3GPP TSG-RAN WG1 #98	R1- 1909027

Prague, Czech Republic, August 26th– 30th, 2019

Agenda Item:	7.2.3.4

Source:	Ericsson

Title:	IAB OTA Timing Alignment - Signalling

Document for:	Discussion

# Introduction

In the RAN1 AdHoc 1901 Meeting, the following agreements on OTA timing alignment were reached [1].

Agreements: An IAB node should set its DL TX timing ahead of its DL Rx timing by TA/2 + T_delta - T_delta is signalled from the parent node, where the value is intended to account for factors such the offset between parent DL Tx and UL Rx, if any due to factors such as Tx to Rx switching time, HW impairments, etc. - TA is the timing gap between UL Tx timing and DL Rx timing, which is derived based on existing Rel-15 mechanism

In the RAN1 #96 Meeting, the following objectives of the IAB work item were agreed [2].

Agreements: - T_delta is indicated by a parent to the child node independently from the existing Rel.15 TA indication from the parent node used to set the UL Tx timing of the child IAB node’s MT

There are currently ongoing discussions on what signalling should be used to provide information about T_delta from the parent node to an IAB-node. Concrete signalling options have been proposed by different parties during the RAN1 #97 Meeting and a discussion is proposed for the RAN1 #98 Meeting on what signalling option to choose.

Any RAN1 preference for a T_delta signalling option should consider different requirements, which we want to discuss.

# Requirements on T_delta Signalling

It has been agreed that the OTA timing alignment basically relies on three different parameters; DL-Rx timing, the timing advance TA as effective at the MT side and adjusted by the parent node and T_delta, a parameter signalled from the parent node.

Only the DL-Rx timing is determined IAB-node internally. The timing advance TA is mainly provided through MAC CE. MAC CE messages are not protected, i.e. there is no integrity/authenticity mechanism, as the messages are exchanged below existing secure protocol layer such as PDCP and F1/NDS, which terminate higher up in the protocol stacks.

Currently there is no support for protecting MAC-based inter-IAB node signalling.

Also, MAC CE messages have not the highest reliability, such as for example control layer messages, which undergo additional feedback loops (RLC-ARQ). However, any change of the timing advance can be observed by a parent IAB node through observing its UL-Rx timing. This, effectively, establishes a feedback loop that would a parent node allow to observe, certainly to correct any wrong or missing TA information in short time.

According to agreements, T_delta is indicated by a parent to the child node independently from the existing TA indication. Also, any error, missing T_delta change-signalling or altering, e.g. by an attack, of T_delta value would have immediate impact on the IAB-node’s DL-Tx timing correctness, and in the following on the DL-Tx timing synchronization performance of all child nodes (both in accuracy and stability). The affected IAB-branch can turn to a major network interference source, affecting performance and RAN reliability.

A difference between the parent node and IAB-node in the assumption about T_delta can have a significant impact on the performance and reliability of RAN.

Due to the IAB half-duplex constraint, a parent node cannot measure its child nodes’ DL-Tx timing, nor have any feedback signalling methods for the DL-Tx timing adjustment been agreed on.

There exist no feedback or measurement mechanism to correct or at least detect significant DL-Tx timing deviations of an IAB-node due to incorrect node-internal T_delta assumptions.

T_delta is assumed to be basically related to the switching gap, Tg, between UL Rx and DL Tx at the parent node [3]. Tg, as an IAB-node internal, possibly implementation related parameter, which does not depend very much on the IAB operation condition itself, can be assumed to not change very frequently. Different to the TA update for (possibly highly) mobile Rel-15 UEs, which requires a certain faster signalling of the TA change, an update of T_delta setting can afford a higher delay, as long as a certain coordination in time of a T_delta change can be guaranteed – if not by timely reception, but by agreeable application procedures. The role of Tg, and therefore T_delta, does not include aspects of dynamicity or non-stationary operation.

T_delta will likely not change frequently, and its signalling has reduced delay requirements, compared to uplink timing advance control.

A broadcasting approach for informing child nodes about T_delta could ensure a frequent provisioning. However, broadcasting, e.g. via MIB/SIB1, could be considered complex (in terms of expensive using a sparse resource type for IAB purposes only) and has significant specification impact. It is also questionable whether this approach is future proof for changes and compatibility.

A broadcasting-based method of T_delta has higher specification impact and requires specification coordination beyond the IAB WI.

Based on the observations, we see no need for IAB-specific changes or improvements with respect to timing advance related signalling. Any signalling specification of T_delta should consider aspects of security, mainly in terms of integrity and authenticity (not necessarily privacy), and reliability.

Any signalling specification of T_delta should provide at least aspects of security, mainly in terms of integrity and authenticity (not necessarily privacy), and reliability.

Our assumption is that, in principle, the IAB-node is provided with T_delta information during the phase 2 of the IAB-node integration procedure [4]. During this phase the IAB-node DU is not active. Therefore, no coordination is needed between parent and IAB-node regarding when the IAB-node should apply the T_delta information in setting its DL-Tx timing and start using this timing setting when transmitting – the parent can know that the IAB-node DU is not yet (but becoming) active, and the IAB-node knows the parent node’s DU is already active. Currently, there are no agreements made on how parent node and IAB-node can coordinate the application of a possibly updated/changed T_delta, when both nodes’ DUs are active. Uncoordinated OTA sync assumptions can lead to significant impact on the performance and reliability of RAN, as we already observed in Observation 2.

Currently, there are no agreements made on how parent node and IAB-node can coordinate the application of a possibly updated/changed T_delta, when both nodes’ DUs are active.

An IAB-node receives information about T_delta and applies T_delta information in its DL-Tx timing determination only when its DU is not active.

- We think RAN1 should agree on, summarize and provide information about the important properties and characteristics of T_delta to RAN2. In the following, RAN2 will specify appropriate signalling for T_delta, based on RAN1 requirements.

RAN1 should summarize and provide information regarding important properties and characteristics of T_delta to RAN2.

The signalling design for a parent node signalling T_delta to an IAB-node is up to RAN2.

# Conclusion

In this contribution, we make the following observations:

Observation 1	Currently there is no support for protecting MAC-based inter-IAB node signalling.

Observation 2	A difference between the parent node and IAB-node in the assumption about T_delta can have a significant impact on the performance and reliability of RAN.

Observation 3	There exist no feedback or measurement mechanism to correct or at least detect significant DL-Tx timing deviations of an IAB-node due to incorrect node-internal T_delta assumptions.

Observation 4	T_delta will likely not change frequently, and its signalling has reduced delay requirements, compared to timing advance control.

Observation 5	A broadcasting-based method of T_delta has higher specification impact and requires specification coordination beyond the IAB WI.

Observation 6	Currently, there are no agreements made on how parent node and IAB-node can coordinate the application of a possibly updated/changed T_delta, when both nodes’ DUs are active.

In this contribution, we propose the following:

Proposal 1	Any signalling specification of T_delta should at least consider aspects of security, mainly in terms of integrity and authenticity (not necessarily privacy), and reliability.

Proposal 2	An IAB-node receives information about T_delta and applies T_delta information in its DL-Tx timing determination only when its DU is not active.

Proposal 3	RAN1 should summarize and provide information regarding important properties and characteristics of T_delta to RAN2.

Proposal 4	The signaling design for a parent node signalling T_delta to an IAB-node is up to RAN2.

# References

RAN1, Chairman's Notes RAN1 AdHoc 1901 final

RAN1, Chairman's Notes RAN1#96 final

R1-1905842, LS on clarification of OTA timing alignment for IAB

3GPP TR 38.874 V16.0.0, Study on Integrated Access and Backhaul