3GPP TSG-RAN WG1 #98		R1-1909044

Prague, Czech Republic, August 26th – 30th 2019

Source: 	Apple Inc.

Title:                     On HARQ and scheduling enhancements for NR-U

Agenda item:	7.2.2.2.3

Document for:	Discussion and Decision

# 1 Introduction

At the RAN1 #97 meeting, enhancement on HARQ and scheduling operation for NR-U was further progressed with following agreements [1]:

Agreement: For multiple PUSCHs scheduled by a single DCI - The following fields in the DCI are signalled per PUSCH - NDI - RV (FFS: compression scheme for RV) - FFS if CBG-based re-transmission is supported for multi-TTI scheduling, at least the following options are considered for signalling the CBGTI field in the DCI - Option 1: per re-transmitted PUSCH - FFS: limitations on number of re-transmitted PUSCH for which CBGTI field is signalled - Option 2: per PUSCH - Option 3: only for a fixed number of PUSCHs - HARQ process ID signalled in the DCI applies to the first scheduled PUSCH. HARQ process ID is then incremented by 1 for subsequent PUSCHs in the scheduled order (with modulo operation as needed) - Time domain resource assignment mechanism is enhanced for indicating the number of scheduled mapping Type A and Type B PUSCHs, and their starting and ending symbols - At least continuous time domain resource assignment is supported - FFS: whether multiple mapping Type B PUSCHs is allowed within the first slot - FFS: whether multiple starting positions is allowed for UE-initiated COT (discussed as part of agenda 7.2.2.1.3) - FFS: configuration/signalling details - CSI request field in the DCI applies to a single PUSCH - FFS: relation between the timing of the triggered CSI-RS and the PUSCH carrying the CSI feedback, and how to determine which PUSCH carries the CSI feedback - FFS: Whether existing DCI formats can be extended or new formats are necessary and associated details for supporting scheduling multiple PUSCHs - FFS: Potential impact on MCS signalling for re-transmission - Note: Indication of the LBT type and priority class should be possible with the DCI - Note: SRS request field in the DCI applies to a single slot with the applicable slot signalled as in Rel-15 - Note: The number of DAI fields is not increased as compared to the single PUSCH scheduling DCI in Rel-16. Applicability of the DAI field is the same as multi-slot PUSCH scheduling in Rel-15. Agreement: For operation with dynamic HARQ codebook (type-2 codebook): - PDSCH grouping by explicitly signalling a group index in DCI scheduling the PDSCH - For any PDSCH scheduled with numerical or non-numerical value of K1 - The number of HARQ-ACK bits for one PDSCH group can change between successive requests for HARQ-ACK feedback for the same PDSCH group - HARQ-ACK feedback for all PDSCHs in the same group is carried in the same PUCCH - One DCI can request HARQ-ACK feedback for one or more PDSCH groups in the same PUCCH - C-DAI/T-DAI is accumulated only within each PDSCH group - FFS: Choose between the following options: - T-DAI is included only for the scheduled group - T-DAI is included for each group - New ACK-Feedback Group Indicator for each PDSCH Group operates as a toggle bit - Maximum number of PDSCH groups: 2 (FFS: maximum number of groups 4 and maximum number of groups for which feedback is requested in the same PUCCH) - A UE can signal support of this feature as part of capability signalling

In this contribution, we provide our views on the FFS aspects of potential enhancements for NR-U operation, focusing on HARQ and scheduling areas.

# 2. Discussion

## 2.1 Multi-TTI scheduling

Time domain resource allocation

One of remaining issues for multi-TTI scheduling is to design the detailed signaling in a single scheduling DCI. In Rel-15, NR supports multi-slots UL/DL transmission direction indication by using a single DCI format 2-0. This mechanism can be extended for multi-slot scheduling to support both contiguous and non-contiguous (i.e. with gap in between) time domain resource allocation without increasing signaling overhead. This unified signaling approach provides additional scheduling flexibility to network as it allows e.g. DL signaling transmission in the middle of multi-TTI UL transmissions. Like SFI indication, the TDRA field in multi-TTI scheduling DCI can be viewed as a pointer into an RRC-configured table where each row in the table is constructed from a set of Rel-15 TDRA values. Moreover, this approach offers the full freedom to gNB to select PUSCH types (i.e. Type A or Type B) and numbers within a first slot based on different use cases. Therefore, we make the following proposal:

Proposal 1:

- Consider support time domain resource allocation for multi-TTI scheduling with each TDRA IE in scheduling DCI to indicate a set of concatenated Rel-15 TDRA values.

RV values

With respect to the RV value, our preference is to explicitly indicate 1-bit RV field per PUSCH to provide certain degree of the scheduling flexibility at acceptable DL control overhead for HARQ operation. Generally, RV #0 is self-decodable and beneficial to use in case the initial PUSCH transmission was not received by network. If the initial transmission was actually received by network, RV #3 can achieve much better performance than that of RV#0 due to incremental combination. 1-bit RV field allows network to select a proper RV i.e. RV #0 or RV #3 based on the PUSCH detection result. Therefore, we propose:

Proposal 2:

- Support 1-bit RV field per PUSCH transmission for multi-TTI scheduling.

CBG-based HARQ operation

NR support CBG-based retransmission that is especially beneficial for a large TB when it is partially interfered by another busty preempting transmission and only a few of CBs in the TB are corrupted. If per-CBG retransmission is configured, HARQ-ACK feedback is provided per CBG and only the erroneously received CBGs are then retransmitted. It can consume less resource for PUSCH retransmission and improve the resource efficiency. This benefit is hold for NR-U operation as well and motivates to support CBG-based HARQ operation for NR-U.

At the same time, support CBG-based HARQ operation for multi-TTI transmission may result in large DL control signaling overhead. One simple solution to address this concern is to limit the maximum number of CBG-based retransmission that can be scheduled within a single multi-TTI PUSCH transmission to achieve a good cost-performance tradeoff. The maximum number of CBG-based retransmission N can be controlled by network and configured on a per UE basis with support disabling this feature (N=0), e.g. based on the channel contention status. In addition, for a single multi-TTI PUSCH, the HARQ processes selected for CBG-based retransmission can be explicitly indicated as part of scheduling DCI format. Considering an example of multi-TTI transmission with 4 PUSCHs and assuming 2 CBG-based transmission, this results in i=02(4i)=14 possibilities. Assuming number of CBGs per TB is 4, it causes 4+2x4 =12 bits signal overhead.

Proposal 3:

- Support CBG-based HARQ retransmission for NR

- The maximum number of HARQ processes with CBG-based retransmission is configured by RRC signaling.

- HARQ-ACK processes associated with CBG-based transmission in a multi-TTI transmission is explicitly indicated by separate field in scheduling DCI format.

## 2.2 HARQ-ACK feedback

T-DAI signaling

Several agreements were made in last meeting [1] to enhance the Rel-15 dynamic HARQ-ACK codebook design, focusing on mechanisms to provide possibility for UE with cross-cot and cross-PDSCH-group HARQ-ACK feedback. This can be achieved by allowing one scheduling DCI to request HARQ-ACK feedback for one or more PDSCH groups in the same PUCCH.

Same as in LTE, two types of DAI are defined for dynamic HARQ-ACK codebook design in Rel-15 NR and signalled as part of DCI format, one is counter DAI (C-DAI) and the other is total DAI (T-DAI). The C-DAI field is used to signal the number of scheduled PDSCH up to the time instance that the DCI was received. The T-DAI indicates the total number of DL transmission across all CCs up to this point in time.

For NR-U operation, it was further agreed that C-DAI and T-DAI is accumulated per PDSCH group. One of FFS aspects for dynamic HARQ-ACK codebook feedback enhancement for NR-U is how to signal the T-DAI IE in the scheduling DCI format. Two options were identified in the past meeting:

- Alt.1: T-DAI is included only for the scheduled group

- Alt.2: T-DAI is included for each group

Alt.1 causes two bits smaller signalling overhead, but it has HARQ-ACK codebook size misalignment problem in case UE missed the last PDCCH detection of another PDSCH group. More seriously, this failure in one group has direct impacts on the HARQ-ACK bits detection of the scheduled PDSCH group due to the concatenation of two HARQ-ACK codebooks that are associated with two PDSCH group. FIG.1 provides an example of codebook size misalignment problem where UE misses the last PDCCH of group 1. This issue can be addressed with Alt.2 by inclusion of 2-bit T-DAI of the other PDSCH groups in the scheduling DCI format if the PUCCH additionally conveys the HARQ-ACK bits for other PDSCH groups. Hence, we proposed the following:

Figure 1: HARQ-ACK codebook size misalignments in case of cross-group HARQ-ACK feedback

Proposal 4:

- T-DAI is included for each PDSCH group to address the misalignment problem of HARQ-ACK codebook.

# 3. Conclusion

In this contribution, we discussed different aspects of HARQ-ACK feedback enhancements and multi-TTI scheduling. Based on the discussions in the previous sections, we make the following proposals:

Proposal 1:

- Consider support time domain resource allocation for multi-TTI scheduling with each TDRA IE in scheduling DCI to indicate a set of concatenated Rel-15 TDRA values.

Proposal 2:

- Support 1-bit RV field per PUSCH transmission for multi-TTI scheduling.

Proposal 3:

- Support CBG-based HARQ retransmission for NR

- The maximum number of HARQ processes with CBG-based retransmission is configured by RRC signaling.

- HARQ-ACK processes associated with CBG-based transmission in a multi-TTI transmission is explicitly indicated by separate field in scheduling DCI format.

Proposal 4:

- T-DAI is included for each PDSCH group to address the misalignment problem of HARQ-ACK codebook.

# References

- RAN1 Chairman’s Notes, RAN1 #97, May 2019