3GPP TSG RAN WG1 #98			                                                                  R1-1909068

Prague, CZ, August 26th – 30th, 2019

Agenda Item:	7.2.3.3

Source:	AT&T

Title:	Mechanisms for supporting access and backhaul link multiplexing

Document for:	Discussion/Approval

# Introduction

As part of the Work Item on Integrated Access and Backhaul for NR [1], 3GPP has agreed to support solutions for efficient operation of integrated access and wireless backhaul for NR including the following requirements:

- Specification of mechanisms for resource multiplexing among backhaul and access links. This includes:

  - Specification of semi-static configuration for IAB-node/IAB-donor DU resources in case of TDM operation subject to half-duplex constraint. This shall be forward compatible to allow the support of half-duplex scenarios with FDM and SDM resource sharing among backhaul and access links.

  - Specification of time resource types for the DU’s child links: DL hard, DL soft, UL hard, UL soft, Flexible hard, Flexible soft, Not Available

  - Specification of dynamic indication (L1 signalling) of the availability of soft resources for a child IAB-node DU

  - Specification of required transmission/reception rules for IAB-nodes and associated behaviours regarding time resource utilization as discussed in TR 38.874 clause 7.3.3.

This contribution discusses the design considerations for the multiplexing of access and backhaul links.

# Frame Structure Design for Access and Backhaul Multiplexing

An example of a network with integrated access and backhaul links is shown in Figure 1 below. The operation of the different links may be on the same or different frequencies (also termed ‘in-band’ and ‘out-band’ relays). Each IAB node has both DU functionality (IAB-DU) as well as UE-like functionality (IAB-MT). The relay is connected to an IAB node of a higher hop order in a similar way as a UE, while the IAB node serves relay UEs inside IAB nodes of lower hop orders or its own access UEs.

Figure 1: Integrated access and backhaul links

In addition, for in-band operation IAB nodes are assumed to operate with a half-duplex constraint as shown in Figure 2, which means they can only do the following at any given time:

- Receive on the access link (UE to IAB node) and/or backhaul link (IAB node to IAB node)

- Transmit on the access link and/or backhaul link

Figure 2: Half-duplex constraint at the relay

A key advantage of IAB is that backhaul and access are integrated and multiplexed in the scheduler, allowing very dynamic resource allocation between the backhaul and access links (in both DL and UL directions). Even though the IAB relay node consists of two logical nodes each with its own protocol stack, the physical transceiver is shared between them. In other words the PHY of the MT and the PHY of the DU run on the same transceiver systems. The backhaul link and the access link can therefore be multiplexed in the following manner:

- Time Division Multiplexing: The access link and the backhaul link are time multiplexed with each other. This implies that IAB-MT PHY and IAB-DU PHY are not active simultaneously.

- Frequency Division Multiplexing: The access link and the backhaul link are active at the same time but on different frequency resources, e.g. on separate CC or on separate PRB on the same CC.

- Spatial Multiplexing: The access link and the backhaul link are active at the same time on the same frequency resources. This case further be divided in to the two following cases:

  - Intra Panel SDM: The access and backhaul use the same panel but different spatial layers

  - Inter Panel SDM: The access and backhaul use different panels

TDM multiplexing is the main priority of the IAB WI, however forward compatibility towards support of FDM/SDM must also be ensured in the design of the signalling and coordination mechanisms, as shown in Figure 3 where a frame-structure across the multiple hops is staggered to achieve this objective.

The remaining sections of this document focus on solutions for IAB to enable efficient multiplexing of backhaul and access links operating on the same frequency to accommodate half-duplex constraints and avoid/mitigate interference.

Figure 3: IAB frame structure

## Semi-static frame structure coordination

During RAN1#97 the following agreements were made:

Agreements:

For the semi-static DU resource configuration, the following is supported:

- Indication of D/U/F resources in the semi-static DU resource configuration includes the following:

  - The flexibility to configure all of the slot patterns and formats supported by the existing Rel-15 TDD-UL-DL-Config RRC configurations and slot format table defined in Table 11.1.1-1 in TS38.213

    - FFS: additional pattern durations than supported in Rel-15

    - FFS: default resources or pattern

  - New slot formats defined only for IAB nodes (DU and MTs) which begin with uplink slots, uplink symbols, or flexible symbols.

    - Note: usage of these slot formats should be compatible with Rel-15 access UEs sharing the same link

    - FFS: whether these slot formats also need to be included in the MT RRC configuration and/or SFI carried on DCI Format 2_0

The new TDD-UL-DL-Pattern configurations should allow for patterns that start with uplink slots, uplink symbols, or flexible symbols unlike the majority of the patterns defined in Rel-15 which start with downlink symbols. This has the benefit of enabling a straightforward shift of the alignment of frame structures across hops as shown in Figure 3, which is needed for both TDM as well as SDM access and backhaul multiplexing.

The existing supported slot formats are provided in 38.213 Table 11.1.1-1

Table 11.1.1-1: Slot formats for normal cyclic prefix

| Format | Symbol number in a slot |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 |
| 0 | D | D | D | D | D | D | D | D | D | D | D | D | D | D |
| 1 | U | U | U | U | U | U | U | U | U | U | U | U | U | U |
| 2 | F | F | F | F | F | F | F | F | F | F | F | F | F | F |
| 3 | D | D | D | D | D | D | D | D | D | D | D | D | D | F |
| 4 | D | D | D | D | D | D | D | D | D | D | D | D | F | F |
| 5 | D | D | D | D | D | D | D | D | D | D | D | F | F | F |
| 6 | D | D | D | D | D | D | D | D | D | D | F | F | F | F |
| 7 | D | D | D | D | D | D | D | D | D | F | F | F | F | F |
| 8 | F | F | F | F | F | F | F | F | F | F | F | F | F | U |
| 9 | F | F | F | F | F | F | F | F | F | F | F | F | U | U |
| 10 | F | U | U | U | U | U | U | U | U | U | U | U | U | U |
| 11 | F | F | U | U | U | U | U | U | U | U | U | U | U | U |
| 12 | F | F | F | U | U | U | U | U | U | U | U | U | U | U |
| 13 | F | F | F | F | U | U | U | U | U | U | U | U | U | U |
| 14 | F | F | F | F | F | U | U | U | U | U | U | U | U | U |
| 15 | F | F | F | F | F | F | U | U | U | U | U | U | U | U |
| 16 | D | F | F | F | F | F | F | F | F | F | F | F | F | F |
| 17 | D | D | F | F | F | F | F | F | F | F | F | F | F | F |
| 18 | D | D | D | F | F | F | F | F | F | F | F | F | F | F |
| 19 | D | F | F | F | F | F | F | F | F | F | F | F | F | U |
| 20 | D | D | F | F | F | F | F | F | F | F | F | F | F | U |
| 21 | D | D | D | F | F | F | F | F | F | F | F | F | F | U |
| 22 | D | F | F | F | F | F | F | F | F | F | F | F | U | U |
| 23 | D | D | F | F | F | F | F | F | F | F | F | F | U | U |
| 24 | D | D | D | F | F | F | F | F | F | F | F | F | U | U |
| 25 | D | F | F | F | F | F | F | F | F | F | F | U | U | U |
| 26 | D | D | F | F | F | F | F | F | F | F | F | U | U | U |
| 27 | D | D | D | F | F | F | F | F | F | F | F | U | U | U |
| 28 | D | D | D | D | D | D | D | D | D | D | D | D | F | U |
| 29 | D | D | D | D | D | D | D | D | D | D | D | F | F | U |
| 30 | D | D | D | D | D | D | D | D | D | D | F | F | F | U |
| 31 | D | D | D | D | D | D | D | D | D | D | D | F | U | U |
| 32 | D | D | D | D | D | D | D | D | D | D | F | F | U | U |
| 33 | D | D | D | D | D | D | D | D | D | F | F | F | U | U |
| 34 | D | F | U | U | U | U | U | U | U | U | U | U | U | U |
| 35 | D | D | F | U | U | U | U | U | U | U | U | U | U | U |
| 36 | D | D | D | F | U | U | U | U | U | U | U | U | U | U |
| 37 | D | F | F | U | U | U | U | U | U | U | U | U | U | U |
| 38 | D | D | F | F | U | U | U | U | U | U | U | U | U | U |
| 39 | D | D | D | F | F | U | U | U | U | U | U | U | U | U |
| 40 | D | F | F | F | U | U | U | U | U | U | U | U | U | U |
| 41 | D | D | F | F | F | U | U | U | U | U | U | U | U | U |
| 42 | D | D | D | F | F | F | U | U | U | U | U | U | U | U |
| 43 | D | D | D | D | D | D | D | D | D | F | F | F | F | U |
| 44 | D | D | D | D | D | D | F | F | F | F | F | F | U | U |
| 45 | D | D | D | D | D | D | F | F | U | U | U | U | U | U |
| 46 | D | D | D | D | D | F | U | D | D | D | D | D | F | U |
| 47 | D | D | F | U | U | U | U | D | D | F | U | U | U | U |
| 48 | D | F | U | U | U | U | U | D | F | U | U | U | U | U |
| 49 | D | D | D | D | F | F | U | D | D | D | D | F | F | U |
| 50 | D | D | F | F | U | U | U | D | D | F | F | U | U | U |
| 51 | D | F | F | U | U | U | U | D | F | F | U | U | U | U |
| 52 | D | F | F | F | F | F | U | D | F | F | F | F | F | U |
| 53 | D | D | F | F | F | F | U | D | D | F | F | F | F | U |
| 54 | F | F | F | F | F | F | F | D | D | D | D | D | D | D |
| 55 | D | D | F | F | F | U | U | U | D | D | D | D | D | D |
| 56 – 254 | Reserved |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 255 | UE determines the slot format for the slot based on TDD-UL-DL-ConfigurationCommon, or TDD-UL-DL-ConfigDedicated and, if any, on detected DCI formats |  |  |  |  |  |  |  |  |  |  |  |  |  |

At least the following slot formats should be defined in Rel.16 for DUs and MTs:

| F | F | F | F | F | F | F | U | U | U | U | U | U | U |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| U | U | U | U | U | U | U | F | F | F | F | F | F | F |
| U | U | U | U | U | U | U | D | D | D | D | D | D | D |

Note that the following symmetric 7-symbol allocation is already supported in 38.213:

| F | F | F | F | F | F | F | D | D | D | D | D | D | D |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

In addition, to support mini-slots with PDSCH duration less than 7 symbols, the slot formats such as rows 10-15 and 34-42 should be supported, based on the existing formats starting with D or F, but flipping the order of the 7-symbol sub-slots.

Proposal 1: The following new slot formats should be defined for use by IAB-nodes (DUs and MTs) in the semi-static F1-AP and RRC configurations, and as part the SFI in DCI Format 2_0:

| F | F | F | F | F | F | F | U | U | U | U | U | U | U |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| U | U | U | U | U | U | U | F | F | F | F | F | F | F |
| U | U | U | U | U | U | U | D | D | D | D | D | D | D |
| U | U | U | U | U | U | U | F | U | U | U | U | U | U |
| U | U | U | U | U | U | U | F | F | U | U | U | U | U |
| U | U | U | U | U | U | U | F | F | F | U | U | U | U |
| U | U | U | U | U | U | U | F | F | F | F | U | U | U |
| U | U | U | U | U | U | U | F | F | F | F | F | U | U |
| U | U | U | U | U | U | U | F | F | F | F | F | F | U |
| U | U | U | U | U | U | U | F | U | U | U | U | U | U |
| U | U | U | U | U | U | U | F | F | U | U | U | U | U |
| U | U | U | U | U | U | U | F | U | U | U | U | U | U |
| U | U | U | U | U | U | U | D | F | U | U | U | U | U |
| U | U | U | U | U | U | U | D | D | F | U | U | U | U |
| U | U | U | U | U | U | U | D | D | D | F | U | U | U |
| U | U | U | U | U | U | U | D | F | F | U | U | U | U |
| U | U | U | U | U | U | U | D | D | F | F | U | U | U |
| U | U | U | U | U | U | U | D | D | D | F | F | U | U |
| U | U | U | U | U | U | U | D | F | F | F | U | U | U |
| U | U | U | U | U | U | U | D | D | F | F | F | U | U |
| U | U | U | U | U | U | U | D | D | D | F | F | F | U |

## Semi-static frame structure coordination

During RAN1#97 the following agreements were made:

Agreements:

For the semi-static DU resource configuration, the following is supported:

- Indication of H/S/NA for the DU resource configuration is based on one of the following alternatives:

  - Alt. 1: H/S/NA is additionally explicitly indicated per-resource type (D/U/F) in each slot

    - To handle potential misalignment in time of the configured DU and MT resources when determining the validity of H/S/NA at the DU one of the following sub-alternatives need to be supported:

      - 1a: H/S/NA is applied relative to the DU resource configuration (D/U/F) slot timing without considering the MT resource configuration or timing.

        - FFS: definition of additional restrictions on the usage of the semi-static configuration (e.g. guard symbols) based on deployment scenario or DL/UL switching times within an IAB node, etc.

        - FFS: How the CU can get information about the required guard symbols for a given DU configuration if needed

      - 1b: H/S/NA is applied relative to the MT resource configuration (D/U/F) slot timing.

        - FFS: Whether and/or how the CU will know the actual H/S/NA resources at the child DU

        - FFS whether S is explicitly indicated or not

      - 1c: H is applied relative to the DU resource configuration (D/U/F) slot timing. S is not explicitly indicated, but implicitly determined by the DU based on whether the corresponding MT configuration indicates the MT resources is F (DU-S). The remaining resources are assumed to be NA at the child DU.

        - FFS: Whether and/or how the CU will know the actual S/NA resources at the child DU

  - Alt. 2: NA is explicitly indicated as a resource type in each slot for both the DU and MT configuration. H/S is not explicitly indicated, but implicitly determined by the DU based on the corresponding MT configuration

    - FFS: how to handle the case where there is not a 1-1 mapping of DUs and MTs in the child IAB node

- Details of the F1-AP signaling design is up to RAN3

In order to achieve TDM partitioning of parent and child links while also enabling FDM or SDM in a forward compatible manner, the following two principles need to be supported in the design of the coordination signalling:

- Explicit configuration of whether a resource for a given DU child link is Hard or Soft

- Explicit indication of whether given time resources are available or not available from the IAB node’s perspective

While implicit indication of available/not-available and hard/soft could be used to support TDM operation, it becomes very challenging to later add support of SDM operation, since additional IAB node behaviour or signalling must be introduced to avoid confusion about the correct utilization of a given resources. This corresponds with Alt.1 from RAN1#97.

Proposal 2: H/S/NA is additionally explicitly indicated per resource type (D/U/F) in each slot.

As additionally described in the RAN1#97 agreements, to handle potential misalignment in time of the configured DU and MT resources when determining the validity of H/S/NA at the DU, the timing of the application of the indicated resources relative to either the DU or MT configuration timing needs to be considered. Since the usage of H/S/NA resources is related to the multiplexing option supported by the network/IAB-node, it is reasonable that the indication can be provided relative to the MT resource configuration slot timing.

This additionally aligns with the explicit indication of soft resources which is also relative to the MT configuration assuming that existing DCI Format 2_0-like signalling is reused. Since the CU applies both the DU and MT resource configurations (via F1-AP and RRC), it should be left to appropriate configuration to avoid any scenarios where an ambiguous or inappropriate configuration may occur (e.g. indication of availability across resource types or conflicting with the enforced duplexing constraint). This solution corresponds with Alt 1b. from RAN1#97.

Proposal 3: H/S/NA is applied relative to the MT resource configuration (D/U/F) slot timing.

## Dynamic frame structure coordination

During RAN1#97 the following conclusion was made:

Conclusion:

The following alternatives are considered (to be down-selected in RAN1#98) for the explicit indication of the availability of soft resources:

- Alt 1) Indicate which MT resources are “IA” for the child DU (DU-IA)

- Alt 2a) Indicate DU-IA and MT resource type (MT-D/MT-U/MT-F)

- Alt 2b) Indicate DU-IA and DU resource type (DU-D/DU-U/DU-F)

- Alt 3) Jointly or separately indicate DU-IA, the DU resource type, and/or MT resource type

- FFS: monitoring occasions for the explicit indication at the MT

- FFS: whether the processing time for applying an explicit indication at the child DU is defined or left to implementation.

To support the different resource partitions discussed in this section the network may utilize semi-static TDD UL/DL configurations to coordinate between DU functions of different IAB nodes (e.g. using the F1 interface). However, more dynamic mechanisms for resource coordination is beneficial. Figure 4 gives an example where the DL and UL alternates to ensure the half-duplex constraint is maintained at the IAB node, however during slot t + 1 and slot t + 2, the IAB parent node does not have any DL traffic to schedule (e.g. for access and backhaul links) for the child IAB node. However, since the frame structure is semi-statically coordinated, the IAB DU cannot adapt the frame structure to allow DL transmissions (to access UEs or other IAB nodes) since it is not aware of the potential scheduling of the IAB parent node.

Figure 4: IAB frame structure without coordination

Figure 5: IAB frame structure with dynamic coordination

However, if the IAB parent and IAB node can perform dynamic frame structure coordination (DFSC), the parent node can indicate to the IAB MT function that a set of resources are available or released, overriding the semi-statically coordinated and configured DL/UL resource pattern. The IAB node can internally determine whether to utilize those newly available resources. Figure 5 gives an example of DFSC signalling and coordination, where at the beginning of the DL portion of slot t + 1 the IAB parent node indicates that the remaining DL resources in slot t + 1 and slot t + 2 are available for usage by the IAB DU for DL transmissions instead of UL taking into account delays associated with the reception processing time at the IAB node’s MT and scheduling processing time at the IAB node’s DU.

While the configuration of the “hard” DL/UL/Flexible resources is performed in a semi-static manner using F1/RRC signalling for the DU and MT respectively, in order to perform DFSC, existing Rel.15 signalling mechanisms can be used, namely the use of the GC-PDCCH to indicate the slot format(s) which can be used in the case of soft resources. As described in 38.213 section 11.1.1:

A SFI-index field value in a DCI format 2_0 indicates to a UE a slot format for each slot in a number of slots for each DL BWP or each UL BWP starting from a slot where the UE detects the DCI format 2_0.

…

This corresponds with Alt 2a from RA1#97. When receiving the GC-PDCCH command, the MT applies the updated configuration and passes the indication to the DU which can assume the resources aligning with its soft resources are indicated as available.

Proposal 4: IAB should support explicit indication of soft resource availability by using GC-PDCCH signalling via DCI Format 2-0 from the parent IAB node to the child IAB node to jointly indicate DU-IA and MT resource type (MT-D/MT-U/MT-F).

Finally, the following agreement was made in RAN1#96:

Agreements:

- Both slot and symbol-level multiplexing of access and backhaul links are supported.

In order to support the full flexibility of the utilization of soft resources at a child IAB-node DU in case of symbol-level multiplexing of access and backhaul links, the dynamic explicit indication should be able to be sent on any symbol within a slot, and not just the first few symbols of the slot.

Proposal 5: The explicit indication of soft resource availability can be sent on any symbol within a slot.

# Conclusion

This contribution analyzed mechanisms for IAB link multiplexing. The following proposals were made:

Proposal 1: The following new slot formats should be defined for use by IAB-nodes (DUs and MTs) in the semi-static F1-AP and RRC configurations, and as part the SFI in DCI Format 2_0:

| F | F | F | F | F | F | F | U | U | U | U | U | U | U |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| U | U | U | U | U | U | U | F | F | F | F | F | F | F |
| U | U | U | U | U | U | U | D | D | D | D | D | D | D |
| U | U | U | U | U | U | U | F | U | U | U | U | U | U |
| U | U | U | U | U | U | U | F | F | U | U | U | U | U |
| U | U | U | U | U | U | U | F | F | F | U | U | U | U |
| U | U | U | U | U | U | U | F | F | F | F | U | U | U |
| U | U | U | U | U | U | U | F | F | F | F | F | U | U |
| U | U | U | U | U | U | U | F | F | F | F | F | F | U |
| U | U | U | U | U | U | U | F | U | U | U | U | U | U |
| U | U | U | U | U | U | U | F | F | U | U | U | U | U |
| U | U | U | U | U | U | U | F | U | U | U | U | U | U |
| U | U | U | U | U | U | U | D | F | U | U | U | U | U |
| U | U | U | U | U | U | U | D | D | F | U | U | U | U |
| U | U | U | U | U | U | U | D | D | D | F | U | U | U |
| U | U | U | U | U | U | U | D | F | F | U | U | U | U |
| U | U | U | U | U | U | U | D | D | F | F | U | U | U |
| U | U | U | U | U | U | U | D | D | D | F | F | U | U |
| U | U | U | U | U | U | U | D | F | F | F | U | U | U |
| U | U | U | U | U | U | U | D | D | F | F | F | U | U |
| U | U | U | U | U | U | U | D | D | D | F | F | F | U |

Proposal 2: H/S/NA is additionally explicitly indicated per resource type (D/U/F) in each slot.

Proposal 3: H/S/NA is applied relative to the MT resource configuration (D/U/F) slot timing.

Proposal 4: IAB should support explicit indication of soft resource availability by using GC-PDCCH signalling via DCI Format 2-0 from the parent IAB node to the child IAB node to jointly indicate DU-IA and MT resource type (MT-D/MT-U/MT-F).

Proposal 5: The explicit indication of soft resource availability can be sent on any symbol within a slot.

# Reference

[1] RP-182882, New WID: Integrated Access and Backhaul for NR, Qualcomm

[2] TR 38.874 1.0.0, Study on Integrated Access and Backhaul