3GPP TSG RAN WG1 Meeting #98	R1- 1909094

Prague, Czech Republic, August 26th – 30th, 2019

Source:	Sharp

Title:	Physical layer structure for NR sidelink

Agenda Item:	7.2.4.1

Document for:	Discussion and Decision

Introduction

In RAN#83, a Rel-16 work item on NR V2X was approved [1]. One of the objectives of the NR V2X WI is to specify sidelink physical layer structure as per the study outcome.

- Support of sidelink signals, channels, bandwidth part, and resource pools [RAN1, RAN2]

In this document, we share our views on a few aspects of physical layer structure for NR sidelink.

This document is a revision of R1-1907216 [2].

Discussion

SL BWP

Simultaneous configuration of SL BWP and UL BWP

The following was agreed during the SI phase,

Agreements: - Configuration for SL BWP is separated from Uu BWP configuration signalling. - UE is not expected to use different numerology in the configured SL BWP and active UL BWP in the same carrier at a given time. - FFS the time scale - FFS relation to DL BWP including initial Uu BWP - FFS relation in terms of frequency location and bandwidth

Regarding simultaneous configuration of SL BWP and UL BWP, our view is that at least in the case of same numerology, the UE should support simultaneous SL and UL transmissions, if the corresponding BWPs are both within the UE TX bandwidth.

Proposal 1: Simultaneous SL BWP operation and UL BWP operation with the same numerology is supported in case both BWPs are confined within the UE TX bandwidth.

SCI design

Single-stage SCI vs. two-stage SCI

The following conclusion was made in RAN1#97,

Conclusion: - If two-stage SCI is supported, the following details are used. - Information related to channel sensing is carried on 1st-stage. - 2nd-stage is decoded by using PSSCH DMRS. - Polar coding used for PDCCH is applied to 2nd-stage - Payload size for 1st-stage in two-stage SCI case is the same for unicast, groupcast, and broadcast in a resource pool. - After decoding the 1st-stage, the receiver does not need to perform blind decoding of 2nd-stage. - FFS other details - Companies are encouraged to perform analysis (e.g., flexibility, complexity, forward compatibility, overhead, spec impact, latency, robustness, reliability, etc.)/evaluations with details of the SCI contents comparing single-stage vs. two-stage SCI. Aim to conclude in RAN1#98

Regarding single-stage SCI vs. two-stage SCI, we prefer a simple design of SCI with reasonable trade-off between performance and complexity, flexibility, and forward compatibility. Therefore, it is preferable to adopt single-stage SCI unless significant issues are found for any of the above aspects. The table below summarizes our review of the issues of single-stage SCI as observed in contributions for RAN1#97 along with our comments to these issues. In short, we don’t see any severe shortcoming of single-stage SCI.

|  | Views shown in contributions for RAN1#97 | Comments from Sharp |
| --- | --- | --- |
| Performance | - R1-1907134 [8] from Ericsson observed 2 dB losses for single-stage SCI. (“Observation 7 Single SCI design for broadcast and unicast/groupcast results in up to 2 dB of performance loss for broadcast.”) - R1-1906436 [9] from Fujitsu observed that single-stage SCI outperforms two-stage SCI, unless the total RBs used in two-stage SCI are significantly larger than those used for single-stage SCI (e.g. 148 vs. 96). - R1-1906074 [3] from Nokia observed 1.5 dB gains for single-stage SCI. - R1-1906571 [6] from Xiaomi observed 1 dB gains for single-stage SCI. - R1-1906457 [4] from ZTE observed at least over 3 dB gains for single-stage SCI. (NOTE 1) - R1-1906553 [5] from MediaTek observed up to 0.5 dB losses for two-stage SCI due to error propagation. | Regarding the “2 dB loss” shown in [8], it should be noted that “single-stage” does not necessarily mean that there is only a single SCI format. For example, there could be two SCI formats, respectively for scheduling PSSCH transmissions requiring and not requiring HARQ feedback. Without this unnecessary “single SCI format” assumption, the so-called 2-dB performance losses claimed in [8] do not exist. (Interestingly, in the same paper [8], multiple SCI formats were assumed for single-stage SCI when discussing complexity, see the entry for complexity in this table.) Regarding the assumptions in Table 1 of [9], it is unclear to us why the SCI fields “Antenna port(s)”, “DMRS sequence initialization”, “PTRS-DMRS association”, “CBGTI” (a total of 9 bits) are only assumed for single-stage SCI and not assumed for two-stage SCI. The performance of single-stage SCI may be under-estimated in [9]. Regarding the up to 0.5 dB losses observed in [5], - It should be noted that this only reflects a fraction of the performance gap between single-stage SCI and two-stage SCI. - It is unclear to us how Figure 1 to Figure 8 in [5] should be interpreted, and how the 0.5 dB losses were derived; in our understanding, the curve with “independent” BLERs for the two stages (i.e. no error propagation from the 1st-stage to the 2nd-stage) should outperform the two-stage case where there is error propagation. However, the figures in [5] showed the opposite. |
| Forward compatibility | - R1-1907134 [8] from Ericsson “Observation 8 Single SCI design restricts future SL enhancements and is not forward compatible.” - R1-1906074 [3] from Nokia “…another drawback of the single-stage SCI/PSCCH structure is the inflexible forward compatibility for the potential new SCI formats that may be needed to support new traffic in the future.” | For a given SCI format, the SCI size could be made (pre-) configurable, meaning a Rel-16 NR V2X UE can already use an SCI size that may only be used in a future release. With configurable SCI sizes for each SCI format there is no problem in introducing SL enhancements in SCI in the future. |
| Complexity | - R1-1907134 [8] from Ericsson “Observation 9 Designing multiple SCI formats for unicast, groupcast and broadcast transmissions will significantly increase the blind decoding complexity of the UE. In a simple example, blind decoding complexity will be doubled if 2 SCI formats are specified.” - R1-1906553 [5] from MediaTek “Observation 1: Up to 40 bits difference between broadcast and unicast/groupcast communications either leads to the inefficient transmission for broadcast communication or the significantly increased blind decoding complexity.” | A NR V2X UE should normally also support NR Uu (e.g. in order to support mode 1 resource allocation). We don’t see any problem in supporting some level of blind decoding (e.g. by using 2 SCI formats) on sidelink especially when the complexity is only a small fraction of what is already supported for NR Uu. |
| Scheduling flexibility | - R1-1907047 [7] from TCL “Observation 2: Two-stage SCI can be utilized for flexible scheduling of different transmissions.” | [7] assumed that the “scheduling flexibility” is provided by the 1st-stage either scheduling a broadcast transmission (i.e. allocating T/F resources for the broadcast transmission) or indicating a 2nd-stage which schedules a unicast/groupcast transmission. However, either this is contradictory to the RAN1 conclusion that the payload size for the 1st-stage is fixed for all cast-types, or it implies heavy padding in case of unicast/groupcast. |

NOTE 1: The over 3 dB gains were read by the sourcing company from Figure 7, 8 and 9 in [4].

Furthermore, we think the following questions have to be clarified for two-stage SCI before any RAN1 decision is taken on single-stage SCI vs. two-stage SCI:

- Is the 2nd-stage necessary for broadcast transmissions (and/or unicast/groupcast transmissions not requiring HARQ feedback) or not?

If the answer is “yes”, the additional “stage” for handling these transmissions seems to introduce additional complexity comparing to single-stage SCI.

If the answer is “no”, as raised in our comment for scheduling flexibility above, either this is contradictory to the RAN1 conclusion that the payload size for the 1st-stage is fixed for all cast-types, or it implies heavy padding in case of unicast/groupcast.

- How is the “PSSCH DMRS” REs (to be used in decoding of the 2nd-stage) known to the UE without knowing the T/F resources allocated to the associated PSSCH?

If the “PSSCH DMRS” REs are only those REs confined within the 2nd-stage PSCCH transmission, no benefit is seen in decoding the 2nd-stage by using “PSSCH DMRS”. Rather, it imposes a major restriction that the DMRS density has to be the same for the 2nd-stage PSCCH and for the associated PSSCH, implying deteriorated performance for the 2nd-stage PSCCH which targets a much lower BLER than the associated PSSCH.

If the “PSSCH DMRS” REs include additional PSSCH DMRS REs, it is unclear how the UE locates these REs without knowing the T/F resources for the associated PSSCH (bearing in mind that these T/F resource are indicated in the 2nd-stage).

- Is decoding of the 2nd-stage required for a sensing UE?

If the answer is “yes”, then one main benefit of two-stage SCI is lost, i.e. a sensing UE has to decode not only the 1st-stage but also the 2nd-stage for the purpose of resource exclusion.

If the answer is “no” (also implied by the RAN1 conclusion that “information related to channel sensing is carried on 1st-stage”), it is unclear how the sensing UE performs T/F resource exclusion since, again, it is expected that T/F resources for the associated PSSCH are indicated in the 2nd-stage, and that T/F resources to be excluded by the sensing UE are closely related to those allocated for the associated PSSCH (otherwise the “information related to channel sensing is carried on 1st-stage” as conveyed in the 1st-stage would have to contain the absolute positions of the reserved T/F resources, implying very big overhead).

Proposal 2: Two-stage SCI is not supported in NR V2X.

Number of SCI formats

In order to support concurrent unicast/groupcast/broadcast sessions, with and without HARQ and various other functionalities (e.g. CSI, resource reservation, QoS etc), it is desirable to define multiple SCI formats.

At least two SCI formats are necessary for scheduling PSSCH: one with HARQ feedback related fields (for unicast/groupcast with HARQ feedback), and the other without HARQ feedback related fields (for unicast/groupcast without HARQ feedback, and broadcast).

Proposal 3: At least two SCI formats are defined for scheduling PSSCH,

- SCI format 1 with HARQ feedback related fields (for unicast/groupcast with HARQ feedback).

- SCI format 2 without HARQ feedback related fields (for unicast/groupcast without HARQ feedback, and broadcast).

Differentiation of cast-types

From the transmitter UE perspective, the cast-type associated with a TB is known to the physical layer along with the TB indicated from higher layers. And from the receiver UE perspective, the cast-type should be known in a way or another after decoding the SCI that scheduled the PSSCH carrying the TB in order to determine e.g. whether HARQ feedback is necessary for the TB. For example, if the cast-type is broadcast, the receiver UE does not schedule any HARQ feedback transmission for the TB, and if the cast-type is unicast, the receiver UE determines whether to schedule a HARQ feedback transmission depending on RRC configurations.

The cast-type can be either encoded in the layer-1 ID, or explicitly contained in an SCI field. The problem of encoding the cast-type in the layer-1 ID is that it further squeezes the layer-1 ID space and increase the possibility of collision. Hence we prefer to explicitly indicate the cast-type in SCI.

Proposal 4: The cast-type is explicitly indicated in SCI,

- For the SCI format with HARQ feedback, the cast-type is used to differentiate between unicast and groupcast.

- For the SCI format without HARQ feedback, the cast-type is used to differentiate between unicast, groupcast and broadcast.

SCI size

An SCI format should be designed in such a way that additional field(s) can be introduced as the NR V2X specifications evolve. On the other hand, it is desirable that a NR V2X UE supporting an old release (e.g. Rel-16) of specifications is able to decode and understand at least some fields (e.g. the resource reservation field) of an enhanced SCI format introduced in a future release (e.g. Rel-17). This can be achieved by e.g. specifying a (pre-) configurable SCI size for any SCI format. For example, for an SCI format specified in Rel-16 with a total number of payload bits to be 36, the size of the SCI format can be configured to be significantly larger than 36 already in Rel-16, such that a Rel-16 UE can decode an enhanced SCI format transmitted by a Rel-17 UE with a total number of payload bits to be e.g. 66 as long as the SCI size is (pre-) configured to be greater than or equal to 66.

Proposal 5: The size of an SCI format is (pre-) configured.

Multiplexing of PSCCH and PSSCH

The following was agreed in RAN1#95,

Working assumption: - Regarding PSCCH / PSSCH multiplexing, at least option 3 is supported for CP-OFDM. - RAN1 assumes that transient period is not needed between symbols containing PSCCH and symbols not containing PSCCH in the supported design of option 3. - FFS how to determine the starting symbol of PSCCH and the associated PSSCH - FFS for other options. e.g. whether some of them are supported to increase PSCCH coverage.

A lot of views have been expressed on this topic since RAN1#95, see e.g. the summary in “issue 3” in [10]. It seems clear that no one has a problem in confirming the support for Option 3, and that there are sustaining different views on whether to additionally support Option 1B and/or Option 2. Given the limited time left for the 5G V2X WI, it is proposed to agree that only Option 3 is supported in Rel-16.

Proposal 6: For PSCCH/PSSCH multiplexing, only Option 3 is supported in Rel-16.

For Option 3, in order to avoid unnecessary signalling overhead, it is proposed to pre-define the relationship between the frequency location of a PSCCH and the associated PSSCH. For example, in case the subchannel size is the same for PSCCH and PSSCH, the central subchannel can be always the same for a given PSCCH and its associated PSSCH. On the other hand, for better flexibility in subchannel configurations, it may be desirable to allow configuring different subchannel sizes for PSCCH and PSSCH.

Proposal 7: For PSCCH/PSSCH multiplexing Option 3, the central frequency of PSCCH and its associated PSSCH is the same.

Proposal 8: The subchannel size is independently configured for PSCCH and PSSCH.

Conclusion

In this contribution, we discuss some issues on physical layer structure in NR sidelink, and make the following proposals:

Proposal 1: Simultaneous SL BWP operation and UL BWP operation with the same numerology is supported in case both BWPs are confined within the UE TX bandwidth.

Proposal 2: Two-stage SCI is not supported in NR V2X.

Proposal 3: At least two SCI formats are defined for scheduling PSSCH,

- SCI format 1 with HARQ feedback related fields (for unicast/groupcast with HARQ feedback).

- SCI format 2 without HARQ feedback related fields (for unicast/groupcast without HARQ feedback, and broadcast).

Proposal 4: The cast-type is explicitly indicated in SCI,

- For the SCI format with HARQ feedback, the cast-type is used to differentiate between unicast and groupcast.

- For the SCI format without HARQ feedback, the cast-type is used to differentiate between unicast, groupcast and broadcast.

Proposal 5: The size of an SCI format is (pre-) configured.

Proposal 6: For PSCCH/PSSCH multiplexing, only Option 3 is supported in Rel-16.

Proposal 7: For PSCCH/PSSCH multiplexing Option 3, the central frequency of PSCCH and its associated PSSCH is the same.

Proposal 8: The subchannel size is independently configured for PSCCH and PSSCH.

References

RP-190766, “New WID on 5G V2X with NR sidelink”, LG Electronics, Huawei, 3GPP RAN#83.

R1-1907216, “Physical layer structure for NR sidelink”, Sharp, 3GPP RAN1#97.

R1-1906074, “Discussion of physical layer structure for sidelink”, Nokia, Nokia Shanghai Bell, 3GPP RAN1#97.

R1-1906457, “NR sidelink physical layer structure”, ZTE, Sanechips, 3GPP RAN1#97.

R1-1906553, “Discussion On sidelink physical layer structure”, MediaTek Inc. , 3GPP RAN1#97.

R1-1906571, “Discussion on physical layer structure for 5G V2X”, Xiaomi Communications, 3GPP RAN1#97.

R1-1907047, “Physical layer structure for sidelink”, TCL Communication Ltd. , 3GPP RAN1#97.

R1-1907134, “PHY layer structure for NR sidelink”, Ericsson, 3GPP RAN1#97.

R1-1906436, “Discussion on physical layer structure for NR sidelink”, Fujitsu, 3GPP RAN1#97.

R1-1907924, “Feature lead summary#4 for 7.2.4.1 Physical layer structure for sidelink”, Samsung, 3GPP RAN1#97.