3GPP TSG-RAN WG1 Meeting #98	R1-1909111

Prague, Czech Republic, 26th – 30th August 2019

Agenda Item:	7.2.5.2

Source:	Ericsson

Title:	On physical layer control procedures for NTN

Document for:	Discussion

# 1	Introduction

In RAN1#97, the following agreements were made related to physical layer control procedures for NTN:

Agreement:

The need and the applicable scenarios for potential enhancements (with respect to the power control schemes in NR Rel-15) for both open-loop and closed-loop power control for NTN are to be studied.

Agreement:

Study the performance of AMC in NTN considering at least the following solutions (some solutions may have no specification impact):

- Prediction-based link adaptation with prediction confidence level

- AMC with CQI reflecting only long-term fading

- Additional BLER targets for CQI reporting to limit number of retransmissions and latency

- CQI offset applied by gNB

- Finer granularity of CQI

- Prediction based CQI reporting

In addition, other issues related to physical layer control procedures for NTN were discussed offline and summarized in [2].  In this contribution, we provide our views on physical layer control procedures for NTN UEs in connected mode.

# 2	Discussion on UL transmission timing for NTN

## The long propagation delays between the gNB and the UE is one of the physical phenomena that requires careful consideration when adapting NR to accommodate NTN.  The one-way delay between the UE and the gNB depends on various factors such as the type of satellite (GEO, LEO, MEO, etc.), the elevation angles associated with the UE and gNB, whether bent pipe or regenerative model is assumed, to name a few.  For the different scenarios considered in TR 38.821[3], the one-way delay can range from ~6ms to ~272ms.

## In NR Release 15, the timing relationships of UCI (uplink control information) transmission in NR PUCCH/PUSCH and of uplink data transmission in PUSCH are designed to be suitable for terrestrial radio propagation environment where the round-trip delay is usually within 1ms.  However, in NTN scenarios, the large propagation delays affect the uplink transmission timing of these physical layer procedures.

There are two types of timing advance (TA) mechanisms in NTN, which we refer to as large TA and small TA. With large TA, each UE has a TA equal to its round-trip time and thus fully compensates its RTT. Accordingly, gNB DL-UL frame timings are aligned.  With large TA, each UE has a TA equal to its round-trip time and thus fully compensates its RTT. This is illustrated in Figure 1. Accordingly, gNB DL-UL frame timings are aligned in the case with large TA.

Figure 1: An illustration of large TA compensating full RTT.

With small TA, each UE has a TA equal to its round-trip time minus a reference round-trip time, i.e., differential RTT. For example, the reference RTT can be the minimum RTT of a cell, and thus the differential RTT of any UE in the cell is always non-negative. With small TA, gNB needs to manage a DL-UL frame timing shift on the order of the reference RTT. In the case of the small TA, it is expected that the gNB broadcasts the reference RTT to the served UEs.

The maximum differential RTT depends on the cell size and may range from sub-millisecond to a few milliseconds. With small TA, gNB needs to manage a DL-UL frame timing shift on the order of the reference RTT, as illustrated in Figure 2.

Figure 2: An illustration of small TA compensating differential RTT.

## Below, we elaborate on the uplink transmission timing of UCI and data transmission.

## 2.1 Transmission timing of HARQ-ACK on PUCCH

HARQ-ACK is transmitted in PUCCH (if not multiplexed on PUSCH) within slot n+K1 under one of the following circumstances:

- if the UE detects a DCI (either DCI format 1_0 or DCI format 1_1) that schedules a PDSCH reception in slot

- if the UE detects a DCI (DCI format 1_0) that indicates SPS PDSCH release with a PDCCH received in slot

The value of K1 which is given in a number of slots, is indicated by the PDSCH-to-HARQ-timing-indicator field in DCI (if the field is present) or is provided by the higher layer parameter dl-DataToUL-ACK.  In the NTN scenarios, which slot a HARQ-ACK is transmitted by the UE will depend on whether small TA or large TA is used.  Figure 3 shows an illustration of HARQ-ACK transmission timing in an NTN scenario, where it is seen that the NTN delay impacts the PUCCH HARQ-ACK transmission timing. Table 1 presents the usable K1 range for HARQ transmission on PUCCH for the two scenarios of small and large TA compensation.  The entries in red-font indicate the following cases:

- Only a small fraction of the current range of K1 values is usable; for example, the K1 range for DCI format 1_0 at 60 kHz numerology with small TA where only the values of 7 and 8 are usable.

- None of the current range of K1 values is usable; this is the case for all entries corresponding to large TA.

Given that the current range of K1 values is not sufficient for NTN scenarios with small TA and large TA, we can consider extending the range of K1 values.  This can be done by either extending the range of K1 values or introducing a configurable offset for HARQ-ACK transmission timing which can be added to K1.  Hence, we make the following proposal:

Proposal 1:  For HARQ-ACK transmission timing with small TA and large TA, either extend the range of K1 values or introduce a configurable offset to compensate for differential RTT and full RTT, respectively.

Figure 3: HARQ-ACK transmission in NTN scenario.

Table 1 Usable K1 range for transmission timing of HARQ-ACK on PUCCH.

| TA scheme | Numerology | Max TA in the unit of slot | Usable K1 range from 1, …, 8 (DCI 1_0) | Usable K1 range from 0, …, 15 (DCI 1_1) |
| --- | --- | --- | --- | --- |
| TA compensating for differential RTT up to 1.6 ms (small TA) | 15 kHz | 1.6 slots | 1, …, 8 | 1, …, 15 |
|  | 30 kHz | 3.2 slots | 3, …, 8 | 3, …, 15 |
|  | 60 kHz | 6.4 slots | 7, 8 | 6, …, 15 |
|  | 120 kHz | 12.8 slots | None | 12, …, 15 |
| TA compensating for full RTT up to 541.46 ms (large TA) | 15 kHz | 541.46 slots | None | None |
|  | 30 kHz | 1082.92 slots | None | None |
|  | 60 kHz | 2165.84 slots | None | None |
|  | 120 kHz | 4331.68 slots | None | None |

## 2.2 Transmission timing for CSI on PUSCH

In NR, two configurations for reporting CSI on PUSCH are possible which are aperiodic CSI reporting and semi-persistent CSI reporting on PUSCH (note that periodic CSI reporting is also supported in NR, but for periodic CSI reporting, the report goes on PUCCH).  Both aperiodic CSI and semi-persistent CSI on PUSCH are activated/triggered by DCI, and the allowed slot offset from the activating/triggering DCI are configured by higher layer parameter reportSlotOffsetList.  The range of slot offsets allowed in reportSlotOffsetList in Release-15 NR is from 0 to 32.  The slot offset K is selected by the activating/triggering DCI.  In the NTN scenarios, which slot a CSI report on PUSCH is transmitted by the UE will depend on whether small TA or large TA is used.  Figure 4 shows an illustration of CSI transmission timing in an NTN scenario, where it is seen that the NTN delay impacts the CSI transmission timing.

Table 2 presents the usable slot offset ranges for aperiodic CSI transmission on PUSCH for the two scenarios of small and large TA compensation.  The red-font entries in the table indicate the entries where none of the current slot offset ranges are usable.

From Table 2, we note that the current range of slot offset values is sufficient for the small TA compensation scenario.  However, the current range of slot offsets is not sufficient for large TA compensation scenario.  Hence, extending the range of slot offset to better support large TA compensation scenario can be considered.  This can be done by either extending the range of slot offset values or introducing a configurable offset which can be added to the slot offset.  Hence, we make the following proposal:

Proposal 2:  For aperiodic CSI transmission timing with large TA, either extend the range of slot offset values or introduce a configurable offset to compensate for full RTT.

Figure 4: Aperiodic CSI transmission in NTN scenario.

Table 2 Usable slot offset ranges for transmission timing of CSI on PUSCH.

| TA scheme | Numerology | Max TA in the unit of slot | Usable slot offset ranges from 0, …, 32 |
| --- | --- | --- | --- |
| TA compensating for differential RTT up to 1.6 ms (small TA) | 15 kHz | 1.6 slots | 1, …, 32 |
|  | 30 kHz | 3.2 slots | 3, …, 32 |
|  | 60 kHz | 6.4 slots | 6, …, 32 |
|  | 120 kHz | 12.8 slots | 12, …., 32 |
| TA compensating for full RTT up to 541.46 ms (large TA) | 15 kHz | 541.46 slots | None |
|  | 30 kHz | 1082.92 slots | None |
|  | 60 kHz | 2165.84 slots | None |
|  | 120 kHz | 4331.68 slots | None |

## 2.3 Transmission timing for data on PUSCH

Dynamic data transmission on PUSCH is scheduled by DCI and the time domain resource assignment field of the DCI provides an index to a table with information on resource allocation in time domain. This information includes, but not limited to, the slot offset K2, the start and length indicator SLIV, or directly the start symbol S and the allocation length L, and the PUSCH mapping type to be applied in the PUSCH transmission.  The slot where the UE shall transmit the PUSCH is determined by K2 as n∙2μPUSCH-μPDCCH+K2, where n is the slot with the scheduling DCI, K2 is based on the numerology of PUSCH, and μPUSCH and μPDCCH are the subcarrier spacing configurations for PUSCH and PDCCH, respectively.  Similar to the problems highlighted in Sections 2.1 and 2.2, which slot the PUSCH is transmitted by the UE will depend on whether small TA or large TA is used.  The value range currently possible in NR Rel-15 for K2 is from 0 to 32.

Similar to the analysis in Table 2, it can be shown that the current range of K2 values is sufficient for the small TA compensation scenario.  However, this range is not sufficient for NTN scenarios with large TA compensation.  Hence, extending the range of K2 values to better support large TA compensation scenario can be considered.  This can be done by either extending the range of K2 values or introducing a configurable offset which can be added to K2.  Hence, we make the following proposal:

Proposal 3:  For PUSCH transmission timing with large TA, either extend the range of K2 values or introduce a configurable offset to compensate for full RTT, respectively.

As discussed above, in the case of the small TA, it is expected that the gNB broadcasts the reference RTT to the served UEs.  Hence, it can be study how to signal the reference RTT in small TA compensation scenario (e.g., broadcasting common RTT on SIB).

Proposal 4:  For small TA, study signaling methods for reference RTT.

# 3	Discussion on CSI for NTN

In this section, we present evaluation results related to CSI in NTN scenarios.  The detailed evaluation assumptions are given in the Appendix.

We first compare the throughput loss resulting from increasing feedback delay.  The results corresponding to UE speeds of 3 km/hr, 30 km/hr, and 60 km/hr are shown in Figures 5, 6, and 7, respectively. We assume that Doppler due to the satellite mobility is compensated for, meaning that the UE mobility is the only factor influencing the Doppler. In these figures, a feedback delay of 1 ms is used. The feedback is limited to CQI reporting (note that we are considering a rank 1 scenario where rank is fixed to 1 and PMI feedback is not needed). From Figure 5, it is noticed that there is notable throughput loss as the feedback delay increases from 1 ms to 40 ms at a UE speed of 3 km/hr.  This is because as the channel ages, the gNB schedules the UE with outdated MCS which in turn results in throughput loss.  However, beyond 40 ms feedback delay, there is no further throughput loss.  The coherence time in the case of Figure 5 is ~180 ms.  So once the feedback delay is beyond a notable fraction of the coherence time, further feedback delays do not hurt the throughput performance further.  In the cases of Figures 6 and 7 with UE speeds of 30 km/hr and 60 km/hr, respectively, the throughput degradation saturates much earlier at around 5 ms feedback delay.  This is because the channel coherence time is much smaller and hence the throughput loss saturation happens much earlier.  Based on these results, we make the following observation:

Observation 1:  There is throughput loss with increasing feedback delay.  However, the throughput loss saturates beyond a certain feedback delay depending on the UE speed.

- At UE speed of 3 km/hr, the throughput loss saturates beyond 40 ms feedback delay.

- At UE speeds of 30 km/hr and 60 km/hr, the throughput loss saturates beyond 5 ms feedback delay.

- At 60km/h the throughput loss is negligible.

Figure 5.  Evaluation results on impact of feedback delay on throughput at UE speed of 3 km/hr

Figure 6.  Evaluation results on impact of feedback delay on throughput at UE speed of 30 km/hr

Figure 7.  Evaluation results on impact of feedback delay on throughput at UE speed of 60 km/hr

One of the schemes proposed in RAN1#97 is ‘AMC with CQI reflecting only long-term fading’.  Hence, we evaluated a scenario where the UE averages channel measurements over consecutive number of CSI-RS periodicities and reports a CSI based on the averaged channel.  Results corresponding to the cases of 3 km/hr and 60 km/hr are shown in Figures 8 and 9, respectively.  In these figures, the feedback delay is fixed at 201 ms, while the number of CSI-RS periodicities over which averaging is performed is varied.  The case with no channel averaging is used as the baseline.  From the results in Figure 8, it is noted that at 3 km/hr UE speed, channel averaging to capture long-term fading does not improve throughput.  At 60 km/hr UE speed (see Figure 9), averaging the channel likely results in pessimistic CSI and thus throughput degrades.  Based on these results, we make the following observation:

Observation 2:  At 3 km/hr UE speed, channel averaging to capture long-term fading does not improve throughput compared to the case with no channel averaging.  At 60 km/hr UE speed, averaging channel likely results in pessimistic CSI and thus throughput degrades compared to the case with no averaging.

Figure 8.  Evaluation results reflecting for channel averaging to capture long-term fading at 3 km/hr

Figure 9.  Evaluation results reflecting for channel averaging to capture long-term fading at 60 km/hr

# 4	Discussion on UL power control for NTN

Even though UL power control was not discussed in RAN1#97, there were some offline discussions as summarized in [2].  According to the summary, some companies think open loop power control should be considered as the baseline for NTN.  However, in NTN scenarios, there are relevant scenarios where pathloss changes drastically in both GEO and LEO scenarios.  One example of such scenarios is blockage.   A satellite ground receiver may first have a clear line of sight to the satellite it is communicating with. If the line of sight to the satellite is blocked by objects such as buildings, trees, bridges, etc., there would be sudden drop in path gains. TR 38.811 models the blocking case by means of a LOS probability and the clutter loss which range up to 44 dB. This implies that the radio link needs to be able to handle quick and drastic changes of the channel strength. In these scenarios it is important to make sure that the UE can quickly adapt the UL power accordingly.  Hence, in addition to open loop power control, closed loop power control may also be relevant in both NTN scenarios.  It should then be studied whether further enhancements are needed to open-loop and/or closed-loop power control in NTN scenarios.

- Observation 3:  In addition to open loop power control, closed loop power control can be relevant in cases where the pathloss drastically changes in which case it is important to ensure that the UE can quickly adapt its UL transmit power.

- Proposal 5:  Study whether enhancements to Rel-15 open-loop and/or closed-loop power control features are needed in NTN scenarios.

# 5	Conclusion

In this contribution, we provide our views on physical layer control procedures for NTN UEs in connected mode.  Based on the discussion in Section 2-4, we make the following observations and proposals:

Observation 1:  There is throughput loss with increasing feedback delay.  However, the throughput loss saturates beyond a certain feedback delay depending on the UE speed.

- At UE speed of 3 km/hr, the throughput loss saturates beyond 40 ms feedback delay.

- At UE speeds of 30 km/hr and 60 km/hr, the throughput loss saturates beyond 5 ms feedback delay.

- At 60km/h the throughput loss is negligible.

Observation 2:  At 3 km/hr UE speed, channel averaging to capture long-term fading does not improve throughput compared to the case with no channel averaging.  At 60 km/hr UE speed, averaging channel likely results in pessimistic CSI and thus throughput degrades compared to the case with no averaging.

- Observation 3:  In addition to open loop power control, closed loop power control can be relevant in cases where the pathloss drastically changes in which case it is important to ensure that the UE can quickly adapt its UL transmit power.

Proposal 1:  For HARQ-ACK transmission timing with small TA and large TA, either extend the range of K1 values or introduce a configurable offset to compensate for differential RTT and full RTT, respectively.

Proposal 2:  For aperiodic CSI transmission timing with large TA, either extend the range of slot offset values or introduce a configurable offset to compensate for full RTT.

Proposal 3:  For PUSCH transmission timing with large TA, either extend the range of K2 values or introduce a configurable offset to compensate for full RTT, respectively.

Proposal 4:  For small TA, study signaling methods for reference RTT.

- Proposal 5:  Study whether enhancements to Rel-15 open-loop and/or closed-loop power control features are needed in NTN scenarios.

# References

Chairman’s Notes RAN1#97, Reno, USA, May 2019.

R1-1907760, ‘Feature lead summary#2 on physical layer control procedures for NTN’, Ericsson, RAN1#97, Reno, USA, May 2019.

3GPP TR 38.821 V0.7.0 (2019-05)

# Appendix: Evaluation Assumptions

| Parameters | Value |
| --- | --- |
| Carrier frequency | 2 GHz |
| Scenario | Suburban (10o Elevation angle) |
| BW | 10 MHz |
| Subcarrier spacing | 15 kHz |
| Number of antennas | 1 at gNB and 2 at UE |
| Pathloss offset of 2nd TRP | 0 dB |
| Channel estimation | Ideal |
| CSI reporting periodicity | 5 ms |
| Time and frequency tracking | Ideal |