3GPP TSG RAN WG1 #98	R1-1909116

Prague, CZ, August 26th – 30th, 2019

Agenda item:	7.2.4.1

Source:		ITL

Title:		Physical layer structure for NR V2X

Document for:	Discussion and Decision

Introduction

In RAN1#97, there has been intensive discussions on the physical layer structure for NR V2X as seen following agreements [1]:

Agreements: - A sequence-based PSFCH format with one symbol (not including AGC training period) is supported. - This is applicable for unicast and groupcast including options 1/2. - Sequence of PUCCH format 0 is the starting point. - FFS: 1 PRB or multiple PRBs is/are used for this PSFCH format - FFS: feasible number of HARQ-ACK bits, mapping of HARQ-ACK bit - FFS whether to support the following formats - X-symbol PSFCH format with a repetition of the one-symbol PSFCH format (not including AGC training period). - E.g. X=2 - A PSFCH format based on PUCCH format 2 - A PSFCH format spanning all available symbols for sidelink in a slot Agreements: - Transmission of PSSCH is mapped onto contiguous PRBs only Agreements: - Sub-channel size is (pre)configurable. - FFS details (e.g., possible sizes, a minimum size etc.) Working assumption: - Rel-15 PDSCH DMRS Configuration type 1 and/or type 2 are reused for frequency-domain pattern of PSSCH DMRS. - FFS whether to support either one or both types - FFS details on multiplexing of different ports for PSSCH DMRS

In this contribution, we discuss about the relevant issues of physical layer structure and provide our proposals and views.

Sidelink BWP

It was agreed in RAN1#AH1901 on SL BWP as following:

Agreements: - Confirm the working assumption - Working assumption: only one SL BWP is configured in a carrier for a NR V2X UE Agreements: - Configuration for SL BWP is separated from Uu BWP configuration signalling. - UE is not expected to use different numerology in the configured SL BWP and active UL BWP in the same carrier at a given time. - FFS the time scale - FFS relation to DL BWP including initial Uu BWP - FFS relation in terms of frequency location and bandwidth

The switching latency between SL BWP and Uu BWP has been discussed in RAN1 and studied in RAN4 as well. In our view, it is desirable that the switching latency between SL BWP and Uu BWP should be avoided so that the resource utilization is increased as much as possible. Therefore, it can be assumed that a UE needs to be implemented by considering a constant RF reception and transmission bandwidth over two BWPs.

Proposal 1:	A UE assumes that there is no switching latency between SL BWP and Uu BWP in a same carrier at a given time

Given that there is only one SL BWP configuration in a carrier, which is separate from Uu BWP configuration signalling, and a UE is not expected to use different numerology between an active SL BWP and an active UL BWP in the same carrier at a given time, a network should ensure to align a single numerology between two BWPs in a carrier. For RRC CONNECTED UE, the numerology alignment may be of course possible with a dedicated RRC signalling signalled by a network, while for IDLE/INACTIVE V2X UEs which has no RRC connection, it may be very challenge, since configurations for initial Uu BWP which consist of cell-specific RRC parameters are primarily provided for e.g. initial access purpose including RACH procedure, which are totally independent from SL BWP configurations. For example, if the numerology of RACH related channels/signals during initial access is imposed to be same as that of SL-BWP in a same carrier, it will negatively impact NR system performance, coverage and so on even for non V2X NR UEs. Moreover, there might be similar situations and limitations even for RRC CONNECTED UEs due to BWP switching via either DCI/timer or RRC signalling even though the network can try to configure it to be same.

To address this issue, there can be two ways. One can be that the NR V2X UE assumes that SL TX/RX operation is deactivated when it switches to the initial Uu BWP of which numerology is different from SL BWP. The other way is that in order to avoid the deactivation of SL TX/RX in above, a network can provide V2X UEs with an additional initial UL BWP configuration only for V2X UEs via system information, which contains at least same numerology as that of SL BWP. That is, NR V2X UE specific initial UL BWP can be newly defined for further flexibility and less V2X performance degradation at least during initial access phase due to potential numerology conflict.

Proposal 2:	It should be discussed how to address the issue related to potential different numerology between initial Uu BWP and SL BWP in a same carrier

Resource pool

Regarding resource pool, it was discussed and there are some related agreements including following one made in RAN1#96 [5]. In time domain resource granularity of resource pool, it is supported to use both slot-level and symbol level resource pool configurations which may be depending on the frequency bands (e.g. ITS band). For further details, it should be discussed whether or not to use only UL symbols configured by cell-specific higher layer signalling or FL (flexible) symbols in addition to UL symbols for NR SL. Given that in Rel-15 NR Uu the dynamic signalling for indicating the slot format can be provided to UEs by a network, which can override that of semi-static signalling, NR SL symbol indication should consider that kind of dynamic nature at least when NR Uu and NR SL are shared in a same carrier if FL symbols is also necessary for NR SL. Therefore, further discussion is needed especially for this issue related to the FL symbols

Agreements:

- For the operation regarding PSSCH, a UE performs either transmission or reception in a slot on a carrier.

- NR sidelink supports for a UE:

  - A case where all the symbols in a slot are available for sidelink.

  - Another case where only a subset of consecutive symbols in a slot is available for sidelink

    - Note: this case is not intended to be used for the ITS spectra, if there is no forward-compatibility issue. Finalize in the WI phase whether there is such an issue or not

    - The subset is NOT dynamically indicated to the UE

    - FFS the supported slot configuration(s)

    - FFS whether/how to operate it in partial coverage scenarios

For frequency domain resource of resource pool, there are two options 1) contiguous PRBs and 2) non-contiguous PRBs. In our view, option 1) contiguous PRBs basically can be preferred since it would allow the simple specification works and flexible/simple resource and interference coordination between NR SL and NR Uu or NR SL resource pools, and there would be no resource fragmentation and better coverage via contiguous PRB allocation. Moreover, if non-contiguous PRB resource in frequency domain is needed to be configured for a UE, then multiple resource pool configurations could be provided to the UE. Therefore, we think contiguous PRBs for the NR V2X resource pool configuration seems be enough in frequency domain.

Proposal 3:	It is preferred to adopt the resource pool consisting of contiguous PRBs in frequency domain.

Furthermore, in last RAN1 meetings, there has been discussion to (pre-)configure SL resource pools based on cast/transmission type (i.e. broadcast/groupcast/unicast). We think that it is not necessary to restrict the SL resource pool with one specific cast/transmission type, since such dimensioning of resource pools specific for a particular cast type would result in resource waste and cannot efficiently address various V2X traffic load in time.

Proposal 4: 	NR V2X supports resource pool configuration without limitation of cast/transmission type.

In LTE V2X, following steps for resource pool indications are specified.

    - 1st step: Among 10240ms, (10240-d1-d2) subframes are calculated.

      - d1: # of subframes for synchronizations

      - d2: # of downlink subframe and special subframe in TDD (d2=0 for FDD)

    - 2nd step: By d3=(10240-d1-d2)mod Lbitmap, (10240-d1-d2-d3) subframes are calculated.

      - Lbitmap: length of indicated bit-map (e.g., 100)

      - d3: # of reserved subframes from that (10240-d1-d2-d3) are divided by Lbitmap

    - 3th step: Apply indicated length Lbitmap bit-map to (10240-d1-d2-d3) subframes

      - the indicated length Lbitmap bit-map is exactly applied by [(10240-d1-d2-d3)/ Lbitmap]times

      - corresponded subframe(1ms) having bit-value 1 belongs to resource pool

In NR V2X, similar to d1, possible time-domain resources for SSBs are not considered for bit-map indication. Furthermore as seen in Figure 2, d3 is not needed if resource pool period(P) is the same or multiple value of SSB period, because 10240ms can be divided by SSB period.

Figure 2. Candidates of resource pool to apply bit-map indication

For the candidates of resource pool as seen in Figure 2, the bit-map indication can be applied. But in this step, slots and/or symbols not used for uplink(similar to d2) is not considered before applying the bit-map indication. However, these could be well coordinated by next step if corresponded time resource unit of each bit in the bit-map is Ams (which corresponds to dl-UL-TransmissionPeriodicity in TDD-UL-DL-ConfigurationCommon), e.g., A=0.625, 1.25 or 2.5.

As seen in Figure 3, it is indicated by the bit-map whether each of Ams time resource unit corresponds to dl-UL-TransmissionPeriodicity can belong to resource pool or not. As the next step, within a each of the Ams time resource unit, symbol-level(or slot-level) resource pool indication could be further applied.

Figure 3. Example of resource pool indication with A ms-level bit-map

Proposal 5: 	The resource pool period(P) is the same or multiple value of SSB period

Proposal 6: 	Ams-level bit-map could be considered for resource pool indication.

- Ams corresponds to dl-UL-TransmissionPeriodicity in TDD-UL-DL-ConfigurationCommon, e.g., A=0.625, 1.25 or 2.5.

PSCCH / PSSCH multiplexing

Figure 4. Options of the PSCCH / PSSCH multiplexing

It was agreed as a working assumption that PSCCH/PSSCH multiplexing with option 3 among the multiplexing options in above figure is at least supported for CP-OFDM as seen in below [2].

Working assumption:

- Regarding PSCCH / PSSCH multiplexing, at least option 3 is supported for CP-OFDM.

  - RAN1 assumes that transient period is not needed between symbols containing PSCCH and symbols not containing PSCCH in the supported design of option 3.

  - FFS how to determine the starting symbol of PSCCH and the associated PSSCH

  - FFS for other options. e.g. whether some of them are supported to increase PSCCH coverage.

The main motivation to support option 3 is to achieve the highest resource utilization between TDM options and the lower latency than FDM (option 2), while option 1B may possibly require a transient period if there is a large power imbalance between PSCCH and PSSCH, because option 1B has a different resource utilization than other options. It was finally agreed in last RAN1 meeting to send a LS to RAN4 to ask the need of the transient period in the above-mentioned options and are waiting for their response [3].

One of remaining questions is whether to additionally support the other options on top of option 3. It has been concerned the PSCCH coverage in option 3 as a possible drawback, since in the extreme cases, the PSCCH coverage may not be enough for NR SL communication. However, it can be compensated by PSCCH power boosting and proper power control mechanism to be defined for NR V2X, while one of the other options (e.g. option 2) can be considered if above approach is not feasible for the reliable PSCCH coverage. Thus, it seems option 3 is currently sufficient to support the variable NR V2X use cases.

Regarding starting symbol of PSCCH and the associated PSSCH, it is related to the PSCCH monitoring occasion and slot/non-slot structure to be used for NR V2X, based on PSCCH/PSSCH multiplexing scheme. In order to reduce UE blind decoding efforts of PSCCH reception in time domain, the candidate starting symbols of PSCCH should be known by a receiving UE. One possible way is to configure the PSCCH monitoring occasion and a slot format during a session/RRC configuration establishment in case of unicast/groupcast, and for the other cases (e.g. broadcast), can be pre-defined or pre-configured if there is no the configuration. This kind of configuration can be associated with a resource pool.

Proposal 7:	Support option 3 only on PSCCH/PSSCH multiplexing

PSFCH format stucture

In last RAN1 meetings, following agreements were made:

RAN1#94bis

Agreements:

Sidelink control information (SCI) is defined.

SCI is transmitted in PSCCH.

SCI includes at least one SCI format which includes the information necessary to decode the corresponding PSSCH.

NDI, if defined, is a part of SCI.

Sidelink feedback control information (SFCI) is defined.

SFCI includes at least one SFCI format which includes HARQ-ACK for the corresponding PSSCH.

FFS whether a solution will use only one of “ACK,” “NACK,” “DTX,” or use a combination of them.

FFS how to include other feedback information (if supported) in SFCI.

FFS how to convey SFCI on sidelink in PSCCH, and/or PSSCH, and/or a new physical sidelink channel

FFS in the context of Mode 1:

whether/how to convey information for SCI on downlink

whether/how to convey information of SFCI on uplink

RAN1#95

Agreements:

- Physical sidelink feedback channel (PSFCH) is defined and it is supported to convey SFCI for unicast and groupcast via PSFCH.

RAN1#AH1901

Agreements:

- For determining the resource of PSFCH containing HARQ feedback, support that the time gap between PSSCH and the associated PSFCH is not signaled via PSCCH at least for modes 2(a)(c)(d) (if respectively supported)

  - FFS whether or not to additionally support other mechanism(s) for modes 2(a)(c)(d)

  - FFS for mode 1

RAN1#96

Agreements:

- At least for sidelink HARQ feedback, NR sidelink supports at least a PSFCH format which uses last symbol(s) available for sidelink in a slot.

Since it was agreed to define PSFCH to convey SFCI for both unicast and groupcast, RAN1 should discuss the details on PSFCH format. Also, in RAN#83, it was agreed that SL-CSI is only delivered using PSSCH using the resource allocation procedure for data transmission. Given that PSFCH can only contain HARQ-ACK feedback, we can focus on PSFCH format design by considering the number of HARQ-ACK information bits.

There can be a PSFCH structure design that it is to modify one or some of NR PUCCH formats(e.g. NR PUCCH format 0/1 or 2), because it has been agreed that HARQ-ACK feedback is delivered by at least PSFCH (and possibly PSSCH as well), while SL CSI feedback is only delivered by PSSCH according to NR V2X WID. Accordingly, it is good starting point to modify the existing NR PUCCH format 0/1 that is targeting to send < 2bit HARQ-ACK information. Same channel coding scheme as NR PUCCH format 0/1 can be fully reused due to same number of feedback information bits. Also, the multiplexing capacity is much higher than other options which may rely on NR shared channel design for higher data rate. In addition, it could be well justified for HARQ-ACK feedback transmission for a groupcast transmission because Tx UE may be able to identify which Rx UE fails to decode the data/control by using same time/frequency resources for PSFCH transmission among Rx UEs, which would allow the Tx UE to identify a certain Rx UE related to the reception of HARQ-ACK feedback among Rx UEs and thus provide better link performance. Furthermore, depending on the how many HARQ-ACK feedback information size are supported for NR V2X, it could be decided whether or not to specify more than one PSFCH format such as PSFCH format 0 (1 or 2bits) and PSFCH format 1 (>2bits). We think it is beneficial to provide flexible HARQ-ACK feedback timing based on the associated PSSCH(s) and HARQ-ACK codebook size determination to V2X UE by introducing at least two PSFCH formats, because NR V2X UE may need to send HARQ-ACK information associated with either a PSSCH or multiple PSSCH, considering various V2X service requirements, use cases and so on.

Therefore, in case of smaller number of HARQ-ACK bits (e.g. ≤2), NR PUCCH format 0 or 1 can be good starting point for one PSFCH format while for larger number of HARQ-ACK bits (e.g. >2), either NR PUCCH format 2 based or PSSCH-based can be considered. Thus, RAN1 needs to consider both options for further comparison when PSFCH details is discussed.

Observation 1:	For PSFCH formats, it is required to support at least two PSFCH formats to accommodate the variable HARQ-ACK information sizes.

Proposal 8:	For larger number of SL HARQ-ACK bits (e.g. >2), a PSFCH format containing more than 2 SL HARQ-ACK bits (e.g. PUCCH format 2) is necessary.

In RAN1#97, a sequence-based PSFCH format with one symbol was agreed with NR PUCCH format 0 sequence as a starting point. Since RAN1 decides to reuse current NR Uu and LTE V2X design as much as possible as agreed in RAN#84, we think most of NR PUCCH format 0 design can be fully reused with some modifications such as layer-1 ID which is used for sequence generation and hopping, when a sequence-based PSFCH format is designed. Therefore, if the layer-1 ID related issue is finalized, it can be directly applied for a sequence-based PSFCH format.

Proposal 9:	It can be considered to reuse most of functionalities of NR PUCCH format 0 for a sequence-based PSFCH format, with some modifications such as how to apply SL layer-1 ID for this PSFCH format.

Regarding on the number of PRB for this PSFCH format, we think there is no big problem to detect this PSFCH format using 1 PRB in terms of AGC. And, a sequence-based PSFCH format seems be targeting to handle up to 2 SL HARQ ACK bits like NR Uu. So, it seems not be necessary to increase the number of PRBs for this PSFCH format. If there is no the further justification to allow the multiple number of PRBs for this PSFCH format, it can be simply assumed a sequence-based PSFCH format is allocated on 1 PRB, similar with NR PUCCH format 0.

Proposal 10:	It is supported to use at least 1 PRB for a sequence-based PSFCH format.

For X-symbol PSFCH format, it can have either a repetition of the one symbol PSFCH format like NR PUCCH 0 or X transmission occasions using one symbol PSFCH format. Since the former has been well evaluated and justified during Rel-15 NR design, it can be simply used. Meanwhile, the latter can increase the multiplexing capacity and flexibility of PSFCH by allowing multiple PSFCH occasions in a slot particularly when performing PSFCH TX to multiple UEs in a same slot. We think both options are beneficial according to the UE’s V2X configuration and channel environments so, one of two options can be (pre-)configured by network.

Proposal 11:	Either a repetition of the one-symbol PSFCH format or X-transmission occasions using one symbol PSFCH format in a slot can be (pre-)configured for X-symbols a sequence-based PSFCH format.

DMRS design for PSCCH/PSSCH

DMRS design is currently discussing taking into account the range of waveform, SCS, channel type, etc. for NR V2X. For PSCCH/PSSCH DMRS design, following multiplexing options can be considered as a starting point as shown in Figure 5.

Figure 5. multiplexing of PSCCH and PSSCH in NR V2X

Particularly for Option 3 that was at this time agreed at least for CP-OFDM, two types of DMRS allocation for PSCCH and PSSCH could be considered as seen in Figure 6.

    - Type 1: Separated DMRS design for PSCCH/PSSCH

      - PSCCH: demodulated with PSCCH DMRS, PSSCH: demodulated with PSSCH DMRS

    - Type 2: Common DMRS design for PSCCH/PSSCH

      - PSCCH: demodulated with PSCCH DMRS, PSSCH: demodulated with PSCCH DMRS & PSSCH DMRS

Figure 6. Two types of DMRS allocation for PSCCH and PSSCH in Option3 in Figure 5

Type 2 (Common DMRS could have several benefits. For example, 1) possibility of faster demodulation of PSSCH (from first symbol), 2) possibility of less PSSCH DMRS resources (by using PSCCH DMRS resources), 3) possibility of efficient resource allocation in non-slot based allocation.

For this, common DMRS sequence mapping (to all allocated PRBs of PSCCH and PSSCH in a symbol) and common(or similar) DMRS pattern design for PSCCH DMRS and PSSCH DMRS should be further studied.

Proposal 12:	Common design for PSCCH DMRS and PSSCH DMRS should be further studied

AGC and Tx-Rx switching

In last RAN1 meeting, there are some proposals to be more efficient resource utilization for the AGC and Tx-Rx switching time assuming that AGC and Tx-Rx switching time can be much less than one symbol depending on SCS. To get more clear assumption, RAN1 sent LS to ask it. In response LS from RAN4, following answer for the questions from RAN1 has been provided especially for the AGC and Tx/Rx switching time [4].

- AGC settling time

  - At least AGC settling time = 1 OFDM symbol for 15 kHz SCS is feasible.

  - RAN4 will further study if smaller AGC time is achievable incl. 15 kHz SCS.

- TX/RX switching time

  - FR1: 13 us

  - FR2: 7us

Considering above response, it seems RAN4 needs also some more time to conclude the required AGC settling time depending on numerology. Thus, RAN1 needs to wait for RAN4 conclusion on AGC settling time.

Regarding potential solution for overhead reduction if smaller AGC time is assumed than 1 OFDM symbol, there are some proposals as followings:

- option 1. AGC/data and Tx-Rx switching/data in two OFDM symbols, respectively

- option 2. Merging AGC/Tx-Rx switching in one OFDM symbol

- option 3. Switching between option 1 or 2 and LTE-V2X way (i.e. first symbol for AGC and last symbol for Tx-Rx switching, depending on SCS

Basically, option 1 may result in potential performance impacts from smaller number of data/control REs in the OFDM symbol due to either AGC or Tx-Rx switching, and additional specification and implementation efforts may be foreseen in order to handle the different OFDM symbol structure (e.g. comb type) over general OFDM symbol in terms of Physical channel/RS design, etc. So, it is preferred to use one symbol entirely for AGC and Tx-Rx switching like option 2 or LTE-V2X-like.

Proposal 13:	It can be considered to adopt either merging AGC and Tx-Rx switching in one OFDM symbol or LTE-V2X-like way

Conclusion

This contribution discusses the synchronization mechanism for NR V2X and makes the following proposal:

Proposal 1:	A UE assumes that there is no switching latency between SL BWP and Uu BWP in a same carrier at a given time

Proposal 2:	It should be discussed how to address the issue related to potential different numerology between initial Uu BWP and SL BWP in a same carrier

Proposal 3:	It is preferred to adopt the resource pool consisting of contiguous PRBs in frequency domain.

Proposal 4: 	NR V2X supports resource pool configuration without limitation of cast/transmission type.

Proposal 5: 	The resource pool period(P) is the same or multiple value of SSB period

Proposal 6: 	Ams-level bit-map could be considered for resource pool indication.

- Ams corresponds to dl-UL-TransmissionPeriodicity in TDD-UL-DL-ConfigurationCommon, e.g., A=0.625, 1.25 or 2.5.

Proposal 7:	Support option 3 only on PSCCH/PSSCH multiplexing

Proposal 8:	For larger number of SL HARQ-ACK bits (e.g. >2), a PSFCH format containing more than 2 SL HARQ-ACK bits (e.g. PUCCH format 2) is necessary.

Proposal 9:	It can be considered to reuse most of functionalities of NR PUCCH format 0 for a sequence-based PSFCH format, with some modifications such as how to apply SL layer-1 ID for this PSFCH format.

Proposal 10:	It is supported to use at least 1 PRB for a sequence-based PSFCH format.

Proposal 11:	Either a repetition of the one-symbol PSFCH format or X-transmission occasions using one symbol PSFCH format in a slot can be (pre-)configured for X-symbols a sequence-based PSFCH format.

Proposal 12:	Common design for PSCCH DMRS and PSSCH DMRS should be further studied

Proposal 13:	It can be considered to adopt either merging AGC and Tx-Rx switching in one OFDM symbol or LTE-V2X-like way

References

- Chairman’s Note 3GPP TSG RAN WG1#97, Reno, USA, May 2019

- Chairman’s Note 3GPP TSG RAN WG1 AH#1901, Taipei, Taiwan, January 2019

- R1-1814165, " LS on transient period in sidelink channels ". RAN WG1, Spokane, USA, Nobemver 2018

- R1-1900010, " Reply LS on NR V2X RF parameters for RAN1 physical layer design aspects ". RAN WG1, Taipei, Taiwan, January 2019

- Chairman’s Note 3GPP TSG RAN WG1#96, Athens, Greece, February 2019