3GPP TSG-RAN WG1 Meeting #98	 R1-1909122

Prague, CZ, August 26th – 30th, 2019

Agenda Item:	7.2.1.1

Source:	Ericsson

Title:	Channel Structure for Two-Step RACH

Document for:	Discussion, Decision

# Introduction

This paper discusses msgA channel structure for 2-step RACH [1]. The structure of PUSCH occasions is investigated, considering how different PUSCH occasion sizes can be supported and how multiple msgA PUSCHs can be multiplexed in time and frequency. The need for differently scrambled DMRS is briefly analyzed, but is discussed in more detail in [3].  Furthermore, how the preambles are mapped to the PUSCH resource units and how the SSBs are mapped to time domain PUSCH occasions is discussed. Finally, how the waveform for msgA PUSCH is assigned, how the numerology is set for PUSCH and PRACH, and how RNTI is determined for msgA and msgB are considered. Use cases and scenarios to target the msgA design toward are discussed in a companion paper [2]. The issue of whether ‘guard’ PRBs can be allocated between neighboring POs is also discussed in [3].

# msgA structure

One of the key requirements for msgA structure is that it supports low latency. In both licensed and unlicensed operation in NR-U, this is enabled by minimizing the duration of PRACH and PUSCH components of msgA. However, for NR-U, one of the main benefits of the 2-step RACH is the reduced need of listen-before-talk (LBT) operations compared to the 4-step RA procedure. In case msgA would need two LBTs, much of the benefit of the 2-step RACH would disappear. Therefore, it is of vital importance that msgA can be transmitted using only one LBT, i.e. that there is no gap between PRACH and PUSCH longer than 16 µs.

Observations:

- For both licensed and unlicensed operation, msgA configurations should support transmission in as few symbols as possible to meet the lowest latency requirements.

- For NR-U, it is important that msgA can be transmitted using only one LBT, and therefore that there are no gaps within msgA longer than 16µs.

## PUSCH occasion slot structure

In RAN1#97, further progress was made on the high level time domain structure of msgA PUSCH: PUSCH occasions (POs) can be FDM’d, and the MCS, the number of PRBs, DMRS settings can all be set within a ‘msgA PUSCH configuration’. However, a number of questions remain:

- What is the time frequency structure of each PRU? How are the PRUs numbered in one or multiple PUSCH occasions?

- How many different PO sizes are there for a given msgA PUSCH transmission?

- How are POs associated with an RO multiplexed in time?

- How does a msgA PUSCH set relate to a msgA PUSCH configuration period?

The PUSCH occasion time domain structure design can be motivated by the following observations:

- In contention-based operation, a UE will randomly select some msgA resources given by a msgA PUSCH configuration. This region in time that contains multiple PUSCH occasions of a msgA PUSCH configuration might be termed a ‘msgA PUSCH set’.

- In order to improve spectral efficiency and to allow different payload sizes, it is desirable to allow the PUSCH occasions to have different sizes.

- Spectral efficiency can be further improved through MU-MIMO techniques, where PUSCHs colliding within a PUSCH occasion have different DMRS. The DMRS enable the gNB to form channel estimates for each of the interfering PUSCH, which then allows an MMSE-IRC receiver in sufficiently good radio conditions to successfully decode all the PUSCHs. This is now supported by PRUs, since UEs transmitting in different PRUs but the same PO can be received using MU-MIMO using the different DMRS associated with the PRUs.

The use of multiple msgA PUSCHs in a PUSCH occasion is illustrated in Figure 1 below. A msgA PUSCH set starting at time and frequency at t0 and f0 is shown. Here, a PUSCH occasion can have one of 3 sizes occupying K=1, K=2, or K=4 PRBs and 7 OFDM symbols. Each PUSCH occasion is identified by its index ‘n’. Each PUSCH occasion contains 2 PUSCH resource units associated with a DMRS configuration (a DMRS port and/or a DMRS sequence initialization) shown as ‘DMRS0’ and ‘DMRS1’.

Time division multiplexing is supported in the figure, with an example of two PUSCH occasions in time. An alternative would be to preclude TDM of POs in a msgA set, and so ROs in a PRACH slot would map to one set of POs in time. If we assume that one RO maps to one PO, there are 64 preambles per PO. Such a configuration could support 3 simultaneous UEs per RO with a preamble collision probability of about 5%, and with 4 receive antennas, the 3 simultaneous UEs should be separable by an MMSE-IRC receiver. Then if a PO uses 6 PRBs and there are 6 TDM’d ROs per RACH slot, then 6*6=36 PUSCH PRBs are needed per RACH slot. If 8 ROs are FDM’d as well, then 8*6*6=288 PUSCH PRBs are needed per RACH slot, which occupies more than the 3300 active subcarriers supported in a carrier in NR. On the other hand, if we allow TDM, say by a factor of 4, then 1 or 8 FDM’d ROs per RACH slot would require 12 or 72 PRBs, respectively.

Observation:

- Precluding TDM of POs in a msgA set may require excessive msgA bandwidth.

The figure also shows that each group of POs that are frequency division multiplexed together in an OFDM symbol is associated with an SSBs. This is so that a gNB can use the same RX beam for this group of POs as for a PRACH preamble that corresponds to the SSB. If an FDM’d PO group corresponds to multiple SSBs, a gNB that requires beam switching among the SSBs would not be able to receive the FDM’d group properly.

If POs are not mapped to SSBs, then UEs will select among POs randomly with respect to SSB.   For example, consider if 4 SSBs are time multiplexed in a PRACH slot and these all map to one PO.  Since there is one PO, a gNB with analog beamforming would need to select one beam to receive the PO with. If 2 UEs were to arrive in that slot with equal probability of choosing each SSB, there is only a 25% chance that they would both be able to be received with the selected beam, and so a 75% chance of having an incorrect beam for one UE. With two POs, the gNB will pick the two SSBs that the 2 UEs actually use, but because the UEs randomly select the POs, the probability of a UE selecting an incorrect beam is again 75%. By contrast, if two POs are mapped to the SSBs used by the ROs the chance of an incorrect beam is 0%.

Observation:

- If POs are not mapped to SSBs, at most one UE can be supported per RACH slot without degraded performance in gNBs using analog beamforming for SSBs.

Given that we TDM msgA POs, the next question is how many to TDM. Since there can be at most 7 ROs TDM’d in a PRACH slot, Supporting the same maximum number of TDM’d POs in a msgA PUSCH set would allow the PUSCH to have the same amount of TDM’d SSBs as the PRACH.

Observation:

- Supporting a number of TDM’d POs in a msgA PUSCH slot equal to the maximum number of ROs in a PRACH slot. allows the same amount of TDM’d SSBs as the PRACH.

Figure 1

Proposal:

- Support configuration of multiple time-domain POs by one msgA PUSCH configuration.

- msgA POs frequency division multiplexed in a set of OFDM symbols can correspond to an SSB

  - The maximum number of TDM’d POs per subcarrier is roughly the maximum number of ROs in a PRACH slot.

## On the need for different scrambling in a PUSCH occasion

A key open issue is whether PRUs in a PO can use different scrambling sequences.  Different scrambling sequences can be beneficial when there are a small number of POs in order to avoid the selection of identical DMRS in contention based operation.

One simple way to illustrate this is to consider the probability that UEs select the same PO and same DMRS.  Figure 2 below shows where there are 1, 2, or 6 POs; 4, 8, or 16 distinct DMRS are available; and 2 to 8 UEs randomly select a PO and a DMRS.  The probability that UEs select the same PO and DMRS can be seen as a lower bound on PUSCH BLER for DMRS based PUSCH reception: when two UEs select the same DMRS, gNB should not be able to decode the two PUSCHs.  Note that this does not take into realistic channel estimation, which will be worse for scrambling than for orthogonal ports; this is addressed in [3].  From the figure, we observe that different scrambling seems needed to avoid error floors when there are few POs and at higher loads:

- With 1 PO and up to 8 DMRS, 10% BLER is not possible

  - A minimum of 25% BLER is possible with 1 PO and 4 DMRS

  - 10% BLER is possible with 16 DMRS and 1 PO

- With 2 UEs per PO, 16 DMRS are needed to reach 10% BLER

  - Less than one UE per PO and 6 POs is needed to reach 1% BLER

- Doubling the number of DMRS seems to reduce the BLER floor by about 2x

Figure 2

Observation:

- The use of different scrambling sequences can avoid BLER floors

We further investigate the relative performance of scrambling and orthogonal ports in [3] using link level simulations where different numbers of POs are available to UEs randomly selecting the POs and their corresponding DMRSs.  While multi-link and/or system level simulations capturing realistic near-far power differences and supported load are needed to make a fully informed decision on the relative merits of differently scrambled vs. orthogonal antenna ports, given the results in [3] where differently scrambled ports are observed to generally outperform orthogonal ports, we tend to think at least differently scrambled ports should be supported.  However, it is intuitive that orthogonal DMRS could provide some benefit as well when there is strong mutual interference.  Additional support of orthogonal DMRS is not expected to overly complicate the specification, since the main design issue would be how to configure the preamble to PRU mapping to account for when only one scrambling ID is used vs. when multiple scrambling IDs are used.

Different DMRS scrambling can be implemented using Rel-15 sequences, but with different DMRS sequence initialization, as discussed in section 2.5.2.  Therefore, we propose:

Proposal:

- At least support differently scrambled DMRS ports among msgA PUSCHs.

  - Orthogonal DMRS operation can also be supported.

- Further study details of DMRS configurations to support, including the number of different scrambling initializations.

## Preamble to PUSCH resource unit mapping

There is a working assumption that multiple preambles can be mapped to a PUSCH RU. One possible motivation for this is to save PUSCH resource overhead, since there can be 64 preambles in a RACH occasion, and reserving a PO for each preamble would require 64 PRBs, which is a substantial amount of overhead. However, the number of preambles could instead be set by the number of POs or PRUs, and only the subset of preambles could be used.

Another line of thinking could be that a gNB could separate two PRUs even though they use the same DMRS. This may be possible if the gNB uses the two different preambles to determine MMSE-IRC combining weights, rather than relying on DMRS for this purpose.  However, the need for another way for channel estimation seems mainly driven by the assumption that there are insufficient unique DMRS.  While there can be as few as 4 ports in some DMRS configurations, DMRS can have a nearly unlimited number of scrambling sequences.  Since the RACH preamble is associated with the DMRS, the number of DMRS configurations is limited by the desired number of RACH preambles rather than the DMRS overhead.

Still other thinking can be that many preambles are needed per PRU to avoid preamble collision. If a preamble collides, then it is difficult to recover which preambles were transmitted. However, if the PUSCH collides but not the preambles, then PUSCH may be able to be detected with an interference suppressing receiver. But the preambles may map to multiple POs, and so having a limited number of preambles per PO is not necessarily a problem. If there is only one PO, a multiple to one preamble to PRU mapping is not necessary if scrambling IDs are used. Therefore, the number of preambles per PO to avoid preamble collision does not seem to be a factor.

In cases where larger payloads can be carried, it may be desirable to allow POs in a given msgA PUSCH set to have different modulation and coding states. In such cases, preambles could be mapped to the MCS states, avoiding the need for the gNB to blindly detect MCS states.

Observations

- A multiple to one preamble to PUSCH RU mapping does not seem beneficial as a way to reduce PUSCH overhead

- The number of PUSCH RUs can match a subset, rather than the entirety, of preambles in an RO.

- While interference rejection via MMSE-IRC may be possible using distinct preambles when the antenna ports are the same, different scrambling IDs can serve the same purpose and make the PUSCH even more identifiable.

- If multiple preamble to PUSCH RU mapping is specified, preambles can be used to convey MCS

Proposal

- If multiple preambles are mapped to one PUSCH resource unit, the preambles convey MCS

In more detail, the following factors should be considered when numbering the PUSCH RUs:

- DMRS antenna ports and/or scrambling

- PUSCH occasions FDMed,

- PUSCH occasions TMDed

- PUSCH occasions for each PO size

And the mapping order can be as below:

- First by DMRS antenna ports and/or scrambling, then by frequency, then by PUSCH occasion size, then by time

Proposal:

- PUSCH RUs are ordered first by DMRS antenna ports and/or scrambling, then by frequency, then by PUSCH occasion size, then by time

A mapping order from the preamble to the PUSCH RUs can be as described below and in Figure 3:

- Step 1: Map to frequency of PUSCH RUs

  - Preambles are first mapped to cyclic shifts, which can be orthogonal. So, preambles mapped to different frequency PUSCH RUs can be orthogonal, like the adjacent PUSCH RUs.

- Step 2: Map to antenna ports and/or DMRS scrambling of PUSCH RUs,

  - Preambles are secondly mapped to root indices, which are not orthogonal, like a PO mapped to different DMRS.

- Step 3: If M preambles are not mapped to each PUSCH RU, then go to step 1.

- Step 4: Map to the PUSCH RU (and PUSCH occasion) time/frequency size

  - Multiple PUSCH RU sizes should be able to correspond to an SSB

- Step 5: Map to the OFDM symbol(s) containing the PUSCH

  - Different SSBs can map to different PUSCH symbols

Proposal:

- Use the five-step procedure below to map preambles to PUSCH resource units in a predetermined order:

  - Step 1: Map to frequency of PUSCH RUs,

  - Step 2: Map to antenna ports and/or DMRS scrambling of PUSCH RUs,

  - Step 3: If M preambles are not mapped to each PUSCH RU, then go to step 1,

  - Step 4: Map to the PUSCH RU (and PUSCH occasion) time/frequency size,

  - Step 5: Map to the OFDM symbol(s) containing the PUSCH

As is shown in Figure 3, preambles in a first RACH occasion map to SSB0 and SSB1, while those in a second RACH occasion map to SSB2 and SSB3. Since there are 14 PRUs per SSB, and 2 PRACHs are mapped to each PRU, 28 preambles are needed for each SSB. Therefore only 56 out of 64 preambles in a RACH occasion is needed to support the msgA PUSCH set example below. It can be observed that an integer number of preambles can’t be mapped to the 14 PRUs such that there are 64 preambles. Therefore, a mechanism is needed to map a subset of preambles in a RACH occasion to PRUs.

At one time instant, the PUSCH occasions for one size can overlap with PUSCH occasions for another size, or alternatively, PUSCH occasions for different sizes can be separated in frequency domain.

Figure 3

## PUSCH occasion configuration

According to current agreements, two options for configuring PUSCH occasions are available: PUSCH occasions are separately configured (‘Option 1’) and may have different configuration period from the RACH occasions, or PUSCH occasions are configured relative to a RACH occasion and have the same configuration period (‘Option 2’). Option 2 has 4 variants, the most flexible of which is:

- Alt 4: Time/frequency relation between each PRACH preamble in PRACH occasion(s) to the PUSCH occasion is semi-statically configured value. Different preambles in different PRACH occasions can have different values.

Figure 4 below illustrates two RACH occasions in one PRACH slot mapped to two msgA PUSCH sets. The two msgA PUSCH sets together form a msgA PUSCH configuration period over which POs can correspond to 4 SSBs. The delay from preambles in the first RO to SSB0 and SSB1 are shown as ΔT0 and ΔT1, respectively, while those in the second RO to SSB2 and SSB3 are ΔT2 and ΔT3, respectively.  Since ΔT0≠ΔT1 (and ΔT2≠ΔT3) the requirement in Alt. 4 above to have one semi-statically configured value between each preamble in a RO with the PO cannot be met.  While this can be solved by using multiple msgA PUSCH configurations, this may lead to greater specification complexity for Option 2 than for Option 1.  However, since there are still significant open issues on PUSCH occasion structure, deciding between these two options seems premature. Such a decision should be made after the PUSCH occasion designs have settled further, to avoid unintended impacts on the PUSCH occasion structure.

Figure 4

Observations

- Mapping multiple RACH occasions in a RACH slot to PUSCH occasions allows better use of the RACH slot.

- When PUSCH occasions that correspond to a RACH occasion are TDM’d, the delay of the PUSCH occasions from the RACH occasion varies with the PUSCH occasion.

Proposals

- Different ROs in one PRACH slot can map to different msgA PUSCH sets

- Defer decisions on whether ROs and POs are separately configured or not until the structure of PUSCH occasions is more clear.

  - Including how POs can be TDM’d in a msgA PUSCH set, how many ROs in a PRACH slot can map to different msgA PUSCH sets, and the structure of a msgA PUSCH configuration period.

## DMRS sequence generation for msgA PUSCH

In RAN1 #97 meeting, below proposal has been agreed by almost all companies, the DMRS port determination and some detailed DMRS sequence design is provided in this section.

Proposals:

- For the definition of PRU, support both DMRS ports and (working assumption) DMRS sequences at least for CP-OFDM

  - FFS how to support multiple sequences for DFT-s-OFDM

  - FFS the maximum number of sequences

### DMRS port determination

As is known that in Rel-15, the DMRS port for msg3 is always 0. By contrast, it has been agreed in Rel-16 for 2-step RACH operation that different DMRS ports, different DMRS sequence initialization, or a combination of the two will be used for PUSCH RUs occupying the same PO. Some mechanism is therefore needed to determine the DMRS port and sequence initialization to be used for each PUSCH RU.

The maximum number of DMRS ports is a function of the DMRS configuration in Rel-15 NR, which can be multiplexed using 3 different mechanisms:

- frequency domain multiplexing (FDM) where different ports occupy different subcarriers,

- code domain multiplexing in frequency (FD-CDM) where ports are spread over subcarriers OFDM symbols,

- code domain multiplexing in time (TD-CDM) where ports are spread over OFDM symbols.

These 3 different multiplexing methods are not equally robust to different impairments. Ports multiplexed with FD-CDM will mutually interfere in the presence of delay spread in a radio channel, while ports multiplexed with TD-CDM will mutually interfere when there is a Doppler shift or spread in the channel.  FDM is more robust than FD-CDM or TD-CDM, since the cyclic prefix tends to mitigate inter-subcarrier interference, although large Doppler shifts or large timing offsets will lead to inter-subcarrier interference.

Since different DMRS ports interfere differently according to how they are multiplexed, it can be beneficial to use a subset of antenna ports that is multiplexed in the most robust way.

So a subset of antenna ports for msgA PUSCH transmission is constructed

- by first including the first antenna port in each CDM group of the DMRS configuration in the subset

- then including the second antenna port in each CDM group in the subset, and so on until the subset contains the desired number of ports.

- The ports in each CDM group occupy a set of subcarriers that is not occupied by other CDM groups.

Proposal

- A subset of antenna ports for msgA PUSCH transmission is constructed

  - first by including the first antenna port in each CDM group of the DMRS configuration in the subset

  - then by including the second antenna port in each CDM group in the subset, and so on until the subset contains the desired number of ports.

### DMRS Sequence generation

When transform precoding is disabled, the DMRS sequence  used for msgA PUSCH can be generated according to clause 6.4.1.1.1.1 of 3GPP TS 38.211 V15.6.0 as the following:

Using same formula:

.

where the pseudo-random sequence  is defined in clause 5.2.1 of 3GPP TS38.211 V15.6.0. The pseudo-random sequence generator shall be initialized with

where  is the OFDM symbol number within the slot, ns,fμ is the slot number within a frame, and

-	NID0,NID1∈0,1,…,65535;

-	NIDnSCID can be a function of Cell ID of current cell NIDcell and preamble id PID of the preamble mapped to corresponding PUSCH occasion.

The quantity nSCID∈0,1 can be configured or fixed in specification.

Proposal:

- When transform precoding is disabled, same formula for normal PUSCH is used for the msgA PUSCH DMRS sequence generation, the nSCID can be a fixed value, and the NIDnSCID can be a function of cell ID, corresponding preamble ID, an index of the PRU, or the number of distinct PO sizes.

For when transform precoding is enabled, for msgA PUSCH transmission, the reference-signal sequence  may be generated according to clause 6.4.1.1.1.1 of 3GPP TS 38.211 V15.6.0 using:

where ru,vα,δ(m) is given by clause 5.2.2 of 38.211 V15.6.0 with δ=1 ,and α, u, v may be determined based on the preamble ID and/or the ID of the PRU and a set of values for one or more of these 3 parameters.

Proposal:

- When transform precoding is enabled, the same formula as for normal PUSCH is used for the msgA PUSCH DMRS sequence generation, α, u, v may be determined based on the preamble ID and/or the ID of the PRU.

## PUSCH waveform

In RAN1 #96, below agreements were made on the waveform of the msgA.

Agreements:

- Both DFT-s-OFDM and CP-OFDM are supported for the payload transmission in msgA

  - FFS how to indicate/configure the waveform

For the signaling of the waveform, in NR release 15, msg3-transformPrecoder was included in RACH-ConfigCommon IE to signal the waveform used for msg3 in 3GPP TS 38.331 V15.4.0. So, similarly, a msgA-transformPrecoder can be included in the same IE to indicate the waveform of the msgA PUSCH for 2-step RA.

Proposal:

- Signal whether transform precoding is enabled for msgA PUSCH in system information.

## PUSCH and preamble numerology

For the preamble of msgA, if the preambles are shared between 2-step and 4-step RA or they are in the same RACH occasion, they will need to have the same numerology to both occupy the RACH occasion.

Regarding the msgA PUSCH numerology, in RAN1 #96b, below agreements were met.

Agreements:

- Support the PRACH and PUSCH for msgA transmission in different slots. In this case, the numerology for msgA PUSCH follow the numerology configured for the UL BWP for msgA transmission.

  - FFS whether to support PRACH and PUSCH in the same slot for msgA transmission. If supported, down-select from the following option

    - Opt 1: the numerology for msgA PUSCH follows that of msgA preamble

    - Opt 2: gNB configure whether the numerology for msgA PUSCH follows that of msgA preamble or UL BWP

    - Opt 3: a UE is not expected to be configured with different numerology among PRACH preamble, msgA PUSCH and UL BWP for msgA transmission

    - Note: in Rel.15 the PRACH and PUSCH transmitted in the same slot for a UE are not supported

From a gNB perspective, allowing the PUSCH and PRACH numerologies to be the same or different enables PRACH and PUSCH resource allocation flexibility to be traded off for reception complexity. As long as the signalling overhead is minimal (i.e. no more than a few bits carried in SIB1), maintaining this flexibility that is already available in Rel-15 seems reasonable.

For the msgA PUSCH part, if the msgA PUSCH and msgA preamble part can be in the same slot, in this case the preamble will definitely use a short format, and the same numerology may be required between preamble and PUSCH part, i.e. option 1.

Thus, a separately configured numerology of the msgA PUSCH can be applied so that all the cases discussed above can be supported.

Proposal:

- The numerology of PRACH in 4-step RA is the same as that of the 2-step preambles.

- Support a separately configured numerology of the msgA PUSCH.

## RNTI for msgA/B

In the 2-step Random Access procedure, the main issue of how to assign RNTIs to msgA is that there is no TC-RNTI given to the UE prior to msgA transmission since the msgA is the first message of the procedure.

So it is needed to select an RNTI which can uniquely define the preamble id and PRACH occasion (equivalent to specific UE if no preamble collision) which is linked to the PUSCH transmission in msgA.

Proposal:

- The RNTI associated with the msgA PUSCH transmission, i.e. nRNTI for the msgA PUSCH scrambling sequence generation is calculate based on at least the preamble ID

In RAN1 #97 meeting, below agreements were made for the msgA PUSCH data scrambling sequence generation. The remain issue is to how to determine the C_init value based on the existing RNTI, preamble index etc.

Agreements:

- The c_init for msgA PUSCH scrambling is at least derived based on a RNTI, preamble index, and/or n_ID (which can be cell ID or configurable, to be FFS).

  - FFS details of the RNTI

  - FFS the inclusion of DMRS index.

For a normal PUSCH, the initial value for the scrambling sequence generation is defined as below based on 3GPP TS38.211:

cinit=nRNTI⋅215+nID

where

-	nID∈0,1,...,1023 equals the higher-layer parameter dataScramblingIdentityPUSCH if configured and the RNTI equals the C-RNTI, MCS-C-RNTI or CS-RNTI, and the transmission is not scheduled using DCI format 0_0 in a common search space,

-	nID=NIDcell otherwise

and where nRNTI corresponds to the RNTI associated with the PUSCH transmission as described in clause 6.1 of [6, TS 38.214].

In order to have different scrambling sequences used for different msgA PUSCHs with different msgA preamble IDs, preamble-ID and some existing RNTI e.g. RA-RNTI can be used to generate nRNTI. For example,

nRNTI=RA_RNTI +P_ID ,

where the RA_RNTI is calculated based on the PRACH resource used to transmit the msgA preamble, and the P_ID is the ID of msgA preamble.

For the msgB monitoring, the RNTI to be used is still being discussed in RAN2. One approach for fall back is that transmitting msgB can imply that msgA was received correctly by gNB, while a msg2 can indicate fallback to the 4-step procedure. Therefore, it is advantageous for UE processing if the RNTI used for msgB cannot collide with the RA-RNTI used for msg2.

Proposal:

- A separate set of RNTIs are used for msgB so that the msgB can be differentiated from msg2.

# Conclusion

In this contribution, we investigated the structure of PUSCH occasions, considering how different PUSCH occasion sizes can be supported and how multiple msgA PUSCHs can be multiplexed in time and frequency. Furthermore, how the preambles are mapped to the PUSCH resource units and how the SSBs are mapped to time domain PUSCH occasions was discussed. Finally, how the waveform for msgA PUSCH is assigned, how the numerology is set for PUSCH and PRACH, and how RNTI is determined for msgA and msgB were considered. We drew the observations below, leading to the following proposals.

Observations:

- For both licensed and unlicensed operation, msgA configurations should support transmission in as few symbols as possible to meet the lowest latency requirements.

- For NR-U, it is important that msgA can be transmitted using only one LBT, and therefore that there are no gaps within msgA longer than 16µs.

- Precluding TDM of POs in a msgA set may require excessive msgA bandwidth.

- If POs are not mapped to SSBs, at most one UE can be supported per RACH slot without degraded performance in gNBs using analog beamforming for SSBs.

- Supporting a number of TDM’d POs in a msgA PUSCH slot equal to the maximum number of ROs in a PRACH slot. allows the same amount of TDM”d SSBs as the PRACH.

- The use of different scrambling sequences can avoid BLER floors

- A multiple to one preamble to PUSCH RU mapping does not seem beneficial as a way to reduce PUSCH overhead

- The number of PUSCH RUs can match a subset, rather than the entirety, of preambles in an RO.

- While interference rejection via MMSE-IRC may be possible using distinct preambles when the antenna ports are the same, different scrambling IDs can serve the same purpose and make the PUSCH even more identifiable.

- If multiple preamble to PUSCH RU mapping is specified, preambles can be used to convey MCS

- Mapping multiple RACH occasions in a RACH slot to PUSCH occasions allows better use of the RACH slot.

- When PUSCH occasions that correspond to a RACH occasion are TDM’d, the delay of the PUSCH occasions from the RACH occasion varies with the PUSCH occasion.

Proposals:

- Support configuration of multiple time-domain POs by one msgA PUSCH configuration.

- msgA POs frequency division multiplexed in a set of OFDM symbols can correspond to an SSB

  - The maximum number of TDM’d POs per subcarrier is roughly the maximum number of ROs in a PRACH slot.

- At least support differently scrambled DMRS ports among msgA PUSCHs.

  - Orthogonal DMRS operation can also be supported.

- Further study details of DMRS configurations to support, including the number of different scrambling initializations.

- If multiple preambles are mapped to one PUSCH resource unit, the preambles convey MCS

- PUSCH RUs are ordered first by DMRS antenna ports and/or scrambling, then by frequency, then by PUSCH occasion size, then by time

- Use the five-step procedure below to map preambles to PUSCH resource units in a predetermined order:

  - Step 1: Map to frequency of PUSCH RUs,

  - Step 2: Map to antenna ports and/or DMRS scrambling of PUSCH RUs,

  - Step 3: If M preambles are not mapped to each PUSCH RU, then go to step 1,

  - Step 4: Map to the PUSCH RU (and PUSCH occasion) time/frequency size,

  - Step 5: Map to the OFDM symbol(s) containing the PUSCH

- Different ROs in one PRACH slot can map to different msgA PUSCH sets

- Defer decisions on whether ROs and POs are separately configured or not until the structure of PUSCH occasions is more clear.

  - Including how POs can be TDM’d in a msgA PUSCH set, how many ROs in a PRACH slot can map to different msgA PUSCH sets, and the structure of a msgA PUSCH configuration period.

- A subset of antenna ports for msgA PUSCH transmission is constructed

  - first by including the first antenna port in each CDM group of the DMRS configuration in the subset

  - then by including the second antenna port in each CDM group in the subset, and so on until the subset contains the desired number of ports.

- When transform precoding is disabled, same formula for normal PUSCH is used for the msgA PUSCH DMRS sequence generation, the nSCID can be a fixed value, and the NIDnSCID can be a function of cell ID, corresponding preamble ID, an index of the PRU, or the number of distinct PO sizes.

- When transform precoding is enabled, the same formula as for normal PUSCH is used for the msgA PUSCH DMRS sequence generation, α, u, v may be determined based on the preamble ID and/or the ID of the PRU.

- Signal whether transform precoding is enabled for msgA PUSCH in system information.

- The numerology of PRACH in 4-step RA is the same as that of the 2-step preambles.

- Support a separately configured numerology of the msgA PUSCH.

- The RNTI associated with the msgA PUSCH transmission, i.e. nRNTI for the msgA PUSCH scrambling sequence generation is calculate based on at least the preamble ID.

- A separate set of RNTIs are used for msgB so that the msgB can be differentiated from msg2.

# References

RP-182894, “New work item: 2-step RACH for NR”, ZTE Corporation, Sanechips, RAN#82, December 2018.

R1-1909121, “Use Cases and Scenarios for 2-Step RACH”, Ericsson, 3GPP TSG-RAN WG1 Meeting #98, Prague, CZ, August 26-30, 2019

R1-1909124, “Performance and design details of 2-step RACH”, Ericsson, 3GPP TSG-RAN WG1 Meeting #98, Prague, CZ, August 26-30, 2019