3GPP TSG RAN WG1 #98			R1-1909192

Prague, CZ, August 26th – 30th, 2019

Source:	NTT DOCOMO, INC.

Title:	RLM/RLF for NR sidelink

Agenda Item:	7.2.4.8

Document for: 	Discussion and Decision

- Introduction

At RAN plenary #83 meeting, NR-V2X WID was endorsed as ‘New WID on 5G V2X with NR sidelink’ [1]. According to the WID, there were a lot of discussions on NR-V2X at the previous RAN1 meetings. In this contribution, we share our views on RLM/RLF for NR sidelink.

- Discussions

  - RS for radio link monitoring

To support RLM/RLF on NR-SL, which RS is used needs to be discussed and defined. It is noted that the following agreement has been reached at the last RAN1 meeting [2].

Agreements: - No standalone RS dedicated to SL RLM/RLF in Rel-16

At the RAN1#97 meeting, many companies provided their views on this point [3], but there is no agreement and still we have the following options as RS for RLM:

- Option 1: S-SSB

- Option 2: PT-RS

- Option 3: CSI-RS

- Option 4: DM-RS

From the above options, we believe that at least DM-RS should be used for RLM, i.e. option 4. In option 1, some UEs would transmit S-SSB at the same time. Due to the half-duplex issue, PC5-RRC connected UEs may not receive each other’s S-SSB. Regarding option 2, PT-RS is not mandatory, especially on FR1. PT-RS is not available depending on the situation; therefore, option 2 is unreasonable for RLM.

In NR-Uu, standalone RS transmission/reception is adopted as CSI-RS and SRS, but standalone RS transmission is not supported for RLM on NR-SL. In other words, to transmit RS for RLM, some physical channel transmission is necessary. Possible channel for this purpose will be PSCCH/PSSCH since each UE can decide whether to transmit or not by itself. PSFCH transmission shall be corresponding to PSCCH/PSSCH reception, which means that PSFCH transmission may not be applicable when measurement is necessary. From this reason, PSFCH is not suitable to be considered for RLM.

Among CSI-RS and DM-RS on PSCCH/PSSCH, our preference is CSI-RS for RLM. The reason is forward compatibility. In future release, beam management would be support in NR-SL. In this case, QCL and TCI-state concept will be introduced, where beam switching can be considered by using CSI-RS. It cannot be considered if DM-RS is used. CSI-RS would be undesirable overhead but it depends on PC5-RRC configuration. CSI-RS on every PSSCH is not unlikely for RLM; therefore, at least CSI-RS should be supported for RLM. Note that DM-RS is also applicable but whether/how to use can be up to UE implementation.

Proposal 1:

- For RLM on NR-SL, at least CSI-RS is used.

  - IS/OOS procedure and metric

At the RAN1#97 meeting, some companies provided their views on periodic TS/OOS procedure and metric [3]. NR-SL does not support standalone RS transmission for RLM/RLF. That is, periodic IS/OOS adopted in NR Rel-15 may need to be updated. And if some update is introduced, Layer-1 aspects for RLM/RLF should consider the update.

At the last RAN2 meeting, the following agreement was reached [4].

- Agreements on PC5 RLM/RLF - Even though transmission of sidelink signal occur irregularly, RAN2 assumes that the physical layer provides periodic indications of IS/OOS to the upper layer as in Uu RLM. - From RAN2 perspective, both side UEs perform RLM/RLF detection mechanism. FFS on whether periodic indications of IS/OOS based RLM/RLF is reused or any additional new mechanism is needed.

According to the RAN2 agreement, IS/OOS procedure for NR-SL is still under discussions. As abovementioned, Layer-1 aspects can be discussed based on RAN2 decision; therefore, RAN1 should wait RAN2 agreements.

Proposal 2:

- Regarding IS/OOS procedure and metric for RLF declaration, RAN1 waits RAN2 decision on IS/OOS mechanism.

- Conclusion

In this contribution, we discussed RLM/RLF for NR sidelink. Proposals are summarized as following:

Proposal 1:

- For RLM on NR-SL, at least CSI-RS is used.

Proposal 2:

- Regarding IS/OOS procedure and metric for RLF declaration, RAN1 waits RAN2 decision on IS/OOS mechanism.

References

- RP-190766		New WID on 5G V2X with NR sidelink		LG Electronics, Huawei

- 3GPP RAN1#97 meeting,	chairman’s notes

- R1-1907714	Feature Lead Summary #1 for NR-V2X AI - 7.2.4.8 Others	InterDigital, Inc.