3GPP TSG RAN WG1 #98		  	   R1-1909199

Prague, CZ, August 26th – 30th, 2019

Source:	NTT DOCOMO, INC.

Title:	Discussions on DL SPS enhancement and UL intra-UE transmission prioritization/multiplexing

Agenda Item:	7.2.6.7

Document for: 	Discussion and Decision

- Introduction

In the RAN#83 meeting, the WIs on physical layer enhancements for NR URLLC and support of NR Industrial Internet of Things (IoT) were approved [1] – [2]. For SPS enhancement, the objective is listed as below:

- Support for multiple simultaneous active semi-persistent scheduling (SPS) configurations for a given BWP of a UE. [RAN2, RAN1].

- Support for shorter SPS periodicities than the existing ones [RAN2, RAN1].

In addition, the detailed objective for NR intra-UE prioritization/multiplexing is:

- Specify enhancements to address resource conflicts between dynamic grant (DG) and configured grant (CG) PUSCH and conflicts involving multiple CGs [RAN2, RAN1].

- Address UL data/control and control/control resource collision by:

  - Specifying prioritization and/or multiplexing behaviour among HARQ-ACK/SR/CSI and PUSCH for traffic with different priorities, including the cases with UCI on PUCCH and UCI on PUSCH [RAN1, RAN2].

In this contribution, we first discuss the physical layer enhancements for DL SPS. Then we discuss solutions to address resource conflicts between dynamic grant (DG) and configured grant (CG) PUSCH and conflicts involving multiple CGs/SPS configurations. Our views for addressing UL data/control and control/control resource collision can be found in our companion contribution [3].

- Physical layer enhancements for DL SPS

For physical layer enhancements for DL SPS, following agreements were made in RAN1 [4][5]:

Agreements in RAN1 #96b: - Support separate activation for different DL SPS configurations for a given BWP of a serving cell. - FFS whether or not to support joint activation in a DCI for two or more DL SPS configurations - Support separate release for different DL SPS configurations for a given BWP of a serving cell. - FFS whether or not to support joint release in a DCI for two or more DL SPS configurations Agreements in RAN1 #97: Regarding Q1 in the LS in R1-1905940: - Although RAN1 has not completely analysed the potential impact of supporting up to 16 SPS configurations for a given BWP of a serving cell, RAN1 has the understanding that 8 SPS configurations for a given BWP of a serving cell is sufficient in Rel-16. Regarding Q2 in LS from RAN2, the following is captured: - RAN1 discussed the feasibility of support of shorter periodicities for DL SPS, it is feasible to support periodicity down to 1 slot for all SCSs and single SPS configuration with certain constraints related to HARQ-ACK feedback and combinations of DL & UL SCSs Conclusion: - RAN1 will continue to further investigate whether or not it is feasible to support periodicities shorter than 1 slot for SPS.

Based on the progress achieved in both RAN1 and RAN2, we discussed following aspects for DL SPS:

- Support for multiple active semi-persistent scheduling (SPS) configurations for a given BWP of a UE

- Support for SPS periodicities shorter than 1 slot

- HARQ-ACK feedback for more than one SPS PDSCH receptions/release in a same PUCCH

  - Multiple active SPS configurations for a given BWP

It was concluded that 8 SPS configurations for a given BWP of a serving cell is sufficient in Rel-16 for current TSC requirements [5][6]. For the feasibility of activation/deactivation of multiple SPS configurations via DCI signalling, it was agreed to support separate activation/deactivation for different DL SPS configurations for a given BWP of a serving cell, while whether support joint activation/release of multiple SPS configurations by a single DCI signalling is still FFS [7]. On the one hand, multiple SPS configurations for a given BWP of a serving cell can be used for different services/traffic types. Separate activation/deactivation for different DL SPS configurations is beneficial for this case. On the other hand, multiple SPS configurations for a given BWP of a serving cell can be used to reduce the transmission alignment delay and enhance the reliability for one given traffic type. For this case, high flexibility of the SPS configuration seems not necessary, low latency is more important. In addition, for separate activation/release of multiple SPS configurations, the delay and control overhead will be large especially for the UE with a large number of SPS configurations, e.g., 8 SPS configurations. Joint SPS activation/release in a DCI for multiple SPS configurations is more proper for this case. Therefore, we propose joint activation/release in a DCI for multiple SPS configurations should be also supported in Rel-16. Whether to use sperate or joint activation/release approach depends on the use case. The details on how to support joint activation/release in a DCI for multiple SPS configurations needs to be further studied.

Proposal 1:

- For DL SPS, support joint activation/release in a DCI for multiple SPS configurations for a given BWP of a serving cell.

  - FFS the details on how to support joint activation/release in a DCI for multiple SPS configurations.

  - SPS periodicity shorter than 1 slot

In Rel.15, the minimum periodicity for SPS configuration is 10ms. While for UL configured grant configuration, the minimum periodicity can be 2 symbols, exact value depends on the SCS, ranging between ~18us and ~143us. In the LS [6], RAN2 asked RAN1 about the feasibility on supporting SPS periodicities of at least 0.5ms and shorter than 0.5ms, e.g. down to 2 symbols, and whether there are any additional limitations for the above periodicities in terms supported SCS and HARQ-ACK feedback. Based on Rel.15 UE feature FG 9 [5-13] ~ FG11 [5-13c], the maximum number for the UE to process the unicast PDSCHs per slot per CC for different TBs for UE capability#2 is 7 [8]. Therefore, from physical layer perspective, it is feasible to support the SPS periodicity shorter than 0.5ms, e.g. down to 2 symbols without any limitations. In Rel.15, the shortest periodicity for CG is 2 symbols, considering the traffic for DL and UL is symmetric, it is desirable that the periodicity for DL SPS can be down to two symbols.

Proposal 2:

- From physical layer perspective, it is feasible to support SPS periodicity shorter than 0.5ms, e.g. down to two symbols without any limitations.

  - HARQ-ACK feedback for more than one SPS PDSCH receptions/release in a same PUCCH

In Rel.15, a UE is not expected to be indicated to transmit HARQ-ACK information for more than one SPS PDSCH receptions on the same PUCCH. In addition, DL SPS shall not be configured for more than one serving cell within a cell group. In the previous meeting, it was agreed to support multiple SPS PDSCH configurations and shorter SPS periodicities in Rel.16. In this case, it is inevitable that HARQ-ACK for more than one SPS PDSCH receptions/release on the same PUCCH will happen. Followings need to be taken into account:

- Type I and Type II HARQ-ACK codebook determination

- The number of HARQ-ACK bits and HARQ-ACK bit position for

  - First SPS PDSCH reception with associated DCI, the DCI format can be 1_0 and 1_1

  - SPS PDSCH release

  - SPS PDSCH reception without associated DCI

- HARQ-ACK feedback and PUCCH resource determination when only SPS PDSCH receptions without associated DCI are detected

In the following, we discuss more detailed specification impact for the above issues.

Type1 HARQ-ACK codebook

For Type I HARQ-ACK codebook, the Rel.15 mechanism can be used as baseline for HARQ-ACK bit(s) generation for first SPS PDSCH reception with associated DCI, SPS PDSCH receptions without associated DCI and SPS PDSCH release. Specifically, HARQ-ACK bit(s) are determined first in increasing order of the candidate PDSCH reception occasion index and then in increasing order of serving cell index where candidate PDSCH reception occasion is derived based on the RRC configured K1 values and the PDSCH time domain resource allocation table, etc.  For one or more DL SPS configurations with SPS periodicity equal or larger than one slot, each SPS PDSCH and/or SPS PDSCH release correspond to one of the entries in the PDSCH resource allocation table, then same HARQ-ACK bit(s) generation mechanism as in Rel.15 can be reused for Type I HARQ-ACK codebook. While for SPS PDSCH configuration(s) with SPS periodicity smaller than one slot, UE may not find corresponding time domain resource allocation in TDRA table for some SPS PDSCH occasions belonging to one SPS configuration [9]. Therefore, the Type I HARQ-ACK codebook construction for SPS configuration with SPS periodicity smaller than one slot needs to be further studied.

Type2 HARQ-ACK codebook

In Rel.15, for Type II HARQ-ACK codebook, HARQ-ACK bit(s) generation for DL SPS are as following:

- For SPS PDSCH reception without DCI, the HARQ-ACK bit(s) is/are placed after ones in response to dynamic PDSCH in the HARQ-ACK codebook;

- While for first SPS PDSCH reception with associated DCI and/or for SPS PDSCH release, the HARQ-ACK bit(s) in the HARQ-ACK codebook are determined based on the value of counter DAI and/or total DAI indicated by the activation/deactivation DCI.

To support more than one HARQ-ACK bits for multiple DL SPS receptions being multiplexed into one HARQ-ACK codebook in Rel.16, the HARQ-ACK codebook construction needs to be enhanced. Following two options can be considered:

- Option 1: Same HARQ-ACK codebook construction mechanism as in Rel.15

  - For SPS PDSCH reception without associated activation DCI, the HARQ-ACK bit(s) is/are placed after ones in response to dynamic PDSCH in the HARQ-ACK codebook.

  - For the first SPS PDSCH reception with activation DCI and/or SPS PDSCH release, the HARQ-ACK bits in the HARQ-ACK codebook are determined based on the value of counter DAI and/or total DAI indicated by the activation/deactivation DCI, as shown in Figure 1.

Figure 1 Type II HARQ-ACK CBS for Option 1

- Option 2: Generate two HARQ-ACK sub-codebooks (sub-CBs), the first sub-CB is for dynamic PDSCH scheduling and the second sub-CB is for DL SPS. Then the two sub-CBs are combined in a single HARQ-ACK codebook as shown in Figure 2.

  - For the second sub-CB, the HARQ-ACK bit(s) are arranged based on the following rules:

    - For first SPS PDSCH reception with activation DCI and/or SPS PDSCH release, the HARQ-ACK bit(s) is/are placed in the second HARQ-ACK sub-codebook based on the value of counter DAI and/or total DAI indicated by the activation/deactivation DCI where the counter DAI is dedicated for SPS PDSCH, which only counts SPS PDSCHs.

    - For SPS PDSCH reception without associated DCI, the HARQ-ACK bit(s) is/are placed after HARQ-ACK bits in response to SPS PDSCH reception with associated activation DCI and/or SPS PDSCH release.

Figure 2 Type II HARQ-ACK CBS for Option 2

More than one HARQ-ACK bits for multiple SPS PDSCHs receptions without associated DCI can be multiplexed into one HARQ-ACK codebook in Rel-16 with discussed options above. In the Type II HARQ-ACK codebook, the HARQ-ACK bit order for dynamic scheduled PDSCH can be determined based on the value of counter DAI and/or total DAI indicated by the corresponding scheduled DCI as Rel-15, while how to arrange the HARQ-ACK bit order for the SPS PDSCH reception without associated DCI needs to be defined in Rel-16. Two possible options are shown as below:

- Alt.1: HARQ-ACK bits for SPS PDSCH receptions without associated DCI are ordered in ascending order across serving cell index and then in descending order across the PDSCH-to-HARQ-feedback timing value K1 for SPS PDSCH.

Figure 3 Alt.1-HARQ-ACK bit order determination for SPS PDSCH receptions without associated DCI

- Alt.2: HARQ-ACK bits for SPS PDSCH receptions without associated DCI are ordered in descending order of the PDSCH-to-HARQ-feedback timing value K1 for SPS PDSCH for a serving cell and then in ascending order across serving cell index.

Figure 4 Alt.2-HARQ-ACK bit order determination for SPS PDSCH receptions without associated DCI

Proposal 3:

- To support HARQ-ACK(s) for more than one SPS PDSCH receptions/releases on the same PUCCH,

  - For one or more DL SPS configurations with SPS periodicity equal or larger than one slot, same HARQ-ACK codebook construction mechanism as in Rel.15 can be reused for Type I HARQ-ACK codebook.

    - FFS Type I HARQ-ACK codebook construction for DL SPS configurations with SPS periodicity shorter than one slot.

  - Down-select following two options for HARQ-ACK codebook construction for Type II HARQ-ACK codebook:

    - Option 1: Same HARQ-ACK codebook construction mechanism as in Rel.15 can be used, place HARQ-ACK bits for SPS reception after dynamic TB-based PDSCH HARQ-ACK codebook.

    - Option 2: HARQ-ACK bits for multiple SPS PDSCH are constructed as a SPS HARQ-ACK codebook, place SPS HARQ-ACK codebook after dynamic HARQ-ACK codebook for scheduled PDSCH.

Proposal 4:

- To support HARQ-ACK(s) for more than one SPS PDSCH receptions on a same PUCCH, down-select following two options for the HARQ-ACK bits order determination for SPS PDSCH receptions without associated DCI in the HARQ-ACK codebook:

  - Alt.1: HARQ-ACK bits for SPS PDSCH receptions without associated DCI are ordered in ascending order across serving cell index and then in descending order across the PDSCH-to-HARQ-feedback timing value for SPS PDSCH.

  - Alt.2: HARQ-ACK bits for SPS PDSCH receptions without associated DCI are ordered in descending order of the PDSCH-to-HARQ-feedback timing value for SPS PDSCH for a serving cell and then in ascending order across serving cell index.

HARQ-ACK feedback and PUCCH resource determination for SPS PDSCH reception without associated DCI only

For HARQ-ACK feedback for SPS PDSCH reception only, two options can be considered as follow:

- Option 1: Bundling HARQ-ACK bits corresponding to multiple SPS PDSCHs into one HARQ-ACK bit.

- Option 2: HARQ-ACK bits corresponding to multiple DL SPS PDSCHs are multiplexed in the same HARQ-ACK codebook where each HARQ-ACK bit is corresponding to one SPS PDSCH. The HARQ-ACK bits order for SPS PDSCH receptions in the HARQ-ACK codebook can be determined with the same method for Type II HARQ-ACK codebook.

For option 1, only one-bit HARQ-ACK is generated and PUCCH resource configuration for DL SPS in Rel.15 can be reused where only PUCCH format 0 or 1 can be configured for DL SPS, while as long as one SPS PDSCH is decoding failure, UE need to retransmit all the SPS PDSCHs. This will lead to very inefficient retransmission. For option 2, UE only need to retransmit the SPS PDSCH which is not detected correctly. In this case, more than two HARQ-ACK bits may need to be transmitted. Therefore, in addition to PUCCH format 0 or 1, format 2 or format 3 or format 4 should also be configured for DL SPS in Rel.16.

For PUCCH resource configuration and determination for SPS PDSCH reception only, in Rel.15, if a UE transmits HARQ-ACK information corresponding only to a PDSCH reception without a corresponding PDCCH, a PUCCH resource for corresponding PUCCH transmission with HARQ-ACK information is provided by n1PUCCH-AN. In Rel.16, the PUCCH resource configuration and determination for HARQ-ACK transmission for more than one SPS PDSCH receptions should be defined. For PUCCH resource configuration and determination, two options can be considered:

- Option 1: Reuse the Rel-15 PUCCH configuration mechanism for HARQ-ACK transmission for SPS PDSCH reception. In Rel-16, each SPS configuration is configured a PUCCH resource in RRC configuration by n1PUCCH-AN. When HARQ-ACK bit(s) for more than one SPS PDSCH receptions (without associated DCI) belonging different SPS configurations are transmitted on the same PUCCH, UE selects one PUCCH resource among all these available PUCCH resources for SPS PDSCH receptions. UE determines which PUCCH resource is used based on following option:

  - Alt.1: The PUCCH resource is derived from the first SPS PDSCH reception associated to the HARQ-ACK codebook.

  - Alt.2: The PUCCH resource is derived from the last SPS PDSCH reception associated to the HARQ-ACK codebook.

- Option 2: Use a similar way as PUCCH resource configuration and determination for HARQ-ACK transmission for dynamic PDSCHs in Rel-15. In Rel-16, multiple SPS configurations share multiple PUCCH resource sets configured by RRC. For the multiple PUCCH resource sets, they can be configured specifically for SPS only, or they can reuse the PUCCH resource sets configured for dynamic scheduled PDSCHs.  Then UE determines one PUCCH resource based on two steps:

  - Step 1: UE firstly determines which PUCCH resource set to use based on the payload size of the HARQ-ACKs for DL SPS.

  - Step 2: Then UE determines which PUCCH resource to use based on the following method:

    - Alt.1: UE selects a PUCCH resource in the PUCCH resource set based on a predefined rule. For example, the PUCCH resource with the earliest starting symbol is selected.

    - Alt.2:  UE selects a PUCCH resource based on the PRI indication of the last activation DCI.

Depending on the traffic arrival time of different services, different SPS configurations may be activated at different time. In this case, the number of HARQ-ACK bits that will be transmitted on a same PUCCH is variable depending on the traffic arrival time.  For option 1, the PUCCH resource for each SPS configuration is preconfigured by gNB,  in order to guarantee the transmission performance for HARQ-ACKs, gNB may need to configure a PUCCH resource with the largest payload for each SPS configuration. For instance, for a UE with 8 SPS configurations in a cell, PUCCH resource for each SPS configuration should be able to transmit 8-bits HARQ-ACK. In this case, only PUCCH format 2/3/4 can be configured. However, UE may only receive two activated SPS configurations which requires at most two HARQ-ACK bits. Therefore, the PUCCH resource configuration flexibility is limited and resource utilization efficiency for HARQ-ACKs for SPS is low.  Option 2 is more flexible and more efficient compared with option 1. Since a PUCCH resource is selected based on the real HARQ-ACK payload. Therefore, option 2 is slightly preferred.

Proposal 5：

- Support HARQ-ACK bits corresponding to multiple SPS PDSCHs without associated DCIs multiplexed in the same HARQ-ACK codebook.

  - All PUCCH formats can be configured for DL SPS.

  - Use a similar way as PUCCH resource configuration and determination for HARQ-ACK codebook transmission for dynamic PDSCHs in Rel-15. Multiple PUCCH resource sets can be used for HARQ-ACK codebook transmission for SPS PDSCH receptions.

    - UE firstly determines which PUCCH resource set to use based on the payload size of the HARQ-ACKs for DL SPS.

    - Then UE determines which PUCCH resource to use in the PUCCH resource set based on a predefined rule or PRI indication of the last activation DCI.

- UL intra-UE transmission prioritization/multiplexing

  - Resource conflicts between dynamic grant (DG) and configured grant (CG) PUSCH

For resource conflicts between DG and CG, and between multiple CGs, following agreements were made in RAN1 and RAN2 [10], [11].

Agreements in RAN1 #96: For scenario 2 as listed in R1-1814342, in case the collision between configured grant and dynamic grant occurs in physical layer, options to determine the prioritization between configured grant and dynamic grant include at least – to be further investigated during the WI phase: - Priority at PHY is determined by MAC layer for the purpose of PHY prioritization. - Note: this may or may not have any RAN1 impact - Priority at PHY is determined via using PHY channel(s)/signal(s)/parameters for the purpose of PHY prioritization. - It is configurable as part of the configured grant configuration whether it should have higher priority than dynamic grant in case of conflict. - Other options are not precluded.

Agreements in RAN2 #105 - RAN2 shall study resource conflicts between multiple active configured grants, in addition to Scenarios 2 and 3, part of UL data-data prioritization. - UE prioritization of a grant when there is at most one dynamic grant in the set of conflicting grants (scenario 2 and CG/CG collision) shall be addressed. MAC specifies currently the UE prioritization of such cases, and modifications to MAC would be required. - RAN2 assumes that the later dynamic grant may always be prioritized over and earlier dynamic grant (scenario 3). One way to realize this is that MAC generate a PDU for each grant and let L1 handle conflicting transmissions. To be confirmed following progress in RAN1. Other solutions are not precluded - For cases when MAC prioritizes a grant, MAC prioritizes the grant on which data of the highest priority can be transmitted according to LCP restrictions and priority configured for each LCH.

Firstly, we define following two cases as resource conflicts;

- Case 1: the two resources conflict only in time domain but do not conflict in the frequency domain;

- Case 2: the two resources conflict in both frequency and time domain.

Whether these two cases are handled in different manners or in the same manner should be considered.

(a) Case 1		 		                                              	(b) Case 2

Figure 5	Resource overlapping.

At least for case 2, a UE needs to prioritize one PUSCH transmission for UL. For case 1, it is possible that a UE can simultaneously process both PUSCHs if the UE has high capability; for example, if the UE is capable of intra-band UL-CA, the UE has at least baseband processing capability of simultaneously transmitting multiple PUSCHs in the given band. From RF point of view, careful study is necessary as this is a kind of non-contiguous transmission. It is useful if it is feasible to enable simultaneous processing in this situation. However, if the UE cannot simultaneously handle multiple PUSCHs, then same handling as for case 2 can be applied. In the following, we will mainly discuss the case where one of two transmissions needs to be prioritized on the conflict resource.

For resource conflicts between configured and dynamic grants, in Rel.15, MAC layer always priorities the resource associated with dynamic grant [12] as long as the UE has time to process the dynamic grant. In order to avoid the case where the UE needs to handle the resource conflicts between configured and dynamic grants without sufficient processing time, physical layer further specifies that the UE does not expect to be scheduled with a PUSCH transmission by a PDCCH that ends less than N2 symbols before the beginning of a valid uplink configured grant transmission occasion if the PUSCH overlaps with the configured grant transmission [13].

Figure 6 UE processing time requirement for dynamic grant overriding the configured grant PUSCH

Therefore, depending on whether or not there is sufficient processing time in physical layer, the prioritized grant selected in MAC layer may or may not be able to be transmitted. Therefore, there are two options for the collision handling between the CG and DG.

Option 1: For the case when MAC can prioritize a grant, MAC prioritizes the grant on which data of the highest priority can be transmitted according to LCP restrictions and priority configured for each LCH, and then MAC delivers the generated data to physical layer. For the case when MAC cannot prioritize a grant, MAC generates a PDU for each grant, implying that MAC may need to generate multiple PDUs if there are multiple grants. Physical layer makes the prioritization based on some explicit or side information related to the priority such as assisted information from MAC or parameters available in physical layer.

Option 2:  MAC generates a PDU for each grant according to the LCP restrictions and priority configured for each LCH, implying that MAC needs to generate multiple PDUs if there are multiple grants. Physical layer makes the prioritization based on some explicit or side information related to the priority such as assisted information from MAC or parameters available in physical layer.

Rel.15 procedure is part of option 1; MAC always knows data priority including CG vs DG since there is no case where the available processing time is less than the need. Two differences for Option 1: first, MAC needs to prioritize the configured grant when it associated with the URLLC traffic over the dynamic grant which is associated eMBB-like traffic. Second, Option 1 allows the case where the available processing time is less than the need, in which case, MAC cannot make the prioritization and deliver to physical layer the SDU with the higher priority. For example, MAC already generates and delivers the SDU associated with the earlier grant before knowing there will be a higher priority data comes. In this case, MAC layer needs to deliver the data with higher priority to the physical layer and provide the priority information to physical layer. Then physical layer can decide whether to it is feasible to transmit the prioritized data.

Option 2 is similar as one way we discussed for collision handling between dynamic grants, in which MAC generates a PDU for each grant and let L1 handle the conflicting transmissions.

Both option 1 and option 2 can work. Which one is better may depend on UE implementation. From specification perspective, Option 1 needs to explicitly define the cases when the MAC can and when the MAC cannot make the prioritization which requires specification efforts. Given that solutions are always needed to cover the case when MAC cannot make the prioritization, option 2 is preferred.

Proposal 6:

- Down-select following two options for resource conflicts handling between DG and CG.

  - Option 1: For the case when MAC can prioritize a grant, MAC prioritizes the grant on which data of the highest priority can be transmitted according to LCP restrictions and priority configured for each LCH. For the case when MAC cannot prioritize a grant, MAC generates a PDU for each grant, then physical layer makes the prioritization based on assisted information from MAC or parameters available in physical layer.

    - Confirm with RAN2 for the cases when the MAC can and cannot prioritize a grant.

    - FFS required assistant information from MAC to facilitate PHY to make the prioritization.

  - Option 2:  MAC generates a PDU for each grant according to the LCP restrictions and priority configured for each LCH. Physical layer makes the prioritization based on assisted information from MAC or parameters available in physical layer.

    - FFS required assistant information from MAC to facilitate PHY to make the prioritization.

  - Confirm with RAN2’s progress about both options.

  - Resource conflicts involving multiple configured grants (CGs)

RAN1 agreed that multiple active configured grant configurations for a given BWP of a serving cell should be supported at least for different services/traffic types and/or for enhancing reliability and reducing latency.

Above underlined parts imply that the following two use cases motivate to support multiple configured grant configurations:

- Use case 1: Support different service/traffic types with different requirements on latency, reliability, packet size etc., running simultaneously at the UE side;

- Use case 2: To ensure K repetitions without sacrificing the latency for a given URLLC service, similar as multiple UL SPS configurations supported in LTE HRLLC. Figure 3 gives an example. The main features for use case 2 are following:

  - The multiple configured grant configurations have the same periodicity but can have different time offsets

  - UE should start PUSCH transmission at the beginning of a first repetition of a transmission occasion of a configured grant configuration and continue K times repetition.

Figure 7 multiple GC configurations for reducing the latency and ensuring K repetitions

For use case 1 in which multiple CG configurations are used to support different services, two options as discussed for resource conflicts handling between DG and CG still applies. However, for use case 2 in which multiple CG configurations are used to guarantee the number of repetitions for a given BWP of a serving cell, an ongoing UL configured grant repetition transmission should not be interrupted by another UL configured grant repetition configuration having new data arriving.

Proposal 7:

- When multiple configured grant configurations are used to support different services/traffic types, same solution for handling the resource conflicts between DG and CG can be used for handling the resource conflicts involving multiple CGs.

Proposal 8:

- When multiple configured grant configurations are used to reduce latency and ensure reliability, an ongoing UL configured grant transmission including repetitions should not be interrupted by another UL configured grant configuration having new data arriving.

  - Collisions between different SPS configurations

Currently, the collision handling between multiple active UL configured grant configurations is listed to be one collision case need to be handled in intra-UE multiplexing. On the other hand, during IIOT/eURLLC SI, we have not discussed the collision case for multiple SPS configurations.  However, this collision case is obvious as long as we are going to specify the multiple SPS configurations and SPS with shorter periodicities. It is same as the collision case for multiple configured grant configurations; if a NW can avoid the SPS collision by configuration, then same logic can be applied to UL configured grant. However, we do not think it can be well handled by gNB if CA/DC is configured for the UE; in addition, using deactivation/activation signalling involves PDCCH overhead/blocking issues and UE may miss detect the deactivation/activation signalling. We understand this collision case has not been discussed in SI and the time for WI is quite limited. To reduce the specification efforts, before precluding this collision case, it is preferred to check whether the solutions discussed/agreed for other collision case like dynamic PDSCH v.s. dynamic PDSCH, collisions involving multiple UL configured grant configurations can be applied to the collision case involving multiple SPS configurations.

Proposal 9:

- To reduce the specification efforts, discuss collisions involving multiple SPS configurations after solutions are identified for other collision scenarios.

- Conclusion

In this contribution, we discussed physical layer enhancements for DL SPS and uplink transmission prioritization/multiplexing for intra-UE. Proposals are summarized as following:

Proposal 1:

- For DL SPS, support joint activation/release in a DCI for multiple SPS configurations for a given BWP of a serving cell.

  - FFS the details on how to support joint activation/release in a DCI for multiple SPS configurations.

Proposal 2:

- From physical layer perspective, it is feasible to support SPS periodicity shorter than 0.5 ms, e.g. down to two symbols without any limitations.

Proposal 3:

- To support HARQ-ACK(s) for more than one SPS PDSCH receptions/releases on the same PUCCH,

  - For one or more DL SPS configurations with SPS periodicity equal or larger than one slot, same HARQ-ACK codebook construction mechanism as in Rel.15 can be reused for Type I HARQ-ACK codebook.

    - FFS Type I HARQ-ACK codebook construction for DL SPS configurations with SPS periodicity smaller than one slot.

  - Down-select following two options for HARQ-ACK codebook construction for Type II HARQ-ACK codebook:

    - Option 1: Same HARQ-ACK codebook construction mechanism as in Rel.15 can be used, place HARQ-ACK bits for SPS reception after dynamic TB-based PDSCH HARQ-ACK codebook.

    - Option 2: HARQ-ACK bits for multiple SPS PDSCH are constructed as a SPS HARQ-ACK codebook, place SPS HARQ-ACK codebook after dynamic HARQ-ACK codebook for scheduled PDSCH.

Proposal 4:

- To support HARQ-ACK(s) for more than one SPS PDSCH receptions without associated DCI in one HARQ-ACK codebook, down-select following two options for the HARQ-ACK bits order determination for SPS PDSCH receptions without associated DCI in ne HARQ-ACK codebook:

  - Alt.1: HARQ-ACK bits for SPS PDSCH receptions without associated DCI are ordered in ascending order across serving cell index and then in descending order across the PDSCH-to-HARQ-feedback timing value for SPS PDSCH.

  - Alt.2: HARQ-ACK bits for SPS PDSCH receptions without associated DCI are ordered in descending order of the PDSCH-to-HARQ-feedback timing value for SPS PDSCH for a serving cell and then in ascending order across serving cell index.

Proposal 5：

- Support HARQ-ACK bits corresponding to multiple SPS PDSCHs without associated DCIs multiplexed in the same HARQ-ACK codebook.

  - All PUCCH formats can be configured for DL SPS.

  - Use a similar way as PUCCH resource configuration and determination for HARQ-ACK codebook transmission for dynamic PDSCHs in Rel-15. Multiple PUCCH resource sets can be used for HARQ-ACK codebook transmission for SPS PDSCH receptions.

    - UE firstly determines which PUCCH resource set to use based on the payload size of the HARQ-ACKs for DL SPS.

    - Then UE determines which PUCCH resource to use in the PUCCH resource set based on a predefined rule or PRI indication of the last activation DCI.

Proposal 6:

- Down-select following two options for resource conflicts handling between DG and CG.

  - Option 1: For the case when MAC can prioritize a grant, MAC prioritizes the grant on which data of the highest priority can be transmitted according to LCP restrictions and priority configured for each LCH. For the case when MAC cannot prioritize a grant, MAC generates a PDU for each grant, then physical layer makes the prioritization based on assisted information from MAC or parameters available in physical layer.

    - Confirm with RAN2 for the cases when the MAC can and cannot prioritize a grant.

    - FFS required assistant information from MAC to facilitate PHY to make the prioritization.

  - Option 2:  MAC generates a PDU for each grant according to the LCP restrictions and priority configured for each LCH. Physical layer makes the prioritization based on assisted information from MAC or parameters available in physical layer.

    - FFS required assistant information from MAC to facilitate PHY to make the prioritization.

  - Confirm with RAN2’s progress about both options.

Proposal 7:

- When multiple configured grant configurations are used to support different services/traffic types, same solution for handling the resource conflicts between DG and CG can be used for handling the resource conflicts involving multiple CGs.

Proposal 8:

- When multiple configured grant configurations are used to reduce latency and ensure reliability, an ongoing UL configured grant transmission including repetitions should not be interrupted by another UL configured grant configuration having new data arriving.

Proposal 9:

- To reduce the specification efforts, discuss collisions involving multiple SPS configurations after solutions are identified for other collision scenarios.

References

- RP-190726, ‘New WID: Physical Layer Enhancements for NR Ultra-Reliable and Low Latency Communication (URLLC),’ Huawei, HiSilicon

- RP-190728, “New WID: Support of NR Industrial Internet of Things (IoT),” Nokia, Nokia Shanghai Bell

- R1-1909194, ‘UCI enhancements for URLLC,’ NTT DOCOMO, INC.

- 3GPP RAN1 #96bis meeting, chairman’s notes.

- 3GPP RAN1 #97 meeting, chairman’s notes.

- R2-1905473, ‘LS on SPS/CG for IIoT,’ TSG RAN WG2

- R1-1907961, ‘Reply LS on SPS/CG for IIoT’, TSG RAN WG1

- R1-1903820, ‘LS on RAN1 NR UE features update,’ RAN1

- R1-1906758, ‘Support for SPS periodicities below a slot,’ Nokia, Nokia Shanghai Bell

- 3GPP, RAN1 #96 meeting, chairman’s notes

- 3GPP, RAN2 #105 meeting, chairman’s notes

- 3GPP TS 38.321 V15.4.0, ‘Medium Access Control (MAC) protocol specification’

- R1-1903783, ‘CR on dynamic grant overriding configured grant,’ Qualcomm, Intel