3GPP TSG RAN WG1 #98			R1-1909205

Prague, CZ, August 26th – 30th, 2019

Source:	NTT DOCOMO, INC.

Title:	Discussion on procedure of cross-slot scheduling power saving techniques

Agenda Item:	7.2.9.2

Document for: 	Discussion and Decision

- Introduction

At the RAN1#97 meeting, the issues related to procedure for cross-slot scheduling power saving techniques were discussed and RAN1 made following agreements [1].

Agreements: For an active DL and an active UL BWP, a UE can be indicated via L1-based signalling(s) from gNB to adapt the minimum applicable value(s) of K0, K2 and/or aperiodic CSI-RS triggering offset (with/without QCL_typeD configured). Agreements: To adapt the minimum applicable value of K0 (K2) for an active DL (UL) BWP, indication of the minimum applicable value is supported. - FFS: Direct assignment of the minimum application value, indication of one value from one or multiple preconfigured or predetermined value(s), and/or implicit indication. - FFS: How the indicated minimum applicable value is applied to the selection of a DL (UL) TDRA entry. Example directions include at least the following: - Excluding the invalid TDRA entries - Re-interpret the selected K0 (K2) value Agreements: When UE is indicated of the minimum applicable value of K0 (K2) for an active DL (UL) BWP, the application method to the selection of a DL (UL) TDRA entry is to be decided from: - An entry in the active DL (UL) TDRA table with K0 (K2) value smaller than the indicated minimum is not expected by or not valid for the UE for the TDRA indication(s) Agreements: At least for the L1-based adaptation on the minimum applicable value of K2, it does not apply to PUSCH scheduled by MAC RAR for at least contention-based RACH procedure. - FFS: Exclude contention-free RACH - FFS: Exclude PUSCH scheduled with TC-RNTI Agreements: - To adapt the minimum applicable value of the aperiodic CSI-RS triggering offset for an active DL BWP, implicit indication by defining the minimum applicable value the same as the minimum applicable K0 value when indicated is supported. Agreements: At least for the L1-based adaptation on the minimum applicable value of K2, it does not apply to: - PUSCH scheduled by RAR UL grants for contention-free RACH procedure - PUSCH scheduled with TC-RNTI Conclusion: Companies are encouraged to check the following proposal for the application delay: For an active DL and an active UL BWP, when UE is indicated by L1-based signalling(s) in slot n to change the minimum applicable value(s) of K0 and/or K2, UE is not expected to apply the new indicated minimum applicable value(s) before slot n+X⋅2μPDSCH2μPDCCH for K0, or slot n+X⋅2μPUSCH2μPDCCH for K2, where - X = max(Y, Z) - Y is the minimum applicable K0 value prior to the indicated change - Z = [1] - Z is the smallest feasible non-zero application delay that may depend on DL SCS - FFS: Z > 1 for 60kHz/120kHz SCS or multi-TRP - FFS: Cross-carrier scheduling with different numerologies - FFS: interruption time, if any

In this contribution, power saving scheme based on cross-slot scheduling is discussed.

- Discussion

Power saving scheme based on cross-slot scheduling was discussed and evaluated, and the power saving gain was shown. Basic motivation on the scheme is to ensure some duration after the PDCCH reception so that the UE does not have to perform preparation for possible subsequent PDSCH reception before the completion of PDCCH decoding, which results in saving the power consumption. If the possible minimum values of K0 is large enough not to start the preparation for PDSCH reception, UE can skip any preparation for PDSCH reception and go to sleep when the UE is not scheduled by the PDCCH. Thus, the value of possible minimum values of K0 for the power saving scheme should be considered based on the assumed UE processing time for PDCCH decoding and preparation for PDSCH reception. Thus, it was agreed that indication of the minimum applicable value is supported. The remaining discussion point is how to indicate the minimum applicable value. The possible minimum applicable value should be at most several slots, and the large number of bits would not be needed. Therefore, it should be enough to indicate one value from predetermined values, i.e., specified values. In addition, also in the case of UL scheduling, the same method can be applied, i.e., for the minimum applicable value of K2.

Proposal 1: One value from predetermined values is indicated as the minimum applicable value of K0 (or K2).

Regarding candidate values of the minimum applicable value, it should be considered whether the value(s) larger than 1 is necessary or not. The minimum applicable value larger than 1 can provide more power saving gain in some cases since it enables the relaxed PDCCH processing depending on PDCCH monitoring periodicity. Thus, the minimum applicable value(s) larger than 1, e.g., the candidate values of 0,1,2 and 3, should be supported in order to increase power saving gain. In addition, considering cross-BWP scheduling and cross-carrier scheduling, the minimum applicable value of 1 may not be sufficient to skip any preparation of switching and PDSCH reception. If so, power saving can be additionally increased.

Proposal 2: The minimum applicable value(s) larger than 1 should be supported.

- Conclusion

In this contribution, power saving scheme based on cross-slot scheduling was discussed. Based on the discussion, the following proposals were made:

Proposal 1: One value from predetermined values is indicated as the minimum applicable value of K0 (or K2).

Proposal 2: The minimum applicable value(s) larger than 1 should be supported.

References

- 3GPP RAN1#97, Chairman’s note, May 2019.