3GPP TSG RAN WG1 #98			R1-1909229

Prague, CZ, August 26th – 30th, 2019

Agenda item:	5

Source: 	Qualcomm Incorporated

Title: 	CSI dropping in C-DRX

Document for:	Discussion/Decision

# Introduction

According to the current MAC-layer specification (TS38.321 v15.5.0, subclause 5.7), when DRX is configured,

1>	if CSI masking (csi-Mask) is setup by upper layers:

2>	in current symbol n, if onDurationTimer would not be running considering grants/assignments/DRX Command MAC CE/Long DRX Command MAC CE received 4 ms prior to symbol n when evaluating all DRX Active Time conditions as specified in this subclause:

3>	not report CSI on PUCCH.

1>	else:

2>	in current symbol n, if the MAC entity would not be in Active Time considering grants/assignments/DRX Command MAC CE/Long DRX Command MAC CE received and Scheduling Request sent 4 ms prior to symbol n when evaluating all DRX Active Time conditions as specified in this subclause:

3>	not report CSI on PUCCH and semi-persistent CSI on PUSCH.

RAN2 has identified an issue in UCI multiplexing due to “not reporting CSI on PUCCH” in C-DRX. The issue is described in a LS (R2-1908159) [1]“RAN2 has discussed UE behavior in the case where a periodic CSI or semi-persistent CSI over PUCCH, in addition to HARQ N/ACK, overlaps with a PUSCH in symbol n. When simultaneousHARQ-ACK-CSI is enabled, if UE performs UCI multiplexing between CSI and HARQ N/ACK according to the current PHY-layer specification (TS38.213 v15.5.0, subclause 9.2.1), then with some PUCCH configurations, it seems possible that the resulting multiplexed UCIs can be relocated to symbols that are in the same slot but no longer overlap with PUSCH. If that happens, the CSI has to be dropped, together with the HARQ N/ACK, according to the requirements above. That is because the multiplexed UCIs would have to be sent over PUCCH instead of PUSCH, as initially expected. This is not a desirable consequence, as HARQ N/ACK is dropped for no good reason. Note that this issue is possible only when DRX is configured.”

The LS (R2-1908159) has been sent to RAN 1 to “ask RAN1 to discuss whether the scenario described above is possible. If it is possible, whether the issue can be resolved in RAN1”.

In this contribution, the root cause for this issue is identified and two alternative solutions are proposed. The two alternatives are then evaluated and compared in a few UCI multiplexing scenarios in Section 3. Based on the observed pros and cons, alternative 2 is proposed as the solution to this issue in Section 4.

# Root cause of the issue

The root case of this issue is that in UCI multiplexing, MAC does not know a CSI report is going to be transmitted on which resource, i.e., on which symbols, when MAC makes the decision to “not report CSI on PUCCH”. More specifically, after UCI multiplexing procedure performed by PHY, a CSI report can be transmitted on PUSCH or on a “new” PUCCH resource, which is on a different resource than the original RRC configured resource for the CSI report. But MAC does not know where is the “new” PUCCH or PUSCH resource. In other words, MAC does not know CSI reports is transmitted on which OFDM symbols, after UCI multiplexing.

Therefore, to resolve this issue, the key question we need to answer is the following:

- Q1: When does a UE check dropping a CSI report or not based on DRX active time on/off condition? In other words, how to interpret “in current symbol n” in RAN2 spec.

There are two alternatives answers to the above question:

- Alternative 1: a UE check dropping a CSI report or not per “current symbol n” where the CSI report transmission occurs.

- Alternative 2: a UE check dropping a CSI report or not at the starting symbol of the RRC configured PUCCH resource for the CSI report.

The pros and cons of the two alternatives are examined in a few typical UCI multiplexing scenarios with C-DRX configured in Section 3.

# CSI Dropping Scenarios in C-DRX

Scenario 1: DRX active time on in the whole slot.

Figure 1: DRX active time on in the whole slot (scenario 1)

This is the nominal scenario. CSI dropping is not triggered per RAN2 spec. UE behavior is clearly defined in RAN1 spec. There is no CSI dropping issue in C-DRX in this scenario.

Scenario 2: DRX active time off in the whole slot.

Figure 2: DRX active time off in the whole slot. UCI Mux with “floating” new PUCCH resource (scenario 2)

Figure 3: UE behaviors in scenario 2 with two different alternatives

- Alt 1: a UE check dropping a CSI report or not per “current symbol n” where the CSI report transmission occurs;

  - UE behavior: As UE does not know CSI will be transmitted on PUSCH or PUCCH, UE cannot drop CSI (follow RAN2 spec). UE then (follow RAN1 spec) calculate a new PUCCH resource. If the new PUCCH resource overlaps with PUSCH, report P-CSI (follow RAN2 spec), and piggyback ACK+P-CSI on PUSCH (follow RAN1 spec). If the new PUCCH resource does not overlap with PUSCH, drop P-CSI (follow RAN2 spec), piggyback ACK on PUSCH (follow RAN1 spec).

  - The UE behavior requires back-and-forward interactive between RAN1/RAN spec. Furthermore, it requires NBC change in RAN1 spec to capture the shadowed box in figure above, as current RAN1 UCI multiplexing procedure does not take DRX active time on/off condition into account.

- Alt 2: a UE check dropping a CSI report or not at the starting symbol of the RRC configured PUCCH resource for CSI.

  - UE behavior: P-CSI is dropped (follow RAN2 spec). From RAN 1 perspective, UE only sees ACK overlap with PUSCH then. ACK is piggybacked on PUSCH (follow RAN1 spec).

  - This UE behavior has no back-and-forward interactive between RAN1/RAN spec. Furthermore, no RAN1 spec change is needed.

Scenario 3: DRX active time on/off switch dissect a slot. UCI Mux on floating new resource

Figure 4: DRX active time on/off switch dissect a slot. UCI Mux on floating new resource (scenario 3)

Figure 5: UE behaviors in scenario 3 with two different alternatives

- Alt 1: a UE check dropping a CSI report or not per “current symbol n” where the CSI report transmission occurs;

  - UE behavior: As UE does not know CSI will be transmitted on PUSCH or PUCCH, neither UE knows DRX active time is on or off after UCI multiplexing, UE cannot drop CSI (follow RAN2 spec). UE then (follow RAN1 spec) calculate new PUCCH resource. If new PUCCH resource overlaps with PUSCH, report P-CSI (follow RAN2 spec), and piggyback ACK+P-CSI on PUSCH (follow RAN1 spec). If new PUCCH resource does not overlap with PUSCH, UE needs to check active time on or off (follow RAN2 spec). if on, report P-CSI (follow RAN2 spec), and piggyback ACK+P-CSI on the new PUCCH resource. If off, drop P-CSI (follow RAN2 spec), piggyback ACK on PUSCH (follow RAN1 spec). (follow RAN1 spec).

  - The UE behavior requires back-and-forward interactive between RAN1/RAN spec. Furthermore, it requires NBC change in RAN1 spec to capture the shadowed box in figure above, as current RAN1 UCI multiplexing procedure does not take DRX active time on/off condition into account.

- Alt 2: a UE check dropping a CSI report or not at the starting symbol of the RRC configured PUCCH resource for CSI.

  - UE behavior: P-CSI is reported (follow RAN2 spec). From RAN 1 perspective, UE sees both P-CSI and ACK overlap with PUSCH. UE (follow RAN1 spec) to do UCI multiplexing.

  - This UE behavior has no back-and-forward interactive between RAN1/RAN spec. Furthermore, no RAN1 spec change is needed.

Scenario 4: DRX active time on/off switch dissect CSI transmission

Figure 6: DRX active time on/off switch dissect CSI transmission (scenario 4)

Figure 7: UE behaviors in scenario 4 with two different alternatives

- Alt 1: a UE check dropping a CSI report or not per “current symbol n” where the CSI report transmission occurs;

  - UE behavior: As UE does not know CSI will be transmitted on PUSCH or PUCCH, neither UE knows DRX active time is on or off after UCI multiplexing, UE cannot drop CSI (follow RAN2 spec). UE then (follow RAN1 spec) calculate new PUCCH resource. If new PUCCH resource overlaps with PUSCH, report P-CSI (follow RAN2 spec), and piggyback ACK+P-CSI on PUSCH (follow RAN1 spec). If new PUCCH resource does not overlap with PUSCH, UE needs to check active time on or off on per symbol basis (follow RAN2 spec). if on, report P-CSI (follow RAN2 spec), and piggyback ACK+P-CSI on the new PUCCH resource (follow RAN1 spec). If off, drop P-CSI (follow RAN2 spec), piggyback ACK on PUSCH (follow RAN1 spec).”

  - The UE behavior requires back-and-forward interactive between RAN1/RAN spec. Furthermore, it requires NBC change in RAN1 spec to capture the shadowed box in figure above, as current RAN1 UCI multiplexing procedure does not take DRX active time on/off condition into account. To make it even worse, DRX active time on/off condition check per symbol is very hard to implement, because the UE needs first following RAN1 spec to determine CSI is transmitted on which symbols in the new PUCCH resource then following RAN2 spec to check DRX active time on/off for each symbol where CSI is transmitted.

- Alt 2: a UE check dropping a CSI report or not at the starting symbol of the RRC configured PUCCH resource for CSI.

  - UE behavior: P-CSI is reported (follow RAN2 spec). From RAN 1 perspective, UE sees both P-CSI and ACK overlap with PUSCH. UE (follow RAN1 spec) to do UCI multiplexing.

  - This UE behavior has no back-and-forward interactive between RAN1/RAN spec. Furthermore, no RAN1 spec change is needed.

Scenario 5: Multi-CSI-PUCCH-ResourceList is configured

Figure 8: DRX active time on/off switch dissect a slot where Multi-CSI resource is configured (scenario 5)

In this scenario, according to RAN1 Spec. 38.213 Subclause 9.2.5, if any of the multiple single-PUCCH resource overlaps, the UE multiplexed all CSI reports and transmitted in one of the two multi-CSI PUCCH resources. Selection of the multi-PUCCH resource for transmission is based on the total payload size of the multiplexed CSI reports. Therefore, the activation of the SP-CSI report might change the resource selection of the multi-CSI resource. Since the DRX active time on/off condition may be different on the two multi-CSI resources, UE behavior will be different depends on when and how does UE perform DRX active time on/off check.

Figure 9: UE behaviors in scenario 5 with two different alternatives

- Alt 1: a UE check dropping a CSI report or not per “current symbol n” where the CSI report transmission occurs;

  - UE behavior: As UE does not know CSI will be transmitted on PUSCH or single-CSI PUCCH resource or multi-CSI PUCCH resource, neither UE knows DRX active time is on or off after UCI multiplexing, UE cannot drop CSI (follow RAN2 spec). UE then (follow RAN1 spec) to determine a multi-CSI PUCCH resource. Furthermore, UE then (follow RAN1 spec) to calculate new PUCCH resource. If the new PUCCH resource overlaps with PUSCH, report P-CSI but drop SP-CSI (follow RAN2 spec), and piggyback ACK+P-CSI on PUSCH (follow RAN1 spec). If the new PUCCH resource does not overlap with PUSCH, UE needs to check active time on or off on per symbol basis (follow RAN2 spec). if on, report P-CSI (follow RAN2 spec), and piggyback ACK+P-CSI on the new PUCCH resource (follow RAN1 spec). If off, drop P-CSI (follow RAN2 spec), piggyback ACK on PUSCH (follow RAN1 spec).”

  - The UE behavior requires back-and-forward interactive between RAN1/RAN spec. Furthermore, it requires NBC change in RAN1 spec to capture the shadowed box in figure above, as current RAN1 UCI multiplexing procedure does not take DRX active time on/off condition into account. To make it even worse, DRX active time on/off condition check per symbol is very hard to implement, because the UE needs first following RAN1 spec to determine CSI is transmitted on which symbols in the new PUCCH resource then following RAN2 spec to check DRX active time on/off for each symbol where CSI is transmitted.

- Alt 2: a UE check dropping a CSI report or not at the starting symbol of the RRC configured PUCCH resource for CSI.

  - UE behavior: Each single P-CSI or SP-CSI is decided to drop or not (follow RAN2 spec). UE (follow RAN1 spec) to do determined multi-CSI resource and perform UCI multiplexing.

  - This UE behavior has no back-and-forward interactive between RAN1/RAN spec. Furthermore, no RAN1 spec change is needed.

Another disadvantage of Alt 1 in this scenario 5 is the strange behavior of “not reporting semi-persistent CSI on PUSCH”, according to RAN2 spec. As shown in Figure 8, suppose combining P-CSI report 1, P-CSI report 2, and SP-CSI ends up at using multi-CSI resource 2. As multi-CSI resource 2 is overlapping with PUSCH, CSI should be multiplexed on PUSCH. However, according to RAN2 spec, UE should not report semi-persistent CSI on PUSCH, when DRX active time is off. Therefore, UE should drop SP-CSI. Dropping SP-CSI leave an ambiguity to UE behavior. Should UE restart UCI multiplexing procedure from beginning which may end up combined P-CSI report 1 and P-CSI report 2 in multi-CSI resource 1, and multi-CSI resource 1 is then transmitted in TDM fashion with PUSCH and HARQ-ACK? Or should UE not restart multiplexing procedure and just multiplex P-CSI report 1, P-CSI report 2 on PUSCH, together with HARQ-ACK? Even put the above ambiguity aside, it is still strange to allow P-CSI piggyback on PUSCH but disallow SP-CSI piggyback on PUSCH, when DRX active time is off. However, with Alt 2, the UE behavior is very clear, as UE decides that P-CSI/SP-CSI is either dropped or not dropped before UCI multiplexing procedure.

# Solution to CSI dropping in C-DRX

Based on the above analysis of UE behaviors under a few typical UCI multiplexing scenarios in DRX, we have the following observations

- Alternative 2 is much simpler than alternative 1. Alternative 2 leads to clean partition between RAN1 and RAN2 specification without iterative UE behavior between PHY/MAC or RAN1/RAN2 specifications. While, alternative 1 requires quite a few iterations between RAN1 and RAN2 specifications.

- Alternative 2 needs no RAN1 spec update. Alternative 1 needs significant modification of RAN1 spec.

- Alternative 1 has no benefit over alternative 2 from technical perspective.

Based on the above observation, we propose to take alternative 2 to resolve issues for CSI dropping and UCI multiplexing in DRX.

Proposal 1: When DRX is configured, for each RRC configured CSI report on PUCCH, UE check dropping the CSI report or not based on DRX active time on or off at the starting symbol of the RRC configured PUCCH resource for the CSI report.

# Extensions

## How to handle TA

MAC layer does not know about PUCCH physical transmission time with TA due to two reasons. The first reason is that MAC does not know UE’s downlink (physical) reception time. The second reason is that the TA tracking loop is performed at PHY layer hence MAC does not know the TA value. MAC layer sees the logic transmission time of SFN index, subframe index, and OFDM symbol index. As according to RAN2 spec, CSI report dropping is decide by MAC layer. Therefore, MAC should use logical transmission time to decide CSI dropping or not without accounting TA.

Figure 10: CSI report dropping in C-DRX with TA

Proposal 2: When DRX is configured, UE decides dropping a CSI report or not based on DRX active time on/off condition at the starting symbol of the RRC configured PUCCH resource for the CSI report, excluding the TA applied to the starting symbol.

## PUCCH repetition cross multiple slots

There are two options to decide drop CSI report(s) or not with PUCCH repetition cross slots, as shown in Figure 11. Since the motivation of PUCCH repetition is for coverage enhancement for link budget limited UEs, it does not make much sense to decide drop CSI reports in some slot while transmit them in the rest. Therefore, option 1, where all repetitions of CSI is transmitted to guarantee the CSI can get through or all of them are dropped to avoid interfering other UEs’ UL transmission, is a more reasonable solution is preferred.

Figure 11: CSI report dropping in C-DRX with PUCCH repetition cross slots

Proposal 3: When DRX is configured, for a CSI report with repetition cross slots, UE decides dropping the CSI report or not based on DRX active time on/off condition at the starting symbol in the first slot of the RRC configured PUCCH resource for the CSI report.

## How to handle on/off switch dissect an OFDM symbol of PUCCH

Due to different numerologies used between DL and UL, it could happen that the DRX active time on/off switch dissects an OFDM symbol on PUCCH, as shown in figure below. In this case, when evaluate the DRX active time on/off condition, the condition is evaluated at the starting point of the symbol, more specifically speaking, which is the first chip (aka Tc) of an OFDM symbol. For example, if the DRX active time on/off condition is evaluated at the first symbol of the RRC configured PUCCH resource, and the first symbol is dissected by the on/off switch, the DRX active time on/off condition is evaluated at the beginning (which is the first chip, aka Tc) of the first symbol, where Tc is the smallest unit of time in NR system.

Figure 11: DRX active on/off switch dissects an OFDM symbol

Proposal 4: When DRX is configured, if DRX active time on/off switch within the starting symbol of the RRC configured PUCCH resource for the CSI report, UE decides dropping the CSI report or not based on DRX active time on/off condition at the first Tc of the starting symbol of the RRC configured PUCCH resource for the CSI report.

# Conclusions

Proposal 1: When DRX is configured, for each RRC configured CSI report on PUCCH, UE check dropping the CSI report or not based on DRX active time on or off at the starting symbol of the RRC configured PUCCH resource for the CSI report.

Proposal 2: When DRX is configured, UE decides dropping a CSI report or not based on DRX active time on/off condition at the starting symbol of the RRC configured PUCCH resource for the CSI report, excluding the TA applied to the starting symbol.

Proposal 3: When DRX is configured, for a CSI report with repetition cross slots, UE decides dropping the CSI report or not based on DRX active time on/off condition at the starting symbol in the first slot of the RRC configured PUCCH resource for the CSI report.

Proposal 4: When DRX is configured, if DRX active time on/off switch within the starting symbol of the RRC configured PUCCH resource for the CSI report, UE decides dropping the CSI report or not based on DRX active time on/off condition at the first Tc of the starting symbol of the RRC configured PUCCH resource for the CSI report.

# References

- 3GPP TSG-RAN WG2 Meeting #106, R2-1908159 “LS on CSI reporting in C-DRX,” Reno, USA, 13 - 17 May 2019.