3GPP TSG-RAN WG1 Meeting #98	R1-1909239

Prague, Czech Republic, 26th – 30th August, 2019

Agenda item:	7.2.1.1

Source: 	Qualcomm Incorporated

Title: 	Channel Structure for Two-Step RACH

Document for:		Discussion and Decision

# 1. 	Introduction

At RAN #84 meeting, the scope and objectives of two-step RACH WI [1] were confirmed for NR Rel-16. The agreements and open issues for RAN1 and RAN2 (up to June 2019) were summarized in [2]. In particular, the following agreements were made on the channel structure for two-step RACH [2-5]:

Agreements:

- PUSCH occasion for 2-step RACH is defined as the time-frequency resource for PUSCH transmission.

Agreements:

- One or more PUSCH occasion(s) within an msgA PUSCH configuration period are configured.

  - FFS msgA PUSCH configuration period

Agreements:

- PUSCH resource unit for 2-step RACH is defined as the PUSCH occasion and DMRS port/DMRS sequence used for an msgA payload transmission.

  - FFS support only one or both of DMRS port / DMRS sequence.

  - The DMRS sequence generation mechanism should follow Rel.15.

Agreements:

- Both DFT-s-OFDM and CP-OFDM are supported for the payload transmission in msgA

  - FFS how to indicate/configure the waveform

- Consider the following numerology for msgA PUSCH (for possible down-selection)

  - Alt 1: ​follow the numerology configured for the UL BWP

    - FFS initial vs. active UL BWP

  - Alt 2:  same as msgA preamble numerology at least for some cases

    - E.g., when short preamble is used (L=139)

Agreements:

- Support the PRACH and PUSCH for msgA transmission in different slots. In this case, the numerology for msgA PUSCH follows the numerology configured for the UL BWP for msgA transmission.

  - FFS whether to support PRACH and PUSCH in the same slot for msgA transmission. If supported, down-select from the following options:

    - Option 1: the numerology for msgA PUSCH follows that of msgA preamble.

    - Option 2: gNB configures whether the numerology for msgA PUSCH follows that of msgA preamble or UL BWP.

    - Option 3: a UE is not expected to be configured with different numerology among PRACH preamble, msgA PUSCH and UL BWP for msgA transmission.

Agreements:

- Further study the granularity of the time advance command, if supported in MsgB.

  - E.g., Based on the subcarrier spacing of MsgA PUSCH using a 12-bit TA command, where the granularity of the TA command is determined according to the following table.

| Subcarrier Spacing (kHz) of the PUSCH | Unit |
| --- | --- |
| 15 | 16*64 Tc |
| 30 | 8*64 Tc |
| 60 | 4*64 Tc |
| 120 | 2*64 Tc |

  - Other options/variations are not precluded.

Agreements:

- The following parameters are defined per msgA PUSCH configuration:

  - Common parameters for both option 1 (separate configuration) and option 2 (relative location), at least include:

    - MCS and/or TBS (to be further decided)

    - Number of FDMed POs

    - POs (including guard band or guard period, if exist) under the same msgA PUSCH configurations are consecutive in frequency domain

    - Number of PRBs per PO

    - Number of DMRS symbols/ports/sequences (if support) per PO

    - FFS whether or not support repetitions for msgA PUSCH

    - FFS bandwidth of PRB-level guard band or duration of guard time

    - FFS PUSCH mapping type

  - Parameters specific to option 1, at least include:

    - Periodicity (msgA PUSCH configuration period)

    - FFS value range

    - Offset(s) (e.g., symbol, slot, subframe, etc.)

    - Time domain resource allocation, details FFS, e.g., in a slot for msgA PUSCH: starting symbol, number of symbols per PO, number of time-domain POs, etc.

    - Frequency starting point

  - Parameters specific to option 2, at least include:

    - Single time offset (combination of slot-level and symbol-level indication) with respect to a reference point

    - FFS, e.g., each PRACH slot (e.g., start or end of the PRACH slot), etc.

    - Number of symbols per PO

    - FFS explicit or implicit indication

    - Single frequency offset with respect to FFS (the start of the first RO in frequency or the end of the last RO in frequency)

  - FFS: Number of TDMed POs

- Support multiple msgA PUSCH configurations for a UE

  - FFS the maximum number of configurations

  - FFS which parameters, if any, are common for all configurations

  - FFS indication of different msgA PUSCH configurations, e.g. by different ROs, by different preamble groups, or by UCI

  - FFS whether or not resources for different msgA PUSCHs can be overlapped in time-frequency, and if so, any spec impact

- FFS whether the frequency resource of msgA PUSCH should be limited to the bandwidth of PRACH

- FFS validation rule of msgA PUSCH

Agreements:

- The c_init for msgA PUSCH scrambling is at least derived based on a RNTI, preamble index, and/or n_ID (which can be cell ID or configurable, to be FFS).

  - FFS details of the RNTI

  - FFS the inclusion of DMRS index

Agreements:

- MsgA shall support all the preamble formats specified for NR release 15.

In this contribution, we will discuss the remaining issues for message A (msgA) and message B (msgB) channel structure. The procedures related to two-step RACH are discussed in a companion paper [6] and the references therein.

# 2. 	Discussion

## 2.1	Overview of Two-Step RACH

Table 1: Physical Signals and Channels Used by Two-Step RACH

Figure 1:  Example of Two-Step RACH Transmission

Table 1 lists the physical signals/channels used by msgA and msgB, as well as the contents of each message type. Figure 1 shows an example of contention-based two-step RACH transmission.

## 2.2 	Channel Structure for msgA

Figure 2: General Channel Structure for msgA

A general channel structure for msgA is shown in Figure 2, which is applicable to both licensed spectrum (FR1 and FR2) and unlicensed spectrum (FR1).

The preamble and payload of msgA are TDMed [1], which are transmitted on pre-define msgA occasions, as shown by Figure 3. Each PRU group comprises multiple PRUs, which share the same time/frequency resources but are configured with different DMRS resources.

The bandwidth configured for msgA preamble and msgA payload can be same or different. The total bandwidth of msgA occasions can be given by [RBmsgA,start, RBmsgA, end], as shown by Figure 3. For RRC IDLE or INACTIVE state, the bandwidth of msgA occasions should be within the initial UL BWP. For RRC CONNECTED state, the total bandwidth of msgA occasions should be within the active UL BWP.

Figure 3: Time and Frequency Resource Configuration for msgA

Therefore, we have the following observation and proposals:

Observation 1:  A msgA transmission occasion comprises a PRACH occasion (RO) and a set of PUSCH occasions (PO). To transmit a msgA preamble, a two-step RACH UE selects a valid RO and a preamble sequence configured on the valid RO. To transmit a msgA payload, a two-step RACH UE selects a set of valid POs and one or multiple PRUs configured on the valid POs.

Proposal 1: The association between SSB and msgA occasion (SSB-to-msgA association) can be defined similar to SSB-to-RO association. The association pattern between SSB and msgA occasion can be indicated in SI or RRC.

Proposal 2: The bandwidth used for msgA preamble and payload transmission can be similar or different. Within an association period of SSB-to-msgA, the total bandwidth configured for msgA occasions is within initial UL BWP (RRC IDLE or INACTIVE) or active UL BWP (RRC CONNECTED).

### 2.2.1	Preamble Transmission of msgA

msgA Preamble Occasion

- Configurations for msgA preamble time occasions can be based on “PRACH Configuration Index” in NR Rel-15 (Table 6.3.3.2-2, 6.3.3.2-3 and 6.3.3.2-4, TS 38.211).

- Two-step RACH and four-step RACH can use separate ROs, or share the same RO but configured with mutually exclusive preamble sequences.

- When new ROs are configured for msgA preamble, the RO distribution in time for two-step RACH and four-step RACH should be able to associate with different SSB beams.

- Two-step RACH and four-step RACH can be configured with different preamble formats, different time/frequency occasions and different SSB-to-RO association patterns.

Configurable Time Gap Before msgA Preamble Transmission

- In FDD or TDD, a configurable transmission gap Tgap, 1 needs to be considered before msgA preamble transmission, if msgA is transmitted after another UL symbol (from the same UE) and the UL symbol is using a SCS different from msgA preamble.

- In TDD, if msgA preamble transmission starts after SSB or any other DL transmission, a transmission gap  Tgap, 1 is needed after the last DL reception symbol. The time duration of Tgap, 1 depends on the preamble format, the numerology of PUSCH in initial/active UL BWP, the frequency band and the processing capability of UE.

  - For initial transmission of msgA, Tgap,1≥Ngap symbols, wherein the symbol numerology is based on the PUSCH SCS configuration in initial/active UL BWP. The values of Ngap have been specified in TS 38.213 Clause 8.1, and are shown in Table 2 as well.

  - For retransmission of msgA, msgB needs to be processed before preamble/payload retransmission. Therefor, Tgap,1≥NT,2+ΔBWPSwitching+ΔDelaymsec, wherein  if the UL BWP does not change and  is defined in TS 38.133 otherwise; ∆Delay depends on the frequency band, namely

∆Delay=0.5  msec,  FR10.25 msec,       FR2

the time duration of NT,2 equals N2 symbols, wherein the value of  N2 depends on the UE processing capability, and the symbol numerology is related to the SCS configuration for PUSCH in the initial/active UL BWP.

Table 2: Minimum TX Gap Before msgA Preamble for Initial Transmission of msgA

| Preamble Format | Preamble SCS | Ngap |
| --- | --- | --- |
| 0, 1, 2, 3 | 1.25 kHz or 5 kHz | 0 |
| A1, A2, A3, B1,B2, B3, C0, C2 | 15 kHz or 30 kHz or 60 kHz or 120 kHz | 2 |
| B4 | 15 kHz or 30 kHz or 60 kHz or 120 kHz | 0 |

Table 3: NT,2 for PUSCH Timing Capability 1

|  | N2 [symbols] |
| --- | --- |
| 0 | 10 |
| 1 | 12 |
| 2 | 23 |
| 3 | 36 |

Table 4: NT,2 for PUSCH Timing Capability 2

|  | N2 [symbols] |
| --- | --- |
| 0 | 5 |
| 1 | 5.5 |
| 2 | 11 (FR1) |

Configurable Time Gap After msgA Preamble Transmission

- After msgA preamble transmission, another time gap Tgap,2 (Tgap,2≥Ngap,2  symbols) needs to be inserted before payload transmission, if there is a change in the transmission parameters for msgA payload such as SCS. Example for Ngap,2  configuration is shown in Table 5.

Table 5: Minimum TX Gap Between msgA Preamble and Payload Due to SCS Change

| Preamble Format | Preamble SCS |  | Ngap,2 [symbols] |
| --- | --- | --- | --- |
| Any | if different from μ | 0, 1 | 2 |
| Any | if different from μ | 2, 3 | 4 |

- A discussion for Tgap,2 configuration in NR-U can be found in [7].

Bandwidth and Numerology for msgA Payload Transmission

- Depending on the payload size, MCS and slot format, the transmission bandwidth for msgA preamble and payload can be same or different.

- In NR Rel-15, frequency hopping (intra-slot and inter-slot) and slot repetition are supported in configured grant transmission. Therefore, frequency hopping and slot repetition should be supported for msgA payload transmission.

- When a UE selects frequency hopping or slot repetition, its msgA payload will be mapped to multiple PRUs.

- The bandwidth of msgA POs should be bounded within the initial UL BWP (RRC IDLE or INACTIVE) or active UL BWP (RRC CONNECTED).

- The combinations of SCS for PRACH and PUSCH supported in NR Rel-15 should also be supported in msgA preamble and payload transmission, as shown in Table 6.

Table 6: Combinations of SCS Supported for msgA Transmission

|  | SCS for msgA Preamble | SCS for msgA Payload | , allocation expressed in number of RBs for PUSCH |
| --- | --- | --- | --- |
| 839 | 1.25 | 15 | 6 |
| 839 | 1.25 | 30 | 3 |
| 839 | 1.25 | 60 | 2 |
| 839 | 5 | 15 | 24 |
| 839 | 5 | 30 | 12 |
| 839 | 5 | 60 | 6 |
| 139 | 15 | 15 | 12 |
| 139 | 15 | 30 | 6 |
| 139 | 15 | 60 | 3 |
| 139 | 30 | 15 | 24 |
| 139 | 30 | 30 | 12 |
| 139 | 30 | 60 | 6 |
| 139 | 60 | 60 | 12 |
| 139 | 60 | 120 | 6 |
| 139 | 120 | 60 | 24 |
| 139 | 120 | 120 | 12 |

To summarize, we have the following observation and proposal for msgA preamble transmission:

Observation 2:  A valid RO for msgA preamble transmission should have a sufficient transmission gap with respect to the UL transmission or DL reception before msgA. The TX gap configurations for msgA preamble can follow the rules specified in NR Rel-15 for PRACH.

Proposal 3:  The SCS combinations specified in NR Rel-15 for PRACH and PUSCH should be supported in msgA transmission.

Proposal 4:  When separate ROs are configured for two-step RACH and four-step RACH, they can be configured with different PRACH configuration index and different FDM patterns.

Proposal 5:  RO validation for msgA preamble should consider at least the time gap required before msgA preamble transmission and the causality rule between SSB/SIB reception and msgA preamble transmission.

### Payload Transmission of msgA

Figure 4:  Block Diagram for msgA Transmission

Figure 4 shows a block diagram for msgA transmission. The NR Rel-15 processing mechanism can be re-used for msgA preamble and payload, with extension in scrambling ID (sets) generation for PUSCH and DMRS. Such extensions aim to meet the new design requirements for two-step RACH, and to minimize the impacts on specifications and implementations.

Bit Level Scrambling for msgA PUSCH

A general formula for the scrambling ID generation of msgA PUSCH can be given by

cinit=nRNTI×215+NIDcell

which includes a cell ID (NIDcell)  and a new RNTI (nRNTI).

- The inclusion of cell ID is necessary to mitigate the inter-cell interference.

- The introduction of a new RNTI is necessary because of the necessity to support msgA transmission in any RRC state. Different from msg3 transmission in four-step RACH, neither UL grant nor TC-RNTI will be assigned for a two-step RACH UE. Therefore, a modified RNTI can be used to initialize the bit level scrambling sequence of msgA PUSCH in any RRC state, which can be defined as a function of RA-RNTI and resource index of preamble/DMRS, i.e.

nRNTI≜RA-RNTI+K1×p_id+K2×r_id16 ,

where p_id and r_id denote the resource index of preamble and DMRS, respectively; K1 and K2 are integers common to all two-step RACH UEs, and 16 represents the operation of truncating the inner variable to 16 bits.

Preamble to PRU Mapping

- Due to the bursty traffic pattern and the nature of CBRA, it is inefficient to dedicate one PUSCH occasion (PO) to a single UE. To improve the spectral efficiency of two-step RACH, one PO can be shared by UEs with similar payload size, MCS and waveform. To accommodate different payload size, MCS and waveform, multiple POs can be configured within a PO configuration period, which can be configured with different TBS/MCS/waveform.

- When multiple ROs and POs are configured for a msgA occasion, the preamble and DMRS resources from multiple ROs and POs can be aggregated into larger pools. The RO and PO can be ordered and indexed. The preamble and DMRS resources can also be ordered and indexed.

- To support frequency hopping and slot repetition, UE can select multiple PRUs to transmit its msgA payload. Therefore, one msgA preamble can be mapped to multiple PRUs, based on pre-configured patterns for frequency hopping and slot repetition.

MCS/TBS Indication

- For a given mapping relationship between preamble and PRU, the MCS/TBS of PRU can be indicated by the partitioning of preamble resources.

- As an option, the MCS/TBS for msgA PUSCH can also be indicated by UCI, which is multiplexed on msgA PUSCH, by re-using the UCI piggyback mechanism specified in NR Rel-15.

DMRS Resource Configuration

- DMRS signal generation can be made as a function of the preamble sequence index.

- The formulas for scrambling ID generation in NR Rel-15 can be extended, by incorporating the index of msgA preamble, the slot/symbol index and the cell ID. For example,

  - When transform precoding for PUSCH is not enabled, the pseudo-random sequence generator for DMRS sequence can be generated by:

cinit=217Nsymbslotns,fμ+l+12NIDnSCID+1+2NIDnSCID+nSCIDmod231

where nSCID=preamble ID, and NIDnSCID=NIDcell+K×nSCID, and K is a constant scalar.

- When transform precoding for PUSCH is enabled, a group of base sequences can be configured for a cell, wherein the configuration of base sequences can be based on the cell ID and preamble ID.

To summarize, we have the following observation and proposals for msgA payload transmission:

Observation 3: For msgA transmission, the following designs should be supported:

- a minimum payload size of 7 or 9 bytes can be supported by DFT-s-OFDM waveform and low MCS; payload sizes on the order of 1000 bits can be supported by higher MCS;

- multiple POs can be configured per PO configuration period, to accommodate different MCS, TBS and waveform;

- multiple two-step RACH UEs can share the same PO, if their msgA transmissions use similar payload size/MCS/waveform;

- resource allocation for PO can be specified relative to RO, by semi-statically or dynamically configured offsets in time and/or frequency domain;

- the mapping rules between msgA preamble sequence and msgA PRU can be indicated by SI or RRC signalling.

Proposal 6:  From the perspective of UE, msgA PO validation should consider at least the following criteria:

- transmission gap between msgA preamble and msgA payload

- RSRP measurements and SSB-to-msgA association pattern

- msgA payload size and maximum TBS supported by PO

- UE capabilities

Proposal 7: The mapping rule between msgA preamble and msgA PRU is configurable. One-to-one, one-to-many and many-to-one mappings can be supported for different use cases.

Proposal 8:  The scrambling ID for msgA PUSCH can be formulated as a linear and weighted combination of cell ID, preamble resource index and DMRS resource index.

Proposal 9: To reduce the DMRS collision probability for CBRA, the pool size of DMRS resources should be increased by extending the scrambling ID sets. The scrambling ID for DMRS sequence generation/configuration should include the cell ID and the preamble ID.

### LLS Results for msgA

Figure 5 shows the BLER performance of msgA payload decoding, for small payload size of 72 bits and TDL-C (300ns) channel. 1 TX & 4 RX antennas configuration is considered, and realistic channel estimation (chest) is performed. Various number of PRBs and UEs are evaluated.

The details of the LLS assumptions can be found in the Appendix.

#### 2.2.3.1	Resource Allocation and UE Multiplexing for Small Payload Size

Figure 5: msgA BLER performance for TBS=72 bits with 1 or 2 UEs

As shown by Figure 5, for small payload size of 72 bits, both single UE and 2 UE cases show similar BLER performances. This example indicates that when the number of RX antennas are greater than the number of UEs, gNB can successfully decode multiple UEs sharing the same PO, even though their PRU multiplexing are non-orthogonal in time/frequency/code domains. Based on the MCL calculation for a target BLER of 10%, 3 RB provides the best performance for 72-bit payload.

#### 2.2.3.2	DFT-s-OFDM Waveform

In Figure 6 and Figure 7, the BLER performance for DFT-s-OFDM waveform is shown for small and large payload size of msgA, respectively. The BLER performances of DFT-s-OFDM waveform are similar to those of CP-OFDM waveform, but the PAPR performance of DFT-s-OFDM waveform is much better. Therefore, DFT-s-OFDM waveform should be supported for two-step RACH UE experiencing large pathloss or in RRC_IDLE/RRC_INACTIVE state.

Figure 6: msgA BLER performance for TBS=72 bits for CP-OFDM and DFT-s-OFDM

Figure 7: msgA BLER performance for TBS=1000 bits for CP-OFDM and DFT-s-OFDM

#### 2.2.3.3	Non-orthogonal DMRS Multiplexing

Figures 8-10 show the LLS performance comparison between orthogonal DMRS resource configuration and non-orthogonal DMRS resource configuration for different payload size of msgA and different waveform.

- For orthogonal DMRS resource configurations, two UEs use different DMRS ports and share the same DMRS scrambling ID.

- For non-orthogonal DMRS resource configuration, two UEs are assigned to the same DMRS port but are configured with different DMRS scrambling IDs.

As shown by Figures 8-10, the non-orthogonal DMRS resource configuration shows negligible performance degradation compared to orthogonal DMRS resource configuration, especially for larger PO bandwidth.

Figure 8: msgA BLER Performance for TBS 72 bits with Orthogonal and Non-Orthogonal DMRS for CP-OFDM

Figure 9: msgA BLER Performance for TBS 1000 bits with Orthogonal and Non-Orthogonal DMRS for CP-OFDM

Figure 10: msgA BLER Performance for TBS 72 bits with Orthogonal and Non-Orthogonal DMRS for DFT-s-OFDM

#### 2.2.3.4	Resource Allocation, MCS and UE Multiplexing for Large Payload Size

Figures 11 show the BLER performance for msgA with large payload size (1000 bits). MCL at 10% BLER is 131.77 dB for 6 RB allocation and 133.26 dB for 12 RB allocation, respectively. Therefore, 12 RB allocation provides a better MCL performance for a 1000-bit payload size of msgA. In addition, the link level performance degradation due to two UE multiplexing is negligible. This indicates non-orthogonal UE multiplexing is also feasible for large payload size.

On the other hand, Figure 12 studies the impacts of different MCS for the same PO bandwidth and the same payload size. It shows that 16 QAM modulation and lower code rate does not provide significant gain in link level performance.

Figure 11: msgA BLER performance for TBS=1000 bits with 1 or 2 UEs

Figure 12: msgA BLER performance for TBS=1000bits with 16 QAM modulation

#### 2.2.3.4	Timing Offset

Figures 13-16 show the LLS performances for different timing offsets between two UEs.

- For small timing offset (less than CP) between two UEs, a common FFT can be shared between two UEs at the receiver. The same receiver for the synchronized case can be reused.

- For large timing offset (much larger than CP) between two UEs, two UEs uses their own FFTs and are separately demodulated at the receiver. Estimated channel can be exchanged for reducing interference and spatial multiplexing.

As shown by Figures 13 and 14, the link level performance degradation due to large timing offset (a half of the OFDM symbol length) is negligible for small payload size. Figure 15 and 16 show less than 1.5 dB performance degradation for large payload size with large timing offset.

Figure 13: msgA BLER performance for TBS=72 bits with Timing Offset for 2 UE and CP-OFDM

Figure 14: msgA BLER performance for TBS=72 bits with Timing Offset for 2 UE and DFT-s-OFDM

Figure 15: msgA BLER performance for TBS=1000 bits with Timing Offset for 2 UE and CP-OFDM

Figure 16: msgA BLER performance for TBS=72 bits with Timing Offset for 2 UE and DFT-s-OFDM

Therefore, we have the following observation:

Observation 4   LLS results for msgA decoding with realistic channel estimation indicate that:

- MCL at a given BLER target can be used as the performance metric for PO size optimization;

- PO size optimization depends on the payload size and slot format;

  - PO size of 3 RB and 1 slot (14 symbols, DMRS overhead 1/7) is good choice for 72 bits;

  - PO size of 12 RB and 1 slot (14 symbols, DMRS overhead 1/7) is a good choice for 1000 bits.

- DFT-s-OFDM waveform should be supported for small payload transmission, especially for UE experiencing large pathloss or in RRC_IDLE/RRC_INACTIVE state;

- For both small and large payload size, non-orthogonal UE multiplexing in both DMRS and PUSCH does not cause performance degradation, if the number of RX antennas is greater than the number of UEs;

- Higher order modulation does not show performance gain for large payload size.

Based on the observation, we have the following proposal for msgA PUSCH resource allocation, MCS and waveform configuration:

Proposal 10: For msgA payload transmission, the following configurations should be supported:

- Low-order linear modulations such as QPSK and pi/2 BPSK should be specified for msgA PUSCH transmission. In particular,

  - QPSK should be specified for CP-OFDM waveform;

  - pi/2 BPSK and QPSK should be specified for DFT-s-OFDM waveform.

- A finite set of PO size can be specified, to support different payload size (in the range of a few bytes to a few hundred bytes) and different coverage requirements;

  - PO bandwidth can be configured proportional to that of RO bandwidth;

  - In FR1, PO bandwidth of {3, 6, 12} PRBs can be considered for a 14-symbol slot;

  - Slot repetition should be supported.

  - Frequency hopping should be supported.

## 2.3	Channel Structure for msgB

After the transmission of msgA, the two-step RACH UE will monitor msgB within a pre-configured RAR window. The RAR window configuration information is broadcast in SI. A general channel structure for msgB is shown in Figure 17, wherein the PDCCH carries the resource assignment for PDSCH, and PDSCH carries the RAR messages in response to the latest msgA transmissions/re-transmissions.

Figure 17:  General Channel Structure of msgB

### 2.3.1	Contents of msgB

In msgA transmission, multiple UEs can be multiplexed on the same RO/PO. Due to the contention-based transmission and channel impairments, not all msgA preamble/payload can be successfully detected by gNB. As a result, gNB will send a msgB in response to the two-step RACH UEs, if their msgA payloads have been decoded, or only their msgA preamble have been detected. In particular, the contents of msgB depend on the processing outcome of msgA as well as the RRC states of UE.

Therefore, we have the following proposals:

Proposal 11: Depending on the processing outcome of msgA and the RRC state of UE, gNB will send different responses in msgB.

- If the msgA preamble is detected but the msgA payload is not decoded, gNB will send a FallbackRAR, which includes at least the RAPID, the TA command, the TC-RNTI and the UL grant for msgA payload re-transmission.

- If the msgA payload is decoded, gNB will send a SuccessRAR, which includes at least the contention resolution information and the TA command. For RRC IDLE or INACTIVE UE, a C-RNTI should also be included in the SuccessRAR.

Proposal 12:  For two-step RACH UEs sharing the same RO, the FallbackRAR or SuccessRAR of RRC IDLE or INACTIVE UE can be aggregated into a MAC PDU. FallbackRAR of RRC CONNECTED UE can also be aggregated into this MAC PDU.

Proposal 13:  Similar to msg2 transmission in four-step RACH, msgB PDCCH and PDSCH can be multicast to a group of UEs sharing the same RO, and the CRC of msgB PDCCH can be masked by a group RNTI.

### 2.3.2	msgB Group RNTI

In four-step RACH, the CRC of msg2 PDCCH is scrambled by a 16-bit RA-RNTI. Based on NR Rel-15 design, the RA-RNTI has max decimal value of 17920, whose binary expansion occupies 15 out of the 16 bits allocation. On the other hand, when the same RO is shared by two-step RACH UE and four-step RACH UE, both msg2 and msgB will be transmitted by gNB. To avoid the ambiguity and reduce the complexity of PDCCH decoding, it is necessary to consider modified RA-RNTI design to differentiate msg2 of four-step RACH from msgB of two-step RACH. A low complexity solution is to modify the RA-RNTI calculation formula for two-step RACH msgB transmission, by considering the un-used space in the 16-bit allocation.  Therefore, we have the following proposal:

Proposal 14:  Group RNTI for msgB should be specified for CRC masking of msgB PDCCH. To reduce the impacts on specification, the RA-RNTI formula in NR Rel-15 can be extended to support msgB group RNTI mapping into the un-used space of the 16-bit allocation. The impacts of RAR window extension should also be considered in the calculation of msgB group RNTI.

# 3.	Conclusions

In this contribution, msgA and msgB channel structure for two-step RACH has been discussed. Based on the performance evaluation and analysis, we have the following observations and proposals:

Observation 1:  A msgA transmission occasion comprises a PRACH occasion (RO) and a set of PUSCH occasions (PO). To transmit a msgA preamble, a two-step RACH UE selects a valid RO and a preamble sequence configured on the valid RO. To transmit a msgA payload, a two-step RACH UE selects a set of valid POs and one or multiple PRUs configured on the valid POs.

Observation 2:  A valid RO for msgA preamble transmission should have a sufficient transmission gap with respect to the UL transmission or DL reception before msgA. The TX gap configurations for msgA preamble can follow the rules specified in NR Rel-15 for PRACH.

Observation 3: For msgA transmission, the following designs should be supported:

- a minimum payload size of 7 or 9 bytes can be supported by DFT-s-OFDM waveform and low MCS; payload sizes on the order of 1000 bits can be supported by higher MCS;

- multiple POs can be configured per PO configuration period, to accommodate different MCS, TBS and waveform;

- multiple two-step RACH UEs can share the same PO, if their msgA transmissions use similar payload size/MCS/waveform;

- resource allocation for PO can be specified relative to RO, by semi-statically or dynamically configured offsets in time and/or frequency domain;

- the mapping rules between msgA preamble sequence and msgA PRU can be indicated by SI or RRC signalling.

Observation 4   LLS results for msgA decoding with realistic channel estimation indicate that:

- MCL at a given BLER target can be used as the performance metric for PO size optimization;

- PO size optimization depends on the payload size and slot format;

  - PO size of 3 RB and 1 slot (14 symbols, DMRS overhead 1/7) is good choice for 72 bits;

  - PO size of 12 RB and 1 slot (14 symbols, DMRS overhead 1/7) is a good choice for 1000 bits.

- DFT-s-OFDM waveform should be supported for small payload transmission, especially for UE experiencing large pathloss or in RRC_IDLE/RRC_INACTIVE state;

- For both small and large payload size, non-orthogonal UE multiplexing in both DMRS and PUSCH does not cause performance degradation, if the number of RX antennas is greater than the number of UEs;

- Higher order modulation does not show performance gain for large payload size.

Proposal 1: The association between SSB and msgA occasion (SSB-to-msgA association) can be defined similar to SSB-to-RO association. The association pattern between SSB and msgA occasion can be indicated in SI or RRC.

Proposal 2: The bandwidth used for msgA preamble and payload transmission can be similar or different. Within an association period of SSB-to-msgA, the total bandwidth configured for msgA occasions is within initial UL BWP (RRC IDLE or INACTIVE) or active UL BWP (RRC CONNECTED).

Proposal 3:  The SCS combinations specified in NR Rel-15 for PRACH and PUSCH should be supported in msgA transmission.

Proposal 4:  When separate ROs are configured for two-step RACH and four-step RACH, they can be configured with different PRACH configuration index and different FDM patterns.

Proposal 5:  RO validation for msgA preamble should consider at least the time gap required before msgA preamble transmission and the causality rule between SSB/SIB reception and msgA preamble transmission.

Proposal 6:  From the perspective of UE, msgA PO validation should consider at least the following criteria:

- transmission gap between msgA preamble and msgA payload

- RSRP measurements and SSB-to-msgA association pattern

- msgA payload size and maximum TBS supported by PO

- UE capabilities

Proposal 7: The mapping rule between msgA preamble and msgA PRU is configurable. One-to-one, one-to-many and many-to-one mappings can be supported for different use cases.

Proposal 8:  The scrambling ID for msgA PUSCH can be formulated as a linear and weighted combination of cell ID, preamble resource index and DMRS resource index.

Proposal 9: To reduce the DMRS collision probability for CBRA, the pool size of DMRS resources should be increased by extending the scrambling ID sets. The scrambling ID for DMRS sequence generation/configuration should include the cell ID and the preamble ID.

Based on the observation, we have the following proposal for msgA PUSCH resource allocation, MCS and waveform configuration:

Proposal 10: For msgA payload transmission, the following configurations should be supported:

- Low-order linear modulations such as QPSK and pi/2 BPSK should be specified for msgA PUSCH transmission. In particular,

  - QPSK should be specified for CP-OFDM waveform;

  - pi/2 BPSK and QPSK should be specified for DFT-s-OFDM waveform.

- A finite set of PO size can be specified, to support different payload size (in the range of a few bytes to a few hundred bytes) and different coverage requirements;

  - PO bandwidth can be configured proportional to that of RO bandwidth;

  - In FR1, PO bandwidth of {3, 6, 12} PRBs can be considered for a 14-symbol slot;

  - Slot repetition should be supported.

  - Frequency hopping should be supported.

Proposal 11: Depending on the processing outcome of msgA and the RRC state of UE, gNB will send different responses in msgB.

- If the msgA preamble is detected but the msgA payload is not decoded, gNB will send a FallbackRAR, which includes at least the RAPID, the TA command, the TC-RNTI and the UL grant for msgA payload re-transmission.

- If the msgA payload is decoded, gNB will send a SuccessRAR, which includes at least the contention resolution information and the TA command. For RRC IDLE or INACTIVE UE, a C-RNTI should also be included in the SuccessRAR.

Proposal 12:  For two-step RACH UEs sharing the same RO, the FallbackRAR or SuccessRAR of RRC IDLE or INACTIVE UE can be aggregated into a MAC PDU. FallbackRAR of RRC CONNECTED UE can also be aggregated into this MAC PDU.

Proposal 13:  Similar to msg2 transmission in four-step RACH, msgB PDCCH and PDSCH can be multicast to a group of UEs sharing the same RO, and the CRC of msgB PDCCH can be masked by a group RNTI.

Proposal 14:  Group RNTI for msgB should be specified for CRC masking of msgB PDCCH. To reduce the impacts on specification, the RA-RNTI formula in NR Rel-15 can be extended to support msgB group RNTI mapping into the un-used space of the 16-bit allocation. The impacts of RAR window extension should also be considered in the calculation of msgB group RNTI.

# References

[1] RP-182894, “New Work Item: Two-Step RACH for NR,” ZTE, 3GPP TSG RAN #82, Sorrento, Italy.

[2] RP-191448, “SR of NR Two-Step RACH,” ZTE, 3GPP TSG RAN #84, Newport Beach, USA.

[3]	3GPP TSG RAN WG1 Meeting #96, “RAN1 Chairman’s Notes,” Athens, Greece.

[4] 3GPP TSG RAN WG1 Meeting #96B, “RAN1 Chairman’s Notes,” Xi’an, China.

[5] 3GPP TSG RAN WG1 Meeting #97, “RAN1 Chairman’s Notes,” Reno, USA.

[6] R1-1909240, “Procedures Related to Two-Step RACH,” Qualcomm, 3GPP TSG RAN1 #98, Prague, Czech Republic.

[7] R1-1909241, “Other Considerations for Two-Step RACH,” Qualcomm, 3GPP TSG RAN1 #98, Prague, Czech Republic.

# Appendix A: Link Level Simulation Assumptions

The following table summarizes the LLS assumptions used in this paper:

| Parameters | Values |
| --- | --- |
| Waveform | CP-OFDM and DFT-s-OFDM |
| Subcarrier spacing for PUSCH | 15kHz at 700MHz |
| Number of symbols in PUSCH slot | 14 symbols |
| DMRS overhead for PUSCH | 2 DMRS symbols |
| TBS | 72, 1000 bits |
| Number of PRBs | 1,2,3,4,6,12 |
| Number of UEs | 1, 2 UEs |
| UE antenna configuration | 1Tx |
| gNB antenna configuration | 4Rx |
| Propagation channel & UE velocity | TDL-C 300ns, 3km/h |
| Receiver | MMSE-IRC |
| Channel Estimation | Ideal and Realistic |
| Target BLER | [10%, 1%] |