3GPP TSG RAN WG1 #98                                                       R1-1909293

Prague, CZ, August 26th – 30th, 2019

Source:	Xiaomi Communications

Title:                  On Synchronization for 5G V2X

Agenda Item:	7.2.4.3

Document for:	Discussion and Decision

Introduction

In RAN1#97 meeting, the following agreements on synchronization for 5G V2X is made [1]:

Agreements:

- In NR V2X, from transmitter perspective, the period (P1 in unit of ms) of S-SSB(s) transmission is the same for all SCS, for further down-selection:

  - Alt 1: the number of S-SSB(s) transmitted within P1is (pre-)configurable.

  - Alt 2: the number of S-SSB(s) transmitted is fixed within P1 per SCS.

  - Alt 3: only one S-SSB for all SCS is transmitted within P1.

Agreements:

At least for evaluation, one S-SSB transmission with at least the following periodicity:

- 160ms period at least for 15kHz SCS.

- FFS other value(s)

In this contribution, several aspects on NR V2x sidelink synchronization are discussed.

Discussion

Synchronization source and its priority

Similar as LTE V2x, following synchronization sources can be considered for NR V2x communications:

- GNSS

- NodeB downlink synchronization signals

- V2x sidelink synchronization signals

- The own timing of V2x UE

Introducing V2x sidelink synchronization channel for sidelink communication is necessary when UEs are out of coverage and with weak GNSS signal, especially in underground parking. A simplified NR based design of synchronization signal and sidelink broadcast channel can be considered for the NR sidelink synchronization. In addition, priority of the above synchronization source should be reconsidered based on the rules in LTE discussion.

On the other hand, multiple synchronization sources should have a clear priority definition at least for single-carrier operation. In RAN1#96 meeting, the priority table was agreed as a working assumption, which we support to be an agreement from our point of view. Furthermore, the one with the highest priority available should be used for synchronization. Similar as LTE, the synchronization signal strength can be used to select the synchronization source if the priority of sources is the same. However, the measurement of synchronization signal strength shall be clarified.

Proposal 1: Synchronization signal strength is used to select the synchronization source among resources with the same priority.

NR V2x synchronization periodicity

In RAN1 #94bis, periodic transmission of S-SSB in NR V2X is supported.  In NR Uu design, UE assumes 20ms synchronization SSB periodicity for initial access, and could be updated a periodicity within {5, 10, 20, 40, 80, 160}ms by higher layer configuration with unit of half radio frames. The periodicity of SLSS is enlarged to 200ms in LTE V2x for overhead consideration after improving synchronization performance by GNSS. Generally speaking, a longer synchronization periodicity could be beneficial for overhead efficiency, however, short periodicity is beneficial to guarantee the sidelink synchronization performance. When the UE are with good synchronization quality e.g. have strong GNSS signal or be close to the base station, the UE can transmit SLSS with a longer periodicity.

A default synchronization periodicity which is longer could be defined for the case with good synchronization quality, and a shorter periodicity could be configured by the base station or synchronization source vehicle in case the synchronization quality is worse.

However, in some underground scenario, shorter period of synchronization signal could be beneficial for V2x application requirements, such as parking and accident avoiding. Without GNSS and NodeB coverage, V2x UE may adjust the periodicity and do SSB repetition, so flexible configured S-SSB transmission in one burst set should be supported.

Proposal 2: A longer default periodicity such as 160ms with configurable periodicity according to synchronization signal coverage could be defined.

Synchronization signal and PSBCH block

Basically, NR V2x sidelink synchronization design should reuse NR Uu design as much as possible to reduce the standardization complexity. Although some timing and frequency design principle of S-SSB were agreed in previous meetings, further details still need study.

As discussed in RAN1 #97, three alternatives are proposed for further down selection. The flexibility of alt3 may be an issue since the beam-based S-SSB transmission may be need for coverage enhancement in higher frequency band of V2x. Comparing Alt 1 and Alt 2, Alt1 is preferred in V2X for more flexibility and forward compatible consideration.

Proposal 3: Alternative 1 should be supported for the S-SSB transmission.

In NR Uu design, the S-SSB repetition is not supported. However, in V2X sidelink, it is a different case, there are multiple S-SSB transmission sources, so the S-SSB repetition shall be supported in V2X.

In frequency domain, the definition on DR-MS of PBCH can be the same as NR and the S-PSS/S-SSS should start from the third subcarrier of the lowest PRB within the 11 PRB of S-SSB as well.

PSBCH content

Similar as LTE V2x, SFN and in-coverage indicator can be included in NR PSBCH. In addition, slot format indicator (SFI) can be included to replace TDD configuration field. In order to support the beam operation in NR, the Sidelink SSB timing index and actual transmitted SSB information should also be included. It needs to be further investigated whether resource pool information can be included in NR PSBCH.

Since S-SSB has no synchronization raster definition, the Kssb is no longer needed in PSBCH.

Besides PBCH, NR RMSI carries some SSB related information such as position, periodicity and power, however, in NR, RMSI may not needed, so some information should be considered in PSBCH, within the payload agreed in RAN1#86bis

Reusing LTE V2x synchronization

NR V2x aims to support advanced V2x services, and the basic V2x services can still be served by LTE V2x. For a UE supporting both NR V2x and LTE V2x, it can communicate with NR V2x and LTE V2x simultaneously for different V2x applications. In such a case, it may be redundant to maintain two independent synchronization mechanism for both NR V2x and LTE V2x. It would be beneficial at least on coexistence if NR V2x communication can reuse LTE V2x synchronization sources as the NR V2x synchronization source, including LTE eNB DL synchronization signals, and LTE V2x sidelink synchronization signal.

Proposal 4: NR V2x communication can select LTE V2x synchronization source as the NR V2x synchronization source.

Synchronization procedure enhancement

In the partial coverage use case where some UEs are in the coverage while other UEs are out of coverage, the principle for the UEs in coverage to transmit S-SSB should be specified. Basically, if all UEs in coverage transmit SSB, unnecessary interference will be introduced and it may not be beneficial for the sidelink synchronization. Thus, the scheme for which and how a UE can become the synchronization reference should be specified. For example, a UE in the center of gNB coverage may not be triggered for S-SSB transmitting.

Proposal 5: S-SSB transmitting triggering scheme should be specified.

Conclusion

In this contribution, the synchronization issue of NR V2x is discussed. Based on the discussion, the following proposals are proposed:

Proposal 1: Synchronization signal strength is used to select the synchronization source among resources with the same priority.

Proposal 2: A longer default periodicity such as 160ms with configurable periodicity according to synchronization signal coverage could be defined.

Proposal 3: Alternative 1 should be supported for the S-SSB transmission.

Proposal 4: NR V2x communication can select LTE V2x synchronization source as the NR V2x synchronization source.

Proposal 5: S-SSB transmitting triggering scheme should be specified.

References

[1] Chairman's Notes RAN1#96bis final

[2] RP-181429, New SID: Study on NR V2X, RAN#80, Jun. 2018