3GPP TSG-RAN WG1 Meeting #98	R1-1909303

Prague, Czech Republic, 26th – 30th August 2019

Agenda Item:	7.2.2.3

Source:	Ericsson

Title:	Evaluation results for enhanced PUCCH and PRACH designs

Document for:	Discussion, Decision

# 1	Introduction

In this paper details regarding PUCCH and PRACH designs are presented.

# 2	Enhanced PUCCH Design

## 2.1		Introduction

In this section, we investigate various details of candidate enhanced PUCCH designs. This contribution provides a more detailed description of the proposed enhanced PUCCH formats mentioned in [1].

## 2.2	Candidates for Enhanced PUCCH Designs

For all enhanced PUCCH designs considered here, the interlace structure in Figure 18 in the appendix for the case of a 20 MHz channel with 30 kHz subcarrier spacing is used. This is the same interlace structure for 30 kHz agreed in RAN1 AH 1901:

Agreement:

For interlace transmission of at least PUSCH and PUCCH, the following PRB-based interlace design is supported for the case of 20 MHz carrier bandwidth:

a.	15 kHz SCS: M = 10 interlaces with N = 10 or 11 PRBs / interlace

b.	30 kHz SCS: M = 5 interlaces with N = 10 or 11 PRBs / interlace

Note: PRACH design to be considered separately, including multiplexing aspects with PUSCH and PUCCH

5 interlaces with 10 PRBs each are defined, and a PUCCH resource occupies one of the interlaces.

### 2.2.1	Interlaced PUCCH Format 0

In RAN1#97, the following agreement was made regarding interlaced PUCCH Format 0 and 1:

Agreement:

Support enhancement of Rel-15 PUCCH formats PF0 and PF1 as follows:

- Mapping to physical resources of one full interlace in 20 MHz.

  - FFS: Sequence type and mapping considering the following alternatives:

    - Alt-1: Repetition of the length-12 Rel-15 PF0 and PF1 sequence in each PRB of an interlace with mechanism to control PAPR/CM considering the following alternatives

      - Alt-1a: Cycling of cyclic shifts across PRBs

      - Alt-1b: Phase rotation across PRBs of an interlace where the phase rotation is can be per RE or per PRB

    - Alt-2: Mapping of different length-12 Rel-15 PF0 and PF1 sequences to the PRBs of an interlace based on different group number u (range is 0 .. 29)

    - Alt-3: Mapping of a single long sequence to the PRBs of an interlace

  - FFS: Impact due to guardbands

- Note: Decisions on the above should be based on at least performance using the agreed MCL metric and specification impact

- Note: Interlaced PF2 and 3 are not enhanced to support 1-2 bit payloads

NR Rel-15 PUCCH format 0 conveys its information using different cyclic shift of a base sequence. For HARQ-ACK feedback, a one-bit payload is created by selecting between two different cyclic shifts and a two-bit payload is created by selecting between four different cyclic shifts. For SR, only one cyclic shift is used in the case of a “positive SR.” For a “negative SR,” i.e., no scheduling request, the UE transmits nothing on the SR resource. In all cases, there are no UCI data symbols as for other PUCCH formats; instead the cyclically shifted base sequence indicates which payload that was transmitted. All possible sequences are pre-defined. In Rel-15, PF0 occupies only 1 PRB. Multiplexing is achieved by allocating different sets of cyclic shifts to different users.

Extending NR Rel-15 PUCCH format 0 to an interlaced form can be done simply by defining longer base sequences. A straight forward way to define longer sequences is to repeat the already defined length-12 CGS sequences in each of the PRBs of the interlace. This is Alt-1 in the above agreement. If only repetition is used, the PAPR/CM will degrade. To eliminate this problem, a simple approach of cycling through a set of the 12 available cyclic shifts on each PRB of the interlace can be used. This breaks up the repetitiveness of the signal and lowers the PAPR/CM. Cyclic shift cycling can start in the first PRB of the interlace with the configuration of an initial cyclic shift as in Rel-15, and then increase the cyclic shift index by one for each PRB, wrapping around after reaching the maximum cyclic shift index (Alt-1a in the above agreement). Since different multiplexed users are configured with different initial cyclic shifts the cycling will maintain orthogonality between users.

Longer sequences can also be defined by using a 10 PRBs long (120 subcarriers) Zadoff-Chu sequence as is used for DMRS in PUCCH Format 3. This is Alt-3 in the above agreement. In Section 2.3 it is shown that the performance for repetition with cyclic shift cycling and long Zadoff-Chu is equal in terms of MCL for a 23 dBm UE. However, the CM of the signal when using repetition with cyclic shift cycling is lower and hence a better choice, considering lower power class UEs that may be introduced in the future.

An additional way of defining longer sequences is by reusing the length-12 CGS sequences and selecting a different sequence for each different PRB corresponding to a different group number u. This is Alt-2 in the above agreement. In Section 2.3 it is shown that the CM for this solution is much worse than the cyclic shift cycling approach. Such a large degradation in CM would result in a substantial degradation of the MCL.

### 2.2.2	Interlaced PUCCH Format 1

NR Rel-15 PUCCH Format 1 is based on TDM between OFDM symbols occupied by a base sequence (used as DMRS) and OFDM symbols occupied by a modulated base sequence conveying the UCI data. The base sequence used in both the DMRS OFDM symbols and the UCI OFDM symbols is of the same type.

The UCI payload is conveyed by a single BPSK symbol (for 1 bit payload) or QPSK symbol (for 2 bit payload) which modulates the base sequence in UCI OFDM symbols. The OFDM symbols containing pure DMRS are used to get a reference phase, which is used to extract the information payload from the data symbols. Since the payload is low, the number of possible resulting modulated sequences are few, and since all possibilities are known in advance, PUCCH Format 1 can be regarded as a sequence-based format as is PUCCH Format 0.

In Rel-15, PF1 occupies 1 PRB. Multiplexing is achieved by allocating different cyclic shifts to different users. In addition, OCCs are applied in the time domain across UCI OFDM symbols and DMRS OFDM symbols separately. The length of the OCC (which determines the user multiplexing capacity) depends on the number of OFDM symbols. For example, for a length-14 PF1 resource, length-7 OCC is used.

Extending NR Rel-15 PUCCH Format 1 to an interlaced form can be done in the same way as for PF0 described above, i.e., repeating the already defined length-12 CGS base sequence in each PRB of the interlace, and then employing cyclic shift cycling on top to manage PAPR/CM (Alt-1). The configuration of the initial cyclic shift can be left the same as in Rel-15. This defines the starting cyclic shift of the cycling pattern. In this way, the orthogonality will be maintained between users. As for PF0, we also investigated the use of a length-120 Z-C sequence as an alternative (Alt-3), but found the cubic metric for the repeated length-12 CGS sequences with cyclic shift cycling to have lower cubic metric. Hence, we prefer the cyclic shift cycling sequence repetition approach (Alt-1a).

### 2.2.3	Enhanced PUCCH Format 2 (E-PF2) Design

The following is a candidate design for an enhanced PUCCH format 2 (E-PF2), based on NR rel-15 PUCCH format 2. It is enhanced in the sense that it uses an interlaced structure and supports multi-user multiplexing with Orthogonal Cover Codes (OCCs).

Since the interlaced structure will make the PUCCH consume more resources, user multiplexing is important to maintain overall capacity. Figure 1 shows an example for the case of 2 OFDM symbols. In this example, 8 users are multiplexed based on a combination of OCC4 in the frequency domain + OCC2 in the time domain. For both the reference symbols and the data symbols, OCC4 is used in the frequency domain and OCC2 in the time domain. Since there are only 4 reference symbols within an OFDM symbol E-PF2 cannot support more than OCC4 in the frequency domain. Each multiplexed user is assigned different OCCs, so that no two users use the same combination of OCCs in time and frequency. The DMRS used to evaluate this format are the same PN-sequences as used for NR Rel-15 PUCCH format 2, truncated to get the desired number of unique elements. For NR Rel-15 PUCCH format 2 the PN-sequence to use depends on the OFDM symbol number. If OCC is to be supported in the time domain for E-PF2 a modification to the way PN-sequences are initialized is required, i.e., the sequence initialization (cinit) would need to be held constant over two OFDM symbols

Figure 1: Candidate E-PF2 PUCCH design for the case of 2 OFDM symbols with OCC4 in the frequency domain and OCC2 in the time domain supporting multiplexing of 8 users.

In Figure 1, the number of coded bits carried by the PUCCH resource of a single user is 10 PRBs * 2 bits/symbol * 2 symbols/PRB = 40 bits. In the evaluation discussed in the next section, up to 25 bits payload is considered using the same Reed Muller block code and Polar code defined for NR PUCCH format 2.

One potential issue with the user multiplexing based on intra data symbol OCC’s is that the I/Q data symbols need to be repeated prior to the application of the OCC’s. In the worst case, if the all 1’s OCC codeword is assigned to a particular user, then each data symbol is repeated as many times as the length of the OCC codeword.

Furthermore, repetition occurs across PRBs in the PUCCH bandwidth due to the assignment of a fixed OCC code to a user that is reused in each PRB. This creates a degradation in the peak to average power ratio (PAPR) and cubic metric (CM) in the time domain transmitted signal. Without a mitigation mechanism, this would require a large amount of back-off of the UE power amplifier, lowering the efficiency and increasing size/cost.

A simple solution is to break up the repetition pattern by cycling the OCC codes across the frequency domain. For example, for the case of multiplexing 4 users, each user can use all 4 OCC codes in a manner that still preserves orthogonality between users. User 1 can apply the OCC codes in the order 1-2-3-4 in the frequency domain; User 2 in the order 2-3-4-1, User 3 in the order 3-4-1-2, and so on. As will be shown in the Section 2.3, this can dramatically reduce the PAPR/CM.

### 2.2.4	Enhanced PUCCH Format 3 (E-PF3) Design

The following is a candidate design for an enhanced PUCCH format 3 (E-PF3), based on NR rel-15 PUCCH format 3. It is enhanced in the sense that it uses an interlaced structure and support multi-user multiplexing with Orthogonal Cover Codes (OCCs).

Since the interlaced structure will make the PUCCH consume more resources, user multiplexing is important to maintain overall capacity. Figure 2 illustrate the usage of intra data symbol OCC by showing the impact on data symbols. All data in an OFDM symbol is repeated block-wise and the OCC is spanning the entire interlace. This is discussed further in Section 2.2.5. Two different levels of user multiplexing are shown: 4 and 6 users. Each multiplexed user is assigned a different OCC. Like for NR PUCCH format 3, a Zadoff-Chu sequence is used for the DMRS symbols and the sequence is mapped to all available subcarriers of the used interlace. The multiplexed users are each assigned the same base sequence but are assigned different cyclic shifts of that base sequence.

In Figure 2(a), the number of coded bits carried by the PUCCH resource of a single user is 10 PRBs * 12 REs * 2 bits/symbol / 4 users = 60 bits. In Figure 2(b) the number of coded bits is reduced to 40 since more users are multiplexed. The coding rate in each case determines the PUCCH payload. In the evaluation discussed in the next section, up to 25 bits payload is considered using the same Reed Muller block code and Polar code defined for NR PUCCH format 3.

In this example, the intention of considering up to OCC6 is that when comparing to legacy NR PUCCH with only 1 PRB, multiplexing of 6 users can make up for much of the loss due to interlacing due to the use of 10 PRBs. Due to varying channel conditions it is important to be able to configure a flexible level of user multiplexing which translates into a flexible length OCC code (1 – 6). In this way, the level of used user-multiplexing can be balanced with respect to the different user payloads and the radio environment in which the system is operated.

(a)	(b)

Figure 2: Candidate E-PF3 PUCCH designs, illustrating intra data symbol OCC impact on data symbols, supporting multiplexing of (a) 4 users based on OCC4 in the frequency domain, and (b) 6 users based on OCC6 in the frequency domain.

Exactly the same design as Rel-15 is assumed in terms of DMRS positions and the use of DFT-s-OFDM, as agreed in RAN1 #97:

Agreement:

For enhanced Rel-15 PF3 supporting interlaced mapping, do not replace DFT-s-OFDM with CP-OFDM

### 2.2.5	OCC Repetition Mapping

Simulations have shown that the performance and PAPR/CM are affected by how the I/Q data symbol repetition used for intra data OCC is mapped over the subcarriers. Three mappings have been evaluated, symbol repetition within a PRB, block repetition within a PRB and block repetition spanning the entire interlace. Block repetition spanning the entire interlace is a natural extension of the way OCC is used in Rel-15 PUCCH Format 4, and similarly to PUCCH Format 4, this will make the different users occupy different combs [3], i.e. not only orthogonal in the code domain, but also orthogonal in frequency.

The block repetition spanning the entire interlace has been evaluated for E-PF3 with DFT-s-OFDM as in Rel-15 PF3. With DFT-s-OFDM, the OCC is applied before the DFT-spreading results in what in Rel-15 is known as “pre-DFT-OCC”. In symbol repetition each symbol is repeated the required number of times and mapped to consecutive subcarriers. In block repetition the entire block of symbols is repeated the required number of times, i.e. the repetitions of the same symbol will not be mapped consecutively. See Figure 3 for an illustration of the differences between symbol and block repetition within a PRB. Simulations show that for DFT-s-OFDM block repetition spanning the entire interlace is best while for CP-OFDM there is a slight advantage of symbol repetition within each PRB. As such, the individually best scheme is used in the remainder of the results shown below.

Figure 3: Illustration of symbol repetition vs block repetition for OCC within a PRB for 4 user OCC, i.e. 3 data symbols per PRB. For simplicity the figure only show how the OCC code is applied to the first data symbol.

## 2.3	Performance of Candidate PUCCH Designs

The candidate PUCCH designs, interlaced PF0, interlaced PF1, E-PF2 and E-PF3 discussed in the previous sections have been evaluated by means of simulation for various payloads in accordance with the evaluation assumptions agreed in RAN1#96b [2].

The simulation assumptions are summarized in Table 1, the reporting metrics are summarized in Table 2, and additional information is summarized in Table 3. The formats are in this evaluation mapped to the agreed PUCCH/PUSCH interlace structure shown in Figure 18 for the case of 30 kHz SCS and a 20 MHz carrier.

Table 1: Evaluation assumptions

| Property | Value |
| --- | --- |
| Carrier frequency | 5 GHz |
| Channel bandwidth | 20 MHz |
| Channel model | TDL-C |
| Delay scaling | At least 10ns, 100ns with 300 ns optional |
| Antenna configuration at BS* | (M,N,P) = (1,1,2) with omni-directional antenna element |
| Antenna configuration at UE | Single omni-directional antenna element |
| Antenna port virtualization | No beamforming and no beam selection |
| Frequency offset | 0 ppm |
| UE speed | 3 km/h |
| Subcarrier spacing | 15/30 kHz (with other SCS optional) |
| Number of code-division multiplexed users if applicable | 1 user, with other cases optional |
| Interference assumption | No inter-cell interference |
| * See Table 7-1 of R1-1704144 |  |

Table 2: Reporting Metrics

| Parameter | Value | Notes |
| --- | --- | --- |
| Enhanced PUCCH Format | Interlaced PF0, Interlaced PF1, E-PF2, E-PF3 | e.g., E-PF0, E-PF1, E-PF2, E-PF3 |
| Number of OFDM symbols used for PUCCH resource | 1,2,4,14 depending on format. | e.g., 1, 2, 4, 14 |
| Number of RBs used for PUCCH resource (N_RB) | 10 | At least 1 full interlace assumed (Assume interlace design for 20 MHz carrier bandwidth as agreed in RAN1AH#1901 for 15kH and 30 kHz is used) |
| Frequency domain OCC configuration details (if applicable) | Specified per simulation, according to description in Section 2. | Include length and type of OCC, mapping of OCCs to control symbols and reference (DMRS) symbols, OCC cycling (if applicable) |
| Time domain OCC configuration details (if applicable) | Specified per simulation, according to description in Section 2. | Include length and type of OCC, mapping of OCCs to control symbols and reference (DMRS) symbols |
| Number of multiplexed users, e.g., by code division if applicable | 1, unless otherwise specified. | 1 user is assumed as baseline Companies are to report other cases if evaluated |
| Waveform | E-PF2: CP-OFDM E-PF3: DFT-s-OFDM Interlaced PF0/1: Sequence based | e.g., CP-OFDM or DFT-s-OFDM |
| PUCCH encoder type | For E-PF2 and E-PF3, Reed Muller for payload ≤ 11 bits; Polar for payload ≥ 12 bits | e.g., Reed Muller or Polar |
| SCS | 30 kHz | 15KHz, 30KHz |
| Noise level, Np (dBm) | As in note => | Np = -174 + 10*log10(SCS*12*N_RB) + NF NF = 5dB |
| Required SNR (dB) | SNR defined as the ratio between the energy of the “desired signal” and noise per PRB. | Required SNR needed to fulfil detection criterion(1)(2), read from simulation curve |
| Cubic Metric | Specified per simulation (see legend of each figure) | Note: Single value reported in dB |
| P_max (dBm) | 20 | Maximum allowed transmit power under PSD limit of 10dBm/MHz measured in any 1MHz chunk and considers the RBs used for the PUCCH resource |
| Backoff (dB) | Cubic Metric | Backoff is computed as the cubic metric. Note: If cubic metric is not used, information on the backoff metric used should be provided. |
| P_TX (dBm) | As in note => | P_TX = min(P_max, 23- Backoff) is maximum allowed transmit power for the waveform considering backoff |
| MCL (dB) | As in note => | MCL = P_TX – SNR – Np |
| PUCCH payload size(s) (bits) | 1-25 depending on format. | If multiple payload sizes evaluated, then MCL to be plotted vs. PUCCH payload size |
| PUCCH encoding rate(s) | Not reported. | If multiple payload sizes evaluated, then multiple encoding rates to be reported (if applicable) |
| - For PUCCH payloads of 1 or 2 bits, detection criterion assumes that the PUCCH payload consists of randomly drawn HARQ ACK/NACK bits and is defined as P(ACK to Error) ≤ 1% and P(NACK to ACK) ≤ 0.1%. Error is defined as NACK or DTX where the decision region for DTX is determined to ensure that the maximum P(DTX to ACK) ≤ 1% for the case when the input to the receiver is noise only. - For PUCCH payloads greater than 2 bits follow the requirements in TS38.104 Section 8.3 Note: - Companies should provide details of RE mapping in frequency and time of enhanced PUCCH formats that are evaluated - Companies should provide details about channel estimation assumptions, e.g. practical, ideal - Definition of SNR to be reported with evaluation metrics |  |  |

Table 3: Additional simulation information

| Property | Value |
| --- | --- |
| RE mapping | According to Rel-15, where applicable. |
| Channel estimation | Practical |
| SNR definition | SNR defined as the ratio between the energy of the “desired signal” and noise per PRB. |

In accordance with the evaluation assumptions Maximum Coupling Loss (MCL) is used as a figure of merit. MCL jointly takes into account the required SNR in order to achieve a target level of performance, the maximum allowed transmit power under a PSD constraint, as well as any required back off from the maximum transmit power due to cubic metric/PAPR for the considered design.

### 2.3.1	Performance of Interlaced PF0

Figure 4 shows the performance for interlaced PF0 for 1-2 bits using the reference sequence corresponding to u=0 and cyclic shift cycling (Alt-1a).

(a)	(b)

Figure 4: Performance of interlaced PF0 for reference sequence u=0, (a) 1 OFDM symbols and (b) 2 OFDM symbols.

### 2.3.2	Performance of Interlaced PF1

Figure 5 shows the performance for interlaced PF1 for 1-2 bits using the reference sequence corresponding to u=0 and cyclic shift cycling.

(a)	(b)

Figure 5: Performance of interlaced PF1 for reference sequence u=0, (a) 4 OFDM symbols and (b) 14 OFDM symbols.

### 2.3.3	Base Sequence for Interlaced PF0 / PF1

Three alternative reference sequence sets for interlaced PF0 and PF1 have been evaluated.

- The Rel-15 CGS with cyclic shift cycling, denoted Alt1a.

- The Rel-15 CGS with a different sequence corresponding to a different group number u in each PRB, denoted Alt2.

- A length 120 Zadoff-Chu sequence, denoted Alt3.

Alt2 has been evaluated by constructing 60211 combinations of different Rel-15 sequences in each PRB. The combinations have been selected as every 499th combination from all possible permutations without order calculated with the matlab function “I = combnk([1:30],numPRB);”. As can be seen in the results this can be regarded as enough combinations for evaluation.

The CM has been calculated for all cyclic shifts of all sequences based on u=0…29 and v=0 for each alternative. As can be seen in Figure 6 the CM for Alt2 is much worse than for the other alternatives. Because of this, simulations to derive MCL for Alt2 were not performed. Figure 6 show that the CM for Alt1a is more than 1 dB better than the CM for Alt3 at the 95th percentile.

Figure 6: CM for alternative sequences for interlaced PF0/PF1.

Figure 7 show the performance differences for Alt1a and Alt3 for interlaced PF0 with 1 OFDM symbol for sequences corresponding to u=0…29 and v=0. As can be seen the performance difference in terms of MCL is negligible. We note that the CM difference does not affect the MCL since the total transmit power plus CM is still less than the maximum UE power (23 dBm). One should also consider forward compatibility in that UEs of lower power class may be introduced in the future. If so, the CM advantage of the repeated CGS sequences then becomes relevant.

Based on the cubic metric advantage of repetition + cyclic shift cycling, we propose the following:

For interlaced PF0 and PF1, support Alt-1a in the RAN1#97 agreement, i.e., repeat the Rel-15 length-12 CGS in each PRB of the interlace. The initial cyclic shift is configured for PF0 and PF1 resources as in Rel-15, and cyclic shifts are cycled over the PRBs of the interlace.

(a)	(b)

Figure 7: Performance of interlaced PF0, 1 OFDM symbol, for reference sequences u=0…29, v=0, for Alt1a and Alt3 for (a) 10 ns and (b) 100 ns.

### 2.3.4	Performance of E-PF2

Figure 8 shows the performance of the candidate E-PF2 PUCCH design in terms of MCL at different PUCCH payloads for the case of 1 and 2 OFDM symbols. Two different delay spread values are considered (10 and 100 ns).

In Figure 8, performance is shown for simulations with 1 UE with different OCC mappings. The mappings are on the form AxB, where A is the intra symbol OCC length and B is the inter symbol OCC length.

The simulations are made with the PN-sequences and a new random sequence is used per block.

(a)	(b)

Figure 8: Performance of candidate E-PF2 PUCCH design for (using PN-sequences, new random sequence per block) (a) 1 OFDM symbols and (b) 2 OFDM symbols.

As discussed in the previous section, the application of OCC codes in the frequency domain to support user multiplexing can, if not mitigated by some means, lead to a degradation (increase) in both peak-to-average power ratio (PAPR) and cubic metric due to the necessary repetition of the data symbols before application of the OCCs. We suggest a simple approach to mitigate PAPR degradation in which each user cycles through all OCCs codes across the frequency domain to break up the repetition pattern. The cycling pattern is chosen such that for any given PRB, all multiplexed users use different OCCs. Table 4 shows the improvement with OCC cycling which is large, (a) vs (b), with OCC cycling there is no degradation in cubic metric from OCC. Based on these results we propose the following:

If PUCCH format PF2 supporting an interlaced mapping is further enhanced to support user multiplexing, support OCC cycling to minimize the PAPR/CM of the transmitted time domain waveform.

Table 4: Cubic metric for evaluation of E-PF2 PUCCH design (using PN-sequences, new random sequence per block)

|  | No OCC | Intra symbol OCC length 2 | Intra symbol OCC length 4 |
| --- | --- | --- | --- |
| E-PF2 without OCC cycling (a) | 4.0 dB | 5.6 dB | 8.6 dB |
| E-PF2 with OCC cycling (b) | 4.0 dB | 3.9 dB | 3.8 dB |

### 2.3.5	Performance of E-PF3

Figure 9 shows the performance of the candidate E-PF3 PUCCH design in terms of MCL at different PUCCH payloads for the case of 4 and 14 OFDM symbols. Two different delay spread values are considered (10 and 100 ns). The reference sequence used corresponds to u=0 and v=0. Performance is shown for simulations with 1 UE with different OCC mappings of length 1, 2, 4, and 6. We note that in Rel-15, OCC lengths 2 and 4 are supported for PF4. While length-6 was discussed during Rel-15 it was not agreed. Furthermore, PF4 can be viewed as PF3 simply with user-multiplexing on top.

Clearly, as the PUCCH duration is increased, the MCL increases, which translates to improved coverage. For example, for the green curves at low payload, the increase from 4 to 14 OFDM symbol duration is roughly 5 dB corresponding to a ratio 14:4 in increased energy collection.

As can be seen from Figure 9, multiplexing of up to 6 users can be supported with only a moderate performance degradation. One can see that the short (4 symbol) PUCCH is more sensitive to dispersion than the longer duration PUCCH. This suggests that the short PUCCH is suitable for lower dispersion and lower levels of user multiplexing, whereas the longer PUCCH durations are more suitable for higher dispersion and higher levels of user multiplexing.

Interlaced PUCCH format PF3 is further enhanced to support multiplexing of at least 2 and 4 users. FFS: Whether or not this can be considered as interlaced PF4.

(a)	(b)

Figure 9: Performance of candidate E-PF3 PUCCH design for reference sequence u=0, v=0, (a) 4 OFDM symbols and (b) 14 OFDM symbols.

# 3	Enhanced PRACH Design

In RAN1#97, the following agreement was made regarding the sequence design and mapping for enhanced PRACH for NR-U:

Agreement:

For a new enhanced design of NR-U PRACH in addition to the Rel-15 design (sequence length of 139) further discussion is limited to the following options

- ZC sequence of the following lengths

- 15 kHz: Choose one of L_RA=[571, 1151]

- 30 kHz: Choose one of L_RA=[283, 571]

- Repetition of Rel-15 PRACH sequences in frequency domain with potentially some mechanisms to improve the cubic metric

  - Consider one of 2 and 4 repetitions for 30 kHz and one of 4 and 8 repetitions for 15 kHz

- Note: Decision will be based on previously agreed evaluation metrics, capacity per cell (i.e., number of preambles per RACH occasion and number of RACH occasions) for the same time and frequency resources, specification impact and implementation complexity.

- Note: Companies should state any deviations in assumptions from the agreed evaluation assumptions.

In this section, we consider the two design options listed in this agreement, specifically long Zadoff-Chu (ZC) sequence, vs. repetition. Evaluations were performed in accordance with the agreed simulations assumptions listed in the appendix. This section provides a more detailed description of the PRACH design proposed in [4].

Additional details are as follows:

- For 15 kHz subcarrier spacing (SCS), the following four PRACH preamble designs were compared:

  - “ZC139 (Rel-15)”: Like Rel-15, i.e. length-139 Zadoff-Chu (ZC) sequence on 139 contiguous subcarriers.

  - “ZC571”: Length-571 ZC sequence on 571 contiguous subcarriers.

  - “ZC139x4”: Repetition 4 times in frequency domain of Rel-15 preamble, using 556 contiguous subcarriers.

  - “ZC139x4, CM reduction”: Same as “ZC139x4”, but with a phase ramps and/or common phase shifts of some of the repetitions to reduce CM and PAPR, see details below.

- For 30 kHz subcarrier spacing, the following four PRACH preamble designs were compared:

  - “ZC139 (Rel-15)”: Like Rel-15, i.e. length-139 Zadoff-Chu (ZC) sequence on 139 contiguous subcarriers.

  - “ZC283”: Length-283 ZC sequence on 283 contiguous subcarriers.

  - “ZC139x2”: Repetition 2 times in frequency domain of Rel-15 preamble, using 278 contiguous subcarriers.

  - “ZC139x2, CM reduction”: Same as “ZC139x2”, but with a phase ramp in the second repetition in order to reduce cubic metric (CM) and peak-to-average power ratio (PAPR), see details below.

- The CM/PAPR reduction for the designs with repeated sequence is obtained by introducing a different phase ramp and/or common phase shift for each repetition. The phase ramp is applied on a per-subcarrier basis within a repetition, and the common phase shift has the same value for all subcarriers within a repetition. The phase ramps and common phase shifts are the same for all Zadoff-Chu roots and cyclic shifts, and should therefore not affect performance in terms of missed detection, false alarm, or delay estimation accuracy, which is confirmed by the simulation results in Sections 3.1 and 3.2. The phase ramps and common phase shifts are applied by modifying the frequency-domain signal yu,vk (see 3GPP TS 38.211, section 6.3.3.1) of repetition r to form a corresponding CM-reduced signal yu,v, r,CMk for repetition r as follows:

yu,v, r,CMk=yu,vk∙exp⁡jπ∆ψrk+ϕr

where
	k 	is a subcarrier index within repetition r (i.e. k=0, 1,…, LRA-1) 
	∆ψr 	is a phase step per subcarrier defining the phase ramp
	ϕr 	is the common phase shift, common to all subcarriers of repetition r 

Note that the parameters ∆ψr and ϕr  do not depend on the ZC root u. However, further optimization could be obtained by making these parameters depend also on u. The used values are listed in Table 5 (ZC139x4) and Table 6 (ZC139x2). These values could likely be somewhat further optimized.

Table 5: Phase ramps and common phase shifts for CM reduction of ZC139x4 (15 kHz)

| Repetition r | Phase ramp step ∆ψr | Common phase shift ϕr |
| --- | --- | --- |
| 1 | 0 | 0 |
| 2 | 0 | -0.05 |
| 3 | 0.008 | -0.05 |
| 4 | 0.008 | 1 |

Table 6: Phase ramps and common phase shifts  for CM reduction of ZC139x2 (30 kHz)

| Repetition r | Phase ramp step ∆ψr | Common phase shift ϕr |
| --- | --- | --- |
| 1 | 0 | 0 |
| 2 | -0.008 | 0 |

For all designs, the 64 preambles in a cell are generated by randomly selecting a first logical sequence index (corresponding to a particular Zadoff-Chu root), and then, starting from that index, generating 64 consecutive sequences in the order defined in 3GPP TS 38.211, i.e. first in the order of increasing cyclic shifts, second in the order of increasing logical root index. The number of cyclic shifts used is configured based on max RTT(function of ISD) + delay spread margin, with the combined margin adjusted to ensure good performance for each scheme (see Table 7 and Table 8). Note that in Rel-15, the N_cs granularity is rather coarse for low N_cs, thereby limiting the amount of fine-tuning of number of cyclic shifts that can be done for the evaluated scenario.

Table 7: Number of cyclic shifts used in simulations for 15 kHz SCS

| Delay spread | Number of cyclic shifts |  |  |  |
| --- | --- | --- | --- | --- |
|  | ZC139 (Rel-15) | ZC571 | ZC139x4 | ZC139x4, CM reduction |
| 10 ns | 23 | 35 | 34 | 34 |
| 100 ns | 23 | 28 | 27 | 27 |

Table 8: Number of cyclic shifts used in simulations for 30 kHz SCS

| Delay spread | Number of cyclic shifts |  |  |  |
| --- | --- | --- | --- | --- |
|  | ZC139 (Rel-15) | ZC283 | ZC139x2 | ZC139x2, CM reduction |
| 10 ns | 17 | 17 | 17 | 17 |
| 100 ns | 13 | 14 | 13 | 13 |

- The receiver algorithm described in [5] was used, with fully coherent combining in time (i.e.  Nnc=1).

- No correlation between gNB receive antennas.

- SNR in the figures is defined as the received signal power on used subcarriers divided by the noise power on used subcarriers, where the received signal power is the average over many fading realizations. Note that for consistency with this definition, the total noise power in the MCL calculation will scale with number of used subcarriers.

- Regarding definition of missed-detection probability, our understanding is that the intention of the agreement in RAN1 AH 1901 was to align the definition with the one in 3GPP TS 38.104:

  - The probability of detection is the conditional probability of correct detection of the preamble when the signal is present. There are several error cases – detecting different preamble than the one that was sent, not detecting a preamble at all or correct preamble detection but with the wrong timing estimation.

It is our understanding that the 38.104 definition would not count the following event as a missed-detection: UE transmits preamble A and gNB detects preambles A and B (i.e, both A and B are detected above the threshold). The reason is that A is detected, so it is not a missed-detection of the transmitted preamble. To avoid this event being counted as a missed detection and thus align with 38.104, we propose the following clarification to the definition agreed in RAN1 AH 1901:

- The missed detection probability is defined as the ratio between the total number of missed detections and the total number of transmitted preambles within an observation interval. A missed detection is defined as the event of that are either not no preamble detected at all, or only different preamble(s) detected as a different preamble than the one that was sent, or preamble that was sent is detected but with timing error greater than the maximum value (i.e., 50% of normal CP length), and the total number of transmitted preambles within an observation interval.

Our intention is only to clarify, not to change the meaning of the agreement. Without change marks, the proposal is as follows:

The missed detection probability is defined as the ratio between the total number of missed detections and the total number of transmitted preambles within an observation interval. A missed detection is defined as the event of no preamble detected at all, or only different preamble(s) detected than the one that was sent, or preamble that was sent is detected, but with timing error greater than the maximum value (i.e., 50% of normal CP length).

## 3.2	Simulation Results for 15 kHz SCS

(a)	(b)

Figure 10: Mis-detection probability results for SCS 15 kHz, delay scaling (a) 10 ns, and (b) 100 ns.

(a)	(b)

Figure 11: False-alarm probability results for SCS 15 kHz, delay scaling (a) 10 ns, and (b) 100 ns.

(a)	(b)

Figure 12: Timing estimation error results for SCS 15 kHz for the SNR where resp. mis-detection probability is 1% for delay scaling (a) 10 ns, and (b) 100 ns.

## 3.1	Simulation Results for 30 kHz SCS

(a)	(b)

Figure 13: Mis-detection probability results for SCS 30 kHz with delay scaling (a) 10 ns, and (b) 100 ns.

(a)	(b)

Figure 14: False-alarm probability results for SCS 30 kHz, delay scaling (a) 10 ns, and (b) 100 ns.

(a)	(b)

Figure 15: Timing estimation error results for SCS 30 kHz for the SNR where respective mis-detection probability is 1% for delay scaling (a) 10 ns and (b) 100 ns.

## 3.3	Cubic Metric (CM) and PAPR

95th Percentile

(a)	(b)

Figure 16: Complementary CDF of CM over all sequences for the different designs for (a) 15 kHz SCS, and (b) 30 kHz SCS

(a)	(b)

Figure 17: Complementary CDF of PAPR over all sequences for the different designs for (a) 15 kHz SCS, and (b) 30 kHz SCS.

Table 9: 95th percentile cubic metrics (CM)

| Design | 95th percentile CM [dB] (1) |
| --- | --- |
| ZC139 (Rel-15) | 2.3 |
| ZC571 | 2.3 |
| ZC139x4 | 5.4 |
| ZC139x4, CM reduction | 0.5 |
| ZC283 | 2.3 |
| ZC139x2 | 2.7 |
| ZC139x2, CM reduction | 1.8 |

(1) Defined as the CM that 95% of all sequences of the design fall below

## 3.4	Metrics Summary

Results for the agreed evaluation metrics are summarized in Table 10 (15 kHz) and Table 11 (30 kHz).

Clearly, the maximum coupling loss for ZC283 and ZC139x2 with or without CM reduction are nearly identical. We do note, however, that while no backoff is needed under the agreed assumption of 23 dBm maximum UE output power capability, there could be need for backoff in case the UE has a lower maximum output power capability. In such case, the MCL of ZC139x2 with CM reduction could improve relative to ZC139x2 without repetition, and even relative to ZC139 (Rel-15).

For ZC139x4, MCL is improved by about 1 dB from CM reduction relative to ZC139x4 without CM reduction. In case of a UE with lower maximum output power capability, the improvement could be substantially larger, up to 5 dB. ZC139x4 with CM reduction could for such a UE even perform almost 2 dB better than single long ZC sequence (ZC571).

Table 10: Metrics summary for 15 kHz SCS and delay scaling 100 ns

| Parameter | Value |  |  |  |
| --- | --- | --- | --- | --- |
| Scheme | ZC139 (Rel-15) | ZC571 | ZC139x4 | ZC139x4, CM reduction |
| SCS (kHz) | 15 | 15 | 15 | 15 |
| PRACH sequence length (L_RA) | 139 | 571 | 139 | 139 |
| # of repetition (R) | 1 | 1 | 4 | 4 |
| N_cs | 6 (i.e. 23 shifts) | 20 (i.e. 28 shifts) | 5 (i.e. 27 shifts) | 5 (i.e. 27 shifts) |
| # of RBs used for one RO (N_RB) | 12 | 48 | 48 | 48 |
| # of interlaces used by one RO (N_interlace) | N/A (Contiguous PRB design) | N/A (Contiguous PRB design) | N/A (Contiguous PRB design) | N/A (Contiguous PRB design) |
| RACH frequency occupancy (MHz) (1) | 2.085 | 8.565 | 8.340 | 8.340 |
| Noise level, Np (dBm) (1) | -105.8 | -99.7 | -99.8 | -99.8 |
| SNR (dB) corresponding to 1% missed detection probability | -4.5 | -11.8 | -11.7 | -11.6 |
| P_max (dBm) (2) | 13.2 | 19.3 | 19.2 | 19.2 |
| Backoff (dB) (3) | 2.3 | 2.3 | 5.4 | 0.5 |
| P_TX (dBm) | 13.2 | 19.3 | 17.6 | 19.2 |
| MCL (dB) | 123.5 | 130.8 | 129.1 | 130.6 |
| N_FDM | 8 | 2 | 2 | 2 |
| Capacity for full network (per time-domain PRACH occasion) (4) | 813823 = 25392 | 257028 = 31920 | 213827 = 7452 | 213827 = 7452 |
| Capacity for one cell (per time-domain PRACH occasion) (5) | 864 = 512 | 264 = 128 | 264 = 128 | 264 = 128 |
| (1) Only includes used subcarriers, for consistency with the SNR definition used in simulations. (2) Based on the assumption of a max PSD of 10 dBm per 1 MHz interval. All four designs have a maximum of 66.7 subcarriers within 1 MHz. (3) Backoff based on 95th percentile of the CCDF of cubic metric. (4) Obtained as # of freq domain ROs * # of roots * # of cyclic shifts, for the agreed simulation assumptions (300 m ISD). (5) Obtained as # of freq domain ROs * 64. |  |  |  |  |

Table 11: Metrics summary for 30 kHz SCS and delay scaling 100 ns

| Parameter | Value |  |  |  |
| --- | --- | --- | --- | --- |
| Scheme | ZC139 (Rel-15) | ZC283 | ZC139x2 | ZC139x2, CM reduction |
| SCS (kHz) | 30 | 30 | 30 | 30 |
| PRACH sequence length (L_RA) | 139 | 283 | 139 | 139 |
| # of repetition (R) | 1 | 1 | 2 | 2 |
| N_cs | 10 (i.e. 13 shifts) | 20 (i.e. 14 shifts) | 10 (i.e. 13 shifts) | 10 (i.e. 13 shifts) |
| # of RBs used for one RO (N_RB) | 12 | 24 | 24 | 24 |
| # of interlaces used by one RO (N_interlace) | N/A (Contiguous PRB design) | N/A (Contiguous PRB design) | N/A (Contiguous PRB design) | N/A (Contiguous PRB design) |
| RACH frequency occupancy (MHz) (1) | 4.170 | 8.490 | 8.340 | 8.340 |
| Noise level, Np (dBm) (1) | -102.8 | -99.7 | -99.8 | -99.8 |
| SNR (dB) corresponding to 1% missed detection probability | -5.0 | -8.7 | -8.6 | -8.6 |
| P_max (dBm) (2) | 16.2 | 19.3 | 19.2 | 19.2 |
| Backoff (dB) (3) | 2.3 | 2.3 | 2.7 | 1.8 |
| P_TX (dBm) | 16.2 | 19.3 | 19.2 | 19.2 |
| MCL (dB) | 124.0 | 127.7 | 127.6 | 127.6 |
| N_FDM | 4 | 2 | 2 | 2 |
| Capacity for full network (per time-domain PRACH occasion) (4) | 413813 = 7176 | 228214 = 7896 | 213813 = 3588 | 213813 = 3588 |
| Capacity for one cell (per time-domain PRACH occasion) (5) | 464 = 256 | 264 = 128 | 264 = 128 | 264 = 128 |
| (1) Only includes used subcarriers, for consistency with the SNR definition used in simulations. (2) Based on the assumption of a max PSD of 10 dBm per 1 MHz interval. All four designs have a maximum of 33.3 subcarriers within 1 MHz. (3) Backoff based on 95th percentile of the CCDF of cubic metric. (4) Obtained as # of freq domain ROs * # of roots * # of cyclic shifts, for the agreed simulation assumptions (300 m ISD). (5) Obtained as # of freq domain ROs * 64. |  |  |  |  |

Based on the above summary and graphs one can see rather similar performance for single long ZC sequence vs. repeated ZC sequence with CM reduction (ZC571 vs. ZC139x4 with CM reduction and ZC283 vs. ZC139x2 with CM reduction) in terms of mis-detection probability, false alarm probability, and timing estimation error. Furthermore, the respective schemes have nearly identical MCL. However, one aspect that is masked by the MCL metric definition is the CM advantage of the repeated ZC sequence scheme + CM reduction compared to the long ZC scheme, especially for the case of 4 repetitions. For the case of 4 repetitions (see Table 9), the CM is 0.5 dB for ZC139x4 + CM reduction, whereas for the ZC571 scheme, the CM is 2.3 dB. Hence, the 4 repetition scheme has 1.8 dB superior CM. This can become very important for the case of lower power class UEs, where the CM advantage would translate directly to a 1.8 dB smaller power amplifier, translating to lower cost and power consumption. Such UE power classes should be considered from a forward compatibility point of view, considering new use cases, e.g., industrial scenarios.

One argument that has been used in favor of the long ZC scheme is that the overall network capacity for RACH is greater compared to the repetition scheme. However, it is debatable whether or not this is important, especially in a small cell scenario. More importantly, the RACH capacity per cell is identical for the long ZC and repeated ZC schemes.

Based on these observations, especially, the almost 2 dB CM improvement possible from repetition for 15 kHz SCS (i.e. ZC139x4 with CM reduction), we have a preference for repeated sequence in that case. In order to minimize implementation and specification complexity, one should then use repeated sequence also for 30 kHz SCS (i.e. ZC139x2 with CM reduction). We acknowledge that we have proposed the opposite previously; however, further investigation has revealed that the CM can be significantly reduced for ZC repetition using very simple phase ramps and common phase shifts, to levels lower than the long ZC scheme. For this reason, we prefer ZC sequence repetition.

Thanks to the short preambles foreseen to be needed in the small cells typically considered for NR-U, both network and cell capacity can be increased, if needed, through time-multiplexing of PRACH occasions. However, if the system is configured to map all SS/PBCH blocks to a single time-domain PRACH occasion, which as explained in Section 2.3 of [6] may be desirable, capacity per cell can only be increased through frequency-multiplexing of PRACH occasions (since the number of preambles per cell per PRACH occasion is fixed to 64). Therefore, each PRACH occasion should occupy at most 50 % of the bandwidth to allow some frequency-domain multiplexing. Based on this we propose:

For the enhanced PRACH sequence design, support repetition of the Rel-15 Zadoff-Chu sequence of length LRA = 139. For 15 kHz SCS, support 4 repetitions with sequence values mapped to 556 contiguous subcarriers. For 30 kHz SCS, support 2 repetitions with sequence values mapped to 278 contiguous subcarriers. In both cases, support a CM reduction mechanism using a phase ramp + common phase shift per repetition. The following items are FFS:

  - Phase ramp and common phase shift per repetition, e.g., values in Table 5 and Table 6Table 5.

  - Supported values of cyclic shift spacing NCS as well as mapping of logical sequence index i to sequence number u with minimal change to Rel-15.

# Conclusion

Based on the discussion in this paper we propose the following:

Proposal 1	For interlaced PF0 and PF1, support Alt-1a in the RAN1#97 agreement, i.e., repeat the Rel-15 length-12 CGS in each PRB of the interlace. The initial cyclic shift is configured for PF0 and PF1 resources as in Rel-15, and cyclic shifts are cycled over the PRBs of the interlace.

Proposal 2	If PUCCH format PF2 supporting an interlaced mapping is further enhanced to support user multiplexing, support OCC cycling to minimize the PAPR/CM of the transmitted time domain waveform.

Proposal 3	Interlaced PUCCH format PF3 is further enhanced to support multiplexing of at least 2 and 4 users. FFS: Whether or not this can be considered as interlaced PF4.

Proposal 4	The missed detection probability is defined as the ratio between the total number of missed detections and the total number of transmitted preambles within an observation interval. A missed detection is defined as the event of no preamble detected at all, or only different preamble(s) detected than the one that was sent, or preamble that was sent is detected, but with timing error greater than the maximum value (i.e., 50% of normal CP length).

Proposal 5	For the enhanced PRACH sequence design, support repetition of the Rel-15 Zadoff-Chu sequence of length LRA = 139. For 15 kHz SCS, support 4 repetitions with sequence values mapped to 556 contiguous subcarriers. For 30 kHz SCS, support 2 repetitions with sequence values mapped to 278 contiguous subcarriers. In both cases, support a CM reduction mechanism using a phase ramp + common phase shift per repetition. The following items are FFS:

a.	Phase ramp and common phase shift per repetition, e.g., values in Table 5 and Table 6Table 5.

b.	Supported values of cyclic shift spacing NCS as well as mapping of logical sequence index i to sequence number u with minimal change to Rel-15.

# References

R1-1909297, “UL signals and channels for NR-U,” Ericsson, RAN1#98, August 2019.

Chairman’s Notes, RAN1#96bis, April 2019.

R1-1813412, “UL signals and channels for NR-U”, Qualcomm Incorporated, RAN1#95, Nov 2018.

R1-1909295, “Initial access signals and channels,” Ericsson, RAN1#98, August 2019.

R1-1609672, “NR PRACH preamble evaluations”, Ericsson, RAN1#86bis, October 2016.

R1-1909299, “Enhancements to initial access procedure”, Ericsson, RAN1#98, August 2019.

# Appendix – Example PUCCH Design

Figure 18: Exemplary PRB-based interlace design PUCCH for 30 kHz with 5 interlaces defined over 51 PRBs. Interlaces 2,3,4,5 have 10 PRBs per interlace; Interlace 1 has 11. Two exemplary PUCCH configurations are shown occupying interlaces 1 and 2, respectively. Both PUCCH configurations use 10 PRBs. The other interlaces may be used for PUSCH.