3GPP TSG RAN WG1 Meeting #98																R1-1909357

Prague, Czech, August 26 – 30, 2019

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.211 | CR | DRAFT | rev | - | Current version: | 15.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on PDSCH type B DMRS location when DMRS collides with a search space set |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  | Date: |  |  | 2019-08-16 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Issue 1: The handling of the Type B PDSCH collision with a search space set does not work as intended if the search space set colliding with PDSCH has muliple monitoring occasions. If two or more monitoring occassions are configured for the search-space, it is not clear after which monitoring occasion CORESET the DMRS should be placed. Issue 2: If multiple search-space sets collide with a PDSCH, it is not clear to which search-space set “a search space set” refers to and thus after which CORESET the DMRS should be placed. For PDSCH mapping type B - if […] the PDSCH allocation collides with resources reserved for a search space set associated with a CORESET, shall be incremented such that the first DM-RS symbol occurs immediately after the CORESET |  |  |  |  |  |  |  |  |
| Summary of change: |  | Modifying the delayed DMRS definition condition to check for the front-loaded DMRS collision with CORESET containing the PDCCH scheduling the PDSCH to make it clear which collision is of interest and avoid confusion in the case of multiple monitoring occasions or multiple SSSs. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The DMRS delay definition with the Type B PDSCH allocation only works when the Type B PDSCH collides only with one monitoring occasion of one search-space set of a UE. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.4.1.1.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Isolated impact analysis - If the gNB implements the change and the UE does not, if there are other monitoring occasions or SSSs colliding with the Type B PDSCH the DMRS locations are not understood the same way and the PDSCH reception fails. However, this is also the case if neither the UE nor the gNB implement the CR. - If the UE implements the change and the gNB does not, the gNB cannot schedule a Type B PDSCH that collides with any other monitoring or SSS than the one scheduling the PDSCH. However, this is also the case if neither the UE nor the gNB implement the CR. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

##### 7.4.1.1.2	Mapping to physical resources

The UE shall assume the PDSCH DM-RS being mapped to physical resources according to configuration type 1 or configuration type 2 as given by the higher-layer parameter dmrs-Type.

The UE shall assume the sequence  is scaled by a factor  to conform with the transmission power specified in [6, TS 38.214] and mapped to resource elements k,lp,μ according to

where , , and Δ are given by Tables 7.4.1.1.2-1 and 7.4.1.1.2-2 and the following conditions are fulfilled:

-	the resource elements are within the common resource blocks allocated for PDSCH transmission

The reference point for  is

-	subcarrier 0 of the lowest-numbered resource block in CORESET 0 if the corresponding PDCCH is associated with CORESET 0 and Type0-PDCCH common search space and is addressed to SI-RNTI;

-	otherwise, subcarrier 0 in common resource block 0

The reference point for  and the position  of the first DM-RS symbol depends on the mapping type:

-	for PDSCH mapping type A:

-	 is defined relative to the start of the slot

-	if the higher-layer parameter dmrs-TypeA-Position is equal to 'pos3' and  otherwise

-	for PDSCH mapping type B:

-	 is defined relative to the start of the scheduled PDSCH resources

-

The position(s) of the DM-RS symbols is given by  and duration ld where

-	for PDSCH mapping type A, ld is the duration is between the first OFDM symbol of the slot and the last OFDM symbol of the scheduled PDSCH resources in the slot

-	for PDSCH mapping type B, ld is the duration is the number of OFDM symbols of the scheduled PDSCH resources

and according to Tables 7.4.1.1.2-3 and 7.4.1.1.2-4. The case dmrs-AdditionalPosition equals to 'pos3' is only supported when dmrs-TypeA-Position is equal to 'pos2'. For PDSCH mapping type A, ld=3 and ld=4 symbols in Tables 7.4.1.1.2-3 and 7.4.1.1.2-4 respectively is only applicable when dmrs-TypeA-Position is equal to 'pos2'. For PDSCH mapping Type A single-symbol DM-RS, l1=11 except if all of the following conditions are fulfilled in which case l1=12:

-	the higher-layer parameter lte-CRS-ToMatchAround is configured and any PDSCH DM-RS symbol conincides with any symbol containing LTE cell-specific reference signals as indicated by the higher-layer parameter lte-CRS-ToMatchAround; and

-	the higher-layer parameters dmrs-AdditionalPosition is equal to 'pos1' and l0=3; and

-	the UE has indicated it is capable of additionalDMRS-DL-Alt

For PDSCH mapping type B

-	if the PDSCH duration ld is 2, 4, or 7 OFDM symbols for normal cyclic prefix or 2, 4, 6 OFDM symbols for extended cyclic prefix, and the front-loaded DM-RS of the PDSCH allocation overlaps with the resources in the CORESET containing the PDCCH scheduling the PDSCH,  shall be incremented such that the first DM-RS symbol occurs immediately after that CORESET and

-	if the PDSCH duration ld is 2 symbols, the UE is not expected to receive a DM-RS symbol beyond the second symbol,

-	if the PDSCH duration ld is 4 symbols, the UE is not expected to receive a DM-RS symbol beyond the third symbol,

-	if the PDSCH duration ld is 7 symbols for normal cyclic prefix or 6 symbols for extended cyclic prefix,

-	the UE is not expected to receive the first DM-RS beyond the fourth symbol, and

-	if one additional single-symbol DM-RS is configured, the UE only expects the additional DM-RS to be transmitted on the 5th or 6th symbol when the front-loaded DM-RS symbol is in the 1st or 2nd symbol, respectively, of the PDSCH duration, otherwise the UE should expect that the additional DM-RS is not transmitted.

-	if the PDSCH duration ld is 2 or 4 OFDM symbols, only single-symbol DM-RS is supported.

The time-domain index l' and the supported antenna ports p are given by Table 7.4.1.1.2-5 where

-	single-symbol DM-RS is used if the higher-layer parameter maxLength in the DMRS-DownlinkConfig IE is not configured

-	single-symbol or double-symbol DM-RS is determined by the associated DCI if the higher-layer parameter maxLength in the DMRS-DownlinkConfig IE is equal to 'len2'.

In absence of CSI-RS configuration, and unless otherwise configured, the UE may assume PDSCH DM-RS and SS/PBCH block to be quasi co-located with respect to Doppler shift, Doppler spread, average delay, delay spread, and, when applicable, spatial Rx parameters. The UE may assume that the PDSCH DM-RS within the same CDM group are quasi co-located with respect to Doppler shift, Doppler spread, average delay, delay spread, and spatial Rx. The UE may assume that DMRS ports associated with a PDSCH are QCL with QCL Type A, Type D (when applicable) and average gain.

The UE may assume that no DM-RS collides with the SS/PBCH block.

Table 7.4.1.1.2-1: Parameters for PDSCH DM-RS configuration type 1.

| p | CDM group | Δ |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| 1000 | 0 | 0 | +1 | +1 | +1 | +1 |
| 1001 | 0 | 0 | +1 | -1 | +1 | +1 |
| 1002 | 1 | 1 | +1 | +1 | +1 | +1 |
| 1003 | 1 | 1 | +1 | -1 | +1 | +1 |
| 1004 | 0 | 0 | +1 | +1 | +1 | -1 |
| 1005 | 0 | 0 | +1 | -1 | +1 | -1 |
| 1006 | 1 | 1 | +1 | +1 | +1 | -1 |
| 1007 | 1 | 1 | +1 | -1 | +1 | -1 |

Table 7.4.1.1.2-2: Parameters for PDSCH DM-RS configuration type 2.

| p | CDM group | Δ |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| 1000 | 0 | 0 | +1 | +1 | +1 | +1 |
| 1001 | 0 | 0 | +1 | -1 | +1 | +1 |
| 1002 | 1 | 2 | +1 | +1 | +1 | +1 |
| 1003 | 1 | 2 | +1 | -1 | +1 | +1 |
| 1004 | 2 | 4 | +1 | +1 | +1 | +1 |
| 1005 | 2 | 4 | +1 | -1 | +1 | +1 |
| 1006 | 0 | 0 | +1 | +1 | +1 | -1 |
| 1007 | 0 | 0 | +1 | -1 | +1 | -1 |
| 1008 | 1 | 2 | +1 | +1 | +1 | -1 |
| 1009 | 1 | 2 | +1 | -1 | +1 | -1 |
| 1010 | 2 | 4 | +1 | +1 | +1 | -1 |
| 1011 | 2 | 4 | +1 | -1 | +1 | -1 |

Table 7.4.1.1.2-3: PDSCH DM-RS positions  for single-symbol DM-RS.

| ld in symbols | DM-RS positions |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | PDSCH mapping type A |  |  |  | PDSCH mapping type B |  |  |  |
|  | dmrs-AdditionalPosition |  |  |  | dmrs-AdditionalPosition |  |  |  |
|  | 0 | 1 | 2 | 3 | 0 | 1 | 2 | 3 |
| 2 | - | - | - | - |  |  |  |  |
| 3 |  |  |  |  | - | - |  |  |
| 4 |  |  |  |  |  |  |  |  |
| 5 |  |  |  |  | - | - |  |  |
| 6 |  |  |  |  |  |  |  |  |
| 7 |  |  |  |  |  |  |  |  |
| 8 |  | , 7 | , 7 | , 7 | - | - |  |  |
| 9 |  | , 7 | , 7 | , 7 | - | - |  |  |
| 10 |  | , 9 | , 6, 9 | , 6, 9 | - | - |  |  |
| 11 |  | , 9 | , 6, 9 | , 6, 9 | - | - |  |  |
| 12 |  | , 9 | , 6, 9 | , 5, 8, 11 | - | - |  |  |
| 13 |  | , l1 | , 7, 11 | , 5, 8, 11 | - | - |  |  |
| 14 |  | , l1 | , 7, 11 | , 5, 8, 11 | - | - |  |  |

Table 7.4.1.1.2-4: PDSCH DM-RS positions  for double-symbol DM-RS.

| ld in symbols | DM-RS positions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | PDSCH mapping type A |  |  | PDSCH mapping type B |  |  |
|  | dmrs-AdditionalPosition |  |  | dmrs-AdditionalPosition |  |  |
|  | 0 | 1 | 2 | 0 | 1 | 2 |
| <4 |  |  |  | - | - |  |
| 4 |  |  |  | - | - |  |
| 5 |  |  |  | - | - |  |
| 6 |  |  |  |  |  |  |
| 7 |  |  |  |  |  |  |
| 8 |  |  |  | - | - |  |
| 9 |  |  |  | - | - |  |
| 10 |  | , 8 |  | - | - |  |
| 11 |  | , 8 |  | - | - |  |
| 12 |  | , 8 |  | - | - |  |
| 13 |  | , 10 |  | - | - |  |
| 14 |  | , 10 |  | - | - |  |

Table 7.4.1.1.2-5: PDSCH DM-RS time index  and antenna ports .

| Single or double symbol DM-RS |  | Supported antenna ports |  |
| --- | --- | --- | --- |
|  |  | Configuration type 1 | Configuration type 2 |
| single | 0 | 1000 – 1003 | 1000 – 1005 |
| double | 0, 1 | 1000 – 1007 | 1000 – 1011 |