3GPP TSG-RAN WG1 Meeting #98	R1-1909381

Prague, CZ, August 26th – 30th 2019

Agenda Item:	5

Source:	Ericsson

Title:	Discussion on NR V2X resource pool configuration and selection

Document for:	Discussion, Decision

# 1	Introduction

RAN1 received the LS in [1] with some requests from RAN2.

# 2	Discussion

The first request is the following:

RAN2 discussed the NR V2X resource pool configuration and selection. In the following, a working assumption has been made: In LTE V2X, the parameter SyncAllowed is used to indicate whether each synchronization reference is allowed to use for each resource pool. Therefore, in NR V2X, the allowed synchronization reference resource also needs to be indicated for each resource pool. RAN2 would like to check with RAN1 whether there is any potential issue on this working assumption from RAN1 side.

From our perspective, there is no problem with reusing this parameter from LTE.

From RAN1 point of view, there is no issue with the working assumption.

The LS contains some further questions from RAN2:

In addition, RAN2 would like to consult with RAN1 on the following questions about resource pool configuration and selection: - Whether resource pool configuration based on zone is considered beneficial to support in NR V2X ? - Whether resource pool configuration based on different cast types should be supported by taking into account the configuration of PSFCH resource? - Whether an NR V2X mode 2 UE can select multiple resource pools on single carrier from RAN1 perspective?

Regarding the first issue, we note that there has been no discussion on zones since Rel-14. In our view, zone-based pools are not useful in practice. Discussion is not necessary and Rel-14 conclusions can be reused.

RAN1 has not discussed or concluded anything zone-based pools since Rel-14.

Regarding the second issue, we note two things:

- RAN1 has agreed to support pools with and without PSFCH resources.

- There is no one-to-one mapping between cast types (e.g., UC/GC/BC) and the use of HARQ. HARQ feedback is supported, but not mandated, for UC and GC.

Given these observations, we think that it is not necessary to have resource pool configuration for specific cast types. However, we expect that it is supported to configure pools with/without PSFCH resources.

The mapping between cast types (i.e., UC/GC/BC) and HARQ feedback configurations is not one-to-one.

RAN1 thinks that it is not necessary to have resource pool configuration based on different cast types. Rather, it is necessary to support pools with and without PSFCH resources, as per RAN1 agreements.

The third issue is related to the second one. Given that RAN1 expects that both pools with and without PSFCH resources can be configured, a UE must be able to select from multiple configured pools. We discuss pool selection in [2]

RAN1 thinks that it is necessary to specify a procedure for pool selection if multiple pools are configured.

# 3	Conclusion

In the previous sections we made the following observations:

Observation 1	RAN1 has not discussed or concluded anything zone-based pools since Rel-14.

Observation 2	The mapping between cast types (i.e., UC/GC/BC) and HARQ feedback configurations is not one-to-one.

Based on the discussion in the previous sections we propose the following:

Proposal 1	From RAN1 point of view, there is no issue with the working assumption.

Proposal 2	RAN1 thinks that it is not necessary to have resource pool configuration based on different cast types. Rather, it is necessary to support pools with and without PSFCH resources, as per RAN1 agreements.

Proposal 3	RAN1 thinks that it is necessary to specify a procedure for pool selection if multiple pools are configured.

# 4	References

R1-1908004, “LS on NR V2X resource pool configuration and selection”, RAN WG2, Prague, Czech Republic, Aug 2019.

R1-1908914, “Miscellaneous issues on resource allocation”, Ericsson, Prague, Czech Republic, Aug 2019.