3GPP TSG-RAN WG1 #98	R1-1909394

Prague, CZ, August 26th – 30th, 2019

Agenda Item:	6.2.1.1

Source:	Qualcomm Incorporated

Title:	Feature lead summary of 6.2.1.1 UE group MWUS

Document for:	Discussion and Decision

# 1	Introduction

This is to summarize the issues for the 6.2.1.1 providing potential proposals on UE group MWUS based on the views expressed by companies in the contributions listed in the references.

So far, we have the following latest agreements for NB-IoT WUS in RAN1#98 meeting.

Agreement

The maximum number of UE groups per WUS resource is 8.

Agreement

The following WA is confirmed:

At least for the group WUS in the same WUS resource, legacy WUS with phase shifts is selected as group WUS sequence design according to w_(group)(m’) = w(m’) exp(j2πgm/G)

Agreement

The specification supports configurability to enable UE group to alternate between WUS resources

Agreement

- Different WUS resources use different scrambling initialization, c_init

Agreement

A UE is required to detect 2 sequences, the common WUS and the group WUS of the group to which it belongs

# 2	Discussion

## 2.1	Multiplexing of WUS resources

#2.1.1 Up to 2 orthogonal WUS resources may be configured in both TDM and FDM (up to 4 orthogonal WUS resources in total)

- Yes: Ericsson, QC, Intel, LGE, DCM, Nokia, NSB

Observation: No objection

Proposal 1: Both options can be configured simultaneously to have up to 4 orthogonal WUS resources including legacy WUS resource

- Up to 2 orthogonal resources may be configured in time domain.

- Up to 2 orthogonal resources may be configured in frequency domain

#2.1.2 Patterns for WUS resource multiplexing associated to one PO

Table 1 Summary of WUS resource pattern ocnfiguration

|  | Patterns | Pattern examples | Companies’ views | Supporting or not |
| --- | --- | --- | --- | --- |
| 1 | 1 WUS resource | Note: WUS0 is the legacy WUS resource (may be shared by Rel-15 WUS and Rel-16 WUS or used for Rel-16 UEs if no Rel-15 UEs) | Yes: QC, Intel, LGE, DCM, Nokia, NSB, Huawei, HiSi No: Ericsson | Yes Agreement Rel-15 WUS and Rel-16 WUS can be configured on the same legacy WUS resource via SI |
| 2a | 2 WUS resources (FDM only) |  | Yes: QC, Intel, LGE, DCM, Nokia, NSB, Huawei, HiSi, Ericsson No: | Yes Agreement Support the option for WUS resource configuration: - Up to 2 orthogonal resources may be configured in frequency domain |
| 2b | 2 WUS resources (TDM only) |  | Yes: QC, Intel, LGE, DCM, Nokia, NSB, Huawei, HiSi No: [Ericsson] | Yes Agreement Support the following options for WUS resource configuration - Up to 2 orthogonal resources may be configured in time domain. |
| 2c | 2 WUS resources (TDM+FDM) |  | Yes: ? No: Ericsson, QC | No Concern: larger latency for WUS1 than 2a; no strong benefits than 2b |
| 3a | 3 WUS resources (FDM+TDM) |  | Yes: QC, Intel, LGE, DCM, Nokia, NSB, Huawei, HiSi No: Ericsson | Yes? (slight majority) |
| 3b | 3 WUS resources (FDM+TDM) |  | Yes: QC, Intel, LGE, Nokia, NSB, DCM, Huawei, HiSi, Ericsson No: | Yes |
| 3c | 3 WUS resources (FDM+TDM) |  | Yes: ? No: Ericsson | No Concern: larger latency for WUS2 than 3a |
| 3d | 3 WUS resources (FDM only) |  | Yes: Ericsson, Sony No: QC, LGE, Intel, Nokia, NSB, Huawei, HiSi, DCM | No? (slight majority) Concern: Requires in total +9.6dB power boosting if +4.8dB per WUS resource |
| 3e | 3 WUS resources (TDM only) |  | Yes: No: Ericsson, QC, LGE, Intel, Nokia, NSB, Huawei, HiSi, DCM | No |
| 4a | 4 WUS resources (FDM+TDM) |  | Yes: QC, LGE, Intel, Nokia, NSB, Huawei, HiSi, DCM, Ericsson No: | Yes |
| 4b | 4 WUS resources (FDM+TDM) |  | Yes: No: QC, LGE, Intel, Nokia, NSB, Huawei, HiSi, DCM, Ericsson | No |
| 4c | 4 WUS resources (FDM+TDM) |  | Yes: No: QC, LGE, Intel, Nokia, NSB, Huawei, HiSi, DCM, Ericsson | No |

Observation: Majority views are to support WUS resource pattern 1, 2a, 2b, 3a, 3b, 4a. Slight majority views to not support WUS resource pattern 3d due to power boosting concern.

Proposal 2: Only support the WUS resource pattern 1, 2a, 2b, 3a, 3b, 4a, [3d] in Table 1.

- FFS the details of how to indicate the WUS resource pattern configuration.

#2.1.3 Other details for WUS resource pattern configuration

Ericsson:

- If configured, legacy WUS is Nresource = 0.

- Group WUS resources are indexed according to

a.	Nresource = 1 is the resource in the center of the narrowband,

b.	Nresource = 2 is the resources that are adjacent to Nresource = 1, in either time or frequency, and

c.	Nresource = 3 is the resource that is adjacent to Nresource = 2 in the frequency domain.

- If group WUS is configured, legacy WUS is not configured in the center of a narrowband.

Observations: Need more discussion.

## 2.2	Number of UE groups and WUS sequences

2.2.1 Number of UE groups per WUS resources:

- Max num>6: Sony

- Max num=8: QC, Huawei, HiSi,

- FFS: Ericsson

Observations: Larger number of UE groups improve the power saving gain. But the sequence design needs to consider the required number of WUS sequences, including the WUS sequence per group and common WUS sequence.

Proposal 3: The maximum number of UE groups per WUS resource is 8.

2.2.2 Number of UE groups uniform for each WUS resource:

- Yes: Ericsson

- No: Nokia, NSB

- FFS: QC

Observations: Need more discussion.

## 2.3	WUS sequence design for MWUS

2.3.1 WUS sequence in different WUS resource

Different WUS sequences associated with the same PO are used for UE groups regardless of whether the UE groups are in the same or different WUS resources.

- Yes: Ericsson, LGE (for FDMed WUS resources), Qualcomm, Nokia, NSB, Intel

  - Ericsson, Qualcomm: 2-bit MSB of scrambling initialization c_init

  - Intel: phase shifted legacy scrambling sequence if up to 3 WUS resources in FDM

  - LGE: phase

- No: Intel

  - Intel: if up to 2 WUS resources in FDM, use phase on 2-PRB WUS for PARA reduction

- FFS: LGE (for TDMed WUS resources)

Observations: Majority views on Yes for multiple benefits in FDM/TDM.

Proposal 4: Different WUS sequences are used in same/different WUS resources.

Proposal 5: Down select one of the following schemes to differentiate WUS sequences indifferent WUS resources.

- Alt1: 2-bit MSB of scrambling initialization c_init

- Alt2: phase shifted legacy scrambling sequence

2.3.2 MWUS sequence mapping in 2PRBs

Ericsson: The NWUS is flipped subcarrier wise in the second PRB such that the complete sequence per symbol is [x0 x1 … x11 x11 … x1 x0], assuming the NB-IoT WUS sequence is [x0 x1 … x11].

- Proposal: UE-group MWUS sequence is same as UE-group NWUS per RB and the same sequence is repeated in two PRBs where the frequency domain sequence order is reversed in the second PRB.

Observations: have agreed to repeat same sequence in two PRBs for group MWUS sequence, as follows

Agreement (RAN#96)

UE-group MWUS sequence is same as UE-group NWUS per RB and the same sequence is repeated in two PRBs.

Need further study on whether need to change the agreement.

## 2.4	Power and Max WUS duration

Working Assumptions

- UE assumes the transmit power for Rel-16 WUS sequence is same as that of Rel-15 WUS sequence.

- Maximum WUS duration for Rel-16 WUS sequence is same as that of Rel-15 WUS sequence

Whether to confirm the above WA

- Yes: Ericsson, DCM, Sony, QC

- No: ZTE

ZTE (R1-1908255): Requires 32/64/64 subframes for 1/2/3- sequence detection

- SNR=-7dB or -6dB?, no TxD, CFO=?, TimeOffset=?

|  | Required repetition number of MWUS |  |  |
| --- | --- | --- | --- |
| SNR=-7dB （1 Tx） | One detection sequence | Two detection sequences | Three detection sequences |
|  | 32 | 64 | 64 |

Figure 1 Miss detection rate of MWUS

Observation: Majority view is to confirm WA since different power boosting/max WUS duration for Rel-15 and Rel-16 WUS leads to complicated configuration/signaling but achieves marginal gain.

Proposal 6: Confirm the WAs:

- UE assumes the transmit power for Rel-16 WUS sequence is same as that of Rel-15 WUS sequence.

- Maximum WUS duration for Rel-16 WUS sequence is same as that of Rel-15 WUS sequence

## 2.5	Common WUS configuration

2.5.1 Common WUS for a subset of group UEs

Agreement

- Each UE monitors up to X WUS sequences.

  - Value of X will be selected between 2 and 3

  - X=3 can only be supported if a common WUS for a subset of UE groups for service-based grouping is supported

Max number X of sequences for UE monitoring for evaluation

- X=2: Ericsson, DCM, LGE, Intel, Sony, QC

- X=3: ZTE

- FFS: Nokia, NSB, Huawei, HiSi

Simulation comparison:

ZTE (R1-1908255)

QC (R1-1908825)

Observations: Majority view is to support up to max X=2 sequences for UE monitoring considering the power saving gain and complexity.

- Note: RAN4 is waiting for RAN1 decision on X for evaluation.

Proposal 7: A UE is required to detect 2 sequences, the common WUS and the group WUS of the group to which it belongs.

## 2.6	Weighting factor for UE distribution

Whether to support weighting factor configuration for different WUS resources of Rel-16 WUS UEs

- Yes: QC, LGE, Sony

- No: Ericsson, ZTE

Whether to support separate paging narrowband configuration for Rel-16 WUS UEs

- Yes:	Ericsson, QC, Sony

Observations: No clear majority view yet. Both topics are mainly up to RAN2 discussion.

## 2.7 	Others

The specification supports configurability to enable UE group to alternate between WUS resources

- ZTE, QC

Proposal 8: The specification supports configurability to enable UE group to alternate between WUS resources.

LGE: Support different starting subframe/actual WUS duration for the Enhanced Coverage Restricted UEs.

Nokia, NSB: separate on/off Rel15 and Rel16 WUS

Nokia, NSB: Support configuration of a single group for Rel-16 UEs associated with a Rel-16 WUS

Observations: No majority view yet. Need further study.

# 3	Summary

Potential proposals are summarized as follows:

Proposal 1: Both options can be configured simultaneously to have up to 4 orthogonal WUS resources including legacy WUS resource

- Up to 2 orthogonal resources may be configured in time domain.

- Up to 2 orthogonal resources may be configured in frequency domain

Proposal 2: Only support the WUS resource pattern 1, 2a, 2b, 3a, 3b, 4a, [3d] in Table 1.

- FFS the details of how to indicate the WUS resource pattern configuration.

Proposal 3: The maximum number of UE groups per WUS resource is 8.

Proposal 4: Different WUS sequences are used in same/different WUS resources.

Proposal 5: Down select one of the following schemes to differentiate WUS sequences indifferent WUS resources.

- Alt1: 2-bit MSB of scrambling initialization c_init

- Alt2: phase shifted legacy scrambling sequence

Proposal 6: Confirm the WAs:

- UE assumes the transmit power for Rel-16 WUS sequence is same as that of Rel-15 WUS sequence.

- Maximum WUS duration for Rel-16 WUS sequence is same as that of Rel-15 WUS sequence

Proposal 7: A UE is required to detect 2 sequences, the common WUS and the group WUS of the group to which it belongs.

Proposal 8: The specification supports configurability to enable UE group to alternate between WUS resources.

# References

| R1-1908018 | Ericsson | Observation 1 False wake-up rates and delayed paging rates may be substantially reduced by allowing independent resource configuration of group WUS and legacy WUS. Observation 2 With frequency multiplexed WUS resources together with a power boosted WUS, PAPR may become a problem. Observation 3 Phase shifting WUS in one PRB increases PAPR if more than one paging narrowband is used. Observation 4 Making WUS symmetric around its center frequency would allow for reduced detection complexity by down sampling x2. Observation 5 Detection of more than two sequences would jeopardize co-existence between group WUS and legacy WUS. Observation 6 Uneven UE distribution among different service classes will result in decreased power saving performance. Based on the discussion in the previous sections we propose the following: Proposal 1 Group WUS and legacy WUS do not share resources. Proposal 2 Four WUS configurations are defined: a. A single group WUS resource frequency multiplexed with legacy WUS, b. Two frequency multiplexed group WUS resources, in turn frequency multiplexed with legacy WUS, c. Two time multiplexed group WUS resources, in turn frequency multiplexed with legacy WUS, d. Three group WUS resources, where each resource is either frequency multiplexed, time multiplexed or both frequency and time multiplexed with legacy WUS. Proposal 3 If configured, legacy WUS is Nresource = 0. Proposal 4 Group WUS resources are indexed according to a. Nresource = 1 is the resource in the center of the narrowband, b. Nresource = 2 is the resources that are adjacent to Nresource = 1, in either time or frequency, and c. Nresource = 3 is the resource that is adjacent to Nresource = 2 in the frequency domain. Proposal 5 If group WUS is configured, legacy WUS is not configured in the center of a narrowband. Proposal 6 The number of UE groups are configured per resource and is configured jointly for all group WUS resources. Proposal 7 Conform working assumption that, at least for the group WUS in the same WUS resource, legacy WUS with phase shifts is selected as group WUS sequence design according to wgroup(m’) = w(m’) exp(j2πgm/G) Proposal 8 The outstanding parameters are set to G = 132 and g = 2+13*UEgroup, 0 ≤ UEgroup ≤ 10. Proposal 9 The WUS resource index, Nresource, is represented in the two MSBs of the WUS scrambling sequence initialization, i.e., cinit_WUS = (NIDcell + 1)(10nf_start_PO + nstart_PO/2 ) mod 2048 + 1) 29 + NIDcell + Nresource229 Proposal 10 The NWUS is flipped subcarrier wise in the second PRB such that the complete sequence per symbol is [x0 x1 … x11 x11 … x1 x0], assuming the NB-IoT WUS sequence is [x0 x1 … x11]. Proposal 11 Multilevel grouping is not supported, i.e., a UE is only required to monitor two sequences. Proposal 12 Confirm working assumption that the UE assumes the transmit power for Rel-16 WUS sequence is the same as that of Rel-15 WUS sequence. Proposal 13 Confirm working assumption that the maximum WUS duration for Rel-16 WUS sequence is the same as that of Rel-15 WUS sequence. |
| --- | --- | --- |
| R1-1908084 | Huawei, HiSilicon | Proposal 1: Up to 4 orthogonal WUS resources may be configured in total for both time and frequency domain. Proposal 2: Support the feature of waking up a subset of all WUS UE groups. Proposal 3: A maximum of eight UE-group WUS can be supported. Proposal 4: WUS including partial UE Paging ID is supported. |
| R1-1908255 | ZTE | Observation 1: The issue of UE unfairness can be solved by UE hopping, but the complexity of introducing UE hopping also needs to be considered. Proposal 1: Max duration and transmit power of Rel-16 MWUS can be configured. Proposal 2: The maximum total number of MWUS resource can be larger than 2 if more than two service types are supported. Proposal 3: UE group weighting is not supported. Proposal 4: Common MWUS is additionally used to wake up a subset of all Rel-16 group MWUS for service-based grouping. Proposal 5: Three detection sequences should be supported. Proposal 6: The details of MWUS sequence design are same as NB-IoT. |
| R1-1909505 (update of R1-1908291) | Nokia, Nokia Shanghai Bell | Observation 1: When a common WUS is configured, there is a common WUS for each of the orthogonal WUS resources that is used to wake up all Rel-16 UEs that are monitoring that resource. Observation 2: Waking up all Rel-16 UEs requires the transmission of a common WUS in each WUS resource. Proposal 1: Up 2 orthogonal WUS resources may be simultaneously configured in time domain and frequency domain to have up to 4 orthogonal WUS resources. Proposal 2: A maximum power boost of 4.8 dB is supported for the WUS in each resource. Proposal 3: Rel-16 WUS location in relation to legacy WUS is configured such that: - If one WUS resource is configured in the time domain, that WUS resource is configured to coincide with the legacy WUS resource, and, - If two WUS resources are configured, the first WUS resource coincides with the legacy WUS resource and the second WUS resource occurs before the first Rel-16 WUS resource. - FFS introduction of a configurable gap between the two resources. - Sharing of a WUS resource between legacy WUS and Rel-16 WUS is configured through assignment of Rel-16 UE groups to WUS resources. Proposal 5: Support configuration of a single group for Rel-16 UEs associated with a Rel-16 WUS. Proposal 6: The network can separately enable or disable operation of Rel-15 WUS and Rel-16 UE-group WUS in each cell. Proposal 7: Support fallback to Rel-15 WUS in a cell if it is enabled and if Rel-16 WUS is disabled in the cell. Proposal 8: Support the feature of waking up a subset of all WUS UE groups if the maximum number of UE groups that can be configured in each WUS resource exceeds X. FFS the value of X. Proposal 9: The Rel-16 WUS sequence assigned to a group depends on its group ID. Proposal 10: A Rel-16 WUS sequence is assigned to a group in only one of the resources associated with a PO. Proposal 11: Consider support for controlling relative UE group sizes or the number of groups associated with different WUS resources. |
| R1-1908519 | LG Electronics | Observation 1: The Enhanced Coverage Restricted UE camp on a cell with normal coverage. Observation 2: The Enhanced Coverage Restricted parameter is provided to both the eNB and the UE. Proposal 1: Consider non-uniform distribution of UEs and/or UE groups over WUS resources. For example, each WUS resource can be allocated with different number of UEs Proposal 2: Support different starting subframe and actual WUS duration for the Enhanced Coverage Restricted UEs. - The Enhance Coverage Restricted UE monitors WUS in the last [X] WUS subframes within the maximum WUS duration configured for the UE. - Here, X is equal to or less than the maximum number of WUS subframes in the WUS duration Proposal 3: Up to 2 orthogonal WUS resources can be configured per dimension. Proposal 4: Legacy WUS is counted as one of up to 2 orthogonal WUS resources in frequency domain. Proposal 5: All UE groups which shares a same time domain WUS resource use different UE group specific WUS sequences regardless of whether the UE groups are in the same or different WUS resources in frequency domain. |
| R1-1909464 (update of R1-1908613) | Intel Corporation | Proposal 1: Support simultaneous configuration of up to 2 orthogonal WUS resources per each dimension (time and frequency), which results in up to 4 orthogonal WUS resources total. - This accounts the legacy WUS resource Proposal 2: Each UE monitors up to 2 WUS sequences within the configured WUS resource. Proposal 3: First agree on the max number of frequency-multiplexed WUS resources corresponding to the same PO. - The use of different WUS sequences for UE groups associated with orthogonal WUS resources depends on the max number of frequency-multiplexed WUS resources. If only up to 2 WUS resources can be multiplexed in the frequency domain, the same WUS sequences can be reused for UE groups associated with orthogonal WUS resources corresponding to the same PO. If configuration of up to 3 frequency-multiplexed WUS resources is allowed, different WUS sequences based on scrambling shifts of the RE-level scrambling Gold code are used for UE group WUSes associated with orthogonal WUS resources corresponding to the same PO. |
| R1-1908756 | Sony | Observation 1: On the supporting of both common MWUS and group MWUS, the MWUS sequence design shall consider UE power consumption and complexity. Proposal 1: UE-group MWUS (rel.16) and legacy UE MWUS (rel.15) are using separate MWUS resource, and preferably multiplex in FDD. Proposal 2: Common-group MWUS signal is applicable to MTC UE release 16. Proposal 3: The maximum number of UE groups supported is at least 6 (i.e. to maximize the benefit of UE-group mechanism, such as reducing false-paging alarm rate). Proposal 4: Each UE monitors up to 2 WUS sequences. Proposal 5: Confirm these working assumptions: - UE assumes the transmit power for Rel-16 WUS sequence is same as that of Rel-15 WUS sequence - Maximum WUS duration for Rel-16 WUS sequence is same as that of Rel-15 WUS sequence |
| R1-1908825 | Qualcomm Incorporated | For group WUS multiplexing: Proposal 1: Up to 2 orthogonal WUS resources per time and frequency can be configured - Pattern A in Figure 4 is used to multiplex up to 2 contiguous WUS resources in TDM and FDM, including legacy WUS resource. Proposal 2: A maximum of 8 UE groups per WUS resource can be configured. For group WUS sequence design: - Proposal 3: Confirm the WA: at least for the group WUS in the same WUS resource, legacy WUS with phase shifts is selected as group WUS sequence design according to wgroup(m’) = w(m’) exp(j2πgm/G) Proposal 4: 9 phase shifts are introduced to support a maximum 8 UE groups and 1 additional common WUS - G=132, g=14(NIDUEgroup+1) with NIDUEgroup=0,…7 and g=126 as the common WUS Proposal 5: Consider using different scrambling sequences (i.e., change c_init of scrambling) for group WUS sequences in different WUS resources associated with the same PO. For group WUS configuration: Proposal 6: Each UE monitors up to 2 WUS sequences. - Configuration of a common WUS for a subset of UE groups is not supported. - Proposal 7: Consider narrowband selection for group WUS configuration. - Proposal 8: Support weighting factor configuration for UE distribution amongst the different WUS resources configured for a narrowband. - Proposal 9: Confirm the WA: - UE assumes the transmit power for Rel-16 WUS sequence is same as that of Rel-15 WUS sequence. - Maximum WUS duration for Rel-16 WUS sequence is same as that of Rel-15 WUS sequence |
| R1-1909167 | NTT DOCOMO, INC. | Proposal 1: Confirm the following working assumption. - UE assumes the transmit power for Rel-16 WUS sequence is same as that of Rel-15 WUS sequence. - Maximum WUS duration for Rel-16 WUS sequence is same as that of Rel-15 WUS sequence Proposal 2: Support to configure up to four WUS resource by using both time and frequency domain multiplexing Proposal 3: Support group common WUS for waking-up all Rel-16 WUS for waking up a subset of UE groups is not supported. |

# Appendix: Agreements in previous meetings

## RAN1#97

Agreement

Support the following options for WUS resource configuration

- Up to 2 orthogonal resources may be configured in time domain.

- Up to 2 orthogonal resources may be configured in frequency domain

- FFS: both options can be configured simultaneously to have up to 4 orthogonal WUS resources

Maximum total number of WUS resources is no larger than 4 including the legacy WUS resource

For further discussion

Different WUS sequences associated with the same PO are used for UE groups regardless of whether the UE groups are in the same or different WUS resources.

Working Assumption

- UE assumes the transmit power for Rel-16 WUS sequence is same as that of Rel-15 WUS sequence.

- Maximum WUS duration for Rel-16 WUS sequence is same as that of Rel-15 WUS sequence

Agreement

Each UE monitors up to X WUS sequences.

- Value of X will be selected between 2 and 3

  - X=3 can only be supported if a common WUS for a subset of UE groups for service-based grouping is supported

## RAN1#96bis

Agreement

For evaluation purpose and down-selection on multiplexing schemes in RAN1#97:

- UE assumes the same WUS max duration and same transmit power for Rel-15 WUS and Rel-16 WUS

Agreement

Down-select one of the following options until RAN1#97 based on evaluation results including power saving gain, usage of resources, etc

- Up to 2 orthogonal WUS resources may be configured in time domain

- Up to 2 orthogonal WUS resources may be configured in frequency domain

- Up to 2 orthogonal WUS resources may be configured per dimension (up to 4 orthogonal WUS resources in total)

- Up to 2 orthogonal WUS resources may be configured either in time or frequency domain (only one of the two can be configured)

Determine in RAN1#97 whether legacy WUS resource is counted as one of the configured WUS resource(s).

Agreement

Rel-15 WUS and Rel-16 WUS can be configured on the same legacy WUS resource via SI

- FFS explicitly or implicitly

- Same WUS parameters are assumed for both Rel-15 and Rel-16 WUS in case both are on the same legacy WUS resource

Agreement

- Per default, all gaps use the same group WUS configuration regarding number of groups and group WUS resource allocation.

- Optionally, eDRX gap(s) may be configured individually if separate from the DRX gap.

Agreement

If the group WUS resource is configured to be shared by Rel-15 WUS and Rel-16 WUS, a common WUS for all the group WUS UEs in the same WUS resource can be configured to be legacy WUS or a non-legacy WUS.

## RAN1#96:

Agreement

UE-group MWUS sequence is same as UE-group NWUS per RB and the same sequence is repeated in two PRBs.

Agreement

- A UE is required to monitor WUS(s) in only one WUS (time/frequency) resource location.

Agreement

- Single-sequence CDM is used in the same WUS resource if more than one UE group is configured to monitor the WUS resource.

- Select one among the following:

  - Up to 2 orthogonal WUS resources may be configured in time domain

  - Up to 2 orthogonal WUS resources may be configured in frequency domain

  - Up to 2 orthogonal WUS resources may be configured per dimension (up to 4 orthogonal WUS resources in total)

  - Up to 2 orthogonal WUS resources may be configured either in time or frequency domain (only one of the two can be configured)

- FFS: whether one of the WUS resources is configured to be shared with legacy WUS

Agreement

If configured, a common WUS is used to wake up all group WUS UEs monitoring the same WUS (time-frequency) resource if more than one UE group occupies the WUS resource.

- FFS: Whether the above is also applicable for Rel-15 WUS UEs

- FFS: Whether to additionally support the feature of waking up a subset of all WUS UE groups

## RAN1#95:

Agreement

For multiplexing between Rel-16 UE-group WUS and Rel-15 WUS, further evaluate and down select among the following options

- TDM

- FDM

- single-seq CDM

- single-seq CDM+TDM

- single-seq CDM+FDM

- FFS whether legacy WUS is the common WUS for all new UEs or only a part of new UEs.

Agreement

For multiplexing between different Rel-16 UE-group WUS, further evaluate and down select among the following options

- single-seq CDM

- FDM

- single-seq CDM+TDM

- single-seq CDM+FDM

Note: At least the maximum number of UE groups should be considered.

Agreement

The number of UE groups is configurable and broadcasted in SIB.

- FFS: Further details on the number of UE groups. For example, whether it is per PO or per gap configuration of a PO

Agreement

UE group ID is used as a parameter to generate WUS UE group sequence(s).

Agreement

One group WUS is designed as a single sequence

- Agreement

Further study false detection (cross/auto correlation) performance properties for the following designs:

- legacy WUS + cover codes,

- legacy WUS + shifted scrambling codes,

- legacy WUS + phase shift + cover code + scrambling bits

  - Including combinations of phase shift, cover code, and/or scrambling bits

Other designs are not precluded.

Agreement

Rel-16 group WUS uses the same gap configurations as for Rel-15 legacy WUS except for differences from possible TDM.

- No new gap higher layer signaling will be introduced for TDM

In RAN1 #94, we have following agreements:

Agreement

UE-group WUSs are only multiplexed in the same NB as associated PO

- FFS TDM/FDM/CDM for UE-group MWUS multiplexing

Agreement

- UE-group MWUS is supported based on eNB’s and UE’s capability.

- Whether the network supports UE-group MWUS is done by higher layer signalling.

- FFS: The number of UE groups is configured by SIB.

- Note that the UE-group MWUS is UE optional

Agreement

Rel-16 UE-group MWUS sequence should consider at least

- Fallback to legacy UE behavior

- Inter-cell interference randomization

- UE group ID for different UE-group MWUS

- Reuse of Rel-15 sequences is not precluded

- Effect of sequence detection on UE complexity

Agreement

Study the RAN1 consequence of UE-grouping on the following basis:

- UE ID

- Coverage

- DRX/eDRX

- Gap configuration

- Services

## RAN1#94:

Agreement

UE grouping is based on at least UE ID or some function of UE ID

Agreement

The legacy UE should not be prevented from using legacy WUS even in the case of Rel-16 group WUS is enabled

- Performance impact on legacy WUS should be carefully considered

Agreement

Group WUS is based on at least legacy WUS and UE-group ID.

Agreement

Configuration of group WUS is at least signaled in SI

Agreement

A Rel-16 group WUS capable UE shall also be capable of Rel-15 legacy WUS