3GPP TSG RAN WG1 Meeting #98																		    R1-1909526

Prague, Czech Republic, 26th August– 30th August 2019

Agenda Item:		7.2.3.2

Source:				Qualcomm Incorporated

Title: 		Summary of section 7.2.3.2 – Extension of RACH occasions and periodicities for backhaul RACH resources

Document for:		Discussion and decision

### Introduction

This document provides a summary of the proposals for section 7.2.3.2 of the agenda on the subject of RACH extensions for IAB and the associated discussion points and offline agreements.

### Discussion points

Based on the contributions there is one main remaining discussion point:

- Clarification on whether backhaul specific RACH configurations should be made available for initial access

Based on the discussion in the offline session on August 27th the consensus is that indeed support for IAB-specific backhaul RACH configurations for initial access was already defined as per the agreements in RAN1 #97. Neverthless for avoidance of doubt the following agreement could be added:

Offline agreement 1:

IAB-specific backhaul RACH configurations are also available for intial access of IAB nodes.

Other points in the contributions relate to signalling aspects for which we had already agreed in RAN1 #97 it would be up to RAN2 to discuss and decide.

Companies input:

| Huawei R1-1908038 | Observation 1: RMSI size with IAB RACH configuration can satisfy the maximum TB size limitation in general configuration scenarios, but it will exceed this value in some configuration scenarios. Observation 2: IAB Backhaul RACH configurations broadcasting along with every RMSI has high overhead. Observation 3: Short broadcasting period of RMSI with IAB RACH configuration may be unnecessary. Proposal: Some enhancements should be considered to reduce the overhead of broadcasting backhaul RACH configurations. - Option 1: Introduce a flag to indicate the presence of IAB-specific RACH configurations - Option 2: Transmit the IAB-specific RACH configurations in OSI |
| --- | --- |
| Samsung R1-1908472 | Observation: the energy waste and processing delay worsen will be created for legacy rel-15 UE if the Rel-15 and rel-16 feature specific information are contained in the same single RMSI PDSCH. Proposal: solutions targeting for separating the new feature-specific configuration from the legacy Rel-15 configuration should be pursued |
| Intel R1-1908630 | Proposal 1: Consider the following two alternative on RACH configuration for an IAB node: - Alt 1: Explicitly signal the following information to an IAB node: - PRACH configuration index (0-255); - scaling factor λ (if any); - time offset in frames Δy (if any); - time offset in subframes/slots Δs (if any). - Alt 2: Pre-define scaling factor and time offsets for each PRACH configuration index. Signal PRACH configuration index as in Rel-15. No explicit signalling on scaling factor and slot/subframe offsets. |
| Ericsson R1-1909025 | Proposal 1 IAB-specific RACH configurations can have a different preamble format, compared to the RACH configuration to be used by Rel-15 UEs for initial access in the same cell. Proposal 2 Information on IAB-specific RACH configuration, including a preamble format that can be different from the one used by Rel-15 UEs for initial access in the same cell, is broadcasted. Proposal 3 Information on IAB-specific RACH configuration, including a preamble format that can be different from the one used by Rel-15 UEs for initial access in the same cell, is included in SIB1 or a new IAB-specific SIB (to be determined by RAN2). |
| ZTE, Sanechips R1-1909146 | Proposal 1: At least the following IEs can be provided separately from common RACH configuration for access UEs. - PRACH configuration index; - prach-RootSequenceIndex; - zeroCorrelationZoneConfig; - restrictedSetConfig; - ssb-perRACH-OccasionAndCB-PreamblesPerSSB |

### References

Agreements from RAN1 #97:

Agreements:

The signaling design for the IAB specific backhaul RACH configurations is up to RAN2.

RAN1 has the following recommendations:

- The ability to configure the IAB specific backhaul RACH configuration additionally to the Rel 15 RACH configuration for access UEs is supported.

  - If the IAB specific RACH configuration is not provided, RAN1 assumes that IAB node will use the configured Rel 15 RACH configuration for IAB node initial access.

- Capability to configure IAB specific backhaul RACH configurations need to be provided for both CBRA & CFRA.

Agreements:

The introduction of scaling factor for PRACH configuration period also scales the SSB-RACH association period and association pattern period accordingly.

Assuming the scaling factor to be λ, for IAB nodes table 8.1-1 of 38.213 gets modified to the following:

Mapping between scaled PRACH Configuration Period (x * λ) and SSB to PRACH Occasion Association Period

| Scaled PRACH configuration period (ms) | Association period |
| --- | --- |
| 10 | {1, 2, 4, 8, 16, 32, 64} |
| 20 | {1, 2, 4, 8, 16, 32} |
| 40 | {1, 2, 4, 8, 16} |
| 80 | {1, 2, 4, 8} |
| 160 | {1, 2, 4} |
| 320 | {1, 2} |
| 640 | {1} |

The definition of the SSB-RACH association pattern period for IAB nodes should be updated as follows: An association pattern period includes one or more association periods and is determined so that a pattern between PRACH occasions and SS/PBCH blocks repeats at most every 640 msec.

Agreements from RAN1 #96bis:

Agreements:

The periodicity of a backhaul RACH configuration in frames takes the form x_iab = x * λ where:

- x is the periodicity of an existing RACH configuration,

- λ is a scaling factor taking values in {1, 2, 4, 8, 16, 32, 64} subject to the constraint x_iab ≤ 64.

Agreements:

The frame containing backhaul ROs is identified by (nSFN mod x_iab) = ((y + Δy) mod x_iab) where Δy denotes a time offset in frames taking values in the range from 0 to x_iab – 1.

Agreements:

The subframe (slot) number for a RO of a backhaul RACH configuration is identified by (Sn + Δs,) mod L, where:

- Sn is the subframe (slot) number of an existing RACH configuration,

- Δs denotes a time offset in subframes (slots) taking values in the range from 0 to L – 1, where L is the number of subframes (slots) in a frame.

NOTE: The usage of the terms subframe or slot is meant to align to the terminology used in the existing RACH configuration tables in TS 38.211, e.g. subframe in Tables 6.3.3.2-2 and 6.3.3.2-3, and slot in Table 6.3.3.2-4.

Agreements:

The validity of ROs for backhaul RACH configurations is regulated by the rules defined in Rel-15 for existing RACH configurations.

Agreements:

Partial overlap of ROs between RACH configurations used in two adjacent links (upstream towards the parent and downstream towards the children from an IAB node perspective) is allowed.