3GPP TSG RAN WG1 Meeting #98	R1-1909532

Prague, Czech Republic, August 26 – 30, 2019

Agenda Item:	7.2.5.1

Source:	Huawei, HiSilicon, CAICT

Title:	Discussion on performance evaluation for NTN

Document for:	Discussion and Decision

Introduction

In RAN1 #97, the following agreements related to NTN SLS assumptions were reached [1]:

Agreement:

A wrap around mechanism should be considered as a baseline for single satellite simulation for intra-satellite interference modeling based on additional bore-sight beam directions which should be computed based on the methodology captured in Table X.4.

- FFS: Details of the wrap around mechanism

Agreement:

Details on the wrap around methodology used should be provided by the companies together with their simulation results.

In this contribution, we discuss the remaining issues of NTN system level simulation assumptions for both single-satellite and multiple-satellite cases, and then provide some preliminary results.

Single-satellite system level simulation assumptions and results

Wrap around methodology

Wrap around method for terrestrial networks

Terrestrial system-level simulation frameworks utilize wrap around method to reduce boundary effects and provide an accurate evaluation of system performance [2]. In the conventional wrap around mechanism, the network is extended to include six additional copies of the original hexagonal cluster. Every cell in the extended clusters has the same properties as its corresponding cell in the central cluster, except that they are at different geographic locations. Specifically, wrap around implementation involves finding the path distance between the UE and the nearest version of the cell whose signal is being received. As shown in Figure 2.1, the original hexagonal cluster (i.e., cluster 0) locates in the center while the other six copies (cluster 1 to cluster 6) are attached symmetrically on six sides. The path distance  from a reference UE at coordinate  to a cell at coordinate  is denoted by the minimum of the seven related distances:

(1)

where is the cell radius. As illustrated in Figure 2.1, to compute the path distance from cell 11 to the reference UE, a total of 7 distances plotted in line are calculated, and the distance from cell 11 in cluster 2 to the reference UE (plotted in blue line) is finally chosen as the path distance .

The conventional wrap around method exhibits the following properties. Firstly, LTE/NR employs full frequency reuse, and thus all simulated cells share the same frequency band. Consequently, all the cells with wrap around operate in the same frequency band. Secondly, as shown in Figure 2.1, for each reference UE located in the original hexagonal cluster, the closest replica of the 19 cells (cells numbered in red) are chosen, with one cell as the serving cell and the other 18 cells as the interfering cells.

Observation 1: The conventional wrap around method is originally applied to the full frequency re-use case.

Observation 2: In the conventional wrap around method, 2-tier interferences (i.e., interferences from 18 interfering cells) are modelled for each reference UE.

Figure 2.1. Conventional wrap-around method with full frequency reuse.

Figure 2.2. Conventional wrap-around method with 3-color frequency reuse.

However, directly applying the conventional wrap around method to NTN may be problematic, due to the fact that NTN generally adopts N-color frequency reuse pattern (e.g., 3-color or 4-color frequency reuse) instead of the full frequency reuse used in LTE/NR terrestrial networks. Simply copying the frequency pattern in the hexagonal cluster to the six additional clusters is not realistic from the network deployment perspective, since it will induce strong interference for certain cells as shown in Figure 2.2, and thus significantly degrades the system performance. Furthermore, if frequency reuse is applied, the conventional wrap around cannot guarantee appropriate 2-tier interferences for a reference UE. For example, as shown in Figure 2.2, if 3-color frequency reuse is employed, interferences from only 6 interfering cells (numbered in red) are finally computed. In this case, the calculated interference is unrealistic, as a neighbor cell 12 acts as an interfering cell for the reference UE. While in reality, if frequency reuse is adopted, two neighboring cells will use different frequency bands and do not generate mutual interference. Therefore, the conventional wrap around method should not be directly employed for single satellite simulation due to the unrealistic interference modeling problem.

Observation 3: NTN generally adopts frequency reuse pattern (e.g., 3-color or 4-color frequency reuse) instead of full frequency reuse pattern used in terrestrial networks.

Observation 4: The conventional wrap around method should not be directly used for single satellite simulation due to the unrealistic interference modeling problem.

Wrap around methods for NTN

In this part, we propose two possible wrap around options for NTN, namely extended wrap around method and NTN-specific wrap around method.

- Wrap around option 1: Extended wrap around method

A natural extension to the conventional wrap around for NTN is to first execute conventional wrap around in the UV plane, and then properly tune the frequency of beams/cells in six additional clusters to form a complete frequency reuse pattern. The details for wrap around option 1 are as follows:

- Step 1: Apply conventional wrap around method in the UV plane;

- Step 2: Tune the frequency of each cell in clusters 1 to 6 using the principle below:

  - Reuse the same frequency if the distance between a cell and its closest co-frequency cell equals to sqrt(FRF)*ABS, where FRF is the frequency reuse factor, and ABS is the adjacent beam spacing.

Figure 2.3. Extended wrap-around method with 3-color frequency reuse.

Clearly, the extended wrap around method for NTN only makes marginal changes to the existing wrap around method. It is easy to implement in the system level simulation platform. Nonetheless, it can lead to the inaccurate interference modeling problem. To verify the interference modeling effect, we use

Figure 2.4. Interfering modeling results for simulation configuration C2.

simulation configuration C2 (listed in Table 2.3 hereafter) as a baseline and provide the geometry calibration results for the extended wrap around method. As can be seen in Figure 2.4, the 2-tier and 3-tier interference modeling exhibits a very similar performance. However, there is a non-negligible interference modeling gap between the extended wrap around method and 2-tier/3-tier interference. The performance gap in this simulation configuration is around 1-2dB. To this end, if the extended wrap around option is adopted, it is recommended to take additional measures to solve the underestimated interference problem. For instance, a simple interference scaling scheme is to multiply the interference by an appropriate scaling factor. However, determining the exact value of the scaling factor for each simulation configuration is FFS.

Observation 5: 2-tier interference can provide an approximate interference modeling.

Observation 6: If frequency reuse is applied, the extended wrap around method cannot model 2-tier interferences and leads to underestimated interference for a reference UE.

- Wrap around option 2: NTN-specific wrap around method

To solve the underestimated interference problem, the NTN-specific wrap around method for single satellite case is proposed. In this method, 4-tier beams (i.e., 108 beams) are first added outside the reference 19-beam layout in the UV plane, and then the configurations (e.g., frequency and UE dropping) of beams/cells in six additional clusters are adjusted. The details for wrap around option 2 are as follows:

- Step 1: Construct the initial 19-beam layout in the UV plane;

- Step 2:  Add 4-tier beams (i.e., 108 beams) outside the initial 19-beam layout in the UV plane. One possible configuration (e.g., frequency and UE dropping) for each beam in the added 4-tier is shown below:

  - For frequency configuration, apply the previous principle: reuse the same frequency if the distance between a cell and its closest co-frequency cell equals to sqrt(FRF)*ABS, where FRF is the frequency reuse factor, and ABS is the adjacent beam spacing.

  - For other configurations (e.g., UE dropping), copy the corresponding configuration from the same numbered beam in the initial 19-beam layout. One possible numbering scheme for beams in the added 4-tier is shown in Figure 2.4. Other numbering schemes can be FFS.

- Step 3: For interference modeling, 2-tier interferences (interferences from 18 interfering beams) are finally calculated;

Figure 2.4. Proposed NTN-specific wrap around method with 3-color frequency reuse.

The underlying reason for adopting the wrap around option is twofold. On the one hand, it resembles a realistic single satellite complete beam layout and thus provides an accurate evaluation of system performance. On the other hand, it well exhibits an approximate 2-tier interference modeling effect for different frequency reuse patterns (i.e., full frequency reuse, 3-color reuse pattern and 4-color reuse pattern) with acceptable simulation complexity. An example is provided in Figure 2.4. However, how to implement the numbering scheme for wrap around option 2 needs further study.

Proposal 1: For wrap around 4-tier beams (i.e., 108 beams) are added outside the initial 19-beam layout in the UV plane for single satellite case.

Observation 7: The proposed NTN-specific wrap around method can guarantee 2-tier interferences for different FRF.

Proposal 2: The numbering scheme in NTN-specific wrap around method needs further study.

Remaining simulation assumptions

Target elevation angle

Table 2.1: Beam layout parameters for single satellite simulation (Table X.6 in [2])

| Scenario | Scenario A | Scenario C2/D2 |
| --- | --- | --- |
| Carrier frequency | S-band : 2 GHz Ka-band : 20 GHz for DL | S-band : 2 GHz Ka-band : 20 GHz for DL |
| Adjacent beam spacing (ABS) on UV plane | S-band : ABS = 0.0061 Ka-band : ABS = 0.0027 | S-band : ABS = 0.0668 Ka-band : ABS = 0.0267 |
| Satellite location | Any position on the geostationary orbit | Any position on the LEO orbit |
| Central beam center elevation angle target | Baseline : - Case 1 : Not considered - Case 2 : 45 degrees | Baseline : - Case 1 : 90 degree - Case 2 : FFS |
| Central beam bore sight direction coordinates in UV plane | Baseline : - Case 1 : Not considered - Case 2 : (0.107,0) | Baseline : - Case 1 : (0,0) - Case 2 : FFS |
| Gateway direction coordinates in UV plane | Baseline : Same as central beam bore sight direction coordinates in UV plane Note : Not needed for calibration |  |

Following the agreements in [1], a set of effective single satellite simulation configurations are first identified based on the combination of six parameters listed below:

- Satellite orbit type: GEO, LEO-1200, and LEO-600

- Band: S-band, and Ka-band

- Central beam bore sight direction: Case 1(nadir point), and Case 2(elevation angle target)

- UE type: Handheld, VSAT, and Others

- Frequency re-use factor: 1, 3, and 2 if polarization re-use is enabled

- Polarization re-use: Disabled, and Enabled

Specifically, as listed in Table 2.1, the central beam bore sight directions for Case 2 in Scenario C2/D2 are not decided. The main purpose for Case 2 is to evaluate the performance of cells/beams far from the satellite nadir point (i.e., cells with low elevation angle).  However, when applying current hexagonal beam layout in the UV plane, as plotted in Figure 2.5, some edge beams will first reach 10 degree elevation angle and exhibit a very different beam shape compared with other central beams in the geodetic plane. The single satellite complete beam layout seems not reasonable from a practical satellite network deployment viewpoint.  Furthermore, it can be seen from Table 2.2 that if Case 2 is set to about 60 degrees, the minimum elevation angles of all beams after wrap around will approach 10 degrees. In other words, we can only derive effective performance of cells with minimum elevation angle larger than 60 degrees in the single satellite simulation. As can be inferred from Figure. 2.6, there is no much CINR performance difference between Case 1 (i.e., 90 degrees) and Case 2 (60 degrees). Therefore, it is not necessary to still simulate Case 2 in Scenario C2/D2 for single satellite SLS from the perspective of reducing simulation complexity.

Observation 8: If Case 2 is set to 60 degrees, the minimum elevation angles of all beams after wrap around will approach 10 degrees.

Proposal 3: The case 2 in Scenario C2/D2 should be studied in the multiple-satellite scenario instead of the single-satellite scenario.

Table 2.2: Minimum elevation angles for all beams after wrap around

| Scenario | Wrap around option | Case 2 (degrees) | Minimum elevation angles (degrees) |
| --- | --- | --- | --- |
| LEO-600 | Wrap around option 1 | 60 | 10.6 |
|  | Wrap around option 2 | 60 | 13.4 |
| LEO-1200 | Wrap around option 1 | 63 | 11.8 |
|  | Wrap around option 2 | 63 | 14.5 |

Figure 2.5. LEO S band beam layout in the geodetic plane.

Figure 2.6. LEO600 S band CINR results with different target elevation angles.

Besides, note that some parameters are correlated, e.g., the Handheld UE type is only present in the S-band. By organically combing those parameters, a total of 18 simulation settings are derived as listed in Table 2.3. Those simulation configurations can be set as the start point for single satellite SLS.

Proposal 4: Adopt simulation configurations in Table 2.3 as the start point for single satellite SLS.

Table 2.3.  System level simulation settings for single satellite cases

| Configuration | S-band | Ka-band |
| --- | --- | --- |
| GEO | *C1:GEO_S-band_Case2_Handheld_1_Disable *C2:GEO_S-band_Case2_Handheld_3_Disable | C3:GEO_Ka-band_Case2_VSAT_1_Disable C4:GEO_Ka-band_Case2_VSAT_2_Enable *C5:GEO_Ka-band_Case2_Others_1_Disable *C6:GEO_Ka-band_Case2_Others_3_Disable |
| LEO-1200 | C7:LEO-1200_S-band_Case1_Handheld_1_Disable C8:LEO-1200_S-band_Case1_Handheld_3_Disable | C9:LEO-1200_Ka-band_Case1_VSAT_1_Disable C10:LEO-1200_Ka-band_Case1_VSAT_2_Enable C11:LEO-1200_Ka-band_Case1_Others_1_Disable C12:LEO-1200_Ka-band_Case1_Others_3_Disable |
| LEO-600 | C13:LEO-600_S-band_Case1_Handheld_1_Disable C14:LEO-600_S-band_Case1_Handheld_3_Disable | C15:LEO-600_Ka-band_Case1_VSAT_1_Disable C16:LEO-600_Ka-band_Case1_VSAT_2_Enable C17:LEO-600_Ka-band_Case1_Others_1_Disable C18:LEO-600_Ka-band_Case1_Others_3_Disable |
| Note 1: * can be optional. |  |  |

Satellite EIRP density

Another observation from Table 2.4 is that satellite EIRP density is defined therein. However, when frequency reuse factor is adopted, it is unclear whether the value of satellite EIRP density changes or not. Take GEO S-band as an illustrative example, the satellite EIRP density is 59 dBW/MHz for the total 30 MHz bandwidth. When a frequency reuse factor of 3 is applied, each beam will finally occupy 10MHz. In this case, it should be clarified that what is the exact method to calculate the total beam power:

- Same EIRP density method: No matter how the bandwidth changes, the EIRP density still remains the same. That is, the available beam power becomes 59+10*log10(10) dBW if FRF=3 is used.

- Same total beam power method: The total power for a beam keeps the same even if the beam bandwidth changes, i.e, the beam EIRP density changes. Under this assumption, the total beam power is also 59+10*log10(30) dBW even if FRF=3 applies.

Indeed, to have a fair performance comparison (e.g., throughput) for different FRF, it is suitable to adopt the same total beam power method. Otherwise, the available beam power for different FRF is different, and thus the performance comparison will be inefficient to some extent. Besides, a satellite RF device typically operates at the saturation point, and thus the total power will be used altogether for transmission.

Observation 9: It is unclear whether the value of satellite EIRP density changes or not when different FRF is used.

Proposal 5: Adopt the same total beam power method when different FRF is used.

Table 2.4: Set-1 Satellite parameters for System Level Simulations (Adopted from Table X.1 [1])

| Satellite orbit |  | GEO | LEO-1200 | LEO-600 |
| --- | --- | --- | --- | --- |
| Satellite altitude |  | 35’786 km | 1’200 km | 600 km |
| Satellite antenna pattern |  | Section 6.4.1 in TR 38.811 | Section 6.4.1 in TR 38.811 | Section 6.4.1 in TR 38.811 |
| Payload characteristics for DL transmissions |  |  |  |  |
| Equivalent satellite antenna aperture (Note 1) | S-band (i.e. 2 GHz) | 22 m | 2 m | 2 m |
| Satellite EIRP density |  | 59 dBW/MHz | 40 dBW/MHz | 34 dBW/MHz |
| Satellite Tx max Gain |  | 51 dBi | 30 dBi | 30 dBi |
| Satellite beam diameter (Note 2) |  | 250 km | 90 km | 50 km |
| Equivalent satellite antenna aperture (Note 1) | Ka-band (i.e. 20 GHz for DL) | 5 m | 0.5 m | 0.5 m |
| Satellite EIRP density |  | 40 dBW/MHz | 10 dBW/MHz | 4 dBW/MHz |
| Satellite Tx max Gain |  | 58.5 dBi | 38.5 dBi | 38.5 dBi |
| Satellite beam diameter (Note 2) |  | 110 km | 40 km | 20 km |
| Payload characteristics for UL transmissions |  |  |  |  |
| Equivalent satellite antenna aperture (Note1) | S-band (i.e. 2 GHz) | 22 m | 2 m | 2 m |
| G/T |  | 19 dB K-1 | 1.1 dB K-1 | 1.1 dB K-1 |
| Satellite Rx max Gain |  | 51 dBi | 30 dBi | 30 dBi |
| Equivalent satellite antenna aperture (Note1) | Ka-band (i.e. 30 GHz for UL) | 5 m | 0.5 m | 0.5 m |
| G/T |  | 28 dB K-1 | 13 dB K-1 | 13 dB K-1 |
| Satellite RX max Gain |  | 62 dBi | 42 dBi | 42 dBi |
| Note 1: This value is equivalent to the antenna diameter to be used in Sec. 6.4.1 of TR 38.811. Note 2: This beam size refers to the Nadir pointing of the satellite |  |  |  |  |

Calibration results

In this part, we provide calibration results (i.e., coupling loss and geometry) for 18 simulation configurations in the single satellite context. Two different wrap around options are employed. Note that S_4 = 0.7 is used in all simulation configurations. Besides, the satellite locations for GEO, LEO-1200 and LEO-600 are set to (42164, 0, 0), (7578, 0, 0) and (6978, 0, 0), respectively.

The coupling loss calibration results for all the 18 simulation configurations can be found in Figures A1-A18 in the Appendix. Note that different wrap around options have no impacts on the coupling loss value. The minimum value of coupling loss is present in configurations C7 and C8, and is approximately equal to -146.5dB. While the maximum value of coupling loss is present in configurations C15 and C16, and is approximately equal to -96dB.

Observation 10: The value of coupling loss for NTN ranges approximately from -146.5dB to -96dB.

Meanwhile, the geometry calibration results for all the 18 simulation configurations under two wrap around methods are shown in Figures A19-A54 in the Appendix. The minimum value of geometry is present in configurations C5 and is approximately equals to -18.5dB. While the maximum value of geometry is present in configurations C10 and C16, and is approximately equal to 10dB. Besides, it can be seen that for a same simulation configuration, wrap around option 2 obtains a smaller geometry than wrap around option 1, and the performance gap varies with different configurations.

Observation 11: The value of geometry for NTN ranges approximately from -18.5dB to 10dB.

Observation 12: There is a gap in geometry performance between two wrap around options.

Preliminary throughput results

Herein, we provide preliminary throughput results for different simulation configurations using wrap around option 1 in Figures 2.7 and 2.8. Notably, the Others type UE achieves nearly zero throughput in all scenarios, due to the very low SINR. This fact can be further verified by the geometry calibration results. Thus, the others type UE characteristics defined in Table X.3 in [1] needs double check.

Observation 13: The Others type UE achieves nearly zero throughput in all scenarios.

Proposal 6: Double check and adjust the Others type UE characteristics defined in Table X.3 in [1].

Figure 2.7. Throughput results with approximate 20% RU under wrap around option 1.

Figure 2.8. Throughput results with approximate 50%~60% RU under wrap around option 1

Multiple-satellite system level simulation assumptions and results

In this section, NTN system level simulation platform for multiple-satellite cases is studied. First, the consideration of constellation design is discussed and reference constellations are proposed. Second, the beam layout and frequency reuse factor for multi-satellite cases are discussed. At last, some preliminary simulation results are given.

Simulation Assumptions

Reference constellations

In throughput evaluation, it is important to consider the inter-satellite interferences for UEs located at satellite edge. In order to simulate the actual conditions of the multiple-satellite system, the reference constellation for different operation scenarios should first be determined. Otherwise, an oversimplified multi-satellite configuration will result in unrealistic results. In this section, the basic considerations for satellite constellation, beam layout and other parameters are proposed for system calibration.

Observation 14: Reference satellite constellations for different scenarios are required for multi-satellite simulation.

In general, in order to achieve an NTN system with global coverage, several constellation parameters should be considered, such as satellite altitude, half power beam width (HPBW), minimum elevation angle and inclination. According to the previous agreement [1], the satellite altitudes for LEO are 600km and 1200km, respectively, and the GEO altitude is 35786 km. Besides, the HPBW of beams for different scenarios can be calculated using the parameters in [1].

The minimum elevation angle determines the maximum satellite service areas. Table 3.1 illustrates the path loss for different elevation angles. For lower minimum elevation angle, the path loss difference between nadir point and edge point is larger. Table 3.1 illustrates the path loss differences between different elevation angles. For LEO 600 km scenarios, the path loss of 10 degree is 10dB larger than nadir point while the difference for 30 degree is only 5dB. For LEO 1200 km scenarios, the path loss of 10 degree is 8dB larger than nadir point while the difference for 30 degree is only 4dB. For GEO scenarios, the path loss differences for different elevation angles are less than 1dB. To avoid large pass loss variation, the minimum elevation angle should be set to 30 degree for both LEO 600km and LEO 1200km scenarios, and 10 degree for GEO scenario.

Table 3.1 Path loss (dB) for different elevation angles

| Path loss (dB) for different elevation angles |  |  |  |
| --- | --- | --- | --- |
| S band (2GHz) |  |  |  |
| Elevation degree | 10 | 30 | 90 (nadir) |
| LEO 600km | 164.2 | 159.1 | 154 |
| LEO 1200km | 168.4 | 164.5 | 160.1 |
| GEO | 190.6 | 190.2 | 189.5 |
| Ka band (20GHz DL) |  |  |  |
| Elevation degree | 10 | 30 | 90 (nadir) |
| LEO 600km | 184.2 | 179.1 | 174 |
| LEO 1200km | 188.4 | 184.5 | 180.1 |
| GEO | 210.6 | 210.2 | 209.5 |

Observation 15: The path loss increases by 4-5 dB when the elevation angle decreases from 90 degree to 30 degree for LEO scenarios, and varies within 1 dB when elevation angle changes for GEO scenarios.

Proposal 7: To avoid large pass loss variation, the minimum elevation angle should be set to 30 degree for both LEO 600km and LEO 1200km scenarios, and 10 degree for GEO scenario, for the satellite reference constellation design.

To obtain the full coverage of the whole earth area, including north and south poles, the inclination is set to 87.5 degrees. Table 3.2 and Table 3.3 show the reference constellations for Set-1 and Set-2, respectively. Those parameters, such as number of satellites per orbit (Ns), number of orbit (Norbit) and number of beams per satellite (Nbeam) are well calculated and optimized to make sure any location on earth can be covered by at least one beam. The HPBW of beam for different scenarios are listed in Table A.1 and Table A.2 in the Appendix.

Table 3.2 Parameters for reference constellation of set-1

| Frequency | Satellite altitude (km) | Parameter |  | Frequency | Satellite altitude (km) | Parameter |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| S-band | LEO 600 | Ns | 30 | Ka-band | LEO 600 | Ns | 30 |
|  |  | Norbit | 17 |  |  | Norbit | 17 |
|  |  | Nbeam | 469 |  |  | Nbeam | 2977 |
|  | LEO 1200 | Ns | 17 |  | LEO 1200 | Ns | 17 |
|  |  | Norbit | 10 |  |  | Norbit | 10 |
|  |  | Nbeam | 397 |  |  | Nbeam | 2437 |
|  | GEO | Ns | 4 |  | GEO | Ns | 4 |
|  |  | Norbit | 4 |  |  | Norbit | 4 |
|  |  | Nbeam | 1801 |  |  | Nbeam | 8911 |

Table 3.3 Parameters for reference constellation of set-2

| Frequency | Satellite altitude (km) | Parameter |  | Frequency | Satellite altitude (km) | Parameter |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| S-band | LEO 600 | Ns | 30 | Ka-band | LEO 600 | Ns | 30 |
|  |  | Norbit | 17 |  |  | Norbit | 17 |
|  |  | Nbeam | 127 |  |  | Nbeam | 469 |
|  | LEO 1200 | Ns | 17 |  | LEO 1200 | Ns | 17 |
|  |  | Norbit | 10 |  |  | Norbit | 10 |
|  |  | Nbeam | 91 |  |  | Nbeam | 397 |
|  | GEO | Ns | 4 |  | GEO | Ns | 4 |
|  |  | Norbit | 4 |  |  | Norbit | 4 |
|  |  | Nbeam | 547 |  |  | Nbeam | 1519 |

All the constellations parameters for different scenarios have been provided in Table 3.2 and Table 3.3, respectively. Take LEO 600km in S band for example, the reference constellation is shown in Figure 3.1.

Figure 3.1 Satellite constellation of LEO-600km in S band

Proposal 8: To simplify the constellation design, the constellations should be walker star constellation and the inclination should be set to 87.5 degree.

Proposal 9: Reference constellation defined in Table 3.2 can be considered for Set-1 satellite parameters in multiple-satellite SLS.

Proposal 10: Reference constellation defined in Table 3.3 can be considered for Set-2 satellite parameters in multiple-satellite SLS.

There are too many satellites and it will take too much time to have full constellation simulation. Therefore, it is suggested to focus on partial geographical area or partial satellites in the simulation.

Proposal 11: A subset of satellites (e.g., 3 or 7 satellites) in the reference constellations are sufficient for multiple-satellite system level simulation.

Beam layout definition

Hexagonal beam layout can be re-used for multiple satellite simulations. The detailed beam layout definition is listed in Table 3.4.

Table 3.4: Beam layout definition for multi satellite simulation

| Scenario | Scenario A, C2 and D2 |
| --- | --- |
| Beam layout definition | Baseline : Hexagonal mapping of the beam bore sight directions on UV plane defined in each satellite reference frame. |
| Number of beams | Case1: N-beam layout for evaluating the whole satellite’s coverage (N is the Nbeams for each scenarios, for example, N=469 for LEO 600 km S band set-1 scenario), with a wrap-around mechanism (i.e., 6 neighbor satellites with full beams surrounding target satellite). Case 2: [TBD]-beam layout including only edge beams), with a wrap-around mechanism (i.e., 2 neighbor satellites with partial neighbor beams). |
| UV plane illustration | The same to single satellite simulation |
| UV plane convention | The method is the same to single satellite simulation |
| Adjacent beam spacing on UV plane | Baseline: Adjacent beam spacing computation based on 3dB beam width of the satellite antenna pattern : ABS = sqrt(3) x sin(HPBW/2 [rad]) |
| Central beam bore sight direction definition | Central beam center is considered at nadir point |
| Multi-satellite beam generation method | 1. Obtain all the locations of the multiple satellites 2. Generate beams using aforementioned parameters for target satellite located at (earth radius + satellite orbit, 0, 0) in ECEF coordinate 3. Duplicate the target satellite’s beam layout and rotate the beam layout in ECEF coordinate 4. Obtain all the beam layout for multiple satellites |

Proposal 12: For multi-satellite case calibration, the target satellite should be located at (earth radius+satellite orbit, 0, 0) in ECEF coordinate.

Proposal 13: For a target satellite, the beam layout generation method should be the same to single satellite case. The beam layout of other satellites can be replicated and rotated from the target satellite.

Take LEO 600km in S band of Set-1 case for example, Figure 3.2 illustrates the UV plane beam layout for one satellite, the total beam number is 469. Figure 3.3 illustrates the multi-satellite beam layout in earth surface (longitude latitude plane). 7 satellites of LEO-600km in S band are chosen and the satellite to be evaluated is located in the center with an ECEF coordinate of (6978000, 0, 0). The beam layout is the same for each satellite. The other surrounding 6 satellites’ beams can be replicated and rotated from the center satellite. In this case, each satellite has full number of beams to make sure there is no coverage gap in the geographical area, as shown in Figure 3.3.

Observation 16: With the identical beam spread angle for all the beams in one satellite, the beam coverage area at low elevation angle is much larger than that of high elevation angle, and there are beam overlapping among inter-satellite beams.

Proposal 14: New beam layout generation mechanism should be studied to solve the beam overlapping problem, such as turning off some beams or changing the HPBW of edge beams.

Figure 3.2 UV plane Beam layout of one satellite for LEO 600km in S band

Figure 3.3 Earth surface beam layout of seven satellites for LEO 600km in S band of Set-1

To evaluate the whole satellite’s performance and to make sure the results to be identical with the real scenario, full beam layout are necessary.

Observation 17: To evaluate the whole satellite’s performance and to make sure the results to be identical with the real scenario, full beam layout are necessary.

If only satellite edge UEs are concerned, we can simplify the simulation setup e.g. 3 satellites can be enough.

Observation 18: To evaluate the inter-satellite interferences for satellite edge UEs, three satellites with edge beam layout definition are sufficient.

Frequency reuse factor

For single satellite cases, the frequency reuse factor is already agreed with 1, 3, 2 (polarization re-use is enabled). For multi-satellite cases, the frequency reuse factor should consider the intra-satellite beams FRF and inter-satellite beams FRF, which can be different. Table 3.5 illustrates the proposed FRF for multi- satellite cases. It should be noticed that the beam layouts are illustrated in longitude-latitude, so the coverage of beams are different. All the possibilities of FRFs are listed here and not all of them are suitable for multi-satellite cases.

Table 3.5 FRF for multi-satellite cases

| Intra-satellite beams | Inter-satellite beams |
| --- | --- |
| FRF=1 | FRF=1 |
| FRF=1 | FRF=3 |
| FRF=1 | FRF=2 (if polarization re-use is enabled) |
| FRF=3 | FRF=3 |
| FRF=2 (polarization re-use is enabled) | FRF=2 (polarization re-use is enabled) |

Observation 19: With FRF=1 for intra-satellite beams and FRF=1 for inter-satellite beams, inter satellite interference is severe due to overlapped coverage area at the satellite edges.

Observation 20: FRF=1 for intra-satellite beams and FRF=3 or 2 for inter-satellite beams, can be a solution to avoid inter-satellite interference with a low frequency reuse efficiency.

Proposal 15: FRF=1 for intra-satellite beams and FRF=3 or 2 for inter-satellite beams should not be considered for multi-satellite cases.

Observation 21: Applying the same FRF configuration for different satellites can be a solution to avoid inter-satellite interference with a high frequency reuse efficiency.

Proposal 16: FRF configuration should be the same for each satellite in multi-satellite simulation, e.g., 1, 3 and 2 (polarization re-use is enabled).

Simulation results and analysis

In this section, some calibration results with full beam layout of 7 satellites are given. The UE distribution and parameters are shown in Table A.1 and Table A.2 in the Appendix, and detailed UE characteristics can be found in Table X.3 of [2]. Frequency reuse factors (FRF) of 1 and 3 are assumed for both inter-satellite and intra-satellite FRF in the example simulation of LEO 600 km S band (Set-1) and the distribution of coupling loss, carrier to noise ratio (CNR), carrier to interference ratio (CIR) and carrier to interference and noise ratio (CINR) are illustrated in the following Figure 3.4 and Figure 3.5. The distributions of coupling loss, CNR, CIR and CINR of LEO 1200 km Ka band (Set-2) are illustrated in the following Figure 3.6 and Figure 3.7. The simulation configurations are detailed in the Table A.1 (for Set-1) and Table A.2 (for set-2) in the Appendix.

Figure 3.4 Illustration of the CL, C/N and CINR for LEO 600 in S band of set-1 with FRF = 1

Figure 3.5 Illustration of the CL, C/N and CINR for LEO 600 in S band of set-1 with FRF= 3

Figure 3.6 Illustration of the CL, C/N and CINR for LEO 1200 in Ka band of set-2 with FRF= 1

Figure 3.7 Illustration of the CL, C/N and CINR for LEO 1200 in Ka band of set-2 with FRF= 2

Observation 22: The interference of FRF 1 is larger than FRF 2 and FRF 3 for the considered scenarios.

Proposal 17: Parameters listed in Table A.1 and Table A.2 should be used for simulation platform calibration.

Conclusions

In this contribution, the system level simulation assumptions are elaborated. Based on the discussion in the previous sections, we made the following observations and proposals:

Observation 1: The conventional wrap around method is originally applied to the full frequency re-use case.

Observation 2: In the conventional wrap around method, 2-tier interferences (i.e., interferences from 18 interfering cells) are modelled for each reference UE.

Observation 3: NTN generally adopts frequency reuse pattern (e.g., 3-color or 4-color frequency reuse) instead of full frequency reuse pattern used in terrestrial networks.

Observation 4: The conventional wrap around method should not be directly used for single satellite simulation due to the unrealistic interference modeling problem.

Observation 5: 2-tier interference can provide an approximate interference modeling.

Observation 6: If frequency reuse is applied, the extended wrap around method cannot model 2-tier interferences and leads to underestimated interference for a reference UE.

Observation 7: The proposed NTN-specific wrap around method can guarantee 2-tier interferences for different FRF.

Observation 8: if Case 2 is set to 60 degrees, the minimum elevation angles of all beams after wrap around will approach 10 degrees.

Observation 9: It is unclear whether the value of satellite EIRP density changes or not when different FRF is used.

Observation 10: The value of coupling loss for NTN ranges approximately from -146.5dB to -96dB.

Observation 11: The value of geometry for NTN ranges approximately from -18.5dB to 10dB.

Observation 12: There is a gap in geometry performance between two wrap around options.

Observation 13: The Others type UE achieves nearly zero throughput in all scenarios.

Observation 14: Reference satellite constellations for different scenarios are required for multi-satellite simulation.

Observation 15: The path loss increases by 4-5 dB when the elevation angle decreases from 90 degree to 30 degree for LEO scenarios, and varies within 1 dB when elevation angle changes for GEO scenarios.

Observation 16: With the identical beam spread angle for all the beams in one satellite, the beam coverage area at low elevation angle is much larger than that of high elevation angle, and there are beam overlapping among inter-satellite beams.

Observation 17: To evaluate the whole satellite’s performance and to make sure the results to be identical with the real scenario, full beam layout are necessary.

Observation 18: To evaluate the inter-satellite interferences for satellite edge UEs, three satellites with edge beam layout definition are sufficient.

Observation 19: With FRF=1 for intra-satellite beams and FRF=1 for inter-satellite beams, inter satellite interference is severe due to overlapped coverage area at the satellite edges.

Observation 20: FRF=1 for intra-satellite beams and FRF=3 or 2 for inter-satellite beams, can be a solution to avoid inter-satellite interference with a low frequency reuse efficiency.

Observation 21: Applying the same FRF configuration for different satellites can be a solution to avoid inter-satellite interference with a high frequency reuse efficiency.

Observation 22: The interference of FRF 1 is larger than FRF 2 and FRF 3 for the considered scenarios.

Proposal 1: For wrap around 4-tier beams (i.e., 108 beams) are added outside the initial 19-beam layout in the UV plane for single satellite case.

Proposal 2: The numbering scheme in NTN-specific wrap around method needs further study.

Proposal 3: The case 2 in Scenario C2/D2 should be studied in the multiple-satellite scenario instead of the single-satellite scenario.

Proposal 4: Adopt simulation configurations in Table 2.3 as the start point for single satellite SLS.

Proposal 5: Adopt the same total beam power method when different FRF is used.

Proposal 6: Double check and adjust the Others type UE characteristics defined in Table X.3 in [1].

Proposal 7: To avoid large pass loss variation, the minimum elevation angle should be set to 30 degree for both LEO 600km and LEO 1200km scenarios, and 10 degree for GEO scenario, for the satellite reference constellation design.

Proposal 8: To simplify the constellation design, the constellations should be walker star constellation and the inclination should be set to 87.5 degree.

Proposal 9: Reference constellation defined in Table 3.2 can be considered for Set-1 satellite parameters in multiple-satellite SLS.

Proposal 10: Reference constellation defined in Table 3.3 can be considered for Set-2 satellite parameters in multiple-satellite SLS.

Proposal 11: A subset of satellites (e.g., 3 or 7 satellites) in the reference constellations are sufficient for multiple-satellite system level simulation.

Proposal 12: For multi-satellite case calibration, the target satellite should be located at (earth radius+satellite orbit, 0, 0) in ECEF coordinate.

Proposal 13: For a target satellite, the beam layout generation method should be the same to single satellite case. The beam layout of other satellites can be replicated and rotated from the target satellite.

Proposal 14: New beam layout generation mechanism should be studied to solve the beam overlapping problem, such as turning off some beams or changing the HPBW of edge beams.

Proposal 15: FRF=1 for intra-satellite beams and FRF=3 or 2 for inter-satellite beams should not be considered for multi-satellite cases.

Proposal 16: FRF configuration should be the same for each satellite in multi-satellite simulation, e.g., 1, 3 and 2 (polarization re-use is enabled).

Proposal 17: Parameters listed in Table A.1 and Table A.2 should be used for simulation platform calibration.

- References

Chairman's Notes RAN1#97 7.2.5 v003.

IEEE, “802.16m Evaluation Methodolody Document (EMD),” 2009.

TR 38.811 Study on solutions evaluation for NR to support Non-Terrestrial Network v15.0.0

- Appendix

- Single-satellite coupling loss results

Figure A1. Coupling loss calibration result of configuration C1.

Figure A2. Coupling loss calibration result of configuration C2.

Figure A3. Coupling loss calibration result of configuration C3.

Figure A4. Coupling loss calibration result of configuration C4.

Figure A5. Coupling loss calibration result of configuration C5.

Figure A6. Coupling loss calibration result of configuration C6.

Figure A7. Coupling loss calibration result of configuration C7.

Figure A8. Coupling loss calibration result of configuration C8.

Figure A9. Coupling loss calibration result of configuration C9.

Figure A10. Coupling loss calibration result of configuration C10.

Figure A11. Coupling loss calibration result of configuration C11.

Figure A12. Coupling loss calibration result of configuration C12.

Figure A13. Coupling loss calibration result of configuration C13.

Figure A14. Coupling loss calibration result of configuration C14.

Figure A15. Coupling loss calibration result of configuration C15.

Figure A16. Coupling loss calibration result of configuration C16.

Figure A17. Coupling loss calibration result of configuration C17.

Figure A18. Coupling loss calibration result of configuration C18.

- Single-satellite geometry results for different wrap around options

Figure A19. Geometry calibration result of configuration C1 under wrap around option 1.

Figure A20. Geometry calibration result of configuration C1 under wrap around option 2.

Figure A21. Geometry calibration result of configuration C2 under wrap around option 1.

Figure A22. Geometry calibration result of configuration C2 under wrap around option 2.

Figure A23. Geometry calibration result of configuration C3 under wrap around option 1.

Figure A24. Geometry calibration result of configuration C3 under wrap around option 2.

Figure A25. Geometry calibration result of configuration C4 under wrap around option 1.

Figure A26. Geometry calibration result of configuration C4 under wrap around option 1.

Figure A27. Geometry calibration result of configuration C5 under wrap around option 1.

Figure A28. Geometry calibration result of configuration C5 under wrap around option 2.

Figure A29. Geometry calibration result of configuration C6 under wrap around option 1.

Figure A30. Geometry calibration result of configuration C6 under wrap around option 2.

Figure A31. Geometry calibration result of configuration C7 under wrap around option 1.

Figure A32. Geometry calibration result of configuration C7 under wrap around option 2.

Figure A33. Geometry calibration result of configuration C8 under wrap around option 1.

Figure A34. Geometry calibration result of configuration C8 under wrap around option 2.

Figure A35. Geometry calibration result of configuration C9 under wrap around option 1.

Figure A36. Geometry calibration result of configuration C9 under wrap around option 2.

Figure A37. Geometry calibration result of configuration C10 under wrap around option 1.

Figure A38. Geometry calibration result of configuration C10 under wrap around option 2.

Figure A39. Geometry calibration result of configuration C11 under wrap around option 1.

Figure A40. Geometry calibration result of configuration C11 under wrap around option 2.

Figure A41. Geometry calibration result of configuration C12 under wrap around option 1.

Figure A42. Geometry calibration result of configuration C12 under wrap around option 2.

Figure A43. Geometry calibration result of configuration C13 under wrap around option 1.

Figure A44. Geometry calibration result of configuration C13 under wrap around option 2.

Figure A45. Geometry calibration result of configuration C14 under wrap around option 1.

Figure A46. Geometry calibration result of configuration C14 under wrap around option 2.

Figure A47. Geometry calibration result of configuration C15 under wrap around option 1.

Figure A48. Geometry calibration result of configuration C15 under wrap around option 2.

Figure A49. Geometry calibration result of configuration C16 under wrap around option 1.

Figure A50. Geometry calibration result of configuration C16 under wrap around option 2.

Figure A51. Geometry calibration result of configuration C17 under wrap around option 1.

Figure A52. Geometry calibration result of configuration C17 under wrap around option 2.

Figure A53. Geometry calibration result of configuration C18 under wrap around option 1.

Figure A54. Geometry calibration result of configuration C18 under wrap around option 2.

- Multiple-satellite simulation assumptions

Table A.1 Illustration of the simulation assumptions for set-1 satellite parameters

| Case | LEO600-S | LEO600-Ka | LEO1200-S | LEO1200-Ka | GEO-S | GEO-Ka |
| --- | --- | --- | --- | --- | --- | --- |
| Satellite altitude | 600km | 600km | 1200km | 1200km | 35786km | 35786km |
| Reference satellite constellation | LEO | LEO | LEO | LEO | GEO | GEO |
| Satellite number per orbit | 30 | 30 | 17 | 17 | 4 | 4 |
| Orbit number | 17 | 17 | 10 | 10 | 4 | 4 |
| Beam number per satellite | 469 | 2977 | 397 | 2437 | 1801 | 8911 |
| Duplexing | FDD | FDD | FDD | FDD | FDD | FDD |
| DL Frequency | 2GHz | 20GHz | 2GHz | 20GHz | 2GHz | 20GHz |
| UL Frequency | 2GHz | 30GHz | 2GHz | 30GHz | 2GHz | 30GHz |
| Bandwidth per Satellite (DL) | 30MHz | 400MHz | 30MHz | 400MHz | 30MHz | 400MHz |
| Frequency reuse factor among beams | 1, 3 | 1, 2 | 1, 3 | 1, 2 | 1, 3 | 1, 2 |
| Channel model | Suburban and Rural scenarios (Large scale model, ref from TR38.811) |  |  |  |  |  |
| Outdoor UE configuration | Handheld | VSAT | Handheld | VSAT | Handheld | VSAT |
| UEs distribution | X=10 UEs per beam with uniform distribution in all the beam |  |  |  |  |  |
| UE orientation | random; | VSAT: Idea Tracking serving beam | random | VSAT: Idea Tracking serving beam | random | VSAT: Idea Tracking serving beam |
| Satellite EIRP density | 34dBW/MHz | 4 dBW/MHz | 40 dBW/MHz | 10 dBW/MHz | 59 dBW/MHz | 40 dBW/MHz |
| Satellite beam HPBW [degree] | 4.4 | 1.76 | 4.4 | 1.76 | 0.4 | 0.18 |
| UE distribution Region | Coverage area of target satellite |  |  |  |  |  |
| UE altitude | 0km |  |  |  |  |  |
| UE antenna Type | 3gpp Class3 | VSAT | 3gpp Class3 | VSAT | 3gpp Class3 | VSAT |
| UE antenna G/T | -30.62dB/K | 15.86dB/K | -30.62dB/K | 15.86dB/K | -30.62dB/K | 15.86dB/K |
| Handover Margin | 0dB | 0dB | 0dB | 0dB | 0dB | 0dB |
| UE attachment | RSRP based | RSRP based | RSRP based | RSRP based | RSRP based | RSRP based |
| Polarization loss | 0dB | 0dB | 0dB | 0dB | 0dB | 0dB |

Table A.2 Illustration of the simulation assumptions for set-2 satellite parameters

| Case | LEO600-S | LEO600-Ka | LEO1200-S | LEO1200-Ka | GEO-S | GEO-Ka |
| --- | --- | --- | --- | --- | --- | --- |
| Satellite altitude | 600km | 600km | 1200km | 1200km | 35786km | 35786km |
| Reference satellite constellation | LEO | LEO | LEO | LEO | GEO | GEO |
| Satellite number per orbit | 30 | 30 | 17 | 17 | 4 | 4 |
| Orbit number | 17 | 17 | 10 | 10 | 4 | 4 |
| Beam number per satellite | 127 | 469 | 91 | 397 | 547 | 1519 |
| Duplexing | FDD | FDD | FDD | FDD | FDD | FDD |
| DL Frequency | 2GHz | 20GHz | 2GHz | 20GHz | 2GHz | 20GHz |
| UL Frequency | 2GHz | 30GHz | 2GHz | 30GHz | 2GHz | 30GHz |
| Bandwidth per Satellite (DL) | 30MHz | 400MHz | 30MHz | 400MHz | 30MHz | 400MHz |
| Frequency reuse factor among beams | 1, 3 | 1, 2 | 1, 3 | 1, 2 | 1, 3 | 1, 2 |
| Channel model | Suburban and Rural scenarios (Large scale model, ref from TR38.811) |  |  |  |  |  |
| UEs distribution | X=10 UEs per beam with uniform distribution in all the beam |  |  |  |  |  |
| Outdoor UE configuration | Handheld | VSAT | Handheld | VSAT | Handheld | VSAT |
| UE orientation | random; | VSAT: Idea Tracking serving beam | random | VSAT: Idea Tracking serving beam | random | VSAT: Idea Tracking serving beam |
| Satellite EIRP density | 28 dBW/MHz | -4 dBW/MHz | 34 dBW/MHz | 2 dBW/MHz | 53.5 dBW/MHz | 32 dBW/MHz |
| Satellite beam HPBW [degree] | 8.8 | 4.4 | 8.8 | 4.4 | 0.74 | 0.44 |
| UE distribution Region | Coverage area of target satellite |  |  |  |  |  |
| UE altitude | 0km |  |  |  |  |  |
| UE antenna Type | 3gpp Class3 | VSAT | 3gpp Class3 | VSAT | 3gpp Class3 | VSAT |
| UE antenna G/T | -30.62dB/K | 15.86dB/K | -30.62dB/K | 15.86dB/K | -30.62dB/K | dB/K |
| Handover Margin | 0dB | 0dB | 0dB | 0dB | 0dB | 0dB |
| UE attachment | RSRP based | RSRP based | RSRP based | RSRP based | RSRP based | RSRP based |
| Polarization loss | 0dB | 0dB | 0dB | 0dB | 0dB | 0dB |