3GPP TSG-RAN WG1 Meeting #98	R1-1909568

# Prague, Czech Republic,  August 26th – 30th, 2019

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| [DRAFT] CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.214 | CR |  | rev | - | Current version: | 15.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on SFTD measurement |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R1 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  | Date: |  |  | 2019-08-26 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | It was agreed in both LS R4-1803506 and R2-1804119, and captured by UE capability sftd-MeasNR-Cell in TS 38.306 that SFTD measurement between E-UTRA PCell and NR neighbouring cells is supported in Rel-15 when no NR PSCell is configured. However, its corresponding definition of SFTD measurement is missing in RAN1 specification TS 38.215. Additionally, none of the SFTD measurements that have been defined in TS 38.215 is intra-frequency SFTD measurement between serving cells. Therefore, the term “intra-frequency” in the row of “Applicable for” is not correct. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Update the SFTD measurement definition to include the case of SFTD measurement between LTE PCell and NR neighbouring cells when no NR PSCell is configured. - Update the row of Applicable for. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incomplete specification on the definition of SFTD measurement for NR standalone and for UEs capable of EN-DC. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.1.34 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | x |  | Other core specifications |  |  |  | TS 38.215 |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Isolated Impact Analysis: The first change in this CR only completes RAN1 specification according to agreements and involves no functional change. Thus, no impact on eNB/UE implementation due to this change is expected. The second change in this CR is only specification alignment. Thus, no impact on eNB/UE implementation due to this change is expected. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Unchanged parts are omitted>

5.1.34	SFN and frame timing difference (SFTD)

| Definition | The observed SFN and frame timing difference (SFTD) between an E-UTRA PCell and an NR PSCell (for EN-DC), or an NR PCell and an E-UTRA PSCell (for NE-DC), or an E-UTRA PCell and an NR neighbouring cell is defined as comprising the following two components: - SFN offset = (SFNPCell - SFNTRGCell) mod 1024, where SFNPCell is the SFN of a PCell radio frame and SFNTRGCell is the SFN of the target-cell radio frame of which the UE receives the start closest in time to the time when it receives the start of the PCell radio frame. - Frame boundary offset = , where TFrameBoundaryPCell is the time when the UE receives the start of a radio frame from the PCell, TFrameBoundaryTRGCell is the time when the UE receives the start of the radio frame, from thetarget cell, that is closest in time to the radio frame received from the PCell. The unit of (TFrameBoundaryPCell - TFrameBoundaryTRGCell) is Ts. |
| --- | --- |
| Applicable for | RRC_CONNECTED inter-frequency for a UE configured with EN-DC/NE-DC RRC_CONNECTED inter-RAT for a UE configured with E-UTRA PCell but without NR PSCell |

<Unchanged parts are omitted>