3GPP TSG RAN WG1 #98 Meeting	    						R1-1909609

Prague, CZ, August 26th – 30th, 2019

Agenda item:	7.2.4.1

Source: 	Samsung

Title: 	Feature lead summary#2 for agenda item 7.2.4.1 Physical layer structure for sidelink

Document for:	Discussion and Decision

Introduction

This document is for the summary and discussion based on the contributions submitted to RAN1#98 AI 7.2.4.1. As work plan (R1-1905436) of this WI, the recommended order of discussion is as below.

Considering priority and the number of discussion paper, the following issues will be discussed first.

- 1-C: Separation of RP for unicast/groupcast/broadcast

Proposal for 1C

  - In physical layer perspective, a (pre-)configured resource pool can be used for all of unicast, groupcast, and broadcast for a given UE.

    - There is no (pre-)configuration to inform which cast types are used for the resource pool.

RAN1 further discuss

- Whether resource pool configuration based on zone is considered beneficial to support in NR V2X ?

  - More discussion

- Whether resource pool configuration based on different cast types should be supported by taking into account the configuration of PSFCH resource?

  - no

- Whether an NR V2X mode 2 UE can select multiple resource pools on single carrier from RAN1 perspective?

  - Yes,, more explaination is needed.

Whether cast types are transparent to physical layer?

No cast type indicator in SCI

  - Resource pool configuration based on different cast types is also supported.

- 5-B: time-domain DMRS pattern for PSSCH

Proposal for 5-B

  - (Pre-)configuration of one or more PSSCH DMRS pattern(s) in time domain per a resource pool is supported.

  - Exact DMRS pattern is indicated by SCI.

    - FFS: how/what is indicated

  - For Mode 2, DMRS pattern is chosen by the transmitter UE from the configured patterns for the resource pool.

    - FFS: case for Mode 1

    - FFS: whether/how to use restrictions for choice of DMRS pattern

  - FFS on details on time-domain pattern

- 4-C: single-stage vs two-stage SCI

Proposal for 4-C

  - RAN1 down-selects

    - Option 1. Single-stage SCI

[vivo], [OPPO], [Samsung], [CATT], [China Telecom], [ZTE, Sanechips], [Sharp]

    - Option 2. Two-stage SCI

[Huawei, HiSilicon], [TCL], [Nokia, NSB], [MediaTek], [Intel], [Fraunhofer], [Lenovo, Motorola], [Furturewei], [Sony], [Panasonic], [Ericsson], [Spreadtrum], [Kyocera], [InterDigital], [ASUSTeK], [Qualcomm], [Fujitsu], [AT&T]

  - For 2-stage SCI, to be discussed

    - For 2-stage SCI, source ID is scrambled in CRC of 2nd SCI.

    - Only 1st SCI is required to decode broadcast PSSCH.

- 4-B: PSCCH DMRS

Proposal

  - For frequency-domain pattern for PSCCH DMRS, reuse Rel-15 NR PDCCH DMRS, i.e., every 4 REs have a RE for PSCCH DMRS.

  - If RAN1 decides that PSCCH is supported up to 3 OFDM symbols, then Rel-15 NR PDCCH DMRS is the baseline for time-domain pattern for PSCCH DMRS.

- 4-A: number of symbols/PRBs for PSCCH

Proposal

  - PSCCH with 2 symbols and 3 symbols are supported in Rel-16.

    - FFS: other length(s) of symbols

  - The number of PSCCH symbols are (pre-)configured.

- 5-A: frequency-domain DMRS pattern for PSSCH

Proposal

  - Use both type 1 and type 2 of Rel-15 PDSCH DMRS as baselines for PSSCH DMRS frequency domain pattern

  - The type is (pre-)configurable per resource pool.

- 6-A: PSFCH format(s)

Proposal

In RAN1#98, RAN1 discusses the followings.

  - Number of PRBs for PSFCH format 0

  - Number of HARQ-ACK bits for PSFCH format 0

  - X-symbol PSFCH format with a repetition of the one-symbol PSFCH format (not including AGC training period). E.g., X=2

  - Whether to support a PSFCH format based on PUCCH format 2

  - Whether to support a PSFCH format spanning all available symbols for sidelink in a slot

- 8-A: AGC handling

Proposal

In RAN1#98, RAN1 discusses the followings.

  - Opt 1. Define AGC training preamble/RS/signal [Samsung], [CATT]

  - Opt 2. Use comb-type mapping [OPPO], [Ericsson]

  - Opt 3. Using mapping of PSCCH and PSSCH [LGE]

  - Opt 4. Leave up to UE implementation

Issues for discussion

- Issue 1. Resource Pool/BWP

- Whether a resource pool consists of contiguous PRBs or not. Configuration and granularity

There are two options as below (as agreed in RAN1#AH1901).

- Option 1. The resource pool always consists of contiguous PRBs. [Huawei, HiSilicon], [Intel], [China Telecom], [ZTE, Sanechips], [InterDigital], [ITL], [NTT DOCOMO], [Qualcomm],

  - To avoid in-band emission problem

  - Non-contiguous configuration can be covered by multiple contiguous configurations

  - To avoid resource fragmentation

  - To reduce signaling overhead

  - More standardization effort to allow non-contiguous PRBs in a RP

  - To avoid large MPR issue

- Option 2. The resource pool can consist of non-contiguous PRBs. [Nokia, Nokia Shanghai Bell 2], [OPPO, 6], [Samsung, 8], [LGE, 21], [Spreadtrum, 23]

  - For flexible and efficient resource pool management

  - For better coexistence with Uu interface

  - With limited number of frequency segments

  - Possible non-contiguous PRBs in a resource pool because of using CP-OFDM

- Observation: 8 papers propose Option 1 while 5 papers propose Option 2. It seems that both options have quite motivations to support. One of the main motivations of Option 2 is for sidelink operations in a licensed band.

- Proposal for agreement

  - Support (pre-)configuration of a resource pool consisting of contiguous PRBs

  - Also, support configuration of a resource pool consisting of non-contiguous PRBs, where this configuration is provided by gNB/eNB only for sidelink operations in a licensed band

- Time-domain granularity of resource pool configuration

- Indication of RP configuration with slot granularity: [TCL], [OPPO], [Samsung], [Intel], [Panasonic], [China Telecom],  [LGE], [Ericsson], [Spreadtrum], [Qualcomm]

- Additional indication of RP configuration with symbol granularity [OPPO], [Panasonic], [Spreadtrum]

- Observation: To avoid complicated sidelink design, slot-level indication for resource pool configuration is proposed, where all the symbols in a slot are available for sidelink. But, RAN1 needs to discuss further resource pool configuration for licensed carrier.

- Proposal for agreement

  - For dedicated sidelink carrier, a slot is used for granularity of resource pool configuration.

  - RAN1 further discusses resource pool configuration for licensed carrier.

- Separation of RP for unicast/groupcast/broadcast

Separation of resource pools based on whether unicast, groupcast, or broadcast was discussed in RAN1#97 with the following candidate alternatives. Some companies showed their view on this in their papers.

- Alt1. For a given UE, a single (pre-)configured resource pool can be used for any combination of unicast, groupcast, and broadcast.

  - [Huawei, HiSilicon], [vivo], [TCL], [Nokia, NSB], [OPPO], [Samsung], [Panasonic], [China Telecom], [Ericsson], [Spreadtrum], [InterDigital], [ITL], [Qualcomm]

  - Better resource utilization considering dynamic changes of data type

- Alt 2. For a given UE, a single (pre-)configured resource pool can be used either for broadcast or for unicast/groupcast.

  - [NEC], [Fraunhofer]

  - It is also proposed to configure RP based on cast type or mode type or both.

  - Increase resource efficiency due to no PSFCH resources in broadcast RP

  - Better near-far problem

- There is LS from RAN2, R1-1908004, which have the following questions to RAN1.

- Whether resource pool configuration based on zone is considered beneficial to support in NR V2X ? - Whether resource pool configuration based on different cast types should be supported by taking into account the configuration of PSFCH resource? - Whether an NR V2X mode 2 UE can select multiple resource pools on single carrier from RAN1 perspective?

The second question above is related to this issue.

- Observation: 13 papers propose to allow a resource pool used for any combination of unicast, groupcast, and broadcast. 2 papers propose to only allow a resource pool used for each of unicast, groupcast, and broadcast. Proponents for both alternatives argue that each proposal has a benefit of resource utilization. Alt 1 can allow dynamic use of different cast types. Alt 2 can avoid resource waste in supporting sidelink broadcast from defining PSFCH for unicast/groupcast.

- Proposal

  - In designing PHY channels and signals, RAN1 assumes a single (pre-)configured resource pool can be used for any combination of unicast, groupcast, and broadcast for a given UE.

  - Resource pool configuration based on different cast types is also supported.

- When SL BWP and Uu BWP are in the same carrier, which symbols are used for sidelink resource pool

There was three FFS points such as the time scale, relation to DL BWP including initial Uu BWP, relation in terms of frequency location and bandwidth.

The following restrictions need to be further discussed.

- SL-BWP/Uu-BWP cannot use the same freq. resources

- SL-BWP/Uu-BWP should have same frequency location

- SL-BWP/Uu-BWP use the same numerology

- No switching delay between UL and SL or define switching delay requirements

Also, the following issues need to be further discussed.

- For available symbol for SL, are SL symbols available in UL or UL+F symbols?

- Whether to allow the case of DL/SL and the case of SL/UL in a slot

- How to define SL BWP in NR Uu

- Observation: Compared to other issues, not many papers discussed which symbols are available for sidelink operations. This may be related to how to signal and how to be related to DL/UL indication in NR Uu.

- Proposal

  - Further discuss the above issues

- Other topics

There are papers discussing others related to resource pool and SL BWP as below. RAN1 needs to discuss the following topics.

  - Supported SL bandwidth in Rel-16

    - Support NR Uu bandwidth

  - Common reference point for SL BWP

  - Size of sub-channel

    - minimum sub-channel size is equal to or larger than PSCCH PRB size

  - LS from RAN2: R1-1908004

- Whether resource pool configuration based on zone is considered beneficial to support in NR V2X ?

- Whether resource pool configuration based on different cast types should be supported by taking into account the configuration of PSFCH resource?

- Whether an NR V2X mode 2 UE can select multiple resource pools on single carrier from RAN1 perspective?

- Issue 3. PSCCH/PSSCH/PSFCH multiplexing within a slot

- Multiplexing of PSCCH and PSSCH

There was a working assumption on Option 3 for multiplexing of PSCCH and PSSCH. There was no concrete conclusion for Option 1A/1B/2.

- Observation: Majority companies support Option 3 multiplexing while Options 1B/2 needs more discussion. Other options seem not to get many supports.

To decide further support Option 1B/2, RAN1 may need the concrete design of PSCCH and PSSCH first.

- Proposal for agreement

  - Confirm the working assumption that, for multiplexing of PSCCH and PSSCH in a slot, Option 3 is supported.

    - FFS: whether/when PSCCH/PSSCH FDMed is allowed

- PSCCH-only transmission within a slot (standalone PSCCH transmission in a slot)

This is related to whether cross-slot scheduling and CSI report triggering are supported or not.

- Observation: RAN1 needs to discuss this topic.

- Proposal

  - RAN1 to discuss whether to support PSCCH-only transmission within a slot for a given UE, i.e., whether PSCCH transmission without the associated PSSCH is supported or not.

  - Discuss in other agenda (Mode 2)

- PSFCH-only transmission within a slot

- Observation: Most of discussion papers seem to discuss PSFCH structure with the assumption of PSFCH-only transmission within a slot. Even though RAN1 did not discuss this topic explicitly, but it seems to be agreeable.

- Proposal for agreement

  - For a given UE, PSFCH-only transmission within a slot is supported for NR sidelink.

- Multiplexing between PSCCH/PSSCH and PSFCH

There are the following alternatives for each issue as below.

- For transmission perspective from a UE

    - Alt 1. Allow FDM between PSCCH/PSSCH and PSFCH

    - Alt 2. Not allow FDM between PSCCH/PSSCH and PSFCH

  - For system/resource pool perspective,

    - Alt 1. Allow FDM between PSCCH/PSSCH and PSFCH

    - Alt 2. Not allow FDM between PSCCH/PSSCH and PSFCH

- Observation: For UE transmission perspective, many of proponents for allowing FDM between PSCCH/PSSCH and PSFCH seems also to propose long PSFCH format. It seems long PSFCH format discussion should be done first.

- Proposal for agreement

  - RAN1 needs to discuss PSFCH formats first.

- Issue 4. PSCCH design

- Number of symbols/PRBs for PSCCH

How many symbols/PRBs are used for PSCCH should be discussed.

  - Symbols:

    - Up to 3 symbols: [Huawei, HiSilicon], [vivo]

    - 4~6 symbols: [Intel], [ZTE, Sanechips]

    - About 7 symbols:

    - All symbols in a slot: [Huawei, HiSilicon],

  - PRBs

    - 2~18 PRBs depending on symbols [Huawei, HiSilicon]

    - 5, 10 PRBs [Intel], [ZTE, Sanechips]

- Observation: It seems there are not much discussion on the number of PSCCH symbols/PRBs. After the discussion of single-stage and two-stage SCI, the number of PSCCH symbols will be discussed first.

- Proposal (from RAN1#97)

  - PSCCH with 2 symbols and 3 symbols are supported in Rel-16.

    - FFS: other length(s) of symbols

  - The number of PSCCH symbols are (pre-)configured.

- DMRS for PSCCH

For frequency-domain pattern for PSCCH DMRS, the following options are proposed.

  - Opt 1. Reuse Rel-15 NR PDCCH DMRS (every 4 REs) [Huawei, HiSilicon], [vivo], [OPPO], [Samsung], [Intel], [NEC], [LGE], [Spreadtrum], [InterDigital], [Qualcomm]

  - Opt 2. Use PSSCH DMRS for decoding PSCCH [Xiaomi]

- Observation: Majority companies propose to reuse the DMRS pattern in the frequency-domain of Rel-15 NR PDCCH DMRS for PSCCH DMRS. It seems better to reuse Rel-15 design as much as possible, not repeating discussion of detailed design. Due to lack of discussion in time-domain resource for PSCCH, the time-domain pattern is FFS. But, with the same principle, if PSCCH is supported up to 3 OFDM symbols, then Rel-15 NR PDCCH DMRS is the baseline for time-domain pattern for PSCCH DMRS.

- Proposal for agreement

  - For frequency-domain pattern for PSCCH DMRS, reuse Rel-15 NR PDCCH DMRS, i.e., every 4 REs have a RE for PSCCH DMRS.

  - If RAN1 decides that PSCCH is supported up to 3 OFDM symbols, then Rel-15 NR PDCCH DMRS is the baseline for time-domain pattern for PSCCH DMRS.

- Single-stage vs Two-stage SCI

Up to RAN1#97, there was discussion whether to support two-stage SCI and its applicability to different transmissions (unicast/broadcast/groupcast). In RAN1#98, companies provide their further analysis and views on this issue as below.

  - Many companies provide their analysis on forward compatibility, spec impact, latency, complexity, robustness, overhead, etc

  - Link-level evaluation results on reliability

    - [vivo] single-stage SCI is better about 1.5 dB

    - [MediaTek] similar (0.1~0.2 dB better or worse)

    - [CATT] single-stage SCI is better about 1.5 dB

    - [Intel] comparable (with assumption of 6 bits CRC for 2nd SCI)

    - [ZTE] single-stage SCI is better

    - [Ericsson] for decoding PSSCH, single-stage SCI is better, For sensing, two-stage SCI is better (since only first SCI is needed)

    - [Xiaomi] single-stage SCI is better

- Observation: 16 papers of [Huawei, HiSilicon], [TCL], [Nokia, NSB], [MediaTek], [Intel], [Fraunhofer], [Lenovo, Motorola], [Furturewei], [Sony], [Panasonic], [Ericsson], [Spreadtrum], [Kyocera], [InterDigital], [ASUSTeK], [Qualcomm], ([Fujitsu], [AT&T]) propose to support two-stage SCI while 8 papers of [vivo], [OPPO], [Samsung], [CATT], [China Telecom], [ZTE, Sanechips], [Sharp], [NTT DOCOMO] propose to support single-stage SCI. There are more papers to show that single-stage SCI is better than two-stage SCI. The proponents of two-stage SCI mention better sensing accuracy, better forward compatibility, low decoding complexity, and scheduling flexibility as the advantages of two-stage SCI, but the proponents of two-stage SCI also argue that single-stage has the merit on those aspects as well.

Status

Single stage: [vivo], [OPPO], [Samsung], [CATT], [China Telecom], [ZTE, Sanechips], [Sharp]

Two stagae: [Huawei, HiSilicon], [TCL], [Nokia, NSB], [MediaTek], [Intel], [Fraunhofer], [Lenovo, Motorola], [Furturewei], [Sony], [Panasonic],[Ericsson], [Spreadtrum], [Kyocera], [InterDigital], [ASUSTeK], [Qualcomm], [Fujitsu], [AT&T]

- Proposal for

  - RAN1 further discusses.

    - Option 1. Single-stage SCI

    - Option 2. Two-stage SCI

- SCI Contents

In RAN1#96bis, the following SCI contents were discussed.

|  |  |  |  |  | For 2-stage SCI |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Bit field | candidate size | unicast | groupcast | broadcast | 1st SCI | 2nd SCI | Note for Need to agree |
| HARQ process number | [1-4] | O | O | [O] | [O] | [O] | agreed in RAN1-#AH1901 |
| NDI | 1 | O | O | [O] | [O] | [O] | agreed in RAN1-#AH1901 |
| RV | 2 | O | O | [O] | [O] | [O] | agreed in RAN1-#AH1901 |
| Layer-1 source ID | [8-24] | O | O |  | [O] | [O] | agreed in RAN1-#AH1901 |
| Layer-1 destination ID | [8-24] | O | O | [O] | O |  | agreed in RAN1-#AH1901 |
| frequency-domain resource assignment for scheduling PSSCH | [8] | O | O | O | O |  | size dependent on BW/sub-channel size |
| MCS | [5] | O | O | O | O |  | seems needed |
| CRC | 24 | O | O | O | O | O | needed |
| QoS indication | 3 | O | O | O | O |  | Discussed in AI of QoS on bit field size and which parameter will be used for this |
| CSI indication | [1] | O |  |  |  | O | need to discuss in AI of procedure |
| CSI request | [1] | O |  |  |  | O | use case agreed only for unicast |
| Antenna port(s) | [2] | O | [O] | [O] | [O] | [O] | related to #layer/DMRS ports |
| DMRS sequence initialization | [1] | O | [O] | [O] | [O] | [O] | whether to support dynamic changes of DRMS sequence initialization - may be not needed for V2X |
| PTRS-DMRS association | [1-2] | O | [O] | [O] | [O] | [O] | whether to support PTRS-DMRS association - may be not needed for V2X |
| CBGTI | [0-8] | O | O |  |  | O | whether to support CBG or not |
| resource reservation | [4] | O | O | O | O |  | in AI of resource allocation |
| Time gap between initial transmission and retransmission | [4] | O | O | O | O |  | in AI of resource allocation |
| Retransmission index | [1-2] | O | O | O | O |  | in AI of resource allocation |
| Transmission format /cast type indicator | [1-2] | O | O | O | O |  | need to discuss |
| Zone ID | [10] |  | O |  |  | O | in AI of procedure |
| NACK distance | [5] |  | O |  |  | O | in AI of procedure |
| HARQ feedback indication | [1-2] | O | O |  |  | O | in AI of procedure |
| time-domain resource assignment for scheduling PSSCH | [3] | O | O |  |  | O | whether to support multi-slot scheduling/slot aggregation |
| second SCI indication | [2-4] |  |  |  | O |  | only for 2-stage SCI |
| DMRS pattern | [2] | O | O | O | O |  | whether to support dynamic indication of DMRS pattern |

    - Note: This is not intended to support the features associated to each bit field.

    - Note: This does not have any implication on the size of each bit field.

  - Other bit fields are not precluded for NR SCI format(s).

- Observation: Most of SCI contents need discussion in resource allocation and procedure agendas.

- Proposal

  - RAN1 further discusses with the following candidates.

| Bit field | candidate size | unicast | groupcast | broadcast |
| --- | --- | --- | --- | --- |
| HARQ process number | [1-4] | O | O | FFS |
| NDI | 1 | O | O | FFS |
| RV | 2 | O | O | FFS |
| Layer-1 source ID | [8-24] | O | O | FFS |
| Layer-1 destination ID | [8-24] | O | O | FFS |
| MCS | [5] | O | O | O |
| CRC | 24 | O | O | O |
| QoS indication | 3 | O | O | O |
| CSI request | [1] | O | X | X |
| DMRS pattern | [1-2] | O | O | FFS |

  - ‘O’ denotes supported while ‘X’ denotes not supported.

  - Other fields not in the above list should be discussed.

- Others topics

  - Single-port transmission of PSCCH [TCL], [Intel]

  - Starting symbol of PSCCH: first symbol [NEC], [ZTE, Sanechips], 3rd symbol [OPPO]

## Issue 5. PSSCH design

- Frequency-domain DMRS pattern for PSSCH

Regarding the frequency-domain DMRS pattern, the following options are proposed.

- Opt 1. Use Rel-15 PDSCH DMRS type 1 as a baseline for PSSCH DMRS: [TCL], [OPPO], [MediaTek], [Intel], [Sony], [China Telecom], [ZTE, Sanechips], [Ericsson], [Spreadtrum], [InterDigital]

- Opt 2. Use Rel-15 PDSCH DMRS type 2 as a baseline for PSSCH DMRS: [Samsung], [NEC]

    - Type 2 has better throughput performance [Samsung]]

- Opt 3. Use Rel-15 PDSCH DMRS type 1 and type 2 as baselines for PSSCH DMRS: [Huawei, HiSilicon], [Nokia, NSB], [CATT], [Fraunhofer], [Sequans], [Xioami]

- Observation: 10 contributions propose to use type 1 DMRS while 2 contributions proposes to use type 2. Also, 6 contributions propose to configure one of them.

- Proposal for Agreement

  - Use both type 1 and type 2 of Rel-15 PDSCH DMRS as baselines for PSSCH DMRS frequency domain pattern

  - The type is (pre-)configurable per resource pool.

- Time-domain DMRS pattern for PSSCH

One issue is whether to support multiple patterns for a given resource pool are supported or not.

Multiple patterns are supported per resource pool for PSSCH DMRS [Huawei, HiSilicon], [vivo], [TCL], [Nokia, NSB], [OPPO], [Samsung], [CATT], [Sequans], [Sony], [NEC], [China Telecom], [Ericsson], [Qualcomm], where [Huawei, HiSilicon] proposed one pattern for broadcast.

Single pattern per resource pool is defined [Futurewei], [ZTE, Sanechip], [Spreadtrum].

  - Not only for SCS, but also for different speed, multiple patterns are needed

  - Communication type/range

  - Reflect channel status

  - Traffic pattern

  - Indicated by PSCCH or SCI

  - DMRS sequence generated based on destination ID

  - DMRS time-domain pattern based on SCS

- Observation: Among companies to discuss this topic, majority supports time-domain multiple pattern of PSSCH DMRS per resource pool.

- Proposal (from RAN1#97)

  - For a given SCS, multiple PSSCH DMRS patterns in time domain per a resource pool for unicast, groupcast, and broadcast, respectively, are supported.

  - Exact DMRS pattern is indicated by SCI.

  - For Mode 2, DMRS pattern is chosen by the transmitter UE from the set or a subset of patterns configured for the resource pool.

    - FFS: case for Mode 1

    - FFS: whether/how to use subset restrictions based on speed and MCS

  - FFS on details on time-domain pattern, e.g., reuse Rel-15 PDSCH DMRS time-domain patterns, or new time-domain patterns

- Number of TB per PSSCH

In RAN1#96bis, the working assumption was made only to use 1 TB per PSSCH. It was considered that only up to 2 layers can be used for PSSCH.

  - Confirm WA: [Samsung], [Mitsubishi], [LGE], [Spreadtrum]

  - Also, support 2 TBs per PSSCH: [Huawei, HiSilicon]

- Observation: There is one company proposing two TBs per PSSCH while 4 companies propose to confirm the WA.

- Proposal

  - Conform the working assumption to support 1 TB per PSSCH.

- Maximum modulation order supported

- Observation: There is not much discussion, while the use of 256QAM is proposed [Huawei, HiSilicon], [Nokia, NSB], [Samsung], [China Telecom], [NTT DOCOMO]

- Proposal

  - 256QAM is supported in Rel-16, at least for unicast.

  - MCS tables supported in Rel-15 NR are also supported in Rel-16 NR V2X.

- Others

- Tx diversity scheme

  - Support pre-coder cycling for PSSCH [Intel]

- DMRS sequence generation and scrambling

- TBS determination [Intel]

- Relationship of frequency locations of PSCCH and PSSCH [vivo], [MediaTeK], [Intel], [Sharp]

- Using LBRM Limited Buffer Rate Matching (LBRM) for Rel-15 NR PDSCH is the baseline for LBRM of PSSCH (from RAN1#97)

- Issue 6. PSFCH design

- Short/Long PSFCH format(s)

In RAN1#97, RAN1 made an agreement regarding PSFCH format.

There are following FFS issues:

- Number of PRBs for PSFCH format 0 (agreed in RAN1#97)

  - [Huawei, HiSilion]: a sub-channel.

  - [Fujitsu]: 4 RBs.

  - [OPPO]: 4/6 RBs

  - [ZTE, Sanechips], [Interdigital], [Ericsson]: 1 RB

- Number of HARQ-ACK bits for PSFCH format 0

  - [Ericsson]: 1 bit

  - [TCL], [NEC]: 2 bits

  - [OPPO], [Intel]: 4 bitss

- X-symbol PSFCH format with a repetition of the one-symbol PSFCH format (not including AGC training period). E.g., X=2

  - [TCL], [ZTE, Sanechips], [Spreadtrum], [Kyocera]: X=2

  - [Huawei, HiSilion], [InterDigital], [Panasonic], [NEC]: X=1

- A PSFCH format based on PUCCH format 2

  - [Spreadtrum]: Support

  - [Samsung], [NEC], [Ericsson], [Kyocera]: Not support

- A PSFCH format spanning all available symbols for sidelink in a slot

  - [Huawei, HiSilion], [Nokia, NSB]: Support

  - [OPPO], [NEC], [Ericsson]: Not support

- Proposal

    - RAN1 further discuss the above issues for conclusion.

- Others

- Issue 7. Other Reference Signals

- Sidelink CSI-RS

- Observation: There are not many discussions on sidelink CSI-RS design itself. One option is to reuse NR Uu CSI-RS as some companies proposed while other option is to newly design SL CSI-RS.

- Proposal

RAN1 further discusses.

- Sidelink PT-RS

- Observation: There are not much detailed discussion of designing PT-RS while there are four papers to reuse NR Uu PT-RS design for SL.

- Proposal for agreement (from RAN1#97 version)

  - Rel-15 PT-RS for UL CP-OFDM is the baseline for sidelink PT-RS.

    - Sidelink PT-RS time-domain pattern(s) reuse one or several of the NR PT-RS time-domain pattern(s) and configuration (time-domain densities, dependency on time-domain DM-RS positions)

    - FFS: If multiple sidelink time-domain PTRS densities are supported, reuse the NR time domain PT-RS density selection mechanism.

    - Sidelink PT-RS frequency-domain pattern(s) in FR2 reuse one or several of the NR PT-RS frequency-domain patterns

    - FFS: If multiple sidelink frequency-domain densities are supported, reuse the NR frequency domain PT-RS density selection mechanism

- Issue 8. Handling AGC and Tx/Rx switching gap

- Whether to define preamble/RS/signal for AGC training

There are the following options being considered for handling AGc.

- Opt 1. Define AGC training preamble/RS/signal [Samsung], [CATT]

- Opt 2. Use comb-type mapping [OPPO], [Ericsson]

- Opt 3. Using mapping of PSCCH and PSSCH [LGE]

- Opt 4. Leave up to UE implementation

- Observation: RAN4 sent an LS to RAN1 for AGC handling.

- Proposal

RAN1 to discuss how to handle AGC issue.

- Issue 9. Other aspects

Reference

- R1-1908039	Sidelink physical layer structure for NR V2X	Huawei, HiSilicon

- R1-1908148	Physical layer structure for NR sidelink	vivo

- R1-1908218	Control Channel Design and its Efficient Transmission in NR-V2X		Fujitsu

- R1-1908275	Physical layer structure for sidelink	TCL Communication Ltd.

- R1-1908282	Discussion of Physical layer structure for sidelink	Nokia, Nokia Shanghai Bell

- R1-1908361	Physical layer structure for NR-V2X	OPPO

- R1-1908396	Discussion on sidelink physical layer structure	MediaTek Inc.

- R1-1908475 (revised to R1-1909461)	On Physical Layer Structures for NR V2X		Samsung

- R1-1908579	Physical layer structure for NR sidelink	CATT

- R1-1908633	Physical structure for NR V2X		Intel Corporation

- R1-1908675	Physical layer design for NR V2X sidelink	Mitsubishi Electric RCE

- R1-1908677	Design of NR V2X Physical Layer Structures		Fraunhofer HHI, Fraunhofer IIS

- R1-1908718	DMRS design for NR V2X sidelink	Sequans Communications

- R1-1908730	Discussion on physical layer structure for NR sidelink	Lenovo, Motorola Mobility

- R1-1908737	Two-stage SCI design and adaptive DMRS support for sidelink		Futurewei

- R1-1908770	Discussion on physical layer structure for NR sidelink	Sony

- R1-1908803	Discussion on physical layer structure for sidelink in NR V2X	Panasonic Corporation

- R1-1908850	Physical layer structure for NR sidelink	NEC

- R1-1908881	On Physical Layer Structure for NR sidelink	China Telecommunications

- R1-1908894	NR sidelink physical layer structure	ZTE, Sanechips

- R1-1908900	Discussion on physical layer structure for NR sidelink	LG Electronics

- R1-1908911	PHY layer structure for NR sidelink	Ericsson

- R1-1908947	Discussion on physical layer structure for sidelink	Spreadtrum Communications

- R1-1909015	Sidelink Physical Layer Structure	Kyocera Corporation

- R1-1909028	On Physical Layer Structure for NR V2X Sidelink	InterDigital, Inc.

- R1-1909070	Physical layer structure design for sidelink	AT&T

- R1-1909094	Physical layer structure for NR sidelink	Sharp

- R1-1909116	Physical layer structure for NR V2X	ITL

- R1-1909185	Sidelink physical layer structure for NR V2X	NTT DOCOMO, INC.

- R1-1909252	Considerations on Physical Layer aspects of NR V2X	Qualcomm Incorporated

- R1-1909290	Discussion on physical layer structure for 5G V2X	Xiaomi Communications

- R1-1909305	Discussion on sidelink structure in NR V2X	ASUSTEK COMPUTER (SHANGHAI)

- R1-1909388	Considerations on PSFCH Design for NR V2X	KT Corp.

- R1-1909419	On Physical Layer Structure for NR V2X sidelink	Convida Wireless

Appendix: Previous agreements

Agreements in RAN1#94

Agreements:

- At least PSCCH and PSSCH are defined for NR V2X. PSCCH at least carries information necessary to decode PSSCH.

  - Note: PSBCH will be discussed in the synchronization agenda.

- RAN1 continues study on the necessity of other channels.

- Further study on

  - Whether/which sidelink feedback information is carried by PSCCH or by another channel/signal.

  - Whether/which information to assist resource allocation and/or schedule UE’s transmission resource(s) is carried by PSCCH or by another channel/signal.

  - PSCCH format(s) and content(s) for unicast, groupcast, and broadcast

Agreements:

- RAN1 to continue study on the physical channel considering at least the following aspects:

- Waveform

  - Candidates: CP-OFDM, DFT-s-OFDM

  - Proposals from companies:

    - CP-OFDM only

    - Support both

  - Consideration points:

    - Different channel can have different waveform?

    - Benefit and impact of supporting only one waveform and supporting both waveforms

- Subcarrier spacing

  - Candidates for further study are:

    - FR1: 15 kHz, 30 kHz, 60 kHz, 120 kHz

    - FR2: 30 kHz, 60 kHz, 120 kHz, 240 kHz

  - Companies are encouraged to consider the potential issues and benefit of introducing new subcarrier spacing.

- CP length

- RS design

  - Candidates are:

    - DM-RS

      - DM-RS defined in Rel-15 NR Uu is the starting point.

    - PT-RS

    - CSI-RS

    - SRS

    - AGC training signal

- Channel coding

  - For data, channel coding defined for data in Rel-15 NR Uu is the starting point.

  - For control, channel coding defined for control in Rel-15 NR Uu is the starting point.

- Modulation

- RE mapping and rate-matching

- Scrambling

Agreements:

- RAN1 continues study on the necessity, benefits and relationship between bandwidth part and resource pool.

Agreements:

Agree the following assumptions as tentative assumptions for the simulation at least till RAN1#94bis

- AGC

  - Up to [15] us in FR1. Up to [10] us in FR2.

- TX/RX switching time

  - [13] us in FR1 and [7] us in FR2

- Time error

  - Up to [0.4] us between a UE and its synchronization reference

- Frequency error

  - Up to [0.1] PPM between a UE and its synchronization reference

Agreements:

RAN1 to continue study on multiplexing physical channels considering at least the above aspects:

- Multiplexing of PSCCH and the associated PSSCH (here, the “associated” means that the PSCCH at least carries information necessary to decode the PSSCH).

  - Study further the following options:

    - Option 1: PSCCH and the associated PSSCH are transmitted using non-overlapping time resources.

      - Option 1A: The frequency resources used by the two channels are the same.

      - Option 1B: The frequency resources used by the two channels can be different.

    - Option 2: PSCCH and the associated PSSCH are transmitted using non-overlapping frequency resources in the all the time resources used for transmission. The time resources used by the two channels are the same.

    - Option 3: A part of PSCCH and the associated PSSCH are transmitted using overlapping time resources in non-overlapping frequency resources, but another part of the associated PSSCH and/or another part of the PSCCH are transmitted using non-overlapping time resources.

Illustration of the above options:

Agreements in RAN1#94bis

Agreements:

- NR sidelink supports the SCSs supported by Uu in a given frequency range, i.e., {15, 30, 60 kHz} in FR1 and {60, 120 kHz} in FR2.

  - FFS the supported CP length

  - Baseline is that a UE is not required to receive sidelink transmissions using different SCSs simultaneously in a given carrier.

    - FFS if this applies to sidelink synchronization signals/channels

  - Baseline is that a UE is not required to transmit sidelink transmissions using different SCSs simultaneously in a given carrier.

    - FFS if this applies to sidelink synchronization signals/channels

Continue discussion on the waveform till next meeting – companies are encouraged to perform more analysis/evaluations.

Agreements:

For PSCCH and associated PSSCH multiplexing

- At least one of Option 1A, 1B, and 3 is supported.

  - FFS whether some options require transient period between PSCCH and PSSCH.

- FFS whether to support Option 2

R1-1812017

Agreements:

Sidelink control information (SCI) is defined.

SCI is transmitted in PSCCH.

SCI includes at least one SCI format which includes the information necessary to decode the corresponding PSSCH.

NDI, if defined, is a part of SCI.

Sidelink feedback control information (SFCI) is defined.

SFCI includes at least one SFCI format which includes HARQ-ACK for the corresponding PSSCH.

FFS whether a solution will use only one of “ACK,” “NACK,” “DTX,” or use a combination of them.

FFS how to include other feedback information (if supported) in SFCI.

FFS how to convey SFCI on sidelink in PSCCH, and/or PSSCH, and/or a new physical sidelink channel

FFS in the context of Mode 1:

whether/how to convey information for SCI on downlink

whether/how to convey information of SFCI on uplink

Agreements:

At least resource pool is supported for NR sidelink

Resource pool is a set of time and frequency resources that can be used for sidelink transmission and/or reception.

FFS whether a resource pool consists of contiguous resources in time and/or frequency.

A resource pool is inside the RF bandwidth of the UE.

FFS how gNB and other UEs know the RF bandwidth of the UE

FFS if BWP (if defined) can be used to in defining at least part of resource pool

FFS if the numerology of a resource pool is indicated as a part of (pre-)configuration for resource pool, carrier, band, or BWP (if defined)

UE assumes a single numerology in using a resource pool.

Multiple resource pools can be configured to a single UE in a given carrier.

FFS how to use multiple resource pools when (pre-)configured.

FFS BWP is supported for NR sidelink

FFS whether RAN1 can assume that at most one BWP is configured in a carrier from the system perspective.

It is RAN1 understanding that, in some cases, the entire system bandwidth is covered by a single BWP.

FFS the details of BWP configurations, including the possibility of restricting the number of BWPs

FFS whether BWP for TX and RX is separated or a common BWP applied to both TX and RX

There is at most one activated sidelink BWP for a UE in a given carrier as in the Uu case

Further study the feasibility, benefit, and impact of sidelink BWP switching

Aim to conclude in RAN1#95

Companies are encouraged to provide more analysis, including checking current Rel-15 specification regarding BWP related text

Agreements in RAN1#95

Agreements:

- At least CP-OFDM is supported.

- Continue study on whether to support DFT-S-OFDM including the potential issues and the following potential benefit:

  - Synchronization coverage enhancement

  - PSCCH coverage enhancement, e.g., with Option 2 of PSCCH/PSSCH multiplexing with the restriction that PSCCH and PSSCH use adjacent frequency resources

  - Feedback channel coverage enhancement

- A single waveform is used in all the sidelink channels in a carrier.

  - Note: A sequence based channel can be supported in any waveform.

  - (Pre-)configuration will be used to determine the used waveform if the specification supports multiple waveforms.

Agreements:

- For PSCCH/PSSCH in FR1, NR V2X supports normal CP for 15kHz, 30kHz, 60kHz, and extended CP for 60kHz.

  - FFS extended CP for 30 kHz in FR1.

- FFS CP for PSCCH/PSSCH in FR2

  - E.g., NR V2X supports normal CP for 60kHz and 120kHz, and extended CP for 60kHz

    - FFS extended CP for 120 kHz in FR2.

- Only one combination of CP length and SCS is used in a carrier at a given time for NR V2X UEs communicating with each other using SL

Agreements:

- BWP is defined for NR sidelink.

  - In a licensed carrier, SL BWP is defined separately from BWP for Uu from the specification perspective.

    - FFS the relation with Uu BWP.

  - The same SL BWP is used for both Tx and Rx.

  - Each resource pool is (pre)configured within a SL BWP.

  - Only one SL BWP is (pre)configured for RRC idle or out of coverage NR V2X UEs in a carrier.

  - For RRC connected UEs, only one SL BWP is active in a carrier. No signalling is exchanged in sidelink for activation and deactivation of SL BWP.

    - Working assumption: only one SL BWP is configured in a carrier for a NR V2X UE

      - Revisit in the next meeting if significant issues are found

  - Numerology is a part of SL BWP configuration.

Note: This does not intend to make restriction in designing the sidelink aspects related to SL BWP.

Note: This does not preclude the possibility where a NR V2X UE uses a Tx RF bandwidth the same as or different than the SL BWP.

Working assumption:

- Regarding PSCCH / PSSCH multiplexing, at least option 3 is supported for CP-OFDM.

  - RAN1 assumes that transient period is not needed between symbols containing PSCCH and symbols not containing PSCCH in the supported design of option 3.

  - FFS how to determine the starting symbol of PSCCH and the associated PSSCH

  - FFS for other options. e.g. whether some of them are supported to increase PSCCH coverage.

Send an LS to RAN4 to ask the following for options 1A/1B/3 (adding details of 1A/1B/3 in the LS) – R1-1814089, which is endorsed with the following updates:

- Fixing email address

- “identified” to “are studying”

Final LS in R1-1814165

Working assumption:

- For RAN1 evaluation purpose only, until RAN4 response on AGC and switching time, it is assumed that one symbol is used for AGC and another one symbol is used for TX/RX switching.

Note: TX/RX switching includes transition in the power amplifier.

Agreements in RAN1AH-1901

Conclusion:

  - No extended CP is supported for 30 kHz in FR1 in Rel-16

  - No extended CP is supported for 120 kHz in FR2 in Rel-16

Agreements:

- Confirm the working assumption

  - Working assumption: only one SL BWP is configured in a carrier for a NR V2X UE

Agreements:

- Configuration for SL BWP is separated from Uu BWP configuration signalling.

  - UE is not expected to use different numerology in the configured SL BWP and active UL BWP in the same carrier at a given time.

    - FFS the time scale

    - FFS relation to DL BWP including initial Uu BWP

    - FFS relation in terms of frequency location and bandwidth

Agreements:

- For time domain resources of a resource pool for PSSCH,

  - Support the case where the resource pool consists of non-contiguous time resources

    - FFS details including granularity

- For frequency domain resources of a resource pool for PSSCH,

  - Down select following options:

    - Option 1: The resource pool always consists of contiguous PRBs

    - Option 2: The resource pool can consist of non-contiguous PRBs

Agreements:

- Multiple DMRS patterns in time domain are supported for PSSCH

  - FFS: Whether a DMRS pattern is selected based on the subcarrier spacing

  - FFS: Single or multiple DMRS pattern(s) per a resource pool

  - FFS: How TX UE and RX UE can be aligned in terms of the DMRS pattern used for PSSCH

  - FFS: RE mapping, sequence generation

- Continue to study DMRS pattern in frequency domain for PSSCH

  - E.g. Whether multiple patterns are supported, whether PDSCH/PUSCH DMRS configuration 1 or 2 is reused.

Agreements:

- Support PT-RS for PSSCH for FR2

Conclusion:

RAN1 to conclude on the need of physical channel for discovery in RAN1#96.

Agreements in RAN1#96

Agreements:

- Rel-16 NR sidelink supports CP-OFDM only.

Agreements:

- For PSCCH/PSSCH in FR2, NR V2X supports normal CP for 60 kHz, 120 kHz, and extended CP for 60 kHz.

- Note: it is understood that PSFCH follows the same CP as PSCCH/PSSCH

Agreements:

- For the operation regarding PSSCH, a UE performs either transmission or reception in a slot on a carrier.

- NR sidelink supports for a UE:

  - A case where all the symbols in a slot are available for sidelink.

  - Another case where only a subset of consecutive symbols in a slot is available for sidelink

    - Note: this case is not intended to be used for the ITS spectra, if there is no forward-compatibility issue. Finalize in the WI phase whether there is such an issue or not

    - The subset is NOT dynamically indicated to the UE

    - FFS the supported slot configuration(s)

    - FFS whether/how to operate it in partial coverage scenarios

Agreements:

- At least for sidelink HARQ feedback, NR sidelink supports at least a PSFCH format which uses last symbol(s) available for sidelink in a slot.

Agreements:

- RAN1 concludes that no additional physical channel needs to be defined for the purpose of discovery in Rel-16.

Agreements:

- (Pre-)configuration indicates the time gap between PSFCH and the associated PSSCH for Mode 1 and Mode 2.

Agreements:

- In mode 1 for unicast and groupcast, it is supported for the transmitter UE via Uu link to report an indication to gNB to indicate the need for retransmission of a TB transmitted by the transmitter UE.

  - FFS the format of the indication, e.g., in the form of HARQ ACK/NACK, or in the form of SR/BSR, etc.

- RAN1 continues discussion on whether to support report from the receiver UE

  - No inter-BS communication will be considered.

To discuss aspects related to 1st sub-bullet & 2nd bullet during this week -revisit later

Agreements:

- Sidelink HARQ ACK/NACK report from UE to gNB is not supported in Rel-16.

Agreements:

- For unicast RX UEs, SL-RSRP is reported to TX UE

- For sidelink open loop power control for unicast for the TX UE, TX UE derives pathloss estimation

  - Revisit during the WI phase w.r.t. whether or not there is a need regarding how to handle pathloss estimation for OLPC before SL-RSRP is available for a RX UE

Agreements:

- TPC commands for SL PC are not supported

R1-1903597

Agreements:

- For sidelink groupcast, it is supported to use TX-RX distance and/or RSRP in deciding whether to send HARQ feedback.

  - Details to be discussed during WI phase, including whether the information on TX-RX distance is explicitly signaled or implicitly derived, whether/how this operation is related to resource allocation, accuracy of distance and/or RSRP, the aspects related to “and/or”, etc.

  - This feature can be disabled/enabled

Working assumption:

- For unicast, the following CSI reporting is supported based on non-subband-based aperiodic CSI reporting mechanism assuming no more than 4-port:

  - CQI

  - RI

  - PMI

- CSI reporting can be enabled and disabled by configuration.

  - It is supported to configure a subset of the above metric for CSI reporting.

- There is no standalone RS transmission dedicated to CSI reporting in Rel-16

- NR sidelink CSI strives to reuse the CSI framework for NR Uu.

  - Discuss details during WI phase

Agreements:

- RAN1 concludes the following regarding beam management:

  - Beam management is beneficial

  - RAN1 has conducted limited study on the beam management.

  - In FR1, it is feasible to support V2X use cases without beam management.

  - In FR2, it is feasible to support some V2X use cases without beam management in some scenarios.

    - Panel selection is necessary to improve the communication range in FR2.

Conclusion:

- There is no consensus in supporting beam management for normative work for NR V2X in Rel-16.

Agreements in RAN1#96bis

Agreements:

- Polar code adopted for Rel-15 NR DCI is applied to PSCCH.

- LDPC codes used for Rel-15 NR DL-SCH is applied to a transport block delivered by PSSCH.

Agreements:

- The starting symbol and the number of symbols for a PSCCH are assumed to be known to the receiving UE before decoding the PSCCH.

Agreements:

- For the purpose of evaluation of PSCCH design, RAN1 assumes 60 bits, 90 bits, 120 bits as the total SCI sizes including 24 bits CRC.

  - Other sizes are not precluded.

Agreements:

  - QPSK is used for PSCCH.

Working assumption:

  - Transmission of 1 TB with up to 2 layers in a PSSCH is supported.

Agreements:

- At least for transmission perspective of a UE in a carrier, at least TDM between PSCCH/PSSCH and PSFCH is allowed for a PSFCH format for sidelink in a slot.

  - FFS the details of the corresponding PSFCH format

  - FFS whether it is also applicable from system/resource pool perspective or not

  - i.e., in this case, there is no simultaneous transmission of PSCCH and PSFCH and there is no simultaneous transmission of PSSCH and PSFCH.

  - FFS FDM between PSCCH/PSSCH and a PSFCH format which uses last symbol(s) available for sidelink in a slot

  - FFS TDM/FDM between PSCCH/PSSCH and other PSFCH format(s), if supported, which is/are different from the PSFCH format which uses last symbol(s) available for sidelink in a slot

Agreements in RAN1#97

Agreements:

- A sequence-based PSFCH format with one symbol (not including AGC training period) is supported.

  - This is applicable for unicast and groupcast including options 1/2.

  - Sequence of PUCCH format 0 is the starting point.

  - FFS: 1 PRB or multiple PRBs is/are used for this PSFCH format

  - FFS: feasible number of HARQ-ACK bits, mapping of HARQ-ACK bit

- FFS whether to support the following formats

  - X-symbol PSFCH format with a repetition of the one-symbol PSFCH format (not including AGC training period).

    - E.g. X=2

  - A PSFCH format based on PUCCH format 2

  - A PSFCH format spanning all available symbols for sidelink in a slot

Agreements:

- Transmission of PSSCH is mapped onto contiguous PRBs only

Agreements:

- Sub-channel size is (pre)configurable.

  - FFS details (e.g., possible sizes, a minimum size etc.)

Conclusion:

- If two-stage SCI is supported, the following details are used.

  - Information related to channel sensing is carried on 1st-stage.

  - 2nd-stage is decoded by using PSSCH DMRS.

  - Polar coding used for PDCCH is applied to 2nd-stage

  - Payload size for 1st-stage in two-stage SCI case is the same for unicast, groupcast, and broadcast in a resource pool.

  - After decoding the 1st-stage, the receiver does not need to perform blind decoding of 2nd-stage.

  - FFS other details

- Companies are encouraged to perform analysis (e.g., flexibility, complexity, forward compatibility, overhead, spec impact, latency, robustness, reliability, etc.)/evaluations with details of the SCI contents comparing single-stage vs. two-stage SCI. Aim to conclude in RAN1#98

Working assumption:

- Rel-15 PDSCH DMRS Configuration type 1 and/or type 2 are reused for frequency-domain pattern of PSSCH DMRS.

  - FFS whether to support either one or both types

  - FFS details on multiplexing of different ports for PSSCH DMRS