3GPP TSG-RAN WG1 #98	R1-1909619

Prague, CZ, August 26th – 30th, 2019

Source:           Apple Inc.

Title:                 Feature lead summary #1 for Uplink Power Control for Supporting NN-DC

Agenda item:	7.2.13.1

Document for:	Discussion and Decision

# 1 Introduction

This document summarizes documents submitted to RAN1#98 on AI 7.2.13.1 to facilitate progress on uplink power control for NN-DC. In the following, some relevant questions are also provided based on the views expressed in the contributions listed in the appendix.

- General discussions on power schemes categorizing

- Semi-Static vs. Dynamic Power Sharing for NN-DC

- Details of Semi-Static Power Sharing for NN-DC

- Details of Dynamic Power Sharing for NN-DC

- PHR reporting

It should be noted that the terms ‘Semi-Static Power Sharing’ and ‘Dynamic Power Sharing’ are just to facilitate the technical discussions and whether to use them in specification or not is fully up to editors.

# 2. General discussions on power schemes categorizing

As in LTE DC and EN-DC, to facilitate the discussions and more importantly move progress on this topic, high level power sharing schemes categorizing seems needed. In previous releases, the schemes are divided into two categories, so called “semi-static” and “dynamic” power sharing based on the fundamental difference i.e. whether or not the dynamic scheduling information of other CG needs to be taken into account for power determination of a given CG. This basic distinction is the root that causes different implementation complexity and also results in various power utilization performance and throughput. This is reason why it is used to differentiate and categorize the power sharing schemes in past.

Conceptually, the existing categorizing rule is still valid for NR-NR DC and can be re-used to discuss the power sharing schemes. It should be also noted that extensive analysis has been conducted by major companies with semi-static and dynamic power sharing framework.

However, there is one different view raised in company contributions on this respect and would better to reach consensus on this first.

| Nokia | Proposal 1: If catergorization is needed, Rel-16 should consider the operational details and the complexity of information sharing between the CGs at UE side. Category based discussion may start with following 3 categories - Power control without inter-CG operation - Inter-CG power sharing without prediction - Power control with prediction of asynchronous transmission |
| --- | --- |

The category rule proposed above is essentially identical to existing categorizing rule since “Power control without inter-CG operation” is equivalent to ‘semi-static’ power sharing and the latter two are sub-categories under the “dynamic” power sharing category (i.e. 2nd is Rel-15 CA power control rule and 3rd is enhanced on 2nd with introducing some sort of look-ahead operation).

Given majority of companies have used the ‘semi-static’ and ‘dynamic’ power sharing category in papers and the purpose of categorizing itself is purely to facilitate the discussions, the following was proposed by feature leader:

[Feature lead proposal]

Proposal 1:

- For Rel-16 NR-NR DC with MCG and SCG in a same frequency range, reusing the existing categorizing rule i.e.  dividing schemes into ‘semi-static’ and ‘dynamic’ power sharing schemes depending on whether or not dynamic scheduling information of one CG needs to be taken into account for power determination of another CG.

It should be noted that the motivation of categorizing is to facilitate the discussions under each category. The categories support should be determined based on the separate discussions on technical merits of each scheme.

Companies are encouraged to provide views on P1 in table below:

| Company | Views |
| --- | --- |
| Ericsson | Agree with above categorization. i.e., dynamic power sharing => dynamic scheduling information of one CG should to be taken into account for power determination of another CG semi-static power sharing => only semi-static information of one CG is taken into account for power determination of another CG |

# 3. Semi-Static vs. Dynamic Power Sharing for NN-DC

## 3.1 Companies views and proposals

Semi-static power sharing

It was widely understood that semi-static power sharing can be generalized to be a scheme where the transmission power of each CG is semi-statically split between two CGs such that transmission power of one CG is not impacted or interrupted by overlapped transmissions that is dynamically scheduled in the other CG.

The analysis semi-static power sharing scheme from different companies can be summarized below:

Table 1: Summary of companies views on semi-static power sharing scheme

|  | Pros | Cons |
| --- | --- | --- |
| UE complexity | - Simpler implementation |  |
| Coverage |  | - Reduced coverage |
| Link adaptation | - Predictability and high efficiency | - Predictability at the cost of pre-emptively degrading the link quality. With dynamic PS, network has more flexibility to implement a trade-off between better link quality and link adaptation predictability |
| Phase discontinuities | - Avoidable |  |
| URLLC support |  | - Limited ability to support high priority URLLC transmission |

Dynamic Power Sharing

Dynamic power sharing can be generally defined as one approach to dynamically adjust transmission power of CCs in one CG based on transmission activity in another CG.

Several potential benefits claimed by different companies include:

  - Maintaining coverage

  - Avoiding UE transmission power underutilization

On dynamic power sharing, several main concerns from companies in contributions against the proposal can be summarized as follows e.g. from UE implementation restriction perspective and throughput performance:

- UE implementation restriction and complexity

  - Tighter timeline requirement to implement a fast coordination interface between two CGs for power adjustment.

  - Checking the UL channel types and contents across CCs on both CGs and go through a list of priority rules that alternates between two CGs.

- Causing phase discontinuity in case of power re-scaling in the middle of UL transmission and resulting in different power of DMRS and PUSCH

- Possible throughput loss in some scenarios due to lack of scheduling decision of other gNB.

- Unpredictable network operation and the interruptions are not under the full control of the scheduling gNB and cannot be avoided or planned.

In summary, it was observed that support of dynamic power sharing represents the majority views but with various solutions. One company [13] shows very strong concern on support dynamic power sharing with arguing that no perceivable performance gain but at the cost of extra complication for UE implementation and restriction on hardware structure of NN-DC product.  In addition, it was observed that a unified signalling framework is preferred by majority companies to operate semi-static and dynamic power sharing (i.e. properly setting PMCG, PSCG, PTotalNR-NR DC values for a given UE).

## 3.2 FL summary and proposals

Summary on semi-static power-sharing scheme

Option 1: Support semi-static power sharing

Qualcomm, Intel, Apple, vivo, Huawei, OPPO (mandatory without UE capability), ZTE/AT&T (Up to network to determine semi-static or dynamic power sharing to operate without UE capability signalling)

Option 2: NOT support semi-static power sharing

CATT, Nokia (Unless necessity is justified)

Observation 1:

- 8 companies shared their views to support ‘semi-static’ power sharing. Within them, 2 companies propose to operate semi-static power sharing as one configuration without UE capability.

- 1 company [6] proposed to not support semi-static power sharing.

- Another 1 company [8] also proposed not to support semi-static power sharing unless necessity is justified.

Summary on dynamic power-sharing scheme

Option 1: Support dynamic power sharing

- With UE capability Samsung, Intel, Apple, OPPO

- Without UE capability ZTE

- Ericsson, Huawei, Nokia, AT&T, Vivo, OPPO, Panasonic, Motorola Mobility, Lenovo

- CATT (Without the minimum reserved power or maximum transmit power)

Option 2: NOT support dynamic power sharing

- Qualcomm

Observation 2:

- Majority of companies’ views is to support dynamic power sharing.

  - 4 companies [4][5][7][9] propose to introduce one UE capability for the support of dynamic power sharing to allow implementation flexibility of NN-DC.

  - 1 company [3] proposes to support dynamic power sharing without UE capability.

  - 1 company [13] propose to NOT support dynamic power sharing for NN-DC.

Summary on the signalling framework to operate semi-static and dynamic power sharing for NN-DC

A unified signalling framework to operate power sharing schemes for NR-NR DC

Huawei, vivo, AT&T, ZTE, OPPO, Intel, Apple

Observation 3:

- Majority of companies’ views is to support dynamic power sharing and semi-static power sharing with a unified signalling framework.

Given that dynamic power sharing receives clearly majority support, but it is strongly concerned by some companies on several aspects especially implementation restriction and complexity, the following way forward was therefore proposed by FL as a compromise between two sides to address the respective concerns and move forward on this topic:

[Feature lead proposal]

Proposal 2:

- For Rel-16 NR-NR DC with MCG and SCG, support both semi-static and dynamic power sharing schemes between MCG and SCG that are in a same frequency range (FR) by properly configuring the values of PMCG and PSCG by RRC signaling as follows:

  - For a same frequency range, semi-static power sharing operation is applied if PMCG+PSCG≤PTotalNR-NR DC where PMCG is the configured maximum transmission power for MCG in a frequency range X and PSCG is the configured maximum transmission power for SCG in the same frequency range X, and PTotalNR-NR DC is the total maximum transmission power for NR-NR DC operation in the same frequency range X;

  - For a same frequency range, dynamic power sharing operation is applied if PMCG+PSCG>PTotalNR-NR DCsubject to UE capability support.

  - Signaling details for PMCG, PSCG, PTotalNR-NR DC are left to RAN2 and RAN4.

- All NR-NR DC capable UEs are mandated to support semi-static power sharing (i.e. PMCG+PSCG≤PTotalNR-NR DC)

- The following alternatives were identified during email discussions:

  - Alt.1: Introducing an optional UE capability for dynamic power sharing support and handling of dynamic power sharing (i.e. PMCG+PSCG>PTotalNR-NR DC).

  - Alt.2: Introducing a UE feature to support dynamic power sharing and handling of dynamic power sharing (i.e. PMCG+PSCG>PTotalNR-NR DC) as mandatory with capability signalling.

  - Alt.3: Discuss later as part of UE feature discussions.

Companies are encouraged to constructively provide views/comments on the P2 above to drive progress on this topic, especially considering the limited leftover time on this WI and limited progress so far. Views on alternatives are appreciated.

| Company | Views |
| --- | --- |
| Ericsson | Proposal 2 is not consistent with Proposal 1. If same PMCG and PSCG are not used/applicable for all slots, the condition PMCG+PSCG≤PTotalNR-NR DC by itself does not imply semi-static power sharing. Depending on exact mechanism, it can also imply dynamic power sharing (i.e., dynamic sharing of information between CGs). For example, according to Ericsson proposal, UE applies PMCG limit on MCG transmissions only if it knows that a) there is an overlapping SCG transmission by detecting SCG DCI before Toffset, or b) there can be a potential overlapping SCG transmission (i.e., via RRC) that may be scheduled after Toffset (likewise for limiting SCG power using PSCG). This would be dynamic power sharing (i.e., dynamic sharing of information between CGs) even when PMCG+PSCG≤PTotalNR-NR DC So, in summary if we use the definition of dynamic/semi-static power sharing as in Proposal 1 - Dynamic power sharing is possible with both PMCG+PSCG≤PTotalNR-NR DC and PMCG+PSCG>PTotalNR-NR DC - For semi-static power sharing, PMCG+PSCG≤PTotalNR-NR DC is necessary but not sufficient condition. |

# 4. Details of Semi-Static Power Sharing for NN-DC

## 4.1 Companies views and proposals

In the RAN1 #96bis meeting, three alternatives were identified and included in [1]:

- The UE determines the maximum transmission power in a PUSCH transmission occasion as follows: - Alt.1 - The maximum transmit power per group is restricted by the configured maximum power of the each group. - Alt. 2 - The maximum transmit power per group is restricted by the configured maximum power of the same group except for the case that the UE is certain that no uplink transmission is possible on the other cell group on the overlapping symbols by relying on the semi-statically indicated symbol directions. In such a case, the maximum transmit power of a cell group can be increased. - Alt.3 - A UE is semi-statically configured with a TDM pattern configuration providing three sets of slots: - On the first set of slots, the maximum configured power for the MCG and SCG are given such that P_max_MCG + P_max_SCG <= Ptot. - On the second set of slots, all available power is allocated to the MCG, i.e., P_max_MCG = P_tot. - On the third set of slots, all available power is allocated to the SCG, i.e., P_max_SCG = P_tot.

Company views on semi-static power sharing are listed in the Table below:

Table 2: Summary on preferred solution by companies for semi-static power sharing

| vivo | Proposal 2: - For NN-DC semi-static power control within the same frequency range, - When the UE determines that there is a possible uplink transmission in other CG, the maximum transmission power of each CG is restricted by the configured maximum transmission power of each CG. Otherwise, the maximum transmission power of each CG is the UE maximum transmission power. - Support to reuse Rel-15 CA power control priority rules for each CG. - Both asynchronous and synchronous cases are supported. |
| --- | --- |
| OPPO | Proposal 6: …. - Semi-static power control: P_max_MCG + P_max_SCG > P_taltal_maxs and TDM-based pattern to avoid the simultaneous transmission of MCG and SCG is configured by network |
| Intel | Proposal 1: If supported, the semi-static power sharing for NR-NR DC is specified as follows - The maximum transmit power per group is restricted by the configured maximum power of the same group except for the case that the UE is certain that no uplink transmission is possible on the other cell group on the overlapping symbols by relying on the semi-statically indicated symbol directions. In such a case, the maximum transmit power of a cell group can be increased. |
| Apple | Proposal 2: Semi-static power sharing is operated as follows: - UE is configured with PMCGFRX+PSCGFRX≤PTotalNR-NR DC, FRX where PMCGFRX is the configured maximum transmission power for MCG for a single Frequency Range X (FRX) and PSCGFRX is the configured maximum transmission power for SCG for FRX, and PTotalNR-NR DC, FRX is the maximum transmission power for NR-NR DC operation on FRX. - For the synchronous NR-NR DC scenario, for the transmission in one CG in FRX, UE checks the semi-statically configured transmission direction of the overlapped symbols of the other CG in the same FRX, - If the transmission direction of overlapped symbols is possible to be UL based on RRC configured UL/DL configuration, UE limits the actual maximum transmission power pCG to be configured maximum transmission power of this CG (i.e. PMCGFRX or PSCGFRX) - Otherwise, UE can be up to PMCGFRX' and PSCGFRX'. FFS on how to determine these two values. |
| Qualcomm | Proposal 2: For uplink power control of synchronous NN-DC, the UE is configured with: - A UE should be configured with a set of maximum allowed powers for each cell group such that P_NR1 + P_NR2 <= P_NN. - Additionally, the UE can be configured with P_NR1’ and P_NR2’, which can each be equal to P_NN. In other words, P_NR1’+P_NR2’ < = > P_NN. From the UE side, the following behavior is expected: For the uplink transmission in cell group 1, the UE checks the semi-statically configured direction of the overlapping symbols of all serving cells of cell group 2 and vice versa. The UE chooses the maximum power allowed per cell group based on whether its uplink transmission collides with only semi-static ‘DL’ symbols, or with semi-static ‘UL’ and ‘flexible’ symbols. |

## 4.2 FL summary and proposal

The preference from majority companies on the candidate of UE behavior for semi-static power sharing is clearly Alt.2, as also summarizing below:

Support Alt.1:

Support Alt.2: Qualcomm, Intel, vivo, Apple

Support Alt.3: OPPO

[Feature lead proposal]

Proposal 3: Adopt Alt.2 for the configuration of ‘PMCG+PSCG≤PTotalNR-NR DC’ (i.e. Semi-Static power sharing operation) of NN-DC as follows, for a same frequency range,

- UE is configured with PMCG+PSCG≤PTotalNR-NR DC where PMCG is the configured maximum transmission power for MCG and PSCG is the configured maximum transmission power for SCG, and PTotalNR-NR DC is the maximum total transmission power for NR-NR DC operation.

- For the synchronous NR-NR dual connectivity, for the uplink transmission in MCG, the UE checks the semi-statically configured direction of the overlapping symbols of all serving cells of SCG and vice versa.

  - If such overlapping with UL transmission on the SCG is possible (i.e. collides with semi-static ‘UL’ and ‘flexible’ symbols on all CCs of SCG), UE limits its actual maximum transmission power pMCGin MCG such that pMCG≤PMCG;

  - Otherwise (i.e. collides with only semi-static ‘DL’ symbols on all CCs of SCG), pMCG can be up to PMCG' and  pSCG can be up to  PSCG'  .

    - Alt.1:  PMCG' and PSCG'  are configured by RRC signaling.

    - Alt.2:  PMCG' and PSCG'  are determined by RAN4 requirement.

- FFS on Semi-Static power sharing to be applied for both synchronous and asynchronous NN-DC or synchronous NR-DC only.

Companies are encouraged to provide views/comments on the P3 above to move forward on the details of semi-static power sharing:

| Company | Views |
| --- | --- |

# 5. Details of Dynamic Power Sharing for NN-DC

## 5.1 Companies views and proposals

There are two design aspects need to be considered to enable dynamic power sharing operation

- Whether and how to support ‘look-ahead’ operation?

- How to define the prioritization order for the overlapping UL transmission in power-limited case?

  - Specially, do we need to treat URLLC traffic with highest priority for power-limited case for NR-NR DC?

‘Look-ahead’ operation and prioritization order application

‘look-ahead’ operation is kind of UE capability that can determine transmission power of a signal/channel at a given time instance t0 with taking into account of overlapped later UL transmissions that are scheduled by a DCI format transmitted until time instance t0-Toffset. Figure 1 provides one example to depict this capability.

Figure 1: ‘look-ahead’ operation and priority order rule for dynamic power sharing of NR-NR DC

Companies views on the first aspect i.e. ‘Look-ahead’ operation is summarized in Table 3 below.

Table 3: Summary on views on ‘Look-ahead’ operation.

| Huawei | Proposal 5: A reference timing for power calculation should be defined to differentiate dynamic power sharing with look-ahead from without look-head. Consider to support look-ahead in uplink power control for NR-DC UE. |
| --- | --- |
| vivo | Proposal 3: Power look-ahead is not required at the UE. |
| ZTE | Proposal 5: The cut-off time of look-ahead operation is determined as an offset before the first symbol of one uplink transmission. - 0.5*Tproc,2 can be applied as the starting point of cut-off time for look-ahead operation. - Once determined, the power cannot be changed due to latter arrived uplink grants. |
| OPPO | Proposal 2: No look-ahead operation is required for Rel-16 NR power control for NR-NR DC. |
| Samsung | Proposal 2: The UE can consider a power for a later scheduled overlapping transmission subject to Rel-15 timelines for determining a channel among overlapping channels for UCI multiplexing. |
| Intel | Proposal 3: If supported, reuse the NR CA Case 2 power control mechanism for NR-NR Dual Connectivity to implement dynamic power sharing scheme. |
| Nokia | Proposal 6: Inter-CG power sharing with look-ahead window is supported. FFS: the proper size of look-ahead window |
| Apple | Proposal 4: Reuse the Rel-15 procedure of UCI multiplexing on PUSCH as staring point to define ‘look-ahead’ behavior for NR-NR DC power control in Rel-16. |
| AT&T | Proposal 4: Look ahead function is considered for dynamic power control in NR-NR DC |
| Ericsson | Proposal 2: T_offset can be similar to the T_proc specified in Rel15 for UCI multiplexing related UE procedures. |
| Motorola, Lenovo | Proposal 1: …..assigning power to higher priority, overlapping transmissions whose power is yet to be determined and scheduling information is known at the cut-off time (T0-T_offset) according to priority rules applied across the two CGs (Rel-15 CA power allocation prioritization rules + MCG>SCG for same priority level); |

Companies views on the 2nd design aspect are included in Table 7 below:

Prioritization order of UL channels in power limited case

Table 4: Summary on views on Prioritization order of UL channels

| Huawei | Proposal 6: Prioritizations for transmission power reductions in Rel-15 are treated as starting points for NR-DC, and additionally support the following prioritizations: - PUCCH format 0/2 is prioritized over PUCCH format 1/3/4 carrying the same type UCI. - Single shot PUCCH is prioritized over PUCCH with repetition in multiple slots carrying the same type UCI. - Single shot PUSCH is prioritized over PUSCH with repetition in multiple slots. |
| --- | --- |
| Vivo | Proposal 3: …. - Support to prioritize MCG transmission to perform power scaling or dropping among CGs. - Support to reuse CA power control priority rules for each CG. - Power look-ahead is not required at the UE. |
| ZTE | Proposal 6: For NR-DC in Rel-16, determine the CG prioritization by RRC time pattern or by the dynamic UL grant information. Proposal 7: Regarding uplink channel/signal prioritization within one CG or across CGs, prioritize URLLC uplink channels/signals for transmission power scaling. |
| OPPO | Proposal 6: …. For the dynamic power sharing for NR-NR DC, the similar priorities rules defined for NR CA can be reused by prioritizing MCG over SCG(s) in case of the same priority. |
| Samsung | Proposal 3: Re-use the Rel-15 EN-DC framework for determining scaling of transmission power and actual/virtual PHR. |
| CATT | Proposal 3: If the UE output power at some OFDM symbols exceeds the maximum power of the UE power class, it would be UE implementation choice of scaling the UL Tx power on one or more UL channels based on the power scaling rule. The physical channel priority used for NR CA power scaling should be reused for NR DC |
| Intel | Proposal 3: If supported, reuse the NR CA Case 2 power control mechanism for NR-NR Dual Connectivity to implement dynamic power sharing scheme. |
| Nokia | Proposal 4: To support URLLC transmission with reasonable resource overhead, service type specific power prioritization should be supported with inter-CG power scaling |
| Apple | Proposal 4: Reuse the Rel-15 procedure of UCI multiplexing on PUSCH as staring point to define ‘look-ahead’ behavior for NR-NR DC power control in Rel-16. |
| Ericsson | Proposal 2: If NW configures P_limit_CG1 and P_limit_CG2 such that P_limit_CG1+P_limit_CG2 > P_limit_total, and if pwr_CG1+ pwr_CG2> P_limit_total, UE scales down transmission power of SCG transmission(s) such that UE transmission power across MCG and SCG does not exceed P_limit_total. |
| Motorola Mobility, Lenovo | Proposal 1: [Alt 3] - UE is configured with a minimum guaranteed power (MGP) limits for MCG and SCG separately. - To compute the transmit power for CG1 UL transmission starting at time T0, a UE considers a power for later scheduled overlapping transmissions up to the cut-off time T0-T_offset - In case of a power-limited situation, the UE allocates power for an UL transmission by: - considering allocated powers for overlapping transmissions whose power are already determined – once the transmit power for a transmission is computed, UE will not re-calculate/re-adjust the transmit power based on upcoming transmission regardless of priority levels; - assigning power to higher priority, overlapping transmissions whose power is yet to be determined and scheduling information is known at the cut-off time (T0-T_offset) according to priority rules applied across the two CGs (Rel-15 CA power allocation prioritization rules + MCG>SCG for same priority level); - respecting the MGPs, such that total power on a CG1 can never exceed P_{NR-DC, Total} – P{CG2, min} (except when it is semi-statically known on no possible overlapping UL transmission on the other CG). |

## 5.2 FL summary and proposal

Summary on ‘look-ahead’ operation

- Option 1: Support the following ‘look-ahead’ operation

  - When determining a total transmission power of transmission occasion i starting from T0 in CG1, the UE needs to consider all PDCCH(s) received before time T0-T_offset that trigger an overlapping CG2 UL transmission.  Huawei, ZTE, Samsung, Intel, Nokia, Apple, AT&T, Ericsson, Motorola mobility (9)

  - Option 1-1: T_offset is equal to T_proc specified in Rel15 for UCI multiplexing related UE procedures. Ericsson, Samsung, Apple,

  - Option 1-2: T_offset is equal to 0.5*Tproc,2 ZTE

- Option 2: NOT support ‘look-ahead’ operation   VIVO, OPPO (2)

As evident, support ‘look-ahead’ operation for dynamic power sharing scheme is clearly majority views.

Summary on ‘Prioritization rule’ in power-limited case

- Option 1: Reuse Rel-15 CA rule  Huawei, CATT, Intel, Motorola Mobility, Lenovo [MCG>SCG for same priority level], Apple (6)

  - Option 1-1: additional consider the PUCCH formats  Huawei

- Option 2: Prioritizing MCG over SCG and reuse CA rule within CG  Ericsson, Vivo, [OPPO], [Samsung],

- Option 3: RRC time pattern or by the dynamic UL grant information  ZTE

- Option 4: URLLC should be prioritized over all channels across CGs  ZTE

[Feature lead proposal]

Proposal 4:

- When UE is configured with PMCG+PSCG>PTotalNR-NR DC   (i.e. dynamic power sharing operation) for NN-DC

- The prioritization rule for UL channels in case of power-limited is as follows

  - Alt.1: MCG>SCG, and then apply Rel-15 rule for each CG.

  - Alt.2: Reusing Rel-15 CA rule across CCs of CGs

  - Alt.3: CG prioritization is determined by time pattern or dynamic indication, and then apply Rel-15 rule with potential update considering URLLC priority for each CG.

  - Note that the above prioritization order does not impact on the minimum guaranteed power if it is agreed to be supported.

Companies are encouraged to provided views on the P4 and also preferred alternative if the P3 is acceptable.

| Company | Views |
| --- | --- |

It was also proposed by two companies [3][4] that if URLLC transmission happens at only one CG, URLLC transmission has higher priority than the others.

It is appreciated if companies share Views on this URLLC related proposal in the following table:

- Alt.1: Prioritize URLLC transmission if it happens only one CG

- Alt.2: No special handling for URLLC at least in this WI.

| Company | Views |
| --- | --- |

Regarding priority order, the following was proposed to make progress:

[Feature lead proposal]

Proposal 5:

- When UE is configured with PMCG+PSCG>PTotalNR-NR DC   (i.e. dynamic power sharing operation) for NN-DC

- The following is considered regarding ‘look-ahead’ operation

  - Alt.1: To compute the transmit power for MCG (or SCG) UL transmission starting at time T0, UE checks for PDCCH(s) received before time T0-T_offset that trigger an overlapping SCG (or MCG) UL transmission.

    - FFS on the exact value of T_offset.

  - Alt.2: Similar to EN-DC, the SCG power is scaled dependent of the power scheduled for MCG.

    - The MCG scheduling timing is slowed down as compared to SCG.

  - Alt.3: Per transmission occasion, the sum power over all serving cells of MCG is computed, and the sum power is passed to SCG. The SCG then determines its sum available power in that occasion. Per CG, then power is distributed according to the NR CA priority rules

Companies views on this P5 are intended to include in the following table, if any.

| Company | Views |
| --- | --- |

# 6. PHR reporting

PHR issue was also mentioned in two company contributions [1][5]. Since this meeting will focus on the issues listed in the first four sections, FL just notes this and proposes to continue discussions.

| Huawei | Proposal 8: Study the UE behaviour on PH calculation in asynchronous NR-DC scenario. |
| --- | --- |
| Samsung | Proposal 3: Re-use the Rel-15 EN-DC framework for determining scaling of transmission power and actual/virtual PHR. |

[Feature lead proposal]

Proposal 6:

- Continue discussion on PHR design for NN-DC

# Reference

- R1-1908076	UL power control for NR-NR dual connectivity	Huawei, HiSilicon

- R1-1908179	Uplink power control for Supporting NR-NR DC	vivo

- R1-1908242	Discussion on uplink power control for NR DC	ZTE

- R1-1908355	UL power control for NR-NR DC	OPPO

- R1-1908514	Power Control for NN-DC	Samsung

- R1-1908577	UL Power control for NR -DC	CATT

- R1-1908664	Uplink power control for NR-NR Dual Connectivity	Intel Corporation

- R1-1908922	Uplink PC for NR-NR Dual Connectivity	Nokia, Nokia Shanghai Bell

- R1-1909062	Uplink power control for NR-NR Dual Connectivity	Apple Inc.

- R1-1909077	Uplink Power Control for NR-NR DC	AT&T

- R1-1909142	Power Control for NR-NR DC	Ericsson

- R1-1909156	Uplink power control for NR-NR DC	Motorola Mobility, Lenovo

- R1-1909285	Uplink Power Control Solutions for NR-NR Dual Connectivity	Qualcomm Incorporated

# Appendix: Contributions used as basis for the summary

## R1-1908076 (Huawei)

  - Proposal 1: Support dynamic power sharing for NR-DC with FR1+FR1 band combination(s) for both synchronous and asynchronous scenarios. The UE modem for each CG has the same ability of obtaining scheduling information of the other CG.

  - Proposal 2: Specify a unified configuration scheme of RRC parameters to cover both dynamic power sharing and semi-static power splitting.

  - Proposal 3: Under the same framework for both dynamic power sharing and semi-static power splitting, consider to support different maximum allowed power in different transmission occasions per CG.

  - Proposal 4: For NR-DC with FR1+FR1 band combination(s), each cell group can be configured with minimum guaranteed power.

  - Proposal 5: A reference timing for power calculation should be defined to differentiate dynamic power sharing with look-ahead from without look-head. Consider to support look-ahead in uplink power control for NR-DC UE.

  - Proposal 6: Prioritizations for transmission power reductions in Rel-15 are treated as starting points for NR-DC, and additionally support the following prioritizations:

    - PUCCH format 0/2 is prioritized over PUCCH format 1/3/4 carrying the same type UCI.

    - Single shot PUCCH is prioritized over PUCCH with repetition in multiple slots carrying the same type UCI.

    - Single shot PUSCH is prioritized over PUSCH with repetition in multiple slots.

## R1-1908179 (vivo)

- Proposal 1:

  - For NN-DC power control within the same frequency range, both dynamic and semi-static power sharing are supported by using a set of common higher layer parameters.

- Proposal 2:

  - For NN-DC semi-static power control within the same frequency range,

    - When the UE determines that there is a possible uplink transmission in other CG, the maximum transmission power of each CG is restricted by the configured maximum transmission power of each CG. Otherwise, the maximum transmission power of each CG is the UE maximum transmission power.

    - Support to reuse Rel-15 CA power control priority rules for each CG.

    - Both asynchronous and synchronous cases are supported.

- Proposal 3:

  - For NN-DC dynamic power sharing within the same frequency range,

    - The UE is configured a maximum power P_MCG_Max for MCG and a maximum power P_SCG_Max for SCG respectively.

      - If P_MCG_Max+P_SCG_Max>P_Total, dynamic power sharing will be used.

        - Support to prioritize MCG transmission to perform power scaling or dropping among CGs.

        - Support to reuse CA power control priority rules for each CG.

        - Power look-ahead is not required at the UE.

    - Both asynchronous and synchronous cases are supported.

## R1-1908242 (ZTE)

- Proposal 1: For Rel-16 NR-DC, support one unified signalling framework for both semi-static power sharing and dynamic power sharing and it’s up to the network whether to adopt semi-static power sharing or dynamic power sharing.

- Proposal 2: A UE supporting NR-DC can support dynamic power sharing mandatorily without capability signalling.

- Proposal 3: Consider the following power control solution for Rel-16 NR-DC.

- Configuration - Configure a UE the parameters Rmin_CG1 , Rmin_CG2 and Rmax . - Configure the minimum guaranteed power ratio for each CG respectively, i.e., Rmin_CG1, Rmin_CG2. Network guarantees that Rmin_CG1+ Rmin_CG2 <= 100%. - Configure one maximum power ratio for the CG with lower priority, i.e., Rmax. Network guarantees that Rmax <= 50%. (Note0) - Power computation - If UE is certain that no UL transmission is possible on CG2 on the overlapping symbols by relying semi-statically indicated symbol directions, the maximum power CG1 can use is Pc_max. - If UE is certain that no UL transmission is possible on CG1 on the overlapping symbols by relying semi-statically indicated symbol directions, the maximum power CG2 can use is Pc_max. - Else - UE checks for PDCCH(s) received before time Ts-Tlook_ahead (Note1) that trigger an overlapping CG2 UL transmission. After checking, the transmission power required by CG1 and CG2 are P1 and P2 respectively. If no PDCCH that triggers an overlapping CG2 UL transmission has been found, P2 = 0. - If P1 + P2 <= 100%, UE allocates P1’=min{P1, 1- Rmin_CG2}power for CG1 UE allocates P2’=min{P2, 1- Rmin_CG1}power for CG2. - Else, i.e., if P1 + P2 > 100%, - If CG1 is prioritized over CG2 (Note2) UE allocates P2’=min{P2, Rmax, , 1- Rmin_CG1}power for CG2 UE allocates P1’=min{P1, 1-P2’, 1- Rmin_CG2}power for CG1 - If CG2 is prioritized over CG1 (Note2) UE allocates P1’=min{P1, Rmax, , 1- Rmin_CG2}power for CG1 UE allocates P2’=min{P2, 1-P1’, 1- Rmin_CG1}power for CG2 - Note0: Another way is to configure one maximum power ratio for the CG with higher priority, i.e., Rmax. Network guarantees that Rmax >= 50%. - Note1: The detailed look-ahead operation and definition of Tlook_ahead is presented in Section 2.2. - Note2: The CG prioritization determination is presented in Section 2.3.

- Proposal 4: Support look-ahead operation for dynamic power sharing in Rel-16.

- Proposal 5: The cut-off time of look-ahead operation is determined as an offset before the first symbol of one uplink transmission.

  - 0.5*Tproc,2 can be applied as the starting point of cut-off time for look-ahead operation.

  - Once determined, the power cannot be changed due to latter arrived uplink grants.

- Proposal 6: For NR-DC in Rel-16, determine the CG prioritization by RRC time pattern or by the dynamic UL grant information.

- Proposal 7: Regarding uplink channel/signal prioritization within one CG or across CGs, prioritize URLLC uplink channels/signals for transmission power scaling.

## R1-1908355 (OPPO)

- Proposal 1: Rel-16 NR supports a common scheme which is focused on asynchronous NR-NR DC and can be used for synchronous NR-NR DC as well.

- Proposal 2: No look-ahead operation is required for Rel-16 NR power control for NR-NR DC.

- Proposal 3: Configuration of minimum reserved power per cell group is not necessary for Rel-16 NR power control for NR-NR DC.

- Proposal 4: For Rel-16 NR power control of NR-NR DC, both semi-static power sharing and dynamic power sharing should be supported in a common solution.

- Proposal 5: Rel-16 NR supports the following UE capability signaling for the power control of NR-NR DC

  - semi-static power sharing is mandatory without UE capability signaling

  - dynamic power sharing is optional with UE capability signaling

- Proposal 6: Rel-16 supports the following power control framework for NR-NR DC

  - gNB configures the maximum available power for each cell group, e.g., P_max_MCG, P_max_SCG

  - The sum of the maximum available powers may exceed the maximum power supported by UE

    - Semi-static power control: P_max_MCG + P_max_SCG <= P_taltal_max

    - Semi-static power control: P_max_MCG + P_max_SCG > P_taltal_maxs and TDM-based pattern to avoid the simultaneous transmission of MCG and SCG is configured by network

    - Dynamic power control: P_max_MCG + P_max_SCG > P_taltal_max and simultaneous transmissions of MCG and SCG are allowed

  - When determining a total transmit power for a transmission scheduled by DCI X, the UE does not consider power for the transmissions whose corresponding DCI(s) is after the DCI X. It means no look-ahead operation is required

  - For the dynamic power sharing for NR-NR DC, the similar priorities rules defined for NR CA can be reused by prioritizing MCG over SCG(s) in case of the same priority.

- Proposal 7: The NR-NR DC power scheme can support semi-static and dynamic power sharing by proper configuration

  - Semi-static power sharing: applicable for both synchronous and asynchronous cases

  - Dynamic power sharing: only applicable for synchronous case

## R1-1908514 (Samsung)

- Proposal 1: For NR-DC in FR1, support inter-CG dynamic sharing of UE transmission power.

- Proposal 2: Introduce a UE capability for a UE to indicate whether or not, for determining a power at a transmission occasion, the UE can consider a power for a later scheduled overlapping transmission subject to Rel-15 timelines for determining a channel among overlapping channels for UCI multiplexing.

- Proposal 3: Re-use the Rel-15 EN-DC framework for determining scaling of transmission power and actual/virtual PHR.

- Proposal 4: For NR-DC in FR1, support configuration to a UE of available powers for transmissions on the MCG and the SCG and indication of whether they are minimum powers or maximum powers.

- Proposal 5: The maximum transmission power on a CG can be  when there is no overlapping transmission on the other CG based on an UL-DL TDD configuration on the other CG.

## R1-1908577 (CATT)

- Proposal 1:  Semi-static power sharing between PCG and SCG is not supported in NR-DC.

- Proposal 2:  The dynamic power sharing with the minimum reserved power or maximum transmit power should not be considered in NR-DC

- Proposal 3: If the UE output power at some OFDM symbols exceeds the maximum power of the UE power class, it would be UE implementation choice of scaling the UL Tx power on one or more UL channels based on the power scaling rule.  The physical channel priority used for NR CA power scaling should be reused for NR DC

## R1-1908664 (Intel)

- Proposal 1: If supported, the semi-static power sharing for NR-NR DC is specified as follows

  - The maximum transmit power per group is restricted by the configured maximum power of the same group except for the case that the UE is certain that no uplink transmission is possible on the other cell group on the overlapping symbols by relying on the semi-statically indicated symbol directions. In such a case, the maximum transmit power of a cell group can be increased.

- Proposal 2:

  - Minimum guaranteed powers or maximum transmission powers for MCG and SCG (e.g. PMgNB, PSgNB) can be separately configured on a UE-specific manner.

- Proposal 3:

  - If supported, reuse the NR CA Case 2 power control mechanism for NR-NR Dual Connectivity to implement dynamic power sharing scheme.

- Proposal 4:

  - Support both semi-static and dynamic power sharing for Rel-16 NR-NR DC

  - Introducing a UE capability to indicate the support of dynamic power sharing operation.

- Proposal 5:

  - A unified signalling framework is adopted to operate semi-static and dynamic power sharing by properly setting the corresponding PMgNB and PSgNB values.

## R1-1908922 (Nokia)

- Proposal 1: If catergorization is needed, Rel-16 should consider the operational details and the complexity of information sharing between the CGs at UE side. Category based discussion may start with following 3 categories

  - Power control without inter-CG operation

  - Inter-CG power sharing without prediction

  - Power control with prediction of asynchronous transmission

- Proposal 2: Specification of separated power control mode for Rel-15 DC like semi-static power control should not be supported till the necessity becomes clear.

- Proposal 3: Full power transmission should be supported for any cell in any cell group.

- Proposal 4: To support URLLC transmission with reasonable resource overhead, service type specific power prioritization should be supported with inter-CG power scaling

- Proposal 5: The existence of UL transmission should be shared between CGs at each occation of UL transmission.

- Proposal 6: Inter-CG power sharing with look-ahead window is supported

  - FFS: the proper size of look-ahead window

- Proposal 7: For further discusison whether larger look-ahead window can be supproted by simplifying the information to be observed shared between cell groups.

## R1-1909062 (Apple)

- Proposal 1:

  - Support both semi-static and dynamic power sharing for Rel-16 NR-NR dual connectivity by properly configuring the value of PMCGFRX and PSCGFRX for a same FRX.

- Proposal 2: Semi-static power sharing is operated as follows:

  - UE is configured with PMCGFRX+PSCGFRX≤PTotalNR-NR DC, FRX where PMCGFRX is the configured maximum transmission power for MCG for a single Frequency Range X (FRX) and PSCGFRX is the configured maximum transmission power for SCG for FRX, and PTotalNR-NR DC, FRX is the maximum transmission power for NR-NR DC operation on FRX.

  - For the synchronous NR-NR DC scenario, for the transmission in one CG in FRX, UE checks the semi-statically configured transmission direction of the overlapped symbols of the other CG in the same FRX,

    - If the transmission direction of overlapped symbols is possible to be UL based on RRC configured UL/DL configuration, UE limits the actual maximum transmission power pCG to be configured maximum transmission power of this CG (i.e. PMCGFRX or PSCGFRX)

    - Otherwise, UE can be up to PMCGFRX' and PSCGFRX'. FFS on how to determine these two values.

- Proposal 3:

  - Support to introduce configurable minimum or maximum transmission powers for MCG and SCG in a same FRX by higher layers.

- Proposal 4:

  - Reuse the Rel-15 procedure of UCI multiplexing on PUSCH as staring point to define ‘look-ahead’ behavior for NR-NR DC power control in Rel-16.

  - Introduce a UE capability to indicate the support of dynamic power sharing for Rel-16 NR-NR DC.

  - Case 2 prioritization rules defined for CA are reused for NR-NR DC

## R1-1909077 (AT&T)

- Proposal 1: Dynamic power control for NR-NR DC is supported.

- Proposal 2: Operation in semi-static control scheme can share the same set of RRC parameters as that of dynamic power control scheme

- Proposal 3: For NR-NR DC where both SCG and MCG are wholly in FR1 a single power control mechanism should be defined for both synchronous and asynchronous DC

- Proposal 4: Look ahead function is considered for dynamic power control in NR-NR DC

- Proposal 5: Dynamic power control based on maximum transmit power per CG should be considered

## R1-1909142 (Ericsson)

- Proposal 1

  - Support dynamic power sharing for NR-NR DC power control where MCG and SCG are in same frequency range.

- Proposal 2

  - NR-DC dynamic power sharing where MCG and SCG are in same frequency range follows the below framework

    - UE is configured with transmit power limits P_limit_CG1 applicable to CG1 and P_limit_CG2 applicable to CG2.

    - To compute the transmit power for CG1 UL transmission starting at time T0,

      - UE checks for PDCCH(s) received before time T0-T_offset that trigger an overlapping CG2 UL transmission, and if such PDCCH(s) are detected, UE limits it’s transmit power in CG1 (pwr_CG1) such that pwr_CG1 <= P_limit_CG1;

      - UE also checks (based on RRC parameters) if any possible overlapping CG2 UL transmissions can be triggered by PDCCH(s) after T0-T_offset, and if such overlapping CG2 UL transmissions are possible, UE limits it’s transmit power in CG1 (pwr_CG1) such that pwr_CG1 <= P_limit_CG1;

      - Otherwise, P_limit_CG1 is not applied and UE transmit power for the CG1 UL transmission can be up to ‘full power’ allowed by RAN4 requirements

      - T_offset can be similar to the T_proc specified in Rel15 for UCI multiplexing related UE procedures.

      - Note: The above behavior is required even if NW configures P_limit_CG1 and P_limit_CG2 such that P_limit_CG1+P_limit_CG2<= P_limit_total.

    - If NW configures P_limit_CG1 and P_limit_CG2 such that P_limit_CG1+P_limit_CG2 > P_limit_total, and if pwr_CG1+ pwr_CG2> P_limit_total, UE scales down transmission power of SCG transmission(s) such that UE transmission power across MCG and SCG does not exceed P_limit_total.

      - Note: if NW configures P_limit_CG1 and P_limit_CG2 such that P_limit_CG1+P_limit_CG2<= P_limit_total, the above SCG scaling behavior is not required.

    - P_limit_total is the power limit for all transmissions in both CG1 and CG2 and will be defined in RAN4

## R1-1909156 (Motorola Mobility, Lenovo)

- Proposal 1: [Alt 3]

  - UE is configured with a minimum guaranteed power (MGP) limits for MCG and SCG separately.

  - To compute the transmit power for CG1 UL transmission starting at time T0, a UE considers a power for later scheduled overlapping transmissions up to the cut-off time T0-T_offset

  - In case of a power-limited situation, the UE allocates power for an UL transmission by:

    - considering allocated powers for overlapping transmissions whose power are already determined – once the transmit power for a transmission is computed, UE will not re-calculate/re-adjust the transmit power based on upcoming transmission regardless of priority levels;

    - assigning power to higher priority, overlapping transmissions whose power is yet to be determined and scheduling information is known at the cut-off time (T0-T_offset) according to priority rules applied across the two CGs (Rel-15 CA power allocation prioritization rules + MCG>SCG for same priority level);

    - respecting the MGPs, such that total power on a CG1 can never exceed P_{NR-DC, Total} – P{CG2, min} (except when it is semi-statically known on no possible overlapping UL transmission on the other CG).

- Proposal 2:

  - In NN-DC, in the case of multiple transmissions overlapping with a given uplink transmission, MGP is defined as an RRC configured fraction of the dual connectivity Pcmax for the transmission with the highest L1 priority level among all overlapping transmissions whose transmission details are known to the UE before the cut-off time.

## R1-1909285 (Qualcomm)

- Proposal 1: Due to the extra complication for UE implementation as well as lack of performance benefit justification, dynamic power sharing should not be supported for Rel. 16 NN-DC.

- Proposal 2: For uplink power control of synchronous NN-DC, the UE is configured with:

  - A UE should be configured with a set of maximum allowed powers for each cell group such that P_NR1 + P_NR2 <= P_NN.

  - Additionally, the UE can be configured with P_NR1’ and P_NR2’, which can each be equal to P_NN. In other words, P_NR1’+P_NR2’ < = > P_NN.

From the UE side, the following behavior is expected:

- For the uplink transmission in cell group 1, the UE checks the semi-statically configured direction of the overlapping symbols of all serving cells of cell group 2 and vice versa. The UE chooses the maximum power allowed per cell group based on whether its uplink transmission collides with only semi-static ‘DL’ symbols, or with semi-static ‘UL’ and ‘flexible’ symbols.