3GPP TSG RAN WG1 #98		        R1-1909664

# Prague, CZ, August 26th – 30th, 2019

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Draft CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.213 | CR |  | rev | - | Current version: | 15.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on RA procedure triggered by higher layers |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ASUSTeK, Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  |  | Date: |  | 2019-08-28 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | When describing PRACH occasions for CSI-RS based RA procedure triggered by higher layers, the terminology used to exclude/describe CBRA case is not aligned with that in TS 38.331. The network can indicate UE to perform CBRA for handover by not configuring cfra in RACH-ConfigDedicated, instead of configuring RA preamble index 0. On the other hand, if CFRA-CSIRS-Resource is provided, a configured RA preamble index is always accompanied with a RA occasions list (including index 0). Hence, even RA preamble index 0 is indicated, there is still a RA occasion list to indicate at least one RRACH occasion for indicated CSI-RS. In addition, the description about PRACH occasions for SSB based CFRA for handover and the description about PRACH occasions indicated by dedicated RACH configuration are missing in 213. |  |  |  |  |  |  |  |  |
| Summary of change: |  | To align TS 38.331 wordings describing CBRA case for CSI-RS based RA procedure triggered by higher layers. To add description about PRACH occasions for SSB based CFRA for handover and description about PRACH occasions indicated by dedicated RACH configuration. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Unnecessary condition check for UE and unclear UE behavior. Unnecessary limitation, for network, on assigning RA preamble index for CSI-RS CFRA. Missing description about PRACH occasions for SSB based CFRA for handover and missing description about PRACH occasions indicated by dedicated RACH configuration. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  | Isolated impact analysis This CR aligns wordings describing RA triggered by higher layers between TS 38.213 and TS 38.331. Hence, it is noted that no new UE or network behavior is defined in this CR, and there is no impact on existing PRACH occasions and RPACH occasions indexing reset for CSI-RS based CFRA triggered by higher layers; there is no impact on existing PRACH occasions determination for SSB based CFRA triggered by higher layers and interpretation of PRACH occasions indicated by dedicated RACH configuration. - If it is not implemented in UE, when RA preamble index 0 and RA occasion list is configured for CSI-RS based RA procedure, UE’s understanding on whether to perform CBRA is not aligned with that of network. In addition, description about PRACH occasions for SSB based CFRA for handover and description about PRACH occasions indicated by dedicated RACH configuration are missing. - If it is not implemented in network, when RA preamble index 0 is configured for CSI-RS based RA procedure, network’s understanding on whether to perform CBRA is not aligned with that of UE. In addition, description about PRACH occasions for SSB based CFRA for handover and description about PRACH occasions indicated by dedicated RACH configuration are missing. |  |  |  |  |  |  |  |  |

## 8.1	Random access preamble

< Unchanged parts are omitted >

For link recovery, a UE is provided  SS/PBCH blocks associated with one PRACH occasion by ssb-perRACH-Occasion in BeamFailureRecoveryConfig. For a dedicated RACH configuration provided by RACH-ConfigDedicated, if cfra is provided, a UE is provided  SS/PBCH blocks associated with one PRACH occasion by ssb-perRACH-Occasion in occasions. If , one SS/PBCH block is mapped to  consecutive valid PRACH occasions. If , all consecutive  SS/PBCH blocks are associated with one PRACH occasion.

< Unchanged parts are omitted >

For a PRACH transmission triggered by a PDCCH order, the PRACH mask index field [5, TS 38.212], if the value of the random access preamble index field is not zero, indicates the PRACH occasion for the PRACH transmission where the PRACH occasions are associated with the SS/PBCH block index indicated by the SS/PBCH block index field of the PDCCH order.

For a PRACH transmission triggered by higher layers, if ssb-ResourceList is provided, the PRACH mask index is indicated by ra-ssb-OccasionMaskIndex which indicates the PRACH occasions for the PRACH transmission where the PRACH occasions are associated with the selected SS/PBCH block index.

The PRACH occasions are mapped consecutively per corresponding SS/PBCH block index. The indexing of the PRACH occasion indicated by the mask index value is reset per mapping cycle of consecutive PRACH occasions per SS/PBCH block index. The UE selects for a PRACH transmission the PRACH occasion indicated by PRACH mask index value for the indicated SS/PBCH block index in the first available mapping cycle.

For the indicated preamble index, the ordering of the PRACH occasions is

-	First, in increasing order of frequency resource indexes for frequency multiplexed PRACH occasions

-	Second, in increasing order of time resource indexes for time multiplexed PRACH occasions within a PRACH slot

-	Third, in increasing order of indexes for PRACH slots

For a PRACH transmission triggered upon request by higher layers, a value of ra-OccasionList [12, TS 38.331], if csirs-ResourceList is provided, indicates a list of PRACH occasions for the PRACH transmission where the PRACH occasions are associated with the selected CSI-RS index. The indexing of the PRACH occasions indicated by ra-OccasionList is reset per association pattern period.

< Unchanged parts are omitted >