3GPP TSG RAN WG1 Meeting #98	R1-1909693

Prague, Czech Republic, August 26th – 30th, 2019

| Source: | Nokia, Nokia Shanghai Bell |
| --- | --- |
| Title: | Simulation Assumptions for Multi-Satellite Evaluation |
| Document for: | Discussion and Decision |
| Agenda Item: | 7.2.5.1 |

# 1 Introduction

A new Study Item (SI) on “Study on Solutions for NR to Support Non-Terrestrial Networks” was approved in RAN#80 meeting [1] and further updated in RAN#82 meeting [2] and RAN#83 meeting [3] with the considered scenarios of transparent GEO satellite and transparent/regenerative LEO satellite (moving beam on earth) for pedestrian UEs and on board vehicle UEs in NTN. The objectives of this SI for physical layer are as follows.

- Consolidation of potential impacts as initially identified in TR 38.811 and identification of related solutions if needed [RAN1]:

  - Physical layer control procedures (e.g. CSI feedback, power control)

  - Uplink Timing advance/RACH procedure including PRACH sequence/format/message

  - Making retransmission mechanisms at the physical layer more delay-tolerant as appropriate. This may also include capability to deactivate the HARQ mechanisms.

- Performance assessment of NR in selected deployment scenarios (LEO based satellite access, GEO based satellite access) through link level (Radio link) and system level (cell) simulations [RAN1]

RAN1#96bis and RAN1#97 discussed the simulation assumptions of performance evaluation in NTN with the agreement summarized in [4][5] with the agreement below for multi-satellite evaluation.

- Both multi-satellite and single satellite simulations should be considered for calibration and performance evaluation.

In this contribution, we shared our views on the rest issues of multi-satellite evaluation in NTN for this SI.

# 2 Multi-Satellite Evaluation

2.1 Scenario

There are fewer satellites in one GEO constellation, for example total 3 GEO satellites. While there are tens of satellites or even hundreds of satellites in one LEO constellation. Therefore, it is proposed to prioritize LEO scenario in multi-satellite evaluation.

Proposal 1: Prioritize LEO scenarios in multi-satellite evaluation.

2.2 Satellite Deployment

Due to the bigger coverage of each satellite and potential massive beams per satellite, it is better to consider few satellites with partial beams per satellites for satellite-edge evaluation. In our opinion, it is fine to use 3 satellites for satellite-edge performance evaluation with the consideration of the trade-off between simulation complexity and performance reliability.

Proposal 2: Use 3 satellites for multi-satellite-based system-level evaluation.

Fig. 1 Satellite deployment in multi-satellite evaluation

In the 3-satellite evaluation, as shown in Fig. 1, it can be assumed that

- The elevation angle of the reference point to each satellite is same.

- These 3 satellites are uniformly distributed around this reference point.

Proposal 3: In multi-satellite evaluation, 3 satellites are uniformly distributed around the reference point with same elevation angle from the reference point to each satellite.

The satellite parameter Set-1 and Set-2 in TR 38.821 can be reused for multi-satellite evaluation. The elevation angle of the reference point to each satellite is provided in Table 1.

Table 1: Elevation angle of the reference point to each satellite in multi-satellite evaluation

| Satellite orbit | LEO-1200 |  | LEO-600 |  |
| --- | --- | --- | --- | --- |
| Satellite altitude | 1200 km |  | 600 km |  |
| Satellite parameter set | Set-1 | Set-2 | Set-1 | Set-2 |
| Elevation angle of the reference point to each satellite | 30 | 30 | 30 | 30 |

Proposal 4: Reuse the satellite parameter Set-1 and Set-2 in TR 38.821 for multi-satellite evaluation with additional parameter configuration provided in Table 1.

# 3 Conclusions

In this contribution, we share our views on simulation assumptions for multi-satellite performance evaluation for NTN with following proposals.

Proposal 1: Prioritize LEO scenarios in multi-satellite evaluation.

Proposal 2: Use 3 satellites for multi-satellite-based system-level evaluation.

Proposal 3: In multi-satellite evaluation, 3 satellites are uniformly distributed around the reference point with same elevation angle from the reference point to each satellite.

Proposal 4: Reuse the satellite parameter Set-1 and Set-2 in TR 38.821 for multi-satellite evaluation with additional parameter configuration provided in Table 1.

# References

- RP-181370, “Study on solutions for NR to support non-terrestrial networks (NTN)”, Thales, RAN#80, La Jolla, USA, June 11 - 14, 2018.

- RP-182444, “Study on solutions for NR to support non-terrestrial networks (NTN)”, Thales, RAN#82, Sorrento, Italy, December 10 - 13, 2018.

- RP-190710, “Study on solutions for NR to support non-terrestrial networks (NTN)”, Thales, RAN#83, Shenzhen, China, March 18 - 21, 2019.

- R1-1905921, “Report of RAN1#96bis meeting”, 3GPP RAN1#97, Reno, USA, May 13th – 17th, 2019.

- R1-190xxxx, “Report of RAN1#97 meeting”, 3GPP RAN1#98, Prague, Czech Republic, August 26th – 30th, 2019

- TR 38.811, “Study on New Radio (NR) to support non terrestrial networks”, 3GPP.

- TR 38.821, “Solutions on NR to support non-terrestrial networks”, 3GPP.