3GPP TSG RAN WG1 #98			R1-1909769

Prague, CZ, August 26th – 30th, 2019

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.213 | CR | 0054 | rev | - | Current version: | 15.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR to 38.213 on clarification of the RNTI used for scrambling a PUSCH transmission scheduled by RAR UL grant |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Spreadtrum communications |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN1 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  | Date: |  |  | 2019-08-29 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | In TS38.211, a reference specification TS38.214 is provided to determine the RNTI used for scrambling a PUSCH transmission as described in clause 6.3.1.1, but there are no related descriptions on the RNTI used for scrambling a PUSCH transmission scheduled by RAR UL grant captured in the TS 38.214, including the CBRA and the CFRA cases. For a PUSCH scheduled by RAR grant, if a TC-RNTI is provided by higher layers, the scrambling initialization of the PUSCH corresponding to the RAR UL grant is by TC-RNTI. Otherwise, the scrambling initialization of the PUSCH is by C-RNTI. Since the related operation of PUSCH scheduled by RAR UL grant has been captured in the clause 8.3 of TS38.213, to make the specification more clear, the descriptions on the RNTI used for scrambling a PUSCH scheduled by RAR UL grant is added to the TS38.213 accordingly. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add the related descriptions on the RNTI used for scrambling a PUSCH transmission scheduled by RAR grant in the specification TS38.213. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | - Incomplete RAN1 specification text that may cause confusion and incorrect implementation. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Isolated impact analysis: This CR completes incomplete specification text. There is no impact from this CR as it reflects common understanding in RAN1. |  |  |  |  |  |  |  |  |

| This CR's revision history: |  |
| --- | --- |

< Unchanged parts are omitted >

8.3	PUSCH scheduled by RAR UL grant

……

A SCS for the PUSCH transmission is provided by subcarrierSpacing in BWP-UplinkCommon. A UE transmits PRACH and the PUSCH on a same uplink carrier of a same serving cell.

A UE transmits a transport block in a PUSCH scheduled by a RAR UL grant in a corresponding RAR message using redundancy version number 0. If a TC-RNTI is provided by higher layers, the scrambling initialization of the PUSCH corresponding to the RAR UL grant in clause 8.2 is by TC-RNTI. Otherwise, the scrambling initialization of the PUSCH corresponding to the RAR UL grant in clause 8.2 is by C-RNTI. Msg3 PUSCH retransmissions, if any, of the transport block, are scheduled by a DCI format 0_0 with CRC scrambled by a TC-RNTI provided in the corresponding RAR message [11, TS 38.321]. The UE always transmits the PUSCH scheduled by a RAR UL grant without repetitions.

With reference to slots for a PUSCH transmission scheduled by a RAR UL grant, if a UE receives a PDSCH with a RAR message ending in slot  for a corresponding PRACH transmission from the UE, the UE transmits the PUSCH in slot , where  and  are provided in [6, TS 38.214].

The UE may assume a minimum time between the last symbol of a PDSCH reception conveying a RAR message with a RAR UL grant and the first symbol of a corresponding PUSCH transmission scheduled by the RAR UL grant is equal to  msec, where  is a time duration of  symbols corresponding to a PDSCH reception time for UE processing capability 1 when additional PDSCH DM-RS is configured,  is a time duration of  symbols corresponding to a PUSCH preparation time for UE processing capability 1 [6, TS 38.214] and, for determining the minimum time, the UE considers that  and  correspond to the smaller of the SCS configurations for the PDSCH and the PUSCH. For , the UE assumes  [6, TS 38.214].

< Unchanged parts are omitted >