3GPP TSG RAN WG1 Meeting RAN1#98	R1-1909790

Prague, Czech Republic, 26th-30th August 2019

Source:	Ericsson

Title:	TP related to Rel-16 TEI on DSS

Agenda Item:	7.2.14

Document for:	Discussion and Decision

# Introduction

In RAN1#98, the following was agreed as part of the Rel-16 TEI on DSS.

Agreements:

- For DSS TEI

  - Maximum number of LTE-CRS non-overlapping rate matching patterns within a NR carrier is X

    - X is three

  - Maximum number of LTE-CRS overlapping rate matching patterns within a part of NR carrier overlapping with a LTE carrier is Y

    - Y is two

  - Maximum number of LTE-CRS rate matching patterns in total within a NR carrier is Z

    - Z is six

    - UE capability for Z={[1,] 2, 3, 4, 5, 6} is defined

Note that there was also an agreement on DMRS positions for Type B scheduling of PDSCH of length 9 and 10 in the NR-U session. However, it is our understanding that these will be included by the editor when capturing NR-U agreements. Hence, only the TP related to the number of LTE CRS patterns are considered here.

# Multiple LTE-CRS rate matching patterns

It is assumed below that RAN2 will introduce a new RRC parameter, here denoted [X], that contains multiple LTE CRS rate matching patterns.

## Changes to TS 38.214

In 38.214, the corresponding changes to reflect the new RRC parameter [X]  is needed as follows

#### 5.1.4.2	PDSCH resource mapping with RE level granularity

A UE may be configured with any of the following higher layer parameters indicating REs declared as not available for PDSCH:

-	lte-CRS-ToMatchAround or [X]  in ServingCellConfig or ServingCellConfigCommon configuring common RS, in 15 kHz subcarrier spacing applicable only to 15 kHz subcarrier spacing PDSCH, of one or multiple LTE carriers in a serving cell. The configuration for each LTE carrier contains v-Shift consisting of LTE-CRS-vshift(s), nrofCRS-Ports consisting of LTE-CRS antenna ports 1, 2 or 4 ports, carrierFreqDL representing the LTE carrier centre subcarrier location determined by offset from (reference) point A, carrierBandwidthDL representing the LTE carrier bandwidth, and may also configure mbsfn-SubframeConfigList representing MBSFN subframe configuration of that carrier. A UE determines the CRS position within the slot according to Subclause 6.10.1.2 in [15, TS 36.211], where slot corresponds to LTE subframe.

## Changes to TS 38.211

Likewise, the corresponding changes to TS 38.211 are

#### 7.3.2.2	Control-resource set (CORESET)

[…]

for both interleaved and non-interleaved mapping, the UE may assume

-	the same precoding being used within a REG bundle if the higher-layer parameter precoderGranularity equals sameAsREG-bundle;

-	the same precoding being used across the all resource-element groups within the set of contiguous resource blocks in the CORESET, and that no resource elements in the CORESET overlap with an SSB or LTE cell-specific reference signals as indicated by the higher-layer parameter lte-CRS-ToMatchAround or [X], if the higher-layer parameter precoderGranularity equals allContiguousRBs.

##### 7.4.1.1.2	Mapping to physical resources

[…]

The position(s) of the DM-RS symbols is given by  and duration ld where

-	for PDSCH mapping type A, ld is the duration is between the first OFDM symbol of the slot and the last OFDM symbol of the scheduled PDSCH resources in the slot

-	for PDSCH mapping type B, ld is the duration is the number of OFDM symbols of the scheduled PDSCH resources

and according to Tables 7.4.1.1.2-3 and 7.4.1.1.2-4. The case dmrs-AdditionalPosition equals to 'pos3' is only supported when dmrs-TypeA-Position is equal to 'pos2'. For PDSCH mapping type A, ld=3 and ld=4 symbols in Tables 7.4.1.1.2-3 and 7.4.1.1.2-4 respectively is only applicable when dmrs-TypeA-Position is equal to 'pos2'. For PDSCH mapping Type A single-symbol DM-RS, l1=11 except if all of the following conditions are fulfilled in which case l1=12:

-	the higher-layer parameter lte-CRS-ToMatchAround or [X]  is configured and any PDSCH DM-RS symbol conincides with any symbol containing LTE cell-specific reference signals as indicated by the higher-layer parameter lte-CRS-ToMatchAround or [X]; and

-	the higher-layer parameters dmrs-AdditionalPosition is equal to 'pos1' and l0=3; and

-	the UE has indicated it is capable of additionalDMRS-DL-Alt

## Changes to TS 38.213

The 38.213 Section 10 requires this change:

[..]

For monitoring of a PDCCH candidate in a slot

-	If the UE has received ssb-PositionsInBurst in SIB1 and has not received ssb-PositionsInBurst in ServingCellConfigCommon for a serving cell and if the UE does not monitor PDCCH candidates in a Type0-PDCCH CSS set and at least one RE for a PDCCH candidate overlaps with at least one RE corresponding to a SS/PBCH block index provided by ssb-PositionsInBurst in SIB1, the UE is not required to monitor the PDCCH candidate.

-	If a UE has received ssb-PositionsInBurst in ServingCellConfigCommon for a serving cell and if the UE does not monitor PDCCH candidates in a Type0-PDCCH CSS set and at least one RE for a PDCCH candidate overlaps with at least one RE corresponding to a SS/PBCH block index provided by ssb-PositionsInBurst in ServingCellConfigCommon, the UE is not required to monitor the PDCCH candidate.

-	If the UE monitors the PDCCH candidate for a Type0-PDCCH CSS set on the serving cell according to the procedure described in Subclause 13, the UE may assume that no SS/PBCH block is transmitted in REs used for monitoring the PDCCH candidate on the serving cell.

-	If at least one RE of a PDCCH candidate on the serving cell overlaps with at least one RE of lte-CRS-ToMatchAround or [X], the UE is not required to monitor the PDCCH candidate.

[…]

The 38.213 Section 10.1 requires this change:

[…]

When precoderGranularity = allContiguousRBs, a UE does not expect

-	to be configured a set of resource blocks of a CORESET that includes more than four sub-sets of resource blocks that are not contiguous in frequency

-	any RE of a CORESET to overlap with any RE determined from lte-CRS-ToMatchAround or [X] or with any RE of a SS/PBCH block.