3GPP TSG RAN WG1 #98			R1-1909843

Prague, CZ, August 26th – 30th, 2019

Source:	NTT DOCOMO, INC.

Title:	Summary#3 on NR Rel-16 TEI related discussion

Agenda Item:	7.2.14

Document for: 	Discussion and Decision

- Introduction

This contribution summarizes the NR Rel-16 TEI related discussions and proposals in AI 7.2.14.

Following is captured in the RAN1 Chairman’s note.

R1-1909544 Summary on NR Rel-16 TEI related discussion NTT DOCOMO, INC. Agreements: - For DSS TEI - Maximum number of LTE-CRS non-overlapping rate matching patterns within a NR carrier is X - X is three - Maximum number of LTE-CRS overlapping rate matching patterns within a part of NR carrier overlapping with a LTE carrier is Y - Y is two - Maximum number of LTE-CRS rate matching patterns in total within a NR carrier is Z - Z is six - UE capability for Z={[1,] 2, 3, 4, 5, 6} is defined Continue offline discussion regarding details on the following two items: - CSI trigger states containing non-active BWP - Aperiodic CSI-RS Triggering for UE reporting beamSwitchTiming values of 224 and 336 R1-1909701 Agreements: TEI – “CSI trigger states containing non-active BWP” - When a UE is triggered with a CSI report for a DL BWP that is non-active, the UE is not expected to report the CSI for the non-active BWP and the CSI report associated with the BWP is omitted. - When a UE is triggered with aperiodic CSI-RS in a DL BWP that is non-active, the UE is not expected to measure the aperiodic CSI-RS. - The above non-active BWP is the non-active BWP when receiving the associated CSI-RS with the following relaxation for UE processing. - In the CC of the associated CSI-RS, if the active BWP when receiving the CSI-RS is different from the active BWP when receiving the triggering DCI - The last symbol of the PDCCH span of the DCI carrying the BWP switching shall be no later than the last symbol of the PDCCH span of the CSI trigger DCI, irrespective of whether they are in the same CC or not and irrespective of whether they are in the same SCS or not. - The UE is not expected to have any other BWP switching in that CC after the last symbol of the PDCCH span covering CSI trigger DCI and before the first symbol of the triggered CSI-RS resource. - Note: the UE is not required to measure P/SP-CSI-RS in the non-active BWP per current specification

- Offline outcome

Offline proposal:

- 48 is used as the beam switching threshold for UEs reporting 224 or 336

  - When/how to use reported value of 224 or 336 will also be clarified

    - When using the higher values of the feature (sym224 and sym336), beamSwitchTiming indicates the minimum number of OFDM symbols between the DCI triggering of aperiodic CSI-RS and aperiodic CSI-RS transmission in a CSI-RS resource configured with repetition ‘ON’ to apply TCI indication in CSI-RS triggering DCI.

R1-1909832	TP of TEI proposal on CSI trigger for non-active BWP	ZTE

R1-1909834	TP related to Rel-16 TEI on DSS	Ericsson

R1-1909845	[Draft] LS on NR Rel-16 TEI	NTT DOCOMO, INC.