3GPP TSG RAN WG1 Meeting #98			R1-1909861

Prague, Czech, August 26th – 30th, 2019

Title:	[Draft] LS response on NR V2X resource pool configuration and selection

Response to:	R1-1908004 (R2-1908486)

Release:	Rel-16

Work Item:	5G_V2X_NRSL-Core

Source:	Samsung, [RAN1]

To:	RAN2

Cc:	-

Contact Person:

Name:	Jeongho Yeo

E-mail Address:	jeongho7 dot yeo (at) samsung dot com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	None

1. Overall Description:

RAN1 thanks RAN2 for consulting with RAN1 with three questions on NR V2X resource pool configuration and selection. Regarding to RAN2’s questions to RAN1, the following includes RAN1’s answers.

Q1: Whether resource pool configuration based on zone is considered beneficial to support in NR V2X?

Answer: RAN1 didn’t conclude whether resource pool configuration based on zone is considered beneficial to support in NR V2X. There are three contributions showing that zone-based resource pool configuration is not beneficial. But, there are also views that the benefit of resource pool configuration based on zone depends on use cases.

Q2: Whether resource pool configuration based on different cast types should be supported by taking into account the configuration of PSFCH resource?

Answer: Regarding this question, RAN1 has made the following agreements.

Agreement in RAN1#96bis - It is supported, in a resource pool, that within the slots associated with the resource pool, PSFCH resources can be (pre)configured periodically with a period of N slot(s) - N is configurable, with the following values - 1 - At least one more value >1 - FFS details - The configuration should also include the possibility of no resource for PSFCH. In this case, HARQ feedback for all transmissions in the resource pool is disabled - HARQ feedback for transmissions in a resource pool can only be sent on PSFCH in the same resource pool Agreement in RAN1#97 - For the period of N slot(s) of PSFCH resource, N=2 and N=4 are additionally supported. Agreement in RAN1#98 - In physical layer perspective, a (pre-)configured resource pool can be used for all of unicast, groupcast, and broadcast for a given UE. - There is no (pre-)configuration to inform which cast types are used for the resource pool.

Q3: Whether an NR V2X mode 2 UE can select multiple resource pools on single carrier from RAN1 perspective?

Answer: RAN1 has concluded that,

- Multiple resource pools for transmission and reception can be (pre-)configured to a UE.

- In mode 2, the UE can choose one and only one of the (pre-)configured resource pools for transmission at a time.

- A UE needs to receive in multiple resource pools.

2. Actions:

To RAN2:

RAN1 kindly asks RAN2 to take the above response into account for RAN2’s future works.

3. Date of Next RAN1 Meetings:

TSG-RAN WG1 Meeting 98bis 	14 – 20 October 2019			Chongqing, China

TSG-RAN WG1 Meeting 99	18 – 22 November 2019 	 		Reno, USA