# Agreements upto RAN1#98

UL full power transmission scheme

Full TX power UL transmission with multiple power amplifier is supported at least for codebook based UL transmission for non-coherent and partial/non-coherent capable UEs

- This specification support is a UE optional feature

Note: UE capability 1, 2, 3 agreed in RAN1#AH1901 mean the PA architectures.

- UE capability 1: for the UE to support full Tx power in UL transmission, full rated PAs on each Tx chain is supported with a new UE capability

- UE capability 2: for the UE to support full Tx power in UL transmission, no Tx chain is assumed to deliver full power with the new UE capability

- UE capability 3: for the UE to support full Tx power in UL transmission, subset of Tx chains with full rated PAs is supported with a new UE capability

Support following scheme for UL full power Tx for UE capability 2 and 3:

- A UE can be configured for one of two modes of full power operation to support ‘Capability 2’ and ‘Capability 3’ subject to UE capability

- A UE can be configured by the network to support full power transmission

- Mode 1: The UE can be configured with one or more SRS resources with same number of SRS ports (according to Rel-15) within an SRS resource set which usage is set to ‘codebook’

  - gNB can configure the UE to use a subset of TPMIs that combine ports in a layer to produce full power transmission.

  - A new codebookSubset is introduced only for the rank value(s) where full power transmission in UL is not achievable includes the TPMI precoders in fullyAndPartialAndNonCoherent defined in Rel-15

    - For mode 1, 2Tx non-coherent UE, the new codebook subset at least includes rank=1 TPMI=2 defined in Rel-15 which can be used for UL full power transmission

    - For mode 1, 4Tx non-coherent UE, the new codebook subset at least includes, rank 1 TPMI= 13 defined in Rel-15, at least includes, rank 2 TPMI=6 defined in Rel-15, at least includes, rank 3 TPMI=1 defined in Rel-15 which can be used for UL full power transmission

- Mode 2: The UE can be configured with one SRS resource or multiple SRS resources with different number of SRS ports within a SRS resource set which usage is set to ‘codebook’

  - UE transmits SRS and PUSCH in same manner, whether antenna virtualization is used or not

  - Rel-15 codebooks and codebook subsets are used

  - Note: Antenna selection precoder can be used to enable full power related PA(s) to produce full power transmission for Capability-3 UE.

  - UL full power Tx is achieved for PUSCH transmission according to indicated SRI and/or TPMI

    - For mode 2, in case of non-coherent with 2 ports, support following TPMI indication for rank 1 which support UL full power transmission:

      - Rank 1: support {TPMI=0} and {TPMI=1}

  - For 4 TX UEs, a maximum of 4 SRS resources are supported in Mode 2 for usage set to ‘codebook’ in a set

    - Depending on UE capability, either up to 2 or 4 SRS resources are supported

  - For 2 TX UEs, a maximum of 4 SRS resources are supported in Mode 2 for usage set to ‘codebook’ in a set

    - Depending on UE capability, either up to 2 or 4 SRS resources are supported

  - For mode 2 UEs, up to 2 different spatial relation info can be configured for all SRS resources with usage set to ‘codebook’

Power scaling

For the the 2TX and 4TX case, the linear value of power after power scaling, is divided equally among the non-zero PUSCH ports

- The above applies for the cases including when UE transmitting at P_c_max

For a capability 1 UE working with full power operations, for PUSCH power control, power scaling factor is fixed to 1

UE capability

- At least for PC3, UE capability 1, 3 can support full power transmission.

- Regardless of UE capability 1, 2, or 3, signaling of “UL full power tx capability” is supported for UEs with full power uplink transmission capability

- The signaling of above information does not imply any specific UE PA architecture implementation.

- Note: Support of Mode 1, Mode 2 have separate UE capability

- Note: UE capability 1, 2, 3 agreed in RAN1#AH1901 mean the PA architectures.

- A set of TPMIs that deliver full power can be signaled by the UE in order to support at least UEcap3, for SRS resource with more than 1 ports