3GPP TSG RAN WG1 Meeting #98bis	                                                                          R1-1909920

Chongqing, China

October 14th – October 20th, 2019

Agenda item:    7.2.2

Source:             Qualcomm Incorporated

Title:                  Agreements in NR-U WI up to RAN1 #98

Document for:  FYI

# Introduction

This paper summarizes the agreements reached during NR-U work item phase up to RAN1 #98.

# 7.2.2	NR-based Access to Unlicensed Spectrum

### RAN1 #AH-1901, Jan 2019

### RAN1 #96, Feb 2019

R1-1903576	[DRAFT] LS to ETSI BRAN on EN 301 893	Qualcomm Incorporated

Final LS is agreed in R1-1903757 with the following modifications:

- Replace the last sentence in the third paragraph with “Hence, it is RAN1’s understanding that for the case where more than one grant is provided to a responding device, 3GPP Rel-14/15 LTE-LAA specifications allow behaviour that is consistent with the provided clarifications and this is also expected to be the case for NR-U specifications currently being developed.”

- Replace the last two sentences of the second last paragraph with, “There is no consensus on the proposal to (1) ban the use of no LBT transmissions and (2) to restrict the use of short LBT transmissions so that it can only be used 1% of time rather than 5% as currently defined in the clause on Short Control Signalling Transmissions. There is also no consensus on limiting NR-U DRS transmissions to a 1% limit. Furthermore, reducing 5% transmission limit to 1% transmission limit would not be consistent with the behaviour of a device implementing LTE-LAA and is not preferred.”

- Modify the action as “RAN1 kindly requests ETSI TC BRAN to take the above information and requests for clarifications into account.”

### RAN1 #96b, April 2019

R1-1905748	Draft Response LS on short control signaling in NR-U	Qualcomm Incorporated

Final LS approved in R1-1905860

### RAN1 #97, May 2019

### RAN1 #98, August 2019

## 7.2.2.1	 Physical Layer Signals and Channels

### 7.2.2.1.1 Initial access signals/channels

#### RAN1 #AH-1901, Jan 2019

Agreement:

- UE assumes 30KHz SCS for SS/PBCH block for 5GHz band and 6GHz band if the SCS is not indicated by higher layers.

- Support configuration by higher layers of 15KHz or 30KHz SCS for SS/PBCH block

- Include this agreement in a LS to RAN4 (cc RAN2) for inclusion in specs managed by RAN4

Conclusion:

No changes are required to the time and frequency position of the PSS/SSS/PBCH relative to each other in one PSS/SSS/PBCH block.

Agreement:

The Type0-PDCCH monitoring configuration for NR-U should satisfy at least the following properties:

- TDM of Type0-PDCCH and SSB similar to existing pattern 1 (already agreed)

- Support the monitoring of Type0 PDCCH of the 2nd SSB position in a slot in the gap between 1st and 2nd SSB within the slot

  - FFS start at symbol #6 of #7 or both

- FFS: The Type0-PDCCH candidates associated with an SSB are confined within a slot carrying the associated SSB (with the same QCL assumptions)

Agreement:

Companies are encouraged to provide results comparing the different alternatives using the following simulation assumptions to select between alternative PRACH designs.

- The Rel-15 PRACH design should be simulated as a baseline

| Property | Value |
| --- | --- |
| Carrier frequency | 5 GHz |
| Channel model | TDL-C |
| Delay scaling | 10ns, 100 ns |
| Antenna configuration at BS(1) | (M,N,P) = (1,1,2) with omni-directional antenna element |
| Antenna configuration at UE | Single omni-directional antenna element |
| Antenna port virtualization | No beamforming and no beam selection |
| Frequency offset | 0.05ppm (fixed) at TRP, and 0.1 ppm (fixed) at UE |
| UE speed | 3 km/h |
| Initial timing offset | Uniformly distributed in [0, 1.2 µs (corresponding to 300 m ISD)] Optional: Uniformly distributed in [0, 2 µs (corresponding to 500 m ISD)] |
| PRACH format | A1 with other formats optional |
| Subcarrier spacing | 15/30 kHz. (with other SCS optional) |
| PRACH sequence and frequency resource allocation | For evaluation purpose, the Rel-15 PRACH ZC sequence (with possible length change) should be simulated. Additional/new sequences can be simulated. Each company should provide details on the sequence (type and length) and the resource allocation (e.g., Alt1~Alt4 and detailed mapping). |
| Total number of preambles per cell | 64, each company should provide details on how these 64 preambles are generated |
| Preamble detector | Each company should provide details on used algorithm |
| Interference assumption | No interference. Optional: -3/0/3dB interference power compared with target PRACH |
| Detection Criteria | 1% maximum mis-detection probability(2) |
|  | 0.1% maximum false alarm probability(3) |
|  | maximum timing estimation error being 50% of the normal CP length |
| Formatting of results (please also reference Section 8 of R1-1704144 for reporting formats) | Mis-detection probability vs. SNR |
|  | False alarm probability vs. SNR(4) |
|  | CDF of timing estimation error |
|  | PRACH capacity (maximum number of preambles) |
|  | Peak-to-average power ratio and cubic metric |
|  | MCL(5) |
| (1) See Table 7-1 of R1-1704144 (2) The missed detection probability is defined as the ratio between the total number of transmitted preambles that are either not detected, or detected as a different preamble, or detected but with timing error greater than the maximum value (i.e., 50% of normal CP length), and the total number of transmitted preambles within an observation interval. (3) Maximum false alarm probability refers to the case when input at receiver is noise only (considering 64 preamble detectors as in 3GPP TS 36.104, section 8.4.1). (4) False alarm probability is defined as the ratio of total number detected but not transmitted preambles, and the total number of possible detection occurrences, where each occurrence (occurrence refers to 64 detections, one for each of the 64 preambles in a cell) is one potential preamble transmission in a RO. (5) In the MCL calculation, needs to consider the maximum transmit power supported by the PRACH design under PSD limitation and PAPR/EVM characteristic of the design. Note: Assumptions on the following should be stated - use of a guard band (if any) - definition of SNR - signal bandwidth used |  |

#### RAN1 #96, Feb 2019

Agreement:

- Down-select from the following options for SSB pattern (symbol index starts at 0)

  - Option 1: SSBs are at symbols (2,3,4,5) and (8,9,10,11) in the slot

  - Option 2: SSBs are at symbols (2,3,4,5) and (9,10,11,12) in the slot

- The down-selected pattern applies no matter if SSB SCS is indicated by higher layer or not, and no matter if RMSI is transmitted or not.

Agreement:

- The SCS for all SSBs and Coreset #0 on a carrier is always the same for operation of NR in unlicensed spectrum.

- CORESET #0 frequency domain resource configuration should be 48 RBs for 30KHz SCS and 96 RBs for 15KHz SCS.

Agreement:

For PRACH evaluations, following metrics should be provided:

| Parameter | Value | Notes |
| --- | --- | --- |
| Scheme |  | Eg. Alt4-ZC139x2 |
| SCS |  | 15KHz or 30KHz |
| PRACH sequence length (L_RA) |  | Eg. 139, |
| # of repetition (R) |  | If repetition of sequence is used in freq domain |
| N_cs |  | Eg. 11 |
| # of RBs used for one RO (N_RB) |  | # of RBs occupied by PRACH. Eg. 12 for ZC139 design |
| # of interlaces used by one RO (N_interlace) |  | # of uniform interlaces (M=5 for 30KHz and M=10 for 15KHz) with RBs used for one PRACH RO |
| RACH frequency occupancy (MHz) |  | The actually used bandwidth with one RO, SCS*L_RA*R |
| Noise level, Np (dBm) |  | Np= -174+10*log10(SCS*L_RA*R)+NF NF=-5dB |
| SNR (dB) |  | SNR needed at 1% misdetection, read from simulation curve |
| P_max (dBm) |  | Maximum allowed transmit power under PSD limit of 10dBm/MHz measured in any 1MHz chunk and considers the RBs used by the proposed scheme |
| Backoff (dB) |  | Backoff is computed as 95% percentile of CCDF of [cubic metric] over the preambles in the RO. Note: If cubic metric is not used, information on the backoff metric used should be provided. |
| P_TX (dBm) |  | P_TX=min(P_max, 23- Backoff) is maximum allowed transmit power for the waveform considering backoff |
| MCL (dB) |  | MCL = P_TX-SNR-Np |
| N_FDM |  | # of ROs in 20MHz |
| Capacity |  | Across all ROs in 20MHz. Should report any constraints on ISD for the scheme evaluated. |

#### RAN1 #96bis, April 2019

Agreement:

PRB/RE-interlaced PRACH is not considered further. Consider the following alternatives should be studied further as options.

- Alt 1: Legacy NR PRACH sequence of length 139 mapped to contiguous subcarriers, with repetitions in frequency

  - FFS: Guard bands between repetitions

  - FFS: Number of repetitions

  - FFS: Whether repetitions are constrained to be contiguous in frequency or not

- Alt 2: A single PRACH sequence mapped to contiguous PRBs according to one of the following alternatives

  - Alt 2.1: ZC sequence with longer length than 139

  - Alt 2.2: New sequence with longer length than 139

Agreement:

Only coreset #0 lengths of 1 and 2 symbols are supported for NR-U

Agreement:

Select one of the following options in RAN1 #97:

- Alt 1: Legacy SSB positions in a slot

  - Support Type0-PDCCH in symbol (#0, #1) for length-2 coreset 0 and symbol (#0) for length-1 coreset 0 at least for the first SSB in a slot

  - Support Type0-PDCCH in symbol (#6, #7) for length-2 coreset 0 and symbol #6 or symbol #7 for length-1 coreset 0 for the second SSB in a slot

    - FFS: configurable between symbols #6 and #7 for the length-1 coreset 0 for the second SSB in a slot

- Alt2: New SSB positions in a slot

  - Support Type0-PDCCH in symbol (#0, #1) for length-2 coreset 0 and symbol (#0) for length-1 coreset 0 for the first SSB in a slot

  - Support Type0-PDCCH in symbol (#7, #8) for length-2 coreset 0 and symbol (#7) for length-1 coreset 0 for the second SSB in a slot

- Alt 3: Legacy SSB positions in a slot

  - Support Type0-PDCCH in symbol (#0, #1) for length-2 coreset 0 and symbol (#0) for length-1 coreset 0 for the first SSB in a slot

  - Support Type0-PDCCH in symbol (#7) for coreset 0 for the second SSB in a slot

#### RAN1 #97, May 2019

Agreement:

For SSB positions and type-0 PDCCH monitoring in a slot, only Alternatives 1 and 2 (from the previous agreement in RAN1#96bis) are considered further.

Note: As per the previous agreement one of Alternative 1 and 2 is to be selected at RAN1#97

Conclusion:

There is no consensus between Alternative 1 and Alternative 2 on the topic of SSB positions and type-0 PDCCH monitoring in a slot. No further online or scheduled offline discussions will be conducted on this topic. If there is consensus on such enhancements, the topic can be revisited in the Rel-16 NR-U work item.

R1-1907884	[DRAFT] LS to RAN4 on impact of sync raster placement for NR-U in RAN1	Qualcomm

The final LS is approved in R1-1907896 with the main body of the LS changed to the following:

“RAN1 understands RAN4 is discussing the sync raster design for NR-U band in 5GHz with the options of placing the sync raster either in approximately the middle of each 20MHz channel, or at approximately the edge of each 20MHz channel. RAN1 evaluated the impact to RAN1 if either of the options are adopted in RAN4.

From a RAN1 perspective, in unlicensed bands, where the DL initial BWP is nominally 20 MHz, RAN4 decisions on sync raster points have consequences on RAN1 design. RAN1 has discussed these consequences.

- If a SSB is not close to the edge, the number of RBs available to carry SIB1 would be limited although further enhancement to introduce rate matching around the SSBs to address this limitation are being discussed in RAN1

- If a SSB is close to the edge, the number of RBs available to carry SIB1 would be less limited although whether or not further enhancement to introduce rate matching around the SSBs is needed is being discussed in RAN1”

Agreement:

For a new enhanced design of NR-U PRACH in addition to the Rel-15 design (sequence length of 139) further discussion is limited to the following options

- ZC sequence of the following lengths

- 15 kHz: Choose one of L_RA=[571, 1151]

- 30 kHz: Choose one of L_RA=[283, 571]

- Repetition of Rel-15 PRACH sequences in frequency domain with potentially some mechanisms to improve the cubic metric

  - Consider one of 2 and 4 repetitions for 30 kHz and one of 4 and 8 repetitions for 15 kHz

- Note: Decision will be based on previously agreed evaluation metrics, capacity per cell (i.e., number of preambles per RACH occasion and number of RACH occasions) for the same time and frequency resources, specification impact and implementation complexity.

- Note: Companies should state any deviations in assumptions from the agreed evaluation assumptions.

#### RAN1 #98, August 2019

Agreement:

No new PRACH formats are specified for operation in unlicensed spectrum.

Agreement:

To support RMSI transmission for ANR purpose on a carrier with an SSB not on a sync raster consider the following alternatives:

- Alt 1: gNB configures the UE to report the CGI for a PCI on a given SSB frequency not on a sync raster point. After the UE detects the SSB it will proceed with RMSI decoding whereby the frequency location of the CORESET #0 scheduling the PDSCH carrying the RMSI is implicitly known.

- Alt 2: gNB configures the UE to report the CGI for a PCI on a given SSB frequency not on a sync raster point and the MIB in SSB will point to the frequency location of the coreset #0.

  - FFS: number of CORESET 0 offsets configurable in MIB when SSB is not on sync raster entry

  - Note: Signalling of offset only necessary if more than one off-sync-raster point offset is agreed

### 7.2.2.1.2	DL signals and channels

#### RAN1 #AH-1901, Jan 2019

Agreement:

- The UE may assume the presence of a signal, such as the DMRS in any [PDCCH or GC-PDCCH] transmission, to detect transmission bursts by the serving gNB, to enable power saving by not necessitating performing blind decodes to detect the transmission burst (Note: The power saving possibility by not necessitating blind decodes assumes performance relaxation for PDCCH decoding is not needed. Also, this does not mandate a two-step PDCCH decoding process for the UE with respect to DMRS detection).

  - If a preamble transmitted at the start of a burst is agreed to be specified (this does not preclude usage of preambles by implementation), it may be used in addition to the DMRS of PDCCH/GC-PDCCH or any other signals in the gNB transmission to detect the start of transmission bursts by the serving gNB and potentially for power saving

    - Note: Whether a preamble, if defined, can be used for power saving in all cases depends on the details of the design.

  - Note: Other signals present in the transmission burst may also be used for the purpose of detection of the transmission burst

  - FFS: Potential enhancements to DMRS design to address issues with detection probability

- The payload of a PDCCH and/or GC-PDCCH transmission can contain information regarding COT structure that may be used by the UE for power saving

#### RAN1 #96, Feb 2019

#### RAN1 #96bis, April 2019

Agreement:

Only further discuss/consider Option 3 (PDSCH mapping type B with durations other than 2/4/7 symbols) as potential enhancements to PDSCH transmissions.

#### RAN1 #97, May 2019

Agreement:

- Additional durations from 2 to 13 (other than 2, 4 and 7 which are already supported in Rel-15) for PDSCH mapping type B are supported.

  - FFS: A duration of 14

  - FFS: DMRS aspects for the additional durations (Reuse Rel-15 patterns or define new patterns for these additional durations)

- Capability signalling will be defined for UEs to indicate which specific subset of durations are supported by the UE.

- FFS: Some duration(s) are mandatory for the UE to support for initial access

- FFS: PDSCH processing times for the new durations (reuse the values for durations of 2/4/7 or define new ones)

- Note: PDCCH monitoring occasions will be discussed separately as part of the NR-U WI

- Note: This agreement does not by itself imply an increased requirement in PDSCH processing and decoding capabilities compared to Rel-15

#### RAN1 #98, August 2019

Agreement:

- The following is agreed for DM-RS positions for PDSCH mapping Type B of length 9 and 10:

| ld in symbols | DM-RS positions |  |  |  |
| --- | --- | --- | --- | --- |
|  | PDSCH mapping type B |  |  |  |
|  | dmrs-AdditionalPosition |  |  |  |
|  | 0 | 1 | 2 | 3 |
| 9 |  | , 7 | ,4,7 | ,4,7 |
| 10 |  | , 7 | ,4,7 | ,4,7 |

- If the PDSCH duration ld is 10 OFDM symbols for normal cyclic prefix, and if at least one of the DM-RS symbols corresponding to the PDSCH allocation collides with resources reserved for at least one of the symbols of a LTE CRS transmission, shall be incremented by 1

  - FFS: Whether the increment is applicable to all slots or a subset of slots

Conclusion:

- PDSCH type B durations 9,10 will not be optimised for double symbol DM-RS for avoiding LTE-CRS collisions

Agreement:

Support bit field corresponding to available LBT bandwidths in GC-PDCCH (add a bitmap in the GC-PDCCH DCI)

### 7.2.2.1.3	UL signals and channels

#### RAN1 #AH-1901, Jan 2019

Agreement:

For interlace transmission of at least PUSCH and PUCCH, the following PRB-based interlace design is supported for the case of 20 MHz carrier bandwidth:

a.	15 kHz SCS: M = 10 interlaces with N = 10 or 11 PRBs / interlace

b.	30 kHz SCS: M = 5 interlaces with N = 10 or 11 PRBs / interlace

Note: PRACH design to be considered separately, including multiplexing aspects with PUSCH and PUCCH

Working assumption:

- For a given SCS, the following interlace design is supported at least for PUSCH:

  - Same spacing (M) between consecutive PRBs in an interlace for all interlaces regardless of carrier BW, i.e., the number of PRBs per interlace is dependent on the carrier bandwidth

  - Point A is the reference for the interlace definition

- For 15 kHz SCS, M = 10 interlaces and for 30 kHz SCS, M = 5 interlaces for all bandwidths

- FFS: Interlace design for PUCCH for bandwidths greater than 20 MHz

- FFS: Whether and how partial interlace allocation is supported

#### RAN1 #96, Feb 2019

Agreement:

- Support short and long PUCCH durations based on enhancements of at least Rel-15 PUCCH formats PF2 and PF3. The enhancements include at least the following aspects:

  - For a 20 MHz carrier bandwidth, support mapping to physical resources of at least one full interlace

  - Mechanism to support user multiplexing for both data and reference symbols of PUCCH

  - The following aspects are FFS:

    - Support for small payloads (1 and 2 bits)

      - Alt-1: Support both small payloads and larger payloads (> 2 bits) for enhanced PF2 and enhanced PF3

      - Alt-2: Small payloads are supported by enhanced PF0 and/or enhanced PF1

    - Whether or not to replace DFT-s-OFDM with CP-OFDM for the enhanced PF3

Agreement:

Suppport configuration of an SRS resource in additional OFDM symbol locations other than the last 6 symbols of a slot with PUSCH and SRS time division multiplexed as in Rel-15.

- FFS: which symbols locations.

Agreement:

Sub-PRB interlace design for PUSCH and PUCCH is not supported.

#### RAN1 #96bis, April 2019

Agreement:

For a 20 MHz carrier bandwidth, if enhancements to PF0 and PF1 are supported, a mapping to physical resources of at least one full interlace is supported

- FFS: Whether or not to support enhancements to PF0/1.

- Companies are encouraged to provide user multiplexing capacity and UCI payload analysis for enabling the decision for relevant use cases

Agreement:

Support RRC configuration of an SRS resource to start at any OFDM symbol within a slot by extending the RRC parameter startPosition of resourceMapping of SRS-Config for Rel-16 to have a value range 0..13.

Agreement:

Decisions on which additional PUCCH formats (enhanced or combination of legacy and enhanced) are supported should be at least based on the following.

- Which PUCCH format(s) are to be used at least for the following use cases:

  - HARQ ACK prior to dedicated PUCCH resource configuration

  - HARQ ACK, SR, CSI and combinations thereof after dedicated PUCCH resource configuration

- Specification impact, e.g., UE procedures in 38.213 and 38.212, for all proposed PUCCH formats to be supported

- User multiplexing capacity and UCI payload analysis for all proposed PUCCH formats to be supported

- In-band and out-of-band emissions

Agreement:

The following common set of evaluation assumptions and reporting metrics are used for enhanced PUCCH design

Evaluation assumptions

| Property | Value |
| --- | --- |
| Carrier frequency | 5 GHz |
| Channel bandwidth | 20 MHz |
| Channel model | TDL-C |
| Delay scaling | At least 10ns, 100ns with 300 ns optional |
| Antenna configuration at BS* | (M,N,P) = (1,1,2) with omni-directional antenna element |
| Antenna configuration at UE | Single omni-directional antenna element |
| Antenna port virtualization | No beamforming and no beam selection |
| Frequency offset | 0 ppm |
| UE speed | 3 km/h |
| Subcarrier spacing | 15/30 kHz (with other SCS optional) |
| Number of code-division multiplexed users if applicable | 1 user, with other cases optional |
| Interference assumption | No inter-cell interference |
| * See Table 7-1 of R1-1704144 |  |

Reporting metrics

| Parameter | Value | Notes |
| --- | --- | --- |
| Enhanced PUCCH Format |  | e.g., E-PF0, E-PF1, E-PF2, E-PF3 |
| Number of OFDM symbols used for PUCCH resource |  | e.g., 1, 2, 4, 14 |
| Number of RBs used for PUCCH resource (N_RB) |  | At least 1 full interlace assumed (Assume interlace design for 20 MHz carrier bandwidth as agreed in RAN1AH#1901 for 15kH and 30 kHz is used) |
| Frequency domain OCC configuration details (if applicable) |  | Include length and type of OCC, mapping of OCCs to control symbols and reference (DMRS) symbols, OCC cycling (if applicable) |
| Time domain OCC configuration details (if applicable) |  | Include length and type of OCC, mapping of OCCs to control symbols and reference (DMRS) symbols |
| Number of multiplexed users, e.g., by code division if applicable |  | 1 user is assumed as baseline Companies are to report other cases if evaluated |
| Waveform |  | e.g., CP-OFDM or DFT-s-OFDM |
| PUCCH encoder type |  | e.g., Reed Muller or Polar |
| SCS |  | 15KHz, 30KHz |
| Noise level, Np (dBm) |  | Np = -174 + 10*log10(SCS*12*N_RB) + NF NF = 5dB |
| Required SNR (dB) |  | Required SNR needed to fulfil detection criterion(1)(2), read from simulation curve |
| Cubic Metric |  | Note: Single value reported in dB |
| P_max (dBm) |  | Maximum allowed transmit power under PSD limit of 10dBm/MHz measured in any 1MHz chunk and considers the RBs used for the PUCCH resource |
| Backoff (dB) |  | Backoff is computed as the cubic metric. Note: If cubic metric is not used, information on the backoff metric used should be provided. |
| P_TX (dBm) |  | P_TX = min(P_max, 23- Backoff) is maximum allowed transmit power for the waveform considering backoff |
| MCL (dB) |  | MCL = P_TX – SNR – Np |
| PUCCH payload size(s) (bits) |  | If multiple payload sizes evaluated, then MCL to be plotted vs. PUCCH payload size |
| PUCCH encoding rate(s) |  | If multiple payload sizes evaluated, then multiple encoding rates to be reported (if applicable) |
| - For PUCCH payloads of 1 or 2 bits, detection criterion assumes that the PUCCH payload consists of randomly drawn HARQ ACK/NACK bits and is defined as P(ACK to Error) ≤ 1% and P(NACK to ACK) ≤ 0.1%. Error is defined as NACK or DTX where the decision region for DTX is determined to ensure that the maximum P(DTX to ACK) ≤ 1% for the case when the input to the receiver is noise only. - For PUCCH payloads greater than 2 bits follow the requirements in TS38.104 Section 8.3 Note: - Companies should provide details of RE mapping in frequency and time of enhanced PUCCH formats that are evaluated - Companies should provide details about channel estimation assumptions, e.g. practical, ideal - Definition of SNR to be reported with evaluation metrics |  |  |

#### RAN1 #97, May 2019

Agreement:

Support enhancement of Rel-15 PUCCH formats PF0 and PF1 as follows:

- Mapping to physical resources of one full interlace in 20 MHz.

  - FFS: Sequence type and mapping considering the following alternatives:

    - Alt-1: Repetition of the length-12 Rel-15 PF0 and PF1 sequence in each PRB of an interlace with mechanism to control PAPR/CM considering the following alternatives

      - Alt-1a: Cycling of cyclic shifts across PRBs

      - Alt-1b: Phase rotation across PRBs of an interlace where the phase rotation is can be per RE or per PRB

    - Alt-2: Mapping of different length-12 Rel-15 PF0 and PF1 sequences to the PRBs of an interlace based on different group number u (range is 0 .. 29)

    - Alt-3: Mapping of a single long sequence to the PRBs of an interlace

  - FFS: Impact due to guardbands

- Note: Decisions on the above should be based on at least performance using the agreed MCL metric and specification impact

- Note: Interlaced PF2 and 3 are not enhanced to support 1-2 bit payloads

Agreement:

For enhanced Rel-15 PF3 supporting interlaced mapping, do not replace DFT-s-OFDM with CP-OFDM

#### RAN1 #98, August 2019

Agreement:

The working assumption from RAN1 AH1901 is converted to an agreement with the following modifications:

- For a given SCS, the following PRB-based interlace design is supported at least for PUSCH and PUCCH:

  - Same spacing (M) between consecutive PRBs in an interlace for all interlaces regardless of carrier BW, i.e., the number of PRBs per interlace is dependent on the carrier bandwidth

  - Point A is the reference for the interlace definition

- For 15 kHz SCS, M = 10 interlaces and for 30 kHz SCS, M = 5 interlaces for all bandwidths

- FFS: Interlace design for PUCCH for bandwidths greater than 20 MHz

- FFS: Whether and how partial interlace allocation is supported considering mechanisms specific to PUSCH and PUCCH

- FFS: PUCCH bandwidth

- FFS: Whether or how an interlace design for PUSCH and/or PUCCH is supported on 10 MHz according to the revised WID objective

Agreement:

Alt-1a (Cycling of cyclic shifts across PRBs of the interlace) is selected from the four alternatives in the RAN1#97 agreement on enhanced Rel-15 PUCCH formats PF0 and PF1

- FFS: Cyclic shift ordering

Agreement:

A bandwidth occupied by a PUCCH resource does not exceed the bandwidth corresponding to a 20 MHz carrier/LBT bandwidth

Agreement:

- For interlaced PUSCH transmission in a BWP, X bits of the PUSCH frequency domain resource allocation field are used for indicating which combination of M interlaces is allocated to the UE.

- This applies to PUSCH of the following types:

  - Msg3 PUSCH

  - PUSCH Scheduled by fallback and non-fallback DCI

  - Type 1 and Type 2 Configured Grant PUSCH

- For 30 kHz SCS

  - Support X = 5 (5-bit bitmap to indicate all possible interlace combinations)

- For 15 kHz SCS

  - Down-select between the following two alternatives:

  - Alt-1: Support X = 10 (10-bit bitmap to indicate all possible interlace combinations)

  - Alt-2: Support X = 6 bits to indicate start interlace index and number of contiguous interlace indices (RIV) and using remaining up to 9 RIV values to indicate specific pre-defined interlace combinations

Conclusion:

For 10 MHz carrier bandwidth, enhancements to Rel-15 UL signals and channels are not necessary.

Agreement:

A PUCCH resource configured with interleaved mapping occupies consecutive PRBs within at least one interlace within a BWP. The PUCCH resource configuration includes the following:

- An indication of the allocated interlace

- An indication of the location of the PUCCH resource within the allocated interlace

  - Note: This may not be needed for a bandwidth part of 20 MHz or less

- The number of PRBs NPUCCH within the allocated interlace given by the following:

  - For Interlaced PF0/1/2:

    - NPUCCH = 10 or 11 depending on the allocated interlace

  - For Interlaced PF3:

    - NPUCCH = 10

- FFS: Whether/how an interlaced PF2/3 resource can be configured on 2 interlaces to increase the number of allocated PRBs to 20, 21, or 22 depending on the allocated interlaces

- FFS: Whether or not the BWP can be configured such that NPUCCH is less than 10 or 11

- FFS: Potential impact due to in-carrier guard bands

- Note: The UE is not expected to be configured with PUCCH transmissions spanning multiple LBT bandwidths

## 7.2.2.2	Physical Layer Procedure

### 7.2.2.2.1	Channel access procedures

#### RAN1 #AH-1901, Jan 2019

Agreement:

- A gap (DLUL, ULUL, or UL DL) of a specific duration is created using one or more of:

  - Timing Advance

  - CP extension

    - max value of not more than one OFDM symbol

  - Shortening of DL or UL transmission duration by one or more OFDM-symbol(s) by puncturing or rate matching

  - Note: the mechanisms applied in each case may be different for different SCSs

- FFS: how to signal the way of creating the gap to the UEs

#### RAN1 #96, Feb 2019

Conclusion:

A common preamble and a common energy detection threshold between NR-U and other technologies for 5 and 6 GHz was discussed. There is no consensus on these aspects at this stage.

Conclusion:

Cat 2 is not used for initiating a UE transmission outside of a gNB COT for the following channels/signals (or any combination of them):

- PUSCH (with or without UCI),

- SRS-only,

- PUCCH-only

Note:

- Cat 4 for these channels was already agreed during the study item

- This does not preclude the use of Cat 2 for transmission on a LBT bandwidth if it is allowed for the case of transmission on multiple LBT bandwidths

Agreement:

For initiation of a gNB transmission:

- LBT other than Cat 4 is not used for DRS multiplexed with unicast data

- LBT other than Cat 4 is not used for PDCCH and/or PDSCH transmission outside of DRS.

Note:

- This does not preclude the use of Cat 2 for transmission on a LBT bandwidth if it is allowed for the case of transmission on multiple LBT bandwidths

Agreement:

LBT other than Cat4 is not considered for UL transmissions that are part of a RACH procedure that initiate a channel occupancy

- Note: This does not preclude the use of Cat 2 for transmission on a LBT bandwidth if it is allowed for the case of transmission on multiple LBT bandwidths

#### RAN1 #96bis, April 2019

Conclusion:

If a category 2 LBT with a duration of 25 microseconds is required, no changes to the baseline method as followed in LAA for such LBT are needed

Agreement:

The following agreement from the SI is updated as shown:

Table 7.2.1.3.1-1: Channel access schemes for initiating a COT by gNB as LBE device

|  | Cat 2 LBT | Cat 4 LBT |
| --- | --- | --- |
| DRS alone or multiplexed with non-unicast data (e.g. OSI, paging, RAR) | When the DRS duty cycle ≤1/20, and the total duration is up to 1 ms: 25 µs Cat 2 LBT is used (as in LAA) | When DRS duty cycle is > 1/20, or total duration > 1 ms Cat4 with any channel access priority class value can be used |
| DRS multiplexed with unicast data | N/A except for the cases discussed in the Note below | Channel access priority class is selected according to the multiplexed data |
| PDCCH and PDSCH | N/A except for the cases discussed in the Note below | Channel access priority class is selected according to the multiplexed data |

Agreement:

For a UCI-only transmission on PUSCH in a channel occupancy initiated by the UE, Cat4 with lowest channel access priority class value can be used by the UE

Agreement:

For LBT by a UE prior to transmission of a UL burst within a gNB-initiated channel occupancy as LBE device, for gap durations shorter than 25 microseconds, choose one of the following alternatives

- Alt 1: Cat 2 LBT can be indicated (FFS: explicit and/or implicit) to the UE if the gap is 16 microseconds (allowing for implementation tolerances)

- Alt 2: Cat 2 LBT is not indicated to the UE for gaps less than 25 microseconds

- Notes (applicable to both alternatives):

  - This means that the gNB ensures that gaps between 16 and 25 microseconds do not occur

  - This doesn’t change the previous agreement for Cat 1 and Cat 2 LBT for gaps of 16 microseconds or less

- FFS: Conditions on channel occupancy after a Cat. 1 or Cat. 2 LBT after a gap of 16 microseconds or less

#### RAN1 #97, May 2019

Agreement:

For LBT by a UE prior to transmission of a UL burst within a gNB-initiated channel occupancy as an LBE device, for gap durations shorter than 25 microseconds, Cat 2 LBT can be indicated (FFS: explicit and/or implicit) to the UE if the gap is 16 microseconds (allowing for implementation tolerances)

Note: this is the Alt 1 identified in RAN1#96bis

Agreement:

Select one of the following alternatives for Cat2 LBT in a 16 us gap.

- Alt 1: The 16us measurement period is split into two slots with the first slot having a duration 7us and second slot having a duration of 9us.

  - Energy measurement is done in the 9us slot with the measurement including averaging for at least 4 us in any portion of the slot. LBT is said to be successful if the measured energy is lower than the ED threshold.

- Alt 2: The 16us measurement period is split into two slots with the first slot having a duration 7us and second slot having a duration of 9us.

  - Energy measurement is done in both the 7us and 9us slot with the measurement including averaging for at least 4 us in any portion of each slot. LBT is said to be successful if the measured energy is lower than the ED threshold in both slots.

- Alt 3: Energy measurement is done in any portion of the 16 us duration including averaging for at least 4 us. LBT is said to be successful if the measured energy is lower than the ED threshold.

Agreement:

Multi-carrier channel access schemes that are applicable to multiple carriers with a single LBT sub-band per carrier that is the same as the carrier bandwidth are also applicable to multiple LBT sub-bands within a carrier when such carriers are used either as a single wideband carrier or as part of carrier aggregation of a set of carriers.

#### RAN1 #98, August 2019

Agreement:

For a gNB initiated channel occupancy the reference duration for CWS adjustment is defined as follows.

- For a CO with unicast PDSCH(s) and for each set of LBT bandwidths for which a single contention window is maintained, the reference duration for CWS adjustment is from the beginning of the CO until the end of the first slot where at least one unicast PDSCH is transmitted over all the resources allocated for the PDSCH, or until the end of the first transmission burst by the gNB that contains unicast PDSCH(s) transmitted over all the resources allocated for the PDSCH, whichever occurs earlier.

  - If the CO has a unicast PDSCH, but doesn’t have any unicast PDSCH transmitted over all the resources allocated for that PDSCH, then, the duration of the first transmission burst by the gNB within the CO that contains unicast PDSCH(s) is the reference duration for CWS adjustment.

Agreement:

For a UE initiated channel occupancy the reference duration for CWS adjustment is defined as follows.

- For a CO with PUSCH(s) and for each set of LBT bandwidths for which a single contention window is maintained, the reference duration for CWS adjustment is from the beginning of the CO until the end of the first slot where at least one PUSCH is transmitted over all the resources allocated for the PUSCH, or until the end of the first transmission burst by the UE that contains PUSCH(s) transmitted over all the resources allocated for the PUSCH, whichever occurs earlier.

  - If the CO has a PUSCH, but doesn’t have any PUSCH transmitted over all the resources allocated for that PUSCH, then, the duration of the first transmission burst by the UE within the CO that contains PUSCH(s) is the reference duration for CWS adjustment.

Agreement:

For a gNB initiated channel occupancy, for a DL burst without unicast PDSCH and with one or multiple UL grants, and for each set of LBT bandwidths for which a single contention window is maintained, CWS adjustment is based on the success or failure (FFS: CB, CBG, or TB) of reception of PUSCH transmissions in the granted resources

- FFS: Details of CWS adjustment based on the reception of a successful transmission

- FFS: Handling of PUCCH/PRACH/SRS and PUSCH without UL-SCH

### 7.2.2.2.2	Enhancements to initial access procedure

#### RAN1 #AH-1901, Jan 2019

#### RAN1 #96, Feb 2019

Agreement:

- For a given cell, UE may assume SS/PBCH blocks in the same candidate position within the DRS transmission window are QCL across DRS transmission windows

  - Alt1: The PBCH DMRS sequence index is also the same

  - Alt2: The PBCH DMRS sequence index may be different

  - Note: The first candidate position of the DRS transmission window is located at the first half slot of a half frame

- FFS: QCL assumption for SSBs in different candidate positions within a DRS transmission window and across DRS transmission windows

Agreement:

- At least the functionalities of Rel-13 LTE-LAA RSSI and channel occupancy reporting as a baseline should be supported

- FFS:

  - Enhanced RSSI metrics, for e.g., sub-band-level interference measurements in a wideband operation scenario

  - Reporting of a new medium contention/load metric other than channel occupancy

  - Any modification of the parameters of the Rel-15 SMTC for operation in unlicensed spectrum

Agreement:

- An RLM measurement window for serving cell RLM measurements based on SSBs in the DRS is supported for in-sync and out-of-sync evaluations.

  - FFS: How RLM measurement window is indicated or determined and relation to DRS transmission window

  - FFS: Whether or not an SSB can fall outside the measurement window and, if so, whether it can be used for in-sync and out-of-sync evaluations.

  - FFS: Any relationship of RLM measurements based on CSI-RS to the measurement window.

- FFS: Mechanism to handle missing RLM-RS due to LBT failure

#### RAN1 #96bis, April 2019

Agreement:

The maximum DRS transmission window duration is 5 ms.

- The maximum number of candidate SSB positions within a DRS transmission window, Y, is selected as Y = 10 for 15 kHz SCS and Y = 20 for 30 kHz SCS.

  - Note: The number of starting points for DRS transmissions with the 5 ms window that can use a Cat. 2 LBT is to be discussed further as part of channel access discussions.

- FFS: If the DRS transmission window is configurable, and if yes, how to configure and indicate the window, including the range of configurable values.

Agreement:

For a given cell, the UE may assume that the PBCH DMRS sequence index is the same for SS/PBCH blocks that are transmitted at the same candidate positions across DRS transmission windows.

Agreement:

UE determines serving cell timing from the detected SSB candidate position, where the SSB candidate positions within the DRS transmission window are indexed from 0,…,Y-1 (Y = 10 for 15 kHz SCS and Y = 20 for 30 kHz SCS).

#### RAN1 #97, May 2019

Agreement:

The mechanism to determine serving cell timing is as follows:

- The SS/PBCH block position index within a DRS transmission window is detected using a combination of PBCH DMRS sequence index and 1 bit/2 bits for 15 kHz SCS/30 kHz SCS of the 3 available bits in the PBCH payload (not in MIB) originally used in Rel-15 FR2 for MSB SSB index

- 10-bits SFN and half-frame indicator are indicated as in Rel-15

- PBCH payload size is not increased compared to Rel-15

  - FFS: Whether reuse of other available bits in PBCH payload is also required for timing determination

- If the UE is required to perform PBCH decoding of neighbor cell(s) (e.g., in asynchronous deployments), an explicit time allowance for acquisition of SSB index is provided to the UE

Agreement:

LBT category for msg 3 initial transmission is provided to the UE in RAR

Agreement:

Reply to the RAN2 LS informing them of the following:

- RAN1 has made the following agreement which facilitates COT sharing between Msg2 and Msg3:

  - LBT category for msg 3 initial transmission is provided to the UE in RAR

- Multiple msg3 tx opportunities with a single or multiple RARs in the time domain is feasible from a RAN1 perspective but there is no consensus at this time in RAN1 to support this. RAN1 will continue discussions on the support of multiple msg3 tx opportunities.

R1-1907858 	Reply LS on RACH agreements for NR-U	Charter

Final LS approved in R1-1907902 with Observation 2 modified as follows:

Extending the maximum RAR window size was concluded to be beneficial during the study item but there has been no discussion in RAN1 during the work item on multiple msg1 transmissions and extending the RAR window size.

Agreement:

For a serving cell, UE may assume a QCL relation between SS/PBCH blocks which are detected across DRS transmission windows and have the same value of modulo(A, Q), once Q is known to the UE

- FFS: A is the SSB candidate position index and/or PBCH DMRS sequence index

- FFS: How Q is indicated or determined

- FFS: Restriction on the range of Q.

Note: Neighbor cell RRM measurements will be addressed separately

Agreement:

For SSB-based RLM, UE may assume the RLM measurement window to be the same as the DRS transmission window.

- Note: This implies that the SSB-based RLM-RS cannot fall outside the measurement window

- FFS: Whether and how DRS transmission window is configured to the UE

#### RAN1 #98, August 2019

Agreement:

For purposes of SSB QCL derivation, the following values of Q are supported: {1, 2, 4, 8}.

- FFS: Further down-selection of allowed values.

Conclusion:

The maximum number of PBCH DMRS sequences used in a cell is unchanged from Rel-15 (=8).

- The number of PBCH DMRS sequences used in a cell is independent of Q.

Agreement:

The 3 LSB bits of the SSB candidate position index are represented by the PBCH DMRS sequence index.

Agreement:

For a cell (either serving or a neighbour cell), UE may assume a QCL relation between SS/PBCH blocks within or across DRS transmission or measurement windows that have the same value of modulo(A, Q), once Q is known to the UE

- A is the PBCH DMRS sequence index.

- Note: This agreement extends a prior agreement for serving cells on QCL relation between SS/PBCH blocks to neighbour cells

Agreement:

The following bits of the PBCH payload (not MIB) are used for serving cell timing determination within a frame in addition to the PBCH DMRS sequence index as agreed earlier:

- aA+7aA+6 for 15 kHz SCS, and  aA+6 and aA+7 for 30 kHz SCS

- aA+4 (the half-frame bit as in Rel-15)

Agreement:

If Q is known, candidate monitoring slots for Type0 PDCCH search space are the PDCCH monitoring slots associated with SS/PBCH blocks that are QCL with the SS/PBCH block from which the UE determines that a CORESET for Type0-PDCCH CSS set is present

- Note: Q may be always known depending on where Q is signalled. This aspect is to be discussed further.

### 7.2.2.2.3	HARQ enhancement

#### RAN1 #AH-1901, Jan 2019

Agreement:

- RRC parameter dl-DataToUL-ACK supports a value that can be signaled by PDSCH-to-HARQ-timing-indicator, which indicates that the UE needs to store the HARQ A/N feedback result for the corresponding PDSCH, and which does not provide any timing for the transmission of this HARQ A/N feedback result

Conclusion:

- No additional value of K1 signaled by PDSCH-to-HARQ-timing-indicator needs to be introduced for the purpose of requesting feedback outside the COT (i.e. Alt3 in TR38.889 section 7.2.1.3.3)

  - Further discuss the details for determining the LBT type for the UE transmission of HARQ A/N feedback for this case

  - Further discuss whether additional values need to be introduced for the purpose of requesting feedback inside the COT

    - If additional values are introduced these can also be used for requesting feedback outside the COT

  - Note: This does not necessitate that the UE needs to know whether the feedback is inside or outside the COT at the time when the feedback is requested

Agreement:

For enabling multiple opportunities for HARQ A/N transmission and for cross-COT HARQ-ACK feedback, at least the following is supported:

gNB requests/triggers feedback for PDSCH from earlier COT(s) or additional reporting of earlier HARQ feedback, where the exact HARQ feedback timing and resource is provided to the UE in another DCI (in the same or in another COT)

Agreement:

Cross-carrier HARQ re-transmissions will not be discussed for NR-U in Rel-16.

#### RAN1 #96, Feb 2019

Agreement:

Scheduling PUSCH over multiple slots/mini-slots by single DCI supports at least multiple continuous PUSCHs with separate TBs

- Each TB is mapped to at most one slot or one mini-slot

#### RAN1 #96bis, April 2019

Agreement:

A non-numerical value is added to the possible range of PDSCH-to-HARQ-timing-indicator values defined in Rel-15, and is used to indicate to the UE that the HARQ-ACK feedback for the corresponding PDSCH is postponed until the timing and resource for the HARQ-ACK feedback is provided by the gNB.

Agreement:

Restrict further discussion on HARQ codebook to the following:

- For dynamic HARQ codebook:

  - PDSCH grouping by explicitly signalling a group index in DCI scheduling the PDSCH

  - gNB can request HARQ-ACK feedback in the same PUCCH for all PDSCHs in the same group

  - Option 1:

    - One PUCCH can carry HARQ-ACK feedback for one or more PDSCH groups

    - DCI can request HARQ-ACK feedback for one or more PDSCH groups

    - FFS one of the two options below

      - C-DAI/T-DAI can be accumulated across multiple PDSCH groups for which feedback is requested in the same PUCCH

      - C-DAI/T-DAI is accumulated only within one PDSCH group

    - FFS: New ACK-Feedback Group Indicator for each PDSCH Group

    - The number of HARQ-ACK bits for one PDSCH group is constant between a first HARQ-ACK feedback transmission and a HARQ-ACK feedback re-transmission, i.e. the PDSCH group cannot be enlarged after the first feedback transmission

  - Option 2:

    - One PUCCH can carry HARQ-ACK feedback for a single PDSCH group

      - FFS: Feedback for more than one PDSCH group

    - DCI can request HARQ-ACK feedback for a single PDSCH group

      - FFS: Requests for more than one PDSCH group

    - C-DAI/T-DAI is accumulated within one PDSCH group

    - A reset indicator signals new HARQ-ACK feedback for a PDSCH group

    - The number of HARQ-ACK bits for one PDSCH group may not be constant between a first HARQ-ACK feedback transmission and a HARQ-ACK feedback re-transmission

- Semi-static codebook. Options FFS.

- If request/trigger for one-shot group HARQ ACK feedback for all configured HARQ processes is introduced (at least for non-CBG HARQ), select one or more of the following candidate schemes:

  - The request is carried in a UE-specific DCI carrying a PUSCH grant

  - The request is carried in a UE-specific DCI carrying a PDSCH assignment

  - The request is carried in a UE-specific DCI not scheduling PDSCH nor PUSCH

  - The request is carried in a UE-common DCI

  - The request can be used for UE configured with dynamic or semi-static HARQ codebook

- Note: The discussion on preconfigured/pre-indicated multiple opportunities in frequency domain in different LBT subbands is a separate discussion

#### RAN1 #97, May 2019

Agreement:

For multiple PUSCHs scheduled by a single DCI

- The following fields in the DCI are signalled per PUSCH

  - NDI

  - RV (FFS: compression scheme for RV)

- FFS if CBG-based re-transmission is supported for multi-TTI scheduling, at least the following options are considered for signalling the CBGTI field in the DCI

  - Option 1: per re-transmitted PUSCH

    - FFS: limitations on number of re-transmitted PUSCH for which CBGTI field is signalled

  - Option 2: per PUSCH

  - Option 3: only for a fixed number of PUSCHs

- HARQ process ID signalled in the DCI applies to the first scheduled PUSCH. HARQ process ID is then incremented by 1 for subsequent PUSCHs in the scheduled order (with modulo operation as needed)

- Time domain resource assignment mechanism is enhanced for indicating the number of scheduled mapping Type A and Type B PUSCHs, and their starting and ending symbols

  - At least continuous time domain resource assignment is supported

  - FFS: whether multiple mapping Type B PUSCHs is allowed within the first slot

  - FFS: whether multiple starting positions is allowed for UE-initiated COT (discussed as part of agenda 7.2.2.1.3)

  - FFS: configuration/signalling details

- CSI request field in the DCI applies to a single PUSCH

  - FFS: relation between the timing of the triggered CSI-RS and the PUSCH carrying the CSI feedback, and how to determine which PUSCH carries the CSI feedback

- FFS: Whether existing DCI formats can be extended or new formats are necessary and associated details for supporting scheduling multiple PUSCHs

- FFS: Potential impact on MCS signalling for re-transmission

- Note: Indication of the LBT type and priority class should be possible with the DCI

- Note: SRS request field in the DCI applies to a single slot with the applicable slot signalled as in Rel-15

- Note: The number of DAI fields is not increased as compared to the single PUSCH scheduling DCI in Rel-16. Applicability of the DAI field is the same as multi-slot PUSCH scheduling in Rel-15.

Agreement:

For operation with dynamic HARQ codebook (type-2 codebook):

- PDSCH grouping by explicitly signalling a group index in DCI scheduling the PDSCH

  - For any PDSCH scheduled with numerical or non-numerical value of K1

- The number of HARQ-ACK bits for one PDSCH group can change between successive requests for HARQ-ACK feedback for the same PDSCH group

- HARQ-ACK feedback for all PDSCHs in the same group is carried in the same PUCCH

- One DCI can request HARQ-ACK feedback for one or more PDSCH groups in the same PUCCH

- C-DAI/T-DAI is accumulated only within each PDSCH group

  - FFS: Choose between the following options:

    - T-DAI is included only for the scheduled group

    - T-DAI is included for each group

- New ACK-Feedback Group Indicator for each PDSCH Group operates as a toggle bit

- Maximum number of PDSCH groups: 2 (FFS: maximum number of groups 4 and maximum number of groups for which feedback is requested in the same PUCCH)

- A UE can signal support of this feature as part of capability signaling

#### RAN1 #98, August 2019

Conclusion:

In Rel-16, the PDSCH-to-HARQ-timing-indicator field is not enhanced to indicate larger values of K1 compared to Rel-15.

Agreement:

When configured for operation with enhanced dynamic HARQ codebook (type-2 codebook), a UE shall support 2 PDSCH groups.

- The number of PDSCH groups that the UE shall support is not RRC configurable

- FFS: More groups, depending on the resolution of the FFS on up to 4 groups from a prior agreement

Agreement:

- For enhanced dynamic HARQ codebook, choose one of the alternatives below:

  - Alt-1: NFI and T-DAI are indicated in non-fallback DCI scheduling PDSCH only for the scheduled group

  - Alt-2: NFI and T-DAI are indicated in non-fallback DCI scheduling PDSCH for both groups

- FFS: Extension to fallback DCI

Agreement:

For enhanced dynamic HARQ-ACK codebook when the feedback is triggered by a DL DCI scheduling a PDSCH, a UE is not expected to be requested in that DL DCI to provide feedback only for a PDSCH group not scheduled in that DL DCI

Agreement:

For enhanced dynamic codebook operation, HARQ-ACK timing for a PDSCH scheduled with a non-numerical value for K1 is derived by the next DL DCI scheduling PDSCH with a numerical value for K1 triggering feedback for PDSCH group(s) including the PDSCH group of this PDSCH.

- FFS: additional methods including

  - based on an earlier DCI

  - one-shot HARQ-ACK feedback

- FFS: dependency on NFI

Conclusion:

There is no consensus to support a 2-stage uplink grant for NR-U in Rel-16.

Conclusion:

For multi-TTI scheduling and for single TTI scheduling for PUSCH, the UE shall only attempt transmitting a PUSCH at the single starting position indicated in the UL grant in PDCCH for this PUSCH.

- FFS: for msg3

### 7.2.2.2.4	Configured grant enhancement

#### RAN1 #AH-1901, Jan 2019

Agreement:

For configured grant resource configuration in time domain, the following alternatives are to be studied with more detailed proposal and analysis, strive to down-select in RAN1#96:

- Alt. 1: Bitmap based approach as baseline with potential enhancement

  - Companies are encouraged to provide detailed design in next meeting

- Alt. 2: NR Rel-15 based time domain resource allocation approach as baseline with potential enhancement

  - Companies are encouraged to provide detailed design in next meeting

Agreement:

- Support multiple UE starting time offsets with sub-symbol granularity with FeLAA AUL approach as the baseline

  - FFS: Enhancements specific to NRU

- Companies are encouraged to provide views and analysis on the following issues:

  - Whether to support allowing the UE to start transmission later than the starting symbol as indicated in configured grant based on LBT outcome

    - If yes, multiple starting positions within a slot for a configured grant configuration;

      - Alt. 1: subset of symbols

      - Alt. 2: any symbol

      - FFS: gNB knowledge of starting symbol, whether UE indicates to gNB

      - FFS signaling details

      - FFS: whether similar design for scheduled grant and configured grant

  - Whether the ending symbol can be punctured

  - Whether the position of the ending symbol can be shifted depending on the starting position due to LBT procedures

Conclusion:

The following aspects should be discussed further as part of the channel access discussions

- Contention window adjustment

- Details of COT sharing related to NRU configured grant including details and limitations on UE-initiated COT sharing with gNB and configured grant UL transmissions within gNB acquired COT

Agreement:

CG-UCI should at least include the following information:

- HARQ ID

- NDI

- RV

- COT sharing information, FFS details

- FFS: other information including UE ID

#### RAN1 #96, Feb 2019

Agreement:

For PUSCH transmitted using CG, CBG-based retransmission is supported at least by using dedicated scheduled resource allocated by an UL grant.

- FFS: CBG-based retransmission using a configured grant

- Note: Include this agreement in an LS to RAN2 informing them of relevant RAN1 agreements

Agreement:

For initial transmission on configured grant resource, HARQ retransmission on configured grant resource upon configured grant timer expiration (assume NACK if no ACK is received) is supported

- Note: Include this agreement in an LS to RAN2 informing them of relevant RAN1 agreements

Agreement:

When a UE initiates a channel occupancy with a transmission using a configured grant, it can signal at least the following

- The duration that the gNB is allowed to transmit in the channel occupancy initiated by the UE

- FFS:

  - How the duration is signalled

  - Whether the UE should signal continued use of the COT for its own transmissions

  - LBT priority class

#### RAN1 #96bis, April 2019

Agreement:

Select from the following additional options for type 1 and type 2 configured grant time domain resource allocation mechanism in NR by RAN1#97

- Option 1: A bitmap to selectively enable or disable configured UL transmission opportunities as per NR Rel-15 configurations.

  - FFS: A bit in the bitmap can correspond to a slot or sub-slot or group of slots

  - FFS: duration of bitmap in time, e.g. 40ms

- Option 2: A mechanism based on multiple NR Rel-15 configurations

  - FFS: Whether any further enhancement is needed to Rel-16 beyond what is being considered in the URLLC WI

- Option 3: Configuration in addition to the Rel-15 baseline of one or more of the following aspects:

  - Multiple offsets within an active configuration

  - Duration of transmission for an offset

- Option 4: A bitmap to configure UL transmission opportunies to replace current time domain resource configuration

  - FFS: A bit in the bitmap can correspond to a slot or sub-slot or group of slots

  - FFS: duration of bitmap in time, e.g. 40ms

  - Note: This is importing LAA AUL functionality into NR

#### RAN1 #97, May 2019

Agreement:

For configured grant time domain resource allocation, the mechanisms in Rel-15 (both Type 1 and Type 2) are extended so that the number of allocated slots following the time instance corresponding to the indicated offset can be configured

- FFS: How to indicate multiple PUSCHs within a slot.

Agreement:

UE can only start transmissions accessing transmission opportunities provided by a configured grant at the configured/indicated starting position.

#### RAN1 #98, August 2019

Agreement:

- For DFI design for configured grants, support at least the following

- DFI including at least TB level HARQ-ACK bitmap for all UL HARQ processes

- Note: Total number of HARQ processes is as defined in Rel-15

- FFS: CBG level HARQ-ACK feedback, if supported

- RRC configured minimum duration, D, from the ending symbol of the PUSCH to the starting symbol of the DFI carrying HARQ-ACK for that PUSCH

  - Note: UE assumes HARQ-ACK is valid only for PUSCH transmissions ending before n-D, where n is the time corresponding to the beginning of the start symbol of the DFI.

  - FFS: the definition of minimum duration for the case of slot aggregation

- UE blind decoding complexity shall not be increased due to DFI size

Agreement:

The number of separately encoded UCIs multiplexed in a PUSCH transmitted using a configured grant is not changed from Rel-15 (maximum of 3).

Working assumption:

CG-UCI is included in every CG-PUSCH transmission

### 7.2.2.2.5	Wide-band operation

#### RAN1 #AH-1901, Jan 2019

Agreement:

- For wideband operation in DL with a single serving cell operation within a carrier with bandwidth larger than 20 MHz

  - Multiple BWPs can be configured, single BWP activated, gNB may transmit PDSCH on parts or whole of single active BWP where CCA is successful at gNB (i.e., option 2 and 3 from previous agreement)

    - FFS: Restrictions on supportable gaps and combinations of gaps between discontiguous blocks where

      - each block spans contiguous (one or) multiple successful LBT sub-bands

      - each gap spans one or multiple contiguous unsuccessful LBT sub-bands

    - FFS: Transmission bandwidth adaptation delay, potentially different delay for e.g., different number of supported gaps, different transmission bandwidths and different positions of the LBT sub-bands where transmissions occur

    - FFS: Limit on the occupied LBT sub-bands due to regulation and coexistence considerations (not intended to imply that regulation and coexistence considerations will not be addressed)

    - FFS: Whether/how to indicate gNB’s transmitted LBT sub-bands

    - FFS: Enhancements to PDCCH/PDSCH configuration/transmission for the parts of BWP where gNB does not transmit due to CCA failure

- Send LS to RAN4 to inform above decision with the description that RAN1 requires RAN4’s feedback on the first three FFS parts in addition to what was requested in earlier LSs.

Agreement:

Operation with multiple active BWPs for a carrier on unlicensed bands is not supported for DL or UL at least in Rel-16 NR-U WI.

- Inform RAN2 of this decision

R1-1901446	[DRAFT] Reply to reply LS on wideband carrier operation for NR-U	LG Electronics

Final LS is agreed with the following changes in R1-1901460

- Include RAN WG2 in the “To” field

- Add an action to RAN WG2: “RAN1 respectfully asks RAN2 to take the above RAN1 agreements into account in their further work”

- Delete the corresponding statement requesting action from RAN2 above in the main body.

- Modify a statement in the main body as follows: “RAN1 respectfully asks RAN4 for a response to FFS parts in the above agreement, in addition to any pending input to questions as requested in [1] not yet answered.”

Conclusion:

The channel access aspects of wideband operation should be discussed further as part of the channel access discussions

#### RAN1 #96, Feb 2019

#### RAN1 #96bis, April 2019

Agreement:

For UL transmissions in a serving cell with carrier bandwidth greater than LBT bandwidth, for the case where UE performs CCA before UL transmission, support at least Alt. 1 among the following alternatives

- Alt. 1: UE transmits the PUSCH only if CCA is successful at UE in all LBT bandwidths of the scheduled PUSCH.

- Alt. 2: UE transmits the PUSCH in all or a subset of LBT bandwidths of the scheduled PUSCH for which CCA is successful at the UE.

  - Decision on whether this alternative is supported will depend on feedback from RAN4

  - FFS on restrictions to the subset of LBT bandwidths, e.g., only contiguous LBT bandwidths allowed, based on feedback from RAN4

- Necessity of guard bands within the scheduled PUSCH should be determined by RAN4

- FFS: Whether this applies also to configured grant PUSCH

- FFS: Whether this applies also to PUCCH

LS to RAN4 on the above agreement captured in R1-1905892.

Agreement:

- Support a mechanism for a UE to detect gNB is transmitting across

  - Multiple carriers

  - Multiple LBT bandwidths in a carrier.

- The following mechanisms are to be considered:

  - Option 1: Explicit indication via PDCCH

    - FFS: The type of PDCCH (e.g., group common PDCCH or UE-specific PDCCH)

    - FFS: Signaling details of the indication

  - Option 2: Explicit indication via selection of a PDCCH DM-RS sequence from a set of PDCCH DM-RS sequences

    - FFS: Details of the indication

  - Option 3: Via UE implementation, i.e., implicit method based on NR-based signal such as DM-RS and/or corresponding PDCCH detection

    - FFS: Which signals/channels or combination of signals/channels could be used by the UE

  - Note: Above options are not mutually exclusive

#### RAN1 #97, May 2019

Agreement:

When GC-PDCCH is configured, explicit indication via GC-PDCCH is supported as a mechanism to inform the UE that one or more carriers and/or LBT bandwidths are not available or available for DL reception, at least for slot(s) that are not at the beginning of DL transmission burst.

- FFS: Signalling details of the indication, including e.g., the time domain validity of the indication

- FFS: Whether and how to support the mechanism at the beginning of DL transmission burst

- FFS: Whether and how to handle the case when GC-PDCCH is not configured or not received by the UE

Conclusion:

A UE can receive a PDSCH scheduled within an LBT bandwidth or over multiple LBT bandwidths as per Rel-15 and current agreements in Rel-16.

#### RAN1 #98, August 2019

Conclusion:

The following are unchanged from Rel-15 for PDCCH.

- The maximum number of monitored PDCCH candidates per slot and per serving cell.

- The maximum number of non-overlapped CCEs per slot and per serving cell.

- CCE-to-REG mapping rule and hashing function.

Agreement:

For CORESET configuration in a serving cell with carrier bandwidth greater than LBT bandwidth,

- For the case where a CORESET is confined within a LBT bandwidth, the search space set configuration associated with the CORESET can have multiple monitoring locations in the frequency domain (per LBT bandwidth)

  - Send an LS to RAN2 informing them of this agreement and providing clarifications on the above if necessary

- Note: For scenarios in which gNB transmits PDCCH/PDSCH on a single BWP if CCA is successful at gNB for the whole BWP, CORESET(s) need not all be confined within an LBT bandwidth, and no specification impact is foreseen

## 7.2.2.3	Others

#### RAN1 #AH-1901, Jan 2019

#### RAN1 #96, Feb 2019

#### RAN1 #96bis, April 2019

#### RAN1 #97, May 2019

#### RAN1 #98, August 2019