3GPP TSG RAN WG1 Meeting #98bis	R1-1910883

Chongqing, China, October 14-20, 2019

Agenda Item:	6.2.2.2

Source:	Futurewei

Title:	Feature lead summary of Support for transmission in preconfigured UL resources

Document for:	Discussion and decision

# Introduction

One of the objectives of the Rel-16 WI on additional enhancements for NB-IoT [1] is to specify support for UL transmission in preconfigured resources.

Improved UL transmission efficiency and/or UE power consumption: - Specify support for transmission in preconfigured resources in idle and/or connected mode based on SC-FDMA waveform for UEs with a valid timing advance[RAN1, RAN2, RAN4] - Both shared resources and dedicated resources can be discussed - Note: This is limited to orthogonal (multi) access schemes

Agreements on transmission on UL preconfigured resources in RAN1#96bis/97 are listed below. All Rel-16 RAN1 agreements are collected in [2].

RAN1#96bis

Agreement In idle mode, a UE can be configured such that TA is always valid within a given cell. - Up to RAN2 how to implement - e.g. PUR Time Alignment Timer or NRSRP Threshold = infinity Agreement The value(s) of NRSRP threshold(s) is UE specific Agreement The UE monitors the NPDCCH for at least a time period after a PUR transmission. - FFS: Details of the time period - FFS: UE behaviour if nothing is received in that time period. - FFS: If and how often UE monitors NPDCCH after a PUR allocation in which it has not transmitted Agreement Reuse existing field(s) of DCI format N0 to convey the dedicated PUR ACK Agreement After data transmission on PUR, upon unsuccessful decoding by eNB, the UE can expect an UL grant for retransmission on NPDCCH. Other behaviors are FFS. - Working Assumption#1 - In idle mode, updating PUR configurations and/or PUR parameters via L1 signalling after a PUR transmission is supported - FFS: Which PUR configurations and PUR parameters will be signaled via L1 - FFS: Definition of PUR configurations and PUR parameters The working assumption will be automatically confirmed if for some cases L2/L3 signaling is not needed. If RAN2 decides that L2/L3 signaling is needed for all cases, the working assumption will be reverted. - Working Assumption#2 For dedicated PUR - During the PUR search space monitoring, the UE monitors for DCI scrambled with a RNTI assuming that the RNTI is not shared with any other UE - Note: It is up to RAN2 to decide how the RNTI is signaled to UE or derived - FFS if the UE monitors any additional RNTI which may be shared with other UEs. - Note: The same RNTI may be used over non-overlapping time and/or frequency resources Send an LS to RAN2 to include two above working assumptions. Ask whether the first bullet in working assumption #2 is feasible. If it is concluded that working assumption #2 feasible, the working assumption #2 will be automatically confirmed. (LS is endorsed in eMTC agenda item) Agreement For dedicated PUR in idle mode, the PUR configuration is configured by UE-specific RRC signaling.

RAN1#97

Agreement For dedicated PUR in idle mode and for HD-FDD UEs, the start of the PUR SS Window is [x] subframes after the end PUR transmission - FFS: Value of x, and if x is fixed or signaled - FFS: Support for monitoring of PUR SS Window before PUR transmission Note: The PUR SS Window is the time period where the UE monitors the NPDCCH for at least a time period after a PUR transmission Agreement NPDCCH candidates are determined by USS like search space - FFS: Other details on the USS like search space - Type2-CSS can also be discussed as part of the FFS Conclusion CBS PUR is not supported in Rel-16 For further discussion - Aspects related to notifying eNB of unused PUR resources. - Potential enhancements of power control mechanisms for PUR. (The baseline is the existing NB-IoT open loop power control.)

RAN1#98

Agreement Confirm the Working Assumption#1 in RAN1#96bis - In idle mode, updating PUR configurations and/or PUR parameters via L1 signalling after a PUR transmission is supported - FFS: Which PUR configurations and PUR parameters will be signaled via L1 - FFS: Definition of PUR configurations and PUR parameters Agreement Confirm the Working Assumption#2 in RAN1#96bis For dedicated PUR - During the PUR search space monitoring, the UE monitors for DCI scrambled with a RNTI assuming that the RNTI is not shared with any other UE - Note: It is up to RAN2 to decide how the RNTI is signaled to UE or derived - FFS if the UE monitors any additional RNTI which may be shared with other UEs. - Note: The same RNTI may be used over non-overlapping time and/or frequency resources Agreement For dedicated PUR in idle mode, the maximum NPDCCH repetitions, X, is included in the PUR configuration and the PUR NPDCCH candidates are derived using legacy USS rules where rmax= X. Agreement RAN1 will downselect among the following indications for PUR L1 ACK in RAN1#98bis Timing advance adjustment (including TA adjustment of 0) UE TX power adjustment NPUSCH repetition adjustment Indication of PUR SS monitoring termination Flag to indicate for L1 ACK None of the above Note: No new indications will be considered. Agreement After data transmission on PUR, if nothing is received by the UE in a time period, the UE shall fallback to legacy RACH/EDT procedure. FFS: Details on time period Agreement Open loop power control is used as power control mechanism for PUR. - The power for PUR transmission is calculated based on pathloss regardless of the number of repetitions. Agreement For dedicated PUR in idle mode and for FDD NB-IoT UEs, the start of the PUR SS Window is 4 subframes after the end of the PUR transmission. Agreement The dedicated PUR ACK DCI at least allows for indicating implicitly or explicitly that the current PUR monitoring is terminated - FFS: The dedicated PUR ACK DCI at least allows for indicating whether the UE continues monitoring the PUR search space after a gap (whether the gap is always zero or it can be zero) Agreement After data transmission on PUR, the UE may expect an explicit indication on NPDCCH for fallback to EDT or RACH Working assumption For NB-IoT allocation with 12 tones, for PUR with R>= [64 or 128 repetitions or ms]: - Allow for UE-specific cyclic shift for DMRS: - For 12 tone allocation, [2 or 4 or 8] cyclic shifts. - For NPUSCH scrambling, the baseline is the current c_init equation by using the configured PUR_RNTI. - Companies are welcome to evaluate baseline c_init, alternative c_init (e.g. c_init for NPDSCH rotation sequence) equations, and other proposals. Note 1: The Working Assumption is also subject to the potential RAN2 and RAN4 specification impacts. Note 2: It is transparent to a given UE whether the eNB is allocating other UEs in the same resource.

This contribution summarizes views and proposals in RAN1#98b [3]-[14].

# Timing Advance (TA)

## UE behavior when the TA is invalid

One remaining issue for obtaining a valid TA (the TA is validated and found to be invalid) when the UE has data to send is which mechanism(s) can be used by the UE to acquire a valid TA.

If the TA is found invalid, the UE may initiate RACH/EDT procedures to transmit data, however UE initiating the RACH procedure for the only purpose of acquiring the TA is unnecessary and has disadvantages (power consumption, UE transitioning from/to CONNECTED/IDLE mode, waste of resources, etc.). The EDT procedure may not be optimal because of the limited supported TBS. It is then beneficial to introduce a mechanism to update the TA so that the UE can continue transmission on PURs. Proposals are summarized in Table 1.

From a feature lead perspective, it is suggested to introduce a mechanism for the UE to acquire the TA only and to continue transmitting on PURs. The details of the mechanism need to be further discussed.

Potential Agreement #1:

For dedicated PUR in idle mode, mechanism(s) to obtain a valid TA and allow the UE transmitting on PUR without transitioning to RRC_CONNECTED mode is introduced.

Potential Agreement #2:

The following mechanisms are considered:

- A contention-free PRACH preamble is configured in the PUR configuration to be used by the UE to acquire a valid TA.

- For the case where UE updates its TA by NPRACH transmission for D-PUR transmission, support early termination of RACH procedure obtaining only a valid TA without transitioning to RRC_CONNECTED state.

Table 1 Acquiring a valid TA

| Source | Proposals |
| --- | --- |
| Huawei, HiSilicon | Proposal 1: When the TA is found to be invalid, it is supported that only TA is acquired through pre-configured PRACH preamble transmission and feedback. Proposal 2: A contention-free PRACH preamble is configured in the PUR configuration to the UE for TA update purpose. |
| ZTE | Proposal 14: When the TA is found to be invalid, mechanism for only obtaining TA for PUR in IDLE mode should be supported. Proposal 15: When the TA is validated and found to be invalid, a simplified legacy contention based random access procedure can be used for only obtaining TA. Proposal 16: Non-contention based random access procedure can be used for only obtaining TA. - Non-contention based preamble can be configured in PUR configuration. |
| LGE | Proposal 2: For the case where UE updates its TA by NPRACH transmission for D-PUR transmission, support early termination of RACH procedure obtaining only a valid TA without transitioning to RRC_CONNECTED state. |
| Ericsson | - Proposal 10: When the TA validation mechanisms are tested and not passed, the fall-back to legacy RACH or EDT agreed in RAN1 #96 is preferred over the FFSs. |
| Sierra | Proposal 8: No new methods to update an invalid TA shall be specified in Rel 16 |
| Lenovo | - Proposal 1: When TA is validated and found to be invalid, TA is updated via mechanism of legacy EDT and RA procedure. |
| DoCoMo | - Proposal 1: New approaches to obtain a valid TA should not be supported in Rel-16. |

## TA validation

Table 2

| Source | Proposals |
| --- | --- |
| ZTE | Proposal 10: For “Serving cell NRSRP changes” based TA validation mechanism, ∆NRSRP is defined as ∆NRSRP = (latest NRSRP measurement) - (NRSRP measurement when TA was evaluated last time) Proposal 11: For evaluation of NRSRP for TA validation, If abs(∆NRSRP)<=R1, (R1 is eNB configured RRC parameter), TA is valid for this NRSRP measurement Proposal 12: For “Serving cell NRSRP changes” based TA validation mechanism, the number of ∆NRSRP calculation result per PUR period, M, can be configured by eNB. - At TA validation time, when M>1, and if at least one ∆NRSRP calculation result satisfies abs(∆NRSRP)>R1 , (R1 is eNB configured RRC parameter), TA is invalid. Proposal 13: For dedicated PUR in idle mode, TA validation should be completed within a time period which is X subframes long and Y subframes before it’s PUR allocation. - X,Y are configured in PUR configuration. |
| Sierra Wireless | At least the following options of how the UE is to obtain the NRSRP threshold(s) are possible: Option 1: Configure the UE with an equation Option 2: Configure the UE with a lookup table Option 3: Configure the UE with the NRSRP threshold(s) each time the TA is updated The same methods can be used for the single and two threshold method. When an equation or table is used to calculate the threshold(s), the signalling overhead is the smallest. When the threshold(s) are signaled each time the TA is updated, there is significant signalling overhead especially since layer 2 signalling is needed. Based on the above observations, the following proposal is made: - If NRSRP is used for TA validation, the PUR configuration shall include an NRSRP threshold table and TA_error if K=2: FSS: The number of table entries and calculation details |

It is suggested to further discuss.

# Transmission in PURs

## Configuration

Progress so far:

Agreement RAN1#96 For dedicated PUR, in idle mode, the PUR resource configuration includes at least the following - Time domain resources including periodicity(s) - Note: also includes number of repetitions, number of RUs, starting position - Frequency domain resources - TBS(s)/MCS(s) - Power control parameters - Legacy DMRS pattern Agreement RAN1#98 - In idle mode, updating PUR configurations and/or PUR parameters via L1 signalling after a PUR transmission is supported - FFS: Which PUR configurations and PUR parameters will be signaled via L1 - FFS: Definition of PUR configurations and PUR parameters

Table 3 PUR configuration

| Source | Proposals |
| --- | --- |
| Huawei, HiSilicon | Proposal 3: For dedicated PUR, the configured number of repetitions and configured TBS are the maximum number of repetitions and maximum TBS supported by the PUR. Proposal 4: The UE is allowed to use a smaller TBS than the TBS configured in PUR configuration, and can scale down the repetition number in a way similar to Rel-15 EDT. |
| Lenovo, MM | Proposal 12: PUR resource can be configured as periodic starting subframe and resource duration for each PUR resource occasion. |
| Samsung | Proposal #5: Validity period should be introduced for dedicated PUR. Proposal #7: Preconfigured UL resources can be activated/deactivated in system information. |
| Qualcomm | Proposal 7: Configuration of PUSCH Format 2 associated with PUR is dedicated for a UE, and the dedicated indication is part of the initial PUR configuration. FFS details of dedicated configuration, for example indication of number of repetitions, subcarrier to be used, etc. |
| LGE | Proposal 5: Among D-PUR configuration parameters, the only parameter that needs L1 update is UL Tx power adjustment if power control is supported for D-PUR. - TA can be updated via MAC CE or MAC RAR and other D-PUR configuration parameters are updated by higher layer |

It is suggested to come back to the configuration discussion after concluding on Sec.4.2 PUR ACK DCI design.

## Skipping resources

NPDCCH monitoring

Progress so far:

In RAN1#95 [6], it has been agreed that for dedicated PUR in idle mode, the UE may skip UL transmissions. In RAN1#96bis [4], there is an FFS: If and how often UE monitors NPDCCH after a PUR allocation in which it has not transmitted.

In order to allow the UE to be reachable by gNB during the period that the UE skips transmission, the UE has to monitor NPDCCH during such period. To resolve the FFS in the above RAN1#96bis agreement, it is suggested to consider the following:

Potential Agreement#3:

A subset of PUR SS windows are monitored by the UE after N PUR occasions in which the UE has not transmitted
    - FFS the value of N and/or periodicity of monitoring.

Information of unused resources to eNB

Another aspect of skipping resources is whether to inform eNB that UE will not use the allocated preconfigured resources. If eNB is aware of UE not using such resources, eNB can stop monitoring for UL transmission and can reallocate the unused resources for other purposes.  Note that in RAN2 it has been agreed an implicit PUR release mechanism, i.e. D-PUR configuration is released when the eNB doesn’t detect “m” consecutive UE transmissions. Hence the eNB will wait for the period of “m” consecutive UE transmissions before releasing PUR resources. By introducing a mechanism that informs eNB that UE will not use the allocated resources, it means that the eNB will be notified earlier and does not need to attempt decoding uplink transmission for the “m” consecutive transmissions to release resources.

Proposals are summarized in Table 4. It is suggested to consider the following:

Potential Agreement#4:

UE sends information to eNB about not using the allocated preconfigured uplink resources

- FFS the information is sent after “n” skipped transmissions, value of n, relation of (n,m) values

- FFS the signaling

Table 4: Proposals related to skipping PUR transmission

| Source | Proposals |
| --- | --- |
| Huawei, HiSilicon | Proposal 5: RAN1 considers eNB blind decoding complexity, power consumption, and waste of UL resource due to unused PUR in the design of PUR mechanisms. Proposal 6: When the UE skips PUR, the UE shall send N continuous slots/RUs of legacy DMRS sequence at the head of the PUR to notify the eNB of unused PUR, where N is given in PUR configuration. |
| Sequans | Proposal 3: Study the use of small highly detectable signaling to notify the serving cell on unused PUR. ‎Proposal 4: Study at which conditions it is most beneficial to use PURPLE. |
| Samsung | Proposal #6: Maximum number of continuous UL skipping should be specified. |

Enabling skipping and eNB detection of UE skipping

UE skipping resources is a mechanism for the UE to retain the PUR resources (for a limited period of time) even if UE has nothing to send in a PUR transmission occasion and/or UE is waiting for some feedback from eNB. For a NB-IoT UE is important to save power and transmission should occur only when there are data to be sent. Based on these considerations, the mechanism of UE skipping resources should be utilized by the UE as needed with the network control (limitation of skipped transmissions). Hence, it is the feature lead opinion that it is not needed to restrict the use of UE skipping by network configuration.

# HARQ

## Successful PUR transmission

Progress so far:

RAN1#96 agreement:

For dedicated PUR in idle mode, the dedicated PUR ACK is at least sent on NPDCCH.

RAN1#98 agreement:

The dedicated PUR ACK DCI at least allows for indicating implicitly or explicitly that the current PUR monitoring is terminated

FFS: The dedicated PUR ACK DCI at least allows for indicating whether the UE continues monitoring the PUR search space after a gap (whether the gap is always zero or it can be zero)

Regarding the FFS from RAN1#98, some companies propose that the UE monitors the PUR search space after receiving a PUR ACK. It is noted that alignment with RAN2 may be needed depending on the RAN1 agreements on this topic. It is suggested to discuss and agree the functionality of the L1 ACK first, and then align with RAN2 work if needed.  Thus, it is proposed to discuss further the following:

Potential Agreement#5:

The dedicated PUR ACK DCI allows for indicating whether the UE continues monitoring the PUR search space after a gap for a time period T

- FFS the gap

- FFS time period T

Table 5 UE behavior upon successful PUR transmission

| Source | Proposals |
| --- | --- |
| Huawei, HiSilicon | Proposal 8: The dedicated PUR ACK DCI at least allows for indicating the UE may continue monitoring the PUR search space after a non-zero gap. - The non-zero gap can be K PDCCH period, where K is indicated in the dedicated PUR ACK DCI |
| Lenovo, MM | - Proposal 4: In response to “ACK”, UE would stop NPDCCH monitoring and go directly to idle mode. Proposal 3: After transmission based PUR, UE start to monitor the NPDCCH with the dedicate RNTI expecting an explicit ACK upon successful decoding by eNB or an explicit NACK/an UL retransmission GRANT upon unsuccessful decoding by eNB. |
| Nokia, NSB | D-PUR DCI should at least support: • An explicit ACK that reuses existing field(s) of DCI format N0 Proposal 6: The D-PUR L1 ACK supports an “Indicator to terminate of SS monitoring ” with a configurable “scheduling delay”. FFS: If the scheduling delay is signalled within the DCI or preconfigured. |
| ZTE | Proposal 3: If nothing is received by the UE in the time period after PUR retransmission, UE can assume that the PUR is successfully received. |
| LGE | Proposal 3: When a UE receives a PUR L1 ACK after D-PUR transmission, the UE stops monitoring PUR search space. |
| Intel | Proposal 4: • The UE terminates monitoring of the current PUR SS after reception of L1 ACK signaling. |

## PUR ACK DCI Design

In RAN1#98 agreement:

RAN1 will downselect among the following indications for PUR L1 ACK in RAN1#98bis

•	Timing advance adjustment (including TA adjustment of 0)

•	UE TX power adjustment

•	NPUSCH repetition adjustment

•	Indication of PUR SS monitoring termination

•	Flag to indicate for L1 ACK

•	None of the above

Note: No new indications will be considered.

Table 6 Indications for PUR L1 ACK

| Source | N | ZTE | Intel | SS | HW | E | L-MM | DCM | LG | SW | Seq | QC |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Timing advance adjustment | yes | yes | FFS | yes | yes | yes | yes | yes | No | yes | No | yes |
| UE TX power adjustment | No | yes | yes |  | No | No | yes | yes | yes | yes |  | yes |
| NPUSCH repetition adjustment | yes | yes | yes |  | No | No | yes | yes | No | yes |  | No |
| Indication of PUR SS monitoring termination |  | yes |  |  | yes | No |  | yes | No | No |  |  |
| Flag to indicate for L1 ACK |  | yes |  |  | yes | FFS |  |  | No |  |  | yes |

Table 7 DCI design

| Source | Proposals |
| --- | --- |
| Ericsson | In case only TA information of 3 bits is added in ACK DCI: - Proposal 7: For the PUR “L1-ACK” in NB-IoT, the legacy L1-ACK as designed for MTC in Rel-15 is re-used. A 3-bit shortened TA update indication is included by not setting to zero and re-purposing the 3-bit “Repetition number” field. |
| Huawei, HiSilicon | Proposal 9: For multi-tone NPUSCH transmission on PUR, the dedicated PUR ACK DCI conveys legacy 6 bits TA adjustment. Proposal 10: For dedicated PUR ACK DCI design, the reserved states of “Subcarrier indication” field are used to convey “Flag to indicate for L1 ACK” and “Indication of PUR SS monitoring termination” as follows: - Isc=48: Flag of L1 ACK and indicates the current PUR SS monitoring is terminated - Isc=49~63: Flag of L1 ACK and indicates the UE may continue monitoring the PUR SS after K=Isc-48 PDCCH period Proposal 11: For dedicated PUR ACK DCI design, introduce “Timing advance adjustment” field to convey the TA adjustment when the “Subcarrier indication” field is to indicate either “Flag to indicate for L1 ACK” or “Indication of PUR SS monitoring termination”: - For multi-tone transmission: this field conveys the legacy 6 bits TA adjustment - For single-tone transmission: 6 bits are reserved |
| Lenovo, MM | Proposal 5: Along with ACK indication in corresponding DCI, the following PUR reconfiguration/parameters can be included: • Timing advance adjustment • UE TX power adjustment • PUSCH Repetition adjustment |
| Nokia, NSB | Proposal 8: The D-PUR L1 ACK supports the following fields: • Timing advance adjustment (including TA adjustment of 0) 6 bits • PUSCH repetition adjustment 3 bit • Flag to indicate L1 ACK 1 bit • Indication of PUR SS monitoring termination 1 bit |
| ZTE | Proposal 7: in idle mode, the following indications are carried in PUR L1 ACK DCI. • Timing advance adjustment (including TA adjustment of 0) • UE TX power adjustment • NPUSCH repetition adjustment • Flag to indicate L1 ACK ‐ Type 1 L1 ACK for indicating explicitly that the current PUR monitoring is terminated. ‐ Type 2 L1 ACK for for indicating explicitly that the UE continues monitoring the PUR search space after a Gap |
| Samsung | Proposal #8: TA adjustment within a small range is indicated in the DCI after transmitting PUR. |
| Qualcomm | Proposal 1: Introduce a timing advance adjustment field in the PUR L1 ACK DCI. Proposal 2: For potentially successive DCIs associated with the same PUR retransmission grant, the UE shall assume that the updated TA for retransmission is determined by adding the TA adjustment in the DCI only once to the TA of the initial PUR transmission. Proposal 3: For a non-zero TA adjustment in the DCI associated with PUR transmission, consider a relaxed NPDCCH-to-NPUSCH timeline with respect to legacy. FFS relaxed value. Proposal 5: For every initial PUR configuration with R repetitions, configure a repetition adjustment set S_rep^R={PUR_rep^(R,1),PUR_rep^(R,2),…,PUR_rep^(R,N) } via RRC signalling for different values of R, where R is always an element of the set S_rep^R. Indicate via DCI, the adjusted repetition number for PUR, using an indication of ⌈〖log〗_2⁡N ⌉ bits. FFS N. Proposal 6: Include a 1-bit flag in the DCI associated with PUR transmission which indicates to the UE the presence or absence of a higher layer acknowledgement from the eNB. |
| LGE | Proposal 1: A UE in RRC_IDLE state can update its TA for D-PUR transmission, and the TA information can be obtained via  MAC CE in NPDSCH scheduled by NPDCCH in PUR search space  MAC RAR, if PDCCH-ordered NPRACH transmission is triggered by NPDCCH in PUR search space |

Based on the inputs regarding the content of the L1 ACK DCI, it is suggested to consider the following agreement. Other indications listed in the RAN1#98 agreement can be further in this meeting since there is some support, particularly the TX power adjustment.

Potential Agreement #6:

The dedicated PUR ACK DCI includes the Timing Advance adjustment (including TA adjustment of 0). The TA adjustment field is 6 bits as legacy.

Regarding the introduction of a flag for L1 ACK, it is suggested to further discuss after agreeing which indication(s) is/are carried by the L1 ACK DCI. It seems the purpose of the L1 flag is different for different companies.

## Unsuccessful PUR transmission

In RAN1#96bis it was agreed that after data transmission on PUR, the UE may expect an explicit indication on NPDCCH for fallback to EDT or RACH.

Based on the additional inputs from companies in Table 8, it is proposed the following:

Potential Agreement #7:

For dedicated PUR in idle mode, the PUR NACK is at least sent on NPDCCH for fallback to EDT or RACH.

Table 8 UE behavior upon unsuccessful PUR transmission

| Source | Proposals |
| --- | --- |
| Ericsson |  |
| Huawei, HiSilicon | Proposal 12: The UE may assume data transmission on PUR is unsuccessful upon receiving an explicit indication on NPDCCH for fallback to EDT or RACH. Observation 12: It is beneficial to allow UE to use the next PUR resource for PUR retransmission with respective to more efficient UL resource usage. Proposal 13: The UL grant for retransmission of a PUR can indicate either the next PUR resource or an explicitly allocated UL resource for PUR retransmission. |
| Lenovo, MM | Proposal 3: After transmission based PUR, UE start to monitor the NPDCCH with the dedicate RNTI expecting an explicit ACK upon successful decoding by eNB or an explicit NACK/an UL retransmission GRANT upon unsuccessful decoding by eNB. - Proposal 6: The NDI field of the PUR DCI is used to differentiate between the L1 PUR ACK and NACK - Proposal 7: In response to “NACK”, UE behaviour depends on the PUR DCI indication. |
| Nokia, NSB | D-PUR DCI should at least support: • An explicit ACK that reuses existing field(s) of DCI format N0 • An UL GRANT for HARQ retransmission • A NACK providing an explicit indication on NPDCCH to fallback to EDT or RACH |
| ZTE | Observation 6: For PUR transmission in RRC_IDLE mode, UE behaviour after receiving L1 NACK or UL grant for retransmission are different. Observation 7: Supporting L1 NACK sent via NPDCCH in PUR search space has benefits for congestion management to eNB. Proposal 9: For PUR transmission in RRC_IDLE mode, L1 NACK is supported and is sent via NPDCCH in PUR search space. |
| Sierra | Proposal 3: The explicitly indication for fallback to EDT or RACH on MPDCCH is included in the L1 ACK DCI and when set, indicates the PUR transmission is not acknowledged. |
| NTT DoCoMo | Proposal 3: For dedicated PUR in idle mode, the PUR NACK is at least sent on NPDCCH. - FFS: details of signalling (e.g. higher layer PUR NACK) |

## No ACK/NACK/UL grant received by the UE after PUR transmission

Agreement RAN1#98 After data transmission on PUR, if nothing is received by the UE in a time period, the UE shall fallback to legacy RACH/EDT procedure. FFS: Details on time period

Table 8 UE behavior if nothing is received

| Source | Proposals |
| --- | --- |
| Sierra | Proposal 1: After data transmission on PUR, if nothing is received by the UE in a PUR search space window, the UE shall fallback to legacy RACH/EDT procedure. |
| LGE | Proposal 4: Discuss how to deal with the contradictory agreements between RAN1 and RAN2 on the UE behavior when UE receives nothing after PUR transmission. |

Potential Agreement #8:

After data transmission on PUR, if nothing is received by the UE in a PUR search space window, the UE shall fallback to legacy RACH/EDT procedure.

## Release PUR

Given the RAN2 agreement that D-PUR configuration is released when the eNB doesn’t detect “m” consecutive UE transmissions, after data transmission on PUR if the UE does not receive any ACK/NACK/UL grant for a period of m consecutive UL transmissions, the UE considers the PUR resources released and may initiate RA/EDT procedures. The eNB can release PUR resources by reconfiguration of allocated resources (the reconfiguration might be the consequence of a notification by the UE, if agreed, as discussed in Section 3.2). No further agreements seem to be needed in RAN1.

## Other

Table 9

| Source | Proposals |
| --- | --- |
| Huawei, HiSilicon | Proposal 13: The UL grant for retransmission of a PUR can indicate either the next PUR resource or an explicitly allocated UL resource for PUR retransmission. |
| ZTE | PUCCH ACK Proposal 8: For PUR transmission in RRC_IDLE mode, PUCCH ACK can be sent by the UE to indicate that the NPDCCH with L1 ACK is successfully decoded. |

# PUR SS overlaps in time domain with Paging CSS

Table 10

| Source | Proposals |
| --- | --- |
| Huawei, HiSilicon | Observation 13: When PUR SS overlaps with Paging CSS, prioritizing PUS SS has the benefit that UE can receive indications related to PUR and do not need to switch DL carrier, but at the cost that UE may miss paging and cause paging delay. Observation 14: The eNB cannot send paging message to a UE through PUR SS since the eNB is not able to map the IMSI/S-TMSI contained in the paging message to a UE configured with PUR, which is due to the fact that higher layer UE_IDs (e.g., IMSI, S-TMSI) for idle mode UEs cannot be maintained by eNB. Observation 15: If the UE prioritizes PUR SS when it overlaps with Paging CSS, - For a UE operated only in DRX mode, the paging delay is small since the DRX cycle is small. - For a UE operated in eDRX mode, the paging delay can be very large since the maximum eDRX cycle can be ~2.9 hours. Proposal 14: After a PUR transmission, when PUR SS overlaps with Paging CSS - For a UE operated only in DRX mode, the UE prioritizes PUR SS - For a UE operated in eDRX mode, the UE prioritizes Paging SS |
| ZTE | Proposal 4: For dedicated PUR in idle mode, when PUR SS in time domain overlaps with paging transmission window (PTW), the UE does not need to monitor paging message in the PTW. |
| LGE | Proposal 6: When PUR search space collides with Type1-CSS, UE is not expected to monitor PUR search space.  FFS whether to drop PUR transmission if the corresponding PUR search space collides with Type1-CSS  FFS whether to count the above case as PUR skipping |

For NB-IoT UE, UE behavior needs to be specified when the UE cannot monitor two search space sets simultaneously. It is suggested to further discuss.

# CFS PUR

Working assumption For NB-IoT allocation with 12 tones, for PUR with R>= [64 or 128 repetitions or ms]: - Allow for UE-specific cyclic shift for DMRS: - For 12 tone allocation, [2 or 4 or 8] cyclic shifts. - For NPUSCH scrambling, the baseline is the current c_init equation by using the configured PUR_RNTI. - Companies are welcome to evaluate baseline c_init, alternative c_init (e.g. c_init for NPDSCH rotation sequence) equations, and other proposals. Note 1: The Working Assumption is also subject to the potential RAN2 and RAN4 specification impacts. Note 2: It is transparent to a given UE whether the eNB is allocating other UEs in the same resource.

The table below summarizes the CFS PUR proposals in this meeting.

Table 12 CFS PUR

| Source | Proposals |
| --- | --- |
| Qualcomm | Proposal 8: Confirm the following working assumption: For NB-IoT allocation with 12 tones, for PUR with R>= 64 repetitions or ms: - Allow for UE-specific cyclic shift for DMRS: - For 12 tone allocation, 8 cyclic shifts. |
| Huawei, HiSilicon | Observation 16: For 12 tones CFS PUR transmission with 128 repetitions and 10% BLER performance, compared with legacy bit-level scrambling, the additional symbol-level scrambling used in legacy NPDSCH can achieve 1.6 dB gain. Proposal 15: Adopt the additional symbol-level scrambling used in legacy NPDSCH on 3/6/12 tones PUR transmission, where the scrambling sequence is initialized at start of each subframe with Cinit=nRNTI+110nf+ns2mod61+1∙29+NIDNcell and the same mapping rule from bit to phase for the symbol scrambling of legacy NPDSCH is applied. Proposal 16: Confirm the working assumption in RAN1#98 for 12 tones PUR transmission: - For NB-IoT allocation with 12 tones, for PUR with R>=16, allow for UE-specific cyclic shift for DMRS: - For 12 tones allocation, 8 cyclic shifts in TS 36.211 Table 5.2.2.1.1-2. Proposal 17: Confirm the working assumption for PUR of 3/6 tones and single tone transmission with R >= [128] repetitions: - For 3/6 tones, allow for UE-specific cyclic shift for DMRS: - For 6 tones allocation, 4 cyclic shifts in TS 36.211 Table 10.1.4.1.2-3. - For 3 tones allocation, 3 cyclic shifts in TS 36.211 Table 10.1.4.1.2-3. - For single tone, allow for UE-specific u in TS 36.211 Table 10.1.4.1.1-1 for DMRS: - For single tone allocation, 16 different u. |
| Intel | Proposal 3 - Confirm the working assumption on CFS PUR supported with orthogonal DMRS for UL MIMO in scenarios with NB-IoT allocation of 12 tones. - Multiple orthogonal DMRS, e.g., by means of UE-specific cyclic shift, are not supported in CFS PUR with a smaller allocation size. - CDMA/OCC-based multiplexing within CFS PUR is not supported in Rel-16. |
| Lenovo, MM | Proposal 15: For CFS PUR, the number of supported UE-specific cyclic shift is configured by eNB. Proposal 16: The scrambling code can be further initialized by the index of cyclic shift. |
| Nokia, NSB | Proposal 9: The RAN1#98 working assumption for the introduction of CFS PUR is agreed with the lower repetition level of 64. |
| Samsung | Proposal #2: CFS PUR using MU-MIMO needn’t be specified with explicit specification change. Proposal #3: CFS PUR using CDM/OCC is not considered. |
| Ericsson | - Observation 19: The WA considers using either 64 or 128 subframes of transmission as starting point for CFS-PUR based on MU-MIMO. In both cases there is a performance degradation with respect to dedicated PUR of ⁓2 dB and ⁓0.89 dB respectively. - Observation 20: CFS-PUR based on MU-MIMO won’t be able to provide the same MCL that dedicated PUR can provide e.g., at 10% BLER. - Observation 21: CFS-PUR based on MU-MIMO will require using a smaller TBS and/or more repetitions to fulfill the BLER target and provide the same MCL that dedicated PUR can provide (i.e., no performance degradation. - Observation 22: According to recent simulations, not even using 256 subframes of transmission provides the same performance as dedicated PUR since still a penalty of 0.37 dB was observed - Proposal 12: Discuss recent CFS-PUR simulation results and possible required modifications to the D-PUR framework before confirming the Working Assumption. |
| ZTE | Proposal 22: More verification is needed before confirming the working assumption. Proposal 23: No further change will be adopted for PUSCH scrambling enhancement in Rel-16. |

The majority of companies suggest introducing CFS PUR in Rel-16 and confirm the working assumptions with some modifications. One company proposes to extend the WA to lower numbers of tone allocation. Two companies propose to further discuss the evaluation results.

The discussed modifications to the working assumptions are related to number of repetitions and cyclic shifts for 12 tone allocation (which are in square brackets in the current WA), and the addition of 6/3/1 tone allocations.

- Number of tones

  - Add 6/3/1 tones

- Number of repetitions

  - With 12 tones, R>=64

  - With 6/3/1 tones, R>=128

- Number of cyclic shifts

  - With 12 tones, 8 cyclic shifts

  - With 6/3 tones, 4/3 cyclic shifts

It is suggested to further discuss the modified WA:

For NB-IoT allocation with 12 tones, for PUR with R>= [64 or 128 repetitions or ms]:

- Allow for UE-specific cyclic shift for DMRS:

  - For 12 tone allocation, [2 or 4 or 8] cyclic shifts in TS 36.211 Table 5.2.2.1.1-2.

For PUR with R >= [128] repetitions:

-	For 3/6 tones, allow for UE-specific cyclic shift for DMRS:

o	For 6 tones allocation, 4 cyclic shifts in TS 36.211 Table 10.1.4.1.2-3.

o	For 3 tones allocation, 3 cyclic shifts in TS 36.211 Table 10.1.4.1.2-3.

-	For single tone, allow for UE-specific u in TS 36.211 Table 10.1.4.1.1-1 for DMRS:

o	For single tone allocation, 16 different u.

# Other

Table 13 collects other proposals that may be reorganized for further discussion during the meeting week.

Table 13

| Source | Proposals |
| --- | --- |
| Samsung | Proposal #1: For Rel-16 NB-IOT, UL SPS in RRC-connected mode can be extended to data transmission from periodic BSR. |
| LGE | - Proposal 7: Discuss whether to support the TPC accumulation mechanism for initial PUR transmissions in NB-IoT. |
| Ericsson | - Proposal 9: Even if the TA is valid, the UE is only allowed to transmit over PUR if the difference between the previous and the current “PL” (i.e., the path loss estimate) is less or equal than X dB. The value of X is [+/-6 dB]. |

# References

- RP-192313	WID revision: Additional enhancements for NB-IoT	Futurewei, RAN#85.

- R1-1909689	RAN1 agreements for Rel-16 Additional Enhancements for NB-IoT	Futurewei, RAN1#98

- R1-1909996	Support for transmission in preconfigured UL resources in NB-IoT	Ericsson

- R1-1910090	Transmission in preconfigured UL resource	Huawei, HiSilicon

- R1-1910151	UL transmission in preconfigured resources for NBIoT		Lenovo, Motorola Mobility

- R1-1910256	Preconfigured Grant for Uplink transmission		Nokia, Nokia Shanghai Bell

- R1-1910272	Support for transmission in preconfigured UL resources for NB-IoT	ZTE

- R1-1910448	Discussion on transmission in preconfigured UL reosurces for NB-IoT		Samsung

- R1-1910525	NB-IOT Pre-configured UL Resources Design Considerations	Sierra Wireless

- R1-1910726	Support for transmission in preconfigured UL resources	Qualcomm

- R1-1910813	Discussion on preconfigured UL resources in NB-IoT	LGE

- R1-1910878	Consideration for preconfigured uplink resources (PUR)	Sequans

- R1-1911153	Discussion on preconfigured UL resources in NB-IoT	NTT DoCoMo

- R1-1911257	UL transmission in preconfigured UL resources for NB-IoT	Intel