3GPP TSG-RAN WG1 Meeting #99	R1-1913151

# Reno, USA, November 18th – 22nd, 2019

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| [DRAFT] CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.215 | CR |  | rev | - | Current version: | 15.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on SFTD measurement in TS 38.215 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R1 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  | Date: |  |  | 2019-11-10 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | It was agreed in both LS R4-1803506 and R2-1804119, and captured by UE capability sftd-MeasNR-Cell in TS 38.306 that SFTD measurement between E-UTRA PCell and NR neighbouring cells is supported in Rel-15 when no NR PSCell is configured. However, its corresponding definition of SFTD measurement is missing in RAN1 specification TS 38.215. It has been agreed in LS R4-1907319 (R1-1907984) that SFTD measurement between NR PCell and NR neighbouring cells should be introduced in Rel-15 for NR SA. However, its corresponding definition of SFTD measurement is missing in RAN1 specification TS 38.215. Additionally, according to received LS’s R1-1911805 and R1-1911806, the row of “Applicable for” of SFTD measurement needs to be updated. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Update the SFTD measurement definition to include the case of SFTD measurement between LTE PCell and NR neighbouring cells when no NR PSCell is configured. - Update the SFTD measurement definition to include the case of SFTD measurement between NR PCell and NR neighbouring cells when no E-UTRA PSCell is configured. - Update the row of Applicable for. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incomplete specification on the definition of SFTD measurement |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.1.14 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | x |  | Other core specifications |  |  |  | TS 36.214 |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Isolated Impact Analysis: The first change in this CR only completes RAN1 specification for SFTD measurement between E-UTRA PCell and NR neighbouring cells according to agreements and involves no functional change. Thus, no impact on gNB/UE implementation due to this change is expected. The second change in this CR only captures changes agreed as part for Rel15 NR standalone and has isolated impact for SFTD measurement: - For a UE not configured with SFTD measurement between NR PCell and NR neighbouring cells in a NR standalone network there is no change to UE behavior. - For a UE in a NR standalone network, SFTD measurement between NR PCell and NR neighbouring cells cannot be configured if either the UE or gNB does not implement this CR. The third change in this CR is only to capture the applicability of SFTD measurements according to RAN4 and RAN2 LS. Thus, no impact on gNB/UE implementation due to this change is expected. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Unchanged parts are omitted>

### 5.1.14	SFN and frame timing difference (SFTD)

| Definition | The observed SFN and frame timing difference (SFTD) between an E-UTRA PCell and an NR PSCell (for EN-DC), or an NR PCell and an E-UTRA PSCell (for NE-DC), or an NR PCell and an NR PSCell (for NR-DC), or an E-UTRA PCell and an NR neighbouring cell, or an NR PCell and an NR neighbouring cell is defined as comprising the following two components: - SFN offset = (SFNPCell - SFNTRGCell) mod 1024, where SFNPCell is the SFN of a PCell radio frame and SFNTRGCell is the SFN of the target-cell radio frame of which the UE receives the start closest in time to the time when it receives the start of the PCell radio frame. - Frame boundary offset = , where TFrameBoundaryPCell is the time when the UE receives the start of a radio frame from the PCell, TFrameBoundaryTRGCell is the time when the UE receives the start of the radio frame, from the target cell, that is closest in time to the radio frame received from the PCell. The unit of (TFrameBoundaryPCell - TFrameBoundaryTRGCell) is Ts. |
| --- | --- |
| Applicable for | RRC_CONNECTED intra-frequency for UEs configured with EN-DC/NE-DC/NR-DC RRC_CONNECTED inter-frequency for UEs configured with NR PCell but no E-UTRA/NR PSCell RRC_CONNECTED inter-RAT for UEs configured with E-UTRA PCell but no NR PSCell |

<Unchanged parts are omitted>