3GPP TSG RAN WG1 Ad-hoc Meeting	R1-151052

Paris, France, 24th – 26th March 2015

Source:	Kyocera

Title:	Channel Selection based on Enhanced UE Measurement Report

Agenda Item:	2.2

Document for:	Discussion/Decision

Introduction

Channel selection is one of the most important features in LAA operation. In addition the hidden node problem is worth studying when designing the channel selection procedures/schemes. In this contribution, we propose enhancements to the UE measurement for channel selection to achieve better performance.

UE measurement report enhancement for channel selection

To deal with the hidden node problem, we propose a UE measurement report mechanism. In the measurement report, UE report the detecting Cell ID and signal power on the unlicensed band in addition to the current RRM measurement result. In our view, UE can detect the non-serving cell’s DRS (including other operator’s LAA) and calculate the DRS RSRP by itself. The eNB that receives this report from the UE can take appropriate action needed to mitigate the hidden node problem. Figure 1 and Table 1 is an example of proposed UE measurement report. In Table 1, UE1 can report to its serving cell indicating Operator B is operating in CC1 and Operator C is operating in CC2. As a result the serving eNB may choose to select CC3 or CC4 to serve its UEs.

Figure 1: Proposed UE measurement report for reporting detected eNBs.

Table 1: Example of proposed UE measurement report

| UE | CC | Detected eNBs | eNB channel selection |
| --- | --- | --- | --- |
| UE 1 report | CC1 | eNB Operator B |  |
|  | CC2 | eNB Operator C |  |
|  | CC3 | Not detected | can be selected by eNB |
|  | CC4 | Not detected | can be selected by eNB |
| UE 2 report | CC1 | eNB Operator D |  |
|  | CC2 | Not detected | can be selected by eNB |
|  | CC3 | eNB Operator E |  |
|  | CC4 | Not detected | can be selected by eNB |

Proposal 1:

New UE measurement report mechanism should be introduced that allows a UE to report the detected non-serving LAA cell’s information.

In addition, there is a potential issue if the same PCI is used by multiple operators. Same PCI should not be allocated to the neighboring cell. Within an operator’s network, it can be achieved by cell planning or SON function. However, the problem remains when the same PCI is used by other operators located in the proximity of the first operator. In our opinion, either the UE assisted or eNB based PCI collision avoidance mechanism in unlicensed spectrum should be introduced.

Proposal 2:

PCI collision avoidance mechanism in unlicensed spectrum should be introduced.

Conclusions

Proposal 1:

New UE measurement report mechanism should be introduced that allows a UE to report the detected non-serving LAA cell’s information.

Proposal 2:

PCI collision avoidance mechanism in unlicensed spectrum should be introduced.