3GPP TSG RAN WG1 Meeting Ad-hoc                                                                              R1-151144

Paris, France, 24th – 26th  March 2015

Source: 	Ericsson

Title:	Summary of Email Discussion [80-04] on DL-only LAA Coexistence Evaluation Results

Agenda Item:	2.1

Document for:	Discussion

# Introduction

The following tables capture initial DL-only coexistence evaluation results using the instructions and table templates provided in R1-150890 as the outcome of the email discussion [80-04].

Some clarifications are given below to facilitate understanding the tables’ entries:

- Step 1 and step 2 in Wi-Fi and LAA coexistence evaluations are described in section 8.1 in TR 36.889.

- LAA operator 1 and LAA operator 2 refer to the two operators in LAA and LAA coexistence case.

- UPT and delay CDF are intended for the FTP traffic.

- VoIP outage percentage refers to the percentage of the VoIP users with 98%-ile latency greater than 50ms.

- 𝜆 denotes the traffic arrival rate.

- 𝜌 denotes the ratio between the mean served cell throughput to the mean offered cell throughput for a given traffic arrival rate as the following:

ρ=Mean served cell throughputMean offered cell throughput

- BO denotes the mean buffer occupancy.

- “Additional comments” is a place holder where companies can provide important information regarding optional assumptions that have been used or assumptions different from the baseline assumptions that are assumed as well as provide brief descriptions of the LBT schemes.

- As agreed in RAN1#80 meeting, the evaluated LBT schemes are classified according to the following categories:

  - Category 1: No LBT

  - Category 2: LBT without random back-off

  - Category 3: LBT with random back-off with fixed size of contention window

  - Category 4: LBT with random back-off with variable size of contention window

# DL-Only LAA Coexistence Evaluation Results

The list of the results tables in this section is provided below for convenience followed by the actual tables. Table 1 to Table 6 capture the indoor coexistence DL-only evaluation results while the corresponding outdoor results are given in Table 7 to Table 12.

- Table 1: Indoor deployment for Wi-Fi and LAA coexistence case with one shared unlicensed carrier and FTP traffic

- Table 2: Indoor deployment for LAA and LAA coexistence case with one shared unlicensed carrier and FTP traffic

- Table 3: Indoor deployment for Wi-Fi and LAA coexistence case with one shared unlicensed carrier and Mixed traffic

- Table 4: Indoor deployment for Wi-Fi and LAA coexistence case with four shared unlicensed carriers and FTP traffic

- Table 5: Indoor deployment for LAA and LAA coexistence case with four shared unlicensed carriers and FTP traffic

- Table 6: Indoor deployment for Wi-Fi and LAA coexistence case with four shared unlicensed carriers and Mixed traffic

- Table 7: Outdoor deployment for Wi-Fi and LAA coexistence case with one shared unlicensed carrier and FTP traffic

- Table 8: Outdoor deployment for LAA and LAA coexistence case with one shared unlicensed carrier and FTP traffic

- Table 9: Outdoor deployment for Wi-Fi and LAA coexistence case with one shared unlicensed carrier and Mixed traffic

- Table 10: Outdoor deployment for Wi-Fi and LAA coexistence case with four shared unlicensed carriers and FTP traffic

- Table 11: Outdoor deployment for LAA and LAA coexistence case with four shared unlicensed carriers and FTP traffic

- Table 12: Outdoor deployment for Wi-Fi and LAA coexistence case with four shared unlicensed carriers and Mixed traffic

Table 1: Indoor deployment for Wi-Fi and LAA coexistence case with one shared unlicensed carrier and FTP traffic

| Tdoc / Company | LAA LBT cat. | Reported parameters |  | Low load BO range for Wi-Fi in Step 1: 10%~25% |  |  | Medium load BO range for Wi-Fi in Step 1: 35%~50% |  |  | High load BO range for Wi-Fi in Step 1: above 55% |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Wi-Fi in step 1 | Wi-Fi in step 2 | LAA in step 2 | Wi-Fi in step 1 | Wi-Fi in step 2 | LAA in step 2 | Wi-Fi in step 1 | Wi-Fi in step 2 | LAA in step 2 |
| Ericsson | 3 | UPT CDF [Mbps] | 5% | 22.58 | (48.17) 35.53 | (90.50) 41.29 | 6.43 | (45.97) 25.52 | (79.95) 26.56 | 2.40 | (44.97) 24.67 | (78.09) 24.89 |
|  |  |  | 50% | 56.98 | (82.54) 75.98 | (127.9) 75.77 | 31.87 | (77.44) 65.78 | (118.8) 63.57 | 16.02 | (76.83) 64.43 | (117.8) 61.73 |
|  |  |  | 95% | 81.89 | (101.9) 97.91 | (149.2) 96.34 | 60.90 | (96.56) 90.13 | (141.8) 87.11 | 38.04 | (96.05) 89.19 | (141.3) 85.83 |
|  |  |  | Mean | 56.96 | (82.43) 74.65 | (127.6) 75.06 | 33.7 | (77.84) 65.18 | (119) 63.58 | 18.5 | (77.21) 64.02 | (118) 61.91 |
|  |  | Delay CDF [s] | 5% | 0.115 | (0.036) 0.039 | (0.023) 0.04 | 0.077 | (0.038) 0.043 | (0.026) 0.045 | 0.128 | (0.038) 0.044 | (0.026) 0.046 |
|  |  |  | 50% | 0.521 | (0.055) 0.071 | (0.034) 0.079 | 0.317 | (0.062) 0.093 | (0.039) 0.107 | 0.690 | (0.063) 0.095 | (0.039) 0.113 |
|  |  |  | 95% | 0.711 | (0.095) 0.26 | (0.051) 0.342 | 1.901 | (0.110) 0.455 | (0.064) 0.675 | 2.774 | (0.114) 0.502 | (0.064) 0.735 |
|  |  |  | Mean | 0.247 | (0.062) (0.112) | (0.037) (0.132) | 0.642 | (0.069) 0.165 | (0.042) 0.216 | 1.104 | (0.07) 0.175 | (0.043) 0.231 |
|  |  | 𝜌 |  | 0.9826 | (0.998)0.993 | (0.999)0.992 | 0.9286 | (0.998) 0.985 | (0.999)0.984 | 0.7947 | (0.998)0.984 | (0.999)0.982 |
|  |  | BO |  | 0.20 | (0.091)0.127 | (0.061)0.072 | 0.40 | (0.115) 0.180 | (0.079)0.106 | 0.60 | (0.119) 0.189 | (0.082) 0.113 |
|  |  | 𝜆 |  | 0.7 files/s |  |  | 0.88 files/s |  |  | 1.05 files/s |  |  |
|  | Additional comments: (*(): results with licensed carrier) 256 QAM, LDPC, 2Tx2Rx, TXOP 4ms, and LBT algorithm based on R1-150584 where the LAA CCA starts and resumes at the subframe boundaries. If LBT succeeds before the 4th OFDM symbol in a subframe the channel is occupied by the LAA node. If the LBT fails by the 4th OFDM symbol in a subframe, the random backoff counter is frozen and the CCA is postponed to the next subframe boundary where the CCA count down is resumed. |  |  |  |  |  |  |  |  |  |  |  |
| InterDigital IDCC | LBE Cat. 3 | UPT CDF [Mbps] | 5% | 8.008 | 10.091 | 10.563 | 4.168 | 3.140 | 3.192 | 0.902 | 1.310 | 0.983 |
|  |  |  | 50% | 48.054 | 46.044 | 67.626 | 30.368 | 23.070 | 36.131 | 12.697 | 12.845 | 9.384 |
|  |  |  | 95% | 114.308 | 120.457 | 143.235 | 120.223 | 120.153 | 143.405 | 55.674 | 72.683 | 91.814 |
|  |  |  | Mean | 56.260 | 57.739 | 77.198 | 42.841 | 36.823 | 56.880 | 17.748 | 21.354 | 21.172 |
|  |  | Delay CDF [s] | 5% | 0.036 | 0.035 | 0.030 | 0.035 | 0.035 | 0.030 | 0.075 | 0.059 | 0.049 |
|  |  |  | 50% | 0.092 | 0.095 | 0.063 | 0.148 | 0.177 | 0.118 | 0.309 | 0.326 | 0.423 |
|  |  |  | 95% | 0.447 | 0.386 | 0.356 | 1.145 | 1.392 | 1.245 | 3.587 | 2.726 | 3.602 |
|  |  |  | Mean | 0.153 | 0.141 | 0.113 | 0.304 | 0.347 | 0.311 | 0.905 | 0.838 | 0.985 |
|  |  | 𝜌 |  | 0.9514 | 0.9519 | 0.8885 | 0.9651 | 0.9644 | 0.8902 | 0.7926 | 0.8074 | 0.7578 |
|  |  | BO |  | 20.909 | 18.418 | 18.264 | 36.191 | 39.4018 | 39.496 | 66.763 | 72.027 | 77.653 |
|  |  | 𝜆 |  | 0.8 |  |  | 0.95 |  |  | 1.2 |  |  |
|  | Additional comments: TXOP=4ms, without licensed carrier LBE Cat.3: Option B from ETSI EN 301 893 v1.8.0, Section 4.8.3.2. CW size fixed and set by q at beginning of simulation. Random backoff N selected from [1;q]. TXOP = q*13/32 ms. |  |  |  |  |  |  |  |  |  |  |  |
| IDCC | LBE Cat. 4 | UPT CDF [Mbps] | 5% | 8.008 | 13.313 | 10.914 | 4.168 | 4.335 | 3.328 | 0.902 | 1.300 | 0.637 |
|  |  |  | 50% | 48.054 | 49.583 | 72.248 | 30.368 | 23.853 | 28.967 | 12.697 | 12.786 | 8.236 |
|  |  |  | 95% | 114.308 | 120.457 | 143.008 | 120.223 | 120.098 | 142.838 | 55.674 | 68.239 | 102.119 |
|  |  |  | Mean | 56.260 | 60.551 | 79.499 | 42.841 | 39.180 | 53.100 | 17.748 | 20.990 | 21.2632 |
|  |  | Delay CDF [s] | 5% | 0.036 | 0.035 | 0.030 | 0.035 | 0.035 | 0.030 | 0.075 | 0.057 | 0.046 |
|  |  |  | 50% | 0.092 | 0.086 | 0.059 | 0.148 | 0.172 | 0.140 | 0.309 | 0.343 | 0.511 |
|  |  |  | 95% | 0.447 | 0.289 | 0.362 | 1.145 | 0.892 | 1.271 | 3.587 | 2.966 | 3.945 |
|  |  |  | Mean | 0.153 | 0.122 | 0.112 | 0.304 | 0.269 | 0.309 | 0.905 | 0.807 | 1.005 |
|  |  | 𝜌 |  | 0.9514 | 0.9513 | 0.8896 | 0.9651 | 0.9641 | 0.8897 | 0.7926 | 0.8463 | 0.7303 |
|  |  | BO |  | 20.909 | 15.327 | 16.764 | 36.191 | 35.902 | 39.324 | 66.763 | 72.241 | 81.895 |
|  |  | 𝜆 |  | 0.8 |  |  | 1.0 |  |  | 1.2 |  |  |
|  | Additional comments: TXOP=4ms, without licensed carrier LBE Cat.4: Modified Option A from ETSI EN 301 893 v1.8.0, Section 4.8.3.2. CW exponentially variable. Initially set to q=16 and doubled whenever N idle slots cannot be found within q observation slots. TXOP independent of q, fixed at beginning of simulation. 4ms and 13 ms (although technically Option A requires TXOP<10ms). |  |  |  |  |  |  |  |  |  |  |  |
| IDCC | LBE Cat. 3 | UPT CDF [Mbps] | 5% | 8.008 | 12.378 | 16.375 | 4.168 | 7.903 | 9.435 | 0.902 | 1.331 | 1.852 |
|  |  |  | 50% | 48.054 | 49.514 | 72.758 | 30.368 | 38.221 | 56.442 | 12.697 | 15.647 | 20.285 |
|  |  |  | 95% | 114.308 | 120.526 | 143.008 | 120.223 | 120.401 | 143.122 | 55.674 | 89.739 | 103.345 |
|  |  |  | Mean | 56.260 | 60.644 | 81.771 | 42.841 | 52.344 | 73.123 | 17.748 | 24.521 | 34.442 |
|  |  | Delay CDF [s] | 5% | 0.036 | 0.035 | 0.030 | 0.035 | 0.035 | 0.030 | 0.075 | 0.053 | 0.046 |
|  |  |  | 50% | 0.092 | 0.089 | 0.059 | 0.148 | 0.104 | 0.073 | 0.309 | 0.323 | 0.206 |
|  |  |  | 95% | 0.447 | 0.324 | 0.261 | 1.145 | 0.534 | 0.429 | 3.587 | 2.370 | 2.195 |
|  |  |  | Mean | 0.153 | 0.124 | 0.090 | 0.304 | 0.178 | 0.128 | 0.905 | 0.607 | 0.568 |
|  |  | 𝜌 |  | 0.9514 | 0.9517 | 0.8906 | 0.9651 | 0.9653 | 0.8934 | 0.7926 | 0.8927 | 0.8954 |
|  |  | BO |  | 20.909 | 17.172 | 14.673 | 36.191 | 25.991 | 21.549 | 66.763 | 63.333 | 54.977 |
|  |  | 𝜆 |  | 0.8 |  |  | 0.95 |  |  | 1.2 |  |  |
|  | Additional comments: TXOP=13ms, without licensed carrier LBE Cat.3: Option B from ETSI EN 301 893 v1.8.0, Section 4.8.3.2. CW size fixed and set by q at beginning of simulation. Random backoff N selected from [1;q]. TXOP = q*13/32 ms. |  |  |  |  |  |  |  |  |  |  |  |
| IDCC | LBE Cat. 4 | UPT CDF [Mbps] | 5% | 8.008 | 13.028 | 16.3125 | 4.168 | 7.652 | 11.444 | 0.902 | 2.119 | 1.766 |
|  |  |  | 50% | 48.054 | 48.886 | 72.804 | 30.368 | 36.948 | 59.516 | 12.697 | 14.988 | 25.103 |
|  |  |  | 95% | 114.308 | 120.443 | 143.008 | 120.223 | 120.401 | 143.292 | 55.674 | 79.364 | 133.417 |
|  |  |  | Mean | 56.260 | 58.372 | 84.178 | 42.841 | 50.159 | 75.921 | 17.748 | 24.240 | 38.253 |
|  |  | Delay CDF [s] | 5% | 0.036 | 0.035 | 0.030 | 0.035 | 0.035 | 0.030 | 0.075 | 0.046 | 0.031 |
|  |  |  | 50% | 0.092 | 0.086 | 0.058 | 0.148 | 0.111 | 0.069 | 0.309 | 0.264 | 0.164 |
|  |  |  | 95% | 0.447 | 0.314 | 0.235 | 1.145 | 0.530 | 0.350 | 3.587 | 1.799 | 2.191 |
|  |  |  | Mean | 0.153 | 0.123 | 0.081 | 0.304 | 0.187 | 0.117 | 0.905 | 0.512 | 0.522 |
|  |  | 𝜌 |  | 0.9514 | 0.9515 | 0.8906 | 0.9651 | 0.9652 | 0.89401 | 0.7926 | 0.8993 | 0.8908 |
|  |  | BO |  | 20.909 | 16.317 | 12.943 | 36.191 | 27.4332 | 20.2035 | 66.763 | 60.236 | 52.213 |
|  |  | 𝜆 |  | 0.8 |  |  | 0.95 |  |  | 1.2 |  |  |
|  | Additional comments: TXOP=13ms, without licensed carrier LBE Cat.4: Modified Option A from ETSI EN 301 893 v1.8.0, Section 4.8.3.2. CW exponentially variable. Initially set to q=16 and doubled whenever N idle slots cannot be found within q observation slots. TXOP independent of q, fixed at beginning of simulation. 4ms and 13 ms (although technically Option A requires TXOP<10ms). |  |  |  |  |  |  |  |  |  |  |  |
| Nokia Corporation / Nokia Networks | 2 | UPT CDF [Mbps] | 5% | 7.25 | 16.41 | 7.76 | 0.00 | 11.25 | 3.23 | 0.00 | 5.68 | 0.74 |
|  |  |  | 50% | 29.75 | 50.55 | 44.17 | 9.44 | 39.15 | 26.86 | 1.23 | 26.69 | 6.58 |
|  |  |  | 95% | 59.78 | 59.54 | 71.40 | 58.83 | 59.35 | 71.24 | 13.47 | 55.33 | 69.81 |
|  |  |  | Mean | 33.18 | 44.55 | 44.93 | 15.02 | 38.64 | 33.66 | 3.45 | 28.84 | 15.56 |
|  |  | Delay CDF [s] | 5% | 0.067 | 0.067 | 0.056 | 0.068 | 0.067 | 0.056 | 0.278 | 0.072 | 0.057 |
|  |  |  | 50% | 0.134 | 0.079 | 0.091 | 0.409 | 0.102 | 0.148 | 2.514 | 0.149 | 0.601 |
|  |  |  | 95% | 0.535 | 0.242 | 0.501 | 4.569 | 0.353 | 1.196 | 12.022 | 0.679 | 5.369 |
|  |  |  | Mean | 0.201 | 0.116 | 0.158 | 1.039 | 0.148 | 0.324 | 3.942 | 0.244 | 1.349 |
|  |  | 𝜌 |  | 0.995 | 0.997 | 0.987 | 0.923 | 0.996 | 0.980 | 0.624 | 0.991 | 0.854 |
|  |  | BO |  | 14.92 | 9.60 | 12.67 | 41.44 | 15.11 | 25.77 | 75.59 | 27.99 | 62.77 |
|  |  | 𝜆 |  | 0.4 |  |  | 0.55 |  |  | 0.8 |  |  |
|  | Additional comments: LBT algorithm according to FBE operation as described in R1-150499. Other details: 1Tx2Rx, TXOP 1ms, BCC, no 256QAM. No user data (i.e. PDSCH) is transmitted on the licensed band carrier in case a UE is configured with LAA. |  |  |  |  |  |  |  |  |  |  |  |
| ALU, ASB | LBE Cat 3 | UPT CDF [Mbps] | 5% | 10.29 | 26.35 | 40.37 | 5.10 | 3.10 | 15.15 | 3.53 | 2.22 | 16.14 |
|  |  |  | 50% | 67.38 | 74.19 | 73.19 | 47.34 | 52.49 | 58.66 | 35.02 | 34.58 | 44.68 |
|  |  |  | 95% | 101.44 | 103.10 | 95.68 | 88.82 | 84.77 | 86.05 | 76.72 | 70.14 | 75.22 |
|  |  |  | Mean | 63.23 | 70.98 | 71.76 | 47.82 | 49.19 | 56.96 | 36.19 | 35.50 | 44.54 |
|  |  | Delay CDF [s] | 5% | 0.032 | 0.032 | 0.036 | 0.032 | 0.032 | 0.036 | 0.032 | 0.032 | 0.036 |
|  |  |  | 50% | 0.064 | 0.057 | 0.055 | 0.095 | 0.092 | 0.071 | 0.163 | 0.159 | 0.100 |
|  |  |  | 95% | 0.852 | 0.426 | 0.171 | 1.629 | 1.930 | 0.375 | 2.675 | 3.857 | 0.683 |
|  |  |  | Mean | 0.203 | 0.119 | 0.077 | 0.362 | 0.389 | 0.133 | 0.585 | 0.715 | 0.201 |
|  |  | 𝜌 |  | 0.95 | 0.96 | 0.88 | 0.87 | 0.88 | 0.84 | 0.79 | 0.75 | 0.79 |
|  |  | BO |  | 0.24 | 0.17 | 0.13 | 0.38 | 0.37 | 0.22 | 0.55 | 0.56 | 0.33 |
|  |  | 𝜆 |  | 0.70 |  |  | 0.85 |  |  | 1.00 |  |  |
|  | Additional comments: LBE, q Value:32, Maximum Occupancy Time : 10ms, LAA without licensed carrier 2Tx2Rx, BCC, no 256QAM CCA threshold: -73 dBm/MHz + 23 - PH, PH specified in dBm EIRP for CCA-ED of LAA, -62 dBm for CCA-ED of Wi-Fi. |  |  |  |  |  |  |  |  |  |  |  |
| ALU, ASB | FBE Cat 2 | UPT CDF [Mbps] | 5% | 10.29 | 26.57 | 24.62 | 5.10 | 8.22 | 8.10 | 3.53 | 5.71 | 5.73 |
|  |  |  | 50% | 67.38 | 87.49 | 70.39 | 47.34 | 66.75 | 51.14 | 35.02 | 51.76 | 36.38 |
|  |  |  | 95% | 101.44 | 112.93 | 93.52 | 88.82 | 102.28 | 83.74 | 76.72 | 92.47 | 70.62 |
|  |  |  | Mean | 63.23 | 82.39 | 67.47 | 47.82 | 61.56 | 49.09 | 36.19 | 49.98 | 36.83 |
|  |  | Delay CDF [s] | 5% | 0.032 | 0.032 | 0.037 | 0.032 | 0.032 | 0.037 | 0.032 | 0.032 | 0.037 |
|  |  |  | 50% | 0.064 | 0.038 | 0.056 | 0.095 | 0.066 | 0.086 | 0.163 | 0.094 | 0.134 |
|  |  |  | 95% | 0.852 | 0.339 | 0.304 | 1.629 | 1.086 | 0.847 | 2.675 | 1.838 | 2.048 |
|  |  |  | Mean | 0.203 | 0.119 | 0.146 | 0.362 | 0.246 | 0.221 | 0.585 | 0.418 | 0.424 |
|  |  | 𝜌 |  | 0.95 | 0.95 | 0.85 | 0.87 | 0.91 | 0.78 | 0.79 | 0.74 | 0.59 |
|  |  | BO |  | 0.24 | 0.15 | 0.17 | 0.38 | 0.28 | 0.31 | 0.55 | 0.43 | 0.48 |
|  |  | 𝜆 |  | 0.7 |  |  | 0.85 |  |  | 1 |  |  |
|  | Additional comments: FBE frame period: 10ms(Maximum Occupancy Time= 9.5ms, idle time = 0.5ms), LAA without licensed carrier 2Tx2Rx, BCC, no 256QAM CCA threshold: -73 dBm/MHz + 23 - PH, PH specified in dBm EIRP for CCA-ED of LAA, -62 dBm for CCA-ED of Wi-Fi; |  |  |  |  |  |  |  |  |  |  |  |
| QCOM | LBT Cat 3 | UPT CDF [Mbps] | 5% | 32.68 | 31.09 | 25.11 | 9.79 | 9.19 | 7.13 | 0.26 | 0.26 | 0.20 |
|  |  |  | 50% | 64.99 | 71.61 | 69.64 | 45.44 | 50.57 | 46.01 | 26.18 | 31.62 | 24.55 |
|  |  |  | 95% | 82.47 | 90.32 | 89.70 | 69.81 | 78.95 | 73.52 | 59.37 | 68.29 | 57.98 |
|  |  |  | Mean | 62.44 | 67.25 | 65.70 | 43.43 | 47.72 | 42.71 | 26.87 | 32.05 | 25.15 |
|  |  | Delay CDF [s] | 5% | 7.65e-3 | 8.04e-3 | 9.96e-3 | 2.55e-2 | 2.72e-2 | 3.51e-2 | 9.62e-1 | 9.62e-1 | 1.25 |
|  |  |  | 50% | 3.85e-3 | 3.49e-3 | 3.59e-3 | 5.50e-3 | 4.94e-3 | 5.43e-3 | 9.55e-3 | 7.91e-3 | 1.02e-2 |
|  |  |  | 95% | 3.03e-3 | 2.77e-3 | 2.79e-3 | 3.58e-3 | 3.17e-3 | 3.40e-3 | 4.21e-3 | 3.66e-3 | 4.31e-3 |
|  |  |  | Mean | 4.00e-3 | 3.72e-3 | 3.81e-3 | 5.76e-3 | 5.24e-3 | 5.85e-3 | 9.30e-3 | 7.80e-3 | 9.94e-3 |
|  |  | 𝜌 |  | 1.00 | 1.00 | 1.00 | 0.98 | 0.98 | 0.96 | 0.92 | 0.93 | 0.91 |
|  |  | BO |  | 20.0% | 19.2% | 20.0% | 40.0% | 39.3% | 40.0% | 60.0% | 58.5% | 58.4% |
|  |  | 𝜆 |  | 1.48 Mbps |  |  | 1.85 Mbps |  |  | 2.21 Mbps |  |  |
|  | Additional comments: For the WiFi operator, CCA-ED = -62dBm, Preamble detect at 4dB SINR, defer period based on AIFS, 256QAM not used, LDPC codes used, RTS/CTS not used and 3ms TXOP. For the LAA operator, all traffic for unlicensed only carrier, CCA-ED = -82dBm, q = 32, LBT Cat 3 as defined in ETSI EN 301 893 v1.8.0, Max TXOP 9.5ms and 256QAM not used |  |  |  |  |  |  |  |  |  |  |  |
| Sharp | 2 | UPT CDF [Mbps] | 5% | 9.50 | 27.87 | 32.93 | 1.13 | 21.53 | 25.21 | 0.23 | 14.51 | 14.92 |
|  |  |  | 50% | 33.63 | 47.81 | 55.56 | 20.19 | 44.25 | 50.50 | 6.67 | 39.09 | 43.50 |
|  |  |  | 95% | 51.75 | 61.41 | 73.24 | 42.53 | 58.30 | 71.70 | 30.35 | 55.52 | 66.27 |
|  |  |  | Mean | 32.73 | 46.70 | 54.68 | 20.73 | 42.63 | 49.43 | 9.87 | 37.57 | 42.49 |
|  |  | Delay CDF [s] | 5% | 0.064 | 0.064 | 0.054 | 0.065 | 0.064 | 0.054 | 0.092 | 0.065 | 0.057 |
|  |  |  | 50% | 0.131 | 0.073 | 0.057 | 0.232 | 0.089 | 0.071 | 0.560 | 0.120 | 0.151 |
|  |  |  | 95% | 0.641 | 0.248 | 0.254 | 1.859 | 0.344 | 0.375 | 3.806 | 0.753 | 1.228 |
|  |  |  | Mean | 0.221 | 0.110 | 0.104 | 0.497 | 0.139 | 0.138 | 1.033 | 0.185 | 0.214 |
|  |  | 𝜌 |  | 1.00 | 1.00 | 1.00 | 0.96 | 1.00 | 1.00 | 0.87 | 1.00 | 0.99 |
|  |  | BO |  | 0.21 | 0.13 | 0.12 | 0.40 | 0.17 | 0.17 | 0.63 | 0.24 | 0.25 |
|  |  | 𝜆 |  | 0.5 |  |  | 0.6 |  |  | 0.7 |  |  |
|  | Additional comments: results without licensed carrier, TXOP and maximum duration time for LAA = 4ms. LBT algorithm according to LBT scheme 2 as described in R1-150694. Other details: 1Tx2Rx, BCC for WiFi, WiFi DL only with ACK modeled, 256 QAM not implemented. |  |  |  |  |  |  |  |  |  |  |  |
| Sharp | 4 | UPT CDF [Mbps] | 5% | 9.50 | 18.73 | 34.10 | 1.13 | 10.69 | 28.06 | 0.23 | 2.80 | 18.40 |
|  |  |  | 50% | 33.63 | 37.78 | 53.52 | 20.19 | 30.88 | 48.72 | 6.67 | 22.01 | 42.11 |
|  |  |  | 95% | 51.75 | 55.15 | 70.76 | 42.53 | 48.54 | 65.92 | 30.35 | 43.45 | 62.56 |
|  |  |  | Mean | 32.73 | 37.44 | 52.91 | 20.73 | 30.46 | 47.69 | 9.87 | 22.46 | 41.51 |
|  |  | Delay CDF [s] | 5% | 0.064 | 0.064 | 0.055 | 0.065 | 0.064 | 0.055 | 0.092 | 0.064 | 0.054 |
|  |  |  | 50% | 0.131 | 0.114 | 0.072 | 0.232 | 0.146 | 0.086 | 0.560 | 0.109 | 0.101 |
|  |  |  | 95% | 0.641 | 0.402 | 0.183 | 1.859 | 0.702 | 0.235 | 3.806 | 0.532 | 0.671 |
|  |  |  | Mean | 0.221 | 0.160 | 0.093 | 0.497 | 0.241 | 0.116 | 1.033 | 0.416 | 0.153 |
|  |  | 𝜌 |  | 1.00 | 1.00 | 1.00 | 0.96 | 1.00 | 1.00 | 0.87 | 0.98 | 1.00 |
|  |  | BO |  | 0.21 | 0.17 | 0.11 | 0.40 | 0.26 | 0.16 | 0.63 | 0.39 | 0.21 |
|  |  | 𝜆 |  | 0.5 |  |  | 0.6 |  |  | 0.7 |  |  |
|  | Additional comments: results without licensed carrier, TXOP and maximum duration time for LAA = 4ms. LBT algorithm according to LBT scheme 1 as described in R1-150694. Other details: 1Tx2Rx, BCC for WiFi, WiFi DL only with ACK modeled, 256 QAM not implemented. |  |  |  |  |  |  |  |  |  |  |  |
| Kyocera | 2 | UPT CDF [Mbps] | 5% | 4.23 | 9.78 | 2.16 | 0.63 | 2.16 | 1.32 | 0.43 | 0.93 | 0.41 |
|  |  |  | 50% | 27.03 | 34.33 | 26.67 | 9.43 | 16.99 | 8.14 | 3.11 | 5.51 | 2.76 |
|  |  |  | 95% | 45.98 | 45.98 | 57.14 | 26.32 | 33.06 | 41.38 | 15.38 | 25.48 | 15.04 |
|  |  |  | Mean | 26.50 | 32.59 | 29.10 | 10.59 | 17.50 | 11.53 | 4.75 | 8.31 | 4.66 |
|  |  | Delay CDF [s] | 5% | 0.09 | 0.09 | 0.07 | 0.15 | 0.12 | 0.10 | 0.22 | 0.16 | 0.19 |
|  |  |  | 50% | 0.15 | 0.12 | 0.15 | 0.39 | 0.23 | 0.52 | 1.36 | 0.84 | 1.22 |
|  |  |  | 95% | 0.95 | 0.46 | 2.38 | 3.92 | 1.20 | 2.43 | 3.94 | 3.25 | 4.96 |
|  |  |  | Mean | 0.27 | 0.17 | 0.36 | 0.82 | 0.38 | 0.84 | 1.54 | 1.15 | 1.84 |
|  |  | 𝜌 |  | 0.97 | 0.90 | 0.88 | 0.90 | 0.79 | 0.74 | 0.84 | 0.68 | 0.64 |
|  |  | BO |  | 0.15 | 0.12 | 0.13 | 0.43 | 0.33 | 0.38 | 0.70 | 0.64 | 0.69 |
|  |  | 𝜆 |  | 0.3 |  |  | 0.5 |  |  | 0.7 |  |  |
|  | Additional comments: Unlicensed carrier only, 1Tx2Rx, no 256 QAM, BCC, no RTS/CTS LBT: Based on FBE 1) a fixed length LBT before the transmission burst 2) at least 1 subframe IDLE period after the transmission burst. 3) the maximum burst length is assumed to be 4msec. |  |  |  |  |  |  |  |  |  |  |  |
| ZTE | 3 | UPT CDF [Mbps] | 5% | 24 | 26 | 52 | 9.85 | 29 | 44 | 2.23 | 2 | 7 |
|  |  |  | 50% | 31 | 38 | 82 | 13.41 | 32 | 68 | 6.44 | 19 | 24 |
|  |  |  | 95% | 36 | 40 | 98 | 20.03 | 40 | 102 | 9.56 | 30 | 58 |
|  |  |  | Mean | 31 | 37 | 81 | 14 | 33 | 69 | 6.6 | 18 | 30 |
|  |  | Delay CDF [s] | 5% | 0.076 | 0.076 | 0.032 | 0.115 | 0.076 | 0.032 | 0.176 | 0.081 | 0.037 |
|  |  |  | 50% | 0.144 | 0.112 | 0.043 | 0.331 | 0.124 | 0.058 | 0.92 | 0.32 | 0.204 |
|  |  |  | 95% | 0.359 | 0.243 | 0.213 | 1.349 | 0.355 | 0.287 | 4.244 | 2.178 | 1.872 |
|  |  |  | Mean | 0.169 | 0.13 | 0.075 | 0.476 | 0.151 | 0.099 | 1.45 | 0.589 | 0.469 |
|  |  | 𝜌 |  | 99.40% | 99.40% | 99.80% | 99.60% | 99.10% | 99.70% | 93.50% | 98.00% | 99.30% |
|  |  | BO |  | 19% | 15% | 10% | 47% | 19% | 16% | 76% | 48% | 47% |
|  |  | 𝜆 |  | 1.5 |  |  | 2.25 |  |  | 2.75 |  |  |
|  | Additional comments: No 256QAM, no LDPC code, 2Tx2Rx, 24 dBm TX power for both LAA and WiFi, no licensed carrier for LAA, LAA max. duration =4 ms, RTS/CTS enabled for WiFi |  |  |  |  |  |  |  |  |  |  |  |
| Fujitsu | 3 | UPT CDF [Mbps] | 5% | 3.792 | 16.756 | 34.871 | 3.569 | 8.055 | 22.119 | 2.749 | 4.544 | 11.820 |
|  |  |  | 50% | 34.551 | 46.414 | 54.354 | 17.625 | 32.096 | 38.061 | 8.504 | 21.591 | 28.883 |
|  |  |  | 95% | 55.275 | 68.443 | 71.396 | 31.873 | 49.731 | 53.901 | 22.540 | 35.231 | 42.292 |
|  |  |  | Mean | 34.037 | 45.968 | 53.820 | 17.773 | 32.030 | 38.072 | 9.807 | 28.190 | 21.613 |
|  |  | Delay CDF [s] | 5% | 0.036 | 0.036 | 0.029 | 0.052 | 0.040 | 0.029 | 0.090 | 0.047 | 0.038 |
|  |  |  | 50% | 0.098 | 0.077 | 0.066 | 0.212 | 0.109 | 0.092 | 0.560 | 0.169 | 0.121 |
|  |  |  | 95% | 0.804 | 0.308 | 0.176 | 1.704 | 0.711 | 0.265 | 4.025 | 1.189 | 0.492 |
|  |  |  | Mean | 0.234 | 0.144 | 0.078 | 0.453 | 0.217 | 0.112 | 1.080 | 0.332 | 0.174 |
|  |  | 𝜌 |  | 0.998 | 0.998 | 0.999 | 0.987 | 0.997 | 0.994 | 0.937 | 0.989 | 0.995 |
|  |  | BO |  | 19.133 | 13.983 | 11.525 | 41.314 | 25.817 | 20.533 | 68.273 | 39.292 | 30.710 |
|  |  | 𝜆 |  | 0.7 |  |  | 0.95 |  |  | 1.1 |  |  |
|  | Additional comments: backoff method 2 in R1-150185, FTP model 1 |  |  |  |  |  |  |  |  |  |  |  |
| Huawei, HiSilicon, | Cat.3 | UPT CDF [Mbps] | 5% | 3.83 | 4.66 | 11.28 | 1.11 | 3.43 | 9.26 | 0.11 | 0.28 | 2.14 |
|  |  |  | 50% | 29.59 | 34.07 | 37.34 | 4.58 | 13.77 | 22.57 | 0.53 | 4.01 | 12.58 |
|  |  |  | 95% | 42.07 | 47.55 | 49.54 | 16.34 | 29.96 | 38.87 | 5.27 | 20.69 | 34.73 |
|  |  |  | Mean | 27.30 | 33.08 | 35.05 | 6.26 | 14.57 | 23.79 | 1.24 | 5.99 | 15.41 |
|  |  | Delay CDF [s] | 5% | 0.094 | 0.083 | 0.080 | 0.24 | 0.13 | 0.10 | 0.70 | 0.18 | 0.11 |
|  |  |  | 50% | 0.13 | 0.12 | 0.11 | 0.79 | 0.28 | 0.17 | 4.77 | 0.92 | 0.30 |
|  |  |  | 95% | 0.92 | 0.69 | 0.31 | 2.59 | 0.95 | 0.39 | 7.27 | 6.46 | 1.32 |
|  |  |  | Mean | 0.25 | 0.17 | 0.14 | 1.01 | 0.41 | 0.20 | 4.37 | 2.02 | 0.49 |
|  |  | 𝜌 |  | 1.00 | 1.00 | 0.98 | 1.00 | 1.00 | 1.00 | 0.73 | 0.93 | 1.00 |
|  |  | BO |  | 0.18 | 0.098 | 0.089 | 0.44 | 0.26 | 0.21 | 0.74 | 0.52 | 0.42 |
|  |  | 𝜆 |  | 0.4 |  |  | 0.7 |  |  | 1.0 |  |  |
|  | Additional comments R1-150042: No 256QAM, no LDPC code, LAA max. duration =13ms, WiFi DL only, LAA on unlicensed carrier only. LBT scheme follows the Option B from ETSI EN 301 893 v1.8.0 |  |  |  |  |  |  |  |  |  |  |  |

| Samsung | 3 | UPT CDF [Mbps] | 5% | 16.1 | 29.6 | 93.6 | 7.3 | 25.5 | 81.0 | 4.1 | 41.0 | 67.7 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 50% | 47.9 | 72.6 | 118.9 | 30.8 | 63.8 | 108.4 | 12.3 | 55.0 | 97.9 |
|  |  |  | 95% | 64.8 | 84.7 | 141.0 | 48.1 | 77.4 | 131.4 | 27.7 | 68.4 | 121.8 |
|  |  |  | Mean | 48.7 | 72.3 | 118.7 | 30.8 | 64.0 | 107.5 | 13.8 | 55.0 | 96.9 |
|  |  | Delay CDF [s] | 5% | 0.034 | 0.034 | 0.022 | 0.038 | 0.034 | 0.022 | 0.093 | 0.034 | 0.022 |
|  |  |  | 50% | 0.103 | 0.059 | 0.035 | 0.184 | 0.070 | 0.040 | 0.462 | 0.084 | 0.044 |
|  |  |  | 95% | 0.782 | 0.315 | 0.085 | 1.782 | 0.421 | 0.105 | 2.692 | 0.276 | 0.127 |
|  |  |  | Mean | 0.142 | 0.072 | 0.041 | 0.286 | 0.088 | 0.048 | 0.788 | 0.110 | 0.055 |
|  |  | 𝜌 |  | 0.99 | 1.00 | 1.00 | 0.98 | 0.99 | 1.00 | 0.91 | 0.96 | 1.00 |
|  |  | BO |  | 0.28 | 0.17 | 0.11 | 0.46 | 0.23 | 0.14 | 0.70 | 0.29 | 0.18 |
|  |  | 𝜆 |  | 0.6 |  |  | 0.9 |  |  | 1.2 |  |  |
|  | Additional comments: LAA with licensed carrier, TXOP: 10ms(LAA), 4ms(WiFi), 2Tx2Rx, BCC, no 256QAM, CCA slot: 20us. For LBE, CCA (20us) is always followed by ECCA |  |  |  |  |  |  |  |  |  |  |  |
| 150812/ ETRI | 3 | UPT CDF [Mbps] | 5% | 7.45 | 9.33 | 13.62 | 1.93 | 4.0595 | 10.91 | 0.94 | 0.8 | 3.49 |
|  |  |  | 50% | 38.15 | 49.0 | 44.01 | 17.08 | 31.95 | 35.13 | 10.43 | 10.63 | 22.05 |
|  |  |  | 95% | 68.71 | 68.97 | 75.32 | 64.91 | 68.79 | 71.49 | 57.37 | 66.7 | 66.45 |
|  |  |  | Mean | 39.85 | 45.07 | 46.08 | 22.92 | 34.60 | 38.13 | 17.02 | 17.97 | 26.72 |
|  |  | Delay CDF [ms] | 5% | 60.24 | 60.03 | 52.60 | 63.71 | 60.18 | 54.09 | 71.98 | 61.46 | 62.06 |
|  |  |  | 50% | 108.39 | 84.50 | 89.36 | 242.44 | 129.61 | 111.99 | 396.95 | 381.35 | 187.07 |
|  |  |  | 95% | 555.88 | 443.84 | 290.76 | 2136.8 | 1020.0 | 361.49 | 4084.7 | 4877.1 | 1135.6 |
|  |  |  | Mean | 170.79 | 157.02 | 118.67 | 565.55 | 254.42 | 145.07 | 997.07 | 1216.2 | 321.38 |
|  |  | 𝜌 |  | 0.94 | 0.95 | 0.95 | 0.93 | 0.95 | 0.95 | 0.87 | 0.89 | 0.89 |
|  |  | BO |  | 17.9% | 16.3% | 15.7% | 40.13% | 22.0% | %19.0 | 59.1% | 60.2% | 36.2% |
|  |  | 𝜆 |  | 0.45 |  |  | 0.6 |  |  | 0.85 |  |  |
|  | Additional comments: TXOP=4ms - Indoor scenario - unlicensed band only - single channel - TXOP 4ms - DL only (for Wi-Fi: ACK modeled) - 10 UEs per operator - For Wi-Fi: both 256-QAM & LDPC are applied (11ac spec.) - For LAA: 256-QAM applied (Rel. 12 TBS table) - 1x2 antenna configuration (1 spatial stream) for both Wi-Fi and LAA - Round robin / auto-rate fallback algorithm applied for Wi-Fi - For LAA: HARQ processed only in licensed band |  |  |  |  |  |  |  |  |  |  |  |
| 150812/ ETRI | 2 | UPT CDF [Mbps] | 5% | 7.45 | 10.1 | 15.6 | 1.93 | 14.99 | 10.58 | 0.94 | 6.05 | 3.53 |
|  |  |  | 50% | 38.15 | 54.27 | 50.12 | 17.08 | 53.89 | 35.97 | 10.43 | 31.14 | 26.29 |
|  |  |  | 95% | 68.71 | 68.8 | 61.81 | 64.91 | 68.70 | 61.84 | 57.37 | 67.16 | 61.42 |
|  |  |  | Mean | 39.85 | 46.99 | 44.57 | 22.92 | 47.47 | 38.5 | 17.02 | 34.49 | 29.65 |
|  |  | Delay CDF [ms] | 5% | 60.24 | 60.16 | 64.07 | 63.71 | 66.95 | 71.14 | 71.98 | 61.64 | 64.47 |
|  |  |  | 50% | 108.39 | 75.81 | 78.75 | 242.44 | 84.56 | 122.33 | 396.95 | 132.94 | 150.65 |
|  |  |  | 95% | 555.88 | 404.33 | 247.43 | 2136.8 | 304.94 | 415.91 | 4084.7 | 684.57 | 1110.0 |
|  |  |  | Mean | 170.79 | 127.03 | 110.91 | 565.55 | 126.45 | 161.1 | 997.07 | 217.89 | 336.09 |
|  |  | 𝜌 |  | 0.95 | 1.0 | 0.91 | 0.91 | 0.93 | 0.89 | 0.87 | 0.85 | 0.84 |
|  |  | BO |  | 17% | 11% | 12% | 40.13% | 15.6% | 18.4% | 59.1% | 29.6% | 34.2% |
|  |  | 𝜆 |  | 0.45 |  |  | 0.6 |  |  | 0.85 |  |  |
|  | Additional comments: TXOP=4ms - Indoor scenario - unlicensed band only - single channel - DL only (for Wi-Fi: ACK modeled) - 10 UEs per operator - For Wi-Fi: both 256-QAM & LDPC are applied (11ac spec.) - For LAA: 256-QAM applied (Rel. 12 TBS table) - 1x2 antenna configuration (1 spatial stream) for both Wi-Fi and LAA - Round robin / auto-rate fallback algorithm applied for Wi-Fi - For LAA: HARQ processed only in licensed band |  |  |  |  |  |  |  |  |  |  |  |

| 150712 Cisco | 4 | UPT CDF [Mbps] | 5% | 58.94 | 62.14 | 60.75 | 11.26 | 5.55 | 5.78 | 0.08 | 0.12 | 0.05 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 50% | 96.91 | 101.40 | 71.37 | 39.53 | 42.31 | 26.85 | 0.51 | 1.69 | 0.19 |
|  |  |  | 95% | 118.17 | 123.49 | 78.14 | 69.08 | 78.14 | 40.77 | 16.33 | 20.25 | 2.96 |
|  |  |  | Mean | 88.79 | 92.09 | 71.10 | 41.19 | 43.80 | 26.49 | 2.50 | 4.91 | 0.78 |
|  |  | Delay CDF [s] | 5% | 0.046 | 0.041 | 0.069 | 0.103 | 0.082 | 0.155 | 1.080 | 0.669 | 100.69 |
|  |  |  | 50% | 0.070 | 0.068 | 0.082 | 0.237 | 0.198 | 0.458 | 351.43 | 201.17 | 591.72 |
|  |  |  | 95% | 0.124 | 11.557 | 9.499 | 19.430 | 150.47 | 14.931 | 736.43 | 657.89 | 763.71 |
|  |  |  | Mean | 0.581 | 2.523 | 0.983 | 7.255 | 18.401 | 17.194 | 356.42 | 255.92 | 532.57 |
|  |  | 𝜌 |  | 0.99 | 0.99 | 0.99 | 0.98 | 0.97 | 0.96 | 0.66 | 0.77 | 0.48 |
|  |  | BO |  | 14% | 13% | 16% | 40% | 37% | 50% | 87% | 83% | 85% |
|  |  | 𝜆 |  | 1 Hz |  |  | 1.5 Hz |  |  | 2.5 Hz |  |  |
|  | Additional comments: CDFs are distributions of the mean UPT or mean latency across all users in the simulation run. In cases where an FTP object enters a transceivers buffer to be sent but 0 bits are transferred in the simulation time the latency is recorded as 999s i.e. infinite for that object. DL buffer occupancy reported here is calculated based on the average buffer occupancy across all eNBs/APs for a given operator across all simulation drops and includes those with no users assigned to them and with no offered traffic. Scheme B(exp) with medium reservation and ED and PD with VCS on Figure 3, Figure 4, Figure 5, Figure 6. Results are for unlicensed carrier only. |  |  |  |  |  |  |  |  |  |  |  |
| 150712 Cisco | 4 | UPT CDF [Mbps] | 5% | 58.94 | 0.02 | 0.0004 | 11.26 | 0.01 | 0.0002 | 0.08 | 0.00 | 0.0001 |
|  |  |  | 50% | 96.91 | 15.92 | 7.63 | 39.53 | 3.57 | 2.57 | 0.51 | 0.18 | 0.07 |
|  |  |  | 95% | 118.17 | 129.72 | 70.34 | 69.08 | 88.22 | 27.32 | 16.33 | 37.22 | 12.57 |
|  |  |  | Mean | 88.79 | 40.33 | 26.03 | 41.19 | 20.36 | 7.82 | 2.50 | 4.74 | 2.22 |
|  |  | Delay CDF [s] | 5% | 0.046 | 0.038 | 0.077 | 0.103 | 0.075 | 0.207 | 1.080 | 0.173 | 0.524 |
|  |  |  | 50% | 0.070 | 0.902 | 2.787 | 0.237 | 25.929 | 89.848 | 351.43 | 680.00 | 720.26 |
|  |  |  | 95% | 0.124 | 913.60 | 989.45 | 19.430 | 943.74 | 992.43 | 736.43 | 971.79 | 995.34 |
|  |  |  | Mean | 0.581 | 238.23 | 296.66 | 7.255 | 328.06 | 363.63 | 356.42 | 567.35 | 603.97 |
|  |  | 𝜌 |  | 0.99 | 0.76 | 0.70 | 0.98 | 0.66 | 0.62 | 0.66 | 0.45 | 0.41 |
|  |  | BO |  | 14% | 51% | 57% | 40% | 66% | 72% | 87% | 84% | 82% |
|  |  | 𝜆 |  | 1 Hz |  |  | 1.5 Hz |  |  | 2.5 Hz |  |  |
|  | Additional comments: CDFs are distributions of the mean UPT or mean latency across all users in the simulation run. In cases where an FTP object enters a transceivers buffer to be sent but 0 bits are transferred in the simulation time the latency is recorded as 999s i.e. infinite for that object. DL buffer occupancy reported here is calculated based on the average buffer occupancy across all eNBs/APs for a given operator across all simulation drops and includes those with no users assigned to them and with no offered traffic Scheme B(exp) with ED and PD with VCS but no medium reservation on Figure6 and Figure 7. Results are for unlicensed carrier only. |  |  |  |  |  |  |  |  |  |  |  |
| 150712 Cisco | 4 | UPT CDF [Mbps] | 5% | 58.94 | 60.39 | 61.62 | 11.26 | 8.51 | 14.95 | 0.08 | 0.05 | 0.13 |
|  |  |  | 50% | 96.91 | 101.60 | 71.41 | 39.53 | 36.47 | 28.95 | 0.51 | 0.27 | 0.41 |
|  |  |  | 95% | 118.17 | 122.73 | 78.16 | 69.08 | 74.44 | 42.25 | 16.33 | 4.63 | 11.39 |
|  |  |  | Mean | 88.79 | 90.76 | 71.19 | 41.19 | 38.02 | 29.44 | 2.50 | 1.00 | 2.37 |
|  |  | Delay CDF [s] | 5% | 0.046 | 0.042 | 0.068 | 0.103 | 0.093 | 0.136 | 1.080 | 16.024 | 0.690 |
|  |  |  | 50% | 0.070 | 0.068 | 0.078 | 0.237 | 0.407 | 0.360 | 351.43 | 513.97 | 343.41 |
|  |  |  | 95% | 0.124 | 0.113 | 0.100 | 19.430 | 49.997 | 43.674 | 736.43 | 823.52 | 624.18 |
|  |  |  | Mean | 0.581 | 0.452 | 0.292 | 7.255 | 10.385 | 6.054 | 356.42 | 495.30 | 315.92 |
|  |  | 𝜌 |  | 0.99 | 0.99 | 0.99 | 0.98 | 0.97 | 0.97 | 0.66 | 0.53 | 0.70 |
|  |  | BO |  | 14% | 14% | 16% | 40% | 44% | 44% | 87% | 90% | 80% |
|  |  | 𝜆 |  | 1 Hz |  |  | 1.5 Hz |  |  | 2.5 Hz |  |  |
|  | Additional comments: CDFs are distributions of the mean UPT or mean latency across all users in the simulation run. In cases where an FTP object enters a transceivers buffer to be sent but 0 bits are transferred in the simulation time the latency is recorded as 999s i.e. infinite for that object. DL buffer occupancy reported here is calculated based on the average buffer occupancy across all eNBs/APs for a given operator across all simulation drops and includes those with no users assigned to them and with no offered traffic. Scheme B(fixed) with medium reservation and ED and PD with VCS on Figure 3 and Figure 4 Results are for unlicensed carrier only. |  |  |  |  |  |  |  |  |  |  |  |
| 150712 Cisco | 4 | UPT CDF [Mbps] | 5% | 58.94 | 51.93 | 93.13 | 11.26 | 5.45 | 67.75 | 0.08 | 0.01 | 21.66 |
|  |  |  | 50% | 96.91 | 81.87 | 99.34 | 39.53 | 26.66 | 82.71 | 0.51 | 0.03 | 39.16 |
|  |  |  | 95% | 118.17 | 109.15 | 105.46 | 69.08 | 50.36 | 88.69 | 16.33 | 0.17 | 45.69 |
|  |  |  | Mean | 88.79 | 77.49 | 99.23 | 41.19 | 28.16 | 81.33 | 2.50 | 0.06 | 36.91 |
|  |  | Delay CDF [s] | 5% | 0.046 | 0.061 | 0.042 | 0.103 | 0.185 | 0.058 | 1.080 | 534.00 | 0.157 |
|  |  |  | 50% | 0.070 | 0.091 | 0.046 | 0.237 | 0.523 | 0.065 | 351.43 | 833.61 | 0.250 |
|  |  |  | 95% | 0.124 | 0.152 | 0.052 | 19.430 | 80.738 | 50.133 | 736.43 | 947.96 | 65.088 |
|  |  |  | Mean | 0.581 | 0.477 | 0.250 | 7.255 | 18.477 | 5.939 | 356.42 | 818.17 | 8.625 |
|  |  | 𝜌 |  | 0.99 | 0.99 | 0.99 | 0.98 | 0.96 | 0.97 | 0.66 | 0.19 | 1.01 |
|  |  | BO |  | 14% | 18% | 10% | 40% | 53% | 20% | 87% | 92% | 55% |
|  |  | 𝜆 |  | 1 Hz |  |  | 1.5 Hz |  |  | 2.5 Hz |  |  |
|  | Additional comments: CDFs are distributions of the mean UPT or mean latency across all users in the simulation run. In cases where an FTP object enters a transceivers buffer to be sent but 0 bits are transferred in the simulation time the latency is recorded as 999s i.e. infinite for that object. DL buffer occupancy reported here is calculated based on the average buffer occupancy across all eNBs/APs for a given operator across all simulation drops and includes those with no users assigned to them and with no offered traffic. Scheme A(fixed) with medium reservation and ED and PD with VCS on Figure 3, Figure 4 and figure 8 Results are for unlicensed carrier only. |  |  |  |  |  |  |  |  |  |  |  |
| 150712 Cisco | 4 | UPT CDF [Mbps] | 5% | 58.94 | 44.62 | 85.67 | 11.26 | 0.34 | 57.15 | 0.08 | 0.01 | 14.75 |
|  |  |  | 50% | 96.91 | 68.43 | 98.26 | 39.53 | 26.29 | 78.47 | 0.51 | 0.05 | 22.09 |
|  |  |  | 95% | 118.17 | 108.30 | 105.05 | 69.08 | 49.47 | 86.74 | 16.33 | 0.37 | 28.20 |
|  |  |  | Mean | 88.79 | 76.43 | 97.51 | 41.19 | 26.51 | 76.13 | 2.50 | 0.10 | 21.84 |
|  |  | Delay CDF [s] | 5% | 0.046 | 0.061 | 0.043 | 0.103 | 0.189 | 0.062 | 1.080 | 505.49 | 0.519 |
|  |  |  | 50% | 0.070 | 0.093 | 0.047 | 0.237 | 0.500 | 0.079 | 351.43 | 805.68 | 8.160 |
|  |  |  | 95% | 0.124 | 1.882 | 0.486 | 19.430 | 95.498 | 45.514 | 736.43 | 922.57 | 73.315 |
|  |  |  | Mean | 0.581 | 0.598 | 0.292 | 7.255 | 27.016 | 4.710 | 356.42 | 785.07 | 19.728 |
|  |  | 𝜌 |  | 0.99 | 0.99 | 0.99 | 0.98 | 0.96 | 0.98 | 0.66 | 0.23 | 1.00 |
|  |  | BO |  | 14% | 18% | 11% | 40% | 55% | 23% | 87% | 92% | 71% |
|  |  | 𝜆 |  | 1 Hz |  |  | 1.5 Hz |  |  | 2.5 Hz |  |  |
|  | Additional comments: CDFs are distributions of the mean UPT or mean latency across all users in the simulation run. In cases where an FTP object enters a transceivers buffer to be sent but 0 bits are transferred in the simulation time the latency is recorded as 999s i.e. infinite for that object. DL buffer occupancy reported here is calculated based on the average buffer occupancy across all eNBs/APs for a given operator across all simulation drops and includes those with no users assigned to them and with no offered traffic. Scheme A(exp) i.e. V1.7.4 Option A, with medium reservation and ED and PD with VCS on figure 8. Results are for unlicensed carrier only. |  |  |  |  |  |  |  |  |  |  |  |
| 150712 Cisco | 4 | UPT CDF [Mbps] | 5% | 58.94 | 0.000000 | 0.000242 | 11.26 | 0.000000 | 0.000150 | 0.08 | 0.000000 | 0.000110 |
|  |  |  | 50% | 96.91 | 0.69 | 2.41 | 39.53 | 0.15 | 1.09 | 0.51 | 0.03 | 0.10 |
|  |  |  | 95% | 118.17 | 77.76 | 35.64 | 69.08 | 68.90 | 21.22 | 16.33 | 30.27 | 18.77 |
|  |  |  | Mean | 88.79 | 16.99 | 9.64 | 41.19 | 12.49 | 7.01 | 2.50 | 4.79 | 3.18 |
|  |  | Delay CDF [s] | 5% | 0.046 | 0.067 | 0.141 | 0.103 | 0.090 | 0.237 | 1.080 | 0.202 | 0.409 |
|  |  |  | 50% | 0.070 | 730.82 | 43.828 | 0.237 | 805.53 | 61.931 | 351.43 | 927.60 | 622.76 |
|  |  |  | 95% | 0.124 | 999.00 | 989.32 | 19.430 | 999.00 | 992.43 | 736.43 | 999.00 | 995.56 |
|  |  |  | Mean | 0.581 | 538.18 | 337.24 | 7.255 | 598.08 | 388.83 | 356.42 | 733.92 | 560.02 |
|  |  | 𝜌 |  | 0.99 | 0.46 | 0.66 | 0.98 | 0.40 | 0.60 | 0.66 | 0.28 | 0.45 |
|  |  | BO |  | 14% | 67% | 66% | 40% | 74% | 72% | 87% | 84% | 81% |
|  |  | 𝜆 |  | 1 Hz |  |  | 1.5 Hz |  |  | 2.5 Hz |  |  |
|  | Additional comments: CDFs are distributions of the mean UPT or mean latency across all users in the simulation run. In cases where an FTP object enters a transceivers buffer to be sent but 0 bits are transferred in the simulation time the latency is recorded as 999s i.e. infinite for that object. DL buffer occupancy reported here is calculated based on the average buffer occupancy across all eNBs/APs for a given operator across all simulation drops and includes those with no users assigned to them and with no offered traffic. Scheme B (exp) with ED only and no medium reservation Figure 7. Results are for unlicensed carrier only. |  |  |  |  |  |  |  |  |  |  |  |
| LG | 2 | UPT CDF [Mbps] | 5% | 26.13 | 32.32 | 33.00 | 1.518 | 19.72 | 18.94 | 0.324 | 10.53 | 5.375 |
|  |  |  | 50% | 32.91 | 44.28 | 43.34 | 16.41 | 36.84 | 33.14 | 6.063 | 31.17 | 24.74 |
|  |  |  | 95% | 41.51 | 50.40 | 55.42 | 32.30 | 46.47 | 49.32 | 26.42 | 43.68 | 45.41 |
|  |  |  | Mean | 33.35 | 43.28 | 43.70 | 16.18 | 35.55 | 33.38 | 8.767 | 29.72 | 25.36 |
|  |  | Delay CDF [s] | 5% | 0.125 | 0.092 | 0.088 | 0.173 | 0.105 | 0.109 | 0.213 | 0.112 | 0.126 |
|  |  |  | 50% | 0.188 | 0.116 | 0.141 | 0.521 | 0.157 | 0.229 | 1.409 | 0.199 | 0.370 |
|  |  |  | 95% | 0.307 | 0.218 | 0.247 | 15.48 | 0.426 | 0.812 | 46.21 | 0.940 | 3.837 |
|  |  |  | Mean | 0.199 | 0.131 | 0.158 | 2.758 | 0.243 | 0.363 | 9.200 | 0.448 | 1.637 |
|  |  | 𝜌 |  | 0.9995 | 0.9998 | 0.9985 | 0.9731 | 0.9989 | 0.9969 | 0.9044 | 0.9966 | 98.53 |
|  |  | BO |  | 0.1832 | 0.1333 | 0.1571 | 0.4463 | 0.2175 | 0.2855 | 0.6074 | 0.2975 | 0.4216 |
|  |  | 𝜆 |  | 0.5 |  |  | 0.65 |  |  | 0.75 |  |  |
|  | Additional comments: LAA without licensed carrier, no 256 QAM, LDPC, 1Tx2Rx, TXOP 4ms CCA threshold: -62dBm for CCA-ED of LAA LBT algorithm: FBE procedure as defined in ETSI EN 301 893 V1.8.0 |  |  |  |  |  |  |  |  |  |  |  |
| LG | 3 | UPT CDF [Mbps] | 5% | 26.13 | 25.57 | 34.21 | 1.518 | 8.303 | 13.91 | 0.324 | 0.721 | 2.570 |
|  |  |  | 50% | 32.91 | 35.94 | 45.26 | 16.41 | 24.27 | 33.82 | 6.063 | 14.25 | 24.72 |
|  |  |  | 95% | 41.51 | 44.91 | 56.30 | 32.30 | 37.80 | 50.66 | 26.42 | 33.21 | 47.22 |
|  |  |  | Mean | 33.35 | 35.75 | 45.02 | 16.18 | 23.97 | 33.60 | 8.767 | 15.50 | 25.06 |
|  |  | Delay CDF [s] | 5% | 0.125 | 0.114 | 0.084 | 0.173 | 0.147 | 0.097 | 0.213 | 0.174 | 0.106 |
|  |  |  | 50% | 0.188 | 0.168 | 0.121 | 0.521 | 0.308 | 0.183 | 1.409 | 0.587 | 0.269 |
|  |  |  | 95% | 0.307 | 0.313 | 0.210 | 15.48 | 15.36 | 0.746 | 46.21 | 22.78 | 5.783 |
|  |  |  | Mean | 0.199 | 0.226 | 0.148 | 2.758 | 0.863 | 0.602 | 9.200 | 3.469 | 1.674 |
|  |  | 𝜌 |  | 0.9995 | 0.9993 | 0.9971 | 0.9731 | 0.9948 | 0.9939 | 0.9044 | 0.9691 | 0.9815 |
|  |  | BO |  | 0.1832 | 0.1742 | 0.1460 | 0.4463 | 0.3357 | 0.2654 | 0.6074 | 0.4977 | 0.3853 |
|  |  | 𝜆 |  | 0.5 |  |  | 0.65 |  |  | 0.75 |  |  |
|  | Additional comments: LAA without licensed carrier, no 256 QAM, LDPC, 1Tx2Rx, TXOP 4ms CCA threshold: -62dBm for CCA-ED of LAA LBT algorithm: LBE procedure with a fixed q(=10) for the contention window as defined in ETSI EN 301 893 V1.8.0 |  |  |  |  |  |  |  |  |  |  |  |
| LG | 4 | UPT CDF [Mbps] | 5% | 26.13 | 31.96 | 33.10 | 1.518 | 19.46 | 16.72 | 0.324 | 10.50 | 3.501 |
|  |  |  | 50% | 32.91 | 44.65 | 43.22 | 16.41 | 36.58 | 32.33 | 6.063 | 30.00 | 23.75 |
|  |  |  | 95% | 41.51 | 50.72 | 54.52 | 32.30 | 46.86 | 49.04 | 26.42 | 44.01 | 44.20 |
|  |  |  | Mean | 33.35 | 43.51 | 43.25 | 16.18 | 35.55 | 32.34 | 8.767 | 29.07 | 24.08 |
|  |  | Delay CDF [s] | 5% | 0.125 | 0.091 | 0.094 | 0.173 | 0.104 | 0.117 | 0.213 | 0.112 | 0.139 |
|  |  |  | 50% | 0.188 | 0.116 | 0.153 | 0.521 | 0.163 | 0.260 | 1.409 | 0.217 | 0.407 |
|  |  |  | 95% | 0.307 | 0.215 | 0.277 | 15.48 | 0.425 | 1.066 | 46.21 | 1.085 | 7.525 |
|  |  |  | Mean | 0.199 | 0.158 | 0.180 | 2.758 | 0.305 | 0.507 | 9.200 | 0.565 | 2.249 |
|  |  | 𝜌 |  | 0.9995 | 0.9995 | 0.9989 | 0.9731 | 0.9984 | 0.9961 | 0.9044 | 0.9962 | 0.9812 |
|  |  | BO |  | 0.1832 | 0.1339 | 0.1647 | 0.4463 | 0.2199 | 0.3006 | 0.6074 | 0.3085 | 0.4365 |
|  |  | 𝜆 |  | 0.5 |  |  | 0.65 |  |  | 0.75 |  |  |
|  | Additional comments: LAA without licensed carrier, no 256 QAM, LDPC, 1Tx2Rx, TXOP 4ms CCA threshold: -62dBm for CCA-ED of LAA LBT algorithm: LBE procedure Op A with a variable q for the contention window as defined in ETSI EN 301 893 V1.8.0 |  |  |  |  |  |  |  |  |  |  |  |
| Intel | 3 | UPT CDF [Mbps] | 5% | 16 | 32 | 17 | 3 | 22 | 16 | 2 | 15 | 13 |
|  |  |  | 50% | 38 | 62 | 110 | 14 | 49 | 74 | 7 | 38 | 57 |
|  |  |  | 95% | 71 | 83 | 191 | 44 | 76 | 167 | 29 | 64 | 145 |
|  |  |  | Mean | 39 | 61 | 103 | 17 | 49 | 80 | 10 | 39 | 65 |
|  |  | Delay CDF [s] | 5% | 0.06 | 0.05 | 0.02 | 0.10 | 0.05 | 0.02 | 0.15 | 0.07 | 0.03 |
|  |  |  | 50% | 0.11 | 0.07 | 0.03 | 0.35 | 0.09 | 0.04 | 0.97 | 0.11 | 0.05 |
|  |  |  | 95% | 0.31 | 0.13 | 0.15 | 3.21 | 0.19 | 0.16 | 6.65 | 0.34 | 0.17 |
|  |  |  | Mean | 0.14 | 0.074 | 0.053 | 0.730 | 0.102 | 0.060 | 2.018 | 0.149 | 0.070 |
|  |  | 𝜌 |  | 1.00 | 1.00 | 0.99 | 1.00 | 0.97 | 0.97 | 1.00 | 0.85 | 1.00 |
|  |  | BO |  | 0.18 | 0.13 | 0.10 | 0.48 | 0.20 | 0.18 | 0.71 | 0.28 | 0.27 |
|  |  | 𝜆 |  | 0.625 |  |  | 0.83 |  |  | 1 |  |  |
|  | Additional comments: results with licensed carrier. 2Tx2Rx, TXOP 4ms, and LBT algorithm based on R1-1500089. |  |  |  |  |  |  |  |  |  |  |  |

Table 2: Indoor deployment for LAA and LAA coexistence case with one shared unlicensed carrier and FTP traffic

| Tdoc / Company | LAA LBT cat. | Reported parameters |  |  | Low load BO range: 10%~25% |  |  |  | Medium load BO range: 35%~50% |  |  |  | High load BO range: above 55% |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | LAA opt. 1 |  |  | LAA opt. 2 | LAA opt. 1 |  | LAA opt. 2 |  | LAA opt. 1 |  | LAA opt. 2 |  |
| Ericsson | 3 | UPT CDF [Mbps] |  | 5% | 38.64 |  |  | 39.18 | 15.31 |  | 12 |  | 5.02 |  | 5.26 |  |
|  |  |  |  | 50% | 69.87 |  |  | 71.06 | 44.7 |  | 41.97 |  | 24.56 |  | 26.32 |  |
|  |  |  |  | 95% | 90.84 |  |  | 91.78 | 72.88 |  | 69.55 |  | 54.8 |  | 56.95 |  |
|  |  |  |  | Mean | 69.72 |  |  | 70.97 | 46.4 |  | 43.52 |  | 28.31 |  | 30.28 |  |
|  |  | Delay CDF [s] |  | 5% | 0.042 |  |  | 0.041 | 0.054 |  | 0.061 |  | 0.074 |  | 0.074 |  |
|  |  |  |  | 50% | 0.084 |  |  | 0.083 | 0.174 |  | 0.204 |  | 0.371 |  | 0.336 |  |
|  |  |  |  | 95% | 0.282 |  |  | 0.298 | 0.868 |  | 0.769 |  | 1.384 |  | 1.229 |  |
|  |  |  |  | Mean | 0.123 |  |  | 0.126 | 0.307 |  | 0.31 |  | 0.564 |  | 0.511 |  |
|  |  | 𝜌 |  |  | 0.99 |  |  | 0.99 | 0.96 |  | 0.93 |  | 0.84 |  | 0.84 |  |
|  |  | BO |  |  | 0.20 |  |  | 0.20 | 0.40 |  | 0.43 |  | 0.60 |  | 0.59 |  |
|  |  | 𝜆 |  |  | 1.035 files/s |  |  |  | 1.408 files/s |  |  |  | 1.734 files/s |  |  |  |
|  | Additional comments: (*(): results with licensed carrier) 256 QAM, LDPC, 2Tx2Rx, TXOP 4ms, and LBT algorithm based on R1-150584 where the LAA CCA starts and resumes at the subframe boundaries. If LBT succeeds before the 4th OFDM symbol in a subframe the channel is occupied by the LAA node. If the LBT fails by the 4th OFDM symbol in a subframe, the random backoff counter is frozen and the CCA is postponed to the next subframe boundary where the CCA count down is resumed. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| InterDigital IDCC | LBE Cat.3 | UPT CDF [Mbps] |  | 5% | 8.268 |  |  | 7.784 | 3.523 |  | 3.646 |  | 0.784 |  | 0.359 |  |
|  |  |  |  | 50% | 85.201 |  |  | 117.848 | 25.292 |  | 29.233 |  | 6.236 |  | 7.540 |  |
|  |  |  |  | 95% | 143.311 |  |  | 144.111 | 133.660 |  | 143.996 |  | 62.660 |  | 118.122 |  |
|  |  |  |  | Mean | 85.437 |  |  | 94.781 | 46.853 |  | 53.357 |  | 19.575 |  | 23.775 |  |
|  |  | Delay CDF [s] |  | 5% | 0.030 |  |  | 0.030 | 0.062 |  | 0.030 |  | 0.051 |  | 0.035 |  |
|  |  |  |  | 50% | 0.048 |  |  | 0.034 | 0.371 |  | 0.134 |  | 0.755 |  | 0.511 |  |
|  |  |  |  | 95% | 0.456 |  |  | 0.513 | 2.703 |  | 1.275 |  | 3.273 |  | 4.295 |  |
|  |  |  |  | Mean | 0.123 |  |  | 0.114 | 0.348 |  | 0.343 |  | 1.116 |  | 1.171 |  |
|  |  | 𝜌 |  |  | 0.943 |  |  | 0.976 | 0.833 |  | 0.865 |  | 0.616 |  | 0.625 |  |
|  |  | BO |  |  | 14.025 |  |  | 12.741 | 36.115 |  | 35.731 |  | 73.571 |  | 68.601 |  |
|  |  | 𝜆 |  |  | 0.6 |  |  |  | 0.85 |  |  |  | 1.2 |  |  |  |
|  | Additional comments: TXOP=4ms, without licensed carrier LBE Cat.3: Option B from ETSI EN 301 893 v1.8.0, Section 4.8.3.2. CW size fixed and set by q at beginning of simulation. Random backoff N selected from [1;q]. TXOP = q*13/32 ms. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| InterDigital IDCC | LBE Cat.4 | UPT CDF [Mbps] |  | 5% | 4.207 |  |  | 4.913 | 2.552 |  | 1.870 |  | 1.211 |  | 0.544 |  |
|  |  |  |  | 50% | 71.831 |  |  | 63.933 | 18.169 |  | 19.868 |  | 13.363 |  | 14.119 |  |
|  |  |  |  | 95% | 143.424 |  |  | 144.169 | 127.892 |  | 142.018 |  | 96.657 |  | 141.571 |  |
|  |  |  |  | Mean | 75.270 |  |  | 77.735 | 33.944 |  | 40.008 |  | 25.934 |  | 33.887 |  |
|  |  | Delay CDF [s] |  | 5% | 0.030 |  |  | 0.030 | 0.031 |  | 0.030 |  | 0.041 |  | 0.030 |  |
|  |  |  |  | 50% | 0.058 |  |  | 0.065 | 0.260 |  | 0.244 |  | 0.346 |  | 0.352 |  |
|  |  |  |  | 95% | 1.100 |  |  | 0.834 | 1.634 |  | 1.711 |  | 2.493 |  | 3.141 |  |
|  |  |  |  | Mean | 0.207 |  |  | 0.178 | 0.437 |  | 0.488 |  | 0.662 |  | 0.763 |  |
|  |  | 𝜌 |  |  | 0.872 |  |  | 0.869 | 0.805 |  | 0.875 |  | 0.728 |  | 0.765 |  |
|  |  | BO |  |  | 21.808 |  |  | 22.616 | 50.036 |  | 49.488 |  | 62.141 |  | 58.163 |  |
|  |  | 𝜆 |  |  | 0.8 |  |  |  | 1.1 |  |  |  | 1.2 |  |  |  |
|  | Additional comments: TXOP=4ms, without licensed carrier LBE Cat.4: Modified Option A from ETSI EN 301 893 v1.8.0, Section 4.8.3.2. CW exponentially variable. Initially set to q=16 and doubled whenever N idle slots cannot be found within q observation slots. TXOP independent of q, fixed at beginning of simulation. 4ms and 13 ms (although technically Option A requires TXOP<10ms). |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| InterDigital IDCC | LBE Cat.3 | UPT CDF [Mbps] |  | 5% | 6.138 |  |  | 5.802 | 1.431 |  | 1.506 |  | 1.141 |  | 1.354 |  |
|  |  |  |  | 50% | 57.631 |  |  | 63.448 | 19.881 |  | 28.228 |  | 9.720 |  | 8.156 |  |
|  |  |  |  | 95% | 143.085 |  |  | 144.111 | 136.420 |  | 143.996 |  | 129.722 |  | 97.702 |  |
|  |  |  |  | Mean | 70.127 |  |  | 76.375 | 44.128 |  | 50.412 |  | 21.247 |  | 34.176 |  |
|  |  | Delay CDF [s] |  | 5% | 0.030 |  |  | 0.030 | 0.030 |  | 0.030 |  | 0.081 |  | 0.030 |  |
|  |  |  |  | 50% | 0.077 |  |  | 0.066 | 0.341 |  | 0.187 |  | 1.042 |  | 0.345 |  |
|  |  |  |  | 95% | 1.118 |  |  | 0.932 | 2.270 |  | 1.837 |  | 5.766 |  | 3.024 |  |
|  |  |  |  | Mean | 0.242 |  |  | 0.212 | 0.591 |  | 0.498 |  | 0.881 |  | 0.722 |  |
|  |  | 𝜌 |  |  | 0.855 |  |  | 0.870 | 0.722 |  | 0.885 |  | 0.724 |  | 0.779 |  |
|  |  | BO |  |  | 23.809 |  |  | 21.588 | 49.705 |  | 43.498 |  | 69.681 |  | 57.843 |  |
|  |  | 𝜆 |  |  | 0.8 |  |  |  | 0.95 |  |  |  | 1.2 |  |  |  |
|  | Additional comments: TXOP=13ms, without licensed carrier LBE Cat.3: Option B from ETSI EN 301 893 v1.8.0, Section 4.8.3.2. CW size fixed and set by q at beginning of simulation. Random backoff N selected from [1;q]. TXOP = q*13/32 ms. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| InterDigital IDCC | LBE Cat.4 | UPT CDF [Mbps] |  | 5% | 5.509 |  |  | 3.863 | 3.010 |  | 2.226 |  | 1.395 |  | 0.949 |  |
|  |  |  |  | 50% | 70.931 |  |  | 76.687 | 22.679 |  | 24.838 |  | 10.177 |  | 11.862 |  |
|  |  |  |  | 95% | 143.481 |  |  | 144.111 | 134.471 |  | 142.690 |  | 96.962 |  | 141.536 |  |
|  |  |  |  | Mean | 75.201 |  |  | 81.381 | 38.002 |  | 43.516 |  | 23.526 |  | 30.368 |  |
|  |  | Delay CDF [s] |  | 5% | 0.030 |  |  | 0.030 | 0.031 |  | 0.031 |  | 0.031 |  | 0.030 |  |
|  |  |  |  | 50% | 0.060 |  |  | 0.050 | 0.175 |  | 0.170 |  | 0.429 |  | 0.337 |  |
|  |  |  |  | 95% | 0.784 |  |  | 1.061 | 1.342 |  | 1.467 |  | 2.407 |  | 3.296 |  |
|  |  |  |  | Mean | 0.173 |  |  | 0.208 | 0.351 |  | 0.360 |  | 0.716 |  | 0.839 |  |
|  |  | 𝜌 |  |  | 0.868 |  |  | 0.869 | 0.837 |  | 0.917 |  | 0.738 |  | 0.750 |  |
|  |  | BO |  |  | 19.710 |  |  | 22.926 | 44.692 |  | 43.932 |  | 65.391 |  | 60.535 |  |
|  |  | 𝜆 |  |  | 0.8 |  |  |  | 1.1 |  |  |  | 1.2 |  |  |  |
|  | Additional comments: TXOP=13ms, without licensed carrier LBE Cat.4: Modified Option A from ETSI EN 301 893 v1.8.0, Section 4.8.3.2. CW exponentially variable. Initially set to q=16 and doubled whenever N idle slots cannot be found within q observation slots. TXOP independent of q, fixed at beginning of simulation. 4ms and 13 ms (although technically Option A requires TXOP<10ms). |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Nokia Corporation / Nokia Networks | 2 | UPT CDF [Mbps] |  | 5% | 11.87 |  |  | 12.54 | 2.24 |  | 2.22 |  | 0.97 |  | 1.03 |  |
|  |  |  |  | 50% | 53.16 |  |  | 55.04 | 17.60 |  | 18.32 |  | 8.79 |  | 9.51 |  |
|  |  |  |  | 95% | 71.42 |  |  | 71.43 | 71.00 |  | 71.00 |  | 70.73 |  | 70.68 |  |
|  |  |  |  | Mean | 49.20 |  |  | 50.11 | 27.02 |  | 26.98 |  | 19.58 |  | 20.29 |  |
|  |  | Delay CDF [s] |  | 5% | 0.056 |  |  | 0.056 | 0.056 |  | 0.056 |  | 0.056 |  | 0.056 |  |
|  |  |  |  | 50% | 0.075 |  |  | 0.073 | 0.225 |  | 0.215 |  | 0.451 |  | 0.418 |  |
|  |  |  |  | 95% | 0.325 |  |  | 0.304 | 1.775 |  | 1.794 |  | 4.071 |  | 3.826 |  |
|  |  |  |  | Mean | 0.118 |  |  | 0.113 | 0.464 |  | 0.470 |  | 1.036 |  | 0.958 |  |
|  |  | 𝜌 |  |  | 0.987 |  |  | 0.984 | 0.967 |  | 0.967 |  | 0.92 |  | 0.906 |  |
|  |  | BO |  |  | 13.56 |  |  | 13.06 | 45.29 |  | 44.30 |  | 62.53 |  | 60.90 |  |
|  |  | 𝜆 |  |  | 0.55 |  |  |  | 1 |  |  |  | 1.2 |  |  |  |
|  | Additional comments: LBT algorithm according to FBE operation as described in R1-150499. Other details: 1Tx2Rx, TXOP 1ms, BCC, no 256QAM. No user data (i.e. PDSCH) is transmitted on the licensed band carrier in case a UE is configured with LAA. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ALU, ASB | LBE Cat 3 | UPT CDF [Mbps] |  | 5% | 43.62 |  |  | 43.21 | 4.84 |  | 2.96 |  | 1.33 |  | 0.82 |  |
|  |  |  |  | 50% | 73.71 |  |  | 74.33 | 48.64 |  | 48.74 |  | 21.46 |  | 26.15 |  |
|  |  |  |  | 95% | 95.31 |  |  | 94.69 | 76.55 |  | 78.66 |  | 64.39 |  | 69.55 |  |
|  |  |  |  | Mean | 72.28 |  |  | 71.80 | 45.92 |  | 45.41 |  | 27.13 |  | 29.74 |  |
|  |  | Delay CDF [s] |  | 5% | 0.036 |  |  | 0.036 | 0.036 |  | 0.036 |  | 0.036 |  | 0.036 |  |
|  |  |  |  | 50% | 0.055 |  |  | 0.055 | 0.097 |  | 0.092 |  | 0.192 |  | 0.170 |  |
|  |  |  |  | 95% | 0.190 |  |  | 0.202 | 1.361 |  | 1.120 |  | 2.532 |  | 2.722 |  |
|  |  |  |  | Mean | 0.078 |  |  | 0.079 | 0.302 |  | 0.291 |  | 0.595 |  | 0.577 |  |
|  |  | 𝜌 |  |  | 0.88 |  |  | 0.83 | 0.72 |  | 0.74 |  | 0.59 |  | 0.60 |  |
|  |  | BO |  |  | 0.12 |  |  | 0.12 | 0.37 |  | 0.35 |  | 0.61 |  | 0.57 |  |
|  |  | 𝜆 |  |  | 0.7 |  |  |  | 0.9 |  |  |  | 1.0 |  |  |  |
|  | Additional comments: LBE, q Value:32, Maximum Occupancy Time : 10ms, LAA without licensed carrier 2Tx2Rx, BCC, no 256QAM CCA threshold: -73 dBm/MHz + 23 - PH, PH specified in dBm EIRP for CCA-ED of LAA, -62 dBm for CCA-ED of Wi-Fi; |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ALU, ASB | FBE Cat 2 | UPT CDF [Mbps] |  | 5% | 51.79 |  |  | 51.61 | 7.04 |  | 7.04 |  | 2.24 |  | 2.49 |  |
|  |  |  |  | 50% | 74.03 |  |  | 73.40 | 49.77 |  | 45.84 |  | 15.48 |  | 18.09 |  |
|  |  |  |  | 95% | 92.40 |  |  | 92.43 | 74.49 |  | 77.36 |  | 53.43 |  | 60.86 |  |
|  |  |  |  | Mean | 73.33 |  |  | 73.22 | 46.50 |  | 45.83 |  | 20.66 |  | 23.20 |  |
|  |  | Delay CDF [s] |  | 5% | 0.037 |  |  | 0.037 | 0.037 |  | 0.037 |  | 0.040 |  | 0.037 |  |
|  |  |  |  | 50% | 0.049 |  |  | 0.054 | 0.090 |  | 0.096 |  | 0.428 |  | 0.356 |  |
|  |  |  |  | 95% | 0.186 |  |  | 0.183 | 1.198 |  | 1.582 |  | 5.000 |  | 4.007 |  |
|  |  |  |  | Mean | 0.076 |  |  | 0.075 | 0.278 |  | 0.328 |  | 1.101 |  | 0.994 |  |
|  |  | 𝜌 |  |  | 0.84 |  |  | 0.79 | 0.69 |  | 0.71 |  | 0.48 |  | 0.47 |  |
|  |  | BO |  |  | 0.14 |  |  | 0.14 | 0.42 |  | 0.43 |  | 0.83 |  | 0.83 |  |
|  |  | 𝜆 |  |  | 0.85 |  |  |  | 1.1 |  |  |  | 1.5 |  |  |  |
|  | Additional comments: FBE frame period: 10ms(Maximum Occupancy Time= 9.5ms, idle time = 0.5ms), LAA with licensed carrier 2Tx2Rx, BCC, no 256QAM CCA threshold: -73 dBm/MHz + 23 - PH, PH specified in dBm EIRP for CCA-ED of LAA, -62 dBm for CCA-ED of Wi-Fi; |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Kyocera | 2 | UPT CDF [Mbps] |  | 5% | 8.50 |  |  | 12.85 | 1.82 |  | 2.66 |  | 1.59 |  | 1.62 |  |
|  |  |  |  | 50% | 31.75 |  |  | 33.17 | 6.14 |  | 8.18 |  | 4.21 |  | 4.98 |  |
|  |  |  |  | 95% | 57.14 |  |  | 57.14 | 20.12 |  | 24.35 |  | 14.56 |  | 17.35 |  |
|  |  |  |  | Mean | 33.52 |  |  | 34.19 | 7.90 |  | 9.97 |  | 5.74 |  | 6.80 |  |
|  |  | Delay CDF [s] |  | 5% | 0.07 |  |  | 0.07 | 0.19 |  | 0.18 |  | 0.27 |  | 0.24 |  |
|  |  |  |  | 50% | 0.12 |  |  | 0.12 | 0.72 |  | 0.51 |  | 1.21 |  | 1.01 |  |
|  |  |  |  | 95% | 0.46 |  |  | 0.33 | 2.74 |  | 2.11 |  | 3.11 |  | 3.77 |  |
|  |  |  |  | Mean | 0.18 |  |  | 0.16 | 0.94 |  | 0.76 |  | 1.40 |  | 1.31 |  |
|  |  | 𝜌 |  |  | 0.91 |  |  | 0.92 | 0.77 |  | 0.82 |  | 0.77 |  | 0.83 |  |
|  |  | BO |  |  | 0.10 |  |  | 0.10 | 0.49 |  | 0.50 |  | 0.63 |  | 0.63 |  |
|  |  | 𝜆 |  |  | 0.3 |  |  |  | 0.60 |  |  |  | 0.70 |  |  |  |
|  | Additional comments: Unlicensed carrier only, 1Tx2Rx, no 256 QAM, BCC, no RTS/CTS LBT: Based on FBE 1) a fixed length LBT before the transmission burst 2) at least 1 subframe IDLE period after the transmission burst. 3) the maximum burst length is assumed to be 4msec. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ZTE | 3 | UPT CDF [Mbps] |  | 5% | 45 |  |  | 41 | 14 |  | 13 |  | 1 |  | 4 |  |
|  |  |  |  | 50% | 65 |  |  | 56 | 31 |  | 30 |  | 14 |  | 7 |  |
|  |  |  |  | 95% | 62 |  |  | 67 | 52 |  | 52 |  | 22 |  | 18 |  |
|  |  |  |  | Mean | 62 |  |  | 58 | 36 |  | 35 |  | 12 |  | 11 |  |
|  |  | Delay CDF [s] |  | 5% | 0.032 |  |  | 0.032 | 0.032 |  | 0.032 |  | 0.072 |  | 0.107 |  |
|  |  |  |  | 50% | 0.079 |  |  | 0.09 | 0.254 |  | 0.195 |  | 0.455 |  | 0.548 |  |
|  |  |  |  | 95% | 0.475 |  |  | 0.509 | 2.35 |  | 0.843 |  | 7.01 |  | 3.27 |  |
|  |  |  |  | Mean | 0.146 |  |  | 0.157 | 0.58 |  | 0.279 |  | 1.775 |  | 0.928 |  |
|  |  | 𝜌 |  |  | 100% |  |  | 99% | 99.70% |  | 99.40% |  |  |  | 97% |  |
|  |  | BO |  |  | 17% |  |  | 19% | 39% |  | 35% |  |  |  | 64% |  |
|  |  | 𝜆 |  |  | 2.3 |  |  |  | 3 |  |  |  |  |  |  |  |
|  | Additional comments: No 256QAM, no LDPC code, 2Tx2Rx, 24 dBm TX power for LAA, no licensed carrier for LAA, LAA max. duration =4 ms |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Coolpad | 2 | UPT CDF [Mbps] |  | 5% | 3.18 |  |  | 1.52 | 1.37 |  | 0.82 |  | 0.22 |  | 0.20 |  |
|  |  |  |  | 50% | 27.63 |  |  | 25.83 | 21.06 |  | 20.12 |  | 1.52 |  | 16.13 |  |
|  |  |  |  | 95% | 50.63 |  |  | 50.63 | 48.78 |  | 50.53 |  | 50.63 |  | 50.63 |  |
|  |  |  |  | Mean | 31.61 |  |  | 27.74 | 22.64 |  | 24.15 |  | 17.43 |  | 21.05 |  |
|  |  | Delay CDF [s] |  | 5% | 0.09 |  |  | 0.08 | 0.08 |  | 0.08 |  | 0.09 |  | 0.09 |  |
|  |  |  |  | 50% | 0.09 |  |  | 0.08 | 0.08 |  | 0.20 |  | 0.23 |  | 1.32 |  |
|  |  |  |  | 95% | 5.60 |  |  | 3.31 | 2.83 |  | 2.60 |  | 2.73 |  | 2.37 |  |
|  |  |  |  | Mean | 1.17 |  |  | 0.49 | 0.56 |  | 0.76 |  | 0.74 |  | 0.75 |  |
|  |  | 𝜌 |  |  | 0.83 |  |  | 0.85 | 0.72 |  | 0.75 |  | 0.48 |  | 0.44 |  |
|  |  | BO |  |  | 8% |  |  | 12.43% | 24.67% |  | 36.07% |  | 73.76% |  | 65.30% |  |
|  |  | 𝜆 |  |  | 0.10 |  |  |  | 0.20 |  |  |  | 0.50 |  |  |  |
|  | Additional comments: No 256QAM, 2Tx2Rx, unlicensed band only, TXOP 4ms, idle time 0.2ms. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Fujitsu | 3 | UPT CDF [Mbps] | 5% |  |  | 44.370 | 48.195 |  |  | 28.572 |  | 34.019 |  | 24.822 |  | 27.773 |
|  |  |  | 50% |  |  | 64.782 | 63.902 |  |  | 49.332 |  | 50.653 |  | 44.050 |  | 45.675 |
|  |  |  | 95% |  |  | 85.048 | 87.891 |  |  | 61.684 |  | 65.511 |  | 58.562 |  | 60.840 |
|  |  |  | Mean |  |  | 64.786 | 64.740 |  |  | 48.191 |  | 50.163 |  | 43.678 |  | 45.409 |
|  |  | Delay CDF [s] | 5% |  |  | 0.029 | 0.029 |  |  | 0.029 |  | 0.029 |  | 0.029 |  | 0.029 |
|  |  |  | 50% |  |  | 0.057 | 0.057 |  |  | 0.073 |  | 0.073 |  | 0.083 |  | 0.081 |
|  |  |  | 95% |  |  | 0.134 | 0.134 |  |  | 0.196 |  | 0.188 |  | 0.224 |  | 0.227 |
|  |  |  | Mean |  |  | 0.065 | 0.066 |  |  | 0.087 |  | 0.084 |  | 0.098 |  | 0.098 |
|  |  | 𝜌 |  |  |  | 0.996 | 0.997 |  |  | 0.995 |  | 0.996 |  | 0.991 |  | 0.993 |
|  |  | BO |  |  |  | 10.057 | 10.405 |  |  | 17.286 |  | 16.971 |  | 21.206 |  | 21.561 |
|  |  | 𝜆 |  |  |  | 0.7 |  |  |  | 0.95 |  |  |  | 1.1 |  |  |
|  | Additional comments: backoff method 2 in R1-150185, FTP model 1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Huawei, HiSilicon, | Cat.3 | UPT CDF [Mbps] |  | 5% | 19.31 |  |  | 19.31 | 2.09 |  | 2.09 |  | 0.024 |  | 0.024 |  |
|  |  |  |  | 50% | 31.48 |  |  | 31.48 | 16.05 |  | 16.05 |  | 7.94 |  | 7.94 |  |
|  |  |  |  | 95% | 45.12 |  |  | 45.12 | 36.27 |  | 36.27 |  | 27.73 |  | 27.73 |  |
|  |  |  |  | Mean | 31.61 |  |  | 31.61 | 17.40 |  | 17.40 |  | 10.03 |  | 10.03 |  |
|  |  | Delay CDF [s] |  | 5% | 0.088 |  |  | 0.088 | 0.11 |  | 0.11 |  | 0.14 |  | 0.14 |  |
|  |  |  |  | 50% | 0.13 |  |  | 0.13 | 0.25 |  | 0.25 |  | 0.49 |  | 0.49 |  |
|  |  |  |  | 95% | 0.20 |  |  | 0.20 | 1.71 |  | 1.71 |  | 7.54 |  | 7.54 |  |
|  |  |  |  | Mean | 0.14 |  |  | 0.14 | 0.54 |  | 0.54 |  | 1.64 |  | 1.64 |  |
|  |  | 𝜌 |  |  | 1.00 |  |  | 1.00 | 0.98 |  | 0.98 |  | 0.89 |  | 0.89 |  |
|  |  | BO |  |  | 0.16 |  |  | 0.16 | 0.38 |  | 0.38 |  | 0.59 |  | 0.59 |  |
|  |  | 𝜆 |  |  | 0.6 |  |  |  | 1.0 |  |  |  | 1.4 |  |  |  |
|  | Additional comments: R1-150043, No 256QAM, LAA max. duration =13ms, LAA on unlicensed carrier only. LBT scheme follows the Option B from ETSI EN 301 893 v1.8.0. The performance metrics is not independently calculated per operator in LAA and LAA coexistence case. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

| Samsung | 3 | UPT CDF [Mbps] | 5% | 62.3 | 62.5 | 23.8 | 23.2 | 3.8 | 4.5 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 50% | 84.6 | 85.4 | 54.3 | 54.0 | 19.6 | 18.2 |
|  |  |  | 95% | 110.9 | 112.0 | 86.2 | 82.6 | 52.2 | 51.7 |
|  |  |  | Mean | 85.6 | 85.7 | 54.6 | 53.9 | 22.3 | 21.9 |
|  |  | Delay CDF [s] | 5% | 0.025 | 0.025 | 0.028 | 0.028 | 0.053 | 0.055 |
|  |  |  | 50% | 0.050 | 0.051 | 0.093 | 0.097 | 0.277 | 0.292 |
|  |  |  | 95% | 0.169 | 0.176 | 0.499 | 0.553 | 2.853 | 3.089 |
|  |  |  | Mean | 0.069 | 0.069 | 0.163 | 0.174 | 0.683 | 0.732 |
|  |  | 𝜌 |  | 1.00 | 1.00 | 0.99 | 0.99 | 0.95 | 0.95 |
|  |  | BO |  | 0.20 | 0.20 | 0.40 | 0.40 | 0.71 | 0.72 |
|  |  | 𝜆 |  | 0.7 |  | 1.0 |  | 1.5 |  |
|  | Additional comments: LAA with licensed carrier, TXOP: 10ms(LAA), 4ms(WiFi), 2Tx2Rx, BCC, no 256QAM, CCA slot: 20us. For LBE, CCA (20us) is always followed by ECCA |  |  |  |  |  |  |  |  |
| 150812/ ETRI | 3 | UPT CDF [Mbps] | 5% | 9.16 | 7.07 | 4.04 | 4.55 | 1.20 | 1.65 |
|  |  |  | 50% | 36.75 | 34.21 | 21.93 | 22.50 | 6.47 | 8.57 |
|  |  |  | 95% | 75.12 | 75.21 | 70.49 | 70.82 | 30.20 | 34.21 |
|  |  |  | Mean | 39.95 | 37.75 | 27.40 | 28.87 | 10.31 | 12.05 |
|  |  | Delay CDF [ms] | 5% | 54.91 | 52.63 | 58.46 | 55.91 | 135.54 | 115.70 |
|  |  |  | 50% | 112.26 | 115.73 | 188.12 | 173.56 | 637.25 | 461.79 |
|  |  |  | 95% | 450.72 | 521.86 | 1012.3 | 808.39 | 3426.3 | 2366.5 |
|  |  |  | Mean | 163.56 | 179.96 | 310.08 | 272.46 | 1055.3 | 783.61 |
|  |  | 𝜌 |  | 0.99 | 0.98 | 0.95 | 0.95 | 0.9 | 0.88 |
|  |  | BO |  | 13% | 15% | 39% | 37% | 63% | 65% |
|  |  | 𝜆 |  | 0.45 |  | 0.6 |  | 0.85 |  |
|  | Additional comments: TXOP=4ms - Indoor scenario - unlicensed band only - single channel - DL only - 10 UEs per operator - For LAA: 256-QAM applied (Rel. 12 TBS table) - 1x2 antenna configuration (1 spatial stream) For LAA: HARQ processed only in licensed band |  |  |  |  |  |  |  |  |
| 150812/ ETRI | 2 | UPT CDF [Mbps] | 5% | 7.29 | 5.11 | 5.03 | 4.53 | 1.58 | 1.29 |
|  |  |  | 50% | 33.53 | 31.33 | 30.50 | 19.54 | 11.34 | 16.84 |
|  |  |  | 95% | 61.71 | 61.67 | 61.94 | 59.13 | 64.09 | 61.59 |
|  |  |  | Mean | 34.99 | 33.77 | 34.43 | 25.98 | 20.07 | 26.18 |
|  |  | Delay CDF [ms] | 5% | 64.14 | 64.17 | 63.79 | 66.97 | 61.5 | 64.27 |
|  |  |  | 50% | 115.92 | 124.77 | 129.80 | 200.18 | 342.49 | 223.7 |
|  |  |  | 95% | 538.09 | 746.18 | 753.25 | 858.6 | 2437.8 | 2917.0 |
|  |  |  | Mean | 194.41 | 216.99 | 227.68 | 290.16 | 680.43 | 611.83 |
|  |  | 𝜌 |  | 0.98 | 0.95 | 0.9 | 0.92 | 0.85 | 0.83 |
|  |  | BO |  | 20.6% | 24.8% | 30.83% | 41.71 | 63.67% | 60.67% |
|  |  | 𝜆 |  | 0.45 |  | 0.6 |  | 0.85 |  |
|  | Additional comments: TXOP=4ms - Indoor scenario - unlicensed band only - single channel - DL only - 10 UEs per operator - For LAA: 256-QAM applied (Rel. 12 TBS table) - 1x2 antenna configuration (1 spatial stream) For LAA: HARQ processed only in licensed band |  |  |  |  |  |  |  |  |
| LG | 2 | UPT CDF [Mbps] | 5% | 39.54 | 39.34 | 9.446 | 8.782 | 1.710 | 1.938 |
|  |  |  | 50% | 49.58 | 48.89 | 29.51 | 28.81 | 15.99 | 15.44 |
|  |  |  | 95% | 58.47 | 58.00 | 45.96 | 46.20 | 38.90 | 36.57 |
|  |  |  | Mean | 49.28 | 48.77 | 29.10 | 28.40 | 17.60 | 16.90 |
|  |  | Delay CDF [s] | 5% | 0.081 | 0.082 | 0.132 | 0.135 | 0.218 | 0.234 |
|  |  |  | 50% | 0.107 | 0.109 | 0.283 | 0.283 | 1.181 | 1.283 |
|  |  |  | 95% | 0.158 | 0.160 | 2.131 | 3.319 | 15.71 | 17.08 |
|  |  |  | Mean | 0.112 | 0.114 | 0.688 | 0.768 | 3.655 | 4.078 |
|  |  | 𝜌 |  | 0.9996 | 0.9987 | 0.9950 | 0.9942 | 0.9602 | 0.9565 |
|  |  | BO |  | 0.1423 | 0.1462 | 0.4220 | 0.4325 | 0.6522 | 0.6631 |
|  |  | 𝜆 |  | 0.6 |  | 1.0 |  | 1.2 |  |
|  | Additional comments: LAA without licensed carrier, no 256 QAM, 1Tx2Rx, TXOP 4ms CCA threshold: -62dBm for CCA-ED of LAA LBT algorithm: FBE procedure as defined in ETSI EN 301 893 V1.8.0 |  |  |  |  |  |  |  |  |
| LG | 3 | UPT CDF [Mbps] | 5% | 29.69 | 30.72 | 1.566 | 1.337 | 0.131 | 0.161 |
|  |  |  | 50% | 40.66 | 41.60 | 24.67 | 25.67 | 10.19 | 11.95 |
|  |  |  | 95% | 54.27 | 54.17 | 47.76 | 47.89 | 41.63 | 41.79 |
|  |  |  | Mean | 40.89 | 41.61 | 24.35 | 25.40 | 13.88 | 15.28 |
|  |  | Delay CDF [s] | 5% | 0.092 | 0.091 | 0.111 | 0.111 | 0.130 | 0.135 |
|  |  |  | 50% | 0.149 | 0.141 | 0.317 | 0.281 | 0.852 | 0.630 |
|  |  |  | 95% | 0.292 | 0.288 | 10.76 | 11.16 | 59.43 | 53.02 |
|  |  |  | Mean | 0.199 | 0.182 | 2.473 | 2.070 | 10.30 | 8.848 |
|  |  | 𝜌 |  | 0.9976 | 0.9954 | 0.9720 | 0.9758 | 0.8905 | 0.9033 |
|  |  | BO |  | 0.1920 | 0.1906 | 0.4277 | 0.4091 | 0.6272 | 0.5992 |
|  |  | 𝜆 |  | 0.6 |  | 0.8 |  | 1.0 |  |
|  | Additional comments: LAA without licensed carrier, no 256 QAM, 1Tx2Rx, TXOP 4ms CCA threshold: -62dBm for CCA-ED of LAA LBT algorithm: LBE procedure with a fixed q(=10) for the contention window as defined in ETSI EN 301 893 V1.8.0 |  |  |  |  |  |  |  |  |
| LG | 4 | UPT CDF [Mbps] | 5% | 38.19 | 38.62 | 7.781 | 8.000 | 0.305 | 0.327 |
|  |  |  | 50% | 46.13 | 46.80 | 27.19 | 28.79 | 14.70 | 16.63 |
|  |  |  | 95% | 56.91 | 56.84 | 47.15 | 46.82 | 42.26 | 41.96 |
|  |  |  | Mean | 46.66 | 47.14 | 27.57 | 28.52 | 17.03 | 18.08 |
|  |  | Delay CDF [s] | 5% | 0.087 | 0.087 | 0.121 | 0.121 | 0.144 | 0.147 |
|  |  |  | 50% | 0.128 | 0.123 | 0.294 | 0.270 | 0.622 | 0.524 |
|  |  |  | 95% | 0.185 | 0.185 | 1.969 | 1.943 | 38.51 | 33.37 |
|  |  |  | Mean | 0.131 | 0.128 | 0.729 | 0.674 | 6.747 | 5.112 |
|  |  | 𝜌 |  | 0.9988 | 0.9986 | 0.9957 | 0.9953 | 0.9346 | 0.9509 |
|  |  | BO |  | 0.1601 | 0.1581 | 0.3961 | 0.3877 | 0.5846 | 0.5671 |
|  |  | 𝜆 |  | 0.6 |  | 0.9 |  | 1.1 |  |
|  | Additional comments: LAA without licensed carrier, no 256 QAM, 1Tx2Rx, TXOP 4ms CCA threshold: -62dBm for CCA-ED of LAA LBT algorithm: LBE procedure Op A with a variable q for the contention window as defined in ETSI EN 301 893 V1.8.0 |  |  |  |  |  |  |  |  |
| Intel | 3 | UPT CDF [Mbps] | 5% | 21 | 16 | 23 | 21 | 15 | 16 |
|  |  |  | 50% | 149 | 162 | 82 | 74 | 35 | 33 |
|  |  |  | 95% | 198 | 209 | 168 | 166 | 63 | 68 |
|  |  |  | Mean | 122 | 134 | 88 | 82 | 37 | 36 |
|  |  | Delay CDF [s] | 5% | 0.02 | 0.02 | 0.02 | 0.02 | 0.06 | 0.06 |
|  |  |  | 50% | 0.03 | 0.02 | 0.04 | 0.04 | 0.12 | 0.12 |
|  |  |  | 95% | 0.13 | 0.14 | 0.09 | 0.10 | 0.31 | 0.40 |
|  |  |  | Mean | 0.04 | 0.04 | 0.05 | 0.05 | 0.14 | 0.16 |
|  |  | 𝜌 |  | 1 | 1 | 1 | 1 | 0.99 | 0.99 |
|  |  | BO |  | 0.15 | 0.16 | 0.35 | 0.36 | 0.60 | 0.61 |
|  |  | 𝜆 |  | 0.7 |  | 1.1 |  | 2.63 |  |
|  | Additional comments: results with licensed carrier. 2Tx2Rx, TXOP 4ms, and LBT algorithm based on R1-1500089. |  |  |  |  |  |  |  |  |

Table 3: Indoor deployment for Wi-Fi and LAA coexistence case with one shared unlicensed carrier and Mixed traffic

| Tdoc / Company | LAA LBT cat. | Reported parameters |  | Low load BO range for Wi-Fi in Step 1: 10%~25% |  |  | Medium load BO range for Wi-Fi in Step 1: 35%~50% |  |  | High load BO range for Wi-Fi in Step 1: above 55% |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Wi-Fi in step 1 | Wi-Fi in step 2 | LAA in step 2 | Wi-Fi in step 1 | Wi-Fi in step 2 | LAA in step 2 | Wi-Fi in step 1 | Wi-Fi in step 2 | LAA in step 2 |
| Ericsson | 3 | UPT CDF [Mbps] | 5% | 23.43 | (48.32) 39.44 | (94.2) 48.04 | 7.11 | (41.74) 30.56 | (83.22) 36.67 | 1.24 | (40.47) 26.69 | (91.08) 30.21 |
|  |  |  | 50% | 59.54 | (82.98) 76.87 | (128.7) 77.99 | 32.85 | (76.98) 68.46 | (119.7)367.9 | 13.41 | (75.73) 65.47 | (117.8) 63.47 |
|  |  |  | 95% | 83.58 | (102.1) 97.51 | (147.5) 96.26 | 63.28 | (96.72) 91.34 | (141.5) 89.13 | 41.90 | (95.50) 89.02 | (140.5) 86.31 |
|  |  |  | Mean | 58.84 | (82.32) 76.25 | (128.2) 77.78 | 35.18 | (76.42) 67.82 | (119.5) 67.77 | 18.21 | (75.14) 64.59 | (117.8) 63.15 |
|  |  | Delay CDF [s] | 5% | 0.049 | (0.037) 0.039 | (0.025) 0.039 | 0.073 | (0.040) 0.042 | (0.027) 0.043 | 0.126 | (0.040) 0.043 | (0.027) 0.045 |
|  |  |  | 50% | 0.123 | (0.055) 0.065 | (0.034) 0.068 | 0.321 | (0.062) 0.079 | (0.038) 0.089 | 0.648 | (0.064) 0.086 | (0.039) 0.101 |
|  |  |  | 95% | 0.783 | (0.095) 0.174 | (0.048) 0.157 | 1.859 | (0.118) 0.294 | (0.058) 0.268 | 3.078 | (0.123) 0.424 | (0.060) 0.416 |
|  |  |  | Mean | 0.264 | (0.061) 0.088 | (0.035) 0.085 | 0.645 | (0.071) 0.128 | (0.04) 0.124 | 1.154 | (0.074) 0.163 | (0.042) 0.17 |
|  |  | VoIP outage (%) |  | 11.3 | (0.76) 9.8 | N/A | 22.4 | (1.2) 19.2 | N/A | 41.4 | (1.1) 22.4 | N/A |
|  |  | 𝜌 |  | 0.9819 | (0.999) 0.998 | (0.998) 0.997 | 0.929 | (0.998) 0.996 | (0.998)0.995 | 0.8233 | (0.998) 0.991 | (0.998)0.988 |
|  |  | BO |  | 0.20 | (0.095) 0.119 | (0.051)0.053 | 0.40 | (0.125)60.169 | (0.068)0.079 | 0.60 | (0.133)0.194 | (0.073)0.096 |
|  |  | 𝜆 |  | 0.60 files/s |  |  | 0.79 files/s |  |  | 0.94 files/s |  |  |
|  | Additional comments: (*(): results with licensed carrier) 256 QAM, LDPC, 2Tx2Rx, TXOP 4ms, and LBT algorithm based on R1-150584 where the LAA CCA starts and resumes at the subframe boundaries. If LBT succeeds before the 4th OFDM symbol in a subframe the channel is occupied by the LAA node. If the LBT fails by the 4th OFDM symbol in a subframe, the random backoff counter is frozen and the CCA is postponed to the next subframe boundary where the CCA count down is resumed. |  |  |  |  |  |  |  |  |  |  |  |
| R1-150551/ Broadcom | Cat 3 | UPT CDF [Mbps] | 5% | 23.177 | 13.81 | 23.116 | 9.554 | 0.032 | 3.687 | 0.013 | 0 | 0.455 |
|  |  |  | 50% | 42.444 | 32.666 | 42.048 | 27.306 | 11.964 | 20.04 | 5.415 | 2.538 | 6.207 |
|  |  |  | 95% | 62.496 | 53.436 | 60.645 | 40.136 | 37.926 | 56.095 | 24.391 | 28.621 | 35.795 |
|  |  |  | Mean | 44.287 | 35.456 | 42.831 | 26.752 | 15.968 | 23.146 | 8.856 | 7.385 | 10.749 |
|  |  | Delay CDF [s] | 5% | 0.007 | 0.009 | 0.007 | 0.017 | 0.34 | 0.035 | 0.326 | 1.125 | 0.314 |
|  |  |  | 50% | 0.059 | 0.091 | 0.053 | 0.141 | 1.466 | 0.286 | 1.154 | 2.556 | 1.281 |
|  |  |  | 95% | 0.196 | 0.587 | 0.236 | 0.557 | 2.716 | 0.886 | 2.731 | 4.196 | 2.599 |
|  |  |  | Mean | 0.075 | 0.18 | 0.079 | 0.191 | 1.509 | 0.347 | 1.318 | 2.694 | 1.336 |
|  |  | VoIP outage (%) |  | 0.00 | 50.00 | N/A | 10 | 70 | N/A | 50.00 | 70.00 | N/A |
|  |  | 98 %ile VoIP latency (ms) |  | 17.43 | 281.78 | N/A | 25.229 | 231.314 | N/A | 51.95 | 620.36 | N/A |
|  |  | 𝜌 |  | 0.986 | 0.976 | 0.987 | 0.98 | 0.813 | 0.947 | 0.804 | 0.514 | 0.840 |
|  |  | BO |  | 0.2 | 0.301 | 0.21 | 0.429 | 0.664 | 0.494 | 0.753 | 0.819 | 0.767 |
|  |  | 𝜆 |  | 0.56 |  |  | 0.74 |  |  | 1 |  |  |
|  | Additional comments: No licensed carrier. Fast fading disabled. Both Wi-Fi & LAA use the closed loop link adaptation, TXOP=5ms, UE noise figure 7dB, antenna configuration 1Tx2Rx, and 256QAM. Wi-Fi uses LDPC. LAA LBT: 0us initial defer as in EU-CCA [3] and q =13: denoted as EU_InitialDefer0_q13 in R1-150551. q is set to 13 (=260us) for obtaining a maximum channel hold time of 5ms |  |  |  |  |  |  |  |  |  |  |  |
| R1-150551/ Broadcom | Cat 3 | UPT CDF [Mbps] | 5% | 23.177 | 13.492 | 19.999 | 9.554 | 0.316 | 3.063 | 0.013 | 0 | 0.342 |
|  |  |  | 50% | 42.444 | 38.819 | 39.284 | 27.306 | 12.35 | 18.696 | 5.415 | 2.09 | 4.858 |
|  |  |  | 95% | 62.496 | 54.795 | 60.052 | 40.136 | 45.833 | 55.16 | 24.391 | 27.815 | 26.923 |
|  |  |  | Mean | 44.287 | 38.176 | 41.063 | 26.752 | 18.466 | 22.941 | 8.856 | 6.888 | 9.766 |
|  |  | Delay CDF [s] | 5% | 0.007 | 0.009 | 0.008 | 0.017 | 0.243 | 0.029 | 0.326 | 0.99 | 0.237 |
|  |  |  | 50% | 0.059 | 0.082 | 0.06 | 0.141 | 0.863 | 0.298 | 1.154 | 2.008 | 1.313 |
|  |  |  | 95% | 0.196 | 0.404 | 0.271 | 0.557 | 1.926 | 0.833 | 2.731 | 4.018 | 2.622 |
|  |  |  | Mean | 0.075 | 0.13 | 0.089 | 0.191 | 0.955 | 0.336 | 1.318 | 2.218 | 1.371 |
|  |  | VoIP outage (%) |  | 0.00 | 30.00 | N/A | 10 | 50 | N/A | 50.00 | 60.00 | N/A |
|  |  | 98 %ile VoIP latency (ms) |  | 17.43 | 53.06 | N/A | 25.229 | 169.554 | N/A | 51.95 | 387.00 | N/A |
|  |  | 𝜌 |  | 0.986 | 0.98 | 0.99 | 0.98 | 0.88 | 0.948 | 0.804 | 0.536 | 0.809 |
|  |  | BO |  | 0.2 | 0.271 | 0.221 | 0.429 | 0.586 | 0.473 | 0.753 | 0.803 | 0.767 |
|  |  | 𝜆 |  | 0.56 |  |  | 0.74 |  |  | 1 |  |  |
|  | Additional comments: No licensed carrier. Fast fading disabled. Both Wi-Fi & LAA use the closed loop link adaptation, TXOP=5ms, UE noise figure 7dB, antenna configuration 1Tx2Rx, and 256QAM. Wi-Fi uses LDPC. LAA LBT: 43us initial defer to match the initial defer for Wi-Fi BE traffic and q =13: denoted as EU_InitialDefer43_q13 in R1-15051 |  |  |  |  |  |  |  |  |  |  |  |
| R1-150551/ Broadcom | Cat 3 | UPT CDF [Mbps] | 5% | 23.177 | 16.384 | 18.439 | 9.554 | 0.421 | 1.781 | 0.013 | 0 | 0.198 |
|  |  |  | 50% | 42.444 | 41.166 | 37.908 | 27.306 | 19.796 | 13.969 | 5.415 | 2.814 | 4.031 |
|  |  |  | 95% | 62.496 | 59.95 | 57.318 | 40.136 | 47.284 | 42.83 | 24.391 | 35.318 | 22.251 |
|  |  |  | Mean | 44.287 | 41.358 | 38.582 | 26.752 | 21.129 | 17.759 | 8.856 | 8.667 | 7.485 |
|  |  | Delay CDF [s] | 5% | 0.007 | 0.008 | 0.008 | 0.017 | 0.178 | 0.064 | 0.326 | 0.816 | 0.31 |
|  |  |  | 50% | 0.059 | 0.07 | 0.066 | 0.141 | 0.661 | 0.513 | 1.154 | 2.065 | 1.599 |
|  |  |  | 95% | 0.196 | 0.344 | 0.29 | 0.557 | 1.584 | 1.175 | 2.731 | 4.175 | 3.085 |
|  |  |  | Mean | 0.075 | 0.113 | 0.096 | 0.191 | 0.728 | 0.532 | 1.318 | 2.303 | 1.64 |
|  |  | VoIP outage (%) |  | 0.00 | 30.00 | N/A | 10 | 50 | N/A | 50.00 | 50.00 | N/A |
|  |  | 98 %ile VoIP latency (ms) |  | 17.43 | 38.568 | N/A | 25.229 | 66.289 | N/A | 51.95 | 159.61 | N/A |
|  |  | 𝜌 |  | 0.986 | 0.982 | 0.985 | 0.98 | 0.881 | 0.90 | 0.804 | 0.586 | 0.769 |
|  |  | BO |  | 0.2 | 0.245 | 0.237 | 0.429 | 0.563 | 0.546 | 0.753 | 0.781 | 0.798 |
|  |  | 𝜆 |  | 0.56 |  |  | 0.74 |  |  | 1 |  |  |
|  | Additional comments: No licensed carrier. Fast fading disabled. Both Wi-Fi & LAA use the closed loop link adaptation, TXOP=5ms, UE noise figure 7dB, antenna configuration 1Tx2Rx, and 256QAM. Wi-Fi uses LDPC. LAA LBT: 43us initial defer to match the initial defer for Wi-Fi BE traffic and q =26: denoted as EU_InitialDefer43_q26 in R1-150551. Even though q is increased to 26, the maximum channel hold time is still 5ms. |  |  |  |  |  |  |  |  |  |  |  |
| R1-150551/ Broadcom | Cat 3 | UPT CDF [Mbps] | 5% | 23.177 | 24.553 | 25.504 | 9.554 | 8.25 | 9.032 | 0.013 | 0.094 | 0.861 |
|  |  |  | 50% | 42.444 | 44.91 | 42.843 | 27.306 | 21.795 | 22.145 | 5.415 | 4.334 | 6.541 |
|  |  |  | 95% | 62.496 | 62.481 | 53.891 | 40.136 | 42.309 | 41.61 | 24.391 | 25.25 | 31.385 |
|  |  |  | Mean | 44.287 | 45.638 | 42.548 | 26.752 | 25.517 | 23.342 | 8.856 | 7.331 | 9.338 |
|  |  | Delay CDF [s] | 5% | 0.007 | 0.007 | 0.007 | 0.017 | 0.016 | 0.019 | 0.326 | 0.582 | 0.162 |
|  |  |  | 50% | 0.059 | 0.054 | 0.052 | 0.141 | 0.153 | 0.14 | 1.154 | 1.558 | 0.837 |
|  |  |  | 95% | 0.196 | 0.201 | 0.206 | 0.557 | 0.702 | 0.466 | 2.731 | 3.853 | 1.942 |
|  |  |  | Mean | 0.075 | 0.072 | 0.071 | 0.191 | 0.235 | 0.179 | 1.318 | 1.989 | 0.931 |
|  |  | VoIP outage (%) |  | 0.00 | 0.00 | N/A | 10 | 20 | N/A | 50.00 | 40.00 | N/A |
|  |  | 98 %ile VoIP latency (ms) |  | 17.43 | 17.355 | N/A | 25.229 | 31.817 | N/A | 51.95 | 51.081 | N/A |
|  |  | 𝜌 |  | 0.986 | 0.987 | 0.986 | 0.98 | 0.984 | 0.972 | 0.804 | 0.660 | 0.870 |
|  |  | BO |  | 0.2 | 0.197 | 0.186 | 0.429 | 0.462 | 0.409 | 0.753 | 0.779 | 0.769 |
|  |  | 𝜆 |  | 0.56 |  |  | 0.74 |  |  | 1 |  |  |
|  | Additional comments: No licensed carrier. Fast fading disabled. Both Wi-Fi & LAA use the closed loop link adaptation, TXOP=5ms, UE noise figure 7dB, antenna configuration 1Tx2Rx, and 256QAM. Wi-Fi uses LDPC. LAA LBT based on ED as EU_InitialDefer43_q13 as well as LAA transmitting and receiving Wi-Fi preambles i.e. CCA-CS: denoted as EU_InitialDefer43_q13_CS in R1-150551 |  |  |  |  |  |  |  |  |  |  |  |
| R1-150551/ Broadcom | Cat 3 | UPT CDF [Mbps] | 5% | 23.177 | 27.551 | 19.525 | 9.554 | 19.439 | 7.732 | 0.013 | 0.746 | 0.381 |
|  |  |  | 50% | 42.444 | 49.527 | 38.252 | 27.306 | 34.913 | 19.902 | 5.415 | 8.911 | 1.886 |
|  |  |  | 95% | 62.496 | 64.75 | 51.062 | 40.136 | 50.115 | 40.72 | 24.391 | 35.579 | 17.011 |
|  |  |  | Mean | 44.287 | 49.325 | 38.031 | 26.752 | 35.442 | 21.435 | 8.856 | 12.707 | 3.737 |
|  |  | Delay CDF [s] | 5% | 0.007 | 0.006 | 0.007 | 0.017 | 0.01 | 0.023 | 0.326 | 0.157 | 0.396 |
|  |  |  | 50% | 0.059 | 0.047 | 0.061 | 0.141 | 0.075 | 0.168 | 1.154 | 0.814 | 1.757 |
|  |  |  | 95% | 0.196 | 0.16 | 0.242 | 0.557 | 0.309 | 0.609 | 2.731 | 1.838 | 3.468 |
|  |  |  | Mean | 0.075 | 0.06 | 0.085 | 0.191 | 0.104 | 0.222 | 1.318 | 0.871 | 1.824 |
|  |  | VoIP outage (%) |  | 0.00 | 0.00 | N/A | 10.00 | 10.00 | N/A | 50.00 | 30.00 | N/A |
|  |  | 98 %ile VoIP latency (ms) |  | 17.43 | 16.495 | N/A | 25.229 | 25.237 | N/A | 51.95 | 45.70 | N/A |
|  |  | 𝜌 |  | 0.986 | 0.99 | 0.988 | 0.98 | 0.979 | 0.96 | 0.804 | 0.861 | 0.699 |
|  |  | BO |  | 0.2 | 0.18 | 0.215 | 0.429 | 0.342 | 0.451 | 0.753 | 0.678 | 0.885 |
|  |  | 𝜆 |  | 0.56 |  |  | 0.74 |  |  | 1 |  |  |
|  | Additional comments: No licensed carrier. Fast fading disabled. Both Wi-Fi & LAA use the closed loop link adaptation, TXOP=5ms, UE noise figure 7dB, antenna configuration 1Tx2Rx, and 256QAM. Wi-Fi uses LDPC. LAA LBT based on ED as EU_InitialDefer43_q26 as well as LAA transmitting and receiving Wi-Fi preambles i.e. CCA-CS: denoted as EU_InitialDefer43_q26_CS in R1-150551 |  |  |  |  |  |  |  |  |  |  |  |
| R1-150551/ Broadcom | Cat 3 | UPT CDF [Mbps] | 5% | 23.177 | 9.01 | 19.11 | 9.554 | 0.156 | 1.758 | 0.013 | 0 | 0.338 |
|  |  |  | 50% | 42.444 | 34.95 | 35.956 | 27.306 | 12.799 | 14.847 | 5.415 | 1.191 | 4.47 |
|  |  |  | 95% | 62.496 | 53.956 | 58.797 | 40.136 | 41.983 | 46.554 | 24.391 | 27.818 | 26.061 |
|  |  |  | Mean | 44.287 | 35.005 | 37.939 | 26.752 | 17.541 | 19.632 | 8.856 | 6.902 | 8.441 |
|  |  | Delay CDF [s] | 5% | 0.007 | 0.011 | 0.01 | 0.017 | 0.143 | 0.036 | 0.326 | 0.921 | 0.246 |
|  |  |  | 50% | 0.059 | 0.116 | 0.07 | 0.141 | 0.978 | 0.407 | 1.154 | 2.225 | 1.369 |
|  |  |  | 95% | 0.196 | 0.546 | 0.313 | 0.557 | 2.198 | 1.041 | 2.731 | 4.688 | 2.822 |
|  |  |  | Mean | 0.075 | 0.167 | 0.106 | 0.191 | 1.083 | 0.446 | 1.318 | 2.492 | 1.438 |
|  |  | VoIP outage (%) |  | 0.00 | 40.00 | N/A | 10.00 | 50.00 | N/A | 50.00 | 50.00 | N/A |
|  |  | 98 %ile VoIP latency (ms) |  | 17.43 | 59.16 | N/A | 25.22 | 303.36 | N/A | 51.95 | 498.94 | N/A |
|  |  | 𝜌 |  | 0.986 | 0.977 | 0.982 | 0.98 | 0.844 | 0.916 | 0.804 | 0.516 | 0.797 |
|  |  | BO |  | 0.2 | 0.305 | 0.245 | 0.429 | 0.627 | 0.52 | 0.753 | 0.805 | 0.79 |
|  |  | 𝜆 |  | 0.56 |  |  | 0.74 |  |  | 1 |  |  |
|  | Additional comments: No licensed carrier. Fast fading disabled. Both Wi-Fi & LAA use the closed loop link adaptation, TXOP=5ms, UE noise figure 7dB, antenna configuration 1Tx2Rx, and 256QAM. Wi-Fi uses LDPC. LAA LBT based on ED as in EU_InitialDefer43_q13 and LAA only receiving but not transmitting Wi-Fi preambles i.e. CCA-CS-Rx-only: denoted as EU_InitialDefer43_q13_CSRx in R1-150551 |  |  |  |  |  |  |  |  |  |  |  |
| R1-150551/ Broadcom | Cat 3 | UPT CDF [Mbps] | 5% | 23.177 | 19.935 | 27.952 | 9.554 | 1.655 | 6.626 | 0.013 | 0 | 0.737 |
|  |  |  | 50% | 42.444 | 37.614 | 45.128 | 27.306 | 18.098 | 21.292 | 5.415 | 0.934 | 9.643 |
|  |  |  | 95% | 62.496 | 54.597 | 62.346 | 40.136 | 43.794 | 53.738 | 24.391 | 24.813 | 36.464 |
|  |  |  | Mean | 44.287 | 39.429 | 45.928 | 26.752 | 19.098 | 27.05 | 8.856 | 5.501 | 12.868 |
|  |  | Delay CDF [s] | 5% | 0.007 | 0.008 | 0.007 | 0.017 | 0.059 | 0.019 | 0.326 | 0.849 | 0.148 |
|  |  |  | 50% | 0.059 | 0.082 | 0.05 | 0.141 | 0.614 | 0.138 | 1.154 | 2.221 | 0.832 |
|  |  |  | 95% | 0.196 | 0.337 | 0.194 | 0.557 | 1.595 | 0.494 | 2.731 | 4.337 | 2.021 |
|  |  |  | Mean | 0.075 | 0.111 | 0.066 | 0.191 | 0.681 | 0.181 | 1.318 | 2.449 | 0.926 |
|  |  | VoIP outage (%) |  | 0.00 | 20.00 | N/A | 10.00 | 70.00 | N/A | 50.00 | 80.00 | N/A |
|  |  | 98 %ile VoIP latency (ms) |  | 17.43 | 38.29 | N/A | 25.229 | 246.11 | N/A | 51.95 | 480.748 | N/A |
|  |  | 𝜌 |  | 0.986 | 0.976 | 0.988 | 0.98 | 0.897 | 0.953 | 0.804 | 0.516 | 0.876 |
|  |  | BO |  | 0.2 | 0.253 | 0.182 | 0.429 | 0.592 | 0.395 | 0.753 | 0.835 | 0.722 |
|  |  | 𝜆 |  | 0.56 |  |  | 0.74 |  |  | 1 |  |  |
|  | Additional comments: No licensed carrier. Fast fading disabled. Both Wi-Fi & LAA use the closed loop link adaptation, TXOP=5ms, UE noise figure 7dB, antenna configuration 1Tx2Rx, and 256QAM. Wi-Fi uses LDPC. LAA LBT based on ED as in EU_InitialDefer43_q13 and LAA only transmitting but not receiving Wi-Fi preambles i.e. CCA-CS-Tx-only: denoted as EU_InitialDefer43_q13_CSTx in R1-150551. |  |  |  |  |  |  |  |  |  |  |  |
| LG | 2 | UPT CDF [Mbps] | 5% | 22.02 | 30.55 | 25.98 | 3.665 | 22.10 | 16.83 | 0.367 | 12.96 | 5.731 |
|  |  |  | 50% | 31.45 | 43.38 | 37.48 | 21.63 | 38.87 | 30.97 | 10.04 | 33.08 | 23.89 |
|  |  |  | 95% | 40.86 | 50.28 | 51.31 | 35.51 | 47.53 | 47.17 | 30.20 | 45.05 | 43.30 |
|  |  |  | Mean | 31.40 | 42.35 | 37.85 | 20.60 | 37.36 | 31.56 | 11.81 | 31.54 | 24.27 |
|  |  | Delay CDF [s] | 5% | 0.126 | 0.092 | 0.096 | 0.150 | 0.099 | 0.113 | 0.184 | 0.108 | 0.128 |
|  |  |  | 50% | 0.204 | 0.119 | 0.167 | 0.364 | 0.142 | 0.229 | 0.803 | 0.189 | 0.356 |
|  |  |  | 95% | 0.425 | 0.234 | 0.311 | 3.834 | 0.361 | 0.660 | 37.62 | 0.787 | 3.037 |
|  |  |  | Mean | 0.259 | 0.138 | 0.195 | 1.433 | 0.211 | 0.325 | 6.160 | 0.465 | 1.195 |
|  |  | VoIP outage (%) |  | 0 | 0 | N/A | 4.5 | 0 | N/A | 16 | 0.5 | N/A |
|  |  | 𝜌 |  | 0.9989 | 0.9997 | 0.9990 | 0.986 | 0.9992 | 0.9963 | 0.9415 | 0.9973 | 0.9894 |
|  |  | BO |  | 0.2411 | 0.1779 | 0.1812 | 0.423 | 0.204 | 0.2746 | 0.5864 | 0.3241 | 0.4049 |
|  |  | 𝜆 |  | 0.5 |  |  | 0.6 |  |  | 0.7 |  |  |
|  | Additional comments: LAA without licensed carrier, no 256 QAM, LDPC, 1Tx2Rx, TXOP 4ms CCA threshold: -62dBm for CCA-ED of LAA LBT algorithm: FBE procedure as defined in ETSI EN 301 893 V1.8.0 |  |  |  |  |  |  |  |  |  |  |  |
| LG | 3 | UPT CDF [Mbps] | 5% | 22.02 | 23.59 | 31.77 | 3.665 | 13.69 | 19.86 | 0.367 | 2.433 | 6.885 |
|  |  |  | 50% | 31.45 | 34.27 | 44.33 | 21.63 | 26.93 | 37.64 | 10.04 | 17.63 | 29.64 |
|  |  |  | 95% | 40.86 | 44.06 | 56.15 | 35.51 | 39.53 | 52.47 | 30.20 | 35.46 | 48.43 |
|  |  |  | Mean | 31.40 | 34.10 | 43.88 | 20.60 | 26.79 | 37.04 | 11.81 | 18.19 | 29.30 |
|  |  | Delay CDF [s] | 5% | 0.126 | 0.116 | 0.084 | 0.150 | 0.135 | 0.092 | 0.184 | 0.157 | 0.101 |
|  |  |  | 50% | 0.204 | 0.180 | 0.121 | 0.364 | 0.263 | 0.154 | 0.803 | 0.469 | 0.211 |
|  |  |  | 95% | 0.425 | 0.352 | 0.232 | 3.834 | 0.789 | 0.428 | 37.62 | 7.501 | 1.563 |
|  |  |  | Mean | 0.259 | 0.228 | 0.159 | 1.433 | 0.546 | 0.262 | 6.160 | 2.069 | 0.956 |
|  |  | VoIP outage (%) |  | 0 | 0 | N/A | 4.5 | 1.5 | N/A | 16 | 6.5 | N/A |
|  |  | 𝜌 |  | 0.9989 | 0.9993 | 0.9963 | 0.986 | 0.997 | 0.9950 | 0.9415 | 0.9832 | 0.9892 |
|  |  | BO |  | 0.2411 | 0.2226 | 0.1526 | 0.423 | 0.3291 | 0.2219 | 0.5864 | 0.4823 | 0.3186 |
|  |  | 𝜆 |  | 0.5 |  |  | 0.6 |  |  | 0.7 |  |  |
|  | Additional comments: LAA without licensed carrier, no 256 QAM, LDPC, 1Tx2Rx, TXOP 4ms CCA threshold: -62dBm for CCA-ED of LAA LBT algorithm: LBE procedure with a fixed q(=10) for the contention window as defined in ETSI EN 301 893 V1.8.0 |  |  |  |  |  |  |  |  |  |  |  |
| Intel | 3 | UPT CDF [Mbps] | 5% | 10 | 33 | 14 | 2 | 13 | 13 | 2 | 14 | 14 |
|  |  |  | 50% | 33 | 62 | 143 | 15 | 45 | 83 | 9 | 37 | 69 |
|  |  |  | 95% | 66 | 85 | 198 | 54 | 74 | 170 | 35 | 67 | 152 |
|  |  |  | Mean | 36 | 60 | 123 | 20 | 45 | 85 | 13 | 38 | 74 |
|  |  | Delay CDF [s] | 5% | 0.061 | 0.049 | 0.02 | 0.08 | 0.055 | 0.024 | 0.123 | 0.062 | 0.026 |
|  |  |  | 50% | 0.123 | 0.066 | 0.027 | 0.296 | 0.094 | 0.04 | 0.565 | 0.114 | 0.047 |
|  |  |  | 95% | 0.452 | 0.128 | 0.171 | 4.107 | 0.32 | 0.188 | 6.667 | 0.39 | 0.177 |
|  |  |  | Mean | 0.179 | 0.078 | 0.048 | 0.874 | 0.146 | 0.07 | 1.509 | 0.184 | 0.069 |
|  |  | VoIP outage (%) |  | 0.8 | 0 | N/A | 4.2 | 0.28 | N/A | 4.2 | 0.84 | N/A |
|  |  | 𝜌 |  | 1 | 1 | 1.0 | 0.966 | 0.994 | 1.0 | 0.936 | 0.996 | 1.0 |
|  |  | BO |  | 0.19 | 0.13 | 0.1 | 0.45 | 0.23 | 0.17 | 0.595 | 0.28 | 0.2 |
|  |  | 𝜆 |  | 0.5 |  |  | 0.71 |  |  | 0.83 |  |  |
|  | Additional comments: results with licensed carrier. 2Tx2Rx, TXOP 4ms, and LBT algorithm based on R1-1500089. |  |  |  |  |  |  |  |  |  |  |  |

Table 4: Indoor deployment for Wi-Fi and LAA coexistence case with four shared unlicensed carriers and FTP traffic

| Tdoc / Company | LAA LBT cat. | Reported parameters |  | Low load BO range for Wi-Fi in Step 1: 10%~25% |  |  | Medium load BO range for Wi-Fi in Step 1: 35%~50% |  |  | High load BO range for Wi-Fi in Step 1: above 55% |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Wi-Fi in step 1 | Wi-Fi in step 2 | LAA in step 2 | Wi-Fi in step 1 | Wi-Fi in step 2 | LAA in step 2 | Wi-Fi in step 1 | Wi-Fi in step 2 | LAA in step 2 |
| Ericsson | 3 | UPT CDF [Mbps] | 5% | 51.17 | (61.82) 54.02 | (104.0) 69.41 | 28.99 | (46.18) 34.13 | (71.20) 37.37 | 10.05 | (30.14) 22.33 | (45.89) 17.05 |
|  |  |  | 50% | 80.60 | (90.98) 89.45 | (131.1) 93.42 | 56.98 | (78.77) 76.64 | (102.5) 69.23 | 35.39 | (69.61) 68.23 | (79.99) 49.46 |
|  |  |  | 95% | 101.2 | (110.7) 109.8 | (150.7) 109.3 | 81.33 | (100.7) 98.80 | (128.8) 93.62 | 63.00 | (92.28) 90.52 | (111.8) 80.84 |
|  |  |  | Mean | 80.24 | (90.24) 87.92 | (130.8) 92.83 | 57.77 | (77.85) 74.73 | (102.8) 69.25 | 36.92 | (68.04) 65.73 | (80.91) 50.51 |
|  |  | Delay CDF [s] | 5% | 0.0410 | (0.035) 0.036 | (0.025) 0.03 | 0.0610 | (0.041) 0.042 | (0.030) 0.045 | 0.092 | (0.047) 0.048 | (0.037) 0.058 |
|  |  |  | 50% | 0.0650 | (0.049) 0.05 | (0.033) 0.05 | 0.1260 | (0.062) 0.065 | (0.049) 0.089 | 0.236 | (0.075) 0.079 | (0.070) 0.152 |
|  |  |  | 95% | 0.1190 | (0.076) 0.116 | (0.045) 0.099 | 0.4100 | (0.151) 0.232 | (0.088) 0.265 | 1.073 | (0.266) 0.351 | (0.163) 0.493 |
|  |  |  | Mean | 0.074 | (0.052) 0.074 | (0.034) 0.063 | 0.181 | (0.077) 0.116 | (0.054) 0.136 | 0.402 | (0.111) 0.143 | (0.084) 0.235 |
|  |  | 𝜌 |  | 0.9987 | (0.999) 0.998 | (0.999)0.998 | 0.9952 | (0.997) 0.993 | (0.998)0.992 | 0.9806 | (0.993)0.985 | (0.997)0.982 |
|  |  | BO |  | 0.20 | (0.164) 0.173 | (0.137)0.195 | 0.40 | (0.291)0.307 | (0.287)0.403 | 0.60 | (0.385)0.401 | (0.432) 0.585 |
|  |  | 𝜆 |  | 0.45 files/s |  |  | 0.79 files/s |  |  | 1.05 files/s |  |  |
|  | Additional comments: (*(): results with licensed carrier) 256 QAM, LDPC, 2Tx2Rx, TXOP 4ms, and LBT algorithm based on R1-150584 where the LAA CCA starts and resumes at the subframe boundaries. If LBT succeeds before the 4th OFDM symbol in a subframe the channel is occupied by the LAA node. If the LBT fails by the 4th OFDM symbol in a subframe, the random backoff counter is frozen and the CCA is postponed to the next subframe boundary where the CCA count down is resumed. |  |  |  |  |  |  |  |  |  |  |  |
| InterDigital IDCC | LBE Cat. 3 | UPT CDF [Mbps] | 5% | 35.650 | 34.530 | 35.908 | 15.321 | 15.396 | 24.651 | 3.659 | 2.086 | 7.575 |
|  |  |  | 50% | 90.623 | 107.366 | 122.448 | 63.307 | 64.069 | 82.126 | 19.439 | 17.990 | 46.704 |
|  |  |  | 95% | 120.679 | 120.791 | 143.691 | 120.609 | 120.581 | 143.691 | 108.932 | 120.443 | 142.782 |
|  |  |  | Mean | 88.721 | 90.852 | 106.327 | 69.479 | 72.033 | 87.410 | 30.916 | 36.899 | 58.792 |
|  |  | Delay CDF [s] | 5% | 0.035 | 0.035 | 0.030 | 0.035 | 0.035 | 0.030 | 0.037 | 0.035 | 0.030 |
|  |  |  | 50% | 0.046 | 0.040 | 0.035 | 0.066 | 0.066 | 0.052 | 0.213 | 0.234 | 0.091 |
|  |  |  | 95% | 0.120 | 0.121 | 0.115 | 0.272 | 0.259 | 0.167 | 1.186 | 2.509 | 0.601 |
|  |  |  | Mean | 0.059 | 0.057 | 0.051 | 0.098 | 0.095 | 0.070 | 0.393 | 0.603 | 0.169 |
|  |  | 𝜌 |  | 1.000 | 1.000 | 0.9094 | 1.000 | 1.000 | 0.9662 | 1.000 | 0.9801 | 0.9951 |
|  |  | BO |  | 21.852 | 20.884 | 16.047 | 41.8716 | 38.254 | 31.5543 | 75.381 | 66.516 | 57.917 |
|  |  | 𝜆 |  | 0.45 |  |  | 0.75 |  |  | 1.125 |  |  |
|  | Additional comments: TXOP=4ms, without licensed carrier LBE Cat.3: Option B from ETSI EN 301 893 v1.8.0, Section 4.8.3.2. CW size fixed and set by q at beginning of simulation. Random backoff N selected from [1;q]. TXOP = q*13/32 ms. |  |  |  |  |  |  |  |  |  |  |  |
| InterDigital IDCC | LBE Cat. 4 | UPT CDF [Mbps] | 5% | 35.650 | 36.527 | 34.425 | 15.321 | 16.569 | 21.401 | 3.659 | 3.400 | 4.345 |
|  |  |  | 50% | 90.623 | 113.758 | 129.206 | 63.307 | 69.290 | 79.918 | 19.439 | 20.206 | 34.728 |
|  |  |  | 95% | 120.679 | 120.692 | 143.691 | 120.609 | 120.637 | 143.691 | 108.932 | 120.360 | 142.556 |
|  |  |  | Mean | 88.721 | 92.962 | 105.458 | 69.479 | 74.953 | 85.808 | 30.916 | 39.482 | 51.215 |
|  |  | Delay CDF [s] | 5% | 0.035 | 0.035 | 0.030 | 0.035 | 0.035 | 0.030 | 0.037 | 0.035 | 0.030 |
|  |  |  | 50% | 0.046 | 0.037 | 0.033 | 0.066 | 0.060 | 0.054 | 0.213 | 0.201 | 0.120 |
|  |  |  | 95% | 0.120 | 0.115 | 0.126 | 0.272 | 0.231 | 0.196 | 1.186 | 1.392 | 1.230 |
|  |  |  | Mean | 0.059 | 0.055 | 0.053 | 0.098 | 0.088 | 0.078 | 0.393 | 0.421 | 0.286 |
|  |  | 𝜌 |  | 1.000 | 1.000 | 0.9097 | 1.000 | 1.000 | 0.9656 | 1.000 | 1.000 | 0.9814 |
|  |  | BO |  | 21.852 | 19.81 | 16.395 | 41.8716 | 36.761 | 32.874 | 75.381 | 64.641 | 64.176 |
|  |  | 𝜆 |  | 0.45 |  |  | 0.75 |  |  | 1.125 |  |  |
|  | Additional comments: TXOP=4ms, without licensed carrier LBE Cat.4: Modified Option A from ETSI EN 301 893 v1.8.0, Section 4.8.3.2. CW exponentially variable. Initially set to q=16 and doubled whenever N idle slots cannot be found within q observation slots. TXOP independent of q, fixed at beginning of simulation. 4ms and 13 ms (although technically Option A requires TXOP<10ms). |  |  |  |  |  |  |  |  |  |  |  |
| InterDigital IDCC | LBE Cat. 3 | UPT CDF [Mbps] | 5% | 35.650 | 36.535 | 39.289 | 15.321 | 20.232 | 24.684 | 3.659 | 3.274 | 9.119 |
|  |  |  | 50% | 90.623 | 112.169 | 133.481 | 63.307 | 67.485 | 86.240 | 19.439 | 25.495 | 45.877 |
|  |  |  | 95% | 120.679 | 120.734 | 143.920 | 120.609 | 120.790 | 143.748 | 108.932 | 120.471 | 142.839 |
|  |  |  | Mean | 88.721 | 93.015 | 107.630 | 69.479 | 74.369 | 90.885 | 30.916 | 41.952 | 59.075 |
|  |  | Delay CDF [s] | 5% | 0.035 | 0.035 | 0.030 | 0.035 | 0.035 | 0.030 | 0.037 | 0.035 | 0.030 |
|  |  |  | 50% | 0.046 | 0.037 | 0.032 | 0.066 | 0.063 | 0.050 | 0.213 | 0.164 | 0.092 |
|  |  |  | 95% | 0.120 | 0.115 | 0.106 | 0.272 | 0.207 | 0.169 | 1.186 | 1.629 | 0.487 |
|  |  |  | Mean | 0.059 | 0.055 | 0.050 | 0.098 | 0.086 | 0.068 | 0.393 | 0.412 | 0.161 |
|  |  | 𝜌 |  | 1.000 | 1.000 | 0.9109 | 1.000 | 1.000 | 0.9665 | 1.000 | 1.000 | 0.9948 |
|  |  | BO |  | 21.852 | 20.004 | 15.746 | 41.8716 | 36.661 | 30.782 | 75.381 | 63.8455 | 57.155 |
|  |  | 𝜆 |  | 0.45 |  |  | 0.75 |  |  | 1.125 |  |  |
|  | Additional comments: TXOP=13ms, without licensed carrier LBE Cat.3: Option B from ETSI EN 301 893 v1.8.0, Section 4.8.3.2. CW size fixed and set by q at beginning of simulation. Random backoff N selected from [1;q]. TXOP = q*13/32 ms. |  |  |  |  |  |  |  |  |  |  |  |
| InterDigital IDCC | LBE Cat. 4 | UPT CDF [Mbps] | 5% | 35.650 | 34.289 | 42.932 | 15.321 | 17.716 | 25.888 | 3.659 | 3.037 | 9.778 |
|  |  |  | 50% | 90.623 | 113.620 | 133.319 | 63.307 | 65.763 | 91.928 | 19.439 | 22.995 | 51.366 |
|  |  |  | 95% | 120.679 | 120.734 | 143.920 | 120.609 | 120.748 | 143.748 | 108.932 | 120.429 | 142.839 |
|  |  |  | Mean | 88.721 | 92.545 | 109.418 | 69.479 | 73.361 | 93.601 | 30.916 | 39.915 | 64.183 |
|  |  | Delay CDF [s] | 5% | 0.035 | 0.035 | 0.028 | 0.035 | 0.035 | 0.030 | 0.037 | 0.035 | 0.030 |
|  |  |  | 50% | 0.046 | 0.037 | 0.032 | 0.066 | 0.063 | 0.046 | 0.213 | 0.181 | 0.083 |
|  |  |  | 95% | 0.120 | 0.121 | 0.097 | 0.272 | 0.235 | 0.161 | 1.186 | 1.770 | 0.461 |
|  |  |  | Mean | 0.059 | 0.056 | 0.048 | 0.098 | 0.089 | 0.064 | 0.393 | 0.455 | 0.141 |
|  |  | 𝜌 |  | 1.000 | 1.000 | 0.9106 | 1.000 | 1.000 | 0.9660 | 1.000 | 1.000 | 0.9956 |
|  |  | BO |  | 21.852 | 19.954 | 15.270 | 41.8716 | 37.211 | 29.477 | 75.381 | 65.047 | 53.928 |
|  |  | 𝜆 |  | 0.45 |  |  | 0.75 |  |  | 1.125 |  |  |
|  | Additional comments: TXOP=13ms, without licensed carrier LBE Cat.4: Modified Option A from ETSI EN 301 893 v1.8.0, Section 4.8.3.2. CW exponentially variable. Initially set to q=16 and doubled whenever N idle slots cannot be found within q observation slots. TXOP independent of q, fixed at beginning of simulation. 4ms and 13 ms (although technically Option A requires TXOP<10ms). |  |  |  |  |  |  |  |  |  |  |  |
| Fujitsu | 3 | UPT CDF [Mbps] | 5% | 5.645 | 27.976 | 54.346 | 6.302 | 8.853 | 38.971 | 4.677 | 6.486 | 28.820 |
|  |  |  | 50% | 63.777 | 66.612 | 87.002 | 40.312 | 46.571 | 59.501 | 20.487 | 33.031 | 43.368 |
|  |  |  | 95% | 102.041 | 100.108 | 137.438 | 64.344 | 67.292 | 80.838 | 36.809 | 46.236 | 59.032 |
|  |  |  | Mean | 62.889 | 65.939 | 88.508 | 40.171 | 45.868 | 59.876 | 20.857 | 31.575 | 43.650 |
|  |  | Delay CDF [s] | 5% | 0.032 | 0.032 | 0.029 | 0.036 | 0.033 | 0.029 | 0.044 | 0.036 | 0.029 |
|  |  |  | 50% | 0.060 | 0.053 | 0.029 | 0.088 | 0.078 | 0.060 | 0.177 | 0.114 | 0.078 |
|  |  |  | 95% | 0.223 | 0.159 | 0.094 | 0.430 | 0.304 | 0.166 | 1.195 | 0.577 | 0.247 |
|  |  |  | Mean | 0.169 | 0.128 | 0.048 | 0.179 | 0.153 | 0.071 | 0.352 | 0.212 | 0.101 |
|  |  | 𝜌 |  | 0.998 | 0.999 | 1.000 | 0.997 | 0.998 | 0.999 | 0.987 | 0.995 | 0.997 |
|  |  | BO |  | 16.046 | 15.369 | 11.471 | 37.881 | 33.546 | 27.515 | 66.612 | 54.547 | 44.582 |
|  |  | 𝜆 |  | 0.3 |  |  | 0.6 |  |  | 0.9 |  |  |
|  | Additional comments: backoff method 2 in R1-150185, FTP model 1 |  |  |  |  |  |  |  |  |  |  |  |
| Huawei, HiSilicon, | Cat.3 | UPT CDF [Mbps] | 5% | 25.31 | 29.18 | 32.19 | 3.34 | 4.51 | 21.47 | 0.25 | 0.37 | 1.26 |
|  |  |  | 50% | 43.53 | 45.45 | 47.51 | 26.69 | 31.73 | 40.97 | 2.01 | 4.12 | 24.07 |
|  |  |  | 95% | 57.49 | 57.69 | 52.93 | 44.94 | 49.64 | 52.22 | 24.71 | 35.25 | 44.23 |
|  |  |  | Mean | 42.44 | 44.46 | 45.81 | 25.36 | 31.23 | 39.21 | 5.70 | 10.92 | 23.97 |
|  |  | Delay CDF [s] | 5% | 0.070 | 0.070 | 0.076 | 0.089 | 0.080 | 0.076 | 0.16 | 0.11 | 0.090 |
|  |  |  | 50% | 0.091 | 0.088 | 0.083 | 0.15 | 0.12 | 0.097 | 1.69 | 0.90 | 0.16 |
|  |  |  | 95% | 0.15 | 0.14 | 0.12 | 1.02 | 0.79 | 0.18 | 6.69 | 6.11 | 2.52 |
|  |  |  | Mean | 0.14 | 0.12 | 0.090 | 0.28 | 0.20 | 0.11 | 2.52 | 1.86 | 0.54 |
|  |  | 𝜌 |  | 1.00 | 1.00 | 1.00 | 1.00 | 1.00 | 1.00 | 0.88 | 0.93 | 0.97 |
|  |  | BO |  | 0.23 | 0.15 | 0.14 | 0.42 | 0.33 | 0.30 | 0.85 | 0.76 | 0.66 |
|  |  | 𝜆 |  | 0.2 |  |  | 0.4 |  |  | 0.8 |  |  |
|  | Additional comments: R1-150042, No 256QAM, no LDPC code, LAA max. duration =13ms, WiFi DL only, LAA on unlicensed carrier only. LBT scheme follows the Option B from ETSI EN 301 893 v1.8.0 |  |  |  |  |  |  |  |  |  |  |  |

| Samsung | 3 | UPT CDF [Mbps] | 5% | 32.1 | 36.9 | 102.6 | 26.1 | 32.3 | 91.2 | 18.1 | 26.3 | 78.9 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 50% | 81.9 | 90.0 | 128.8 | 69.9 | 80.9 | 115.9 | 55.2 | 69.3 | 103.4 |
|  |  |  | 95% | 99.3 | 105.3 | 158.4 | 88.9 | 96.7 | 147.9 | 79.0 | 89.1 | 136.1 |
|  |  |  | Mean | 81.8 | 89.7 | 129.3 | 69.9 | 81.0 | 117.3 | 56.0 | 70.1 | 104.9 |
|  |  | Delay CDF [s] | 5% | 0.034 | 0.034 | 0.022 | 0.034 | 0.034 | 0.022 | 0.034 | 0.034 | 0.023 |
|  |  |  | 50% | 0.050 | 0.042 | 0.029 | 0.062 | 0.050 | 0.033 | 0.083 | 0.060 | 0.039 |
|  |  |  | 95% | 0.280 | 0.214 | 0.073 | 0.400 | 0.278 | 0.090 | 0.668 | 0.399 | 0.118 |
|  |  |  | Mean | 0.062 | 0.052 | 0.036 | 0.081 | 0.062 | 0.042 | 0.120 | 0.081 | 0.051 |
|  |  | 𝜌 |  | 1.00 | 1.00 | 1.00 | 1.00 | 1.00 | 1.00 | 0.99 | 1.00 | 1.00 |
|  |  | BO |  | 0.30 | 0.27 | 0.21 | 0.41 | 0.36 | 0.28 | 0.55 | 0.46 | 0.37 |
|  |  | 𝜆 |  | 0.4 |  |  | 0.7 |  |  | 1.1 |  |  |
|  | Additional comments: LAA with licensed carrier, TXOP: 10ms(LAA), 4ms(WiFi), 2Tx2Rx, BCC, no 256QAM, CCA slot: 20us. For LBE, CCA (20us) is always followed by ECCA |  |  |  |  |  |  |  |  |  |  |  |
| Intel | 3 | UPT CDF [Mbps] | 5% | 45 | 55 | 17 | 28 | 44 | 13 | 10 | 29 | 17 |
|  |  |  | 50% | 69 | 75 | 159 | 47 | 60 | 75 | 26 | 44 | 59 |
|  |  |  | 95% | 89 | 93 | 215 | 69 | 76 | 157 | 41 | 57 | 127 |
|  |  |  | Mean | 69 | 75 | 133 | 48 | 60 | 80 | 26 | 44 | 63 |
|  |  | Delay CDF [s] | 5% | 0.05 | 0.04 | 0.02 | 0.06 | 0.05 | 0.02 | 0.10 | 0.07 | 0.03 |
|  |  |  | 50% | 0.06 | 0.05 | 0.02 | 0.09 | 0.07 | 0.03 | 0.17 | 0.10 | 0.04 |
|  |  |  | 95% | 0.09 | 0.08 | 0.08 | 0.16 | 0.10 | 0.07 | 0.65 | 0.16 | 0.06 |
|  |  |  | Mean | 0.06 | 0.06 | 0.03 | 0.10 | 0.07 | 0.04 | 0.25 | 0.11 | 0.04 |
|  |  | 𝜌 |  | 0.99 | 1.00 | 1.00 | 0.99 | 0.99 | 1.00 | 1.00 | 1.00 | 1.00 |
|  |  | BO |  | 0.17 | 0.16 | 0.16 | 0.39 | 0.34 | 0.32 | 0.66 | 0.51 | 0.5 |
|  |  | 𝜆 |  | 0.34 |  |  | 0.71 |  |  | 1.01 |  |  |
|  | Additional comments: results with licensed carrier. 2Tx2Rx, TXOP 4ms, and LBT algorithm based on R1-1500089. |  |  |  |  |  |  |  |  |  |  |  |

Table 5: Indoor deployment for LAA and LAA coexistence case with four shared unlicensed carriers and FTP traffic

| Tdoc / Company | LAA LBT cat. | Reported parameters |  | Low load BO range: 10%~25% |  | Medium load BO range: 35%~50% |  | High load BO range: above 55% |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | LAA opt. 1 | LAA opt. 2 | LAA opt. 1 | LAA opt. 2 | LAA opt. 1 | LAA opt. 2 |
| Ericsson | 3 | UPT CDF [Mbps] | 5% | 76.28 | 75.06 | 40.18 | 40.52 | 24.42 | 25.89 |
|  |  |  | 50% | 97.9 | 96.23 | 78.5 | 75.46 | 53.82 | 52.78 |
|  |  |  | 95% | 111 | 111.1 | 97.3 | 95.96 | 76.98 | 75.32 |
|  |  |  | Mean | 97.3 | 95.9 | 76.57 | 74.26 | 53.65 | 53.02 |
|  |  | Delay CDF [s] | 5% | 0.034 | 0.034 | 0.041 | 0.043 | 0.088 | 0.081 |
|  |  |  | 50% | 0.043 | 0.044 | 0.06 | 0.064 | 0.286 | 0.173 |
|  |  |  | 95% | 0.061 | 0.066 | 0.215 | 0.236 | 0.737 | 0.47 |
|  |  |  | Mean | 0.046 | 0.047 | 0.105 | 0.109 | 0.364 | 0.231 |
|  |  | 𝜌 |  | 1.00 | 1.00 | 0.99 | 0.99 | 0.94 | 0.93 |
|  |  | BO |  | 0.20 | 0.20 | 0.40 | 0.41 | 0.60 | 0.60 |
|  |  | 𝜆 |  | 0.558 files/s |  | 0.971 files/s |  | 1.356 files/s |  |
|  | Additional comments: (*(): results with licensed carrier) 256 QAM, LDPC, 2Tx2Rx, TXOP 4ms, and LBT algorithm based on R1-150584 where the LAA CCA starts and resumes at the subframe boundaries. If LBT succeeds before the 4th OFDM symbol in a subframe the channel is occupied by the LAA node. If the LBT fails by the 4th OFDM symbol in a subframe, the random backoff counter is frozen and the CCA is postponed to the next subframe boundary where the CCA count down is resumed. |  |  |  |  |  |  |  |  |
| InterDigital IDCC | LBE Cat.3 | UPT CDF [Mbps] | 5% | 34.526 | 30.973 | 4.705 | 3.068 | 1.055 | 1.138 |
|  |  |  | 50% | 137.968 | 131.097 | 63.531 | 59.521 | 25.324 | 31.872 |
|  |  |  | 95% | 143.767 | 143.996 | 143.197 | 143.311 | 142.914 | 142.492 |
|  |  |  | Mean | 108.398 | 107.392 | 72.789 | 69.305 | 50.349 | 50.613 |
|  |  | Delay CDF [s] | 5% | 0.030 | 0.030 | 0.030 | 0.030 | 0.030 | 0.030 |
|  |  |  | 50% | 0.031 | 0.032 | 0.067 | 0.072 | 0.177 | 0.131 |
|  |  |  | 95% | 0.114 | 0.139 | 0.783 | 1.750 | 3.178 | 3.101 |
|  |  |  | Mean | 0.051 | 0.054 | 0.209 | 0.309 | 0.719 | 0.616 |
|  |  | 𝜌 |  | 1.000 | 0.922 | 0.944 | 0.827 | 0.873 | 0.737 |
|  |  | BO |  | 19.743 | 16.925 | 47.827 | 45.479 | 68.293 | 65.786 |
|  |  | 𝜆 |  | 0.45 |  | 0.88 |  | 1.13 |  |
|  | Additional comments: TXOP=4ms, without licensed carrier LBE Cat.3: Option B from ETSI EN 301 893 v1.8.0, Section 4.8.3.2. CW size fixed and set by q at beginning of simulation. Random backoff N selected from [1;q]. TXOP = q*13/32 ms. |  |  |  |  |  |  |  |  |
| IDCC | LBE Cat.4 | UPT CDF [Mbps] | 5% | 49.018 | 46.320 | 13.183 | 13.406 | 5.798 | 5.881 |
|  |  |  | 50% | 140.032 | 139.494 | 94.480 | 74.516 | 64.041 | 53.897 |
|  |  |  | 95% | 143.767 | 143.996 | 143.653 | 143.767 | 143.140 | 142.914 |
|  |  |  | Mean | 116.631 | 115.739 | 91.684 | 81.637 | 72.966 | 66.539 |
|  |  | Delay CDF [s] | 5% | 0.030 | 0.030 | 0.030 | 0.030 | 0.030 | 0.030 |
|  |  |  | 50% | 0.031 | 0.031 | 0.045 | 0.057 | 0.066 | 0.079 |
|  |  |  | 95% | 0.086 | 0.092 | 0.444 | 0.539 | 0.771 | 1.238 |
|  |  |  | Mean | 0.043 | 0.044 | 0.120 | 0.147 | 0.201 | 0.240 |
|  |  | 𝜌 |  | 1.000 | 0.925 | 1.000 | 0.877 | 0.963 | 0.868 |
|  |  | BO |  | 17.483 | 14.902 | 44.544 | 40.626 | 62.530 | 60.812 |
|  |  | 𝜆 |  | 0.45 |  | 1.00 |  | 1.38 |  |
|  | Additional comments: TXOP=4ms, without licensed carrier LBE Cat.4: Modified Option A from ETSI EN 301 893 v1.8.0, Section 4.8.3.2. CW exponentially variable. Initially set to q=16 and doubled whenever N idle slots cannot be found within q observation slots. TXOP independent of q, fixed at beginning of simulation. 4ms and 13 ms (although technically Option A requires TXOP<10ms). |  |  |  |  |  |  |  |  |
| InterDigital IDCC | LBE Cat.3 | UPT CDF [Mbps] | 5% | 42.709 | 37.130 | 15.344 | 14.473 | 4.024 | 3.646 |
|  |  |  | 50% | 139.923 | 143.767 | 81.747 | 79.737 | 51.894 | 46.806 |
|  |  |  | 95% | 138.489 | 143.996 | 143.367 | 143.881 | 143.140 | 142.521 |
|  |  |  | Mean | 112.826 | 112.250 | 86.392 | 83.800 | 65.748 | 59.296 |
|  |  | Delay CDF [s] | 5% | 0.030 | 0.030 | 0.030 | 0.030 | 0.030 | 0.030 |
|  |  |  | 50% | 0.031 | 0.031 | 0.053 | 0.055 | 0.083 | 0.090 |
|  |  |  | 95% | 0.098 | 0.111 | 0.274 | 0.367 | 0.964 | 1.256 |
|  |  |  | Mean | 0.045 | 0.047 | 0.102 | 0.106 | 0.244 | 0.268 |
|  |  | 𝜌 |  | 1.000 | 0.924 | 1.000 | 0.900 | 0.989 | 0.855 |
|  |  | BO |  | 18.227 | 15.629 | 38.847 | 36.623 | 62.527 | 58.925 |
|  |  | 𝜆 |  | 0.45 |  | 0.88 |  | 1.25 |  |
|  | Additional comments: TXOP=13ms, without licensed carrier LBE Cat.3: Option B from ETSI EN 301 893 v1.8.0, Section 4.8.3.2. CW size fixed and set by q at beginning of simulation. Random backoff N selected from [1;q]. TXOP = q*13/32 ms. |  |  |  |  |  |  |  |  |
| IDCC | LBE Cat.4 | UPT CDF [Mbps] | 5% | 46.816 | 47.518 | 13.112 | 13.355 | 4.765 | 6.057 |
|  |  |  | 50% | 140.086 | 139.495 | 90.117 | 74.454 | 61.277 | 52.670 |
|  |  |  | 95% | 143.767 | 143.996 | 143.595 | 143.767 | 143.140 | 142.744 |
|  |  |  | Mean | 117.490 | 116.141 | 91.503 | 82.020 | 71.206 | 65.185 |
|  |  | Delay CDF [s] | 5% | 0.030 | 0.030 | 0.030 | 0.030 | 0.030 | 0.030 |
|  |  |  | 50% | 0.031 | 0.031 | 0.048 | 0.058 | 0.069 | 0.080 |
|  |  |  | 95% | 0.086 | 0.089 | 0.452 | 0.536 | 1.007 | 1.415 |
|  |  |  | Mean | 0.042 | 0.043 | 0.118 | 0.148 | 0.223 | 0.258 |
|  |  | 𝜌 |  | 1.000 | 0.925 | 1.000 | 0.876 | 0.959 | 0.861 |
|  |  | BO |  | 17.247 | 14.755 | 44.394 | 40.418 | 62.485 | 60.505 |
|  |  | 𝜆 |  | 0.45 |  | 1.00 |  | 1.38 |  |
|  | Additional comments: TXOP=13ms, without licensed carrier LBE Cat.4: Modified Option A from ETSI EN 301 893 v1.8.0, Section 4.8.3.2. CW exponentially variable. Initially set to q=16 and doubled whenever N idle slots cannot be found within q observation slots. TXOP independent of q, fixed at beginning of simulation. 4ms and 13 ms (although technically Option A requires TXOP<10ms). |  |  |  |  |  |  |  |  |
| Fujitsu | 3 | UPT CDF [Mbps] | 5% | 60.654 | 64.273 | 51.855 | 48.031 | 36.420 | 37.542 |
|  |  |  | 50% | 94.629 | 94.723 | 70.331 | 69.237 | 51.854 | 52.819 |
|  |  |  | 95% | 137.608 | 137.608 | 95.256 | 92.240 | 67.208 | 69.266 |
|  |  |  | Mean | 94.458 | 96.561 | 71.099 | 69.612 | 52.352 | 53.340 |
|  |  | Delay CDF [s] | 5% | 0.029 | 0.029 | 0.029 | 0.029 | 0.029 | 0.029 |
|  |  |  | 50% | 0.029 | 0.029 | 0.057 | 0.057 | 0.060 | 0.060 |
|  |  |  | 95% | 0.086 | 0.086 | 0.122 | 0.126 | 0.182 | 0.176 |
|  |  |  | Mean | 0.044 | 0.044 | 0.060 | 0.060 | 0.081 | 0.079 |
|  |  | 𝜌 |  | 1.000 | 1.000 | 0.999 | 0.998 | 0.994 | 0.997 |
|  |  | BO |  | 10.413 | 10.487 | 23.410 | 23.030 | 39.141 | 39.155 |
|  |  | 𝜆 |  | 0.3 |  | 0.6 |  | 0.9 |  |
|  | Additional comments: backoff method 2 in R1-150185, FTP model 1 |  |  |  |  |  |  |  |  |
| Huawei, HiSilicon, | Cat.3 | UPT CDF [Mbps] | 5% | 34.87 | 34.87 | 9.00 | 9.00 | 1.15 | 1.15 |
|  |  |  | 50% | 48.19 | 48.19 | 34.67 | 34.67 | 25.85 | 25.85 |
|  |  |  | 95% | 52.93 | 52.93 | 48.97 | 48.97 | 44.99 | 44.99 |
|  |  |  | Mean | 46.57 | 46.57 | 32.41 | 32.41 | 24.44 | 24.44 |
|  |  | Delay CDF [s] | 5% | 0.076 | 0.076 | 0.081 | 0.081 | 0.088 | 0.088 |
|  |  |  | 50% | 0.083 | 0.083 | 0.12 | 0.12 | 0.15 | 0.15 |
|  |  |  | 95% | 0.11 | 0.11 | 0.43 | 0.43 | 2.27 | 2.27 |
|  |  |  | Mean | 0.087 | 0.087 | 0.18 | 0.18 | 0.52 | 0.52 |
|  |  | 𝜌 |  | 1.00 | 1.00 | 1.00 | 1.00 | 0.99 | 0.99 |
|  |  | BO |  | 0.14 | 0.14 | 0.47 | 0.47 | 0.66 | 0.66 |
|  |  | 𝜆 |  | 0.2 |  | 0.6 |  | 0.8 |  |
|  | Additional comments: R1-150043, No 256QAM, LAA max. duration =13ms, LAA on unlicensed carrier only. LBT scheme follows the Option B from ETSI EN 301 893 v1.8.0. The performance metrics is not independently calculated per operator in LAA and LAA coexistence case. |  |  |  |  |  |  |  |  |

| Samsung | 3 | UPT CDF [Mbps] | 5% | 78.6 | 78.2 | 58.5 | 59.4 | 35.3 | 35.8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 50% | 98.1 | 97.4 | 79.8 | 79.3 | 59.0 | 58.9 |
|  |  |  | 95% | 142.7 | 142.2 | 127.0 | 123.8 | 109.1 | 107.7 |
|  |  |  | Mean | 98.9 | 98.8 | 80.5 | 80.1 | 60.1 | 60.0 |
|  |  | Delay CDF [s] | 5% | 0.027 | 0.027 | 0.028 | 0.028 | 0.031 | 0.032 |
|  |  |  | 50% | 0.039 | 0.039 | 0.053 | 0.054 | 0.077 | 0.078 |
|  |  |  | 95% | 0.120 | 0.119 | 0.207 | 0.206 | 0.494 | 0.476 |
|  |  |  | Mean | 0.052 | 0.052 | 0.076 | 0.077 | 0.146 | 0.142 |
|  |  | 𝜌 |  | 1.00 | 1.00 | 1.00 | 1.00 | 0.97 | 0.97 |
|  |  | BO |  | 0.32 | 0.33 | 0.48 | 0.48 | 0.67 | 0.67 |
|  |  | 𝜆 |  | 0.6 |  | 0.9 |  | 1.3 |  |
|  | Additional comments: LAA with licensed carrier, TXOP: 10ms(LAA), 4ms(WiFi), 2Tx2Rx, BCC, no 256QAM, CCA slot: 20us. For LBE, CCA (20us) is always followed by ECCA |  |  |  |  |  |  |  |  |
| Intel | 3 | UPT CDF [Mbps] | 5% | 18 | 17 | 14 | 16 | 18 | 17 |
|  |  |  | 50% | 140 | 140 | 89 | 90 | 64 | 63 |
|  |  |  | 95% | 197 | 205 | 166 | 166 | 123 | 119 |
|  |  |  | Mean | 121 | 122 | 89 | 90 | 65 | 66 |
|  |  | Delay CDF [s] | 5% | 0.02 | 0.02 | 0.02 | 0.02 | 0.03 | 0.03 |
|  |  |  | 50% | 0.03 | 0.03 | 0.03 | 0.03 | 0.04 | 0.04 |
|  |  |  | 95% | 0.09 | 0.07 | 0.06 | 0.06 | 0.06 | 0.06 |
|  |  |  | Mean | 0.04 | 0.03 | 0.04 | 0.04 | 0.04 | 0.04 |
|  |  | 𝜌 |  | 1.00 | 1..0 | 1.00 | 1.00 | 1.00 | 1.00 |
|  |  | BO |  | 0.26 | 0.26 | 0.42 | 0.42 | 0.63 | 0.62 |
|  |  | 𝜆 |  | 0.43 |  | 0.66 |  | 1.05 |  |
|  | Additional comments: results with licensed carrier. 2Tx2Rx, TXOP 4ms, and LBT algorithm based on R1-1500089. |  |  |  |  |  |  |  |  |

Table 6: Indoor deployment for Wi-Fi and LAA coexistence case with four shared unlicensed carriers and Mixed traffic

| Tdoc / Company | LAA LBT cat. | Reported parameters |  | Low load BO range for Wi-Fi in Step 1: 10%~25% |  |  | Medium load BO range for Wi-Fi in Step 1: 35%~50% |  |  | High load BO range for Wi-Fi in Step 1: above 55% |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Wi-Fi in step 1 | Wi-Fi in step 2 | LAA in step 2 | Wi-Fi in step 1 | Wi-Fi in step 2 | LAA in step 2 | Wi-Fi in step 1 | Wi-Fi in step 2 | LAA in step 2 |
| Ericsson | 3 | UPT CDF [Mbps] | 5% | 48.51 | (59.24) 56.25 | (102.7) 64.88 | 26.81 | (45.73) 33.42 | (71.47) 35.32 | 10.61 | (31.28) 20.21 | (46.95) 16.45 |
|  |  |  | 50% | 79.71 | (89.9) 88.81 | (127.9) 88.18 | 57.60 | (77.97 75.92 | (100.7) 66.67 | 37.12 | (67.51) 66.35 | (77.29) 48.56 |
|  |  |  | 95% | 100.04 | (109.4) 108.2 | (148.3) 106.5 | 80.38 | (99.28 97.41 | (127.7) 92.74 | 63.37 | (89.91) 89.42 | (108.4) 78.76 |
|  |  |  | Mean | 78.93 | (89.08) 87.58 | (127.7) 88.25 | 57.43 | (77.2) 73.88 | (101.1) 67.02 | 37.99 | (66.75) 64.51 | (78.71) 49.23 |
|  |  | Delay CDF [s] | 5% | 0.0420 | (0.036) 0.107 | (0.025) 0.123 | 0.0610 | (0.031) 0.045 | (0.042) 0.042 | 0.0870 | (0.048) 0.049 | (0.038) 0.06 |
|  |  |  | 50% | 0.0660 | (0.049) 0.05 | (0.033) 0.053 | 0.1220 | (0.049) 0.089 | (0.063) 0.065 | 0.2180 | (0.080) 0.081 | (0.072) 0.161 |
|  |  |  | 95% | 0.1270 | (0.077) 0.036 | (0.046) 0.036 | 0.3590 | (0.083) 0.269 | (0.128) 0.191 | 0.8720 | (0.219) 0.303 | (0.143) 0.486 |
|  |  |  | Mean | 0.078 | (0.053) 0.064 | (0.034) 0.07 | 0.173 | (0.076) 0.1 | (0.053) 0.133 | 0.355 | (0.118) 0.141 | (0.082) 0.23 |
|  |  | VoIP outage % |  | 0 | 0 | N/A | 0 | 0 | N/A | 0 | 0 | N/A |
|  |  | 𝜌 |  | 0.9989 | (0.999)0.998 | (0.999)0.998 | 0.9957 | (0.998)0.994 | (0.998) 0.993 | 0.989 | (0.996)0.987 | (0.995)0.985 |
|  |  | BO |  | 0.20 | (0.133) 0.197 | (0.182)0.194 | 0.40 | (0.279) 0.341 | (0.313)0.395 | 0.60 | (0.428) 0.441 | (0.416)0.579 |
|  |  | 𝜆 |  | 0.42 files/s |  |  | 0.74 files/s |  |  | 0.98 files/s |  |  |
|  | Additional comments: (*(): results with licensed carrier) 256 QAM, LDPC, 2Tx2Rx, TXOP 4ms, and LBT algorithm based on R1-150584 where the LAA CCA starts and resumes at the subframe boundaries. If LBT succeeds before the 4th OFDM symbol in a subframe the channel is occupied by the LAA node. If the LBT fails by the 4th OFDM symbol in a subframe, the random backoff counter is frozen and the CCA is postponed to the next subframe boundary where the CCA count down is resumed. |  |  |  |  |  |  |  |  |  |  |  |
| Intel | 3 | UPT CDF [Mbps] | 5% | 43 | 53 | 18 | 28 | 43 | 14 | 13 | 33 | 17 |
|  |  |  | 50% | 64 | 74 | 155 | 47 | 61 | 77 | 30 | 50 | 65 |
|  |  |  | 95% | 86 | 90 | 210 | 67 | 78 | 158 | 50 | 65 | 128 |
|  |  |  | Mean | 64 | 73 | 134 | 47 | 61 | 83 | 31 | 50 | 67 |
|  |  | Delay CDF [s] | 5% | 0.047 | 0.045 | 0.019 | 0.061 | 0.052 | 0.024 | 0.084 | 0.063 | 0.028 |
|  |  |  | 50% | 0.063 | 0.055 | 0.024 | 0.089 | 0.067 | 0.032 | 0.145 | 0.083 | 0.037 |
|  |  |  | 95% | 0.096 | 0.077 | 0.096 | 0.156 | 0.098 | 0.069 | 0.411 | 0.134 | 0.062 |
|  |  |  | Mean | 0.066 | 0.057 | 0.034 | 0.097 | 0.07 | 0.037 | 0.189 | 0.092 | 0.041 |
|  |  | VoIP outage % |  | 0 | 0 | N/A | 0 | 0 | N/A | 0.22 | 0 | N/A |
|  |  | 𝜌 |  | 0.98 | 0.99 | 1.0 | 0.99 | 0.99 | 1.0 | 0.99 | 1.00 | 1.0 |
|  |  | BO |  | 0.25 | 0.29 | 0.21 | 0.45 | 0.42 | 0.47 | 0.65 | 0.58 | 0.56 |
|  |  | 𝜆 |  | 0.34 |  |  | 0.66 |  |  | 0.90 |  |  |
|  | Additional comments: results with licensed carrier. 2Tx2Rx, TXOP 4ms, and LBT algorithm based on R1-1500089. |  |  |  |  |  |  |  |  |  |  |  |

Table 7: Outdoor deployment for Wi-Fi and LAA coexistence case with one shared unlicensed carrier and FTP traffic

| Tdoc / Company | LAA LBT cat. | Reported parameters |  | Low load BO range for Wi-Fi in Step 1: 10%~25% |  |  | Medium load BO range for Wi-Fi in Step 1: 35%~50% |  |  | High load BO range for Wi-Fi in Step 1: above 55% |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Wi-Fi in step 1 | Wi-Fi in step 2 | LAA in step 2 | Wi-Fi in step 1 | Wi-Fi in step 2 | LAA in step 2 | Wi-Fi in step 1 | Wi-Fi in step 2 | LAA in step 2 |
| Ericsson | 3 | UPT CDF [Mbps] | 5% | 8.23 | (30.28) 20.12 | (73.82) 28.17 | 1.19 | (25.2) 12.82 | (68.81) 20.91 | 0.35 | (20.55) 7.14 | (63.5) 14.11 |
|  |  |  | 50% | 43.04 | (82.90) 69.82 | (127.8) 76.51 | 19.22 | (77.19) 56.72 | (122.7) 67.8 | 7.38 | (70.63) 42.8 | (116.9) 56.55 |
|  |  |  | 95% | 75.86 | (109.5) 101.1 | (152.9) 101.9 | 50.12 | (105.5) 94.43 | (151.2) 97.99 | 30.96 | (100.9) 82.9 | (148.3) 90.66 |
|  |  |  | Mean | 43.95 | (80.55) 68.54 | (124.5) 74.2 | 22.27 | (75.12) 57.44 | (120.1) 66.34 | 11.32 | (69.14) 45.19 | (114.8) 56.48 |
|  |  | Delay CDF [s] | 5% | 0.051 | (0.032) 0.034 | (0.023) 0.033 | 0.081 | (0.033) 0.037 | (0.023) 0.036 | 0.129 | (0.034) 0.047 | (0.023) 0.04 |
|  |  |  | 50% | 0.156 | (0.048) 0.07 | (0.03) 0.06 | 0.359 | (0.054) 0.098 | (0.032) 0.077 | 0.68 | (0.062) 0.162 | (0.034) 0.108 |
|  |  |  | 95% | 1.006 | (0.122) 0.275 | (0.048) 0.192 | 2.423 | (0.15) 0.491 | (0.054) 0.291 | 3.366 | (0.207) 1.037 | (0.062) 0.495 |
|  |  |  | Mean | 0.318 | (0.062) 0.109 | (0.033) 0.082 | 0.74 | (0.071) 0.176 | (0.035) 0.112 | 1.141 | (0.089) 0.33 | (0.038) 0.179 |
|  |  | 𝜌 |  | 0.97 | (1.00) 1.00 | (1.00) 1.00 | 0.89 | (1.00) 0.99 | (1.00) 1.00 | 0.74 | (0.99) 0.96 | (1.00) 0.99 |
|  |  | BO |  | 0.20 | (0.06) 0.09 | (0.03) 0.08 | 0.40 | (0.07) 0.13 | (0.04) 0.11 | 0.60 | (0.10) 0.22 | (0.05) 0.16 |
|  |  | 𝜆 |  | 0.436 files/s |  |  | 0.552 files/s |  |  | 0.7566 files/s |  |  |
|  | Additional comments: (*(): results with licensed carrier) 256 QAM, LDPC, 2Tx2Rx, TXOP 4ms, and LBT algorithm based on R1-150584 where the LAA CCA starts and resumes at the subframe boundaries. If LBT succeeds before the 4th OFDM symbol in a subframe the channel is occupied by the LAA node. If the LBT fails by the 4th OFDM symbol in a subframe, the random backoff counter is frozen and the CCA is postponed to the next subframe boundary where the CCA count down is resumed. |  |  |  |  |  |  |  |  |  |  |  |
| CATT | LBT Cat. 1 | UPT CDF [Mbps] | 5% | 3.10 | 1.56 | 13.3 | 0 | 0 | 3.19 | 0 | 0 | 2.02 |
|  |  |  | 50% | 64.89 | 30.95 | 56.34 | 6.14 | 2.43 | 27.03 | 1.95 | 0.27 | 18.31 |
|  |  |  | 95% | 128.6 | 85.70 | 114.29 | 96.74 | 61.07 | 88.88 | 45.72 | 30.20 | 67.80 |
|  |  |  | Mean | 66.35 | 36.49 | 58.12 | 20.98 | 12.35 | 33.64 | 9.45 | 4.86 | 24.72 |
|  |  | Delay CDF [s] | 5% | 0.0621 | 0.0908 | 0.0660 | 0.0706 | 0.1094 | 0.0820 | 0.1276 | 0.1965 | 0.1120 |
|  |  |  | 50% | 0.1201 | 0.2460 | 0.1380 | 0.6312 | 1.0710 | 0.2760 | 1.0384 | 1.8767 | 0.3990 |
|  |  |  | 95% | 1.6396 | 2.1179 | 0.566 | 2.8749 | 3.9230 | 1.2670 | 2.986 | 4.4714 | 1.9070 |
|  |  |  | Mean | 0.3735 | 0.5424 | 0.2223 | 0.9825 | 1.4147 | 0.4226 | 1.2993 | 2.0750 | 0.5948 |
|  |  | 𝜌 |  | 0.9528 | 0.9315 | 0.9836 | 0.7602 | 0.6756 | 0.9525 | 0.6046 | 0.4630 | 0.9639 |
|  |  | BO |  | 12% | 16.03% | 9.23% | 44.41% | 52.83% | 28.42% | 58.47% | 69.77% | 48.53% |
|  |  | 𝜆 |  | 1 |  |  | 2 |  |  | 3 |  |  |
|  | Additional comments: FTP model 1, maximum transmission duration is 4ms for both LAA and WIFI. For LAA, traffic is served on the unlicensed carrier only. For WIFI, RTS/CTS, 256QAM, LDPC codes are not modeled. |  |  |  |  |  |  |  |  |  |  |  |
| CATT | LBT Cat. 2 | UPT CDF [Mbps] | 5% | 3.10 | 2.05 | 5.65 | 0 | 0 | 0.969 | 0 | 0 | 0.53 |
|  |  |  | 50% | 64.89 | 73.63 | 79.65 | 6.14 | 5.64 | 18.95 | 1.95 | 1.40 | 10.58 |
|  |  |  | 95% | 128.6 | 129.98 | 127.59 | 96.74 | 122.96 | 125.58 | 45.72 | 72.56 | 100.67 |
|  |  |  | Mean | 66.35 | 71.55 | 75.94 | 20.98 | 25.3 | 34.98 | 9.45 | 12.16 | 24.22 |
|  |  | Delay CDF [s] | 5% | 0.0621 | 0.0615 | 0.0625 | 0.0706 | 0.0635 | 0.0637 | 0.1276 | 0.1037 | 0.0761 |
|  |  |  | 50% | 0.1201 | 0.1062 | 0.0972 | 0.6312 | 0.6269 | 0.3552 | 1.0384 | 1.0402 | 0.7409 |
|  |  |  | 95% | 1.6396 | 2.3308 | 1.1735 | 2.8749 | 3.273 | 2.3656 | 2.986 | 2.714 | 2.4427 |
|  |  |  | Mean | 0.3735 | 0.3969 | 0.2867 | 0.9825 | 1.0089 | 0.6526 | 1.2993 | 1.1831 | 0.9887 |
|  |  | 𝜌 |  | 0.9528 | 0.9235 | 0.9723 | 0.7602 | 0.7366 | 0.8978 | 0.6046 | 0.5870 | 0.8427 |
|  |  | BO |  | 12% | 13.04% | 10.55% | 44.41% | 43.3% | 34.18% | 58.47% | 55.79% | 48.22% |
|  |  | 𝜆 |  | 1 |  |  | 2 |  |  | 3 |  |  |
|  | Additional comments: FTP model 1, maximum transmission duration is 4ms for both LAA and WIFI. For LAA, the frame based LBT as defined in ETSI EN 301 893 v1.8.0 section 4.8.3.1 is used with frame period of 4ms and CCA time slot of 20us. For LAA, traffic is served on the unlicensed carrier only. For WIFI, RTS/CTS, 256QAM, LDPC codes are not modeled. |  |  |  |  |  |  |  |  |  |  |  |
| CATT | LBT Cat.3 | UPT CDF [Mbps] | 5% | 3.10 | 2.71 | 6.33 | 0 | 0 | 1.05 | 0 | 0 | 0.62 |
|  |  |  | 50% | 64.89 | 70.79 | 78.19 | 6.14 | 4.57 | 18.48 | 1.95 | 1.08 | 10.55 |
|  |  |  | 95% | 128.6 | 128.77 | 125.2 | 96.74 | 113.79 | 121.07 | 45.72 | 62.81 | 91.56 |
|  |  |  | Mean | 66.35 | 70.08 | 75.43 | 20.98 | 21.87 | 33.5 | 9.45 | 10.67 | 23.03 |
|  |  | Delay CDF [s] | 5% | 0.0621 | 0.0621 | 0.0638 | 0.0706 | 0.0655 | 0.0650 | 0.1276 | 0.1052 | 0.0752 |
|  |  |  | 50% | 0.1201 | 0.1126 | 0.1009 | 0.6312 | 0.6853 | 0.3563 | 1.0384 | 1.0561 | 0.7235 |
|  |  |  | 95% | 1.6396 | 1.6065 | 1.0823 | 2.8749 | 3.4184 | 2.1366 | 2.986 | 2.802 | 2.4162 |
|  |  |  | Mean | 0.3735 | 0.3608 | 0.2725 | 0.9825 | 1.0947 | 0.6140 | 1.2993 | 1.2784 | 0.9768 |
|  |  | 𝜌 |  | 0.9528 | 0.9562 | 0.9753 | 0.7602 | 0.7144 | 0.9075 | 0.6046 | 0.5643 | 0.8564 |
|  |  | BO |  | 12% | 12.56% | 10.11% | 44.41% | 46.44% | 33.88% | 58.47% | 57.40% | 48.30% |
|  |  | 𝜆 |  | 1 |  |  | 2 |  |  | 3 |  |  |
|  | Additional comments: FTP model 1, maximum transmission duration is 4ms for both LAA and WIFI. For LAA, load based LBT as defined in ETSI EN 301 893 v1.8.0 section 4.8.3.2 option B is used with q=32 and 20us CCA time slot. For LAA, traffic is served on the unlicensed carrier only. For WIFI, RTS/CTS, 256QAM, LDPC codes are not modeled. |  |  |  |  |  |  |  |  |  |  |  |
| ALU, ASB | LBE Cat 3 | UPT CDF [Mbps] | 5% | 23.39 | 16.23 | 32.40 | 2.30 | 4.59 | 14.80 | 1.36 | 1.16 | 7.08 |
|  |  |  | 50% | 65.92 | 55.35 | 72.11 | 28.07 | 21.06 | 46.48 | 18.34 | 11.51 | 39.11 |
|  |  |  | 95% | 104.37 | 95.87 | 96.68 | 79.00 | 62.16 | 80.14 | 63.51 | 51.66 | 72.57 |
|  |  |  | Mean | 65.21 | 55.46 | 69.33 | 32.86 | 25.24 | 47.38 | 24.74 | 17.18 | 39.75 |
|  |  | Delay CDF [s] | 5% | 0.032 | 0.032 | 0.036 | 0.032 | 0.037 | 0.036 | 0.036 | 0.044 | 0.038 |
|  |  |  | 50% | 0.064 | 0.084 | 0.054 | 0.184 | 0.265 | 0.087 | 0.258 | 0.434 | 0.107 |
|  |  |  | 95% | 0.535 | 0.886 | 0.229 | 2.390 | 2.506 | 0.514 | 2.967 | 3.710 | 0.688 |
|  |  |  | Mean | 0.139 | 0.242 | 0.092 | 0.535 | 0.635 | 0.161 | 0.731 | 0.988 | 0.215 |
|  |  | 𝜌 |  | 0.98 | 0.94 | 0.87 | 0.81 | 0.84 | 0.77 | 0.71 | 0.70 | 0.75 |
|  |  | BO |  | 0.17 | 0.21 | 0.12 | 0.49 | 0.57 | 0.25 | 0.64 | 0.74 | 0.33 |
|  |  | 𝜆 |  | 0.60 |  |  | 0.85 |  |  | 1.00 |  |  |
|  | Additional comments: LBE, q Value:32, Maximum Occupancy Time : 10ms, LAA without licensed carrier 2Tx2Rx, BCC, no 256QAM CCA threshold: -73 dBm/MHz + 23 - PH, PH specified in dBm EIRP for CCA-ED of LAA, -62 dBm for CCA-ED of Wi-Fi; |  |  |  |  |  |  |  |  |  |  |  |
| ALU, ASB | FBE Cat 2 | UPT CDF [Mbps] | 5% | 23.39 | 23.20 | 38.74 | 2.30 | 4.40 | 6.93 | 1.36 | 1.55 | 4.55 |
|  |  |  | 50% | 65.92 | 68.23 | 69.00 | 28.07 | 29.96 | 41.61 | 18.34 | 17.88 | 30.24 |
|  |  |  | 95% | 104.37 | 107.73 | 96.80 | 79.00 | 77.27 | 72.83 | 63.51 | 67.99 | 59.05 |
|  |  |  | Mean | 65.21 | 67.72 | 68.17 | 32.86 | 34.51 | 41.27 | 24.74 | 23.68 | 30.97 |
|  |  | Delay CDF [s] | 5% | 0.032 | 0.033 | 0.037 | 0.032 | 0.033 | 0.037 | 0.036 | 0.035 | 0.037 |
|  |  |  | 50% | 0.064 | 0.060 | 0.056 | 0.184 | 0.180 | 0.107 | 0.258 | 0.339 | 0.166 |
|  |  |  | 95% | 0.535 | 0.481 | 0.266 | 2.390 | 1.647 | 0.835 | 2.967 | 3.344 | 1.465 |
|  |  |  | Mean | 0.139 | 0.130 | 0.092 | 0.535 | 0.450 | 0.231 | 0.731 | 0.803 | 0.380 |
|  |  | 𝜌 |  | 0.98 | 0.94 | 0.85 | 0.81 | 0.89 | 0.74 | 0.71 | 0.75 | 0.68 |
|  |  | BO |  | 0.17 | 0.15 | 0.12 | 0.49 | 0.46 | 0.31 | 0.64 | 0.66 | 0.46 |
|  |  | 𝜆 |  | 0.60 |  |  | 0.85 |  |  | 1 |  |  |
|  | Additional comments: FBE frame period: 10ms(Maximum Occupancy Time= 9.5ms, idle time = 0.5ms), LAA without licensed carrier 2Tx2Rx, BCC, no 256QAM CCA threshold: -73 dBm/MHz + 23 - PH, PH specified in dBm EIRP for CCA-ED of LAA, -62 dBm for CCA-ED of Wi-Fi; |  |  |  |  |  |  |  |  |  |  |  |
| QCOM | LBT Cat 3 | UPT CDF [Mbps] | 5% | 19.04 | 14.64 | 11.10 | 2.03 | 2.67 | 1.03 | 0.63 | 0.91 | 0.37 |
|  |  |  | 50% | 53.40 | 61.34 | 59.33 | 33.60 | 38.78 | 25.66 | 20.70 | 29.48 | 13.86 |
|  |  |  | 95% | 81.58 | 91.26 | 88.76 | 70.04 | 80.31 | 65.44 | 58.64 | 73.23 | 52.37 |
|  |  |  | Mean | 52.36 | 58.37 | 56.32 | 34.37 | 39.76 | 28.00 | 23.44 | 32.22 | 18.08 |
|  |  | Delay CDF [s] | 5% | 1.31e-2 | 1.71e-2 | 2.25e-2 | 1.23e-1 | 9.36e-2 | 2.43e-1 | 3.97e-1 | 2.75e-1 | 6.76e-1 |
|  |  |  | 50% | 4.68e-3 | 4.08e-3 | 4.21e-3 | 7.44e-3 | 6.45e-3 | 9.74e-3 | 1.21e-2 | 8.48e-3 | 1.80e-2 |
|  |  |  | 95% | 3.06e-3 | 2.74e-3 | 2.82e-3 | 3.57e-3 | 3.11e-3 | 3.82e-3 | 4.26e-3 | 3.41e-3 | 4.77e-3 |
|  |  |  | Mean | 4.77e-3 | 4.28e-3 | 4.44e-3 | 7.27e-3 | 6.29e-3 | 8.93e-3 | 1.07e-2 | 7.76e-3 | 1.38e-2 |
|  |  | 𝜌 |  | 1.00 | 1.00 | 1.00 | 0.97 | 0.98 | 0.97 | 0.91 | 0.96 | 0.90 |
|  |  | BO |  | 20.0% | 14.8% | 13.9% | 40.0% | 35.3% | 42.3% | 60.0% | 47.3% | 58.8% |
|  |  | 𝜆 |  | 0.83Mbps |  |  | 1.02Mbps |  |  | 1.18Mbps |  |  |
|  | Additional comments: For the WiFi operator, CCA-ED = -62dBm, Preamble detect at 4dB SINR, defer period based on AIFS, 256QAM not used, LDPC codes used, RTS/CTS not used and 3ms TXOP. For the LAA operator, all traffic for unlicensed only carrier, CCA-ED = -82dBm, LBT Cat 3 as defined in ETSI EN 301 893 v1.8.0 Option B, q = 32 , Max TXOP 9.5ms and 256QAM not used |  |  |  |  |  |  |  |  |  |  |  |
| NTT DOCOMO | 2 | UPT CDF [Mbps] | 5% | 1.38 | 2.32 | 4.51 | 0.708 | 0.826 | 1.27 | 0.664 | 0.654 | 0.686 |
|  |  |  | 50% | 34.5 | 53.3 | 57.6 | 23.4 | 32.0 | 43.0 | 18.1 | 23.1 | 30.3 |
|  |  |  | 95% | 121.2 | 121.2 | 121.2 | 102.6 | 121.2 | 121.2 | 85.1 | 114.3 | 117.7 |
|  |  |  | Mean | 43.3 | 56.7 | 62.3 | 33.1 | 43.3 | 49.5 | 27.3 | 35.4 | 40.1 |
|  |  | Delay CDF [s] | 5% | 0.033 | 0.033 | 0.033 | 0.039 | 0.033 | 0.033 | 0.046 | 0.034 | 0.034 |
|  |  |  | 50% | 0.115 | 0.074 | 0.069 | 0.169 | 0.125 | 0.093 | 0.218 | 0.170 | 0.131 |
|  |  |  | 95% | 2.23 | 1.50 | 0.887 | 0.472 | 0.419 | 2.62 | 4.66 | 5.33 | 3.75 |
|  |  |  | Mean | 0.530 | 0.414 | 0.262 | 0.837 | 0.744 | 0.463 | 0.915 | 0.892 | 0.783 |
|  |  | 𝜌 |  | 0.978 | 0.986 | 0.997 | 0.980 | 0.982 | 0.984 | 0.976 | 0.971 | 0.976 |
|  |  | BO |  | 0.241 | 0.183 | 0.088 | 0.441 | 0.370 | 0.180 | 0.592 | 0.499 | 0.239 |
|  |  | 𝜆 |  | 2 |  |  | 2.5 |  |  | 3 |  |  |
|  | Additional comments: Results without licensed carrier. FTP model 1, maximum 4ms burst length for both Wi-Fi and LAA. 1ms CCA period for LAA when channel is busy as described in R1-150414. |  |  |  |  |  |  |  |  |  |  |  |
| NTT DOCOMO | 2 | UPT CDF [Mbps] | 5% | 1.38 | 3.75 | 51.3 | 0.708 | 1.77 | 41.2 | 0.664 | 1.88 | 40.8 |
|  |  |  | 50% | 34.5 | 58.8 | 121.2 | 23.4 | 47.6 | 108.1 | 18.1 | 43.0 | 102.6 |
|  |  |  | 95% | 121.2 | 121.2 | 181.8 | 102.6 | 121.2 | 181.8 | 85.1 | 121.2 | 181.8 |
|  |  |  | Mean | 43.3 | 62.1 | 120.0 | 33.1 | 53.0 | 108.6 | 27.3 | 50.1 | 106.4 |
|  |  | Delay CDF [s] | 5% | 0.033 | 0.033 | 0.022 | 0.039 | 0.033 | 0.022 | 0.046 | 0.033 | 0.022 |
|  |  |  | 50% | 0.115 | 0.067 | 0.033 | 0.169 | 0.084 | 0.037 | 0.219 | 0.091 | 0.039 |
|  |  |  | 95% | 2.23 | 0.924 | 0.078 | 4.72 | 1.84 | 0.097 | 4.66 | 1.98 | 0.098 |
|  |  |  | Mean | 0.531 | 0.360 | 0.040 | 0.837 | 0.433 | 0.047 | 0.915 | 0.439 | 0.048 |
|  |  | 𝜌 |  | 0.978 | 0.989 | 0.999 | 0.980 | 0.983 | 0.999 | 0.976 | 0.983 | 0.999 |
|  |  | BO |  | 0.241 | 0.158 | 0.019 | 0.441 | 0.239 | 0.028 | 0.592 | 0.297 | 0.033 |
|  |  | 𝜆 |  | 2 |  |  | 2.5 |  |  | 3 |  |  |
|  | Additional comments: Results with licensed carrier. FTP model 1, maximum 4ms burst length for both Wi-Fi and LAA. 1ms CCA period for LAA when channel is busy as described in R1-150414. |  |  |  |  |  |  |  |  |  |  |  |
| ZTE | 3 | UPT CDF [Mbps] | 5% | 6 | 23 | 56 | 1 | 9 | 31 | 0.309 | 0.28 | 1.975 |
|  |  |  | 50% | 19 | 34 | 91 | 8 | 26 | 72 | 0.607 | 1.423 | 23.946 |
|  |  |  | 95% | 28 | 41 | 113 | 19 | 38 | 106 | 8.155 | 23.756 | 78.133 |
|  |  |  | Mean | 18 | 33 | 89 | 9 | 25 | 73 | 1.451 | 6.216 | 30.271 |
|  |  | Delay CDF [s] | 5% | 0.096 | 0.077 | 0.032 | 0.151 | 0.079 | 0.032 | 0.39 | 0.122 | 0.04 |
|  |  |  | 50% | 0.249 | 0.124 | 0.042 | 0.62 | 0.172 | 0.062 | 5.817 | 1.361 | 0.172 |
|  |  |  | 95% | 1.101 | 0.329 | 0.19 | 6.774 | 0.792 | 0.303 | 23.281 | 18.172 | 1.707 |
|  |  |  | Mean | 0.382 | 0.153 | 0.072 | 1.566 | 0.284 | 0.108 | 7.975 | 4.4433 | 0.447 |
|  |  | 𝜌 |  | 98.90% | 99.80% | 99.80% | 93.80% | 98.90% | 99.30% | 57.70% | 74.10% | 91.00% |
|  |  | BO |  | 24% | 13% | 7% | 50% | 24% | 13% | 82% | 72% | 48% |
|  |  | 𝜆 |  | 1.05 |  |  | 1.45 |  |  | 3.2 |  |  |
|  | Additional comments: No 256QAM, no LDPC code, 2Tx2Rx, 24 dBm TX power for both LAA and WiFi, no licensed carrier for LAA, LAA max. duration =4 ms, RTS/CTS enabled for WiFi |  |  |  |  |  |  |  |  |  |  |  |
| ZTE | 2 | UPT CDF [Mbps] | 5% | 6 | 23.8 | 59.2 | 1 | 12.1 | 29.8 | 0.309 | 0.33 | 2 |
|  |  |  | 50% | 19 | 34.1 | 86.7 | 8 | 27 | 71.3 | 0.607 | 1.68 | 20 |
|  |  |  | 95% | 28 | 42 | 104.6 | 19 | 37.8 | 97.7 | 8.155 | 22.31 | 69.6 |
|  |  |  | Mean | 18 | 33.6 | 85.4 | 9 | 26.5 | 69.8 | 1.451 | 6.21 | 26.6 |
|  |  | Delay CDF [s] | 5% | 0.096 | 0.077 | 0.035 | 0.151 | 0.078 | 0.035 | 0.39 | 0.136 | 0.046 |
|  |  |  | 50% | 0.249 | 0.121 | 0.044 | 0.62 | 0.16 | 0.063 | 5.817 | 1.393 | 0.198 |
|  |  |  | 95% | 1.101 | 0.314 | 0.185 | 6.774 | 0.574 | 0.291 | 23.281 | 16.28 | 3.155 |
|  |  |  | Mean | 0.382 | 0.147 | 0.073 | 1.566 | 0.229 | 0.107 | 7.975 | 4.194 | 0.607 |
|  |  | 𝜌 |  | 98.90% | 99. 8% | 99.80% | 93.80% | 99.10% | 99.20% | 57.70% | 78.00% | 88.80% |
|  |  | BO |  | 24% | 13% | 7% | 50% | 22% | 13% | 82% | 70% | 50% |
|  |  | 𝜆 |  | 1.05 |  |  | 1.45 |  |  | 3.2 |  |  |
|  | Additional comments: No 256QAM, no LDPC code, 2Tx2Rx, 24 dBm TX power for both LAA and WiFi, no licensed carrier for LAA, LAA max. duration =4 ms, RTS/CTS enabled for WiFi |  |  |  |  |  |  |  |  |  |  |  |
| Huawei, HiSilicon, | Cat.3 | UPT CDF [Mbps] | 5% | 2.67 | 3.06 | 13.35 | 0.83 | 1.65 | 4.98 | 0.10 | 0.15 | 0.21 |
|  |  |  | 50% | 20.37 | 27.01 | 26.34 | 5.99 | 11.75 | 17.96 | 0.77 | 1.39 | 7.27 |
|  |  |  | 95% | 33.33 | 41.34 | 37.94 | 21.40 | 30.80 | 34.39 | 11.53 | 14.61 | 24.78 |
|  |  |  | Mean | 19.17 | 25.18 | 26.68 | 8.77 | 13.55 | 18.44 | 2.64 | 3.63 | 9.96 |
|  |  | Delay CDF [s] | 5% | 0.12 | 0.10 | 0.10 | 0.18 | 0.12 | 0.11 | 0.32 | 0.26 | 0.16 |
|  |  |  | 50% | 0.19 | 0.14 | 0.15 | 0.62 | 0.33 | 0.22 | 3.67 | 2.21 | 0.51 |
|  |  |  | 95% | 1.15 | 0.97 | 0.29 | 3.27 | 1.59 | 0.70 | 7.57 | 7.17 | 6.20 |
|  |  |  | Mean | 0.36 | 0.29 | 0.17 | 0.98 | 0.60 | 0.30 | 3.91 | 3.02 | 1.18 |
|  |  | 𝜌 |  | 1 | 0.97 | 1 | 0.97 | 1 | 1 | 0.75 | 0.84 | 0.96 |
|  |  | BO |  | 0.23 | 0.11 | 0.13 | 0.41 | 0.26 | 0.24 | 0.71 | 0.61 | 0.52 |
|  |  | 𝜆 |  | 0.4 |  |  | 0.6 |  |  | 1.0 |  |  |
|  | Additional comments: R1-150042, No 256QAM, no LDPC code, LAA max. duration =4ms, WiFi DL only, LAA on unlicensed carrier only. LBT scheme follows the Option B from ETSI EN 301 893 v1.8.0 |  |  |  |  |  |  |  |  |  |  |  |

| Samsung | 3 | UPT CDF [Mbps] | 5% | 1.9 | 4.5 | 25.3 | 0.9 | 2.2 | 24.1 | 0.7 | 1.2 | 19.2 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 50% | 36.8 | 50.6 | 128.1 | 22.3 | 41.8 | 117.2 | 8.1 | 28.1 | 100.8 |
|  |  |  | 95% | 90.7 | 102.2 | 155.8 | 77.0 | 95.1 | 148.1 | 56.6 | 85.2 | 139.3 |
|  |  |  | Mean | 34.5 | 44.3 | 116.1 | 24.7 | 38.4 | 107.5 | 13.3 | 28.4 | 94.5 |
|  |  | Delay CDF [s] | 5% | 0.034 | 0.034 | 0.022 | 0.034 | 0.034 | 0.022 | 0.047 | 0.034 | 0.022 |
|  |  |  | 50% | 0.145 | 0.095 | 0.034 | 0.246 | 0.121 | 0.038 | 0.603 | 0.200 | 0.044 |
|  |  |  | 95% | 2.142 | 0.990 | 0.156 | 3.894 | 1.610 | 0.177 | 7.059 | 3.453 | 0.226 |
|  |  |  | Mean | 0.662 | 0.368 | 0.059 | 1.061 | 0.539 | 0.067 | 2.038 | 0.938 | 0.081 |
|  |  | 𝜌 |  | 0.94 | 0.98 | 0.99 | 0.91 | 0.96 | 0.99 | 0.8 | 0.93 | 1.00 |
|  |  | BO |  | 0.31 | 0.21 | 0.09 | 0.45 | 0.30 | 0.12 | 0.67 | 0.45 | 0.18 |
|  |  | 𝜆 |  | 0.4 |  |  | 0.6 |  |  | 0.8 |  |  |
|  | Additional comments: LAA with licensed carrier, TXOP: 10ms(LAA), 4ms(WiFi), 2Tx2Rx, BCC, no 256QAM, CCA slot: 20us. For LBE, CCA (20us) is always followed by ECCA |  |  |  |  |  |  |  |  |  |  |  |
| CMCC | LBT Cat.3 | UPT CDF [Mbps] | 5% | 3.02 | 2.32 | 1.00 | 0.60 | 0.51 | 0.77 | 0.22 | 0.19 | 0.18 |
|  |  |  | 50% | 25.31 | 26.07 | 10.00 | 8.89 | 7.56 | 4.99 | 2.44 | 2.67 | 2.79 |
|  |  |  | 95% | 59.64 | 61.11 | 44.27 | 40.22 | 45.80 | 29.44 | 22.52 | 28.83 | 20.62 |
|  |  |  | Mean | 34.25 | 34.56 | 38.75 | 12.74 | 13.02 | 16.41 | 3.84 | 4.39 | 5.77 |
|  |  | Delay CDF [s] | 5% | 0.07 | 0.07 | 0.10 | 0.10 | 0.09 | 0.13 | 0.17 | 0.13 | 0.17 |
|  |  |  | 50% | 0.16 | 0.15 | 0.45 | 0.41 | 0.48 | 0.79 | 1.36 | 1.23 | 1.17 |
|  |  |  | 95% | 1.24 | 1.62 | 4.07 | 4.42 | 5.51 | 3.59 | 8.93 | 9.87 | 5.91 |
|  |  |  | Mean | 0.26 | 0.31 | 0.29 | 1.03 | 1.24 | 0.73 | 2.55 | 2.61 | 1.38 |
|  |  | 𝜌 |  | - | - | - | - | - | - | - | - | - |
|  |  | BO |  | 9% | 10% | 11% | 38% | 40% | 24% | 69% | 67% | 40% |
|  |  | 𝜆 |  | 0.2 |  |  | 0.4 |  |  | 0.6 |  |  |
|  | Additional comments: 𝜆 is defined as packet arrival rate with unit in #/s/UE FTP model 3, maximum transmission duration is 4ms for both LAA and WIFI. For LAA, load based LBT as defined in R1-150989 is used with q=10 and 20us CCA time slot. For LAA, traffic is served on the unlicensed carrier only. For WIFI, RTS/CTS, 256QAM, LDPC codes are not modeled. CCA mode 1: WIFI has dual CCA thresholds with CCA-ED threshold = -62dBm, and CCA-CS threshold = -82dBm |  |  |  |  |  |  |  |  |  |  |  |

| LG | 2 | UPT CDF [Mbps] | 5% | 12.91 | 26.25 | 31.61 | 0.737 | 7.667 | 6.046 | 0.052 | 0.694 | 0.728 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 50% | 40.15 | 48.17 | 50.25 | 20.72 | 38.12 | 35.18 | 8.746 | 25.80 | 19.04 |
|  |  |  | 95% | 52.74 | 56.91 | 62.51 | 44.68 | 51.53 | 53.18 | 38.97 | 46.39 | 45.79 |
|  |  |  | Mean | 37.29 | 46.20 | 48.33 | 21.53 | 34.99 | 33.13 | 13.03 | 24.27 | 20.32 |
|  |  | Delay CDF [s] | 5% | 0.086 | 0.076 | 0.071 | 0.110 | 0.088 | 0.094 | 0.129 | 0.103 | 0.123 |
|  |  |  | 50% | 0.134 | 0.100 | 0.109 | 0.312 | 0.144 | 0.198 | 0.835 | 0.238 | 0.461 |
|  |  |  | 95% | 0.625 | 0.292 | 0.511 | 12.26 | 1.260 | 2.036 | 21.23 | 11.75 | 12.26 |
|  |  |  | Mean | 0.319 | 0.127 | 0.382 | 1.810 | 0.433 | 0.963 | 4.593 | 1.633 | 2.616 |
|  |  | 𝜌 |  | 0.9949 | 0.9990 | 0.9888 | 0.9370 | 0.9918 | 0.9680 | 0.8280 | 0.9417 | 0.9060 |
|  |  | BO |  | 0.1410 | 0.0948 | 0.1481 | 0.3720 | 0.2202 | 0.3276 | 0.5542 | 0.3834 | 0.5191 |
|  |  | 𝜆 |  | 0.35 |  |  | 0.55 |  |  | 0.75 |  |  |
|  | Additional comments: LAA without licensed carrier, no 256 QAM, LDPC, 1Tx2Rx, TXOP 4ms CCA threshold: -62dBm for CCA-ED of LAA LBT algorithm: FBE procedure as defined in ETSI EN 301 893 V1.8.0 |  |  |  |  |  |  |  |  |  |  |  |
| LG | 3 | UPT CDF [Mbps] | 5% | 12.91 | 22.70 | 28.02 | 0.737 | 4.223 | 5.455 | 0.052 | 0.158 | 0.356 |
|  |  |  | 50% | 40.15 | 43.64 | 50.81 | 20.72 | 27.81 | 35.44 | 8.746 | 12.15 | 20.18 |
|  |  |  | 95% | 52.74 | 54.73 | 62.92 | 44.68 | 45.15 | 55.12 | 38.97 | 39.90 | 48.55 |
|  |  |  | Mean | 37.29 | 42.03 | 48.80 | 21.53 | 27.05 | 32.99 | 13.03 | 15.29 | 21.47 |
|  |  | Delay CDF [s] | 5% | 0.086 | 0.082 | 0.071 | 0.110 | 0.106 | 0.086 | 0.129 | 0.127 | 0.105 |
|  |  |  | 50% | 0.134 | 0.119 | 0.101 | 0.312 | 0.237 | 0.168 | 0.835 | 0.604 | 0.341 |
|  |  |  | 95% | 0.625 | 0.375 | 0.549 | 12.26 | 3.264 | 2.115 | 21.23 | 17.09 | 12.58 |
|  |  |  | Mean | 0.319 | 0.156 | 0.468 | 1.810 | 0.896 | 0.987 | 4.593 | 3.152 | 2.627 |
|  |  | 𝜌 |  | 0.9949 | 0.9986 | 0.9836 | 0.9370 | 0.9797 | 0.9586 | 0.8280 | 0.8849 | 0.9083 |
|  |  | BO |  | 0.1410 | 0.1105 | 0.1389 | 0.3720 | 0.2926 | 0.3177 | 0.5542 | 0.5120 | 0.5344 |
|  |  | 𝜆 |  | 0.35 |  |  | 0.55 |  |  | 0.75 |  |  |
|  | Additional comments: LAA without licensed carrier, no 256 QAM, LDPC, 1Tx2Rx, TXOP 4ms CCA threshold: -62dBm for CCA-ED of LAA LBT algorithm: LBE procedure with a fixed q(=10) for the contention window as defined in ETSI EN 301 893 V1.8.0 |  |  |  |  |  |  |  |  |  |  |  |
| LG | 4 | UPT CDF [Mbps] | 5% | 12.91 | 27.79 | 28.04 | 0.737 | 7.348 | 3.228 | 0.052 | 0.749 | 0.145 |
|  |  |  | 50% | 40.15 | 48.94 | 49.35 | 20.72 | 38.60 | 33.44 | 8.746 | 25.33 | 14.99 |
|  |  |  | 95% | 52.74 | 57.56 | 62.29 | 44.68 | 52.65 | 53.10 | 38.97 | 48.14 | 42.45 |
|  |  |  | Mean | 37.29 | 46.93 | 47.70 | 21.53 | 35.74 | 31.31 | 13.03 | 24.89 | 17.48 |
|  |  | Delay CDF [s] | 5% | 0.086 | 0.075 | 0.073 | 0.110 | 0.087 | 0.103 | 0.129 | 0.098 | 0.147 |
|  |  |  | 50% | 0.134 | 0.098 | 0.117 | 0.312 | 0.141 | 0.246 | 0.835 | 0.259 | 0.640 |
|  |  |  | 95% | 0.625 | 0.291 | 0.462 | 12.26 | 1.182 | 4.692 | 21.23 | 9.047 | 15.52 |
|  |  |  | Mean | 0.319 | 0.125 | 0.381 | 1.810 | 0.532 | 1.141 | 4.593 | 1.414 | 3.678 |
|  |  | 𝜌 |  | 0.9949 | 0.9989 | 0.9874 | 0.9370 | 0.9890 | 0.9566 | 0.8280 | 0.9506 | 0.8592 |
|  |  | BO |  | 0.1410 | 0.0934 | 0.1469 | 0.3720 | 0.2189 | 0.3511 | 0.5542 | 0.3832 | 0.5921 |
|  |  | 𝜆 |  | 0.35 |  |  | 0.55 |  |  | 0.75 |  |  |
|  | Additional comments: LAA without licensed carrier, no 256 QAM, LDPC, 1Tx2Rx, TXOP 4ms CCA threshold: -62dBm for CCA-ED of LAA LBT algorithm: LBE procedure Op A with a variable q for the contention window as defined in ETSI EN 301 893 V1.8.0 |  |  |  |  |  |  |  |  |  |  |  |

| Intel | 3 | UPT CDF [Mbps] | 5% | 3 | 15 | 8 | 1 | 10 | 6 | 1 | 4 | 8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 50% | 12 | 32 | 100 | 7 | 25 | 61 | 4 | 16 | 35 |
|  |  |  | 95% | 32 | 70 | 163 | 25 | 53 | 156 | 18 | 38 | 107 |
|  |  |  | Mean | 14 | 36 | 90 | 10 | 28 | 65 | 6 | 18 | 46 |
|  |  | Delay CDF [s] | 5% | 0.13 | 0.06 | 0.02 | 0.16 | 0.08 | 0.03 | 0.22 | 0.11 | 0.04 |
|  |  |  | 50% | 0.35 | 0.13 | 0.04 | 0.59 | 0.17 | 0.05 | 1.30 | 0.29 | 0.09 |
|  |  |  | 95% | 2.10 | 0.30 | 0.48 | 4.16 | 0.43 | 0.43 | 8.57 | 1.42 | 0.32 |
|  |  |  | Mean | 0.62 | 0.15 | 0.10 | 1.27 | 0.20 | 0.11 | 2.64 | 0.49 | 0.13 |
|  |  | 𝜌 |  | 1.0 | 0.98 | 1.00 | 0.98 | 0.98 | 1.0 | 0.90 | 1.0 | 1.0 |
|  |  | BO |  | 0.30 | 0.11 | 0.12 | 0.47 | 0.17 | 0.17 | 0.69 | 0.35 | 0.22 |
|  |  | 𝜆 |  | 0.35 |  |  | 0.41 |  |  | 0.55 |  |  |
|  | Additional comments: results with licensed carrier. 2Tx2Rx, TXOP 4ms, and LBT algorithm based on R1-1500089. |  |  |  |  |  |  |  |  |  |  |  |

| NEC | 3 | UPT CDF [Mbps] | 5% | - | 2.61 | 6.93 | - | 2.22 | 2.68 | - | 2.12 | 2.94 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 50% | - | 20.20 | 28.37 | - | 11.40 | 14.08 | - | 8.89 | 11.73 |
|  |  |  | 95% | - | 47.06 | 56.34 | - | 42.55 | 54.05 | - | 33.90 | 52.63 |
|  |  |  | Mean | - | 22.58 | 31.20 | - | 16.34 | 20.03 | - | 12.26 | 16.62 |
|  |  | Delay CDF [s] | 5% | - | 0.09 | 0.07 | - | 0.09 | 0.08 | - | 0.12 | 0.08 |
|  |  |  | 50% | - | 0.19 | 0.14 | - | 0.35 | 0.28 | - | 0.45 | 0.34 |
|  |  |  | 95% | - | 1.54 | 0.58 | - | 1.80 | 1.49 | - | 1.89 | 1.36 |
|  |  |  | Mean | - | 0.39 | 0.20 | - | 0.55 | 0.45 | - | 0.67 | 0.48 |
|  |  | 𝜌 |  | - | 0.97 | 0.98 | - | 0.86 | 0.87 | - | 0.70 | 0.81 |
|  |  | BO |  | - | 0.25 | 0.13 | - | 0.41 | 0.35 | - | 0.61 | 0.47 |
|  |  | 𝜆 |  | 0.4 |  |  | 0.8 |  |  | 1.2 |  |  |
|  | Additional comments: FTP model 3 is used and 𝜆 is defined as packet arrival rate per UE per second; Maximum channel occupancy time is 4 ms for both WiFi and LAA; Traffic is served on unlicensed carrier only for both WiFi and LAA; For WiFi, RTS/CTS, 256QAM, LDPC are not used; CCA model: for WiFi, intra-RAT preamble detection with CCA threshold -82dbm, and inter-RAT energy detection with CCA threshold -62dBm; for LAA, inter-RAT energy detection with CCA threshold -62dBm. |  |  |  |  |  |  |  |  |  |  |  |

| KDDI | 4 |  | UPT CDF [Mbps] |  | 5% | 4.5 |  | 1 |  | 1 |  | 0 | 0 | 0 | 0 | 0 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | 50% | 12.5 |  | 8 |  | 7.5 |  | 5.5 | 0 | 2 | 1.5 | 0 | 0.5 |
|  |  |  |  |  | 95% | 52 |  | 51.5 |  | 47.5 |  | 32 | 18 | 13.5 | 10 | 2.5 | 6.5 |
|  |  |  |  |  | Mean | 17.26 |  | 13.51 |  | 11.00 |  | 9.74 | 3.13 | 3.96 | 2.78 | 0.74 | 1.75 |
|  |  |  | Delay CDF [s] |  | 5% | 0 |  | 0 |  | 0 |  | 0 | 0 | 0 | 0 | 0 | 0 |
|  |  |  |  |  | 50% | 0 |  | 0 |  | 0 |  | 0.5 | 1.4 | 1.2 | 2.3 | 2.6 | 1.9 |
|  |  |  |  |  | 95% | 0.8 |  | 1.5 |  | 1.6 |  | 2.4 | 2.5 | 2.5 | 2.8 | 2.8 | 2.8 |
|  |  |  |  |  | Mean | 0.18 |  | 0.29 |  | 0.31 |  | 0.82 | 1.18 | 1.15 | 1.91 | 2.26 | 1.75 |
|  |  |  | 𝜌 |  |  | 1 |  | 0.95 |  | 0.95 |  | 0.91 | 0.38 | 0.69 | 0.65 | 0.23 | 0.47 |
|  |  |  | BO |  |  | 0.17 |  | 0.28 |  | 0.29 |  | 0.40 | 0.61 | 0.54 | 0.71 | 0.84 | 0.69 |
|  |  |  | 𝜆 |  |  | 0.5 |  |  |  |  |  | 1.35 |  |  | 4 |  |  |
|  | Additional comments: [LAA] LAA without licensed carrier, TxOP: 4ms, SISO, no 256QAM, CCA slot: 72us, CCA-ED: -62dB [Wi-Fi] TxOP: 0, no 256QAM, CCA-CS:-82dBm, CCA-ED: -62dBm |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| KDDI | 4 | UPT CDF [Mbps] |  | 5% |  |  | 4.5 |  | 5.5 |  | 2 | 0 | 3.5 | 0 | 0 | 0.5 | 0 |
|  |  |  |  | 50% |  |  | 12.5 |  | 17.5 |  | 5.5 | 5.5 | 15.5 | 1 | 1.5 | 3.5 | 0 |
|  |  |  |  | 95% |  |  | 52 |  | 53.5 |  | 28.5 | 32 | 49.5 | 9.5 | 10 | 26.5 | 1.5 |
|  |  |  |  | Mean |  |  | 17.26 |  | 22.64 |  | 9.74 | 9.74 | 19.63 | 2.65 | 2.78 | 7.49 | 0.41 |
|  |  | Delay CDF [s] |  | 5% |  |  | 0 |  | 0 |  | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
|  |  |  |  | 50% |  |  | 0 |  | 0 |  | 0 | 0.5 | 0.2 | 1.6 | 2.3 | 1.3 | 2.2 |
|  |  |  |  | 95% |  |  | 0.8 |  | 0.5 |  | 1.5 | 2.4 | 1.3 | 2.5 | 2.8 | 2.7 | 2.8 |
|  |  |  |  | Mean |  |  | 0.18 |  | 0.13 |  | 0.34 | 0.82 | 0.40 | 1.24 | 1.91 | 1.36 | 1.55 |
|  |  | 𝜌 |  |  |  |  | 1 |  | 1.00 |  | 0.99 | 0.91 | 1.00 | 0.51 | 0.65 | 0.94 | 0.16 |
|  |  | BO |  |  |  |  | 0.17 |  | 0.12 |  | 0.31 | 0.40 | 0.20 | 0.63 | 0.71 | 0.51 | 0.86 |
|  |  | 𝜆 |  |  |  |  | 0.5 |  |  |  |  | 1.35 |  |  | 4 |  |  |
|  | Additional comments: [LAA] LAA without licensed carrier, TxOP: 4ms, SISO, no 256QAM, CCA slot: 72us, CCA-ED: -82dB [Wi-Fi] TxOP: 0, no 256QAM, CCA-CS:-82dBm, CCA-ED: -62dBm |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 8: Outdoor deployment for LAA and LAA coexistence case with one shared unlicensed carrier and FTP traffic

| Tdoc / Company | LAA LBT cat. | Reported parameters |  | Low load BO range: 10%~25% |  | Medium load BO range: 35%~50% |  | High load BO range: above 55% |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | LAA opt. 1 | LAA opt. 2 | LAA opt. 1 | LAA opt. 2 | LAA opt. 1 | LAA opt. 2 |
| ALU, ASB | LBE Cat 3 | UPT CDF [Mbps] | 5% | 27.67 | 22.00 | 1.82 | 1.90 | 0.08 | 0.42 |
|  |  |  | 50% | 72.79 | 72.29 | 29.44 | 30.10 | 6.46 | 9.44 |
|  |  |  | 95% | 100.40 | 100.44 | 67.68 | 70.27 | 42.03 | 38.45 |
|  |  |  | Mean | 69.64 | 69.60 | 31.67 | 32.10 | 12.08 | 13.98 |
|  |  | Delay CDF [s] | 5% | 0.036 | 0.036 | 0.037 | 0.037 | 0.055 | 0.057 |
|  |  |  | 50% | 0.055 | 0.055 | 0.146 | 0.145 | 0.382 | 0.344 |
|  |  |  | 95% | 0.214 | 0.224 | 1.861 | 1.680 | 4.041 | 3.225 |
|  |  |  | Mean | 0.109 | 0.097 | 0.446 | 0.390 | 0.905 | 0.802 |
|  |  | 𝜌 |  | 0.84 | 0.87 | 0.64 | 0.66 | 0.41 | 0.51 |
|  |  | BO |  | 0.10 | 0.11 | 0.44 | 0.42 | 0.78 | 0.75 |
|  |  | 𝜆 |  | 0.5 |  | 0.7 |  | 1.0 |  |
|  | Additional comments: LBE, q Value:32, Maximum Occupancy Time : 10ms, LAA without licensed carrier CCA threshold: -73 dBm/MHz + 23 - PH, PH specified in dBm EIRP for CCA-ED of LAA, -62 dBm for CCA-ED of Wi-Fi; |  |  |  |  |  |  |  |  |
| ALU, ASB | FBE Cat 2 | UPT CDF [Mbps] | 5% | 18.68 | 15.38 | 7.28 | 8.83 | 2.76 | 3.69 |
|  |  |  | 50% | 72.29 | 71.53 | 38.59 | 37.85 | 27.99 | 29.21 |
|  |  |  | 95% | 94.91 | 97.12 | 73.55 | 73.70 | 70.72 | 71.46 |
|  |  |  | Mean | 67.38 | 67.65 | 40.91 | 39.67 | 31.94 | 32.86 |
|  |  | Delay CDF [s] | 5% | 0.037 | 0.037 | 0.037 | 0.037 | 0.037 | 0.037 |
|  |  |  | 50% | 0.056 | 0.054 | 0.117 | 0.126 | 0.182 | 0.168 |
|  |  |  | 95% | 0.317 | 0.629 | 1.584 | 1.854 | 3.155 | 2.631 |
|  |  |  | Mean | 0.140 | 0.257 | 0.357 | 0.392 | 0.699 | 0.576 |
|  |  | 𝜌 |  | 0.75 | 0.79 | 0.65 | 0.69 | 0.55 | 0.61 |
|  |  | BO |  | 0.16 | 0.16 | 0.39 | 0.42 | 0.56 | 0.56 |
|  |  | 𝜆 |  | 0.6 |  | 0.9 |  | 1.0 |  |
|  | Additional comments: FBE frame period: 10ms(Maximum Occupancy Time= 9.5ms, idle time = 0.5ms), LAA with licensed carrier CCA threshold: -73 dBm/MHz + 23 - PH, PH specified in dBm EIRP for CCA-ED of LAA, -62 dBm for CCA-ED of Wi-Fi; |  |  |  |  |  |  |  |  |
| ZTE | 3 | UPT CDF [Mbps] | 5% | 34.464 | 34.258 | 4 | 3 | 2.043 | 1.345 |
|  |  |  | 50% | 76.923 | 77.633 | 35 | 30 | 23.153 | 20.45 |
|  |  |  | 95% | 108.207 | 113.543 | 88 | 101 | 83.241 | 96.32 |
|  |  |  | Mean | 75.662 | 78.189 | 40 | 39 | 30.962 | 30.646 |
|  |  | Delay CDF [s] | 5% | 0.032 | 0.032 | 0.032 | 0.034 | 0.035 | 0.036 |
|  |  |  | 50% | 0.054 | 0.057 | 0.119 | 0.149 | 0.146 | 0.178 |
|  |  |  | 95% | 0.254 | 0.27 | 2.059 | 3.354 | 2.057 | 4.384 |
|  |  |  | Mean | 0.084 | 0.093 | 0.429 | 0.571 | 0.484 | 0.739 |
|  |  | 𝜌 |  | 99.90% | 99.70% | 94.80% | 93.40% | 87.90% | 83. 8% |
|  |  | BO |  | 18% | 20% | 47% | 50% | 57% | 59% |
|  |  | 𝜆 |  | 2.5 |  | 3.75 |  | 4.5 |  |
|  | Additional comments: No 256QAM, no LDPC code, 2Tx2Rx, 24 dBm TX power for LAA, no licensed carrier for LAA, LAA max. duration =4 ms |  |  |  |  |  |  |  |  |
| Ericsson | 3 | UPT CDF [Mbps] | 5% | 33.28 | 26.89 | 11.32 | 9.69 | 1.01 | 1.28 |
|  |  |  | 50% | 67.8 | 67.81 | 37.76 | 37.84 | 17.06 | 19.49 |
|  |  |  | 95% | 94.51 | 92.84 | 74.8 | 71.64 | 54.68 | 55.87 |
|  |  |  | Mean | 67.83 | 67.05 | 40.94 | 40.2 | 21.86 | 23.7 |
|  |  | Delay CDF [s] | 5% | 0.044 | 0.047 | 0.057 | 0.064 | 0.083 | 0.082 |
|  |  |  | 50% | 0.132 | 0.132 | 0.254 | 0.257 | 0.487 | 0.413 |
|  |  |  | 95% | 0.772 | 0.652 | 1.798 | 1.472 | 2.709 | 2.105 |
|  |  |  | Mean | 0.252 | 0.228 | 0.545 | 0.486 | 0.884 | 0.718 |
|  |  | 𝜌 |  | 97 | 97 | 93 | 93 | 82 | 86 |
|  |  | BO |  | 20 | 20 | 40 | 40 | 60 | 56 |
|  |  | 𝜆 |  | 0.649 files/s |  | 0.888 files/s |  | 1.121 files/s |  |
|  | Additional comments: |  |  |  |  |  |  |  |  |
| Huawei, HiSilicon, | Cat.3 | UPT CDF [Mbps] | 5% | 13.13 | 13.13 | 1.58 | 1.58 | 0.03 | 0.03 |
|  |  |  | 50% | 28.30 | 28.30 | 10.57 | 10.57 | 4.24 | 4.24 |
|  |  |  | 95% | 39.93 | 39.93 | 27.12 | 27.12 | 17.28 | 17.28 |
|  |  |  | Mean | 28.19 | 28.19 | 12.42 | 12.42 | 5.67 | 5.67 |
|  |  | Delay CDF [s] | 5% | 0.10 | 0.10 | 0.15 | 0.15 | 0.23 | 0.23 |
|  |  |  | 50% | 0.14 | 0.14 | 0.37 | 0.37 | 0.90 | 0.90 |
|  |  |  | 95% | 0.29 | 0.29 | 2.06 | 2.06 | 7.57 | 7.57 |
|  |  |  | Mean | 0.16 | 0.16 | 0.62 | 0.62 | 2.32 | 2.32 |
|  |  | 𝜌 |  | 1 | 1 | 1 | 1 | 0.84 | 0.84 |
|  |  | BO |  | 0.12 | 0.12 | 0.41 | 0.41 | 0.69 | 0.69 |
|  |  | 𝜆 |  | 0.4 |  | 0.8 |  | 1.2 |  |
|  | Additional comments: R1-150043, No 256QAM, LAA max. duration =4ms, LAA on unlicensed carrier only. LBT scheme follows the Option B from ETSI EN 301 893 v1.8.0. The performance metrics is not independently calculated per operator in LAA and LAA coexistence case. |  |  |  |  |  |  |  |  |

| Samsung | 3 | UPT CDF [Mbps] | 5% | 16.8 | 17.0 | 7.1 | 7.6 | 2.8 | 2.5 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 50% | 60.8 | 61.5 | 42.5 | 41.4 | 26.9 | 25.6 |
|  |  |  | 95% | 106.0 | 107.3 | 86.4 | 87.6 | 70.3 | 69.2 |
|  |  |  | Mean | 59.6 | 59.5 | 44.2 | 43.2 | 31.6 | 30.4 |
|  |  | Delay CDF [s] | 5% | 0.023 | 0.023 | 0.025 | 0.025 | 0.028 | 0.028 |
|  |  |  | 50% | 0.071 | 0.071 | 0.111 | 0.116 | 0.180 | 0.191 |
|  |  |  | 95% | 0.371 | 0.389 | 1.039 | 1.107 | 2.495 | 2.474 |
|  |  |  | Mean | 0.163 | 0.184 | 0.395 | 0.421 | 0.745 | 0.787 |
|  |  | 𝜌 |  | 0.99 | 0.99 | 0.98 | 0.97 | 0.95 | 0.93 |
|  |  | BO |  | 0.22 | 0.23 | 0.40 | 0.40 | 0.57 | 0.58 |
|  |  | 𝜆 |  | 0.5 |  | 0.8 |  | 1.2 |  |
|  | Additional comments: LAA with licensed carrier, TXOP: 10ms(LAA), 4ms(WiFi), 2Tx2Rx, BCC, no 256QAM, CCA slot: 20us. For LBE, CCA (20us) is always followed by ECCA |  |  |  |  |  |  |  |  |
| LG | 2 | UPT CDF [Mbps] | 5% | 31.90 | 32.15 | 6.484 | 7.984 | 1.151 | 1.561 |
|  |  |  | 50% | 51.96 | 51.97 | 32.49 | 32.25 | 20.53 | 20.09 |
|  |  |  | 95% | 61.18 | 60.94 | 50.92 | 50.40 | 43.83 | 43.84 |
|  |  |  | Mean | 50.02 | 50.02 | 31.42 | 31.21 | 21.09 | 21.26 |
|  |  | Delay CDF [s] | 5% | 0.075 | 0.076 | 0.111 | 0.113 | 0.154 | 0.158 |
|  |  |  | 50% | 0.100 | 0.101 | 0.243 | 0.244 | 0.715 | 0.718 |
|  |  |  | 95% | 0.361 | 0.348 | 2.214 | 2.411 | 15.02 | 17.53 |
|  |  |  | Mean | 0.190 | 0.194 | 1.154 | 0.915 | 3.456 | 3.897 |
|  |  | 𝜌 |  | 0.9889 | 0.9924 | 0.9773 | 0.9829 | 0.9479 | 0.9573 |
|  |  | BO |  | 0.1623 | 0.1547 | 0.4063 | 0.4062 | 0.5928 | 0.5978 |
|  |  | 𝜆 |  | 0.5 |  | 0.9 |  | 1.1 |  |
|  | Additional comments: LAA without licensed carrier, no 256 QAM, 1Tx2Rx, TXOP 4ms CCA threshold: -62dBm for CCA-ED of LAA LBT algorithm: FBE procedure as defined in ETSI EN 301 893 V1.8.0 |  |  |  |  |  |  |  |  |
| LG | 3 | UPT CDF [Mbps] | 5% | 19.23 | 20.45 | 1.173 | 2.023 | 0.086 | 0.089 |
|  |  |  | 50% | 44.43 | 45.78 | 29.74 | 31.62 | 14.20 | 17.40 |
|  |  |  | 95% | 57.49 | 57.73 | 51.04 | 51.87 | 45.82 | 47.76 |
|  |  |  | Mean | 42.58 | 43.55 | 28.16 | 29.63 | 17.45 | 19.01 |
|  |  | Delay CDF [s] | 5% | 0.082 | 0.081 | 0.097 | 0.095 | 0.109 | 0.103 |
|  |  |  | 50% | 0.129 | 0.121 | 0.239 | 0.211 | 0.530 | 0.427 |
|  |  |  | 95% | 0.617 | 0.661 | 4.474 | 4.417 | 54.98 | 53.48 |
|  |  |  | Mean | 0.364 | 0.371 | 1.849 | 1.821 | 8.090 | 7.793 |
|  |  | 𝜌 |  | 0.9796 | 0.9848 | 0.9517 | 0.9617 | 0.8829 | 0.8926 |
|  |  | BO |  | 0.2191 | 0.2055 | 0.4093 | 0.3941 | 0.5936 | 0.5721 |
|  |  | 𝜆 |  | 0.5 |  | 0.7 |  | 0.9 |  |
|  | Additional comments: LAA without licensed carrier, no 256 QAM, 1Tx2Rx, TXOP 4ms CCA threshold: -62dBm for CCA-ED of LAA LBT algorithm: LBE procedure with a fixed q(=10) for the contention window as defined in ETSI EN 301 893 V1.8.0 |  |  |  |  |  |  |  |  |
| LG | 4 | UPT CDF [Mbps] | 5% | 27.22 | 27.58 | 2.6884 | 3.043 | 0.092 | 0.101 |
|  |  |  | 50% | 49.24 | 49.83 | 30.78 | 31.65 | 11.80 | 14.36 |
|  |  |  | 95% | 59.18 | 59.86 | 50.42 | 51.19 | 43.37 | 43.82 |
|  |  |  | Mean | 47.28 | 47.90 | 29.20 | 30.28 | 15.40 | 17.11 |
|  |  | Delay CDF [s] | 5% | 0.081 | 0.080 | 0.106 | 0.103 | 0.128 | 0.129 |
|  |  |  | 50% | 0.116 | 0.113 | 0.258 | 0.242 | 0.816 | 0.657 |
|  |  |  | 95% | 0.448 | 0.397 | 3.657 | 4.380 | 57.74 | 58.06 |
|  |  |  | Mean | 0.298 | 0.290 | 1.703 | 1.774 | 10.68 | 9.248 |
|  |  | 𝜌 |  | 0.9872 | 0.9907 | 0.9725 | 0.9746 | 0.8804 | 0.9002 |
|  |  | BO |  | 0.1834 | 0.1737 | 0.4039 | 0.3934 | 0.6350 | 0.6095 |
|  |  | 𝜆 |  | 0.5 |  | 0.8 |  | 1.1 |  |
|  | Additional comments: LAA without licensed carrier, no 256 QAM, 1Tx2Rx, TXOP 4ms CCA threshold: -62dBm for CCA-ED of LAA LBT algorithm: LBE procedure Op A with a variable q for the contention window as defined in ETSI EN 301 893 V1.8.0 |  |  |  |  |  |  |  |  |
| Intel | 3 | UPT CDF [Mbps] | 5% | 13 | 10 | 17 | 13 | 10 | 8 |
|  |  |  | 50% | 67 | 59 | 48 | 44 | 23 | 22 |
|  |  |  | 95% | 155 | 155 | 122 | 110 | 45 | 45 |
|  |  |  | Mean | 76 | 68 | 55 | 51 | 24 | 23 |
|  |  | Delay CDF [s] | 5% | 0.03 | 0.03 | 0.03 | 0.03 | 0.09 | 0.09 |
|  |  |  | 50% | 0.04 | 0.05 | 0.06 | 0.07 | 0.20 | 0.19 |
|  |  |  | 95% | 0.21 | 0.19 | 0.14 | 0.16 | 0.91 | 1.91 |
|  |  |  | Mean | 0.07 | 0.07 | 0.07 | 0.08 | 0.30 | 0.52 |
|  |  | 𝜌 |  | 1.00 | 1.00 | 1.00 | 1.00 | 0.99 | 0.96 |
|  |  | BO |  | 0.22 | 0.24 | 0.26 | 0.31 | 0.73 | 0.73 |
|  |  | 𝜆 |  | 0.7 |  | 0.95 |  | 2.27 |  |
|  | Additional comments: results with licensed carrier. 2Tx2Rx, TXOP 4ms, and LBT algorithm based on R1-1500089. |  |  |  |  |  |  |  |  |

| NEC | 3 | UPT CDF [Mbps] | 5% | 4.41 | 8.97 | 2.60 | 2.34 | 1.84 | 1.68 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 50% | 26.32 | 35.71 | 10.93 | 12.35 | 7.86 | 7.02 |
|  |  |  | 95% | 65.57 | 65.57 | 52.63 | 54.05 | 45.45 | 33.90 |
|  |  |  | Mean | 32.33 | 38.53 | 16.37 | 18.42 | 12.06 | 10.71 |
|  |  | Delay CDF [s] | 5% | 0.06 | 0.06 | 0.08 | 0.08 | 0.09 | 0.12 |
|  |  |  | 50% | 0.15 | 0.11 | 0.37 | 0.32 | 0.51 | 0.57 |
|  |  |  | 95% | 0.96 | 0.45 | 0.95 | 1.71 | 2.17 | 2.39 |
|  |  |  | Mean | 0.27 | 0.16 | 0.53 | 0.53 | 0.75 | 0.82 |
|  |  | 𝜌 |  | 0.96 | 0.99 | 0.87 | 0.83 | 0.72 | 0.64 |
|  |  | BO |  | 0.16 | 0.10 | 0.44 | 0.40 | 0.67 | 0.69 |
|  |  | 𝜆 |  | 0.4 |  | 1 |  | 1.4 |  |
|  | Additional comments: FTP model 3 is used and 𝜆 is defined as packet arrival rate per UE per second; Maximum channel occupancy time is 4 ms for LAA; Traffic is served on unlicensed carrier only for LAA; CCA model: for LAA, inter-operator energy detection with CCA threshold -62dBm. |  |  |  |  |  |  |  |  |

Table 9: Outdoor deployment for Wi-Fi and LAA coexistence case with one shared unlicensed carrier and Mixed traffic

| Tdoc / Company | LAA LBT cat. | Reported parameters |  | Low load BO range for Wi-Fi in Step 1: 10%~25% |  |  | Medium load BO range for Wi-Fi in Step 1: 35%~50% |  |  | High load BO range for Wi-Fi in Step 1: above 55% |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Wi-Fi in step 1 | Wi-Fi in step 2 | LAA in step 2 | Wi-Fi in step 1 | Wi-Fi in step 2 | LAA in step 2 | Wi-Fi in step 1 | Wi-Fi in step 2 | LAA in step 2 |
| Ericsson | 3 | UPT CDF [Mbps] | 5% | 7.30 | (20.96) 16 | (78.16) 29.66 | 0.50 | (14.61) 8.78 | (67.19) 16.41 | 0.24 | (10.51) 3.73 | (58.64) 9.39 |
|  |  |  | 50% | 34.60 | (71.02) 61.84 | (126.9) 75.44 | 11.48 | (61.64) 46.98 | (117.7) 59.40 | 5.45 | (53.79) 32.04 | (107.3) 45.56 |
|  |  |  | 95% | 69.51 | (102.9) 97.26 | (151.3) 98.37 | 41.66 | (95.66) 86.23 | (145.8) 89.50 | 26.42 | (89.08) 74.97 | (141.6) 81.55 |
|  |  |  | Mean | 37.58 | (69.38) 61.18 | (124.2) 73.2 | 15.89 | (60.91) 47.95 | (115.3) 58.59 | 9.03 | (54.26) 36.44 | (106.6) 46.84 |
|  |  | Delay CDF [s] | 5% | 0.058 | (0.034) 0.036 | (0.023) 0.035 | 0.104 | (0.024) 0.039 | (0.036) 0.042 | 0.138 | (0.040) 0.050 | (0.024) 0.043 |
|  |  |  | 50% | 0.213 | (0.057) 0.076 | (0.030) 0.060 | 0.479 | (0.033) 0.086 | (0.069) 0.115 | 0.835 | (0.082) 0.185 | (0.037) 0.123 |
|  |  |  | 95% | 0.058 | (0.179) 0.274 | (0.047) 0.184 | 3.419 | (0.056) 0.373 | (0.255) 0.562 | 4.598 | (0.338) 1.191 | (0.066) 0.683 |
|  |  |  | Mean | 0.502 | (0.032) 0.115 | (0.079) 0.082 | 1.007 | (0.103) 0.207 | (0.036) 0.14 | 1.435 | (0.128) 0.387 | (0.041) 0.231 |
|  |  | VoIP outage (%) |  | 10.3 | (0.0) 5.7 | N/A | 23.9 | (0.3) 15.8 | N/A | 38.6 | (0.9) 33.6 | N/A |
|  |  | 𝜌 |  | 0.96 | (1.00) 1.00 | (1.00) 1.00 | 0.87 | (1.00) 0.99 | (1.00) 0.99 | 0.71 | (0.99) 0.97 | (1.00) 0.98 |
|  |  | BO |  | 0.20 | (0.08) 0.11 | (0.03) 0.06 | 0.40 | (0.11) 0.18 | (0.04) 0.12 | 0.60 | (0.15) 0.28 | (0.05) 0.19 |
|  |  | 𝜆 |  | 0.3398 files/s |  |  | 0.4918 files/s |  |  | 0.6667 files/s |  |  |
|  | Additional comments: (*(): results with licensed carrier) 256 QAM, LDPC, 2Tx2Rx, TXOP 4ms, and LBT algorithm based on R1-150584 where the LAA CCA starts and resumes at the subframe boundaries. If LBT succeeds before the 4th OFDM symbol in a subframe the channel is occupied by the LAA node. If the LBT fails by the 4th OFDM symbol in a subframe, the random backoff counter is frozen and the CCA is postponed to the next subframe boundary where the CCA count down is resumed. |  |  |  |  |  |  |  |  |  |  |  |
| Intel | 3 | UPT CDF [Mbps] | 5% | 6 | 12 | 4 | 2 | 7 | 5 | 0 | 4 | 6 |
|  |  |  | 50% | 18 | 35 | 111 | 9 | 22 | 72 | 4 | 15 | 39 |
|  |  |  | 95% | 46 | 69 | 193 | 28 | 49 | 149 | 17 | 37 | 106 |
|  |  |  | Mean | 21 | 38 | 98 | 11 | 25 | 67 | 5 | 17 | 46 |
|  |  | Delay CDF [s] | 5% | 0.088 | 0.058 | 0.021 | 0.145 | 0.081 | 0.027 | 0.176 | 0.11 | 0.038 |
|  |  |  | 50% | 0.226 | 0.116 | 0.034 | 0.472 | 0.178 | 0.048 | 1.315 | 0.289 | 0.078 |
|  |  |  | 95% | 0.788 | 0.356 | 0.892 | 3.31 | 0.705 | 0.503 | 9.047 | 2.085 | 0.377 |
|  |  |  | Mean | 0.381 | 0.142 | 0.148 | 0.911 | 0.244 | 0.142 | 2.753 | 0.532 | 0.129 |
|  |  | VoIP outage (%) |  | 17.22 | 0 | N/A | 21.84 | 5.88 | N/A | 17.6 | 8.4 | N/A |
|  |  | 𝜌 |  | 0.943 | 0.943 | 0.984 | 0.98 | 1 | 1 | 0.8 | 1 | 1 |
|  |  | BO |  | 0.19 | 0.13 | 0.12 | 0.41 | 0.24 | 0.17 | 0.685 | 0.41 | 0.22 |
|  |  | 𝜆 |  | 0.24 |  |  | 0.35 |  |  | 0.5 |  |  |
|  | Additional comments: results with licensed carrier. 2Tx2Rx, TXOP 4ms, and LBT algorithm based on R1-1500089. |  |  |  |  |  |  |  |  |  |  |  |

Table 10: Outdoor deployment for Wi-Fi and LAA coexistence case with four shared unlicensed carriers and FTP traffic

| Tdoc / Company | LAA LBT cat. | Reported parameters |  | Low load BO range for Wi-Fi in Step 1: 10%~25% |  |  | Medium load BO range for Wi-Fi in Step 1: 35%~50% |  |  | High load BO range for Wi-Fi in Step 1: above 55% |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Wi-Fi in step 1 | Wi-Fi in step 2 | LAA in step 2 | Wi-Fi in step 1 | Wi-Fi in step 2 | LAA in step 2 | Wi-Fi in step 1 | Wi-Fi in step 2 | LAA in step 2 |
| Ericsson | 3 | UPT CDF [Mbps] | 5% | 20.25 | (34.14) 21.4 | (74.1) 31.91 | 3.52 | (19.17) 3.89 | (49.54) 8.45 | 0.63 | (5.3) 0.41 | (25.69) 2.24 |
|  |  |  | 50% | 69.42 | (88.21) 75.92 | (127.0) 86.37 | 37.18 | (70.46) 47.25 | (108.6) 62.04 | 18.82 | (42.36) 24.83 | (80.82) 39.09 |
|  |  |  | 95% | 104.01 | (115.2) 110.83 | (153.1) 111.33 | 84.89 | (105.8) 97.68 | (142.1) 101.61 | 62.78 | (92.68) 81.97 | (125.7) 90.23 |
|  |  |  | Mean | 68.52 | (85.34) 73.81 | (123.8) 82.32 | 40.64 | (68.73) 49.84 | (105.7) 60.32 | 24.32 | (45.54) 31.54 | (80.48) 42.45 |
|  |  | Delay CDF [s] | 5% | 0.034 | (0.031) 0.031 | (0.023) 0.031 | 0.044 | (0.034) 0.036 | (0.025) 0.034 | 0.064 | (0.039) 0.043 | (0.028) 0.04 |
|  |  |  | 50% | 0.067 | (0.047) 0.062 | (0.029) 0.047 | 0.135 | (0.066) 0.116 | (0.036) 0.075 | 0.248 | (0.146) 0.198 | (0.056) 0.141 |
|  |  |  | 95% | 0.304 | (0.12) 0.421 | (0.05) 0.275 | 1.201 | (0.276) 1.263 | (0.081) 0.791 | 2.361 | (1.249) 2.59 | (0.261) 1.565 |
|  |  |  | Mean | 0.116 | (0.059) 0.151 | (0.033) 0.111 | 0.366 | (0.11) 0.366 | (0.043) 0.228 | 0.63 | (0.371) 0.6 | (0.096) 0.4 |
|  |  | 𝜌 |  | 1.00 | (1.00) 0.99 | (1.00) 0.99 | 0.97 | (1.00) 0.96 | (1.00) 0.98 | 0.85 | (0.94) 0.81 | (0.99) 0.94 |
|  |  | BO |  | 0.20 | (0.14) 0.2 | (0.10) 0.18 | 0.40 | (0.26) 0.38 | (0.17) 0.35 | 0.60 | (0.46) 0.58 | (0.33) 0.55 |
|  |  | 𝜆 |  | 0.333 files/s |  |  | 0.5156 files/s |  |  | 0.7999 files/s |  |  |
|  | Additional comments: (*(): results with licensed carrier) 256 QAM, LDPC, 2Tx2Rx, TXOP 4ms, and LBT algorithm based on R1-150584 where the LAA CCA starts and resumes at the subframe boundaries. If LBT succeeds before the 4th OFDM symbol in a subframe the channel is occupied by the LAA node. If the LBT fails by the 4th OFDM symbol in a subframe, the random backoff counter is frozen and the CCA is postponed to the next subframe boundary where the CCA count down is resumed. |  |  |  |  |  |  |  |  |  |  |  |
| NTT DOCOMO | 2 | UPT CDF [Mbps] | 5% | 4.74 | 9.13 | 56.3 | 2.65 | 3.35 | 46.0 | 2.19 | 2.24 | 39.2 |
|  |  |  | 50% | 59.7 | 65.6 | 129.0 | 46.0 | 51.2 | 117.7 | 40.8 | 42.6 | 111.1 |
|  |  |  | 95% | 121.2 | 121.2 | 181.8 | 121.2 | 121.2 | 181.8 | 121.2 | 121.2 | 181.8 |
|  |  |  | Mean | 63.1 | 69.8 | 127.2 | 51.9 | 56.8 | 116.8 | 47.8 | 48.7 | 111.6 |
|  |  | Delay CDF [s] | 5% | 0.033 | 0.033 | 0.022 | 0.033 | 0.033 | 0.022 | 0.033 | 0.033 | 0.022 |
|  |  |  | 50% | 0.066 | 0.061 | 0.031 | 0.087 | 0.078 | 0.034 | 0.098 | 0.094 | 0.036 |
|  |  |  | 95% | 0.798 | 0.433 | 0.071 | 1.411 | 1.180 | 0.087 | 1.663 | 1.684 | 0.102 |
|  |  |  | Mean | 0.225 | 0.166 | 0.036 | 0.313 | 0.258 | 0.042 | 0.352 | 0.344 | 0.046 |
|  |  | 𝜌 |  | 0.995 | 0.995 | 0.999 | 0.993 | 0.993 | 0.999 | 0.991 | 0.991 | 0.999 |
|  |  | BO |  | 0.277 | 0.201 | 0.050 | 0.483 | 0.448 | 0.090 | 0.548 | 0.583 | 0.113 |
|  |  | 𝜆 |  | 6 |  |  | 10 |  |  | 12 |  |  |
|  | Additional comments: Results with licensed carrier. FTP model 1, maximum 4ms burst length for both Wi-Fi and LAA. 1ms CCA period for LAA when channel is busy as described in R1-150414. |  |  |  |  |  |  |  |  |  |  |  |
| Huawei, HiSilicon, | Cat.3 | UPT CDF [Mbps] | 5% | 1.75 | 2.48 | 24.64 | 0.98 | 1.84 | 22.39 | 0.67 | 0.14 | 3.83 |
|  |  |  | 50% | 29.68 | 36.53 | 46.78 | 13.91 | 27.98 | 44.12 | 0.93 | 2.82 | 33.88 |
|  |  |  | 95% | 52.29 | 55.67 | 52.93 | 41.58 | 48.73 | 52.98 | 20.30 | 31.59 | 48.42 |
|  |  |  | Mean | 28.52 | 35.05 | 44.12 | 16.39 | 26.03 | 41.35 | 3.97 | 8.26 | 30.88 |
|  |  | Delay CDF [s] | 5% | 0.76 | 0.07 | 0.08 | 0.10 | 0.08 | 0.08 | 0.19 | 0.12 | 0.08 |
|  |  |  | 50% | 0.13 | 0.11 | 0.09 | 0.27 | 0.14 | 0.09 | 2.97 | 1.18 | 0.12 |
|  |  |  | 95% | 1.70 | 1.24 | 0.15 | 2.61 | 1.55 | 0.19 | 7.49 | 7.22 | 0.84 |
|  |  |  | Mean | 0.36 | 0.24 | 0.10 | 0.76 | 0.37 | 0.12 | 3.59 | 2.39 | 0.29 |
|  |  | 𝜌 |  | 1 | 1 | 1 | 1 | 1 | 1 | 0.77 | 0.87 | 1 |
|  |  | BO |  | 0.25 | 0.17 | 0.15 | 0.46 | 0.30 | 0.22 | 0.84 | 0.73 | 0.50 |
|  |  | 𝜆 |  | 0.2 |  |  | 0.3 |  |  | 0.6 |  |  |
|  | Additional comments: R1-150042, No 256QAM, no LDPC code, LAA max. duration =4ms, WiFi DL only, LAA on unlicensed carrier only. LBT scheme follows the Option B from ETSI EN 301 893 v1.8.0 |  |  |  |  |  |  |  |  |  |  |  |

| Samsung | 3 | UPT CDF [Mbps] | 5% | 11.3 | 11.8 | 37.8 | 5.7 | 6.6 | 30.2 | 1.3 | 1.9 | 23.3 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 50% | 59.4 | 63.7 | 139.8 | 50.7 | 55.2 | 129.0 | 34.7 | 44.8 | 114.9 |
|  |  |  | 95% | 117.8 | 118.0 | 166.8 | 105.9 | 110.4 | 160.4 | 94.6 | 103.0 | 152.9 |
|  |  |  | Mean | 52.4 | 55.4 | 127.2 | 45.1 | 48.5 | 117.4 | 32.8 | 40.7 | 106.4 |
|  |  | Delay CDF [s] | 5% | 0.034 | 0.034 | 0.022 | 0.034 | 0.034 | 0.022 | 0.034 | 0.034 | 0.022 |
|  |  |  | 50% | 0.073 | 0.066 | 0.028 | 0.092 | 0.083 | 0.032 | 0.151 | 0.107 | 0.036 |
|  |  |  | 95% | 0.454 | 0.432 | 0.117 | 0.798 | 0.891 | 0.163 | 2.482 | 1.726 | 0.206 |
|  |  |  | Mean | 0.209 | 0.175 | 0.050 | 0.322 | 0.325 | 0.068 | 0.745 | 0.545 | 0.090 |
|  |  | 𝜌 |  | 0.99 | 0.99 | 1.00 | 0.98 | 0.98 | 1.00 | 0.91 | 0.93 | 0.98 |
|  |  | BO |  | 0.24 | 0.23 | 0.12 | 0.39 | 0.37 | 0.22 | 0.58 | 0.51 | 0.32 |
|  |  | 𝜆 |  | 0.2 |  |  | 0.4 |  |  | 0.7 |  |  |
|  | Additional comments: LAA with licensed carrier, TXOP: 10ms(LAA), 4ms(WiFi), 2Tx2Rx, BCC, no 256QAM, CCA slot: 20us. For LBE, CCA (20us) is always followed by ECCA |  |  |  |  |  |  |  |  |  |  |  |
| Intel | 3 | UPT CDF [Mbps] | 5% | 9 | 15 | 6 | 2 | 10 | 6 | 1 | 4 | 5 |
|  |  |  | 50% | 29 | 45 | 134 | 14 | 34 | 96 | 7 | 21 | 56 |
|  |  |  | 95% | 77 | 87 | 214 | 49 | 77 | 192 | 37 | 62 | 153 |
|  |  |  | Mean | 34 | 47 | 123 | 18 | 37 | 93 | 11 | 25 | 65 |
|  |  | Delay CDF [s] | 5% | 0.05 | 0.05 | 0.02 | 0.08 | 0.05 | 0.02 | 0.11 | 0.07 | 0.03 |
|  |  |  | 50% | 0.14 | 0.09 | 0.03 | 0.31 | 0.12 | 0.04 | 0.63 | 0.19 | 0.05 |
|  |  |  | 95% | 0.48 | 0.27 | 0.21 | 2.36 | 0.40 | 0.21 | 6.33 | 1.11 | 0.17 |
|  |  |  | Mean | 0.19 | 0.12 | 0.06 | 0.64 | 0.16 | 0.07 | 1.54 | 0.34 | 0.07 |
|  |  | 𝜌 |  | 0.99 | 1.00 | 1.00 | 0.98 | 0.99 | 0.99 | 0.95 | 1.00 | 1.00 |
|  |  | BO |  | 0.22 | 0.15 | 0.14 | 0.49 | 0.26 | 0.2 | 0.70 | 0.47 | 0.42 |
|  |  | 𝜆 |  | 0.17 |  |  | 0.25 |  |  | 0.33 |  |  |
|  | Additional comments: results with licensed carrier. 2Tx2Rx, TXOP 4ms, and LBT algorithm based on R1-1500089. |  |  |  |  |  |  |  |  |  |  |  |

Table 11: Outdoor deployment for LAA and LAA coexistence case with four shared unlicensed carriers and FTP traffic

| Tdoc / Company | LAA LBT cat. | Reported parameters |  | Low load BO range: 10%~25% |  | Medium load BO range: 35%~50% |  | High load BO range: above 55% |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | LAA opt. 1 | LAA opt. 2 | LAA opt. 1 | LAA opt. 2 | LAA opt. 1 | LAA opt. 2 |
| Huawei, HiSilicon, HiSilicon | Cat.3 | UPT CDF [Mbps] | 5% | 34.36 | 34.36 | 5.77 | 5.77 | 0.04 | 0.04 |
|  |  |  | 50% | 48.71 | 48.71 | 35.57 | 35.57 | 19.68 | 19.68 |
|  |  |  | 95% | 52.94 | 52.94 | 49.10 | 49.10 | 43.17 | 43.17 |
|  |  |  | Mean | 46.66 | 46.66 | 32.27 | 32.27 | 20.20 | 20.20 |
|  |  | Delay CDF [s] | 5% | 0.08 | 0.08 | 0.08 | 0.08 | 0.09 | 0.09 |
|  |  |  | 50% | 0.08 | 0.08 | 0.11 | 0.11 | 0.20 | 0.20 |
|  |  |  | 95% | 0.12 | 0.12 | 0.65 | 0.65 | 7.36 | 7.36 |
|  |  |  | Mean | 0.09 | 0.09 | 0.24 | 0.24 | 0.99 | 0.99 |
|  |  | 𝜌 |  | 1 | 1 | 1 | 1 | 0.93 | 0.93 |
|  |  | BO |  | 0.14 | 0.14 | 0.48 | 0.48 | 0.73 | 0.73 |
|  |  | 𝜆 |  | 0.2 |  | 0.6 |  | 1.0 |  |
|  | Additional comments: R1-150043, No 256QAM, LAA max. duration =4ms, LAA on unlicensed carrier only. LBT scheme follows the Option B from ETSI EN 301 893 v1.8.0. The performance metrics is not independently calculated per operator in LAA and LAA coexistence case. |  |  |  |  |  |  |  |  |

| Samsung | 3 | UPT CDF [Mbps] | 5% | 29.5 | 27.6 | 13.6 | 11.9 | 8.9 | 7.6 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 50% | 83.3 | 83.4 | 66.0 | 66.9 | 59.8 | 60.1 |
|  |  |  | 95% | 139.0 | 141.7 | 123.4 | 125.5 | 120.2 | 120.7 |
|  |  |  | Mean | 76.1 | 76.6 | 61.5 | 61.9 | 56.5 | 56.7 |
|  |  | Delay CDF [s] | 5% | 0.024 | 0.023 | 0.026 | 0.025 | 0.026 | 0.026 |
|  |  |  | 50% | 0.047 | 0.047 | 0.063 | 0.063 | 0.073 | 0.070 |
|  |  |  | 95% | 0.166 | 0.180 | 0.423 | 0.439 | 0.637 | 0.662 |
|  |  |  | Mean | 0.095 | 0.097 | 0.215 | 0.237 | 0.289 | 0.311 |
|  |  | 𝜌 |  | 1.00 | 1.00 | 0.98 | 0.98 | 0.97 | 0.96 |
|  |  | BO |  | 0.19 | 0.20 | 0.45 | 0.46 | 0.53 | 0.54 |
|  |  | 𝜆 |  | 0.4 |  | 0.7 |  | 0.9 |  |
|  | Additional comments: LAA with licensed carrier, TXOP: 10ms(LAA), 4ms(WiFi), 2Tx2Rx, BCC, no 256QAM, CCA slot: 20us. For LBE, CCA (20us) is always followed by ECCA |  |  |  |  |  |  |  |  |
| Intel | 3 | UPT CDF [Mbps] | 5% | 9 | 7 | 6 | 8 | 10 | 8 |
|  |  |  | 50% | 137 | 137 | 81 | 81 | 50 | 46 |
|  |  |  | 95% | 210 | 214 | 181 | 178 | 117 | 115 |
|  |  |  | Mean | 121 | 123 | 86 | 85 | 54 | 51 |
|  |  | Delay CDF [s] | 5% | 0.02 | 0.02 | 0.02 | 0.02 | 0.03 | 0.03 |
|  |  |  | 50% | 0.03 | 0.03 | 0.03 | 0.04 | 0.06 | 0.06 |
|  |  |  | 95% | 0.16 | 0.14 | 0.11 | 0.13 | 0.11 | 0.13 |
|  |  |  | Mean | 0.05 | 0.05 | 0.05 | 0.05 | 0.06 | 0.07 |
|  |  | 𝜌 |  | 1.0 | 1.0 | 1.0 | 1.0 | 1.0 | 1.0 |
|  |  | BO |  | 0.22 | 0.28 | 0.42 | 0.40 | 0.59 | 0.66 |
|  |  | 𝜆 |  | 0.22 |  | 0.35 |  | 0.66 |  |
|  | Additional comments: results with licensed carrier. 2Tx2Rx, TXOP 4ms, and LBT algorithm based on R1-1500089. |  |  |  |  |  |  |  |  |

Table 12: Outdoor deployment for Wi-Fi and LAA coexistence case with four shared unlicensed carriers and Mixed traffic

| Tdoc / Company | LAA LBT cat. | Reported parameters |  | Low load BO range for Wi-Fi in Step 1: 10%~25% |  |  | Medium load BO range for Wi-Fi in Step 1: 35%~50% |  |  | High load BO range for Wi-Fi in Step 1: above 55% |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Wi-Fi in step 1 | Wi-Fi in step 2 | LAA in step 2 | Wi-Fi in step 1 | Wi-Fi in step 2 | LAA in step 2 | Wi-Fi in step 1 | Wi-Fi in step 2 | LAA in step 2 |
| Ericsson | 3 | UPT CDF [Mbps] | 5% | 14.29 | (23.34) 18.33 | (77.05) 35.98 | 1.8 | (12.96) 4.07 | (52.64) 9.99 | 0.42 | (4.87) 0.64 | (28.3) 2.97 |
|  |  |  | 50% | 64.55 | (81.79) 74.09 | (127.9) 85.92 | 31.45 | (63.78) 46.74 | (108.2) 64.73 | 15.54 | (41.29) 23.31 | (83.47) 41.57 |
|  |  |  | 95% | 101.96 | (112.8) 110.19 | (153.7) 111.48 | 80.96 | (103.2) 96.26 | (142.4) 103.8 | 57.79 | (89.84) 81.35 | (127.2) 92.77 |
|  |  |  | Mean | 63.3 | (77.89) 71.46 | (125.2) 83.22 | 36.4 | (62.66) 49.2 | (105.8) 63.02 | 21.48 | (44.71) 31.74 | (82.8) 44.64 |
|  |  | Delay CDF [s] | 5% | 0.036 | (0.031) 0.032 | (0.023) 0.031 | 0.047 | (0.034) 0.037 | (0.024) 0.034 | 0.069 | (0.04) 0.045 | (0.028) 0.039 |
|  |  |  | 50% | 0.075 | (0.049) 0.061 | (0.029) 0.047 | 0.163 | (0.073) 0.115 | (0.036) 0.07 | 0.286 | (0.143) 0.215 | (0.053) 0.132 |
|  |  |  | 95% | 0.573 | (0.157) 0.386 | (0.048) 0.181 | 1.891 | (0.324) 1.199 | (0.079) 0.631 | 2.63 | (1.262) 2.531 | (0.207) 1.443 |
|  |  |  | Mean | 0.174 | (0.069) 0.129 | (0.032) 0.08 | 0.512 | (0.125) 0.352 | (0.043) 0.189 | 0.7 | (0.365) 0.632 | (0.081) 0.373 |
|  |  | VoIP outage % |  | 3 | (0) | N/A | 9 | (1) | N/A | 8 | (8) | N/A |
|  |  | 𝜌 |  | 0.99 | (1) 0.99 | (1) 1 | 0.95 | (1) 0.96 | (1) 0.99 | 0.8 | (0.99) 0.83 | (0.94) 0.95 |
|  |  | BO |  | 0.20 | (0.17) 0.2 | (0.08) 0.14 | 0.40 | (0.3) 0.39 | (0.16) 0.31 | 0.60 | (0.47) 0.56 | (0.3) 0.51 |
|  |  | 𝜆 |  | 0.339 files/s |  |  | 0.492 files/s |  |  | 0.667 files/s |  |  |
|  | Additional comments: (*(): results with licensed carrier) 256 QAM, LDPC, 2Tx2Rx, TXOP 4ms, and LBT algorithm based on R1-150584 where the LAA CCA starts and resumes at the subframe boundaries. If LBT succeeds before the 4th OFDM symbol in a subframe the channel is occupied by the LAA node. If the LBT fails by the 4th OFDM symbol in a subframe, the random backoff counter is frozen and the CCA is postponed to the next subframe boundary where the CCA count down is resumed. |  |  |  |  |  |  |  |  |  |  |  |
| Intel | 3 | UPT CDF [Mbps] | 5% | 9 | 15 | 6 | 6 | 12 | 6 | 1 | 6 | 5 |
|  |  |  | 50% | 34 | 48 | 129 | 25 | 43 | 113 | 11 | 29 | 73 |
|  |  |  | 95% | 80 | 86 | 204 | 71 | 84 | 194 | 45 | 69 | 167 |
|  |  |  | Mean | 39 | 49 | 117 | 30 | 45 | 103 | 15 | 32 | 76 |
|  |  | Delay CDF [s] | 5% | 0.05 | 0.046 | 0.019 | 0.056 | 0.048 | 0.02 | 0.065 | 0.057 | 0.024 |
|  |  |  | 50% | 0.118 | 0.083 | 0.029 | 0.161 | 0.095 | 0.033 | 0.325 | 0.141 | 0.045 |
|  |  |  | 95% | 0.43 | 0.271 | 0.335 | 0.738 | 0.349 | 0.309 | 3.916 | 0.682 | 0.249 |
|  |  |  | Mean | 0.167 | 0.114 | 0.073 | 0.26 | 0.134 | 0.075 | 0.876 | 0.238 | 0.079 |
|  |  | VoIP outage % |  | 1.6 | 0 | N/A | 15.6 | 4.2 | N/A | 48.64 | 9.88 | N/A |
|  |  | 𝜌 |  | 4.08 | 1 | 1 | 1.0 | 0.99 | 0.99 | 1.0 | 0.9 | 0.92 |
|  |  | BO |  | 0.21 | 0.245 | 0.24 | 0.21 | 0.35 | 0.28 | 0.25 | 0.625 | 0.42 |
|  |  | 𝜆 |  | 0.125 |  |  | 0.125 |  |  | 0.16 |  |  |
|  | Additional comments: results with licensed carrier. 2Tx2Rx, TXOP 4ms, and LBT algorithm based on R1-1500089, random channel selection. |  |  |  |  |  |  |  |  |  |  |  |