3GPP TSG RAN WG1 Meeting Ad-hoc                                                                     R1-151146

Paris, France, 24th – 26th March 2015

Source:	Ericsson

Title:		Additional Coexistence Evaluation Results for DL-only LAA and UL+DL Wi-Fi

Agenda Item:	2.1

Document for:	Discussion and Decision

# Introduction

Substantial progress was made for the Study Item on Licensed-Assisted Access (LAA) [1] in RAN1 WG #80 [2]. An additional optional simulation scenario for DL-only LAA was agreed as the following:

Agreements:

- Include an additional optional simulation scenario with Y=1 (single channel scenario) with the following assumptions

  - Non replaced WiFi network has both DL and UL traffic

  - WiFi network, which is replaced by LAA, has only DL FTP traffic

  - Assume 20UEs per operator

  - For all other parameters, use the existing DL + UL simulation assumptions whenever applicable

  - For traffic load and split (Overall offered load is the same for both the coexisting networks) at least the following case should be simulated:

    - Traffic load on DL-only Wi-Fi and LAA networks is 25% greater than that of the DL nodes in the DL+UL non-replaced Wi-Fi network

    - DL to UL ratio is 80% to 20% for this scenario

We have considered the above scenario for the coexistence study and in this contribution, we report the coexistence evaluation results for the LAA solution without UL transmissions discussed in [3] for indoor scenarios when the non-replaced Wi-Fi network supports both UL and DL traffic while the replaced-Wi-Fi network as well as LAA both support only DL traffic.

# Coexistence Evaluation Assumptions

The coexistence methodology agreed in RAN1#78bis [4] is the following:

Agreements:

- Wi-Fi-LAA coexistence

  - For each UE and eNB/AP drop

    - Step 1: Performance metrics for two Wi-Fi networks coexisting in a given evaluation scenario are evaluated and recorded.

    - Step 2: Wi-Fi is replaced with LAA for the group of eNBs and UEs served by one of the Wi-Fi operators. Performance metrics of the Wi-Fi network coexisting with the LAA network are evaluated and recorded.

  - Performance metrics for the Wi-Fi operator common to the two steps are compared.

- LAA-LAA coexistence

  - Performance metrics for two LAA operators coexisting in a given evaluation scenario are evaluated and recorded.

  - Performance metrics for the two LAA operators are compared.

The above agreed methodology is used for the coexistence evaluations. Briefly, in the Wi-Fi and LAA coexistence scenario, in the first step Operator A and B both use Wi-Fi. In the second step, operator A and its corresponding UEs are replaced by an LAA operator and LAA UEs while operator B and its UEs remain unchanged. Additionally operator A supports only DL traffic while both DL and UL traffic are supported by operator B. Moreover, the licensed PCell carrier is not used in the LAA network. More details on the simulation assumptions are provided in the Appendix, Annex A.

# Coexistence Evaluation Results

In this evaluation scenario, two operators deploy X=4 small cells each in the single-floor building sharing Y=1 unlicensed carrier. 20 UEs for each operator network are modeled. Operator A supports DL-only traffic while the non-replaced Wi-Fi operator B supports both DL and UL traffic. The results presented in Figure 1 and Figure 2 correspond to an 80/20 split between DL and UL traffic in the Wi-Fi network B while the results available in Figure 3 and Figure 4 correspond to a 50/50 split between DL and UL traffic in the Wi-Fi network B. Results are shown for FTP traffic as well as the mixed traffic model with FTP and VoIP traffic, where two additional VoIP traffic UEs are modeled for the non-replaced Wi-Fi network. Additional results that provide a comprehensive picture of the observed performance are shown in the Appendix, Annex B.

Figure 1: User throughput vs. served traffic per operator per AP for FTP traffic with 80/20 split between DL and UL traffic where ∇ and O correspond to 5%-ile and mean user throughput, respectively. Operator A network has only DL traffic and Operator B network has DL and UL traffic. For LAA, licensed band PCell is not used for DL traffic. The left and right plots correspond to DL and UL user throughput results, respectively.

Figure 2: VoIP outage with 80/20 split between DL and UL traffic where black and blue lines correspond to the non-replaced Wi-Fi network coexisting with Wi-Fi and with LAA, respectively. Operator A network has only DL traffic and Operator B network has DL and UL traffic. For LAA, licensed band PCell is not used for DL traffic. The left and right plots correspond to DL and UL user results, respectively.

Figure 3: User throughput vs. served traffic per operator per AP for FTP traffic with 50/50 split between DL and UL traffic where ∇ and O correspond to 5%-ile and mean user throughput, respectively. Operator A network has only DL traffic and Operator B network has DL and UL traffic. For LAA, licensed band PCell is not used for DL traffic. The left and right plots correspond to DL and UL user throughput results, respectively.

Figure 4: VoIP outage with 50/50 split between DL and UL traffic where black and blue lines correspond to the non-replaced Wi-Fi coexisting with Wi-Fi and with LAA, respectively. Operator A network has only DL traffic and Operator B network has DL and UL traffic. For LAA, licensed band PCell is not used for DL traffic. The left and right plots correspond to DL and UL user results, respectively.

# Discussion

The results presented here provide an overview on the coexistence of DL-only LAA with a Wi-Fi network that carries both DL and UL traffic. The system performance results clearly show that not only does DL-only LAA coexists in a fair manner with Wi-Fi but also boosts Wi-Fi performance as compared to the case where two Wi-Fi networks coexist with each other. Some of the major elements which make LAA a good neighbor to Wi-Fi in the unlicensed band where both technologies have to share the medium are listed below:

- LTE, as compared to Wi-Fi, enjoys more robust interference mitigation, error correction and retransmission schemes which results in greater efficiency in serving the traffic and reducing resource utilization thus providing more opportunities for other systems to access the medium.

- LAA uses the same sensing threshold irrespective of transmissions from LAA or Wi-Fi nodes as opposed to Wi-Fi which uses a higher sensing threshold for LAA as compared to Wi-Fi.

- The LBT algorithm is a load based algorithm with freeze periods that provides lots of opportunities for Wi-Fi nodes to access the channel.

Overall, the results show that DL-only LAA does not degrade VoIP and FTP performance of a co-existing UL+DL Wi-Fi network when it is used instead of another Wi-Fi network for the evaluated scenarios. However another interesting behavior is observed due to the unbalanced DL and UL traffic in coexisting networks which is explained in the following:

- The Wi-Fi network with both DL and UL traffic (and consequently more nodes contending for channel access) manages to achieve higher served traffic even when the offered load is the same as the other Wi-Fi network with only DL traffic (and consequently fewer nodes contending for channel access).

- As one of the Wi-Fi networks attempts to serve more and more UL traffic, the performance of the other Wi-Fi network becomes more and more severely degraded, which can be clearly observed in the “bending back” of the served traffic for the Operator A network. This degradation increases as the traffic split shifts towards increased UL traffic in the Wi-Fi network with bidirectional traffic.

- The Wi-Fi network with both DL and UL traffic manages to achieve higher served traffic than the DL-only LAA network when the offered loads to both networks are identical.

- As the Wi-Fi network attempts to serve more and more UL traffic, the LAA network performance becomes more and more severely depressed.

Based on the above discussion, we can arrive at the following observation from these evaluation results:

Observation:

- DL-only LAA can improve the performance of the FTP and VoIP services of a coexisting UL+DL Wi-Fi network.

- A network with few nodes contending for channel access is found to be able to serve much lower traffic than a co-channel Wi-Fi network with more nodes contending for channel access even with identical offered loads for both networks. The disparity in served traffic increases with the difference between the numbers of nodes contending for channel access in the two networks.

More insights are provided in our companion contribution [6].

# Conclusions

In this contribution, we report coexistence evaluation results for DL-only LAA for indoor deployments when the Wi-Fi network supports both UL and DL traffic where FTP and VoIP traffic can be served in either direction. In these evaluations, LAA is not utilizing the licensed band carrier. We have also provided simulation results for different DL and UL traffic split in the UL+DL Wi-Fi network.

Based on the simulation results, we make the following observations:

Observations:

- DL-only LAA can improve the performance of the FTP and VoIP services of a coexisting UL+DL Wi-Fi network.

- A network with few nodes contending for channel access is found to be able to serve much lower traffic than a co-channel Wi-Fi network with more nodes contending for channel access even with identical offered loads for both networks. The disparity in served traffic increases with the difference between the numbers of nodes contending for channel access in the two networks.

# References

RP-141664 Study on Licensed-Assisted Access using LTE, Ericsson, Qualcomm, Huawei, ALU

Chairman’s notes, 3GPP RAN1 WG #79

R1-151131 Further details on LBT design in DL for LAA, Ericsson

Chairman’s notes, 3GPP RAN1 WG#78bis

R1-150955, 3GPP TR36.889 v0.3.1, “Study on Licensed-Assisted Access to Unlicensed Spectrum”

R1-151130 Discussion on the LBT Protocol, Ericsson

# Appendix

Annex A: Coexistence Evaluation Assumptions

The simulation assumptions are based on the agreed coexistence assumptions in [5]. However our preferences on the assumptions that remained optional or need clarifications when results are presented are provided below. In all the indoor coexistence evaluations, the transmit power of the base station in the unlicensed band is assumed to be 18 dBm. Moreover, FTP model 3 is used for generating FTP traffic.

Additionally, the LBT algorithm used for LAA is based on the proposed load-based algorithm presented in [3] where the LAA CCA starts and resumes at the subframe boundaries. In more detail, if LBT succeeds before the 4th OFDM symbol in a subframe, the channel is occupied by the LAA node by initial signals until the 4th OFDM symbol and then followed by data transmission from the 4th OFDM symbol in that subframe. If the LBT fails by the 4th OFDM symbol in a subframe, the random backoff counter is frozen and the CCA is deferred to the next subframe boundary for a freeze period of 11 OFDM symbol where the CCA count down is resumed. A maximum channel occupancy time of 4ms is assumed for LAA. Finally, Table 5 and Table 6 capture our assumptions for Wi-Fi and LAA systems.

Table 5: Additional Wi-Fi system evaluation assumptions

| Parameter |  | Value |
| --- | --- | --- |
| MCS |  | 802.11ac MCS table with 256 QAM |
| Antenna configuration |  | 2Tx2Rx in DL, Cross-polarized Baseline: open loop |
| Channel coding |  | LDPC |
| Frame aggregation |  | A-MPDU |
| MPDU size |  | 1500B MSDU + 14 B header |
| Max PPDU duration |  | Baseline:< 4 ms (Asynchronous to LTE timing) |
| MAC | Coordination | EDCA |
|  | SIFS, DIFS | SIFS, DIFS |
|  | Detection | Energy detection & preamble detection |
|  | RTS/CTS | No |
|  | Contention window | Per EDCA |
| CCA-CS |  | -82dBm and preamble decoding (Note preamble occupies the 20MHz system bandwidth with rate 1/2 coding and BPSK modulation) |
| CCA-ED |  | -62dBm |
| ACK Modeled (successful reception, resources utilized) |  | Yes |
| DL/UL Duplexing |  | For the DL-only LAA coexistence evaluations: - DL traffic only for the replaced Wi-Fi netwrok - DL and UL for the non-replaced Wi-Fi network |
| Rate control |  | Minstrel algorithm |
| Channel selection |  | Based on the minimum interference level while ensuring that each unlicensed carrier is shared by two operators in each cluster |
| OFDM symbol length |  | 4 micro second |

Table 6: Additional LAA system evaluations assumptions

| Parameters | Value |
| --- | --- |
| PCI planning for each NW | Planned |
| Antenna configuration | 2Tx2Rx in DL, Cross-polarized. |
| Transmission schemes | Based on TM10, QPSK/16QAM/64QAM/256QAM |
| Turbo code block interleaving depth | Per LTE specs (1-14 LTE OFDM symbols dependent on MCS and PRB allocation) |
| Scheduling | Proportional fair |
| Link adaptation | Realistic |
| CCA-ED | -82 dBm |
| Channel selection | Based on the minimum interference level while ensuring that each unlicensed carrier is shared by two operators in each cluster |
| Cyclic Prefix | Normal |

Annex B: Additional Coexistence Evaluation Results

Figure 5: Ratio of DL served traffic over the offered traffic and DL and UL buffer occupancy for the case with 80/20 DL and UL traffic split

Figure 6: Ratio of DL served traffic over the offered traffic and DL and UL buffer occupancy for the case with 50/50 DL and UL traffic split