3GPP TSG RAN WG1 Ad-Hoc Meeting 1901	Tdoc R1-1900173

Taipei, Taiwan, 21st – 25th January 2019

Agenda Item:	7.2.6.4

Source:	Ericsson

Title:	Evaluation of Rel-15 Enabled URLLC Use Cases

Document for:	Discussion, Decision

# 1	Introduction

In RAN1#94bis [1], requirements and simulation assumptions for different URLLC use cases are defined.

Rel-15 enabled use case is one of the use cases with two sets of requirements, one for small packet size with strict reliability and latency requirements and another for large packet size with more relaxed requirements. In this contribution, we evaluate the Rel-15 enabled use case using the stricter requirement, i.e., reliability of 99.999% within 1ms latency for a small packet size of 32 bytes. We note that these requirements are also aligned with the ITU IMT 2020 reliability requirement [2].

# 2	Discussion

Reliability and latency requirements for Rel-15 enabled use case defined in [1] are shown in Table 1 below. In the following, we focus on the reliability requirement of 99.999% within 1ms latency for a small packet of 32 bytes.

Table 1. Requirements for Rel-15 enabled use case.

| Use case (Clause #) | Reliability (%) | Latency (ms) | Data packet size and traffic model |
| --- | --- | --- | --- |
| Rel-15 enabled use case (e.g. AR/VR) | 99.999 | 1ms (air interface delay) for 32 bytes 1 ms and 4 ms (air interface delay) for 200 bytes | DL & UL: 32 and 200 bytes FTP model 3 or periodic with different arrival rates |
|  | 99.9 | 7ms (air interface delay) | DL & UL: 4096, 10 K FTP model 3 or periodic with different arrival rates |

## 2.1	System-Level Simulations (SLS)

We perform system level simulation based on the assumptions outlined in [1] and also shown in Table 2 below. Full buffer traffic model is assumed which may be seen as the worst-case assumption in terms of interference where users always have data to transmit. UL and DL SINR distributions are shown in Figure 1.

Table 2. Assumptions for the system-level simulations for Rel-15 enabled use case (Urban macro).

| Parameters | Value |
| --- | --- |
| Layout | Single layer - Macro layer: Hex. Grid |
| Inter-BS distance | 500m |
| Carrier frequency | 4 GHz |
| Channel model | UMa in TR 38.901 |
| UE Tx power | 23dBm |
| BS antenna configurations | 16Tx/16Rx ports (M, N, P, Mg, Ng; Mp, Np) = (8, 8, 2, 1, 1; 1, 8), dH = 0.5λ, dV = 0.8λ; 9 degrees electrical antenna tilt |
| BS antenna height | 25m |
| BS antenna element gain + connector loss | 8 dBi |
| BS receiver noise figure | 5dB |
| UE antenna configuration | 2 Tx/4 Rx antenna ports |
| UE antenna height | Follow the modelling of TR 38.901 (e.g. 1.5m) |
| UE antenna gain | 3dBi |
| UE receiver noise figure | 9 dB |
| Total transmit power per TRxP | 49 dBm |
| Simulation bandwidth | 40 MHz DL and 40 MHz UL |
| Duplex mode | FDD |
| SCS | 30 kHz |
| UE distribution | 80% of users are outdoors and 20% of users are indoors Indoor penetration loss is modelled according to low loss model |
| Traffic | Full buffer |

Figure 1. Full buffer DL and UL SINR distribution for URLLC Rel-15 enabled use case.

For macro scenario, the 5th%-tile SINR is typically considered to representing a cell-edge user SINR. Here the 5th%-tile SINR is 1.69 dB (DL) and 0.92 dB (UL).

- The cell-edge SINR for URLLC Rel-15 enabled use case (macro scenario) is approximately 1.69 dB (DL) and 0.92 dB (UL).

## Link-level simulations (LLS)

Next, we evaluate link level performance for Urban macro scenario using the assumptions in Table 3. For PDCCH a DCI of size 40 bits excluding CRC is assumed. For PUCCH we have assumed 1-bit UCI carried by PUCCH format 0 with 2os duration and frequency hopping (evaluated at 1% D2A rate). For data, different MCSs are evaluated for a small packet size of 32 bytes transmitted over 4os PDSCH/PUSCH.

The resulting BLER and error rate as a function of SNR for the DL and UL control channels are shown in Figure 2, and for the data channels in Figure 3.

Table 3. Assumptions for the link-level simulations for Rel-15 enabled use case (Urban macro).

| Parameter | Value |
| --- | --- |
| Carrier frequency for evaluation | 4GHz |
| Channel model | TDL-C (delay spread: 300ns) as in 38.901 |
| UE speed | 3 km/h for Rel-15 enabled use case |
| BS antenna configuration | 2 Tx/4 Rx antenna ports |
| UE antenna configuration | 2 Tx/4 Rx antenna ports |
| System bandwidth | 40 MHz |
| Sub-carrier spacing | 30 kHz |
| Channel estimation | Practical |
| Receiver type | MMSE |

Figure 2. PUCCH format 0 error rate and PDCCH BLER as function of SNR.

Figure 3. BLER for 4OS-data with different MCSs (from Table 5.1.3.1-3 [3]) as function of SNR.

## Total reliability for URLLC Rel-15 enabled use case

We evaluate the overall reliability of a single DL and configured grant (CG) UL transmission as follows.

(1)  BLER after one DL transmission = Pr(PDCCH failure) + Pr(PDCCH success)*Pr(PDSCH failure)

(2)  BLER after one CG-UL transmission = Pr(PUSCH failure)

From the results in Figures 1-3, it can be seen that at the 5th%-tile DL SINR (1.69 dB), Pr(PDCCH failure) can be made lower than 1E-5 using AL4 or higher and Pr(PDSCH failure) can be made lower than 1E-5 using MCS9 or lower. Thus, BLER after one DL transmission according to (1) can be made lower than 1E-5.

Similarly, it can be seen that at the 5th%-tile UL SINR (0.92 dB), Pr(PUSCH failure) can be made lower than 1E-5 using MCS9 or lower.

That is, there exist several scheduling options for a single DL and CG-UL transmission with 4-os duration which fulfill the reliability requirement, i.e., achieving overall BLER of 1E-5 at the 5th%-tile SNR. Moreover, based on latency analysis in [4], it is found that single DL and single CG-UL transmissions with 4os, 30 kHz SCS are both possible within 1ms latency bound. Thus, the URLLC Rel-15 enabled use case requirement (macro scenario) of 1E-5 error rate within 1ms latency can be met.

- The reliability target of 99.999% (total BLER of 1E-5) can be met at the 5th%-tile SINR for DL and configured grant UL with a single transmission using MCS9 or lower.

- URLLC Rel-15 enabled use case requirement can be met, i.e., 1E-5 error rate within 1ms for a packet size of 32 bytes.

# Conclusion

In section 2 we made the following observations:

Observation 1	The cell-edge SINR for URLLC Rel-15 enabled use case (macro scenario) is approximately 1.69 dB (DL) and 0.92 dB (UL).

Observation 2	The reliability target of 99.999% (total BLER of 1E-5) can be met at the 5th%-tile SINR for DL and configured grant UL with a single transmission using MCS9 or lower.

Observation 3	URLLC Rel-15 enabled use case requirement can be met, i.e., 1E-5 error rate within 1ms for a packet size of 32 bytes.

# References

- 3GPP RAN1#94bis Chairman note

- 5D/TEMP/216-E, Preliminary draft new report ITU-R M., [IMT-2020.TECH PERF REQ], ITU-R Working Party 5D, (WP5D-TD-0300).

- 3GPP TS 38.214 V15.3.0

- R1-1900175 Latency Evaluation of Rel-15 URLLC, Ericsson, January 2019