3GPP TSG RAN WG1 Ad-Hoc Meeting 1901	R1-1900671

Taipei, Taiwan, 21st – 25th January, 2019

Agenda Item:	7.2.4.1.4

Source: 	NEC

Title:	Resource allocation mechanism for NR V2X

Document for:	Discussion/Decision

Introduction

NR-V2X SI was approved in the RAN#80 meeting [1], in the last RAN1 meeting, resource allocation mechanism was discussed with following agreements [2]:

- Agreements:

Sensing procedure is defined as SCI decoding from other UEs and/or sidelink measurements

  - FFS information extracted from SCI decoding

  - FFS sidelink measurements used

  - FFS UE behavior and timescale of sensing procedure

  - Note: It is up to further discussion whether SFCI is to be used in sensing procedure

  - Note: Sensing procedure can be discussed in the context of other modes

Resource (re)-selection procedure uses results of sensing procedure to determine resource(s) for sidelink transmission

  - FFS timescale and conditions for resource selection or re-selection

  - FFS resource selection / re-selection details for PSCCH and PSSCH transmissions

  - FFS details for PSFCH (e.g. whether resource (re)-selection procedure based on sensing is used or there is a dependency/association b/w PSCCH/PSSCH and PSFCH resource)

  - FFS impact of sidelink QoS attributes on resource selection / re-selection procedure

For Mode-2(a), the following schemes for resource selection are evaluated, including

  - Semi-persistent scheme: resource(s) are selected for multiple transmissions of different TBs

  - Dynamic scheme: resource(s) are selected for each TB transmission

- Agreements:

Mode-2(b) to be studied as a functionality that can be a part of Mode-2(a)(c)(d) operation, when one UE assists sidelink resource selection for other UE(s)

Note: Mode-2(b) is not supported/studied as a standalone sidelink resource allocation mode

- Agreements:

  - For out of coverage operation, Mode-2(c) assumes (pre)-configuration of single or multiple sidelink transmission patterns (patterns are defined on each sidelink resource pool).

  - For in-coverage operation, Mode-2(c) assumes that gNB configuration indicates single or multiple sidelink transmission patterns (patterns are defined on each sidelink resource pool)

  - FFS pattern design in time and frequency for periodic and aperiodic traffic

  - If single pattern is configured to transmitting UE there is no sensing procedure executed by UE

  - If multiple patterns are configured to transmitting UE there is a possibility of sensing procedure executed by UE

  - Pattern is defined as follows

    - Size of the resource in time and frequency

    - Position(s) of the resource in time and frequency

    - Number of resources

  - FFS pattern selection procedure by UE

- Agreements:

For Mode-2(d) operation, further study the following potential radio-layer procedures including at least the following

  - Procedures to become/serve as a scheduling UE for in-coverage and out-of-coverage scenarios

    - The following options are identified for further study:

      - Scheduling UE is configured by gNB

      - Application layer or pre-configuration selects scheduling UE

      - Receiver UE schedules transmissions of the transmitter UE during the session

      - Scheduling UE is decided by multiple UEs including the one that is finally selected

        - UE may autonomously decide to serve as a scheduling UE (self-nomination) / offer scheduling UE functions

- Agreements:

Initialization of Mode-2(d) operation is FFS

For Mode-2(d) operation, further study the following potential radio-layer procedures including at least the following

  - Procedure to determine a set of sidelink resources a scheduling UE can use for scheduling of other UEs

    - The following options are identified:

      - Based on sensing procedure by scheduling UE

      - Configured by gNB if scheduling UE is in-coverage

      - Pre-configured if scheduling UE is out of coverage

      - Transmitting UE provides information about sidelink resources to scheduling UE

  - FFS behavior/algorithm of scheduling UE

  - Behavior of scheduling UE to signal scheduling decisions for transmission/reception of other UEs

    - The following options are identified:

      - Physical layer signaling

      - Higher layer signaling

  - UE behavior to (re)-select scheduling UE(s)

  - UE behavior to associate to scheduling UE(s)

  - UE behavior when scheduling UE stop scheduling

  - Resource management to address collision/interference and half-duplex issues b/w UEs scheduled by different scheduling UEs

  - Relationship between scheduling UE and UE groups from upper layer perspective

    - Whether UEs from the same upper layer group are served by the same scheduling UE

  - Resources used for communication before UE is associated with a scheduling UE

  - Procedures to switch between Mode-2(d) from/to other submodes

In this contribution, we will discuss and give our views on the issues that need to be considered for resource allocation mechanism in NR-V2X.

Discussion

Mode 2(a)

NR V2X is supposed to complement LTE V2X and targeted to support the traffics that LTE V2X cannot support. These traffics may include both periodic and aperiodic. Furthermore, depending on some regional policy, NR V2X and LTE V2X may not be deployed simultaneously, which implies that NR V2X may also need to support the traffic that originally supposed to be supported by LTE V2X. The autonomous resource selection mechanism defined in Rel-14 LTE-V2X is tailored for periodic traffic, and it has been justified that the performance of this mechanism is far beyond DSRC in the real network. Hence, it is desirable that LTE-V2X autonomous resource selection mechanism can be supported in NR V2X, such that periodic traffic can be well served with minimal specification efforts. If LTE Mode 4 is supported in NR V2X, to ensure S-RSSI measurement used in LTE Mode 4 is accurate, periodic traffic and aperiodic traffic should be separated in different resource pools.

Proposal 1:

- LTE-V2X autonomous resource selection mechanism (LTE Mode 4) should be supported for periodic traffic in NR V2X.

Sensing procedure is used to identify resources reserved by other UEs or resources with high interference, such that these resources can be precluded in the resource (re-)selectin procedure. In the last RAN1 meeting, it is agreed that sensing procedure in NR V2X includes at least SCI decoding from other UEs. In our view, based on SCI decoding the sensing UE should at least identify the resources that have been reserved by other UEs in the upcoming slots or upcoming symbols. The sensing procedure is highly related to the location of the decoded SCI and reserved resources indicated in the decoded SCI. Basically, there are four possible cases on resource reservation indicated by SCI:

- Case 1: resources reserved semi-statically;

- Case 2: resources reserved for retransmission of a TB;

- Case 3: resources reserved dynamically for initial transmission of a TB in upcoming slot;

- Case 4: resources reserved dynamically for initial transmission of a TB in the same slot;

Figure 1 4 cases on resource reservation indicated in the decoded SCI

Case 1may occur when there is a UE with semi-persistent transmission in the resource pool (although we prefer dedicated resource pool for semi-persistent transmission). In this case, SCI should convey information of resource reservation period and also indicate how many periods the resource will be reserved, similar as LTE Mode 4.

In Case 2 one TB is transmitted multiple times, and the SCI indicates the resources for both initial transmission and retransmissions.

In Case 3, it is assumed that a new SCI format is defined for reservation indication, the SCI is transmitted several slots before the associated PSSCH. The new SCI enables other UEs to identify the reserved resource in advance, however it may create new problems. E.g. the PSCCH carrying the new SCI and the associated PSSCH may be transmitted in different slot, to avoid the AGC issue, the PSCCH should occupy the entire slot, which implies a new PSCCH structure. Moreover, as the performance of the sensing procedure in this case is highly dependent on the decoding of the new SCI, an effective way to avoid resource collision and half-duplex constraint on the resources for the PSCCH carrying the new SCI is indispensable.

In case 4 sensing UE identifies the reserved resources based on SCI transmitted in the same slot. In this case, if the symbols after the SCI is not reserved by other UEs, in order to use the resource in the symbols, additional TX/Rx switching time (to switch from sensing to transmission), additional AGC settling time, and additional SCI transmission occasion are necessary.

Furthermore, a reserved resource can also be reused by another UE if the energy on the resource is low, hence, sidelink measurement should be performed to evaluate the energy on the reserved resource. In Case 1 and Case 2, sidelink measurement can be performed on the PSSCH transmitted in the SCI slot. In Case 3 and Case 4, sidelink measurement on the decoded SCI can be used to estimate the energy on the reserved resources. The time scale for sensing depends on which case is supported, e.g. in Case 1, the sensing time scale should be as long as the transmission period of the UE using semi-persistent transmission such that all semi-persistent reservation can be identified.

Proposal 2:

- SCI decoding should at least provide resource reservation information of other UEs;

- Sidelink measurement should be performed during sensing procedure;

- Further study which one or some of cases on resource reservation indicated by SCI should be supported in NR V2X.

Mode-2(c)

In Mode-2 (c), a UE is configured with a pattern consisting of multiple resources semi-statically, in this case, mode-2(c) can be considered as one implementation of Mode 1. If the UE is out of coverage, the pattern can only be pre-configured or autonomously selected by the UE, in that sense, mode-2(c) looks more like a resource pattern design rather than a resource selection mechanism. Anyhow, as the size of configured pattern is fixed, this scheme is preferable to be used for resource allocation of periodic traffic or control channel.

Proposal 3:

- Mode-2(c) should be supported as one option of Mode 1 for resource allocation of periodic traffic or control channel.

Mode-2(d)

In platooning, the vehicles are moving with similar speed and trajectory [3], during a platooning is operational, the relative location of the group of vehicles are almost stable, the lead vehicle of the platoon can schedule the transmission and reception of the platoon members. Especially for the intra platooning communication, since the information exchanged during intra group communication is exclusive to group members, a dedicated set of resources can be reserved for a platoon, and up to the lead vehicle to schedule platoon members within the resource set. However, in normal V2X communication scenarios, the speed and trajectory of vehicles are independent, and the topology of vehicles may not be stable, forming, maintenance, reforming of a scheduling group may not be feasible anytime/anywhere.

Proposal 4:

- Mode-2(d) can be supported for platooning, FFS other scenarios.

In the last meeting 4 options to become/serve as a scheduling UE are agreed for further study. In Mode-2 (d) scheduling UE controls the transmission and reception of a group of UEs, the scheduling algorithm of the scheduling UE may impact the system performance, furthermore, the scheduling algorithm has to guarantee that all UEs in the group are fairly scheduled, different QoS levels are supported, and network configuration e.g. congestion control, is respected, etc.. It seems extremely complicated to specify the scheduling algorithm to satisfy all the requirements.  If the scheduling algorithm is not specified or partially specified, some authentication process is necessary before allowing a UE to become as a scheduling UE, the authentication process can be performed by gNB, application layer of UEs, or in pre-configuration. If this is the case, a UE should not be allowed to be self-nominated as a scheduling UE.  As to the third option, receiver UE schedules the transmission of the transmitter UE, the benefit is not clear, especially considering that the channel status at the receiver side and the transmitter side may not be the same, and the scheduling of the receiver may conflict with other transmissions of the transmitter if the transmitter has multiple sessions.

Proposal 5:

- Scheduling behaviour of scheduling UE is not specified;

- Scheduling UE should be configured by gNB, or selected by application layer or pre-configuration, FFS other procedures to become/serve as a scheduling UE.

As agreed in the last meeting, the set of resources for scheduling UE to use can be either semi-statically (pre-) configured, or dynamically selected. The semi-static schemes can ensure that the set of resources are dedicated for the scheduling UEs to control. However, in the dynamic schemes, the set of resources are selected either based on sensing of scheduling UE (Option 1) or information provided by transmitting UE (Option 4). As scheduling UE and transmitting UE are not collocated, the sensing results at the scheduling UE may not be consistent with the sensing results at the transmitting UE, Option 1 seems problematic.  Option 4 may significantly increase the overhead on feedback channel if sufficient information is provided. In view of above, in contrast to Mode-2(a), the advantage of Mode-2(d) is not clear.

Proposal 6:

- The set of resources a scheduling UE can use for scheduling of other UEs should be semi-statically configured or pre-configured.

Conclusion

In this contribution, we discussed the issues that need to be considered for resource allocation in NR-V2X, we have following proposals:

Proposal 1:

- LTE-V2X autonomous resource selection mechanism (LTE Mode 4) should be supported for periodic traffic in NR V2X.

Proposal 2:

- SCI decoding should at least provide resource reservation information of other UEs;

- Sidelink measurement should be performed during sensing procedure;

- Further study which one or some of cases on resource reservation indicated by SCI should be supported in NR V2X.

Proposal 3:

- Mode-2(c) should be supported as one option of Mode 1 for resource allocation of periodic traffic or control channel.

Proposal 4:

- Mode-2(d) can be supported for platooning, FFS other scenarios.

Proposal 5:

- Scheduling behaviour of scheduling UE is not specified;

- Scheduling UE should be configured by gNB, or selected by application layer or pre-configuration, FFS other procedures to become/serve as a scheduling UE.

Proposal 6:

- The set of resources a scheduling UE can use for scheduling of other UEs should be semi-statically configured or pre-configured.

References

- RP-181429, New SID: Study on NR V2X;

- Chairman's Notes RAN1#95;

- 3GPP TR 22.886 V16.0.0, “3rd Generation Partnership Project; Technical Specification Group Services and System Aspects; Study on enhancement of 3GPP Support for 5G V2X Services (Release 16)”