# 3GPP TSG RAN WG1 Ad-Hoc Meeting 1901						R1-1900685

# Taipei, Taiwan, 21st – 25th January, 2019

# Source:	ETRI

# Title:		Discussion on NR V2X sidelink synchronization

# Agenda item:	7.2.4.1.3

# Document for:	Discussion/Decision

Introduction

The following agreements on the NR V2X sidelink synchronization were made in the 3GPP RAN1#94bis meeting [1]:

Agreements:

- The design of NR V2X sidelink synchronization signals and PSBCH uses NR SSB structure as the starting point with the following properties,

  - NR V2X synchronization signals include sidelink PSS (S-PSS) and sidelink SSS (S-SSS) and are structured with PSBCH in a block format (S-SSB)

Agreements:

- Periodic transmission of S-SSB in NR V2X is supported

  - FFS:  whether one/more S-SSB is transmitted in a period

More agreements were made in the 3GPP RAN1#95 meeting [2]:

Agreements:

- For evaluation of V2X S-SSB, the transmission bandwidth of S-SSB is in proportion to the SCS for the design of V2X S-SSB.

  - Alt1: 24 PRBs

  - Alt2: 20 PRBs

  - Other values are not precluded

Agreements:

- The aspects of synchronization sequence for NR V2X to be considered for the evaluation include,

  - The length of S-PSS and S-SSS sequences

  - If and how to distinguish from NR Uu PSS and SSS sequences

  - The number of NR SL-SSID targeted in the design of NR V2X S-PSS/S-SSS

    - Use cases of NR SL-SSID should be addressed

In this contribution, we discuss some considerations in designing NR V2X sidelink synchronization mechanism.

Discussions

Sidelink SSB structure

For NR V2X study, it was agreed to employ mmWave frequency bands (FR2) such as around 30 GH and/or 63 GHz for the V2I, V2V, V2P links [3]. It is well known that such mmWave signals are prone to blockage. Hence, the NR V2X study will consider vehicle-induced blockage, namely NLOSv as well as LOS cases [4]. If a vehicle UE is not able to obtain timing reference from the gNodeB due to such blockage, nearby in-coverage vehicle UEs can assist synchronization by sending sidelink synchronization signals (SLSS) and broadcast channel (SL-BCH) such as PSLSS, SSLSS, and PSBCH. In addition, by reusing the NR downlink SSB (DL-SSB) structure, this synchronization procedure can be involved in multi-beam operation so that the out-of-coverage vehicle UE can select the best beam containing the sidelink synchronization signal block (SL-SSB) using the periodic SL-SSB transmission structure, as seen in Figure 1.

Proposal 1: In NR V2X periodic SL-SSB structure, multiple SL-SSBs are transmitted in a period at least FR2.

Figure 1. SS block structure for sidelink synchronization

Regarding the transmission bandwidth of SL-SSB, there will be several advantages to have the same bandwidth between the SL-SSB and DL-SSB, i.e., 20 PRBs. First, the UE can reuse the DL-SSB detection module for SL-SSB. Second, since the payload of the SL-BCH is not considered to be large, the bandwidth of 20 PRBs will be sufficient. Third, the same bandwidth between SL-SSB and DL-SSB enables efficient multiplexing between them.

Proposal 2: For NR V2X, the transmission bandwidth of SL-SSB should be the same as that of DL-SSB, i.e., 20 PRBs.

If SL-SSB is transmitted in addition to DL-SSB in the same cell area, a multiplexing scheme between DL-SSB and SL-SSB should be studied. Even though the SL-SSB is intended for the out-of-coverage UEs that cannot detect DL-SSB, the in-coverage UEs will receive both the DL-SSB and SL-SSB due to their broadcast nature. There will be several options, namely time-division multiplexing (TDM), frequency-division multiplexing (FDM), and FDM with time-domain offset (i.e., a combination of TDM and FDM), as seen in Figure 4. The TDM scheme allows the transmission of SL-SSB at the same frequency resources with DL-SSB. In this case, the UEs only need to search the common frequency region for the downlink and sidelink synchronizations. However, it will not be possible to configure short SSB burst periodicity such as 5 ms, leading to collision between DL-SSB and SL-SSB. The FDM scheme allocates DL-SSB and SL-SSB in the different frequency resources. In this case, the UE need to search both frequency regions at the same time, making the UE receiver implementation more complex. Another option will be a hybrid of TDM and FDM. The UEs search different frequency regions at the different time.

Figure 4. Multiplexing of DL-SSB and SL-SSB: a) TDM; b) FDM; c) FDM with time-domain offset

Proposal 3: NR V2X should investigate a multiplexing scheme between DL-SSB and SL-SSB, with the following options:

  - TDM

  - FDM

  - FDM with time-domain offset

Sidelink SSB waveform

Basically, NR supports both CP-OFDM and DFT-s-OFDM for the waveform in the uplink. CP-OFDM is advantageous for achieving high spectral efficiency while DFT-s-OFDM is advantageous for improving power efficiency. However, supporting both waveforms for the sidelink SSB is not beneficial due to the increased UE complexity and latency occurring from the blind detection of the waveform. This problem is particularly significant to V2X applications with tight latency requirements. Hence, regardless of whether other sidelink channels (e.g., PSCCH, PSSCH) supports both waveforms, only single waveform should be supported for the sidelink SSB.

Proposal 4: NR V2X should support only single waveform for the sidelink SSB.

Conclusion

In this contribution, we discussed NR V2X sidelink synchronization methods with special emphasis on mmWave applications. Our observations and proposal are reproduced below:

Proposal 1: In NR V2X periodic SL-SSB structure, multiple SL-SSBs are transmitted in a period at least FR2.

Proposal 2: For NR V2X, the transmission bandwidth of SL-SSB should be the same as that of DL-SSB, i.e., 20 PRBs.

Proposal 3: NR V2X should investigate a multiplexing scheme between DL-SSB and SL-SSB, with the following options:

  - TDM

  - FDM

  - FDM with time-domain offset

Proposal 4: NR V2X should support only single waveform for the sidelink SSB.

References

- 3GPP Chairman’s notes RAN1#94, Aug. 2018.

- 3GPP Chairman’s notes RAN1#95, Nov. 2018.

- RP-181429, “New SID: Study on NR V2X,” Jun. 2018.

- 3GPP TR 37.885, “Study on evaluation methodology of new Vehicle-to-Everything V2X use cases for LTE and NR,” V 15.0.0, Jun. 2018.