3GPP1 TSG RAN WG1 		Tdoc TSG RAN WG1  9/99

Meeting # 1

Espoo, Finland

January 21-22, 1999

Source: 	Nokia

Title: 	Proposal for merging ETSI and ARIB Layer 1 documents to one common 3GPP Layer 1 document

# Introduction

The intention of this paper is to help the procedure of generating one common 3GPP Layer 1 specification document by combining two source documents: "UTRA Physical Layer Description"  from ETSI UMTS-L1 and "Volume 3, Specifications of Air Interface for 3G Mobile" from ARIB SWG2. A lot of effort has been put in making both of the source documents, and for that reason we should utilise both of their contents in such a way that the end result is as comprehensive as possible.

This paper contains a comparison of ETSI and ARIB Layer 1 documents chapter by chapter, and item by item, clarifying the differences and giving a suggestion what could be done with each section.

# Classification of document comparison results

The comparison of "UTRA Physical Layer Description"  from ETSI and "Volume 3, Specifications of Air Interface for 3G Mobile" from ARIB is shown in the next section. The comparison is divided into tables according to ETSI XX-documentation division. For each item a short description of ETSI and ARIB document contents is given. In the last column there is a classification of the comparison result. The interpretation of the classification is following:

a) Same contents

This means that the main technical contents is the same in both ETSI and ARIB documents. However, since the text may be more complete in the other document, both documents should be used as a source in the merging process. Thus it is proposed that the editor rules are such, that the editor-in-charge should take the main text from that document which is more complete, and add the possible missing parts from the other document.

b) Differences & Suggestion

This means that there are some differences between the technical contents of ETSI and ARIB document. The main differences are listed. Additionally, a suggestion is given what could be done to the differences when merging the ETSI and ARIB document into 3GPP document. In case the suggestion is not accepted with the whole group, the alternative proposal is that both ETSI and ARIB contents for that particular item will be put to square brackets in the first version of the 3GPP L1 document, and the decision between them will be made later.

c) ETSI contents only / ARIB contents only

This means that only one of the documents, ETSI or ARIB, contains this particular item. In this case we propose that the contents will be directly taken to the 3GPP document.

# 3.  ETSI and ARIB Layer 1 document comparison

Table 1. Transport and physical channels (FDD)

|  |  | ETSI, XX.03 | ARIB Vol. 3, v.1.0-0.5 | Classification of comparison result |
| --- | --- | --- | --- | --- |
| 1 | Transport channels | 4. Transport channels Dedicated: DCH Common: BCH,FACH, PCH,RACH | 3.2.1.2 Transport channels Dedicated: DCH Common: BCH, FACH, PCH,RACH | Same contents |
| 2 | Physical channel configuration | 5. Physical channels - Primary CCPCH (1) - Secondary CCPCH (2) - PRACH (2) - DPDCH (3) - DPCCH (3) - SCH (1) | 3.2.1.4 Physical channel configuration - Perch (1) * - Common physical channel (2) - Dedicated physical channel (3) * the numbers (1),(2),(3) show the correspondent channels in ARIB and ETSI | Differences - terms are different - ETSI : SCH code multiplexing - ARIB: SCH time multiplexing into Perch Suggestion - Select terms - Decision is needed about SCH multiplexing |
| 3 | Dedicated uplink physical channels | 5.2.1 Dedicated uplink physical channels DPDCH: data DPCCH: pilot / tpc / tfci - SF=256…4 - number of pilot, tfci and tpc bits are not defined | 3.2.2.4.2 Reverse link dedicated physical channels DPDCH: (pilot/tpc/) data DPCCH: pilot / tfci / tpc - SF=256…1 - number of pilot, tfci and tpc bits are defined | Differences Suggestion ETSI: - SF range ARIB: - number of bits for different fields - order of fields |
| 4 | Dedicated downlink physical channels | 5.3.1 Dedicated downlink physical channels pilot / tpc / tfci / data - SF=256…4 - number of pilot, tfci and tpc bits are not defined | 3.2.2.4.1 Forward Link Dedicated Physical channels tfci / data1 / tpc / data2 / pilot - SF=256…1 - number of pilot, tfci and tpc bits are defined | Differences Suggestion ETSI: - SF range ARIB: - number of bits for different fields - order of fields |
| 5 | Multicode transmission | 5.2, 5.3 Dedicated physical channels in uplink, downlink Uplink: same as in ARIB Downlink: - L1 control is put on only the first downlink DPCH | 3.2.6.8.2 Multicode transmission Uplink: same as in ETSI Downlink: - L1 control is put on only the first downlink DPCH - Power of L1 control part can be different than the data part | Same contents - main structure of multicode usage ARIB contents only - possibility to have smaller power level for L1 control part than data part |
| 6 | Physical random access channel | 5.2.2.1 PRACH - preamble/idleperiod/message 1 ms / x ms / 10 ms - message: I:data, Q:pilot+tfci - power ramping for preamble - 8 access slots, 16 signatures -Signature + tfci together define the short code and spreading factor for message | 3.2.2.3.2 Reverse link common physical channel - 10 ms message, without preamble - I:data, Q: signature - no power ramping -4 access slots, 16 signatures -32 kbit/s or 128 kbit/s | Differences - preamble - power ramping Suggestion - ETSI' s RACH structure, since use of preamble and power ramping is probably needed |
| 7 | Primary CCPCH (ETSI) vs. Perch (ARIB) | 5.3.2.1 Primary CCPCH slot structure in symbols, SF=256: pilot / data 4 / 6 | 3.2.2.2 Perch channel slot structure in symbols, SF=256: Nsch / Ndata /Npilot 1 / 5 / 4 | Differences - Time multiplexing of SCH symbol into this channel or not - order of fields Suggestion - decision is needed first about SCH multiplexing, then about order of fields |
| 8 | Secondary CCPCH (ETSI) vs. FL common physical channel (ARIB) | 5.3.2.2 Secondary CCPCH slot structure in symbols (SF=256…4): pilot / data Npilot / Ndata - rate and SF is broadcasted on BCH => cell specific rate | 3.2.2.3.1 FL common physical ch. slot structure in symbols (SF=64): data / pilot 36 / 4 | Differences - Number of bits - Order of fields - fixed vs. cell specific bit rate Suggestion - take ETSI structure, its probably good to have different bit rates |
| 9 | Synchronisation channel | 5.3.2.3 Synchronisation channel - primary and secondary SCH transmitted in parallel to all other channels - otherwise format is the same as in ARIB | 3.2.2.2 - primary and secondary SCH are included in the perch channel - otherwise the format is the same as in ETSI | Same contents - main structure and idea of synch channel Differences - SCH multiplexing scheme : time or code multiplexing Suggestion - Decision needed about SCH multiplexing |
| 10 | Mapping of transport channels onto physical channels | 6. Mapping of transport channels onto physical channels Phys.ch Transp.ch Primary CCPCH --- BCH Secondary CCPCH --- PCH, FACH PRACH --- RACH DPDCH,DPCCH --- DCH SCH --- | 3.2.1.5 Mapping of transport channels to physical channels Phys.ch Transp.channels Perch --- BCH Common physical channel --- PCH,FACH,RACH Dedicated physical Channel ---- DCH | Differences - Mapping of RACH and SCH is different. Suggestion - use ETSI mapping for RACH - decision needed about SCH mapping |
| 11 | Timing relationship between physical channels | 7. ETSI 3.2.2.1 ARIB - Tcell ----- Tsect (different timing for SCH in different sectors) - Td ----- Tcpch (avoid matched tx patterns in common channels) - Td ----- Tframe (averaging fixed line ATM transmission) - Td ------ Tslot (average interference from dedicated channels) - Tm ------ Tdho (timing needed in soft handover) - To ------ 250 us (timing difference between DL and UL DPCH) - ------ Toffset (method to get several DL scrambling codes) |  | Differences - different names, idea is the same Suggestion - take ETSI names for simplification ARIB contents only - Toffset parameter in order to get multiple DL scrambling codes |

Table 2. Multiplexing, channel coding and interleaving (FDD)

|  |  | ETSI, XX.04 | ARIB Vol. 3, v.1.0-0.5 | Classification of comparison result |
| --- | --- | --- | --- | --- |
| 1 | Transport channel multiplexing | 7.2 Transport channel coding/multiplexing Following order is used: - Add CRC per Tr. block - channel coding - 1st interleaving - rate matching - service multiplexing - 2nd interleaving - mapping to physical channels | 3.2.3.1.1. Multiplexing of different QoS services Following order is used: - Transport channel mux - channel coding - service specific rate matching - interleaving, (MIL, 10-80 ms) - radio frame segmentation - service multiplexing - physical channel rate matching - physical channel FIFO mapping | Differences - combined vs. 2 state rate matching - order of 1st interleaving and rate matching - interleaver structures Suggestion - take ETSI structure, since this has been discussed a long time there and discussion is still going on |
| 2 | Channel coding | 7.2.1 channel coding convolutional coding - BER=10e-3 services - rate 1/3, ½, constraint length 9 turbo coding - BER=10e-6 services, and data rates >32 kbit/s - 8 state parallel concatenated convolutional code - conventional termination - interleaver TBD - not defined that several/segmented frames can be combined for turbo coding | 3.2.3.2.2 Error correcting coding convolutional coding - ½ rate: BCH, PCH,FACH,RACH - 1/3 rate: <32 kbit/s DCH turbo coding - 1/3 rate: >= 32 kbit/s DCH - 4- state parallel concatenated convolutional code - CBP for termination - MIL interleaver - several/segmented frames can be combined for turbo coding | Differences Suggestion ETSI: based on turbo code adhoc group results - Turbo code only for BER=10e-6 services and >=32 kbit/s. Conv. coding for others. - 8 state parallel turbo - conventional termina-tion method for turbo ARIB: - conv. coding rates * ½ rate for BCH, PCH, FACH, RACH * 1/3 rate for DCH - turbo coding block can be combined for seve-ral /segmented frames TBD: - turbo interleaver |
| 3 | Channel interleaver | 7.2.3, 7.2.7 1st and 2nd interleaver - structures are TBD | 3.2.3.3.1 Channel interleaving 1st interleaver: MIL interleaver 2nd interleaver: FIFO mapping | Differences Suggestion - leave it open now since structures are still open in ETSI and discussion is still going on. |
| 4 | Coding for Layer 1 control | 7.4.1 TFI coding - biorthogonal (32,6) coding | 3.2.2.4.1.3 TFCI - biorthogonal (32,6) coding | Same contents |
| 5 | Error detection | - details not defined | 3.2.3.4 Error detection - usage of CRC : retransmission, outer loop power control, selective combinining in uplink soft handover - 16 bit CRC for all logical ch. | ARIB contents only |
| 6 | Coding for slotted mode | 7.5 Coding for slotted mode - either by puncturing or by higher spreading factors | 3.2.6.6.3 inter-frequency handover either by puncturing or by higher spreading factor | Same contents |

Table 3. Spreading and modulation (FDD)

|  |  | ETSI, XX.05 | ARIB Vol. 3, v.1.0-0.5 |  | Classification of comparison result |  |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | Uplink modulation and spreading | 6. Uplink spreading and modulation - BPSK data modulation - different channelisation code for I and Q branch - complex scrambling | 6.Uplink spreading and modulation - BPSK data modulation - different channelisation code for I and Q branch - Gain factor for Q branch - complex scrambling |  | Same contents - main contents Differences - In uplink, the gain factor of Q branch is missing from ETSI. Suggestion - include the gain factor for Q branch in uplink |  |
| 2 | Downlink modulation and spreading | 7. Downlink spreading and modulation - QPSK data modulation - complex scrambling | 3.2.4. 2.1.1 FL Spreading Modulation scheme - QPSK data modulation - complex scrambling |  | Same contents |  |
| 3 | Uplink code generation | 6.2 Uplink Code generation and allocation -Channelisation codes: OVSF - Scrambling codes: HPSK - short scrambling codes: Very large Kasami length 256 - long scrambling codes: same as in ARIB * 10 ms period * quadrature component is 1024 chip shifted - Synchronisation codes: => same as ARIB - Random access codes: HPSK => different from ARIB | 3.2.4.2.2 Code generation - channelisation codes:OVSF codes - Scrambling codes: HPSK method - short scrambling codes:TBD codes of length 256 - long scrambling codes: same as in ETSI * 36864* 10 ms period * quadrature component is 1024 chip shifted version - Synchronisation codes: => same as ETSI |  | Differences - Which code set is used for short scrambling codes - period of UL scrambling code - RACH scheme is different => so will be its spreading Suggestion Take from ETSI: - short scrambling code set - 10 ms UL code period - RACH codes Other parts are same |  |
| 4 | Downlink code generation | 7.2 Downlink code generation and allocation -Channelisation codes: OVSF - Scrambling codes: - 512 codes in 32 code groups - Gold sequences: same as in ARIB * 10 ms period * quadrature component is not yet defined | 3.2.4.2.2.Code generation -Channelisation codes:OVSF codes - Scrambling codes: - 33 code groups - Gold sequences: same as in ETSI * 10 ms period * quadrature component is 875 us shifted version of the in-phase component |  | Same contents - Except that quadrature component for the scrambling code is not yet defined in ETSI |  |
| 5 | Code allocation | 3.2.4.2.3 Code allocation - uplink channelisation code: from top half of the code tree - downlink channelisation code: decided by network - search code :same as ARIB | 3.2.4.2.3 Code allocation - reverse link: minimising peak to average ratio - forward link: TBD - search code: same as in ETSI |  | Same contents - main contents are same |  |
| 6 | Modulation | - chip rates : 4.096, 8.192, 16,384 Mcps - pulse shaping : roll-off 0.22 |  | - chip rates : 1.024, 4.096, 8.192, 16,384 Mcps - pulse shaping : roll-off 0.22 |  | Differences - lowest chip rate differs Suggestion - take ETSI chip rates |

Table 4. Physical Layer Procedures (FDD)

|  |  | ETSI, XX.07 | ARIB Vol. 3, v.1.0-0.5 | Classification of comparison result |
| --- | --- | --- | --- | --- |
| 1 | Power control | 4. Power control Closed loop power control - SIR based for UL and DL, cycle=0.625 ms - step size 0.25-1.5 dB - In soft handover the MS increases its Tx power only if all commands indicate an increase value Outer loop adjusts the SIR target Open loop power control - used only for RACH bursts - based on downlink pathloss and uplink interference | 3.2.6.7. Transmitter power control Closed loop power control - SIR based for UL and DL, Cycle=0.625 ms - Step size UL: 1 dB, DL: 1 dB - In soft handover MS decreases its Tx power if there is at least one decrease command that satisfied required reliability. Reliability is based on SIR. - Downlink power offset possibility Outer loop adjusts the SIR target Open loop power control - used only for RACH bursts - based on downlink pathloss and uplink interference | Differences - step sizes - power offsets - power control in soft handover - level of details is quite different Suggestion Main description from both ETSI and ARIB ETSI: - step sizes TBD later: - DL power offsets - Power control in soft handover |
| 2 | Cell search | 5. Cell search - Initial cell search => same as in ARIB - Idle mode cell search - Active mode cell search | 6.1 Cell detection control - Initial cell search => same as in ETSI - Idle operation/handover => periodical search process | Same contents |
| 3 | Synchronisation procedures | 6. Random access procedure - syncronisation to a cell - read BCH and select parameters for RACH burst etc. | 3.2.6.2. Synchronisation procedures - common ph. ch. synchronisation -dedicated channel synchronisation => defines power control and use of idle patterns during synchronisation phase - soft handover synchronisation - softer handover synchronisation - superframe synchronisation | Differences - ETSI contains only RACH part of the synchronisation Suggestion - RACH part from ETSI - Other synchroni-sation description from ARIB |
| 4 | Idle mode tasks | 7. Idle mode tasks - Paging control | 3.2.6.3 Idling operation - Paging control | Same contents |
| 5 | Rate detection | - | 3.2.6.4 Rate detection - Blind rate detection by CRC - TFCI : mapping of TFCI bits to service rate combinations | ARIB contents only |
| 6 | Quality supervision | - | 3.2.6.5 Quality supervision - SIR definition for MS | ARIB contents only |
| 7 | DTX | - | 3.2.6.9 Discontinuous transmission - DTX within a radio frame - DTX of radio frames | ARIB contents only |
| 8 | Reverse link synchronous transmission | - | 3.2.6.11 Reverse link synchronous transmission - alternative technology for low mobility terminals | ARIB contents only |
| 9 | Asymmetric transmission control | - | 3.2.6.11.2 Asymmetric tx control - independent SFs in UL and DL - Possibility to have UL: RACH and DL: DCH | ARIB contents only |
| 10 | DownlinkTx diversity | Still open in ETSI | 3.2.6.11.4 Forward link TDTD - optional in BS, mandatory in MS - contains predetermined mode and feedback mode | Differences - Discussion still going on in ETSI Suggestion - Put the options in square brackets |

Table 5, Physical Layer procedures, continued (FDD)

|  |  | ETSI, XX.15 | ARIB Vol. 3, v.1.0-0.5 | Classification of comparison result |
| --- | --- | --- | --- | --- |
| 1 | Handover types | 6. Handover types - soft handover - UTRA to UTRA hard handover - UTRA to GSM hard handover | 3.2.6.6 - soft handover - hard handover | Differences Suggestion - Take ETSI types, since they cover also intersystem handovers |
| 2 | Cell sets for handover | 8.2 Contents of Cell sets for handover 1) Handover monitoring set in FDD 2) Handover candidate set 3) Active set | - | ETSI contents only |
| 3 | Set maintenance | 8.3 Set maintenance - MS sends regular (or event driven) measurement reports to network. - Network decides the update of the sets and reports decisions to MS. | 3.2.6.6 Handover - same idea as in ETSI | Same contents |
| 4 | Handover preparation from UTRA to UTRA/ Soft handover | 9.1 Handover preparation from UTRA to UTRA Measurement report includes at least : - cell ID - relative signal strength - relative timing information | 3.2.6.6.1 Diversity handover MS measures: - relative timing difference | Differences ETSI concentrates more on measurement list, ARIB more on synchronisation procedure Suggestion ETSI : measurement list ARIB: synchronisation procedure |
|  | UTRA to UTRA/ Intercell hard handover | - | 3.2.6.6.2 Hard handover Conditions to measure received signal strength of other cells: - if signal strength of own cell is below certain threshold - during DTX period | ARIB contents only |
|  | UTRA to GSM | 9.2 Handover preparation from UTRA to GSM | - | ETSI contents only |
| 5 | Transmission in handover and handover execution procedure | 10.1 Soft handover New cell should get following information through the network - max. data rate and other service parameters - MS ID and UL scrambling code - Relative timing information MS should get following information through own cell - from which frame the new cell initiates the transmission - channelisation codes - relative timing information | 3.2.6.6.1 Diversity handover Synchronisation establishment explained in detail upon soft/softer handover | Differences - ETSI gives a list what information will be changed. ARIB explains the synchronisation procedure step by step. Suggestion ETSI : measurement list ARIB: synchronisation procedure |

# Documents from ETSI

All the TDD documents are proposed to be taken straightly from ETSI, since they have been mainly developed there:

- XX.09, UTRA TDD, Transport and physical channels description

- XX.10, UTRA TDD, Multiplexing, channel coding and interleaving description

- XX.11, UTRA TDD, spreading and modulation description and relevant parts from

- XX.12, UTRA TDD, Radio transmission and reception description

- XX.13, UTRA TDD, Physical Layer procedures description

# Conclusion

This paper was made for a tool for generating the 3GPP Layer 1 documentation by merging ETSI and ARIB Layer 1 documents. For that purpose it contains a comparison of ARIB and ETSI Layer 1 document contents.

In section 2  we have also given our proposal how the editor rules should be defined for generating the first version of 3GPP document. The editor rules should be defined in such a way that the 3GPP document will be as comprehensive as possible right from the beginning.

This means that if the main technical contents is the same in both documents, the editor-in-charge should take the main text from that document which is more complete, and add the possible missing parts from the other document. In that way the end result will be as complete as possible.

ARIB Vol. 3 ver 1.0-0.5 12/98 and ETSI XX-documents after 9th L1 EG meeting 12/98 were used as source documents.