3GPP TSG-RAN WG1 Meeting #100-e	R1-2001014

e-Meeting, February 24th- March 6th, 2020

Agenda Item:	7.2.4.7

Source:	Ericsson

Title:	NR Uu controlling LTE sidelink transmissions

Document for:	Discussion, Decision

# 1	Introduction

This contribution addresses some remaining issues regarding NR Uu controlling LTE sidelink.

# 2	Timing aspects

In RAN1#99 the following agreement was made

Agreements: The supported values of X signaled in the DCI are: - 0.75ms, 1ms, [1.25ms], [1.5ms], 2ms, 4ms, 5ms, 8ms, 10ms, 20 ms - Additional value(s) can be discussed during the Feb. meeting - Spare values are reserved for future deployments

In our view the values 1.25ms and 1.5ms should be supported and no more values are needed.

The supported values of X signaled in the DCI are: 0.75ms, 1ms, 1.25ms, 1.5ms, 2ms, 4ms, 5ms, 8ms, 10ms, 20 ms.

Besides this, there was a discussion on whether to apply a correction to the transmission time to ensure that NR Uu can control LTE sidelink even if each of the RATs uses an independent timing, in the same way as for NR Uu controlling NR sidelink.

In our view, this was already agreed in RAN1#97:

Agreements: - RRC-based activation/deactivation is not supported - DCI-based activation/deactivation is supported - Support of LTE PC5 scheduling by NR Uu (mode 3-like ) is based on UE capability - NR DCI provides the fields of DCI 5A in LTE-V that are related to SPS scheduling - The size of DCI for activation/deactivation is one of the DCI size(s) that will be defined for NR Uu scheduling NR V2V - FFS whether the DCI format is the same as one of the DCI formats that will be defined for NR Uu scheduling NR V2V - Activation/deactivation applies to the first LTE subframe after Z+X ms after receiving the DCI - Z is the same timing offset in current LTE V2X specs - X>0. FFS value(s) of X, and if one or multiple values of X are possible

More specifically, the value for Z is found in 36.213 Section 14.2.1, as copied below:

If the UE receives in subframe n DCI format 5A with the CRC scrambled by the SL-V-RNTI, one transmission of PSCCH is in the PSCCH resource (described in Subclause 14.2.4) in the first subframe that is included in and that starts not earlier than TDL-NTA2×TS+4+m×10-3. is the value indicated by "Lowest index of the sub-channel allocation to the initial transmission" associated with the configured sidelink grant (described in [8]), is determined by Subclause 14.1.5, the value m is indicated by 'SL index' field in the corresponding DCI format 5A according to Table 14.2.1-1 if this field is present and m=0 otherwise, TDL is the start of the downlink subframe carrying the DCI, and NTA and TS are described in [3].

The current agreements already cover the correction to apply to the timing of transmission for ensuring that NR Uu can control LTE sidelink even if they each of the RATs uses an independent timing.

# 3	Specification

We note that the changes to the LTE specifications corresponding to this feature have not been provided or approved (cf. [1]).

The LTE specifications have not been modified for supporting NR Uu controlling LTE sidelink transmissions.

# Conclusion

In the previous sections we made the following observations:

Observation 1	The current agreements already cover the correction to apply to the timing of transmission for ensuring that NR Uu can control LTE sidelink even if they each of the RATs uses an independent timing.

Observation 2	The LTE specifications have not been modified for supporting NR Uu controlling LTE sidelink transmissions.

Based on the discussion in the previous sections we propose the following:

Proposal 1	The supported values of X signaled in the DCI are: 0.75ms, 1ms, 1.25ms, 1.5ms, 2ms, 4ms, 5ms, 8ms, 10ms, 20 ms.

# References

TS 36.213, v16.0.0