3GPP TSG-RAN WG1 Meeting #100-e		R1-2001376

e-Meeting, February 24th – March 6th, 2020

Title:	Reply LS on UL skipping

Response to:	R1-2000163 (R2-1916572)

Release:	Rel-15

Work Item:	NR_newRAT-Core

Source:	RAN1

To:	RAN2

Cc:

Attachments:

# Contact Person:

# Name:	               Xueming Pan

E-mail Address:	panxueming@vivo.com

# 1. Overall Description:

RAN1 would like to thank RAN2 for the LS R1-2000163 regarding dynamic UL skipping. RAN1 has discussedthe issue related to dynamic PUSCH skipping and the handling of overlapping CSI/HARQ-ACK on PUCCH forthe following two cases

Case 1: dynamic PUSCH skipping without overlapping CSI/HARQ-ACK on PUCCH

For case 1, RAN1 reached the following conclusion

•        When a UL grant without UL-SCH field or UL-SCH =1 (if present) is detected by a UE configured with skipUplinkTxDynamic, the corresponding PUSCH transmission is skipped by the UE if no transport block for the PUSCH transmission is generated by MAC and there is no CSI/HARQ-ACK on PUCCH overlapping with the PUSCH.

•        Current RAN1 spec should be corrected, the CR is to be discussed/decided after the conclusion of case 2 is reached.

Case 2: dynamic PUSCH skipping with overlapping CSI/HARQ-ACK on PUCCH

For case 2, a majority of companies in RAN1 agree with the RAN2 common understanding on the desired UE  behavior for UCI handling, as described in the LS R1-2000163. However, no consensus could be made in RAN1#100-e, due to implementation concerns raised from both gNB (increased gNB blind detection) and UE side (change of UCI multiplexing behavior).

# 2. Actions:

RAN1 respectfully asks RAN2 to take the above information into account in their work.

# 3. Date of Next RAN1 Meetings:

TSG RAN WG1 Meeting #100-e-Bis			20 - 24 April 2020       	 				     Online

TSG RAN WG1 Meeting #101			25 - 29 May 2020       	 				     Athens, Greece