3GPP TSG RAN WG1 #100bis                                                                                R1-2001616

e-Meeting, April 20th – 24th, 2020

Source:	ZTE

Title:                 Remaining issues on enhancements for UL configured grant transmission

Agenda item:	7.2.5.6

Document for:  Discussion and Decision

- Introduction

In this contribution, we discuss the remaining issues related to UL CG enhancements, mainly including

- DMRS configuration for Type 2 CG activated by DCI format 0_2.

- Discussion

In the RAN1#100 e-meeting,  there is one remaining issue on DMRS configuration for Type 2 CG in the third email discussion thread [1]. The proposal below was discussed but not concluded due to the potential impact from the discussion on DMRS configuration for DG PUSCH.

- When the Antenna ports field is not present in DCI format 0_2 with CRC scrambled by C-RNTI, and UE in this case shall not follow the parameter cg-DMRS-Configuration in configuredGrantConfig to derive the bit width of the Antenna ports field in DCI format 0_2 with CRC scrambled by CS-RNTI and NDI set to 0. For the determination of the DMRS configuration in this case, - Opt.1: the rules defined in Rel-15 for DMRS determination for Type 2 configured grant when activated by DCI format 0_0 can be reused for DCI format 0_2. - Opt.2: Antenna port(s) are defined assuming bit field index value 0 in Tables 7.3.1.1.2-6 to 7.3.1.1.2-23 of 38.212;

For DG PUSCH, it has been agreed that antenna port(s) are defined assuming bit field index value 0 in Tables 7.3.1.1.2-6 to 7.3.1.1.2-23 of 38.212 when the antenna ports bit filed is not present in DCI 0_2[2]. It’s straightforward to reuse the same rule, i.e., adopting Opt.2 for Type 2 CG PUSCH. This could also bring potential flexibility since the first row of those DMRS tables can be configured differently among different UEs, which could potentially enable MU-MIMO among UEs.

Thus, we have the following proposal:

- Proposal 1: When the Antenna ports field is not present in DCI format 0_2 with CRC scrambled by C-RNTI, a UE in this case shall not follow the parameter cg-DMRS-Configuration in configuredGrantConfig to derive the bit width of the Antenna ports field in DCI format 0_2 with CRC scrambled by CS-RNTI and NDI set to 0. Antenna port(s) are defined assuming bit field index value 0 in Tables 7.3.1.1.2-6 to 7.3.1.1.2-23 of 38.212.

- Given the current text in TS 38.212 can also cover the case for Type 2 CG PUSCH, no text proposal is needed.

- Conclusion

According to the analysis given above, we have the following proposal:

- Proposal 1: When the Antenna ports field is not present in DCI format 0_2 with CRC scrambled by C-RNTI, a UE in this case shall not follow the parameter cg-DMRS-Configuration in configuredGrantConfig to derive the bit width of the Antenna ports field in DCI format 0_2 with CRC scrambled by CS-RNTI and NDI set to 0. Antenna port(s) are defined assuming bit field index value 0 in Tables 7.3.1.1.2-6 to 7.3.1.1.2-23 of 38.212.

- Reference

- R1-2001424, [URLLC-eCG-03] Email discussion/approval on DMRS configuration for Type 2 CG activated by DCI format 0_2, NTT DOCOMO, INC.

- R1-2001404 Summary of email discussion [100e-NR-L1enh_URLLC_PDCCH-01] on remaining issues on DCI format, Huawei.