3GPP TSG RAN WG1 #100bis		R1-2001731

e-Meeting, April 20th – 30th, 2020

Source:	OPPO

Title:	Remaining Issues on DL Positioning Reference Signal

Agenda Item:	7.2.8.1

Document for:	Discussion and Decision

- Introduction

In this contribution, we present our views on various remaining issues of downlink positioning reference signals and provide text proposals for specification update.

- Discussion

  - TS 38.211

The periodicity value of a DL PRS resource takes into account the SCS used by the PRS. The candidate values are 2μ4, 5, 8, 10, 16, 20, 32, 40, 64, 80, 160, 320, 640, 1280, 2560, 5120, 10240, which is captured in TS 38.214 and TS 37.355.  The corresponding configuration in TS 37.355 is copied as follows

NR-DL-PRS-Periodicity-and-ResourceSetSlotOffset-r16 ::= CHOICE {

scs15-r16		CHOICE {

n4-r16					INTEGER (0..3),

n5-r16					INTEGER (0..4),

n8-r16					INTEGER (0..7),

n10-r16					INTEGER (0..9),

n16-r16					INTEGER (0..15),

n20-r16					INTEGER (0..19),

n32-r16					INTEGER (0..31),

n40-r16					INTEGER (0..39),

n64-r16					INTEGER (0..63),

n80-r16					INTEGER (0..79),

n160-r16				INTEGER (0..159),

n320-r16				INTEGER (0..319),

n640-r16				INTEGER (0..639),

n1280-r16				INTEGER (0..1279),

n2560-r16				INTEGER (0..2559),

n5120-r16				INTEGER (0..5119),

n10240-r16				INTEGER (0..10239),

...},

scs30-r16		CHOICE {

n8-r16					INTEGER (0..7),

n10-r16					INTEGER (0..9),

n16-r16					INTEGER (0..15),

n20-r16					INTEGER (0..19),

n32-r16					INTEGER (0..31),

n40-r16					INTEGER (0..39),

n64-r16					INTEGER (0..63),

n80-r16					INTEGER (0..79),

n128-r16				INTEGER (0..127),

n160-r16				INTEGER (0..159),

n320-r16				INTEGER (0..319),

n640-r16				INTEGER (0..639),

n1280-r16				INTEGER (0..1279),

n2560-r16				INTEGER (0..2559),

n5120-r16				INTEGER (0..5119),

n10240-r16				INTEGER (0..10239),

n20480-r16				INTEGER (0..20479),

...},

scs60-r16		CHOICE {

n16-r16					INTEGER (0..15),

n20-r16					INTEGER (0..19),

n32-r16					INTEGER (0..31),

n40-r16					INTEGER (0..39),

n64-r16					INTEGER (0..63),

n80-r16					INTEGER (0..79),

n128-r16				INTEGER (0..127),

n160-r16				INTEGER (0..159),

n256-r16				INTEGER (0..255),

n320-r16				INTEGER (0..319),

n640-r16				INTEGER (0..639),

n1280-r16				INTEGER (0..1279),

n2560-r16				INTEGER (0..2559),

n5120-r16				INTEGER (0..5119),

n10240-r16				INTEGER (0..10239),

n20480-r16				INTEGER (0..20479),

n40960-r16				INTEGER (0..40959),

...},

scs120-r16		CHOICE {

n32-r16					INTEGER (0..31),

n40-r16					INTEGER (0..39),

n64-r16					INTEGER (0..63),

n80-r16					INTEGER (0..79),

n128-r16				INTEGER (0..127),

n160-r16				INTEGER (0..159),

n256-r16				INTEGER (0..255),

n320-r16				INTEGER (0..319),

n512-r16				INTEGER (0..511),

n640-r16				INTEGER (0..639),

n1280-r16				INTEGER (0..1279),

n2560-r16				INTEGER (0..2559),

n5120-r16				INTEGER (0..5119),

n10240-r16				INTEGER (0..10239),

n20480-r16				INTEGER (0..20479),

n40960-r16				INTEGER (0..40959),

n81920-r16				INTEGER (0..81919),

...},

...

}

However, in the specification text in TS 38.211, the description of TperPRS is not aligned with the counterparts of TS 38.214 and TS 37.355.

Proposal 1: Correct the periodicity values of a DL PRS resource to be aligned with TS 38.214 and TS 37.355

The following gives the corresponding text proposal for Proposal 1.

In TS 38.211 Section 7.4.1.7 <omitted text> ##### 7.4.1.7.4 Mapping to slots in a downlink PRS resource set For a downlink PRS resource in a downlink PRS resource set, the UE shall assume the downlink PRS resource being transmitted when the slot and frame numbers fulfil Nslotframe,μnf+ns,fμ-ToffsetPRS-Toffset,resPRS mod 2μTperPRS∈iTgapPRSi=0TrepPRS-1 and one of the following conditions are fulfilled: - the higher-layer parameter DL-PRS-MutingPattern is not provided; - the higher-layer parameter DL-PRS-MutingPattern is provided and bitmap b1 but not bitmap b2 is provided, and bit bi1 is set; - the higher-layer parameter DL-PRS-MutingPattern is provided and bitmap b2 but not bitmap b1 is provided, and bit bi2 is set; - the higher-layer parameter DL-PRS-MutingPattern is provided and both bitmaps b1 and b2 are provided, and both bit bi1 and bi2 are set. where - bi1 is bit i=Nslotframe,μnf+ns,fμ-ToffsetPRS-Toffset,resPRS 2μTmutingPRSTperPRSmod L in the bitmap given by the higher-layer parameter DL-PRS-MutingPattern where L∈2, 4, 8, 16, 32 is the size of the bitmap; - bi2 is bit i=Nslotframe,μnf+ns,fμ-ToffsetPRS-Toffset,resPRS mod 2μTperPRS TgapPRS mod TrepPRS in the bitmap given by the higher-layer parameter DL-PRS-MutingPattern; - the slot offset ToffsetPRS∈0,1,…,TperPRS-1 is given by the higher-layer parameter DL-PRS-ResourceSetSlotOffset; - the downlink PRS resource slot offset Toffset,resPRS is given by the higher-layer parameter DL-PRS-ResourceSlotOffset; - the periodicity TperPRS∈2μ4, 5, 8, 10, 16, 20, 32, 40, 64, 80, 160, 320, 640, 1280, 2560, 5120, 10240 is given by the higher-layer parameter dl-PRS-Periodicity-and-ResourceSetSlotOffset-r16; - the repetition factor TrepPRS∈1,2,4,6,8,16,32 is given by the higher-layer parameter DL-PRS-ResourceRepetitionFactor; - the muting repetition factor TmutingPRS is given by the higher-layer parameter DL-PRS-MutingBitRepetitionFactor; - the time gap TgapPRS∈1,2,4,8,16,32 is given by the higher-layer parameter DL-PRS-ResourceTimeGap; For a downlink PRS resource in a downlink PRS resource set configured, the UE shall assume the downlink PRS resource being transmitted as described in clause 5.1.6.4 of [6, TS 38.214]. <omitted text>

In the description of PRS muting, the variable of i is used in the formula, and also in the bitmap bi1 and bi2. It may lead to some confusion that the same variable of  i is used for the formula and {bi1 , bi2}. However, they are different variables and should be differentiated with different variables.

Proposal 2: Use different variables to differentiate i  in the formula and i  in {bi1 , bi2}

In TS 37.355, the DL PRS muting configuration introduces two IEs for the two options of DL PRS muting as follows

dl-PRS-MutingPatternList-r16		SEQUENCE {

mutingOption1-r16					SEQUENCE {

mutingPattern-r16					MutingPattern-r16,

dl-PRS-MutingBitRepetitionFactor-r16	ENUMERATED {n1, n2, n4, n8, ...}	OPTIONAL	--Need OR

},

mutingOption2-r16					SEQUENCE {

mutingPattern-r16					MutingPattern-r16

}

},

MutingPattern-r16 ::= CHOICE {

po2-r16								BIT STRING (SIZE(2)),

po4-r16								BIT STRING (SIZE(4)),

po6-r16								BIT STRING (SIZE(6)),

po8-r16								BIT STRING (SIZE(8)),

po16-r16							BIT STRING (SIZE(16)),

po32-r16							BIT STRING (SIZE(32)),

...

}

Thus, some modifications in TS 38.211 are needed for the alignment of TS 37.355.

Proposal 3: Update the description to match the DL PRS muting configuration of TS 37.355

The following gives the corresponding text proposal for Proposal 2 and 3.

In TS 38.211 Section 7.4.1.7 <omitted text> ##### 7.4.1.7.4 Mapping to slots in a downlink PRS resource set For a downlink PRS resource in a downlink PRS resource set, the UE shall assume the downlink PRS resource being transmitted when the slot and frame numbers fulfil Nslotframe,μnf+ns,fμ-ToffsetPRS-Toffset,resPRS mod 2μTperPRS∈ikTgapPRSik=0TrepPRS-1 and one of the following conditions are fulfilled: - the higher-layer parameter dl-PRS-MutingPatternList-r16 is not provided; - the higher-layer parameter dl-PRS-MutingPatternList-r16 is provided and bitmap b1 of mutingOption1-r16 but not bitmap b2 of mutingOption2-r16 is provided, and bit bi1 is set; - the higher-layer parameter dl-PRS-MutingPatternList-r16 is provided and bitmap b2 of mutingOption1-r16 but not bitmap b1 of mutingOption2-r16 is provided, and bit bi2 is set; - the higher-layer parameter dl-PRS-MutingPatternList-r16 is provided and both bitmaps b1 of mutingOption1-r16 and b2 of mutingOption2-r16 are provided, and both bit bi1 and bi2 are set. where - bi1 is bit i=Nslotframe,μnf+ns,fμ-ToffsetPRS-Toffset,resPRS 2μTmutingPRSTperPRSmod L in the bitmap given by the higher-layer parameter mutingOption1-r16 where L∈2, 4, 8, 16, 32 is the size of the bitmap; - bi2 is bit i=Nslotframe,μnf+ns,fμ-ToffsetPRS-Toffset,resPRS mod 2μTperPRS TgapPRS mod TrepPRS in the bitmap given by the higher-layer parameter mutingOption2-r16; - the slot offset ToffsetPRS∈0,1,…,TperPRS-1 is given by the higher-layer parameter DL-PRS-ResourceSetSlotOffset; - the downlink PRS resource slot offset Toffset,resPRS is given by the higher-layer parameter DL-PRS-ResourceSlotOffset; - the periodicity TperPRS∈4, 5, 8, 10, 16, 20, 32, 40, 64, 80, 160, 320, 640, 1280, 2560, 5120, 10240 is given by the higher-layer parameter DL-PRS-Periodicity; - the repetition factor TrepPRS∈1,2,4,6,8,16,32 is given by the higher-layer parameter DL-PRS-ResourceRepetitionFactor; - the muting repetition factor TmutingPRS is given by the higher-layer parameter dl-PRS-MutingBitRepetitionFactor-r16; - the time gap TgapPRS∈1,2,4,8,16,32 is given by the higher-layer parameter DL-PRS-ResourceTimeGap; For a downlink PRS resource in a downlink PRS resource set configured, the UE shall assume the downlink PRS resource being transmitted as described in clause 5.1.6.4 of [6, TS 38.214]. <omitted text>

  - TS 38.214

Regarding the RE mapping pattern of DL PRS, we made the following agreement in RAN1#98bis:

Agreement: - At least the following DL PRS RE patterns, with comb size N equal to number of symbols M are supported (figures for information)df - Comb-2: Symbols {0, 1} have relative RE offsets {0, 1} - Comb-4: Symbols {0, 1, 2, 3} have relative RE offsets {0, 2, 1, 3} - Comb-6: Symbols {0, 1, 2, 3, 4, 5} have relative RE offsets {0, 3, 1, 4, 2, 5}

And in RAN1#99, we finalized the RE mapping design with the 2nd agreement:

Agreement: - Configuration of 12 symbols for DL PRS Resource is supported. - Configuration of CombSize-12 for DL PRS Resource is supported - At most one pattern is to be supported for each agreed pair of number of symbols and combSize - The following relative RE offsets are supported for different combinations of number of symbols and combSize for DL PRS Resource: 2 symbols; 4 symbols; 6 symbols; 12 symbols. Comb-2; {0,1}; {0,1,0,1}; {0,1,0,1,0,1}; {0,1,0,1,0,1,0,1,0,1,0,1}. Comb-4; NA; {0,2,1,3}; NA; {0,2,1,3,0,2,1,3,0,2,1,3}}. Comb-6; NA; NA; {0,3,1,4,2,5}; {0,3,1,4,2,5,0,3,1,4,2,5}. Comb-12; NA; NA; NA; Working assumption: {0,6,3,9,1,7,4,10,2,8,5,11}.

As we have agreed, in the configuration of a PRS resource, the number of OFDM symbols shall be always equal to or larger than the Comb Size. However, the current specifications (38.214, 38.211) do not implement that.

Proposal 4: Clarify in 38.214 that the number of symbols in one PRS resource shall be not less than configured Comb size.

The following gives the corresponding text proposal for Proposal 4.

In TS 38.214 Section 5.1.6.5 <omitted text> A DL PRS resource is defined by: - DL-PRS-ResourceList determines the DL PRS resources that are contained within one DL PRS resource set. - DL-PRS-ResourceId determines the DL PRS resource configuration identity. All DL PRS resource IDs are locally defined within a DL PRS resource set. - DL-PRS-SequenceId is used to initialize cinit value used in pseudo random generator [4, TS38.211, 7.4.1.7.2] for generation of DL PRS sequence for a given DL PRS resource. - DL-PRS-ReOffset defines the starting RE offset of the first symbol within a DL PRS resource in frequency. The relative RE offsets of the remaining symbols within a DL PRS resource are defined based on the initial offset and the rule described in Clause 7.4.1.7.3 of [4, TS38.211]. - DL-PRS-ResourceSlotOffset determines the starting slot of the DL PRS resource with respect to corresponding DL-PRS-ResourceSetSlotOffset - DL-PRS-ResourceSymbolOffset determines the starting symbol of the DL PRS resource within the starting slot. - DL-PRS-NumSymbols defines the number of symbols of the DL PRS resource within a slot where the allowable values are given in Clause 7.4.1.7.1 of [4, TS38.211]. For the combination of {number of symbols, comb size} for a PRS resource, the UE can be configured with one of { 2, 2},{ 4, 2}, { 6, 2}, { 12, 2}, { 4, 4}, { 12, 4}, { 6, 6}, { 12, 6} and { 12, 12} - DL-PRS-QCL-Info defines any quasi-colocation information of the DL PRS resource with other reference signals. The DL PRS may be configured to be 'QCL-Type-D' with a DL PRS or SS/PBCH Block from a serving cell or a non-serving cell. The DL PRS may be configured to be 'QCL-Type-C' with a SS/PBCH Block from a serving or non-serving cell. If the DL PRS is configured as both 'QCL-Type-C' and 'QCL-Type-D' with a SS/PBCH Block then the SSB index indicated should be the same. <omitted text>

In TS 37.355, the configuration of a positioning frequency layer is as following

NR-DL–PRS-PositioningFrequencyLayer-r16 ::= SEQUENCE {

dl-PRS-SubcarrierSpacing-r16		ENUMERATED {kHz15, kHz30, kHz60, kHz120, ...},

dl-PRS-ResourceBandwidth-r16		INTEGER (1..63),

dl-PRS-StartPRB-r16					INTEGER(0..2176),

dl-PRS-PointA-r16					ARFCN-ValueNR-r15,

dl-PRS-CombSizeN-r16				ENUMERATED {n2, n4, n6, n12, ...},

dl-PRS-CyclicPrefix-r16				ENUMERATED {normal, extended, ...},

...

}

However, some of the above IEs are described within the configuration DL PRS resource set in TS 38.214, which is not aligned with TS 38.355.

There are also some misalignment of configurations of DL PRS resource set and DL PRS resource and the TS 38.214 descriptions:

NR-DL-PRS-ResourceSet-r16 ::= SEQUENCE {

nr-DL-PRS-ResourceSetId-r16			NR-DL-PRS-ResourceSetId-r16,

dl-PRS-Periodicity-and-ResourceSetSlotOffset-r16-r16	NR-DL-PRS-Periodicity-and-ResourceSetSlotOffset-r16,

dl-PRS-ResourceRepetitionFactor-r16	ENUMERATED {n1, n2, n4, n6, n8, n16, n32, ...},

dl-PRS-ResourceTimeGap-r16			ENUMERATED {s1, s2, s4, s8, s16, s32, ...},

dl-PRS-ResourceList-r16				SEQUENCE (SIZE (1..nrMaxResourcesPerSet)) OF NR-DL-PRS-Resource-r16,

dl-PRS-NumSymbols-r16				ENUMERATED {n2, n4, n6, n12, ...},

dl-PRS-MutingPatternList-r16		SEQUENCE {

mutingOption1-r16					SEQUENCE {

mutingPattern-r16					MutingPattern-r16,

dl-PRS-MutingBitRepetitionFactor-r16	ENUMERATED {n1, n2, n4, n8, ...}	OPTIONAL	--Need OR

},

mutingOption2-r16					SEQUENCE {

mutingPattern-r16					MutingPattern-r16

}

},

dl-PRS-ResourcePower-r16			INTEGER (-60..50),

...

}

NR-DL-PRS-Resource-r16 ::= SEQUENCE {

nr-DL-PRS-ResourceId-r16				NR-DL-PRS-ResourceID-r16,

dl-PRS-SequenceId-r16				INTEGER {0.. 4095},

dl-PRS-ReOffset-r16					CHOICE {

n2-r16							INTEGER (0..1),

n4-r16							INTEGER (0..3),

n6-r16							INTEGER (0..5),

n12-r16							INTEGER (0..11)

},

dl-PRS-ResourceSlotOffset-r16		INTEGER (0..nrMaxResourceOffsetValue-1),

dl-PRS-ResourceSymbolOffset-r16		INTEGER (0..12),

dl-PRS-QCL-Info-r16					DL-PRS-QCL-Info-r16	OPTIONAL,

...

}

Proposal 5: Correct the description on the configurations of positioning frequency layer, DL PRS resource set, DL PRS resource in TS 38.214 accordingly.

The following gives the corresponding text proposal for Proposal 5.

In TS 38.211 Section 7.4.1.7 <omitted text> A positioning frequency layer consists of one or more PRS resource sets and it is defined by: - DL-PRS-SubcarrierSpacing defines the subcarrier spacing for the DL PRS resource. All DL PRS Resources and DL PRS Resource sets in the same DL-PRS-PositioningFrequencyLayer have the same value of DL-PRS-SubcarrierSpacing. The supported values of DL-PRS-SubcarrierSpacing are given in Table 4.2-1 of [4, TS38.211]. - DL-PRS-CyclicPrefix defines the cyclic prefix for the DL PRS resource. All DL PRS Resources and DL PRS Resource sets in the same DL-PRS-PositioningFrequencyLayer have the same value of DL-PRS-CyclicPrefix. The supported values of DL-PRS-CyclicPrefix are given in Table 4.2-1 of [4, TS38.211]. - DL-PRS-PointA defines the absolute frequency of the reference resource block. Its lowest subcarrier is also known as Point A. All DL PRS resources belonging to the same DL PRS Resource Set have common Point A and all DL PRS Resources sets belonging to the same DL-PRS-PositioningFrequencyLayer have a common Point A. - dl-PRS-CombSizeN-r16 defines the comb size of a DL PRS resource where the allowable values are given in Clause 7.4.1.7.1 of [TS38.211]. - dl-PRS-ResourceBandwidth-r16 defines the number of resource blocks configured for PRS transmission. The parameter has a granularity of 4 PRBs with a minimum of 24 PRBs and a maximum of 272 PRBs. - dl-PRS-StartPRB-r16 defines the starting PRB index of the DL PRS resource with respect to reference Point A, where reference Point A is given by the higher-layer parameter DL-PRS-PointA. The starting PRB index has a granularity of one PRB with a minimum value of 0 and a maximum value of 2176 PRBs. The UE expects that it will be configured with [IDs] each of which is defined such that it is associated with multiple DL PRS Resource Sets from the same cell. The UE expects that one of these [IDs] along with a DL-PRS-ResourceSetId and a DL-PRS-ResourceId can be used to uniquely identify a DL PRS Resource. A DL PRS resource set consists of one or more DL PRS resources and it is defined by: - DL-PRS-ResourceSetId defines the identity of the DL PRS resource set configuration. - dl-PRS-ResourceList-r16 determines the DL PRS resources that are contained within one DL PRS resource set. - DL-PRS-Periodicity defines the DL PRS resource periodicity and takes values TperPRS∈2μ4, 5, 8, 10, 16, 20, 32, 40, 64, 80, 160, 320, 640, 1280, 2560, 5120, 10240 slots, where μ=0, 1, 2, 3 for DL-PRS-SubcarrierSpacing=15, 30, 60 and 120kHz respectively. All the DL PRS resources within one DL PRS resource set are configured with the same DL PRS periodicity. - DL-PRS-ResourceRepetitionFactor defines how many times each DL-PRS resource is repeated for a single instance of the DL-PRS resource set and takes values TrepPRS∈1,2,4,6,8,16,32,. All the DL PRS resources within one resource set have the same ResourceRepetitionFactor - DL-PRS-ResourceTimeGap defines the offset in number of slots between two repeated instances of a DL PRS resource with the same DL-PRS-ResourceID within a single instance of the DL PRS resource set and takes values TgapPRS∈1,2,4,8,16,32. The UE only expects to be configured with DL-PRS-ResourceTimeGap if DL-PRS-ResourceRepetitionFactor is configured with value greater than 1. The time duration spanned by one instance of a DL-PRS-ResourceSet is not expected to exceed the configured value of DL-PRS-Periodicity. All the DL PRS resources within one resource set have the same DL-PRS-ResourceTimeGap. - dl-PRS-NumSymbols-r16 defines the number of symbols of the DL PRS resource within a slot where the allowable values are given in Clause 7.4.1.7.1 of [4, TS38.211]. - dl-PRS-MutingPatternList-r16 defines up to two bitmaps of the time locations where the DL PRS resource is expected to not be transmitted for a DL PRS resource set. The bitmap size can be {2, 4, 6, 8, 16, 32} bits long. each bit in the bitmap given by mutingOption1-r16 corresponds to a configurable number provided by higher layer parameter DL-PRS.MutingBitRepetitionFactor of consecutive instances of a DL-PRS-ResourceSet where all the DL-PRS-Resources within the set are muted for the instance that is indicated to be muted. each bit in the bitmap given by mutingOption2-r16 corresponds to a single repetition index for each of the DL-PRS-Resources within each instance of a DL-PRS-ResourceSet and the length of the bitmap is equal to DL-PRS-ResourceRepetitionFactor. Both options may be configured at the same time in which case the logical AND operation is applied to the bit maps as described in clause 7.4.1.7.4 of [4, TS 38.211]. - DL-PRS-SFN0-Offset defines the time offset of the SFN0 slot 0 for the transmitting cell with respect to SFN0 slot 0 of [FFS in RAN2]. - DL-PRS-ResourceSetSlotOffset defines the slot offset with respect to SFN0 slot 0 and takes values ToffsetPRS∈0,1,…,TperPRS-1. - A DL PRS resource is defined by: - DL-PRS-ResourceId determines the DL PRS resource configuration identity. All DL PRS resource IDs are locally defined within a DL PRS resource set. - DL-PRS-SequenceId is used to initialize cinit value used in pseudo random generator [4, TS38.211, 7.4.1.7.2] for generation of DL PRS sequence for a given DL PRS resource. - DL-PRS-ReOffset defines the starting RE offset of the first symbol within a DL PRS resource in frequency. The relative RE offsets of the remaining symbols within a DL PRS resource are defined based on the initial offset and the rule described in Clause 7.4.1.7.3 of [4, TS38.211]. - DL-PRS-ResourceSlotOffset determines the starting slot of the DL PRS resource with respect to corresponding DL-PRS-ResourceSetSlotOffset - DL-PRS-ResourceSymbolOffset determines the starting symbol of the DL PRS resource within the starting slot. - DL-PRS-QCL-Info defines any quasi-colocation information of the DL PRS resource with other reference signals. The DL PRS may be configured to be 'QCL-Type-D' with a DL PRS or SS/PBCH Block from a serving cell or a non-serving cell. The DL PRS may be configured to be 'QCL-Type-C' with a SS/PBCH Block from a serving or non-serving cell. If the DL PRS is configured as both 'QCL-Type-C' and 'QCL-Type-D' with a SS/PBCH Block then the SSB index indicated should be the same. <omitted text>

There are also some other modifications needed for DL PRS reception.

Proposal 6: Correct the following on PRS reception in TS 38.214

- TRP should be used instead of cell

- DL-PRS-ResourceSymbolOffset not only determines the starting symbol of the DL PRS resource within the starting slot, but also that within the other slot used for transmission.

- SS/PBCH Blocks with the same SSB index may be from the same beam, or may be from different beams, since SS/PBCH block with the same SSB index may be from different servicing cells

- The condition “as long as the condition that the DL PRS resources used belong to a single DL PRS resource set is met” should refer to the case with different DL PRS resource rather than different LD PRS resource sets

The following gives the corresponding text proposal for Proposal 4.

In TS 38.214 Section 5.1.6.5 <omitted text> #### 5.1.6.5 PRS reception procedure The UE can be configured with one or more DL PRS resource set configuration(s) as indicated by the higher layer parameters DL-PRS-ResourceSet and DL-PRS-Resource. Each DL PRS resource set consists of K≥1 DL PRS resource(s) where each has an associated spatial transmission filter. The UE can be configured with one or more DL PRS Positioning Frequency Layer configuration(s) as indicated by the higher layer parameter DL-PRS-PositioningFrequencyLayer. A DL PRS Positioning Frequency Layer is defined as a collection of DL PRS Resource Sets which have common parameters configured by DL-PRS-PositioningFrequencyLayer. The UE assumes that the following parameters for each DL PRS resource(s) are configured via higher layer parameters DL-PRS-PositioningFrequencyLayer, DL-PRS-ResourceSet and DL-PRS-Resource. A positioning frequency layer consists of one or more PRS resource sets and it is defined by: - DL-PRS-SubcarrierSpacing defines the subcarrier spacing for the DL PRS resource. All DL PRS Resources and DL PRS Resource sets in the same DL-PRS-PositioningFrequencyLayer have the same value of DL-PRS-SubcarrierSpacing. The supported values of DL-PRS-SubcarrierSpacing are given in Table 4.2-1 of [4, TS38.211]. - DL-PRS-CyclicPrefix defines the cyclic prefix for the DL PRS resource. All DL PRS Resources and DL PRS Resource sets in the same DL-PRS-PositioningFrequencyLayer have the same value of DL-PRS-CyclicPrefix. The supported values of DL-PRS-CyclicPrefix are given in Table 4.2-1 of [4, TS38.211]. - DL-PRS-PointA defines the absolute frequency of the reference resource block. Its lowest subcarrier is also known as Point A. All DL PRS resources belonging to the same DL PRS Resource Set have common Point A and all DL PRS Resources sets belonging to the same DL-PRS-PositioningFrequencyLayer have a common Point A. The UE expects that it will be configured with [IDs] each of which is defined such that it is associated with one or two DL PRS Resource Sets associated with the same trp-ID-r16. The UE expects that one of these [IDs] along with a DL-PRS-ResourceSetId and a DL-PRS-ResourceId can be used to uniquely identify a DL PRS Resource. A DL PRS resource set consists of one or more DL PRS resources and it is defined by: - DL-PRS-ResourceSetId defines the identity of the DL PRS resource set configuration. - DL-PRS-Periodicity defines the DL PRS resource periodicity and takes values TperPRS∈2μ4, 5, 8, 10, 16, 20, 32, 40, 64, 80, 160, 320, 640, 1280, 2560, 5120, 10240 slots, where μ=0, 1, 2, 3 for DL-PRS-SubcarrierSpacing=15, 30, 60 and 120kHz respectively. All the DL PRS resources within one DL PRS resource set are configured with the same DL PRS periodicity. - DL-PRS-ResourceRepetitionFactor defines how many times each DL-PRS resource is repeated for a single instance of the DL-PRS resource set and takes values TrepPRS∈1,2,4,6,8,16,32,. All the DL PRS resources within one resource set have the same ResourceRepetitionFactor - DL-PRS-ResourceTimeGap defines the offset in number of slots between two repeated instances of a DL PRS resource with the same DL-PRS-ResourceID within a single instance of the DL PRS resource set and takes values TgapPRS∈1,2,4,8,16,32. The UE only expects to be configured with DL-PRS-ResourceTimeGap if DL-PRS-ResourceRepetitionFactor is configured with value greater than 1. The time duration spanned by one instance of a DL-PRS-ResourceSet is not expected to exceed the configured value of DL-PRS-Periodicity. All the DL PRS resources within one resource set have the same DL-PRS-ResourceTimeGap. - DL-PRS-MutingPattern defines a bitmap of the time locations where the DL PRS resource is expected to not be transmitted for a DL PRS resource set. The bitmap size can be {2, 4, 6, 8, 16, 32} bits long. The bitmap has two options for applicability. In the first option each bit in the bitmap corresponds to a configurable number provided by higher layer parameter DL-PRS.MutingBitRepetitionFactor of consecutive instances of a DL-PRS-ResourceSet where all the DL-PRS-Resources within the set are muted for the instance that is indicated to be muted. In the second option each bit in the bitmap corresponds to a single repetition index for each of the DL-PRS-Resources within each instance of a DL-PRS-ResourceSet and the length of the bitmap is equal to DL-PRS-ResourceRepetitionFactor. Both options may be configured at the same time in which case the logical AND operation is applied to the bit maps as described in clause 7.4.1.7.4 of [4, TS 38.211]. - DL-PRS-SFN0-Offset defines the time offset of the SFN0 slot 0 for the transmitting cell with respect to SFN0 slot 0 of [FFS in RAN2]. - DL-PRS-ResourceSetSlotOffset defines the slot offset with respect to SFN0 slot 0 and takes values ToffsetPRS∈0,1,…,TperPRS-1. - DL-PRS-CombSizeN defines the comb size of a DL PRS resource where the allowable values are given in Clause 7.4.1.7.1 of [TS38.211]. All DL PRS resource sets belonging to the same positioning frequency layer have the same value of DL-PRS-combSizeN. - DL-PRS-ResourceBandwidth defines the number of resource blocks configured for PRS transmission. The parameter has a granularity of 4 PRBs with a minimum of 24 PRBs and a maximum of 272 PRBs. All DL PRS resources sets within a positioning frequency layer have the same value of DL-PRS-ResourceBandwidth. - DL-PRS-StartPRB defines the starting PRB index of the DL PRS resource with respect to reference Point A, where reference Point A is given by the higher-layer parameter DL-PRS-PointA. The starting PRB index has a granularity of one PRB with a minimum value of 0 and a maximum value of 2176 PRBs. All DL PRS Resource Sets belonging to the same Positioning Frequency Layer have the same value of Start PRB. A DL PRS resource is defined by: - DL-PRS-ResourceList determines the DL PRS resources that are contained within one DL PRS resource set. - DL-PRS-ResourceId determines the DL PRS resource configuration identity. All DL PRS resource IDs are locally defined within a DL PRS resource set. - DL-PRS-SequenceId is used to initialize cinit value used in pseudo random generator [4, TS38.211, 7.4.1.7.2] for generation of DL PRS sequence for a given DL PRS resource. - DL-PRS-ReOffset defines the starting RE offset of the first symbol within a DL PRS resource in frequency. The relative RE offsets of the remaining symbols within a DL PRS resource are defined based on the initial offset and the rule described in Clause 7.4.1.7.3 of [4, TS38.211]. - DL-PRS-ResourceSlotOffset determines the starting slot of the DL PRS resource with respect to corresponding DL-PRS-ResourceSetSlotOffset - DL-PRS-ResourceSymbolOffset determines the starting symbol of the DL PRS resource within the slot. - DL-PRS-NumSymbols defines the number of symbols of the DL PRS resource within a slot where the allowable values are given in Clause 7.4.1.7.1 of [4, TS38.211]. - DL-PRS-QCL-Info defines any quasi-colocation information of the DL PRS resource with other reference signals. The DL PRS may be configured to be 'QCL-Type-D' with a DL PRS or SS/PBCH Block from a serving cell or a non-serving cell. The DL PRS may be configured to be 'QCL-Type-C' with a SS/PBCH Block from a serving or non-serving cell. If the DL PRS is configured as both 'QCL-Type-C' and 'QCL-Type-D' with a SS/PBCH Block then the SSB index indicated should be the same and should be from the same cell. The UE assumes constant EPRE is used for all REs of a given DL PRS resource. The UE may be indicated by the network that a DL PRS resources can be used as the reference for the DL RSTD, DL PRS-RSRP, and UE Rx-Tx time difference measurements in a higher layer parameter DL-PRS-RstdReferenceInfo. The reference time indicated by the network to the UE can also be used by the UE to determine how to apply higher layer parameters DL-PRS-expectedRSTD and DL-PRS-expectedRSTD-uncertainty. The UE expects the reference time to be indicated whenever it is expected to receive the DL PRS. This reference time provided by DL-PRS-RstdReferenceInfo may include an [ID], a PRS resource set ID, and optionally a single PRS resource ID or a list of PRS resource IDs. The UE may use different DL PRS resources as long as the condition that the DL PRS resources used belong to a single DL PRS resource set is met or a different DL PRS resource set to determine the reference time for the RSTD measurement. If the UE chooses to use a different reference time than indicated by the network, then it is expected to report the [ID], the DL PRS resource ID(s) or the DL PRS resource set ID used to determine the reference. The UE may be configured to report quality metrics corresponding to the RSTD and UE Rx-Tx time difference measurements which include the following fields: - TimingMeasQuality-Value which provides the best estimate of the uncertainty of the measurement - TimingMeasQuality-Resolution which specifies the resolution levels used in the TimingMeasQuality-Value field The UE expects to be configured with higher layer parameter DL-PRS-expectedRSTD, which defines the time difference with respect to the received DL subframe timing the UE is expected to receive DL PRS, and DL-PRS-expectedRSTD-uncertainty, which defines a search window around the expectedRSTD. For DL UE positioning measurement reporting in higher layer parameters DL-PRS-RstdMeasurementInfo or DL-PRS-UE-Rx-Tx-MeasurementInfo the UE can be configured to report the DL PRS resource ID(s) or the DL PRS resource set ID(s) associated with the DL PRS resource(s) or the DL PRS resource set(s) which are used in determining the UE measurements DL RSTD, UE Tx-Rx time difference or the DL PRS-RSRP. The UE can be configured in higher layer parameter UE Rx-Tx Time-MeasRequestInfo to report multiple UE Rx-Tx time difference measurements corresponding to a single configured SRS resource or resource set for positioning. Each measurement corresponds to a single received DL PRS resource or resource set which can be in difference positioning frequency layers. For the DL RSTD, DL PRS-RSRP, and UE Rx-Tx time difference measurements the UE can report an associated higher layer parameter Timestamp. The Timestamp can include the SFN and the slot number for a subcarrier spacing. These values correspond to the reference which is provided by DL-PRS-RSTDReferenceInfo. The UE is expected to measure the DL PRS resource outside the active DL BWP or with a numerology different from the numerology of the active DL BWP if the measurement is made during a configured measurement gap. When not configured with a measurement gap, the UE is only required to measure DL PRS within the active DL BWP and with the same numerology as the active DL BWP. When the UE is expected to measure the DL PRS resource outside the active DL BWP it may request a measurement gap in higher layer parameter [XYZ]. The UE assumes that the DL PRS from the serving cell is not mapped to any symbol that contains SS/PBCH block from the serving cell. If the time frequency location of the SS/PBCH block transmissions from non-serving cells are provided to the UE then the UE also assumes that the DL PRS from a non-serving cell is not mapped to any symbol that contains the SS/PBCH block of the same non-serving cell. The UE may be configured to report, subject to UE capability, up to 4 DL RSTD measurements per pair of trp-ID-r16 with each measurement between a different pair of DL PRS resources or DL PRS resource sets within the DL PRS configured for those trp-ID-r16. The up to 4 measurements being performed on the same pair of cells and all DL RSTD measurements in the same report use a single reference timing. The UE may be configured to measure and report up to 8 DL PRS RSRP measurements on different DL PRS resources associated with the same trp-ID-r16. When the UE reports DL PRS RSRP measurements from one DL PRS resource set, the UE may indicate which DL PRS RSRP measurements have been performed using the same spatial domain filter for reception. If the UE is configured with DL-PRS-QCL-Info and the QCL relation is between two DL PRS resources, then the UE assumes those DL PRS resources are associated with the same trp-ID-r16. If DL-PRS-QCL-Info is configured to the UE with 'QCL-Type-D' with a source DL-PRS-Resource then the DL-PRS-ResourceSetId and the DL-PRS-ResrouceId of the source DL-PRS-Resource are expected to be indicated to the UE. The UE does not expect to process the DL PRS in the same symbol where other DL signals and channels are transmitted to the UE when there is no measurement gap configured to the UE. <omitted text>

- Conclusion

In this contribution, we presented our views on procedures for DL PRS transmission and UL SRS transmission. Based on the discussion, the following proposals are provided:

Proposal 1: Correct the periodicity values of a DL PRS resource to be aligned with TS 38.214 and TS 37.355

Proposal 2: Use different variables to differentiate i  in the formula and i  in {bi1 , bi2}

Proposal 3: Update the description to match the DL PRS muting configuration of TS 37.355

Proposal 4: Clarify in 38.214 that the number of symbols in one PRS resource shall be not less than configured Comb size.

Proposal 5: Correct the description on the configurations of positioning frequency layer, DL PRS resource set, DL PRS resource in TS 38.214 accordingly.

Proposal 6: Correct the following on PRS reception in TS 38.214

- TRP should be used instead of cell

- DL-PRS-ResourceSymbolOffset not only determines the starting symbol of the DL PRS resource within the starting slot, but also that within the other slot used for transmission.

- SS/PBCH Blocks with the same SSB index may be from the same beam, or may be from different beams, since SS/PBCH block with the same SSB index may be from different servicing cells

- The condition “as long as the condition that the DL PRS resources used belong to a single DL PRS resource set is met” should refer to the case with different DL PRS resource rather than different LD PRS resource sets

- Reference

TS 38.211, NR; Physical channels and modulation, V16.1.0

- TS 38.214, NR; Physical layer procedures for data, V16.1.0

TS 37.355, LTE Positioning Protocol (LPP), V16.0.0