3GPP TSG RAN WG1 #100bis	   	                      R1-2001932

e-Meeting, April 20th – 30th, 2020

Agenda Item:	7.2.2.1.1

Source: 	LG Electronics

Title: 	Remaining issues of initial access signals and channels for NR-U

Document for:	Discussion and decision

- Introduction

In this contribution, we discuss the following remaining issue of initial access signals and channels for NR-U, to be handled in RAN1#100bis-e.

- Determination of LSB of k_SSB to define PRB grid

- Determination of LSB of k_SSB to define PRB grid

Based on reply LS [1] from RAN2, if RAN1 decide to use LSB of ssb-SubcarrierOffset (1 bit) for signalling of Q (i.e., ssbPositionQCL-Relationship-r16), it should be discussed how to set LSB of k_SSB since LSB of ssb-SubcarrierOffset was originally used for signalling LSB of k_SSB. If the corresponding SS/PBCH block is located at GSCN that will be defined for NR-U in RAN4 specification (i.e., TS 38.101-1), LSB of k_SSB would be equal to ‘0’ due to the fact that channel and synchronization rasters for NR-U will be defined on 30 kHz granularity.

Proposal #1: If LSB of ssb-SubcarrierOffset is used for signalling of ssbPositionQCL-Relationship-r16, LSB of k_SSB is set to ‘0’ for SS/PBCH on a sync raster.

Furthermore, we should consider the SS/PBCH block not on a synchronization raster for the purpose of ANR.

- The frequency position of the SS/PBCH block provided by ssbFrequency in a measurement configuration associated with a reporting configuration providing reportCGI is configured with NR-ARFCN parameter which has 15 kHz granularity.

- In RAN1#100-e meeting, it was agreed to apply k_SSB from the SS/PBCH configured for the purpose of ANR

Therefore, based on the above two considerations, LSB of k_SSB can be set to ‘0’ or ‘1’ depending on whether the distance between a sync raster and the center frequency of off-sync SS/PBCH block is integer multiple of 15 kHz or 30 kHz.

Proposal #2: If LSB of ssb-SubcarrierOffset is used for signalling of ssbPositionQCL-Relationship-r16, for a SS/PBCH not on a sync raster,

- If the distance between a synchronization raster for NR-U and the center frequency of the SS/PBCH is equal to integer multiple of 30 kHz, LSB of k_SSB is set to ‘0’.

- Otherwise, if the distance between a synchronization raster for NR-U and the center frequency of the SS/PBCH is not equal to integer multiple of 30 kHz but equal to integer multiple of 15 kHz, LSB of k_SSB is set to ‘1’.

Based on proposals #1 and #2, the following text proposal can be given.

Proposal #3: Adopt the following text proposal in TS 38.211 section 7.4.3.1.

#### 7.4.3.1 Time-frequency structure of an SS/PBCH block In the time domain, an SS/PBCH block consists of 4 OFDM symbols, numbered in increasing order from 0 to 3 within the SS/PBCH block, where PSS, SSS, and PBCH with associated DM-RS are mapped to symbols as given by Table 7.4.3.1-1. In the frequency domain, an SS/PBCH block consists of 240 contiguous subcarriers with the subcarriers numbered in increasing order from 0 to 239 within the SS/PBCH block. The quantities and represent the frequency and time indices, respectively, within one SS/PBCH block. The UE may assume that the complex-valued symbols corresponding to resource elements denoted as 'Set to 0' in Table 7.4.3.1-1 are set to zero. The quantity in Table 7.4.3.1-1 is given by v=NIDcell mod 4. The quantity is the subcarrier offset from subcarrier 0 in common resource block NCRBSSB to subcarrier 0 of the SS/PBCH block, where NCRBSSB is obtained from the higher-layer parameter offsetToPointA and the 4 least significant bits of are given by the higher-layer parameter ssb-SubcarrierOffset and for SS/PBCH block type A the most significant bit of is given by aA+5 in the PBCH payload as defined in clause 7.1.1 of [4, TS 38.212]. For operation with shared spectrum channel access, the least significant bit of kSSB is set to 0 if the frequency offset between the lowest subcarrier of the SS/PBCH block and the lowest subcarrier of a SS/PBCH block located at the GSCN of a synchronization raster entry as defined in [X, TS 38.101-1] is equal to 0 or integer multiple of 30 kHz, otherwise, the least significant bit of kSSB is set to 1. If ssb-SubcarrierOffset is not provided, kSSB is derived from the frequency difference between the SS/PBCH block and Point A.

- Reference

R1-2001512, “Reply LS on signaling of Q for a serving cell in NR-U,” RAN2, Qualcomm, RAN1#100bis-e, Apr. 2020