3GPP TSG RAN WG1 #100bis	R1-2001953

e-Meeting, April 20th – 30th, 2020

Agenda Item:	7.2.3.4

Source: 	LG Electronics

Title: 	Text proposals on case1 timing alignment

Document for:	Discussion and decision

Introduction

In [1], RAN2 informs RAN1 of its preference to have the T_delta index and the actual value of T_delta captured in RAN1 specifications. In this contribution, we discuss and provide text proposals on this mapping issue.

Discussion on Mapping between T_delta index and the actual value of T_delta

In the current specification, case 1 timing alignment (i.e., DL Tx timing alignment) is based on NTA+NTA,offset⋅Tc/2+Tdelta where NTA is calculated based on the index values of TA and Tdelta is assumed to be the actual value. As in [1], RAN2 has its preference to have the T_delta index signalling, so that the mapping relation between T_delta index and actual T_delta value needs to be specified in TS 38.213.

According to [2], the granularity of T_delta for IAB OTA timing alignment is 64Tc for FR1 and 32Tc for FR2, and the ranges of T_delta for different numerologies are as shown in the below Table 1.

Table 1. Range of T_delta

| SCS [kHz] | Max T_delta [Tc] | Min T_delta [Tc] |
| --- | --- | --- |
| 15 | - /2 + 6256 | - /2 - 70528 |
| 30 | - /2 + 6128 | - /2 - 35328 |
| 60 | - /2 + 6032 | - /2 - 17664 |
| 120 | - /2 + 6032 | - /2 - 8816 |

As observed in Table 1, the maximum bit-width for T_delta indication is 11 bits (index range is 0, …, 1200) in case of SCS=15kHz. Thus we think 11 bits for the T_delta field (containing the index value of T_delta) are sufficient.

Observation 1: 11 bits for the T_delta field (containing the index value of T_delta) are sufficient.

A simple way to map the T_delta index to the actual value of T_delta is using the following equations:

T_delta value = Min T_delta + granularity  * T_delta index,  or

T_delta value = Max T_delta - granularity  * T_delta index

For the minimum value based approach, the Tdelta ,actual can be replaced with

Tdelta, actual=Tdelta_min+Tdelta_gran*Tdelta, index Tc,

where Tdelta,index is an index delivered by MAC-CE and Tdelta_min and Tdelta_gran are listed in Table 2.

Table 2. Granularity and value range of Tdelta

| Frequency range (FR) | Tdelta_gran [Tc] | SCS [kHz] | Tdelta_min [Tc] | Index range of Tdelta,index |
| --- | --- | --- | --- | --- |
| FR1 | 64 | 15 | -NTA,offset2-70528 | 0, …, 1200 |
|  |  | 30 | -NTA,offset2-35328 | 0, … ,648 |
|  |  | 60 | -NTA,offset2-17664 | 0, … ,371 |
| FR2 | 32 | 60 | -NTA,offset2-17664 | 0, … ,741 |
|  |  | 120 | -NTA,offset2-8816 | 0, … ,464 |

Also, both TA and T_delta have NTA,offset commonly, the equation of case 1 timing alignment can be further simplified as

NTA2+T'delta_min+Tdelta_gran⋅Tdelta⋅Tc,

where T'delta_min is the T'delta_min=Tdelta_min+ NTA,offset2.

Based on the above discussion, we provide text proposals for mapping between T_delta index and the actual value of T_delta in the next section.

Summary

In this contribution, we discuss on the mapping between T_delta index and the actual value of T_delta. Based on the discussion, we provide following observation and text proposal.

Observation 1: 11 bits for the T_delta field (containing the index value of T_delta) are sufficient.

Text proposal:

- 14	Integrated access-backhaul operation

-------------------------------------------------------- Omitted -----------------------------------------------------

If an IAB-node is provided a value Tdelta from a serving cell, the IAB-node may assume that NTA+NTA,offset⋅Tc/2+Tdelta 
NTA2+Tdelta_min+Tdelta_gran⋅Tdelta⋅Tc

is a time difference between a DU transmission of a signal from the serving cell and a reception of the signal by the IAB-node MT when NTA+NTA,offset⋅Tc2+Tdelta>0NTA2+Tdelta_min+Tdelta_gran⋅Tdelta⋅Tc>0, where NTA and NTA,offset are obtained as for a "UE" in Clause 4.2 for the TAG containing the serving cell, Tdelta_min and Tdelta_gran are listed in Table 14-x, and  Tdelta is given by Timing Delta command [11, 38.321]. The IAB-node may use the time difference to determine a DU transmission time.

Table 14-x. Granularity and value range of Tdelta

| Frequency range (FR) | Tdelta_gran [Tc] | SCS [kHz] | Tdelta_min [Tc] | Index range of Tdelta |
| --- | --- | --- | --- | --- |
| FR1 | 64 | 15 | -70528 | 0,…,1200 |
|  |  | 30 | -35328 | 0, … ,648 |
|  |  | 60 | -17664 | 0, … ,371 |
| FR2 | 32 | 60 | -17664 | 0, … ,741 |
|  |  | 120 | -8816 | 0, … ,464 |

-------------------------------------------------------- Omitted -----------------------------------------------------

Reference

[1] R1-2001510, “LS to RAN1 on T_delta in IAB”.

[2] R1-1911807, “Reply LS on T_delta for IAB OTA timing alignment”.