3GPP TSG RAN WG1 #102-e			R1-200xxxx

e-Meeting, August 17th – 28th, 2020

Agenda Item:	7.1

Source:			Moderator (ASUSTeK)

Title:   Summary for [102-e-NR-7.1CRs-12] Maintenance on UCI bit sequence

generation

Document for: 	Discussion and Decision

# Introduction

This document is a summary for the following email discussion [1]:

[102-e-NR-7.1CRs-12] Maintenance on UCI bit sequence generation by 8/19 – Eddie (ASUSTek)

- For Rel-16, issue described in R1-2006585

This issue was discussed in R1-2004375 [2] in RAN#1 101-e and the outcome is no changes for Rel-15 and it can be considered in Rel-16. The focus of this email discussion is whether to fix the UCI bit indexing error in Rel-16. Note that previous comments expressed could be found in annex of this document.

Please provide your input at the end of section 2.

# Discussion

The following is the TP to fix the UCI bit indexing error in Rel-16:

******************Start of the TP******************

- 6.3.1.1.1	HARQ-ACK/SR only

If only HARQ-ACK bits are transmitted on a PUCCH, the UCI bit sequence  is determined by setting  for  and , where the HARQ-ACK bit sequence  is given by Clause 9.1 of [5, TS38.213].

If only HARQ-ACK and SR bits are transmitted on a PUCCH, the UCI bit sequence  is determined by setting  for ,   for , and , where the HARQ-ACK bit sequence  is given by Clause 9.1 of [5, TS 38.213], and the SR bit sequence  is given by Clause 9.2.5.1 of [5, TS 38.213].

<Unchanged Text omitted>

- 6.3.1.1.3	HARQ-ACK/SR and CSI

If none of the CSI reports for transmission on a PUCCH is of two parts, the UCI bit sequence  is generated according to the following, where :

-	if there is HARQ-ACK for transmission on the PUCCH, the HARQ-ACK bits are mapped to the UCI bit sequence , where  for , the HARQ-ACK bit sequence  is given by Clause 9.1 of [5, TS38.213], and  is number of HARQ-ACK bits; if there is no HARQ-ACK for transmission on the PUCCH, set ;

-	if there is SR for transmission on the PUCCH, set   for , where the SR bit sequence  is given by Clause 9.2.5.1 of [5, TS 38.213]; if there is no SR for transmission on the PUCCH, set ;

-	the CSI fields of all CSI reports, in the order from upper part to lower part in Table 6.3.1.1.2-12, are mapped to the UCI bit sequence  starting with , where  is the number of CSI bits.

If at least one of the CSI reports for transmission on a PUCCH is of two parts, two UCI bit sequences are generated,  and , according to the following, where  and :

-	if there is HARQ-ACK for transmission on the PUCCH, the HARQ-ACK bits are mapped to the UCI bit sequence , where  for , the HARQ-ACK bit sequence  is given by Clause 9.1 of [5, TS38.213], and  is number of HARQ-ACK bits; if there is no HARQ-ACK for transmission on the PUCCH, set ;

-	if there is SR for transmission on the PUCCH, set   for , where the SR bit sequence  is given by Clause 9.2.5.1 of [5, TS 38.213]; if there is no SR for transmission on the PUCCH, set ;

******************End of the TP******************

Given most comments received so far, it seems agreeable that the changes in above TP are correct while the question is to adopt it or not in Rel-16. Please share your view in the following table with reasoning when proper:

Q: Do you agree to adopt above TP in Rel-16?

| Company Name | Comments |
| --- | --- |
| ASUSTeK | Yes, adopt it in Rel-16. The TP corrects an indexing error in the spec. |
| CATT | Agree to adopt above TP in Rel-16. |

# Conclusion

To be added after the discussion

# Reference

- Chairman’s Notes RAN1#102-e v002

- R1-2004375, “CR on UCI bit sequence determination”.

Annex 1:

Comments received on this issue in RAN1#101-e based on the summary (v20 of [101-e-NR-7.1CRs-14]):

| [101-e-NR-7.1CRs-14] Moderator's initial view | Rejected for Rel-15 but consider for Rel-16 (in RAN1#102-e) |
| --- | --- |
| Updated Summary | #1: Nokia (0 -> 1) #2: DCM, ZTE, Samsung, Ericsson, Intel, MTK (6) #3: CATT, QC, HW, Spreadtrum, ASUSTeK (5) |
| CATT | Further consider for Rel-15 in future meetings (We agree with the CR.) |
| NTT DOCOMO | Rejected for Rel-15 but consider for Rel-16 (in RAN1#102-e), or Further consider for Rel-15 in future meetings This CR is editorial one but correct update. We are OK with either way, |
| Qualcomm | Further consider for Rel-15 in future meetings This is an editorial change but is technically correct. |
| Huawei | Further consider for Rel-15. We are fine with the change. |
| ZTE | Not essential correction and can be considered in Rel-16. |
| Samsung | Rejected for Rel-15 but consider for Rel-16 (in RAN1#102-e). Editorial CR. Given no Rel-15 editor's alignment CR, we suggest to discuss this for Rel-16 editor's alignment CR. |
| Spreadtrum | Further consider for Rel-15 in future meetings |
| Ericsson | Consider for Rel-16. Agree with CR. Good to fix to have correct indexing. |
| Intel | Rejected for Rel-15 but consider for Rel-16. This can be viewed as editorial change. |
| ASUSTeK | Further consider for Rel-15 in future meetings. Many companies has confirmed that it is indeed an indexing error i.e. there is something wrong rather than unclear in the spec. Our first preference would be to correct it in Rel-15, while could be fine to go with Rel-16 resolution if there is more concern expressed for Rel-15 correction. |
| MediaTek | Rejected for Rel-15 but consider for Rel-16. |
| Nokia | #1 |

Annex 2:

Comments received on this issue in RAN1#102-e under preparation phase of AI 7.2.12 in summary (v9):

| Company | Comment |
| --- | --- |
| Ericsson | We support to adopt the CR. This is a non-controversial issue which addresses an error in indexing which is important to be fixed from specification point of view. The CR was submitted last meeting in R1-2004375 where we shared the same view. |
| Intel | Should be the CR, not TEI. |
| Nokia | OK to discuss, but this is not a TEI and should have been submitted under AI 7.1 with a clear statement that this is intented for Rel-16. |
| ZTE | It’s better to treat it as a Rel-15 CR instead of Rel-16 TEI. So it should be discussed in AI 7.1. |
| vivo | Share the view this should be treated in AI 7.1. |
| Moderator | Thank you very much for the inputs! According to the inputs so far, the issue should be discussed in RAN1#102-e, not as part of TEI but as part of CR under AI7.1. Therefore, the updated FL proposal for this issue is to ask chairman/vice chairmen to treat R1-2006585 under AI 7.1. |
| ASUSTeK | We agreed to discuss this issue under AI 7.1 and we updated R1-2006585 to R1-2006964 with a clear statement that this is intented for Rel-16. |
| Huawei, HiSilicon | Basically, it is common understanding that SR bits are appended to HARQ bits when they are multiplexed in one PUCCH resource, and SR bits starts from first one, the change does not make any functional changes. |