3GPP TSG RAN WG1 #102-e		                                   					       												                 R1-200nnnn

e-Meeting, August 17th – 28th, 2020

Agenda Item:	7.2.11

Source:	Moderator (AT&T)

Title:	Summary of email discussion/approval [102-e-NR-UEFeatures-MobEnh-01]

Document for:	Discussion/Decision

# Introduction

This document presents the summary of email discussion/approval [102-e-NR-UEFeatures-MobEnh-01] during RAN1 #102-e. According to the Chairman’s Notes:

[102-e-NR-UEFeatures-MobEnh-01] Email discussion/approval for remaining issues on UE features for NR mobility enhancements, till 8/20 – Ralf (AT&T) - whether to update the consequence if FG 21-1a and/or 21-1b is not supported by the UE

The following was discussed and agreed during RAN1 #102-e within the scope of [102-e-NR-UEFeatures-MobEnh-01]. All proposals are based on the latest RAN1 UE features list for Rel-16 NR in Error: Reference source not found.

# Summary of email discussion/approval [102-e-NR-UEFeatures-MobEnh-01]

One company [2] is proposing to update the consequence if FG 21-1a and/or 21-1b is not supported by the UE and it was agreed during the preparation phase to discuss this proposal.

Proposal: The consequence if FG 21-1a is not supported is changed to “the network cannot configure the UE with intra-frequency DAPS handover” and the consequence if FG 21-1b is not supported is changed to “the network cannot configure the UE with inter-frequency DAPS handover”.

| 21-1a | Intra-frequency DAPS HO | Support of intra-frequency DAPS-HO - Support of simultaneous DL reception of PDCCH and PDSCH from source and target cell in DAPS-HO - Support of PDCCH blind decoding capability in the first MCG and second MCG. - Support of cancelling UL transmission to the source cell for intra-frequency DAPS-HO | DAPS (Note: RAN2 feature) | Yes | N/A | The network cannot configure UE with intra-frequency DAPS HO | Per Band | No | N/A | N/A |  | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 21-1b | Inter-frequency DAPS HO | Support of inter-frequency DAPS-HO 1) Support of simultaneous DL reception of PDCCH and PDSCH from source and target cell in DAPS-HO 2) Support of PDCCH blind decoding capability in the first MCG and second MCG. | DAPS (Note: RAN2 feature) | Yes | N/A | The network cannot configure UE with inter-frequency DAPS HO | Per BC | No | N/A | N/A |  | [Optional with capability signalling] |

Companies are invited to express their views in the table below.

| Company | Comments/Questions/Suggestions |
| --- | --- |
| ZTE [2] | The consequence if the UE does not support FG 21-1a or FG 21-1b in the agreed UE feature list is the network cannot configure the UE with DAPS handover, which is not correct. Because it is possible that a UE can only supports DAPS handover for one case, e.g. intra-frequency DAPS handover. Therefore, the consequence if FG 21-1a is not supported is the network cannot configure the UE with intra-frequency DAPS handover and the consequence if FG 21-1b is not supported is the network cannot configure the UE with inter-frequency DAPS handover. |
| MTK | We support the FL proposal. It seems like a reasonable modification. |
| Huawei, HiSilicon | We support the changes from FL though it was discussed and the changes are kind of obvious in essence. |
| Apple | We support the FL proposal. The network determines whether to configure the DAPS handover according to UE reported capability, i.e., FG 21-1a or FG 21-1b. |
| Samsung | Fine to the proposed update. However, as Huawei said, it is obvious and all the other FGs (for all Rel-16 WIs) may need to be clarified along this way for the consistency. Alternatively, we can empty this column for FG21-1a/1b. |
| Nokia, NSB | We do not support the FL proposal. In our view the UE needs to be able to support both types of HO to be operational with DAPS in real deployments. |

…

# Conclusion

…

# References

- R1-2006462, Updated RAN1 UE features list for Rel-16 NR, Moderators (AT&T, NTT DOCOMO, INC.)

- R1-2005423, Discussion on NR Rel-16 UE Features, ZTE