3GPP TSG RAN WG1 Meeting #102-e                                                                    R1-200xxxx

E-meeting, August 17 - 28, 2020

Title:	[Draft] Reply LS on feasibility of UL FPTx modes and transparent TxD for certain UE implementation

Response to: 	R1-2005224 (R4-2009171)

Release:	Rel-16

Work Item:	NR-eMIMO-Core

Source:	Huawei, RAN WG1

To:	RAN WG4

CC:

Contact Person:

#### Name:	Leiming Zhang

E-mail Address:	Zhangleiming.zhang {at } huawei.com

1. Overall Description:

RAN1 thanks RAN4 for the LS. Based on the discussion, RAN1 would like to provide following answers:

In RAN4 study of ULFPTx under eMIMO WI, enabling transparent Tx diversity (TxD) was agreed at least from Rel-16, and the applicability of transparent TxD is NOT related to UE supporting or not supporting Rel-16 ULFPTx. Two possible cases were identified in RAN4 to use transparent TxD to achieve the required transmission power, i.e. for a FR1 UE having two TX branches/antennae,

- First case: Transparent TxD for UE configured with single SRS port (either with DCI_0_0 or single SRS port with DCI_0_1);

- Second case: Transparent TxD for UE configured with 2 SRS ports (FFS whether TxD is feasible in this case).

For the second case, two possible methods to transmit a multi-port SRS resource (i.e. 2Tx ports) with two PAs (PA1 and PA2) were considered, i.e.

- Method-1: SRS port-1 maps to PA1, SRS port-2 maps to PA2

- Method-2: SRS port-1 maps to PA1+PA2, SRS port-2 maps to PA1+PA2

In order to make progress of corresponding discussion of transparent TxD related issues, RAN4 would like to get some clarification from RAN1 for the feasibility of the second case.

Question 1: Whether the two mentioned methods are both feasible to transmit the full output power?

Answer 1: From RAN1’s perspective, both Method-1 and Method-2 can be supported for UL full power transmission.

For method 2, the UE will need to transmit two SRS ports on each PA for two ports case in a symbol. If the UE is 2 layer capable, it will need to transmit 2 PUSCH layers on each PA in a symbol. For method 1, each PA carries one SRS port and, in the case of non-coherent 2 layer transmission, one layer per PA; but for UEs that support coherent 2 layers PUSCH transmission, it needs to transmit two layers on each PA in a symbol.  RAN1 understand that multi-layer transmission on a PA in a symbol could increase PAPR for PUSCH and multiple SRS port transmission on a PA in a symbol could increases PAPR for SRS, but RAN1 has not evaluated yet.

Question 2: If answer is yes, which ULFPTx modes can be supported for these two methods?

Answer 2: From RAN1’s perspective, Method-1 can be supported for any UL full power transmission modes, and Method-2 can be supported at least for Mode-2 and Mode-full power (i.e., the other mode).

Furthermore, RAN4 discussed that the feasibility of different ULFPTx modes in FR2 UE. The ULFPTx mode-1 for FR2 UE has been confirmed to be feasible, while companies have different understandings for the other two modes (i.e., mode-1 and the other mode). RAN4 would like to get some clarification from RAN1 for the feasibility of the other two modes.

Question 3: Whether the ULFPTx mode-2 and the other ULPFTx mode are feasible for FR2 UE?

Answer 3: From RAN1’s perspective, UL full power modes {Mode-1, Mode-2 and Mode-full power (i.e., the other mode)} can be supported for FR2.

2. Actions:

To RAN4

ACTION: RAN1 would like to respectfully ask RAN4 to take the above information into account.

3. Date of Next RAN WG1 Meetings:

TSG-RAN WG1 Meeting #103e	26 Oct - 13 Nov 2020, 	             Online

TSG-RAN WG1 Meeting #104e	1  -  5 Mar 2021, 		Athens, GR