3GPP TSG RAN WG1 Meeting #102-e			R1-20xxxxx

e-Meeting, August 17 – 28, 2020

Title:	Draft LS on propagation delay compensation enhancements

Response to:	-

Release:	Rel-17

Study Item:	NR_IIOT_URLLC_enh

Source:	Huawei [TSG RAN WG1]

To:	TSG RAN WG2

Cc:

Contact Person:

#### Name:	Yan Cheng

E-mail Address:	chengyan.cheng@huawei.com

1. Overall Description:

RAN1 discussed propagation delay compensation enhancements and the following agreement was achieved on the representative use cases for further study on propagation delay compensation enhancements in Rel-17.

Agreements:

- Take the following use cases as the representative use cases for further study on propagation delay compensation enhancements in Rel-17.

| User-specific clock synchronicity accuracy level | Number of devices in one Communication group for clock synchronisation | 5GS synchronicity budget requirement (note) | Service area | Scenario |
| --- | --- | --- | --- | --- |
| 2 | Up to 300 UEs | ≤900 ns | ≤ 1000 m x 100 m | - Control-to-control communication for industrial controller |
| 4 | Up to 100 UEs | <1 µs | < 20 km2 | - Smart Grid: synchronicity between PMUs |

The requirement for the two representative use cases as show in the above table is the overall synchronicity budget. In order to decide whether any enhancements needed on propagation delay compensation in Rel-17 or what compensation methods we should take in RAN, we need the design target on synchronicity budget for Uu interface.

2. Actions:

To RAN2:

ACTION: 	RAN1 respectfully asks RAN2 to take the above into account for the future work, and also provide feedback on the synchronicity budget for Uu interface for the above two representative use cases.

3. Date of Next TSG-RAN WG1 Meetings:

TSG RAN WG1 Meeting #103-e                       26th October – 13th November, 2020	E-meeting

TSG RAN WG1 Meeting #104                          1st – 5th March, 2021			Athens, GR