3GPP TSG RAN WG1 Meeting #104-e 	R1-21xxxxx

e-Meeting, January 25th – February 5th, 2021

Agenda Item: 7.1

Source: Moderator (MediaTek)

Title: Summary for [104-e-NR-7.1CRs-10] Clarification on numerology for active DL and UL BWPs

Document for: Discussion and decision

# Introduction

In RAN1 #103-e meeting, the following email discussion is assigned by Chairman to discuss “[104-e-NR-7.1CRs-10] Clarification on numerology for active DL and UL BWPs”. The email thread is triggered by Issue #13 of [1] and originates from the draft CR in [2].

[104-e-NR-7.1CRs-10] Clarification on Numerology for Active DL and UL BWPs – Pei-Kai (MediaTek) by Jan 29

# Discussion

## Background

In TR38.822 “NR UE feature list (Release 15)”, one of technical components of FG6-4 is “same numerology between DL and UL per cell except for SUL at a given time”, as shown in Table 1. However, in TS38.306, there is no such description in the definition of the UE capability parameter “bwp-DiffNumerology”, as shown in Table 2.

Table 1. FG6-4 in NR UE feature list

| Index | Feature group | Components | Prerequisite |
| --- | --- | --- | --- |
| 6-4 | BWP adaptation with different numerology | 1) Up to 4 UE-specific RRC configured DL BWPs per carrier 2) Up to 4 UE-specific RRC configured UL BWPs per carrier 3) Active BWP switching by DCI and timer 4) More than one numerologies for the UE-specific RRC configured BWPs per carrier 5) Same numerology between DL and UL per cell except for SUL at a given time 6) BW of a UE-specific RRC configured BWP includes BW of the CORESET#0 (if CORESET#0 is present) and SSB for PCell/PSCell (if configured) and BW of the UE-specific RRC configured BWP includes SSB for SCell if there is SSB on SCell | 6-1 |

Table 2. Definition of the UE capability parameter “bwp-DiffNumerology”

| Definitions for parameters | Per | M | FDD-TDD DIFF | FR1-FR2 DIFF |
| --- | --- | --- | --- | --- |
| bwp-DiffNumerology Indicates whether the UE supports BWP adaptation up to 4 BWPs with the different numerologies, via DCI and timer. For the UE capable of this feature, the bandwidth of a UE-specific RRC configured DL BWP includes the bandwidth of the CORESET#0 (if CORESET#0 is present) and SSB for PCell and PSCell (if configured). For SCell(s), the bandwidth of the UE-specific RRC configured DL BWP includes SSB, if there is SSB on SCell(s). | Band | No | No | No |

For unpaired spectrum (e.g. TDD spectrum), if active DL BWP and active UL BWP have different numerology, UE will need longer guard time (potentially equal to DCI/timer-based BWP switching delay defined in TS38.133) for DL-UL switch to process numerology change. Since there is DL/UL BWP pairing and either DCI format 1_1 or DCI format 0_1 can be used to switch both DL BWP and UL BWP in unpaired spectrum, it’s feasible for the network to guarantee that a UE’s active DL BWP and active UL BWP share the same numerology in unpaired spectrum.

For paired spectrum (e.g. FDD spectrum), there is no strong needs and it’s also difficult for the network to guarantee that a UE’s active DL BWP and active UL BWP share the same numerology because there is no DL/UL BWP pairing and DCI format 1_1 (or DCI format 0_1) can be used to switch DL (or UL) BWP only.

For FG6-1 “Basic BWP operation with restriction”, it’s also not clear whether a UE can assume that its active DL BWP and active UL BWP share the same numerology at a given time, especially for unpaired spectrum.

## Company view

Q1: Do you think any CR needed for RAN1 specifications to clarify the following issues?

- Whether UEs supporting FG6-1 can assume the active DL BWP and the active UL BWP share the same numerology

- Whether UEs supporting FG6-4 can assume the active DL BWP and the active UL BWP share the same numerology

| Company | Comments |
| --- | --- |
| vivo | FG6-1 is mandatory without capability signalling, there was a general discussion in RANP that whether move all the “mandatory without capability signalling” features from TR38.802 to the core specification, unfortunately no conclusion has been reached yet. FG6-4 is optional feature, we are open to capture it in RAN1 specification if majority companies are willing to do so. But we think at least a 38.306 CR is needed. |
| CATT | We are open to clarify in RAN1 specifications. |
| Apple | CR to 38.306 might be sufficient. But we are also okay to have RAN1 CR, for example, to 38.213. But we want to clarify the scenario for paired spectrum (FDD), does UE have to support different numerology between the active DL BWP and the active UL BWP? What is the RAN1 understanding regarding numerology for the active DL BWP and the active UL BWP for FDD band? |
| ZTE | From our perspective, we didn’t see the need to update RAN1 specification. RAN1 specification mainly defines functionalities, it seems more appropriate to have the description as a RAN1 conclusion or update the 306 spec. |
| Intel | We share same view that same numerology between active DL and UL BWPs should apply for unpaired spectrum. On the other hand, it is possible to have different numerology for the active DL and UL BWPs for paired spectrum. - On FG 6-1, it is fine to allow different numerology for the RRC configured active DL and UL BWPs. In fact, UE needs to handle the different numerology for initial DL and UL BWPs, since a SCS for initial UL BWP could be separately configured in by subcarrierSpacing in BWP-UplinkCommon. - FG 6-4 stands for a powerful UE capability, so it is fine to allow different numerology for the RRC configured active DL and UL BWPs |
| Nokia, NSB | In our view, there is no reason why RAN1 specs should hard-code something that is a matter of UE capability definition. The limitation on the UE needing to be provided with the same SCS for DL and UL can be seen in the UE features. |
| Qualcomm | We do not think any CR is needed for RAN1 specification. The current spec should be clear enough. |
| OPPO | It is better to capture the related description in TS 38.308. But we are also open to some clarification in RAN1 spec Meanwhile, our understanding is that the numerology of active UL BWP and active DL BWP should be the same for both TDD and FDD. As in FG 6-2/6-3, there is a component “4) Same numerology for all the UE-specific RRC configured BWPs per carrier”. It means all BWP (no matter UL and DL) in FG6-1/6-2/6-3 should have the same numerology and doesn’t differentiate FDD from TDD. Component 5 of FG 6-4 should also applicable for both TDD and FDD. |
| Spreadtrum | Yes for CR in RAN1 for both of 6-1 and 6-4, including paired and unpaired spectrum. |
| Samsung | Our understanding is that if the UE reports FG 6-4, the BS should employ the same numerology across all the UL and DL BWPs in order to accommodate paired spectrum. This obviously puts some restriction in the implementation. But it doesn’t entail further clarification in the specification. |
| Huawei, HiSilicon | We don’t think RAN1 specification change is needed, while we are open to clarify this. Our understanding is same SCS restriction applied to both FDD and TDD. |
| Ericsson | We do not see need for RAN1 spec clarification. |

Q2: Do you think any CR needed to carry over RAN1 agreements for the UE capability of “bwp-DiffNumerology” in TS38.306 to clarify the following issue?

- Whether UEs supporting FG6-4 can assume the active DL BWP and the active UL BWP share the same numerology

| Company | Comments |
| --- | --- |
| vivo | We support to capture it in 38.306 |
| CATT | Yes, we think CR to 38.306 is needed. |
| Apple | We think CR to 38.306 is needed |
| ZTE | We don’t see the need of RAN1 CR. We prefer to have a RAN1 conclusion or update the 306 spec. |
| Intel | Yes, we think a CR is beneficial |
| Nokia, NSB | We don’t see the need for a RAN1 CR. 38.306 could be clarified in this respect, if companies feel this is necessary |
| Qualcomm | We don’t think TS 38.306 update is needed. It should be already clear based on TR 38.822. It is understood that UL BWP and DL BWP should have the same numerology for Rel-15, so no conclusion on 38.306 update needs to be made. If anything is to be agreed, it should be for the same neurology between UL BWP and DL BWP for both unpaired and paired spectrum. In particular, FG 6-4 mentioned “Same numerology between DL and UL per cell” unconditionally for both paired spectrum and unpaired spectrum. Different numerology between UL BWP and DL BWP for paired spectrum should be a more advanced feature than the optional FG 6-4. Without such a feature defined, it should be assumed that UE does not support different numerology between UL BWP and DL BWP for paired spectrum for Rel-15. |
| OPPO | OK to clarify in TS 38.306 that the numerology of active UL BWP and active DL BWP should be the same for both TDD and FDD |
| Spreadtrum | Yes for CR in 38.306 for both of 6-1 and 6-4, including paired and unpaired spectrum. |
| Samsung | We believe no specification change in RAN1 is required. |
| Huawei, HiSilicon | No RAN1 CR needed. |
| Ericsson | Whether/how to update 38.806 can be discussed in RAN2. We do not see need for RAN1 CR. |

Q3: Do you agree to send an LS from RAN1 to RAN2 if any potential RAN2 actions (e.g. potential CR for TS38.306) concluded?

| Company | Comments |
| --- | --- |
| vivo | We support to sent an LS to RAN2 to request a 38.306 CR. |
| CATT | Yes, we agree to send an LS to RAN2 to request 38.306 update. |
| Apple | We think LS is helpful |
| ZTE | If only a RAN1 conclusion is agreed, then no LS is needed. If majority companies prefer to update 306 spec, then an LS can be sent. |
| Intel | We agree to send LS to RAN2 for the update of 38.306 |
| Nokia, NSB | We are OK with an LS to RAN2 if the RAN1 agreements suggest a clarification to 38.306 (or other RAN2 action) |
| Qualcomm | We don’t think the LS is needed. If LS is sent, it should clarify the same numerology for paired spectrum as we mentioned under Q2. |
| OPPO | Support to send an LS to RAN2 for the clarification that the numerology of active UL BWP and active DL BWP should be the same for both TDD and FDD |
| Spreadtrum | We agree to send LS to RAN2. |
| Samsung | We are OK with sending an LS to RAN2. |
| Huawei, HiSilicon | Fine to tell RAN2. |
| Ericsson | We do not see need for RAN1 LS as FG 6-4 has been communicated to RAN2 already |

## Summary

There is no consensus to have a RAN1 CR to clarify the issues raised in R1-2100610. But majority companies are OK to send a LS from RAN1 to RAN2 for a CR on TS38.306 for FG6-4.

Companies’ views for Q1

- OK for RAN1 CR: 5 companies

- No RAN1 CR: 9 companies

Companies’ views for Q2

- OK for RAN2 CR on TS38.306: 13 companies

- No RAN2 CR on TS38.306: 1 company

Companies’ views for Q3

- OK to send LS from RAN1 to RAN2: 12 companies

- No LS from RAN1 to RAN2: 2 company

## Moderator’s proposal

Proposal:

- It’s RAN1’s understanding that for both paired spectrum and unpaired spectrum UE may assume the same SCS and CP length for its active DL BWP and active UL BWP in a serving cell except for SUL at a given time

  - No RAN1 CR is needed

- Send LS to RAN2 for the above conclusion and it’s up to RAN2 to decide whether/how to clarify the definition of UE capability parameter bwp-DiffNumerology in Rel-15 & Rel-16 based on the above conclusion

# Conclusion

To be added after discussion.

# Reference

- R1-2101768, “RAN1#104-e preparation phase initial summary on NR Rel-15 CRs”, Ad-hoc Chair (Samsung)

- R1-2100610, “Clarification on Numerology for Active DL and UL BWPs”, MediaTek Inc.